3GPP TSG-RAN4 Meeting #111	R4-2407076

# Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 1188 | rev |  | Current version: | 16.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-3 on enabling missing UL feature support for inter-band CA between FR1 and FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R16_2BDL_xBUL-Core |  |  |  |  |  | Date: |  |  | 2024-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Configurations for inter-band CA and NR-DC between FR1 and FR2 have been introduced since Rel-15. However, there is no explicit indication on UL MIMO support for FR1 carrier within the configurations in the current RAN4 specifications. Considering that for inter-band combinations between FR1 and FR2, UL RF requirements are independently specified for FR1 and FR2, and UL MIMO in FR1 has been supported since Rel-15, and yet many existing UEs are already capable of supporting FR1 UL MIMO under inter-band CA or NR-DC between FR1 and FR2 where the feature is also supported by the current RAN2 signaling design, it would be desired to enable the missing FR1 UL MIMO feature in inter-band CA and NR-DC between FR1 and FR2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the specifications for inter-band CA between FR1 and FR2 by replacing the explicit feature indication in UE maximum output power sub-clause with a general referencing to FR1 and FR2 specifications respectively such that all the existing independent UL features between FR1 and FR2 can be implicitly supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR1 UL MIMO feature is missing and cannot be supported for inter-band CA and NR-DC between FR1 and FR2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of changed sections >>>

#### 6.2A.1.1	Inter-band CA between FR1 and FR2

Table 6.2A.1.1-1: Void

For inter-band NR CA in FR1 and FR2 combined, the UE shall meet each transmitter power requirement specified in TS 38.101-1 [2] and TS 38.101-2 [3] for each frequency range independently.

<<< End of changed sections >>>