3GPP TSG-WG4 Meeting #111	R4-2408204

# Fukuoka, Japan, 20 - 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-1 | CR | 0052 | rev |  | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.176-1: Correction of test tolerance value for IAB-MT total power dynamic range (rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2024-5-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TT for IAB-MT total power dynamic range is specified for ±0.7 dB for BW ≤ 40MHz, and ±1.0 dB for 40MHz < f ≤ 100MHz. An error of condition exists above. Condition for ±1.0 dB TT shall be 40MHz < BW ≤ 100MHz |  |  |  |  |  |  |  |  |
| Summary of change: |  | In table C.1-1, “±1.0 dB for 40MHz < f ≤ 100MHz” was corrected to “±1.0 dB for 40MHz < BW ≤ 100MHz” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Condition for IAB-MT total power dynamic range TT value does not make sense. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

C.1	Measurement of transmitter

Table C.1-1: Derivation of test requirements (Transmitter tests)

| Test | Minimum requirement in TS 38.174 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 IAB output power | See TS 38.174 [2], clause 6.2 | Normal and extreme conditions: 0.7 dB, f ≤ 3.0 GHz 1.0 dB, 3.0 GHz < f ≤ 6GHz (Note 1) | Formula: Upper limit + TT, Lower limit - TT |
| 6.3.1 IAB-DU Output power dynamics | See TS 38.174 [2], clause 6.3.1 | 0.4 dB | Formula: Total power dynamic range – TT (dB) |
| 6.3.2 IAB-MT Total power dynamic range | See TS 38.174 [2], clause 6.3.2 | ±0.7 dB, BW ≤ 40MHz ±1.0 dB, 40MHz < BW ≤ 100MHz | Formula: Total power dynamic range – TT (dB) |
| 6.4.1 Transmitter OFF power | See TS 38.174 [2], clause 6.4.1 | 2.0 dB, f ≤ 3.0 GHz 2.5 dB, 3.0 GHz < f ≤ 6 GHz (Note 1) | Formula: Minimum Requirement + TT |
| 6.4.2 Transient period | See TS 38.174 [2], clause 6.4.2 | N/A |  |
| 6.5.2.1 IAB-DU Frequency error | See TS 38.174 [2], clause 6.5.1.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.5.2.2 IAB-MT Frequency error | See TS 38.174 [2], clause 6.5.1.2 | ±15 Hz, f ≤ 3.0GHz ±36 Hz, f > 3.0GHz | Formula: Frequency Error limit + TT |
| 6.5.4 Modulation quality (EVM) | See TS 38.174 [2], clause 6.5.2 | 1% | Formula: EVM limit + TT |
| 6.5.4 Time alignment error | See TS 38.174 [2], clause 6.5.3.1 | 25ns | Formula: Time alignment error limit + TT+ TT |
| 6.6.2 Occupied bandwidth | See TS 38.174 [2], clause 6.6.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 6.6.3 Adjacent Channel Leakage Power Ratio (ACLR) | See TS 38.174 [2], clause 6.6.3 | ACLR/CACLR: BW ≤ 20MHz: 0.8dB BW > 20MHz: 1.2 dB Absolute ACLR/CACLR: 0 dB | Formula: ACLR Minimum Requirement - TT Absolute limit +TT |
| 6.6.4 Operating band unwanted emissions | See TS 38.174 [2], clause 6.6.4 | Offsets < 10MHz 1.5 dB, f ≤ 3.0GHz 1.8 dB, 3.0GHz < f ≤ 6GHz (Note 1) Offsets ≥ 10MHz 0dB | Formula: Minimum Requirement + TT |
| 6.6.5.5.1 General transmitter spurious emissions requirements Category A | See TS 38.174 [2], clause 6.6.5.2.1 | 0dB | Formula: Minimum Requirement + TT |
| 6.6.5.5.1 General transmitter spurious emissions requirements Category B | See TS 38.174 [2], clause 6.6.5.2.1 | 0dB | Formula: Minimum Requirement + TT |
| 6.6.5.5.2 Additional spurious emissions requirements | See TS 38.174 [2], clause 6.6.5.2.2 | 0dB | Formula: Minimum Requirement + TT |
| 6.6.5.5.3 Co-location with other base stations | See TS 38.174 [2], clause 6.6.5.2.3 | 0dB | Formula: Minimum Requirement + TT |
| 6.7 Transmitter intermodulation | See TS 38.174 [2], clause 6.7 | 0dB | Formula: Ratio + TT |
| NOTE 1: TT values for 4.2 GHz < f ≤ 6.0 GHz apply for IAB operates in licensed spectrum only. NOTE 2: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

--------------End of change-------------