3GPP TSG-RAN4 Meeting #111		R4-2409755

Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4601 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_redcap-Core) Formal CR to Rel-18 TS 38.133: on eDRX non-RedCap for INACTIVE mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing requirements for eDRX in INACTIVE mode for non-RedCap UEs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rel-17 eDRX for INACTIVE mode (without PTW) - eDRX for non-RedCap - eDRX for serving cell --> [complete] - eDRX for neighbouring cells --> [complete] - eDRX for relaxed RRM neighbouring cells --> [Missing] - eDRX for relaxed RRM based based low mobilityand/or rel-16 not-at-cell edge --> [Missing] - Editor’s Note: requirements with relaxed measurement criterion can be further revisited. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be missing requirements for relaxed eDRX. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.3, 5.1.2.4, 5.1.2.5, and new clauses 5.1.2.8, 5.1.2.9, 5.1.2.10, 5.1.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 5.1.2.3	Measurements of intra-frequency NR cells

The requirements in this clause apply when UE is configured with eDRX_IDLE, otherwise the requirements in clause 4.2.2.3 shall apply. The requirements in clause 4.2.2.9 apply for UE configured with relaxed measurement criterion when UE is not configured with eDRX_IDLE.

When UE is configured with eDRX_IDLE and UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, the requirements defined in section 4.2.2.3 shall apply with Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra defined in Table 5.1.2.3-1 and Table 5.1.2.3-2.

When UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2.2.3 shall apply with Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra defined in Table 5.1.2.3-3 and Table 5.1.2.3-4.

Table 5.1.2.3-1: Tdetect, NR_Intra, Tmeasure, NR_Intra and Tevaluate, NR_Intra for UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra [s] (number of DRX or INACTIVE eDRX cycles) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x M2 (36 x M2) | 1.28 x M2 (4 x M2) | 5.12 x M2 (16 x M2) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
|  | 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) |
|  | 10.24 | 235.52 (23) | 10.24 (1) | 30.72 (3) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |

Table 5.1.2.3-2: Tdetect,NR_Intra, Tmeasure, NR_Intra and Tevaluate,NR_Intra for UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 8 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
|  | 0.64 | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
|  | 1.28 | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
|  | 2.56 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
|  | 5.12 | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 15.36 x N1 (3 x N1) |
|  | 10.24 | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 30.72 x N1 (3 x N1) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Table 5.1.2.3-3: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle, (Frequency range FR1)

| eDRX_IDLE cycle and eDRX INACTIVE cycle length [s] | RANDRX cycle length [s] | eDRX INACTIVEPTW length [s] (number of 1.28s periods) | Tdetect,NR_Intra [s] (number of RAN DRX cycles) | Tmeasure,NR_Intra [s] (number of RAN DRX cycles Note 3) | Tevaluate,NR_Intra [s] (number of RAN DRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 20.48 ≤ eDRX_INACTIVE cycle length ≤10485.76 in the first column | 0.32 | ≥[1.28] ([1]) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x M2 (1 x M2) | 0.64 x M2 (2 x M2) |
|  | 0.64 | ≥[1.28] ([1]) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥[2.56] ([2]) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥[5.12] ([4]) |  | 2.56 (1) | 5.12 (2) |
| Note 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 2: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 3: eDRX INACTIVE PTW in this table is RAN configured PTW. Note 4: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 5: The lower bound of PTW length is derived based on Tevaluate,NR_Intra*DRX_cycle1.28*1.28. Note 6: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Table 5.1.2.3-4: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle, (Frequency range FR2)

| eDRX_IDLE cycle and eDRX INACTIVE cycle length [s] | RAN DRX length [s] | eDRX INACTIVEPTW length [s] (number of 1.28s periods) | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of RAN DRX cycles) | Tmeasure,NR_Intra [s] (number of RAN DRX cycles Note 5) | Tevaluate,NR_Intra [s] (number of RAN DRX cycles Note 5) |
| --- | --- | --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 20.48 ≤ eDRX_INACTIVE cycle length ≤10485.76 in the first column | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| Note 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 2: Applies for UE of all power class. Note 3: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 4: eDRX INACTIVE PTW in this table is RAN configured PTW. Note 5: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 6: The lower bound of PTW length is derived based on Tevaluate,NR_Intra*DRX_cycle1.28*1.28. Note 7: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |  |

When the UE transitions between any two states when changing eDRX_IDLE cycle length, eDRX_INACTIVE cycle length, INACTIVE RAN DRX length or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

#### 5.1.2.4	Measurements of inter-frequency NR cells

When UE is configured with eDRX_IDLE and UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, the requirements defined in section 4.2.2.4 shall apply with Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter defined in Table 5.1.2.4-1 and Table 5.1.2.4-2, and with Kcarrier defined as follows.

When UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2.2.4 shall apply with Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter defined in Table 5.1.2.4-3 and Table 5.1.2.4-4.

Table 5.1.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x 1.5 (36 x 1.5) | 1.28 x 1.5 (4 x 1.5) | 5.12 x 1.5 (16 x 1.5) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
|  | 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) |
|  | 10.24 | 235.52(23) | 10.24 (1) | 30.72 (3) |

Table 5.1.2.4-2: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
|  | 0.64 | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
|  | 1.28 | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
|  | 2.56 | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
|  | 5.12 | 3 | 117.76 x N1 (23 x N1) | 5.12 x N1 (1 x N1) | 15.36 x N1 (3 x N1) |
|  | 10.24 | 3 | 235.52 x N1 (23 x N1) | 10.24 x N1 (1 x N1) | 30.72 x N1 (3 x N1) |

Table 5.1.2.4-3: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle (Frequency range FR1)

| eDRX_IDLE cycle and eDRX_INACTIVE cycle length [s] | RAN DRX cycle length [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48≤eDRX_IDLE cycle length ≤ 10485.76 20.48 ≤eDRX_INACTIVE cycle length ≤ 10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x 1.5 (1 x 1.5) | 0.64 x 1.5 (2 x 1.5) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| Note 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 2: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 3: eDRX INACTIVE PTW in this table is RAN configured PTW [1]. Note 4: The number of DRX cycles in this table is given for the DRX cycles within RAN PTWs. Note 5: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 6: The lower bound of PTW length is derived based on Tevaluate,NR_Inter*DRX_cycle1.28*1.28. |  |  |  |  |  |

Table 5.1.2.4-4: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE cycle (Frequency range FR2)

| eDRX_IDLE cycle and eDRX_INACTIVE cycle length [s] | RAN DRX cycle length [s] | eDRX INACTIVE PTW length [s] (number of 1.28s periods) | Scaling Factor (N1) Note1 | Tdetect,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- | --- |
| 20.48≤eDRX_IDLE cycle length ≤ 10485.76 20.48 ≤eDRX_INACTIVE cycle length ≤ 10485.76 | 0.32 | ≥5.12 (4) | 8 | eDRX_cycle_length×23×N1PTW/DRX_cycle_length (23 x N1) | 0.32 x N1 (1 x N1) | 0.64 x N1 (2 x N1) |
|  | 0.64 | ≥6.4 (5) | 5 |  | 0.64 x N1 (1 x N1) | 1.28 x N1 (2 x N1) |
|  | 1.28 | ≥10.24 (8) | 4 |  | 1.28 x N1 (1 x N1) | 2.56 x N1 (2 x N1) |
|  | 2.56 | ≥15.36 (12) | 3 |  | 2.56 x N1 (1 x N1) | 5.12 x N1 (2 x N1) |
| Note 1: Applies for UE of all power class. Note 2: The number of DRX cycles in this table is given for the DRX cycles within RAN PTWs. Note 3: The eDRX_INACTIVE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter*DRX_cycle1.28*1.28. Note 5: When eDRX_INACTIVE=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX _INACTIVE cycles. Note 6: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. Note 7: The number of RAN DRX cycles in this table is given for the DRX cycles within RAN configured PTWs. Note 8: eDRX INACTIVE PTW in this table is RAN configured PTW [1]. |  |  |  |  |  |  |

When the UE transitions between any two states when being configured with eDRX_INACTIVE, being configured with eDRX_INACTIVE cycle, changing eDRX_INACTIVE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

If UE is not configured to perform PRS measurement, or if UE is configured to perform PRS measurement and supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier in clause 4.2.2.4 shall apply regardless of whether the serving cell is subject to CCA or not.

If UE is configured to perform PRS measurement but does not support parallelPRS-MeasRRC-Inactive-r17, Kcarrier is replaced with Kcarrier + 1, regardless of whether the serving cell is subject to CCA or not.

If UE is not configured to perform PRS measurement, or if UE is configured to perform PRS measurement and supports parallelPRS-MeasRRC-Inactive-r17, the requirements in clause 4.2.2.10 shall apply regardless of whether the serving cell is subject to CCA or not for UE configured with relaxed measurement criterion.

If UE is configured to perform PRS measurement but does not support parallelPRS-MeasRRC-Inactive-r17, the requirements in clause 4.2.2.10 shall apply with Kcarrier being replaced with Kcarrier + 1, regardless of whether the serving cell is subject to CCA or not for UE configured with relaxed measurement criterion.

#### 5.1.2.5	Measurements of inter-RAT E-UTRAN cells

When UE is configured with eDRX_IDLE and UE is not configured with eDRX by [ran-ExtendedPagingCycle-r18] or eDRX-AllowedInactive-r18 is not signalled in SIB1, the requirements defined in section 4.2.2.5 shall apply with Tdetect, EUTRAN, Tmeasure, EUTRAN and Tevaluate, EUTRAN defined in Table 5.1.2.5-1.

When UE is configured with eDRX by [ran-ExtendedPagingCycle-r18] and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2B.2.5 shall apply with Tdetect, EUTRAN, Tmeasure, EUTRAN and Tevaluate, EUTRAN  defined in Table 5.1.2.5-2.

Table 5.1.2.5-1: Tdetect, EUTRAN, Tmeasure, EUTRAN and Tevaluate, EUTRAN for inactive UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect, EUTRAN [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure, EUTRAN [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate, EUTRAN [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | 11.52 x 1.5 (36 x 1.5) | 1.28 x 1.5 (4 x 1.5) | 5.12 x 1.5 (16 x 1.5) |
|  | 0.64 | 17.92 (28) | 1.28 (2) | 5.12 (8) |
|  | 1.28 | 32 (25) | 1.28 (1) | 6.4 (5) |
|  | 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
|  | 5.12 | 117.76 (23) | 5.12 (1) | 15.36 (3) |
|  | 10.24 | 235.52(23) | 10.24 (1) | 30.72 (3) |

Table 5.1.2.5-2: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN for UE configured with eDRX_IDLE cycle and eDRX_INACTIVE ≥ 20.48s, (Frequency range FR1)

| eDRX_IDLE cycle and eDRX_Inactive cycle length [s] | RAN DRX cycle length [s] | eDRX Inactive PTW length [s] (number of 1.28s periods) | Tdetect,EUTRAN [s] (number of RAN DRX cycles Note 3) | Tmeasure,EUTRAN [s] (number of RAN DRX cycles Note 3) | Tevaluate,E-UTRAN [s] (number of RAN DRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 20.48 ≤ eDRX_INACTIVE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 (1) | 0.64 (2) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| NOTE 1: RAN DRX cycle in this table is UE specific DRX value configured by RRC specified in [1]. NOTE 2: The number of RAN DRX cycles in this table is given for the RAN DRX cycles within RAN configured PTWs. NOTE 3: eDRX Inactive PTW in this table is RAN configured PTW [1]. NOTE 4: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [34]. NOTE 5: The eDRX_INACITVE cycle lengths are ran-ExtendedPagingCycle-r18 as specified in [2] NOTE 6: The lower bound of PTW length is derived based on Tevaluate,E-UTRAN*DRX_cycle1.28*1.28. NOTE 7: When eDRX=20.48s and DRX=0.32s, UE is allowed to perform cell evaluation within PTW in every 2 eDRX cycles. |  |  |  |  |  |

When the UE transitions between any two states when changing eDRX_IDLE cycle length, eDRX_INACTIVE cycle length, INACTIVE RAN DRX length or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

<End of Change 1>

<Start of Change 2>

#### 5.1.2.8	Minimum requirement at transitions

The requirements in sub-clause 4.2.2.8 shall apply.

#### 5.1.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion

The requirements in clause 4.2.2.9 apply for UE configured with relaxed measurement criterion except when UE is configured with eDRX_IDLE cycle greater than 10.24 s and UE has fulfilled low mobility criterion or not-at-cell edge criterion.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR1 and FR2, and UE has fulfilled low mobility criterion or not-at-cell edge criterion, then the requirements in Table 5.1.2.9-1 and Table 5.1.2.9-2 respectively apply provided that eDRX cycle is ≤ [10485.76] sec.

Table 5.1.2.9-1: Tdetect,NR_Intra_Relax, Tmeasure,NR_Intra_Relax and Tevaluate,NR_Intra_Relax for inactive UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect,NR_Intra_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra_Relax [s] (number of DRX or INACTIVE eDRX cycles) |
| --- | --- | --- | --- | --- |
| 20.48 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 | 11.52 x M2 x K1 (36 x M2 x K1) | 1.28 x M2 x K1 (4 x M2 x K1) | 5.12 x M2 x K1 (16 x M2 x K1) |
|  | 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
|  | 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
|  | 2.56 | 58.88 x K1 (23 x K1) | 2.56 x K1 (1 x K1) | 7.68 x K1 (3 x K1) |
|  | 5.12 | 117.76 x K1 (23 x K1) | 5.12 x K1 (1 x K1) | 15.36 x K1 (3 x K1) |
|  | 10.24 | 235.52 x K1 (23) | 10.24 x K1 (1 x K1) | 30.72 x K1 (3 x K1) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |

Table 5.1.2.9-2: Tdetect,NR_Intra_Relax, Tmeasure,NR_Intra_Relax and Tevaluate,NR_Intra_Relax for inactive UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Intra_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Intra_Relax [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 | 8 | 11.52 x N1 x M2 x K1 (36 x N1 x M2 x K1) | 1.28 x N1 x M2 x K1 (4 x N1 x M2 x K1) | 5.12 x N1 x M2 x K1 (16 x N1 x M2 x K1) |
|  | 0.64 | 5 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
|  | 1.28 | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
|  | 2.56 | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
|  | 5.12 | 3 | 117.76 x N1 x K1 (23 x N1 x K1) | 5.12 x N1 x K1 (1 x N1 x K1) | 15.36 x N1 x K1 (3 x N1 x K1) |
|  | 10.24 | 3 | 235.52 x N1 x K1 (23 x N1 x K1) | 10.24 x N1 x K1 (1 x N1 x K1) | 30.72 x N1 x K1 (3 x N1 x K1) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

#### 5.1.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion

The requirements in clause 4.2.2.10 apply for UE configured with relaxed measurement criterion except when UE is configured with eDRX_IDLE cycle greater than 10.24 s and UE has fulfilled or low mobility criterion or not-at-cell edge criterion.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR1 and FR2, and UE has fulfilled low mobility criterion or not-at-cell edge criterion, then the requirements in Table 5.1.2.10-1 and Table 5.1.2.10-2 respectively apply provided that eDRX cycle is ≤ [10485.76] sec.

Table 5.1.2.10-1: Tdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax for inactive UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect,NR_Inter_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter_Relax [s] (number of DRX or INACTIVE eDRX cycles) |
| --- | --- | --- | --- | --- |
| 20.48 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 | 11.52 x M2 x K1 (36 x M2 x K1) | 1.28 x M2 x K1 (4 x M2 x K1) | 5.12 x M2 x K1 (16 x M2 x K1) |
|  | 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
|  | 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
|  | 2.56 | 58.88 x K1 (23 x K1) | 2.56 x K1 (1 x K1) | 7.68 x K1 (3 x K1) |
|  | 5.12 | 117.76 x K1 (23 x K1) | 5.12 x K1 (1 x K1) | 15.36 x K1 (3 x K1) |
|  | 10.24 | 235.52 x K1 (23) | 10.24 x K1 (1 x K1) | 30.72 x K1 (3 x K1) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |

Table 5.1.2.10-2: TTdetect,NR_Inter_Relax, Tmeasure,NR_Inter_Relax and Tevaluate,NR_Inter_Relax for inactive UE configured with eDRX_IDLE cycle, (Frequency range FR2)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Inter_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure,NR_Inter_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate,NR_Inter_Relax [s] (number of DRX or eDRX INACTIVE cycles) |
| --- | --- | --- | --- | --- | --- |
| 20.48 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 | 8 | 11.52 x N1 x M2 x K1 (36 x N1 x M2 x K1) | 1.28 x N1 x M2 x K1 (4 x N1 x M2 x K1) | 5.12 x N1 x M2 x K1 (16 x N1 x M2 x K1) |
|  | 0.64 | 5 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
|  | 1.28 | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
|  | 2.56 | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
|  | 5.12 | 3 | 117.76 x N1 x K1 (23 x N1 x K1) | 5.12 x N1 x K1 (1 x N1 x K1) | 15.36 x N1 x K1 (3 x N1 x K1) |
|  | 10.24 | 3 | 235.52 x N1 x K1 (23 x N1 x K1) | 10.24 x N1 x K1 (1 x N1 x K1) | 30.72 x N1 x K1 (3 x N1 x K1) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |  |

#### 5.1.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion

The requirements in clause 4.2.2.11 apply for UE configured with relaxed measurement criterion except when UE is configured with eDRX_IDLE cycle greater than 10.24 s and UE has fulfilled low mobility criterion or not-at-cell edge criterion.

If the UE is configured with eDRX_IDLE cycle greater than 10.24 s in FR1 and FR2, and UE has fulfilled low mobility criterion or not-at-cell edge criterion, then the requirements in Table 5.1.2.11-1 apply provided that eDRX cycle is ≤ [10485.76] sec.

Table 5.1.2.11-1: Tdetect, EUTRAN_Relax, Tmeasure, EUTRAN_Relax and Tevaluate, EUTRAN_Relax for inactive UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length [s] | Tdetect, EUTRAN_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tmeasure, EUTRAN_Relax [s] (number of DRX or eDRX INACTIVE cycles) | Tevaluate, EUTRAN_Relax [s] (number of DRX or INACTIVE eDRX cycles) |
| --- | --- | --- | --- | --- |
| 20.48 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 | 11.52 x M2 x K1 (36 x M2 x K1) | 1.28 x M2 x K1 (4 x M2 x K1) | 5.12 x M2 x K1 (16 x M2 x K1) |
|  | 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
|  | 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
|  | 2.56 | 58.88 x K1 (23 x K1) | 2.56 x K1 (1 x K1) | 7.68 x K1 (3 x K1) |
|  | 5.12 | 117.76 x K1 (23 x K1) | 5.12 x K1 (1 x K1) | 15.36 x K1 (3 x K1) |
|  | 10.24 | 235.52 x K1 (23 x K1) | 10.24 x K1 (1 x K1) | 30.72 x K1 (3 x K1) |
| Note1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion or fulfilling the cellEdgeEvaluation [2] criterion. |  |  |  |  |

<End of Change 2>