3GPP TSG-RAN WG4 Meeting # 111				                                               R4-2407813

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2259 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_RF_FR1_enh-Core) CR to 38.101-1 R18 corrections on Pcmax tolerance for intra-band contiguous CA with UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current spec, the same Pcmax tolerance is defined for intra-band UL contiguous CA with UL MIMO and without UL MIMO. But based on the requirement for UL MIMO, the tolerance of lower bound Pcmax for UL MIMO is more relaxed compared with single carrier case by considering the tolerance for UL MIMO comes from 2Tx link. If follow the same approach, the tolerance of lower bound Pcmax for intra-band CA with UL MIMO shall also have some relaxations compared with intra-band CA without UL MIMO. In addition, it could be found the requirement for UL MIMO for intra-band CA case is even more stringent compared with UL MIMO for single carrier case, which is obviously unreasonable. The detail analysis could be seen from R4-2401180. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the table on PCMAX tolerance for intra-band UL contiguous CA with UL MIMO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Pcmax tolerance requirements for intra-band UL contiguous CA with UL MIMO is too stringent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change>

6.2H.1.4	Configured transmitted power for intra-band UL contiguous CA with UL MIMO

For UE supporting intra-band UL contiguous CA with UL MIMO, the transmitted power is configured per each UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in clause 6.2A.4.1.1 shall apply to UE supporting intra-band UL contiguous CA with UL MIMO, where

-	ΔPPowerClass,CA and ∆TC,c are specified in clause 6.2A.4 unless otherwise stated;

-	PPowerClass,CA is the maximum UE power specified in Table 6.2H.1.1-1 without taking into account the tolerance;

-	MPR, AMPR is specified in clause 6.2H.1.2 and 6.2H.1.3;

The measured configured maximum output power PUMAX over all serving cells shall be within the following bounds:

PCMAX_L  –  MAX{TL, T LOW(PCMAX_L)}  ≤  PUMAX  ≤  PCMAX_H  +  T HIGH(PCMAX_H)

where TLOW(PCMAX_L) and THIGH(PCMAX_H) are defined as the tolerance and applies to PCMAX_L and PCMAX_H separately, while TL is the absolute value of the lower tolerance in Table 6.2 H.1.1-1 for the applicable operating band.

For UE supporting intra-band UL contiguous CA with UL MIMO, the tolerance is specified in Table 6.2H.1.4-1.

Table 6.2H.1.4-1: PCMAX tolerance for intra-band UL contiguous CA with UL MIMO

| PCMAX (dBm) |  | Tolerance TLOW(PCMAX_L) (dB) |  | Tolerance THIGH(PCMAX_H) (dB) |  |
| --- | --- | --- | --- | --- | --- |
| 23 ≤ PCMAX ≤ 26 |  | 3.0 |  | 2.0 |  |
| 22 ≤ PCMAX < 23 |  | 5.0 |  | 2.0 |  |
| 21 ≤ PCMAX < 22 |  | 5.0 |  | 3.0 |  |
| 20 ≤ PCMAX < 21 |  | 5.0 |  | 4.0 |  |
| 16 ≤ PCMAX < 20 |  | 5.0 |  |  |  |
| 11 ≤ PCMAX < 16 |  | 6.0 |  |  |  |
| -40 ≤ PCMAX < 11 |  | 7.0 |  |  |  |

6.2H.2	Void

# <End of change >