3GPP TSG-RAN4 Meeting #111		R4-2409574

# Fukuoka, Japan, 20th  – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4575 | rev |  | Current version: | 16.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_unlic-Perf] Correction for NR-U test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2024-20-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Io values for test config 1, 2 are not defined for 40 MHz CBW |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Io values for test config 1, 2 for 40 MHz CBW |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN5 will not be able to test the R16 NR-U test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.11.2.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE 1----------------------------

##### A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA

A.11.2.2.3.2.1	Test Purpose and Environment

This test is to verify RRC connection release with redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA specified in clause 6.2.3.2.3.

A.11.2.2.3.2.2	Test Parameters

Supported test configurations are shown in table A.11.2.2.3.2.2-1. The time delay is tested by using the parameters in table A.11.2.2.3.2.2-2, and A.11.2.2.3.2.2-3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The RRCRelease message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

Table A.11.2.2.3.2.2-1: Redirection from NR to NR test configurations

| Configuration | Source cell without CCA | Target cell with CCA |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD | 30 kHz SSB SCS, 40 MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Table A.11.2.2.3.2.2-2: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 | On the carrier without CCA |
|  | Neighbouring cell |  | Cell 2 | On the carrier under CCA |
| Final condition | Active cell |  | Cell 2 | On the carrier under CCA |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| DL CCA model | Dynamic channel accessNote 1, 3 |  | As specified in clause A.3.26.2.1 |  |
|  | Semi-static channel access Note 2, 3 |  |  |  |
| UL CCA model | Dynamic channel access Note 1, 3 |  | As specified in clause A.3.26.2.2 |  |
|  | Semi-static channel access Note 2,3 |  |  |  |
| T1 |  | s | 5 |  |
| T2 |  | s | ≥ Tconnection_release_redirect_NR_CCA | Tconnection_release_redirect_NR_CCA is defined in clause 6.2.3.2.3 |
| NOTE 1: For a UE supporting dynamic channel access and network configuring dynamic channel occupancy. NOTE 2: For a UE supporting semi-static channel access and network configuring semi-static channel occupancy. NOTE 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table A.11.2.2.3.2.2-3: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 2 |  |  |
| PCCA_DL for dynamic channel access Note 4,6 |  |  |  | N/A |  | PCCA_DL_1=0.75 PCCA_DL_2=0.75 | PCCA_DL_1=0.75 PCCA_DL_2=0.75 |  |
| PCCA_DL for semi-static channel access Note 5,6 |  |  |  | N/A |  | PCCA_DL=0.9375 | PCCA_DL=0.9375 |  |
| PCCA_UL for dynamic channel access Note 4,6 |  |  |  | N/A |  | 1 | 1 |  |
| PCCA_UL for semi-static channel access Note 5,6 |  |  |  | N/A |  | 1 | 1 |  |
| LCCA_DL Note 7 |  |  |  | N/A |  | 8 |  |  |
| WCCA_DL Note 7 |  |  | ms | N/A |  | Tidentify-NR_CCA |  |  |
| Duplex mode |  | Config 1 |  | FDD |  | TDD |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  | TDDConf.1.1 CCA |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  | TDDConf.1.1 CCA |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  | TDDConf.1.1 CCA |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  | 40: NRB,c = 106 |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | SR.1.1 CCA |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 CCA |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR.1.1 CCA |  |  |
| RMSI CORESET RMC configuration |  | Config 1 |  | CR.1.1 FDD |  | CR.1.1 CCA |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 CCA |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.1.1 CCA |  |  |
| Dedicated CORESET RMC configuration |  | Config 1 |  | CCR.1.1 FDD |  | CCR.1.1 CCA |  |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 CCA |  |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.1.1 CCA |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |
| SSB Configuration | Semi-static channel acces | Config 1,2 |  | SSB.1 FR1 |  | SSB.1 CCA |  |  |
|  | Dymamic channel acces | Config 3 |  | SSB.2 FR1 |  | SSB.2 CCA |  |  |
|  | Semi-static channel acces | Config 1,2 |  | SSB.1 FR1 |  | SSB.1 CCA |  |  |
|  | Dymamic channel acces | Config 3 |  | SSB.2 FR1 |  | SSB.2 CCA |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 FR1 |  | SMTC.2 FR1 |  |  |
|  |  | Config 3 |  | SMTC.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  | 30 kHz |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  | 30 kHz |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 under CCA in Table A.3.8A.2.1-1 |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |
| Note2 |  | Config 1,2 | dBm/SCS | -98 |  | -95 |  |  |
|  |  | Config 3 |  | -95 |  |  |  |  |
|  |  |  | dB | 4 | 4 | -infinity |  | 4 |
|  |  |  | dB | 4 | 4 | -infinity |  | 4 |
| IoNote3 |  | Config 1,2 | dBm/9.36MHz | -64.59 | -64.59 | N/A |  | N/A |
|  |  |  | dBm/38.16MHz | NA | NA | -63.94 |  | -58.49 |
|  |  | Config 3 | dBm/38.16MHz | -58.49 | -58.49 | -63.94 |  | -58.49 |
| Propagation condition |  |  | - | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. Note 5: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 6: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. Note 7: As defined in clause 6.2.3.2.3 for Trs ≤ 40 ms. |  |  |  |  |  |  |  |  |

A.11.2.2.3.2.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than Tconnection_release_redirect_NR_CCA ms from the beginning of time period T2, where Tconnection_release_redirect_NR_CCA is defined in clause 6.2.3.2.3.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR_CCA = TRRC_procedure_delay + Tidentify-NR_CCA + TSI-NR_CCA + TRACH_CCA,

where:

TRRC_procedure_delay = 110 ms in the test.

Tidentify-NR_CCA = MAX (680 ms, (L1+11)  20 ms) in the test.

TSI-NR_CCA = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target NR cell.

TRACH_CCA is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TRACH_CCA = (1+L2)´TSSB,RO + 10 ms; where TSSB,RO = 10 ms for FR1 PRACH configuration 1.

L1 is the number of SMTC occasions not available at the UE due to DL CCA failures. The test equipment shall ensure that L1 does not exceed L1,max. In the test L1,max= LCCA_DL which is defined in clause A.3.26.2.1.

L2 is the consecutive number of SSB to PRACH occasion association periods during which no PRACH occasion is available for PRACH transmission due to UL CCA failures. L2 = 0 in the test.

The total delay, Tconnection_release_redirect_NR_CCA, shall be less than 1410 + MAX (680, (L1+11)´20) ms.

----------------------END OF CHANGES 1----------------------------