3GPP TSG-RAN WG4 Meeting # 111	R4-2408753

# Fukuoka City, Fukuoka, Japan, 20th – 24th, May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0536 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BigCR to 38.101-4 on Introducing PDCCH demodulation requirements for DSS enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DSS_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-05-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 agreed to introduce PDCCH requirement for the WI: NR_DSS_enh |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Combine all endoresed draft CRs in RAN4 #111: R4-2409990: [NR_DSS_enh-Perf] Draft CR to 38.101-4 Applicability rule for NR PDCCH demodulation requirements under DSS scenario -- Qualcomm R4-2409991: Draft CR to 38.101-4: PDCCH requirement for DSS enhancement (FDD 2Rx) -- MediaTek R4-2409873: DraftCR for PDCCH requirements for eDSS - TDD with 2RX -- Apple R4-2409992: Draft CR on the Introduction of PDCCH requirement for DSS enhancement -FDD 4Rx -- Nokia R4-2409849: Draft CR to TS38.101-4: Introduction of PDCCH requirement for DSS enhancement -TDD 4Rx -- ZTE - Combine the endorsed draft CRs that are not resubmitted (no further updates) in RAN4 #111 R4-2406016: Draft CR: Introduction of Reference measurement channel for DSS enhancement demodulation requirement – Huawei - Combine some contents in one of endorsed draft CR in the last RAN4 110bis meeting but are missed in this RAN4 #111 resubmission draft CR. R4-2406014: Draft CR to 38.101-4 PDCCH requirement for DSS enhancement (FDD 2Rx) -- MediaTek |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demodulation requirement for the WI: NR_DSS_enh is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.3, 5.3.2.1, 5.3.2.1.6 (NEW), 5.3.2.2.6 (New), 5.3.3.1.5 (New), 5.3.3.2.5 (New), A.3.3.1, A.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------------- Beginning of R4-2409990 -------------------------------------------------------

#### 5.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |  |
| --- | --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1) |  |  |
| Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 (Test 1-2) Clause 5.2.3.2.4 (Test 1-2) |  |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 5.5A.1 | 1)Up to 16 DL carriers 2)Same numerology across carrier for data/control channel at a given time |  |
| Enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h | FR1 FDD | PDSCH | Clause 5.2.2.1.9 (Test 1-1) Clause 5.2.3.1.9 (Test 1-1) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.9 (Test 1-1) Clause 5.2.3.2.9 (Test 1-1) |  |  |
| Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |  |
| CQI table with target BLER of 10^-5New CQI table (cqi-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 |  |  |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR1 FDD | PDSCH | Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |  |
| UE PDSCH processing capability #2 (pdsch-ProcessingType2) | FR1 FDD | PDSCH | Clause 5.2.2.1.7 Clause 5.2.3.1.7 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.7 Clause 5.2.3.2.7 |  |  |
| Pre-emption indication for DL (pre-EmptIndication-DL) | FR1 FDD | PDSCH | Clause 5.2.2.1.8 Clause 5.2.3.1.8 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.8 Clause 5.2.3.2.8 |  |  |
| Single DCI based SDM transmission for multi-TRxP (singleDCI-SDM-scheme-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.11 Clause 5.2.3.1.11 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.11 Clause 5.2.3.2.11 |  |  |
| Multi DCI based multi-TRxP support (multiDCI-MultiTRP-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.12 Clause 5.2.3.1.12 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.12 Clause 5.2.3.2.12 |  |  |
| Single DCI based FDM Scheme-A for multi-TRxP(supportFDM-SchemeA-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.13 Clause 5.2.3.1.13 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.13 Clause 5.2.3.2.13 |  |  |
| Single DCI based inter-slot TDM for multi-TRxP (supportInter-slotTDM-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |  |
| Maximum number of TCI states in Single-DCI based inter-slot TDM (maxNumberTCI-states-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 | The requirements apply only when maxNumberTCI-states-r16 = 2. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |  |
| DRX Adaptation (drx-Adaptation-r16) | FR1 FDD | PDCCH | Clause 5.3.2.1.3 | If the Test 1 in Clause 5.3.2.1.3 is passed, the test coverage can be considered fulfilled without executing Test 3 in clause 5.3.2.1.1. |  |
|  | FR1 TDD | PDCCH | Clause 5.3.2.2.3 | If the Test 1 in Clause 5.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 2 in clause 5.3.2.2.1. |  |
|  | FR1 FDD | PDCCH | Clause 5.3.3.1.3 | If the Test 1 in Clause 5.3.3.1.3 is passed, the test coverage can be considered fulfilled without executing Test 3 in clause 5.3.3.1.1. |  |
|  | FR1 TDD | PDCCH | Clause 5.3.3.2.3 | If the Test 1 in Clause 5.3.3.2.3 is passed, the test coverage can be considered fulfilled without executing Test 2 in clause 5.3.3.2.1. |  |
| Validating P/SP-CSI-RS reception (periodicAndSemi-PersistentCSI-RS-r16) | FR1 TDD | PDSCH | Clause 5.2.2.2.15 Clause 5.2.3.2.15 Clause 5.2A.2.3 Clause 5.2A.3.3 | The requirements apply only in case tested UE supporting operations in shared spectrum access and validation of P/SP-CSI-RS reception based on DCI |  |
| Supported UL channels for dynamic channel access mode (ul-DynamicChAccess-r16) or UL channel access for semi-static channel access mode (ul-Semi-StaticChAccess-r16) or both | FR1 TDD | PDSCH | Clause 5.2.2.2.15 Clause 5.2.3.2.15 | The requirements apply only in case tested UE supports one of UL channels for dynamic channel access mode and UL channel access for semi-static channel access mode |  |
| 1024QAM modulation for PDSCH for FR1 (pdsch-1024QAM-FR1-r17 or pdsch-1024QAM-2MIMO-FR1-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-8) Clause 5.2.3.1.1 (Test 1-8) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 1-12) Clause 5.2.3.2.1 (Test 1-12) |  |  |
|  |  | SDR | Clause 5.5.1 Clause 5.5A.1 | 1024QAM MCS indexes are used only if UE supports 1024QAM for FR1 DL. |  |
| Support of neighboring LTE cell CRS-IM in DSS scenario with NR 15 kHz SCS ( CRS-IM-DSS-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.18 Clause 5.2.3.1.17 | UE can support the feature on the CC(s) in a band only if the UE indicates support of rateMatchingLTE-CRS on that band. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.19 Clause 5.2.3.2.18 |  |  |
| Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth (CRS-IM-nonDSS-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 (Test 2-1) Clause 5.2.3.1.18 (Test 2-1) | The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 2-1) Clause 5.2.3.2.19 (Test 2-1) |  |  |
| Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth (CRS-IM-nonDSS-NWA-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 (Test 1-1) Clause 5.2.3.1.18 (Test 1-1) | If the Test 2-1 in Clause 5.2.2.1.19 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.2.1.19. If the Test 2-1 in Clause 5.2.3.1.18 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.3.1.18. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-1) Clause 5.2.3.2.19 (Test 1-1) | If the Test 2-1 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.2.2.20. If the Test 2-1 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.3.2.19. |  |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth (crs-IM-nonDSS-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 2-2) Clause 5.2.3.2.19 (Test 2-2) | The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position. |  |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth (crs-IM-nonDSS-NWA-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-2) Clause 5.2.3.2.19 (Test 1-2) | If the Test 2-2 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.2.2.20. If the Test 2-2 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.3.2.19. |  |
| Support for SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH (sfn-SchemeA-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.20 Clause 5.2.3.1.19 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.21 Clause 5.2.3.2.20 |  |  |
| Support for SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH (sfn-SchemeB-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.21 Clause 5.2.3.1.20 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.22 Clause 5.2.3.2.21 |  |  |
| Support for PDCCH with intra-slot repetition (mTRP-PDCCH-Repetition-r17) | FR1 FDD | PDCCH | Clause 5.3.2.1.5 Clause 5.3.3.1.4 |  |  |
|  | FR1 TDD | PDCCH | Clause 5.3.2.2.5 Clause 5.3.3.2.4 |  |  |
| Table 5.1.1.3-1: Requirements applicability for optional UE features | Table 5.1.1.3-1: Requirements applicability for optional UE features | Table 5.1.1.3-1: Requirements applicability for optional UE features | Table 5.1.1.3-1: Requirements applicability for optional UE features | Table 5.1.1.3-1: Requirements applicability for optional UE features |  |
| Reception of NR PDCCH candidates overlapping with LTE CRS REs (nr-PDCCH-OverlapLTE-CRS-RE-r18) | FR1 FDD | PDCCH | Clause 5.3.2.1.6 Clause 5.3.2.2.6 |  |
|  | FR1 TDD | PDCCH | Clause 5.3.3.1.5 Clause 5.3.3.2.5 |  |

------------------------------------------------------------ End of R4-2409990 -----------------------------------------------------------

-------------------------------------------------------- Beginning of R4-2406014 -------------------------------------------------------

#### 5.3.2.1	FDD

The parameters specified in Table 5.3.2.1-1 are valid for all FDD tests unless otherwise stated.

Table 5.3.2.1-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna | 4 Tx Antenna |
| --- | --- | --- | --- | --- |
| CCE to REG mapping type |  | nonInterleaved |  |  |
| REG bundle size |  | 6 |  |  |
| Shift index |  | 0 |  |  |

------------------------------------------------------------ End of R4-2406014 -----------------------------------------------------------

-------------------------------------------------------- Beginning of R4-2409991 -------------------------------------------------------

##### 5.3.2.1.6	Minimum requirements for PDCCH overlapping with LTE CRS

The parameters specified in Table 5.3.2.1.6-1 are additional parameters for requirements with PDCCH overlapping with LTE CRS.

Table 5.3.2.1.6-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| PDCCH configuration | Symbols with PDCCH |  | Symbol#1 and #2 |  |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: NR PDCCH data REs and DMRS REs overlapped with LTE CRS are punctured at the transmitter side. |  |  |  |

For the parameters specified in Table 5.3.2.1.6-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.6-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.2.1.6-2: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 8 | R.PDCCH.1-2.9 FDD | TDLC300-100 | 4x2 Low | 1 | [-0.2] |

------------------------------------------------------------ End of R4-2409991 -----------------------------------------------------------

-------------------------------------------------------- Beginning of R4-2409873 -------------------------------------------------------

##### 5.3.2.2.6	Minimum requirements with PDCCH overlapping with LTE CRS

The parameters specified in Table 5.3.2.2.6-1 are additional prarameters for requirements with PDCCH overlapping with LTE CRS.

Table 5.3.2.2.6-1: Test Parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.15-1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| CCE to REG mapping type |  |  | Non-interleaved |
| REG bundle size |  |  | 6 |
| Shift Index |  |  | 0 |
| PDCCH Configuration | Symbols with PDCCH |  | Symbol#1 and #2 |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: LTE carrier is configured with Uplink-downlink configuration 2 [Table 4.2-2, TS 36.211] and Special subframe configuration 7 [Table 4.2-1, TS 36.211]. The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame. Note 3: NR PDCCH data REs and DMRS REs overlapped with LTE CRS are punctured at the transmitter side |  |  |  |

For the parameters specified in Table 5.3.2.2.6-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.6-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.2.2.6-2: Minimum performance for PDCCH with 15KHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 8 | R.PDCCH.1-2.8 TDD | TDLC300-100 | 2x2 Low | 1 | [0.5] |

------------------------------------------------------------ End of R4-2409873 -----------------------------------------------------------

---------------------------------------------------- Beginning of R4-2409992 -----------------------------------------------------------

##### 5.3.3.1.5	Minimum requirements for PDCCH overlapping with LTE CRS

The parameters specified in Table 5.3.3.1.5-1 are additional parameters for requirements with PDCCH overlapping with LTE CRS.

Table 5.3.3.1.5-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| PDCCH configuration | Symbols with PDCCH |  | Symbol#1 and #2 |  |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: NR PDCCH data REs and DMRS REs overlapped with LTE CRS are punctured at the transmitter side. |  |  |  |

For the parameters specified in Table 5.3.3.1.5-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.5-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.3.1.5-2: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | R.PDCCH.1-2.8 FDD | TDLA30-10 | 2x4 Low | 1 | [0.6] |

------------------------------------------------------------ End of R4-2409992 -----------------------------------------------------------

---------------------------------------------------- Beginning of R4-2409849 -----------------------------------------------------------

##### 5.3.3.2.5	Minimum requirement for PDCCH overlapping with LTE CRS

The parameters specified in Table 5.3.3.2.5-1 are additional parameters for requirements with PDCCH overlapping with LTE CRS.

Table 5.3.3.2.5-1: Test Parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.15-1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| CCE to REG mapping type |  |  | Non-interleaved |
| REG bundle size |  |  | 6 |
| Shift Index |  |  | 0 |
| PDCCH Configuration | Symbols with PDCCH |  | Symbol#1 and #2 |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: LTE carrier is configured with Uplink-downlink configuration 2 [Table 4.2-2, TS 36.211] and Special subframe configuration 7 [Table 4.2-1, TS 36.211]. The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame. Note 3: NR PDCCH data REs and DMRS REs overlapped with LTE CRS are punctured at the transmitter side |  |  |  |

For the parameters specified in Table 5.3.3.2.5-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.5-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.3.2.5-2: Minimum performance for PDCCH with 15kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | R.PDCCH.1-2.7 TDD | TDLA30-10 | 4x4 Low | 1 | [-0.1] |

------------------------------------------------------------ End of R4-2409849 -----------------------------------------------------------

-------------------------------------------------------- Beginning of R4-2406016 -------------------------------------------------------

## A.3.3	Reference measurement channels for PDCCH performance requirements

### A.3.3.1	FDD

#### A.3.3.1.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.3.1.1-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-1.1 FDD | R.PDCCH.1-1.2 FDD | R.PDCCH.1-1.3 FDD |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |  |
| CORESET frequency domain allocation |  | 48 | 48 | 48 |  |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 |  |  |  |
| Aggregation level |  | 4 | 4 | 8 |  |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 |  |  |  |
| Payload (without CRC) | Bits | 39 | 52 | 52 |  |  |  |

Table A.3.3.1.1-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-2.1 FDD | R.PDCCH.1-2.2 FDD | R.PDCCH.1-2.3 FDD | R.PDCCH.1-2.4 FDD | R.PDCCH.1-2.5 FDD | R.PDCCH.1-2.6 FDD | R.PDCCH.1-2.7 FDD | R.PDCCH.1-2.8 FDD | R.PDCCH.1-2.9 FDD |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 24 | 24 | 48 | 48 | 48 | 48 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Aggregation level |  | 2 | 4 | 2 | 4 | 8 | 16 | 8 | 4 | 8 |
| DCI Format |  | 1_0 | 1_0 | 1_1 | 1_1 | 1_1 | 1_0 | 2_6 | 1_0 | 1_0 |
| Payload (without CRC) | Bits | 39 | 39 | 52 | 52 | 52 | 39 | 12 | 39 | 39 |

### A.3.3.2	TDD

#### A.3.3.2.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.3.2.1-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-1.1 TDD | R.PDCCH.1-1.2 TDD | R.PDCCH.1-1.3 TDD |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |  |
| CORESET frequency domain allocation |  | 48 | 48 | 48 |  |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 |  |  |  |
| Aggregation level |  | 4 | 4 | 8 |  |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 |  |  |  |
| Payload (without CRC) | Bits | 39 | 52 | 52 |  |  |  |

Table A.3.3.2.1-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-2.1 TDD | R.PDCCH.1-2.2 TDD | R.PDCCH.1-2.3 TDD | R.PDCCH.1-2.4 TDD | R.PDCCH.1-2.5 TDD | R.PDCCH.1-2.6 TDD | R.PDCCH.1-2.7 TDD | R.PDCCH.1-2.8 TDD |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 24 | 24 | 48 | 48 | 48 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Aggregation level |  | 2 | 4 | 2 | 4 | 8 | 16 | 4 | 8 |
| DCI Format |  | 1_0 | 1_0 | 1_1 | 1_1 | 1_1 | 1_0 | 1_0 | 1_0 |
| Payload (without CRC) | Bits | 39 | 39 | 52 | 52 | 52 | 39 | 39 | 39 |

------------------------------------------------------------ End of R4-2406016 -----------------------------------------------------------