3GPP TSG-WG4 Meeting #111	R4-2407676

# Fukuoka City, Fukuoka, Japan,  20th - 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4393 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR on performance part of NR_MIMO_evo_DL_UL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Perf |  |  |  |  |  | Date: |  |  | 2024-05-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce RRM performance requiements for NR_MIMO_evo_DL_UL, including TDCP mapping tables and Test cases for NR_MIMO_evo_DL_UL-Perf |  |  |  |  |  |  |  |  |
| Summary of change: |  | This Big CR includes all endorsed draft CRs for TS 38.133 for performance part of NR_MIMO_evo_DL_UL in RAN4#111 meeting: - R4-2410318, Draft CR for TDCP mapping, Apple - R4-2410319, Draft CR to 38.133 on TDCP tests, Ericsson - R4-2410320, Draft CR on test cases of UE transmit timing from two TRPs in FR1, Samsung - R4-2410321, draft CR on test cases for FR2 UE transmit timing from two TRPs, vivo - R4-2410322, DraftCR on TC for MAC CE based DL separate dual TCI state activation in sDCI, Xiaomi - R4-2410323, Draft CR on TC for FR1 TCI state switching for mDCI with two TA, Huawei, HiSilicon - R4-2410324, TC: sDCI mTRP, FR2 separate UL TCI state switching, Nokia - R4-2410416, TC for sDCI MAC-CE based joint dual TCI state switching, MediaTek Inc. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM performance requiements of Rel-18 MIMO evolution are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3 10.1.41 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== Start of change #1 including: R4-2410318 =====

===== change #1 in R4-2410318 =====

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

AoA	Angle of Arrival

AoD	Angle of Departure

ATG	Air to Ground

BFD	Beam Failure Detection

BFD-RS	BFD Reference Signal

BLER	Block Error Rate

BM-RS	Beam Management Reference Signal

BWP	Bandwidth Part

CA	Carrier Aggregation

CBD	Candidate Beam Detection

CBW	Channel Bandwidth

CC	Component Carrier

CCA	Clear Channel Assessment

CG-SDT	Configured Grant Small Data Transmisison

CLI	Cross Link Interference

CMR	Channel Measurement Resource

CORESET	Control Resource Set

CP	Cyclic Prefix

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

CSI-RSRP	CSI Reference Signal based Reference Signal Received Power

CSI-RSRQ	CSI Reference Signal based Reference Signal Received Quality

CSI-SINR	CSI Reference Signal based Signal to Noise and Interference Ratio

CSI_RP	Received (linear) average power of the resource elements that carry NR CSI-RS signals and channels, measured at the UE antenna connector

DBT	Discovery Burst Transmission

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DL-AoD	Downlink Angle-of-Departure

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell ID

eDRX	Extended DRX

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EMW	Effective measurement window

EMWRP	Effective measurement window repetition period

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FH	Frequency Hopping

FR	Frequency Range

GEO	Geostationary Earth Orbit

HARQ	Hybrid Automatic Repeat Request

HO	Handover

GAP	Refers to any of Measurement Gap, activated Pre-MG and NCSG

IMR	Interference Measurement Resource

L1-RSRP	Layer 1 RSRP

L1 SL-RSRP	Layer 1 Sidelink RSRP which corresponds to PSCCH-RSRP and/or PSSCH-RSRP

LEO	Low Earth Orbit

LMF	Location Management Function

LPP	LTE Positioning Protocol

LTM	L1/L2 triggered mobility

MAC	Medium Access Control

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MG	Measurement Gap

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

ML	Measurement Length

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

MUSIM	Multi-Universal Subscriber Identity Module

NCSG	Network Controlled Small Gap

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NTN	Non-Terrestrial Network

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OTDOA	Observed Time Difference Of Arrival

PBCH	Physical Broadcast Channel

PCC	Primary Component Carrier

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PRACH	Physical RACH

Pre-MG	Pre-configured Measurement Gap

ProSe	Proximity-based Service

PRP	PRS Received Power

PRS	Positioning Reference Signal

PRS-RSRP	Positioning Reference Signal based Reference Signal Received Power

PPW	PRS Processing Window

PSBCH	Physical Sidelink Broadcast Channel

PSBCH-RSRP	Physical Sidelink Broadcast Channel DMRS based Reference Signal Received Power

PSCCH	Physical Sidelink Control Channel

PSCCH-RSRP	Physical Sidelink Control Channel DMRS based Reference Signal Received Power

PSCell	Primary SCell

PSS	Primary Synchronization Signal

PSSCH	Physical Sidelink Shared Channel

PSSCH-RSRP	Physical Sidelink Shared Channel DMRS based Reference Signal Received Power

pTAG	Primary Timing Advance Group

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-Location

RACH	Random Access Channel

RAT	Radio Access Technology

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMSI	Remaining Minimum System Information

RRC	Radio Resource Control

RRH	Remote Radio Head

RRM	Radio Resource Management

RSCP	Reference Signal Carrier Phase

RSCPD	Reference Signal Carrier Phase Difference

RSSI	Received Signal Strength Indicator

RSRP	Reference Signal Received Power

RSRPP	Reference Signal Received Path Power

RSRQ	Reference Signal Received Quality

RSTD	Reference Signal Time Difference

RTOA	Relative Time Of Arrival

RTT	Round Trip Time

S-SSB	Sidelink Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector or radiated interface boundary.

SA	Standalone operation mode

SAB	Satellite access band

SAN	Satellite Access Node

SCC	Secondary Component Carrier

SCCH	Sidelink Control Channel

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SDT	Small Data Transmission

SFN	System Frame Number

SFTD	SFN and Frame Timing DifferenceSI	System Information

SIB	System Information Block

SL	Sidelink

SL AoA	Sidelink AoA

SL PRS-RSRP	Sidelink PRS-based RSRP

SL PRS-RSRPP	Sidelink PRS-based RSRPP

SL RSTD	Sidelink RSTD

SL RTOA	Sidelink RTOA

SL Rx-Tx	Sidelink Receive-Transmit time difference

SL-PRS	Sidelink PRS

SL-RSSI	Sidelink Received Signal Strength Indicator

SLPP	Sidelink Positioning Protocol

SLSS	Sidelink Synchronization Signal

SMTC	SSB-based Measurement Timing configuration

SpCell	Special Cell

SRS	Sounding Reference Signal

SRS-RSRP	Sounding Reference Signal based Reference Signal Received Power

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

sTAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TDCP	Time Domain Channel Properties

TDD	Time Division Duplex

TDOA	Time Difference Of Arrival

TN	Terrestrial Network

TRP	Transmission-Reception Point

TTI	Transmission Time Interval

U2N	UE-to-Network

U2U	UE-to-UE

UE	User Equipment

UL	Uplink

V2X	Vehicle-to-Everything service

VIL	Visible Interruption Length

VIRP	Visible Interruption Repetition Period

VSAT	Very Small Aperture Terminal

===== change #2 in R4-2410318 =====

### 10.1.41 TDCP Measurement Report Mapping

The reporting range of TDCP amplitude is defined from 0 to 1. The reporting range of TDCP phase is 0 to 2p. The mapping of measured quantity is defined in Tables 10.1.41-1, 10.1.41-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.41-1: TDCP amplitude measurement report mapping

| Reported value | Measured quantity value |
| --- | --- |
| TDCP_0 | 1- 11282 < TDCP ≤1 |
| TDCP_1 | 1-1128 < TDCP ≤ 1- 11282 |
| TDCP_2 | 1-1642 < TDCP ≤ 1- 1128 |
| TDCP_3 | 1-164 < TDCP ≤ 1- 1642 |
| TDCP_4 | 1-1322 < TDCP ≤ 1- 164 |
| TDCP_5 | 1-132 < TDCP ≤ 1- 1322 |
| TDCP_6 | 1-1162 < TDCP ≤ 1- 132 |
| TDCP_7 | 1-116 < TDCP ≤ 1- 1162 |
| TDCP_8 | 1-182 < TDCP ≤ 1- 116 |
| TDCP_9 | 1-18 < TDCP ≤ 1- 182 |
| TDCP_10 | 1-142 < TDCP ≤ 1- 18 |
| TDCP_11 | 1-14 < TDCP ≤ 1- 142 |
| TDCP_12 | 1-122 < TDCP ≤ 1- 14 |
| TDCP_13 | 1-12 < TDCP ≤ 1- 122 |
| TDCP_14 | 1-12 < TDCP ≤ 1- 12 |
| TDCP_15 | 0 ≤ TDCP ≤ 1- 12 |

Table 10.1.41-2: TDCP phase measurement report mapping

| Reported value | Measured quantity value |
| --- | --- |
| TDCP_PH_0 | 0 ≤ TDCP_PH < 2π16 |
| TDCP_PH_1 | 2π16≤ TDCP_PH < 2π*216 |
| TDCP_PH_2 | 2π*216 ≤ TDCP_PH < 2π*316 |
| TDCP_PH_3 | 2π*316 ≤ TDCP_PH < 2π*416 |
| TDCP_PH_4 | 2π*416 ≤ TDCP_PH < 2π*516 |
| TDCP_PH_5 | 2π*516≤ TDCP_PH < 2π*616 |
| TDCP_PH_6 | 2π*616 ≤ TDCP_PH < 2π*716 |
| TDCP_PH_7 | 2π*716 ≤ TDCP_PH < 2π*816 |
| TDCP_PH_8 | 2π*816 ≤ TDCP_PH < 2π*916 |
| TDCP_PH_9 | 2π*916 ≤ TDCP_PH < 2π*1016 |
| TDCP_PH_10 | 2π*1016 ≤ TDCP_PH < 2π*1116 |
| TDCP_PH_11 | 2π*1116≤ TDCP_PH < 2π*1216 |
| TDCP_PH_12 | 2π*1216 ≤ TDCP_PH < 2π*1316 |
| TDCP_PH_13 | 2π*1316 ≤ TDCP_PH < 2π*1416 |
| TDCP_PH_14 | 2π*1416 ≤ TDCP_PH < 2π*1516 |
| TDCP_PH_15 | 2π*1516 ≤ TDCP_PH ≤ 2π |

=====End of change #1 =====

===== Start of change #2 including change 1 in R4-2410416 =====

A.3.10.2.19	SSB pattern 17 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz

Table A.3.10.2.1-1: SSB.17 FR2: SSB Pattern 17 for SSB SCS = 120 kHz in 100 MHz channel with 3 SSBs per SS-burst

| SSB Parameters | Values |  |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | 100 MHz |  |  |  |
| SSB SCS | 120 kHz |  |  |  |
| SSB periodicity (TSSB) | 20 ms |  |  |  |
| Number of SSBs per SS-burst | 3 |  |  |  |
| SS/PBCH block index | 0 | 1 | 2 | 3 |
| Symbol numbers containing SSBs Note 2 | 4-7 | 8-11 | 2-5 | 6-9 |
| Slot numbers containing SSB Note 2 | 0 | 0 | 1 | 1 |
| SFN containing SSB | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |  |  |
| RB numbers containing SSBs within channel BW | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |  |  |

=====End of change #2 =====

===== Start of change #3 including change 1 in R4-2410324 =====

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases

>>> Unchanged sections omitted <<<<<

A.3.15.x	Setup X: 3 AoAs

There are 3 active probes in the test. The DL signals, and noise if applicable, transmitted from the three active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. The relative angular offset between the directions (AoAs) of the 3 active probes, shall be changed for each test iteration. Any combinations of two relative angular offsets between 2 active probes specified in Table A.3.15.3-1, is considered as a valid applicable relative angular offsets between 3 active probes for the respective power class.

=====End of change #3 =====

===== Start of change #4 including change 2 in R4-2410416 and change 1 in R4-2410323 =====

A.3.16A.2	DLorJoint TCI states

Table A.3.16A.2-1: DLorJoint TCI States

| Parameter | DLorJoint TCI.State.0 | DLorJoint TCI.State.1 | DLorJoint TCI.State.2 | DLorJoint TCI.State.3 | DLorJoint TCI.State.4 | DLorJoint TCI.State.5 | DLorJoint TCI.State.6 | DLorJoint TCI.State.7 | DLorJoint TCI.State.8 | DLorJoint TCI.State.9 | DLorJoint TCI.State.10 | DLorJoint TCI.State.11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tci-StateUnifiedId | Id0 | Id1 | Id2 | Id3 | Id4 | Id5 | Id6 | Id7 | Id8 | Id9 | Id10 | Id11 |
| qcl-Type1 | typeA | typeA | typeA | typeA | typeC | typeC | typeA | typeA | typeA | typeC | typeC | typeC |
| qcl-Type2Note1 | typeD | typeD | typeD | typeD | typeD | typeD | typeD | typeD | typeD | typeD | typeD | typeD |
| referenceSignal Note2 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note 5 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note3 | SSB0 | SSB1 from the cell with different PCI | Resource #4 in TRS resource set 3 Note 6 | Resource #4 in TRS resource set 4 Note 7 | Resource #4 in TRS resource set 5 Note 8 | SSB1 | SSB2 | SSB3 |
| pathlossReferenceRS | N/A | N/A | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 1 Note3 | N/A | N/A | Resource #4 in TRS resource set 3 Note 6 | Resource #4 in TRS resource set 4 Note 7 | Resource #4 in TRS resource set 5 Note 8 | N/A | N/A | N/A |
| additionalPCI | N/A | configured Note4 | N/A | N/A | N/A | configured Note4 | N/A | N/A | N/A | N/A | N/A | N/A |
| Note 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 Note 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 3: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.4. Note 4: Only one PCI than serving cell PCI is included in the additionalPCIList, and the additionalPCIIndex is configured as 0. Note 5: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.5. Note 6: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DlorJoint TCI.State.9. Note 7: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DlorJoint TCI.State.10. Note 7: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.11. |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.16A.2-2: DLorJoint TCI States for mTRP FR1

| Parameter | DLorJoint TCI.State.1.1 | DLorJoint TCI.State.1.2 | DLorJoint TCI.State.1.3 | DLorJoint TCI.State.1.4 |
| --- | --- | --- | --- | --- |
| tci-StateUnifiedId | Id1 | Id2 | Id3 | Id4 |
| qcl-Type1 | typeA | typeA | typeC | typeC |
| qcl-Type2 | N/A | N/A | N/A | N/A |
| referenceSignal Note2 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 2 Note3 | SSB1 | SSB2 |
| pathlossReferenceRS | N/A | N/A | N/A | N/A |
| additionalPCI | N/A | N/A | N/A | N/A |
| Note 1: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 2: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.1.1. Note 3: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. The TCI state of the TRS is the DLorJoint TCI.State.1.2. |  |  |  |  |

=====End of change #4 =====

===== Start of change #5 including change 2 in R4-2410324 =====

A.3.16A.3	UL TCI states

Table A.3.16A.3-1: UL TCI States

| Parameter | UL TCI.State.0 | UL TCI.State.1 | UL TCI.State.2 | UL TCI.State.3 | UL TCI.State.4 |
| --- | --- | --- | --- | --- | --- |
| ul-TCIState-Id | Id0 | Id1 | Id2 | Id3 | Id4 |
| referenceSignal Note1 | SSB0 | SSB1 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 2 Note2 | Resource #4 in TRS resource set 3 Note2 |
| pathlossReferenceRS | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 1 Note2 | Resource #4 in TRS resource set 2 Note2 | Resource #4 in TRS resource set 3 Note2 |
| additionalPCI | N/A | configured Note3 | N/A | N/A | N/A |
| Note 1: referenceSignal configurations towards which the UL TCI states are configured are defined in a test-specific manner. Note 2: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. Note 3: Only one PCI than serving cell PCI is included in the additionalPCIList, and the additionalPCIIndex is configured as 0. |  |  |  |  |  |

=====End of change #5 =====

===== Start of change #6 including change 1 in R4-2410319 & change 2 in R4-2410323 =====

A.3.17	Configurations of CSI-RS for tracking

A.3.17.1	Configuration of CSI-RS for tracking for FR1

A.3.17.1.1	FDD

Table A.3.17.1.1-1: CSI-RS for tracking for SCS=15kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.1 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.1-2: CSI-RS for tracking for SCS=30kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.2 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.1-3: Aperiodic CSI-RS for tracking for SCS=15kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.3 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.1-4: Aperiodic CSI-RS for tracking for SCS=30kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.4 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.1-5: CSI-RS for tracking for SCS=15kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.5 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | DLorJoint TCI.State.1.3 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.1-6: CSI-RS for tracking for SCS=15kHz Set 2

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.6 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | DLorJoint TCI.State.1.4 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.X1-1: CSI-RS for tracking for SCS=15kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.7 FDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

A.3.17.1.2	TDD

Table A.3.17.1.2-1: CSI-RS for tracking for SCS=15kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.1 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |

Table A.3.17.1.2-2: CSI-RS for tracking for SCS=30kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.2 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.2-3: Aperiodic CSI-RS for tracking for SCS=15kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.3 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |

Table A.3.17.1.2-4: Aperiodic CSI-RS for tracking for SCS=30kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.4 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.2-5: CSI-RS for tracking for SCS=15kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.5 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | DLorJoint TCI.State.1.3 |
| Note: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |

Table A.3.17.1.2-6: CSI-RS for tracking for SCS=15kHz Set 2

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.6 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 15 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | DLorJoint TCI.State.1.4 |
| Note: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |

Table A.3.17.1.2-7: CSI-RS for tracking for SCS=30kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.7 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | DLorJoint TCI.State.1.3 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.2-8: CSI-RS for tracking for SCS=30kHz Set 2

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.8 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | DLorJoint TCI.State.1.3 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

Table A.3.17.1.X2-2: CSI-RS for tracking for SCS=30kHz

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.1.9 TDD |
| Bandwidth |  | BW of Active BWPNote 1 |
| SCS | kHz | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case |  |  |

=====End of change #6 =====

===== Start of change #7 including change 3 in R4-2410324 =====

A.3.17.2	Configuration of CSI-RS for tracking for FR2

A.3.17.2.1	TDD

Table A.3.17.2.1-1: CSI-RS for tracking for SCS=120kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.1 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 l0 = 5 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

Table A.3.17.2.1-2: CSI-RS for tracking for SCS=120kHz Set 2

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.2 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 2 for CSI-RS resource 1 and 3 l0 = 6 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.1 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

Table A.3.17.2.1-3: Aperiodic CSI-RS for tracking for SCS=120kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.3 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 l0 = 5 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| aperiodicTriggeringOffsetL2 | slots | 2 |
| Aperiodic CSI-RS offset | slots | 2 for CSI-RS resource 1 and 2 3 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

Table A.3.17.2.1-z: CSI-RS for tracking for SCS=120kHz Set 3

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.4 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 3 for CSI-RS resource 1 and 3 l0 = 7 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.2 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

Table A.3.17.2.1-w: CSI-RS for tracking for SCS=120kHz Set 4

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.5 TDD |
| Bandwidth |  | BW of Active BWPNote 1,3 |
| SCS | kHz | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 4 for CSI-RS resource 1 and 3 l0 = 8 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0Note 2 |
| TCI state |  | TCI.State.3 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: Unless otherwise specified in the test case Note 3: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |

=====End of change #7 =====

===== Start of change #8 including change 1 in R4-2410320 =====

A.4.4.1	UE transmit timing

A.4.4.1.X	NR UE Transmit Timing Test for two TRPs in FR1

A.4.4.1.X.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timings change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits for both TRPs. The test is configured with two TRPs in NR PSCell. This test will verify the requirements in clause 7.1.2. Supported test configurations are shown in Table 4.4.1.X.1-1.

Table A.4.4.1.X.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | LTE FDD, NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | LTE FDD, NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 4 | LTE TDD, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 5 | LTE TDD, NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 6 | LTE TDD, NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

The test consists of E-UTRA PCell and NR PSCell. For NR PSCell, two TRPs are configured. The configuration for E-UTRA is given in A.3.7.2.1. Table A.4.4.1.X.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.4.4.1.X.1-3.

For UE not support the capability of “rxTimingDiff-r18”, the UE is only required to be tested in Test1 and Test3.

For UE supports the capability of “rxTimingDiff-r18”, the UE is only required to be tested in Test2 and Test4.

Table A.4.4.1.X.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  | Test2 |  | Test3 |  | Test4 |  | Band Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TRP#1 | TRP#2 | TRP#1 | TRP#2 | TRP#1 | TRP#2 | TRP#1 | TRP#2 |  |
| SSB ARFCN |  | 1,2,3,4,5,6 | Freq1 |  |  |  |  |  |  |  |  |
| Duplex Mode |  | 1,4 | FDD |  |  |  |  |  |  |  |  |
|  |  | 2,3,5,6 | TDD |  |  |  |  |  |  |  |  |
| TDD configuration |  | 1,4 | Not Applicable |  |  |  |  |  |  |  |  |
|  |  | 2,5 | TDDConf.1.1 |  |  |  |  |  |  |  |  |
|  |  | 3,6 | TDDConf.2.1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 1,4 | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |
|  |  | 2,5 | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |
|  |  | 3,6 | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3,4,5,6 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3,4,5,6 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |  |  |  |
| DRx Cycle | ms | 1,2,3,4,5,6 | N/A |  |  |  | DRX.8Note5 |  |  |  |  |
| PDSCH Reference |  | 1,4 | SR.1.1 FDD |  |  |  |  |  |  |  |  |
| measurement channel |  | 2,5 | SR.1.1 TDD |  |  |  |  |  |  |  |  |
|  |  | 3,6 | SR.2.1 TDD |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  | 1,4 | CR.1.1 FDD |  |  |  |  |  |  |  |  |
| Channel |  | 2,5 | CR.1.1 TDD |  |  |  |  |  |  |  |  |
|  |  | 3,6 | CR.2.1 TDD |  |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | 1,4 | CCR.1.1 FDD |  |  |  |  |  |  |  |  |
|  |  | 2,5 | CCR.1.1 TDD |  |  |  |  |  |  |  |  |
|  |  | 3,6 | CCR.2.1 TDD |  |  |  |  |  |  |  |  |
| coresetPoolIndex for dedicated CORESET Reference Channel |  | 1,2,3,4,5,6 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  |
| Timing difference compared to TRP#1 | us | 1,2,4,5 | 0 | 3 | 0 | 30 | 0 | 3 | 0 | 30 |  |
|  |  | 3,6 |  | 1.5 |  |  |  | 1.5 |  |  |  |
| OCNG Patterns |  | 1,2,3,4,5,6 | OP.1 |  |  |  |  |  |  |  |  |
| SSB configuration |  | 1,4 | SSB.3 FR1 |  |  |  |  |  |  |  |  |
|  |  | 2,5 | SSB.3 FR1 |  |  |  |  |  |  |  |  |
|  |  | 3,6 | SSB.4 FR1 |  |  |  |  |  |  |  |  |
| SSB index |  | 1,4 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  |
|  |  | 2,5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  |
|  |  | 3,6 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  |
| SMTC configuration |  | 1,2,3,4,5,6 | SMTC.2 |  |  |  |  |  |  |  |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD |  |  |  |  |  |  |  |  |
|  |  | 2,5 | TRS.1.1 TDD |  |  |  |  |  |  |  |  |
|  |  | 3,6 | TRS.1.2 TDD |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH | kHz | 1,2,4,5 | 15 |  |  |  |  |  |  |  |  |
| subcarrier spacing |  | 3,6 | 30 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 1,2,3,4,5,6 | 0 |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4,5,6 | -98 |  |  |  | -98 |  |  |  |  |
| Note2 | dBm/SCS | 1,2,4,5 | -98 |  |  |  | -98 |  |  |  |  |
|  |  | 3,6 | -95 |  |  |  | -95 |  |  |  |  |
|  |  | 1,2,3,4,5,6 | 3 |  |  |  | 3 |  |  |  |  |
|  |  | 1,2,3,4,5,6 | 3 |  |  |  | 3 |  |  |  |  |
| SS-RSRPNote3 | dBm/SCS | 1,2,4,5 | -95 |  |  |  | -95 |  |  |  |  |
|  |  | 3,6 | -92 |  |  |  | -92 |  |  |  |  |
| IoNote3 | dBm/9.36MHz | 1,2,4,5 | -65.2 |  |  |  | -65.2 |  |  |  |  |
|  | dBm/38.1MHz | 3,6 | -59.2 |  |  |  | -59.2 |  |  |  |  |
| Propagation condition |  | 1,2,3,4,5,6 | AWGN |  |  |  |  |  |  |  |  |
| SRS Config |  | 1,2,4,5 | SRSConf.1Note6 |  |  |  | SRSConf.3Note6 |  |  |  |  |
|  |  | 3, 6 | SRSConf.1Note6 |  |  |  | SRSConf.2Note6 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRx related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.4.4.1.X.1-3 |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.4.1.X.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | SRSConf.3 | Comments |
| --- | --- | --- | --- | --- | --- |
| SRS- | srs-ResourceSetId | 0 | 0 | 0 |  |
| ResourceSet | srs-ResourceIdList | 0 | 0 | 0 |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | 14 for test configuration 1,2,4,5 25 for test configuration 3,6 | 25 | 14 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl640, 5 | sl320, 3 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | 0 | Any 10 bit number |

A.4.4.1.X.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1)	Set up E-UTRA PCell according to parameters given in Table A.3.7.2.1-1 and setup NR PSCell according to parameters given in Table A.4.4.1.X.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset)×Tc ± Te of the first detected path of DL SSB of TRP#1 and TRP#2.

a.	The NTA offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in Table A.4.4.1.X.2-1 for only TRP#1. The timing of the DL path of TRP#2 is not changed.

Table A.4.4.1.X.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1&2 | Test3&Test4 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first detected path (in time) of DL SSB for TRP#1. For TRP#2, the test system shall verify there is no adjustment.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB of TRP#1.

=====End of change #8 =====

===== Start of change #9 including change 3 in R4-2410319 =====

A.4.7.X	TDCP amplitude measurement accuracy

A.4.7.X.1        TDCP amplitude measurement accuracy in EN-DC

A.4.7. X.1.1	Test Purpose and Environment

The purpose of this test is to verify that the TRS based TDCP amplitude measurement accuracy is within the specified limits in the test requirements section. The cofigurations for the test are specified in Table A.4.7.X.1.1-1.

The test consists of two tests, Test 1 and Test 2. Each test further consists of two subtests Test 1A, 1B and Test 2A, 2B.

Test 1A: 10 Hz doppler + 15kHz SCS FDD + 20dB SNR

Test 1B:  10 Hz doppler + 30kHz SCS TDD + 20 dB SNR

Test 2A: 300 Hz doppler + 15kHz SCS FDD + [10] dB SNR

Test 2B: 300 Hz doppler + 30kHz SCS TDD + [10] dB SNR

Relevant parmeters for each test are provided in the table A.4.7.X.1.2-1. UE needs to pass Test 1A, 1B, 2A, 2B.

Table A.4.7.X.1.1-1: Applicable NR configurations for FR1 TRS based TDCP test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |

A.4.7. X.1.2	Test parameters

In this set of test cases there are two cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2). The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 2 are given in Table A.4.7.X.1.2-1. Ampliutude of TDCP is tested by using the parameters in Table A.4.7.X.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured with 1 TRS set with the TRS resources in the set are configured in adjacent slot. UE is configured to perform L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.4.7.X.1.2-1: FR1 TRS based TDCP test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 | freq1 |
| Duplex mode | 1 |  | FDD | FDD |
|  | 2 |  | TDD | TDD |
| TDD Configuration | 1 |  | N/A | N/A |
|  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  | 2 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  | 2 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  | 2 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  | 2 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  | 2 |  | SSB.4 FR1 | SSB.4 FR1 |
| OCNG Patterns | 1~2 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration | 1 |  | TRS.1.5 FDD | TRS.1.5 FDD |
|  | 2 |  | TRS.1.5 TDD | TRS.1.5 TDD |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~2 |  | SMTC.1 | SMTC.1 |
| reportConfigType | 1~2 |  | periodic | periodic |
| reportQuantity | 1~2 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS | 1~2 |  | 2 | 2 |
| L1-RSRP reporting period | 1~2 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Noc | dBm/15kHz | 1 | -98 | -98 |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 | -98 |
|  |  | 2 | -95 | -95 |
|  | 1~2 | dB | TBD | TBD |
|  | 1~2 | dB | 10 | 20 |
| SSB_RP | 1 |  | -88 | -78 |
| Propagation condition | 1~2 |  | TDL-A 30ns 10 Hz | TDL-A 30ns 300 Hz |
| Antenna configuration | 1~2 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

A.4.7. X.1.3	Test Requirements

For Test 1A, the reported TDCP index shall be smaller than or equal to 6 for the 80% of the times over repeated tests.

For Test 1B:  the reported TDCP index shall be smaller than or equal to 5 for the 80% of the times over repeated tests.

For Test 2A: the reported TDCP index shall be lrager than 8 for the 80% of the times over repeated tests.

For Test 2B: the reported TDCP index shall be larger than [or equal to] 6 for the 80% of the times over repeated tests.

=====End of change #9 =====

===== Start of change #10 including change 1 in R4-2410321 =====

A.5.4.1.x	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18

A.5.4.1.x.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits, for UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 and is configured with 2 TAGs for multi-DCI multi-TRP operation. UE is also configured with dl-OrJointTCI-StateList or ul-TCI-State-List. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in Table 5.4.1.x.1-1.

Table A.5.4.1.x.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |
| 2 | LTE TDD, NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |

The test consists of E-UTRA PCell and NR PSCell. The configuration for E-UTRA is given in A.3.7.2.1. Tables A.5.4.1.x.1-2 and A.5.4.1.x.1-2A define the parameters to be configured and strength of the transmitted signals. The NR PSCell is configured with two TRPs in the test. Each TRP is associated with a CORESET, with coresetPoolIndex-r16 is set to 0 for the first TRP and set to 1 for the second TRP. UE is also configured with tag2 in ServingCellConfig. Two SRS resource sets are configured and associated to different TAGs via TCI state configuration. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.5.4.1.x.1-3.

For UE not support the capability of “rxTimingDiff-r18”, the UE is only required to be tested in Test1 and Test3.

For UE supports the capability of “rxTimingDiff-r18”, the UE is only required to be tested in Test2 and Test4.

Table A.5.4.1.x.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  | Test2 |  | Test3 |  | Test4 |  | Band Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TRP #1 | TRP #2 | TRP #1 | TRP #2 | TRP #1 | TRP #2 | TRP #1 | TRP #2 |  |
| SSB ARFCN |  | 1,2 | Freq1 |  |  |  |  |  |  |  |  |
| Duplex Mode |  | 1,2 | TDD |  |  |  |  |  |  |  |  |
| TDD configuration |  | 1,2 | TDDConf.3.1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 1,2 | 100: NRB,c = 66 |  |  |  |  |  |  |  |  |
| Data RBs allocated |  | 1,2 | 66 |  |  |  |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |  |  |  |
| TRS Configuration |  | 1,2 | TRS.2.1 TDD TRS.2.2 TDD |  |  |  |  |  |  |  |  |
| DRx Cycle | ms | 1,2 | N/A |  |  |  | DRX.8Note5 |  |  |  |  |
| PDSCH Reference measurement channel |  | 1,2 | SR.3. 3 TDD |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | 1,2 | CR.3.1 TDD |  |  |  |  |  |  |  |  |
| coresetPoolIndex for dedicated CORESET Reference Channel |  | 1,2 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  |
| Dedicated CORESET Reference Channel |  | 1,2 | CCR.3.4 TDD | CCR.3.6 TDD | CCR.3.4 TDD | CCR.3.6 TDD | CCR.3.4 TDD | CCR.3.6 TDD | CCR.3.4 TDD | CCR.3.6 TDD |  |
| TCI configuration |  | 1,2 | DLorJoint TCI.State.0 with tag-Id-ptr-r18 = n0 | DLorJoint TCI.State.1 with tag-Id-ptr-r18 = n1 | DLorJoint TCI.State.0 with tag-Id-ptr-r18 = n0 | DLorJoint TCI.State.1 with tag-Id-ptr-r18 = n1 | DLorJoint TCI.State.0 with tag-Id-ptr-r18 = n0 | DLorJoint TCI.State.1 with tag-Id-ptr-r18 = n1 | DLorJoint TCI.State.0 with tag-Id-ptr-r18 = n0 | DLorJoint TCI.State.1 with tag-Id-ptr-r18 = n1 |  |
| Timing difference compared to TRP#1 | us | 1,2 | 0 | 0.1 | 0 | 8 | 0 | 0.1 | 0 | 8 |  |
| OCNG Patterns |  | 1,2 | O P. 1 |  |  |  |  |  |  |  |  |
| SSB Configuration |  | 1,2 | SSB.1 FR2 |  |  |  |  |  |  |  |  |
| SMTC Configuration |  | 1,2 | SMTC.1 |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2 | 120 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2 | 0 |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition |  | 1,2 | AWGN |  |  |  |  |  |  |  |  |
| SRS Config |  | 1,2 | SRSConf.1Note6 |  |  |  | SRSConf.2Note6 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: VoidNote 4: Void Note 5: DRx related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.5.4.1.x.1-3 |  |  |  |  |  |  |  |  |  |  |  |

Table A.5.4.1.x.1-2A: OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 3 as specified in clause A.3.15 Note 7 |  |
| Assumption for UE beamsNote 6 |  | Fine |  |
| Note1 | dBm/15kHzNote4 | -112 |  |
| Note1 | dBm/SCSNote3 | -100 |  |
|  | dB | 4 |  |
| SSB_RPNote2 | dBm/SCS Note4 | -99 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 7: AoA1 for TRP1 and AoA2 for TRP2 |  |  |  |

Table A.5.4.1.x.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 |  | SRSConf.2 |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 1 | 0 | 1 |  |
|  | srs-ResourceIdList | 0 | 1 | 0 | 1 |  |
|  | resourceType | Periodic | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 1 | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | 17 | 17 | 17 | 17 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl2, 0 | sl2, 1 | sl2560, 4 | sl2560, 9 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | 0 | 0 | Any 10 bit number |
|  | TCI state | DLorJoint TCI.State.0 | DLorJoint TCI.State.1 | DLorJoint TCI.State.0 | DLorJoint TCI.State.1 |  |

Table A.5.4.1.x.1-4: Void

A.5.4.1.x.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1)	Set up E-UTRA PCell according to parameters given in Table A.3.7.2.2-1 and setup NR PSCell according to parameters given in Table A.5.4.1.x.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB for each TAG.

a.	The NTA offset value (in Tc units) is 13792

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in Table A.5.4.1.x.2-1 for only TRP#1. The timing of the DL path of TRP#2 is not changed.

Table A.5.4.1.x.2-1 Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 240 | +8*64Tc | +4*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first path (in time) of DL SSB of each TAG used by the UE to determine downlink timing is received from the reference cell at the UE antenna. For TRP#2, the test system shall verify there is no adjustment. Skip this step for Test 3 and Test 4 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first path (in time) of DL SSB of each TAG used by the UE to determine downlink timing is received from the reference cell at the UE antenna. For Test 3 and Test 4 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment

=====End of change #10 =====

===== Start of change #11 including change 2 in R4-2410320 =====

A.6.4.1	UE transmit timing

A.6.4.1.X	NR UE Transmit Timing Test for two TRPs in FR1

A.6.4.1.X.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits for both TRPs. The test is configured with two TRPs in NR PCell. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in Table A.6.4.1.1.1-1.

Table A.6.4.1.X.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 2 | NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 3 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

For this test a single NR cell is used. Table A.6.4.1.X.1-2 defines the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.6.4.1.X.1-3.

For UE not support the capability of “rxTimingDiff-r18”, the UE is only required to be tested in Test1 and Test3.

For UE supports the capability of “rxTimingDiff-r18”, the UE is only required to be tested in Test2 and Test4.

Table A.6.4.1.X.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  |  |  | Test2 |  |  |  |  |  | Test3 |  |  |  |  |  |  |  |  | Test4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TRP#1 | TRP#2 |  |  | TRP#1 |  |  | TRP#2 |  |  | TRP#1 |  |  |  |  |  | TRP#2 |  |  | TRP#1 |  |  | TRP#2 |
| SSB ARFCN |  | 1,2,3 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | 1 | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DRx Cycle | ms | 1,2,3 | N/A |  |  |  |  |  |  |  |  |  |  |  |  | DRX.8Note5 |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| coresetPoolIndex for dedicated CORESET Reference Channel |  | 1,2,3 | 0 |  |  | 1 |  |  | 0 |  |  | 1 |  |  | 0 |  | 1 |  |  | 0 |  |  | 1 |  |  |
| Timing difference compared to TRP#1 | us | 1, 2 | 0 |  |  | 3 |  |  | 0 |  |  | 30 |  |  | 0 |  | 3 |  |  | 0 |  |  | 30 |  |  |
|  |  | 3 |  |  |  | 1.5 |  |  |  |  |  |  |  |  |  |  | 1.5 |  |  |  |  |  |  |  |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  | 1,2 | SSB.3 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | SSB.4 FR1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSB index |  | 1,2,3 | 0 |  | 1 |  |  | 0 |  |  | 1 |  |  | 0 |  |  |  | 1 |  |  | 0 |  |  | 1 |  |
| SMTC Configuration |  | 1,2 | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3 | -98 |  |  |  |  |  |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1,2 | -98 |  |  |  |  |  |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |  |  |  |  |  |  | -95 |  |  |  |  |  |  |  |  |  |
|  |  | 1,2,3 | 3 |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |
|  |  | 1,2,3 | 3 |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |
| SS-RSRPNote3 | dBm/SCS | 1,2 | -95 |  |  |  |  |  |  |  |  |  |  |  |  | -95 |  |  |  |  |  |  |  |  |  |
|  |  | 3 | -92 |  |  |  |  |  |  |  |  |  |  |  |  | -92 |  |  |  |  |  |  |  |  |  |
| IoNote3 | dBm/9.36MHz | 1,2 | -65.2 |  |  |  |  |  |  |  |  |  |  |  |  | -65.2 |  |  |  |  |  |  |  |  |  |
|  | dBm/38.1MHz | 3 | -59.2 |  |  |  |  |  |  |  |  |  |  |  |  | -59.2 |  |  |  |  |  |  |  |  |  |
| Propagation condition |  | 1,2,3 | AWGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SRS Config |  | 1,2 | SRSConf.1Note6 |  |  |  |  |  |  |  |  |  |  |  |  | SRSConf.3Note6 |  |  |  |  |  |  |  |  |  |
|  |  | 3 | SRSConf.1Note6 |  |  |  |  |  |  |  |  |  |  |  |  | SRSConf.2Note6 |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRx related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.6.4.1.X.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.4.1.X.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | SRSConf.3 | Comments |
| --- | --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 | 0 |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | 14 for test configuration 1,2 25 for test configuration 3 | 25 | 14 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1, 0 | sl640, 0 | sl320, 0 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | 0 | Any 10 bit number |

A.6.4.1.X.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1) Setup NR PCell according to parameters given in Table A.6.4.1.1.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB of TRP#1 and TRP#2.

a.	The NTA offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in Table A.6.4.1.X.2-1 for only TRP#1. The timing of the DL path of TRP#2 is not changed.

Table A.6.4.1.X.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (KHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1&Test2 | Test3&Test4 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first path (in time) of DL SSB used by the UE to determine downlink timing is received from the reference cell at the UE antenna for TRP#1. For TRP#2, the test system shall verify there is no adjustment. Skip this step for test 3&4 with DRX configured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first path (in time) of DL SSB used by the UE to determine downlink timing is received from the reference cell at the UE antenna of TRP#1. For Test 3&4 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

=====End of change #11 =====

===== Start of change #12 including change 3 in R4-2410323 =====

A.6.5.X1	Active TCI state switch delay

A.6.5.X1.1	MAC-CE based joint TCI state switch for mDCI with two TA when RTD is larger than CP

##### A.6.5.X1.1.1	Test Purpose and Environment

The purpose of this test is to verify the active TCI state switch delay requirement defined in clause 8.22.3. Supported test configuration is shown in Table A.6.5.X1.1.1-1.

The test scenario comprises of one NR PCell (Cell 1) containing two TRPs (i.e., TRP 0 and TRP 1) belonging to two TAGs as given in Table A.6.5.X1.1.1-2. Cell-specific parameters of NR PCell are specified in Table A.6.5.X1.1.1-3 below.

PDCCHs associated with corsetPoolIndex 0 and 1 indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending on PUCCH associated with TRP 0.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is configured with 1 different TCI states associated with coresetPoolIndex 0 for PCell, PDCCH TCI state 0 (QCL’d to SSB0), in Cell 1 before starting the test. UE is configured with 2 TCI states associated with coresetPoolIndex 1 for PCell, PDCCH TCI state 0 (QCL’d to SSB1) and TCIstate 1 (QCL’d to SSB2),.

-	UE is indicated in TCI state 0 as the active PDCCH TCI state for PDCCH associated with coresetPoolIndex 0, and UE is indicated in TCI state 0 as the active PDCCH TCI state for PDCCH associated with coresetPoolIndex 1.

-	Target TCI state is not in the active TCI state list.

The test consists of two time periods, T1 and T2. During T1 only SSB0 and SSB2 are transmitted. At the beginning of T2, the SSB1 corresponding to TCI state 1 starts transmitting. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280ms of UE providing L1-RSRP report with results for both SSB0 and SSB1, UE receives a MAC-CE command indicating a switch to TCI state 1 for PDCCH associated with coresetPoolIndex 1. tci-PresentInDCI is not configured in the PDSCH configuration, i.e. TCI state for the PDSCH is identical to the PDCCH TCI state.

The test equipment verifies that UE can be scheduled on PCell on TCI state 0 associated with coresetPoolIndex 0  till n+ THARQ +3 ms. The test equipment also verifies the TCI state switch time in PCell by scheduling the UE on TCI state 1 associated with coresetPoolIndex 0  after n+ THARQ +3 ms + (Tfirst-SSB + TSSB-proc).

Table A.6.5.X1.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 2 | TDD duplex mode, 15 kHz SSB SCS, 10 MHz bandwidth |
| 3 | TDD duplex mode, 30 kHz SSB SCS, 40 MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.X1.1.1-2: General test parameters for TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| Timing offset between SSB 0 and SSB1 | us | [25] |  |
| Timing offset between SSB0 and SSB2 | us | [33] |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |

Table A.6.5.X1.1.1-3: NR Cell specific test parameters for TCI state switch

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2,3 |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.1.2 |
| BWchannel | Config 1,2 |  | 10: NRB,c = 52 |
|  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| PDSCH Reference | Config 1 |  | SR.1.1 FDD |
| measurement channel Note 4 | Config 2 |  | SR.1.1 TDD |
|  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET | Config 1 |  | CR.1.1 FDD |
| parameters | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET | Config 1 |  | CCR.1.2 FDD |
| parameters | Config 2 |  | CCR.1.2 TDD |
|  | Config 3 |  | CCR.2.4 TDD |
| TRS Configuration for TRP 0 | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| TRS Configuration for TRP 1 Set 1 | Config 1 |  | TRS.1.5 FDD |
|  | Config 2 |  | TRS.1.5 TDD |
|  | Config 3 |  | TRS.1.7 TDD |
| TRS Configuration for TRP 1 Set 2 | Config 1 |  | TRS.1.6 FDD |
|  | Config 2 |  | TRS.1.6 TDD |
|  | Config 3 |  | TRS.1.8 TDD |
| OCNG Patterns | Config 1,2 |  | OP.1 |
|  | Config 3 |  | OP.1 |
| SSB Configuration for TRP 0 | Config 1,2 |  | SSB.2 FR1 |
|  | Config 3 |  | SSB.4 FR1 |
| SSB Configuration for TRP 1 | Config 1,2 |  | SSB.7 FR1 |
|  | Config 3 |  | SSB.8 FR1 |
| TCI state 0 associated with coresetPoolIndex 0 |  |  | DLorJoint TCI.State.0 |
| TCI state 0 associated with coresetPoolIndex 1 |  |  | DLorJoint TCI.State.6 |
| TCI state 1 associated with coresetPoolIndex 1 |  |  | DLorJoint TCI.State.7 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |
| NocNote 2 | Config 1,2 | dBm/SCS | -104 |
|  | Config 3 |  | -101 |
| NocNote 2 |  | dBm/15KHz | -104 |
| SS-RSRP Note 3 | Config 1,2 | dBm/SCS | -87 |
|  | Config 3 |  | -84 |
| Ês/Iot |  | dB | 17 |
| Ês/Noc |  | dB | 17 |
| IoNote3 | Config 1,2 | dBm/ 9.36MHz | -58.96 |
|  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4 PDSCH RMC for TRP0 and TRP1 are scheduled in non-overlapping RBs |  |  |  |

##### A.6.5.X1.1.2	Test Requirements

During T2, UE shall send L1-RSRP report with results for both SSB0 and SSB1.

After receiving MAC-CE command in slot n, UE shall be able to continue receive on TRP 0, and for TRP 1, UE shall:

-	be able to continue to receive on TCI state 0 till   n+ THARQ +3 ms

-	be able to start receiving on TCI state 1 after n+ THARQ +5 ms + Tfirst-SSB

The rate of correct events observed during repeated tests shall be at least 90%.

=====End of change #12 =====

===== Start of change #13 including change 2 in R4-2410319 =====

A.6.7.X	TDCP amplitude measurement accuracy

A.6.7.X.1        TDCP amplitude measurement accuracy in FR1

A.6.7. X.1.1	Test Purpose and Environment

The purpose of this test is to verify that the TRS based TDCP amplitude measurement accuracy is within the specified limits in the test requirements section. The cofigurations for the test are specified in Table A.6.7.X.1.1-1.

The test consists of two tests, Test 1 and Test 2. Each test further consists of two subtests Test 1A, 1B and Test 2A, 2B.

Test 1A: 10 Hz doppler + 15kHz SCS FDD + 20dB SNR

Test 1B:  10 Hz doppler + 30kHz SCS TDD + 20 dB SNR

Test 2A: 300 Hz doppler + 15kHz SCS FDD + [10] dB SNR

Test 2B: 300 Hz doppler + 30kHz SCS TDD + [10] dB SNR

Relevant parmeters for each test are provided in the table A.6.7.X.1.2-1. UE needs to pass Test 1A, 1B, 2A, 2B.

Table A.6.7.X.1.1-1: Applicable NR configurations for FR1 TRS based TDCP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

A.6.7. X.1.2	Test parameters

In this set of test cases there is one cell in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.6.7.X.1.2-1. Ampliutude of TDCP is tested by using the parameters in Table A.6.7.X.1.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured with 1 TRS set with the TRS resources in the set are configured in adjacent slot. UE is configured to perform L1-RSRP measurement based on the SSB resources 0 and 1.

Table A.6.7.X.1.2-1: FR1 TRS based TDCP test parameters

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 | freq1 |
| Duplex mode | 1 |  | FDD | FDD |
|  | 2 |  | TDD | TDD |
| TDD Configuration | 1 |  | N/A | N/A |
|  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  | 2 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  | 2 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  | 2 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  | 2 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  | 2 |  | SSB.4 FR1 | SSB.4 FR1 |
| OCNG Patterns | 1~2 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration | 1 |  | TRS.1.5 FDD | TRS.1.5 FDD |
|  | 2 |  | TRS.1.5 TDD | TRS.1.5 TDD |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~2 |  | SMTC.1 | SMTC.1 |
| reportConfigType | 1~2 |  | periodic | periodic |
| reportQuantity | 1~2 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS | 1~2 |  | 2 | 2 |
| L1-RSRP reporting period | 1~2 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Noc | dBm/15kHz | 1 | -98 | -98 |
|  |  | 2 |  |  |
| Note 2 | dBm/SCS | 1 | -98 | -98 |
|  |  | 2 | -95 | -95 |
|  | 1~2 | dB | TBD | TBD |
|  | 1~2 | dB | 10 | 20 |
| SSB_RP | 1 |  | -88 | -78 |
| Propagation condition | 1~2 |  | TDL-A 30ns 10 Hz | TDL-A 30ns 300Hz |
| Antenna configuration | 1~2 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |

A.6.7.X.1.3	Test Requirements

For Test 1A, the reported TDCP index shall be smaller than or equal to 6 for the 80% of the times over repeated tests

For Test 1B:  the reported TDCP index shall be smaller than or equal to 5 for the 80% of the times over repeated tests

For Test 2A: the reported TDCP index shall be lrager than 8 for the 80% of the times over repeated tests

For Test 2B: the reported TDCP index shall be larger than [or equal to] 6 for the 80% of the times over repeated tests

=====End of change #13 =====

===== Start of change #14 including change 2 in R4-2410321 =====

A.7.4.1.1	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18

A.7.4.1.1.1	Test Purpose and environment

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits, for UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 and is configured with 2 TAGs for multi-DCI multi-TRP operation. UE is also configured with dl-OrJointTCI-StateList or ul-TCI-State-List. This test will verify the requirements in clause 7.1.2.

Supported test configurations are shown in Table 7.4.1.1.1-1.

Table A.7.4.1.1.1-1: Supported test configurations for FR2 PCell

| Configuration | Description |
| --- | --- |
| 1 | NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz |

For this test a single NR cell is used. Tables A.7.4.1.1.1-2 and A.7.4.1.1.1-2A define the parameters to be configured and strength of the transmitted signals. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.4.1.1.1-3.

For UE not support the capability of “rxTimingDiff-r18”, the UE is only required to be tested in Test1 and Test3.

For UE supports the capability of “rxTimingDiff-r18”, the UE is only required to be tested in Test2 and Test4.

Table A.7.4.1.1.1-2: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  | Test2 |  | Test3 |  | Test4 |  | Band Group |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TRP #1 | TRP #2 | TRP #1 | TRP #2 | TRP #1 | TRP #2 | TRP #1 | TRP #2 |  |
| SSB ARFCN |  | 1,2 | Freq1 |  |  |  |  |  |  |  |  |
| Duplex Mode |  | 1,2 | TDD |  |  |  |  |  |  |  |  |
| TDD configuration |  | 1,2 | TDDConf.3.1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 1,2 | 100: NRB,c = 66 |  |  |  |  |  |  |  |  |
| Data RBs allocated |  | 1,2 | 66 |  |  |  |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |  |  |  |
| TRS Configuration |  | 1,2 | TRS.2.1 TDD TRS.2.2 TDD |  |  |  |  |  |  |  |  |
| DRx Cycle | ms | 1,2 | N/A |  |  |  | DRX.8Note5 |  |  |  |  |
| PDSCH Reference measurement channel |  | 1,2 | SR.3. 3 TDD |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | 1,2 | CR.3.2 TDD |  |  |  |  |  |  |  |  |
| coresetPoolIndex for dedicated CORESET Reference Channel |  | 1,2 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  |
| Dedicated CORESET Reference Channel |  | 1,2 | CCR.3.4 TDD | CCR.3.6 TDD | CCR.3.4 TDD | CCR.3.6 TDD | CCR.3.4 TDD | CCR.3.6 TDD | CCR.3.4 TDD | CCR.3.6 TDD |  |
| TCI configuration |  | 1,2 | DLorJoint TCI.State.0 with tag-Id-ptr-r18 = n0 | DLorJoint TCI.State.1 with tag-Id-ptr-r18 = n1 | DLorJoint TCI.State.0 with tag-Id-ptr-r18 = n0 | DLorJoint TCI.State.1 with tag-Id-ptr-r18 = n1 | DLorJoint TCI.State.0 with tag-Id-ptr-r18 = n0 | DLorJoint TCI.State.1 with tag-Id-ptr-r18 = n1 | DLorJoint TCI.State.0 with tag-Id-ptr-r18 = n0 | DLorJoint TCI.State.1 with tag-Id-ptr-r18 = n1 |  |
| Timing difference compared to TRP#1 | us | 1,2 | 0 | 0.1 | 0 | 8 | 0 | 0.1 | 0 | 8 |  |
| OCNG Patterns |  | 1,2 | O P. 1 |  |  |  |  |  |  |  |  |
| SSB Configuration |  | 1,2 | SSB.1 FR2 |  |  |  |  |  |  |  |  |
| SMTC Configuration |  | 1,2 | SMTC.1 |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2 | 120 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2 | 0 |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition |  | 1,2 | AWGN |  |  |  |  |  |  |  |  |
| SRS Config |  | 1,2 | SRSConf.1Note6 |  |  |  | SRSConf.2Note6 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: VoidNote 4: Void Note 5: DRx related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.5.4.1.x.1-3 |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.4.1.1.1-2A: OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 3 as specified in clause A.3.15 Note 7 |  |
| Assumption for UE beamsNote 6 |  | Fine |  |
| Note1 | dBm/15kHzNote4 | -112 |  |
| Note1 | dBm/SCSNote3 | -100 |  |
|  | dB | 4 |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -96 |  |
|  | dB | 4 |  |
| IoNote2 | dBm/95.04 MHz Note4 | -68.5 |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 7: AoA1 for TRP1 and AoA2 for TRP2 |  |  |  |

Table A.7.4.1.1.1-3: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 |  | SRSConf.2 |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 1 | 0 | 1 |  |
|  | srs-ResourceIdList | 0 | 1 | 0 | 1 |  |
|  | resourceType | Periodic | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 1 | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | 17 | 17 | 17 | 17 | Matches NRB,c |
|  | freqHopping b-SRS | 0 | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl2, 0 | sl2, 1 | sl2560, 4 | sl2560, 9 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | 0 | 0 | Any 10 bit number |

Table A.7.4.1.1.1-4: Void

A.7.4.1.1.2	Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test:

1)	Setup NR PCell according to parameters given in Table A.7.4.1.1.1-1.

2)	After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB for each TAG.

a.	The NTA offset value (in Tc units) is 13792

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

3)	The test system shall adjust the timing of the DL path by values given in Table A.7.4.1.1.2-1 for only TRP#1. The timing of the DL path of TRP#2 is not changed.

Table A.7.4.1.1.2-1 Adjustment Value for DL Timing

| SCS of SSB signals (kHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 240 | +8*64Tc | +4*64Tc |

4)	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in clause 7.1.2 Table 7.1.2.1-1 until the UE transmit timing offset is within (NTA + NTA_offset) ×Tc ± Te respective to the first path (in time) of DL SSB of each TAG used by the UE to determine downlink timing is received from the reference cell at the UE antenna. For TRP#2, the test system shall verify there is no adjustment. Skip this step for Test 3 and Test 4 with DRX confiured.

5)	The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first path (in time) of DL SSB used by the UE to determine downlink timing is received from the reference cell at the UE antenna. For Test 3 and Test 4 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

=====End of change #14 =====

===== Start of change #15 including change 4 in R4-2410416 =====

A.7.5.13.X	sDCI MAC-CE based joint TCI state switching

A.7.5.13.X.1	Test Purpose and Environment

The purpose of this test is to verify both active downlink and uplink TCI state switch delay requirement defined in clause 8.21 and 8.23, respectively, by using joint TCI state of unified TCI state switch framework. Supported test configuration is shown in Table A.7.5.13.X.1-1.

Table A.7.5.13.X.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.13.X.2-2: General test parameters for dual TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| L1-RSRP reporting period | slot | 320 | Periodic L1-RSRP reporting configured |
| L1-RSRP measured RS |  | SSB0 and SSB2 of TRP0, SSB1 and SSB3 of TRP1 | L1-RSRP measurements of SSB0, SSB1 SSB2 and SSB3. |
| Number of RS for L1-RSRP reporting |  | 4 | SSB0, SSB1, SSB2 and SSB3 in Joint TCI state 0, 1, 2, and 3. |
| Cell2 timing offset to cell1 | us | <CP |  |
| T1 | s | 0.2 |  |
| T2 | s | 1 | UE is required to activate dual joint TCI states (TCI state 2 and TCI state 3). |

A.7.5.13.X.2	Test parameters

The test scenario comprises of one NR PCell (Cell 1) as given in Table A.7.5.13.X.2-1. Cell-specific parameters of NR PCell are specified in Table A.7.5.13.X.2-2 below. The OTA related test parameters for FR2 are shown in Table A.7.5.13.X.2-3.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	PDCCHs indicating new transmissions shall be sent continuously on Pcell to ensure that the UE would have ACK/NACK sending.

-	UE is provided with dl-OrJoint-TCIStateList-r17 and UE’s higher layer signalling unifiedTCI-StateType-r17 in IE MIMOParam-r17 is set to joint.

-	applyIndicatedTCI-State-r18 is set as {first} for TRP0 (CORESET index p associated with Joint TCI state 0 and Joint TCI state 2) and as {second} for TRP1 (CORESET index q associated with Joint TCI state 1 and Joint TCI state 3).

-	tci-SelectionPresentInDCI-r18 is configured in the BWP configuration, i.e. TCI state for the PDSCH is indicated by DCI format 1_1 and PDSCHs on two TRPs are scheduled in TDM manner.

-	UE is configured with two joint TCI states (TCI state 0 and TCI state 2) for TRP0 and two joint TCI states (TCI state 1 and TCI state 3) for TRP1. QCL info to Joint TCI state 0,1,2 and 3 are provided by SSB0, SSB1, SSB2 and SSB3, respectively.

-	UE is indicated in TCI state 0 and TCI state 1 as the active TCI state for TRP0 and TRP1.

The test consists of two time periods, T1 and T2. During T1, SSB0 in joint TCI state 0 and SSB1 in joint TCI state 1 are transmitted.

At the beginning of T2, SSB2 in joint TCI state 2 and SSB3 in joint TCI state 3 start transmitting. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280ms after the slot in which UE provides L1-RSRP report with results for SSB2 and SSB3 in joint TCI state 2 and joint TCI state 3, UE receives a MAC-CE command indicating a switch to dual joint TCI state 2 and TCI state 3 for two TRPs.

The test equipment verifies that UE can be scheduled by two TRPs on joint TCI state 0 and joint TCI state 1 till slot n+ THARQ +3Nslotsubframe,µ. The test equipment also verifies the TCI state switch time for two TRPs by scheduling the UE on joint TCI state 2 and joint TCI state 3 after slot n+THARQ + 3Nslotsubframe,µ + max{NM1* (Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2ms), NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2ms) }/ NR slot length.

Table A.7.5.13.X.2-1: General test parameters for dual TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| L1-RSRP reporting period | slot | 320 | Periodic L1-RSRP reporting configured |
| L1-RSRP measured RS |  | SSB0 and SSB2 of TRP0, SSB1 and SSB3 of TRP1 | L1-RSRP measurements of SSB0, SSB1 SSB2 and SSB3. |
| Number of RS for L1-RSRP reporting |  | 4 | SSB0, SSB1, SSB2 and SSB3 in Joint TCI state 0, 1, 2, and 3. |
| Cell2 timing offset to cell1 | us | <CP |  |
| T1 | s | 0.2 |  |
| T2 | s | 1 | UE is required to activate dual joint TCI states (TCI state 2 and TCI state 3). |

Table A.7.5.13.X.2-2: NR Cell specific test parameters for dual TCI state switch

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Data RBs allocated |  | 66 |
| Initial BWP Configuration |  | DLBWP.0.2 ULBWP.0.2 |
| Dedicated BWP Configuration |  | DLBWP.1.1 ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3. 2 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters (CORESET index p) |  | CCR.3.1 TDD |
| Dedicated CORESET parameters (CORESET index q) |  | CCR.3.2 TDD |
| OCNG Patterns |  | OP.5 defined in A.3.2.1 |
| SSB Configuration |  | SSB.17 FR2 |
| SMTC Configuration |  | SMTC.1 |
| Joint TCI State 0 |  | DLorJoint TCI.State.2 |
| Joint TCI State 1 |  | DLorJoint TCI.State.6 |
| Joint TCI State 2 |  | DLorJoint TCI.State.7 |
| Joint TCI State 3 |  | DLorJoint TCI.State.8 |
| TRS Configuration |  | TRS.2.1 TDD for DLorJoint TCI.State.2 TRS.2.4 TDD for DLorJoint TCI.State.6 TRS.2.5 TDD for DLorJoint TCI.State.7 TRS.2.6 TDD for DLorJoint TCI.State.8 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.5.13.X.2-3: OTA related test parameters for dual TCI state switch

| Parameter | Units | TRP0 |  |  |  | TRP1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB2 |  | SSB1 |  | SSB3 |  |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  | Setup [TBD] according to clause A.3.XX |  |  |  |  |  |  |  |
|  |  | AoA1 |  | AoA2 |  | AoA2 |  | AoA3 |  |
| Assumption for UE beams Note 6 |  | Rough |  |  |  |  |  |  |  |
| Ês | dBm/SCS | -80.6 | -80.6 | -Infinity | -80.6 | -80.6 | -80.6 | -Infinity | -80.6 |
| SS B_RP Note 2 | dBm/ SCS | -80.6 | -80.6 | -Infinity | -80.6 | -80.6 | -80.6 | -Infinity | -80.6 |
| BB Note 7 | dB | 8.3 | 8.3 | -Infinity | 8.3 | 8.3 | 8.3 | -Infinity | 8.3 |
| IoNote2 | dBm/95.04 MHz Note4 | -56.0 | -56.0 | - Infinity | -56.0 | -56.0 | -56.0 | - Infinity | -56.0 |
| Note 1: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 3: As observed with 0dBi gain antenna at the center of the quiet zone. Note 4: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |  |

A.7.5.13.X.3	Test Requirements

During T2, the test verifies that UE can be scheduled by two TRPs on joint TCI state 2 and joint TCI state 3.

After receiving MAC-CE command in slot n, UE shall:

-	be able to continue to receive and transmit with joint TCI state 0 and joint TCI state 1 till slot n+ THARQ +3Nslotsubframe,µ

-	be able to start receiving and transmitting with joint TCI state 2 and joint TCI state 3 after slot n+THARQ + 3Nslotsubframe,µ + max{NM1* (Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2ms), NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS 2+ 2ms) }/ NR slot length.

-	where NM1=1, NM2=1.

The rate of correct events observed during repeated tests shall be at least 90%.

=====End of change #15 =====

===== Start of change #16 including change 1 in R4-2410322 =====

A.7.5.13.x	MAC-CE based dual downlink TCI state switching delay for unified TCI for single-DCI mTRP

A.7.5.13.x.1	NR PCell FR2 dual downlink TCI state switch in sDCI for known case

A.7.5.13.x.1.1	Test Purpose and Environment

The purpose of this test is to verify the MAC-CE based dual downlink TCI state switch delay requirement defined in clause 8.21.3. Supported test configuration is shown in Table A.7.5.13.x.1.1-1.

Table A.7.5.13.x.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

A.7.5.13.x.1.2	Test Parameters

There is one active serving cell (Cell 1) configured with two TRPs in the test. Cell-specific parameters of Cell 1 are specified in Table A.7.5.13.x.1.2-2 below. The OTA related test parameters for FR2 are shown in Table A.7.5.13.x.1.2-3.

PDCCHs indicating new transmissions shall be sent continuously on TRP0 to ensure that the UE would have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1  on radio channel 1 (PCC).

-	UE is provided with dl-OrJoint-TCIStateList-r17 and UE’s higher layer signalling unifiedTCI-StateType-r17 in IE MIMOParam-r17 is set to separate.

-	UE is configured with SSB-based L1-RSRP measurements on Cell 1

-	UE is configured with two TCI states (TCI state 0 and TCI state 2) for TRP0 and two TCI states(TCI state 1 and TCI state 3) for TRP1. QCL info to DL TCI state 0, 1 , 2 and 3 are provided by SSB0, SSB1, SSB2 and SSB3, respectively.

-	UE is indicated in TCI state 0 and TCI state 1 as the active TCI state for TRP0 and TRP1

The test consists of two time periods, T1 and T2. During T1, source RS in TCI state 0 and TCI state 1 are transmitted. At the beginning of T2, source RS in TCI state 2 and source RS in TCI state 3 start transmitting. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280ms after the slot in which UE provides L1-RSRP report with results for source RSs in TCI state 2 and 3, UE receives a MAC-CE command indicating a switch to TCI state 2 and 3 for two TRPs. tci-PresentInDCI is not configured in the PDSCH configuration, i.e. TCI state for the PDSCH is identical to the PDCCH TCI state.

The test equipment verifies that UE can be scheduled by two TRPs on TCI state 0 and TCI state 1 till slot n+ THARQ +3Nslotsubframe,µ. The test equipment also verifies the TCI state switch time for two TRPs by scheduling the UE on TCI state 2 and TCI state 3 after slot after slot n+ THARQ + 3Nslotsubframe,µ+ max{TOk1*(Tfirst-SSB1 + AD1*TSSB1  + TSSB-proc), TOk2*(Tfirst-SSB2 + AD2*TSSB2 + TSSB-proc)} / NR slot length.

Table A.7.5.13.x.1.2-1: General test parameters for TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| L1-RSRP reporting period | slot | 160 | Periodic L1-RSRP reporting configured |
| L1-RSRP measured RS |  | SSB0 and SSB2 of TRP0, SSB1 and SSB3 of TRP1 | L1-RSRP measurements of SSB0, SSB1 SSB2 and SSB3. |
| Number of RS for L1-RSRP reporting |  | 4 | four source RSs in TCI state 0,1, 2,3. |
| Cell2 timing offset to cell1 | us | <CP |  |
| T1 | s | 0.2 |  |
| T2 | s | 1 |  |

Table A.7.5.13.x.1.2-2: NR Cell specific test parameters

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Data RBs allocated |  | 24 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3. 2 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP. 5 |
| SSB Configuration |  | SSB.17 FR2 |
| SMTC Configuration |  | SMTC.1 |
| DL TCI State 0 |  | DLorJoint TCI.State.0 |
| DL TCI State 1 |  | DLorJoint TCI.State.1 |
| DL TCI State 2 |  | DLorJoint TCI.State.6 |
| DL TCI State 3 |  | DLorJoint TCI.State.7 |
| TRS Configuration |  | TRS.2.1 TDD for DLorJoint TCI.State.0 TRS.2.2 TDD for DLorJoint TCI.State.1 TRS.2.4 TDD for DLorJoint TCI.State.6 TRS.2.5 TDD for DLorJoint TCI.State.7 |
| Pathloss RS Configuration |  | Resource #4 in TRS.2.1 TDD |
| reportQuantity for SSB |  | ssb-Index-RSRP-Index-r17 |
| reportConfigType for SSB |  | periodic |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.5.13.x.1.2-3: OTA related test parameters for TCI state switch

| Parameter | Units | TRP0 |  |  |  | TRP1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB2 |  | SSB1 |  | SSB3 |  |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  | Setup [TBD] according to clause A.3.XX |  |  |  |  |  |  |  |
|  |  | AoA1 |  | AoA2 |  | AoA2 |  | AoA3 |  |
| Assumption for UE beams Note 6 |  | Rough |  |  |  |  |  |  |  |
| Ês | dBm/SCS | -80.6 | -80.6 | -Infinity | -80.6 | -80.6 | -80.6 | -Infinity | -80.6 |
| SS B_RP Note 2 | dBm/ SCS | -80.6 | -80.6 | -Infinity | -80.6 | -80.6 | -80.6 | -Infinity | -80.6 |
| BB Note 7 | dB | 8.3 | 8.3 | -Infinity | 8.3 | 8.3 | 8.3 | -Infinity | 8.3 |
| IoNote2 | dBm/95.04 MHz Note4 | -56.0 | -56.0 | - Infinity | -56.0 | -56.0 | -56.0 | - Infinity | -56.0 |
| Note 1: Void Note 2: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |  |

A.7.5.13.x.1.3	Test Requirements

During T2, the test verifies that UE can be scheduled by two TRPs on TCI state 2 and TCI state 3.

After receiving MAC-CE command in slot n, UE shall:

-	be able to continue to receive on DL TCI state 0 and DL TCI state 1 till slot n+ THARQ +3Nslotsubframe,µ

-	be able to start receiving on DL TCI state 2 and DL TCI state 3  after slot n+ THARQ + 3Nslotsubframe,µ+ max{TOk1*(Tfirst-SSB1 + AD1*TSSB1  + TSSB-proc), TOk2*(Tfirst-SSB2 + AD2*TSSB2 + TSSB-proc)} / NR slot length

The rate of correct events observed during repeated tests shall be at least 90%.

=====End of change #16 =====

===== Start of change #17 including change 4 in R4-2410324 =====

A.7.5.13		Unified TCI state switching delay

>>> Unchanged sections omitted <<<<<

A.7.5.13.y		MAC-CE based active uplink TCI state switch for single-DCI mTRP

A.7.5.13.y.1		NR FR2 PCell uplink TCI state switch for two known TCI states

A.7.5.13.y.1.1	Test Purpose and Environment

The purpose of this test is to verify fulfillment of the uplink TCI switch delay requirement defined in clause 8.23.3 by a UE capable of beam correspondence without the need for UL beam sweeping. The test scenario comprises one PCell (Cell 1) with two TRPs.

Throughout the test, PDCCH indicating new transmissions shall be sent continuously on PCell to ensure that the UE will send ACK/NACKs on PUCCH.

Before the test starts,

-	UE is connected to Cell 1 on radio channel 1.

-	UE is configured with a unified DL TCI state, TCI State-0, and SSB0 is configured as QCL source for the TCI state. At the start of test UE is connected to DL TCI state 0.

-	UE is configured with 4 UL TCI states, UL TCI state 0, UL TCI state 1, UL TCI state 2, and UL TCI state 3. QCL info to UL TCI state 0, 1, 2, and 3 is provided by SSB0, SSB1, SSB2, and SSB3, respectively. Initially only UL TCI 0 is in the active TCI state list.

-	PL-RS is configured for each of the UL TCI states. CSI-RS 0, CSI-RS 1, CSI-RS 2, and CSI-RS 3 are associated with UL TCI state 0, 1, 2, and 3 respectively as PL-RS.

-	At the start of the test UE is connected to DL TCI state 0, UL TCI state 0, and UL TCI state 1.

Index of CSI-RS#1 is configured for UE as PUSCH-PathlossReferenceRS-Id-r17 which is indicated in TCI-UL-State-r17 of uplink TCI state 1. CSI-RS#1 is QCLed typeD with SSB#1. UE does not maintain CSI-RS#1 as pathloss RS before the uplink TCI state switching.

Index of CSI-RS#2 is configured for UE as PUSCH-PathlossReferenceRS-Id-r17 which is indicated in TCI-UL-State-r17 of uplink TCI state 2. CSI-RS#2 is QCLed typeD with SSB#2. UE does not maintain CSI-RS#2 as pathloss RS before the uplink TCI state switching.

The test consists of two time periods, T1 and T2. During T1, only the SSBs associated with DL TCI state-0, UL TCI state 0, and UL TCI state 1 are transmitted. At the beginning of T2, transmission of the SSB 2 associated with UL TCI state 2 and transmission of the SSB 3 associated with UL TCI state 3 start. The UE conducts periodic L1-RSRP (i.e., SSB-Index-RSRP) reporting for SSB0, SSB1, SSB2, and SSB3. In slot n, which is within 1280 ms after UE receiving SSB0, SSB1, SSB2, and SSB3, and after reporting valid results for the SSB0, the SSB1, the SSB2 and the SSB3, the UE receives a MAC-CE indicating a TCI state switch to UL TCI state 2 and to UL TCI state 3.

The test equipment verifies that the UE transmits according to UL TCI state 0 and UL TCI state 1 up until slot n + THARQ + 3Nslotsubframe,µ, and according to UL TCI state 2 and to UL TCI state 3 from slot n + THARQ + 3Nslotsubframe,µ + max{NM1* (Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2ms), NM2* (Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS2 + 2ms)} / NR slot length and onwards. NM1 is equal to 1. NM2 is equal to 1. Where, THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

A.7.5.13.y.1.2	Test parameters

The supported test configurations are provided in Table A.7.5.13.y.1.2-1.

General test parameters are provided in Table A.7.5.13.y.1.2-2.

Cell-specific parameters are provided in Table A.7.5.13.y.1.2-3.

OTA-related test parameters are provided in Table A.7.5.13.y.1.2-4.

Table A.7.5.13.y.1.2-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.13.y.1.2-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| L1-RSRP reporting period | slot | 160 | Periodic L1-RSRP reporting configured |
| L1-RSRP measured RS |  | SSB0, SSB1, SSB2, SSB3 | L1-RSRP measurements of SSB0, SSB1, SSB2 and SSB3. |
| Number of reported RS |  | 3 | L1-RSRP reporting of measurements on SSB0, SSB1, SSB2, and SSB3. |
| T1 | s | 0.2 |  |
| T2 | s | 2 |  |

Table A.7.5.13.y.1.2-3: NR Cell specific test parameters

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Data RBs allocated |  | 24 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP.5 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| DL TCI State |  | DLorJoint TCI.State.0 |
| PL-RS Configuration for CSI-RS#0 |  | Resource #4 in TRS.2.1 TDD for UL TCI.State.2 |
| PL-RS Configuration for CSI-RS#1 |  | Resource #4 in TRS.2.2 TDD for UL TCI.State.3 |
| PL-RS Configuration for CSI-RS#2 |  | Resource #4 in TRS.2.4 TDD for UL TCI.State.4 |
| PL-RS Configuration for CSI-RS#3 |  | Resource #4 in TRS.2.5 TDD for UL TCI.State.5 |
| UL TCI State-0 Configuration |  | UL TCI.State.2 |
| UL TCI State-1 Configuration |  | UL TCI.State.3 |
| UL TCI State-2 Configuration |  | UL TCI.State.4 |
| UL TCI State-3 Configuration |  | UL TCI.State.5 |
| reportConfigType |  | ssb-Index-RSRP |
| reportConfigType |  | periodic |
| timeRestrictionForChannelMeasurements |  | configured |
| TRS Configuration |  | TRS.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote 1 |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.5.13.y.1.2-4: OTA related test parameters

| Parameter | Unit | Cell 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  | SSB2 |  | SSB3 |  |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  | Setup X according to clause A.3.15.x |  |  |  |  |  |  |  |
|  |  | AoA1 |  | AoA2 |  | AoA1 |  | AoA3 |  |
| Assumption for UE beams Note 6 |  | Rough |  |  |  |  |  |  |  |
| NocNote 1 | dBm/15 kHz | -92.1 |  |  |  |  |  |  |  |
| NocNote 1 | dBm/SCS | -83.1 |  |  |  |  |  |  |  |
| Ês/Noc | dB | 1 |  | -infinity | 1 | -infinity | 1 | -infinity | 1 |
| SS-RSRP Note 2 | dBm/120 kHz Note3 | -82.1 |  | -infinity | -82.1 | -infinity | -82.1 | -infinity | -82.1 |
| IoNote2 | dBm/95.04 MHz Note4 | -55.0 |  | -58.5 | -55.0 | -58.5 | -55.0 | -58.5 | -55.0 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone. Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |

A.7.5.13.y.1.3	Test Requirements

During T2, the UE shall send L1-RSRP report with results for SSB0 and SSB1.

After receiving MAC-CE command in slot n, the UE shall:

-	Continue transmitting using UL TCI state 0 and UL TCI state 1 up to and including slot n + THARQ + 3Nslotsubframe,µ

-	Start transmitting using UL TCI state 2 and UL TCI state 3, from slot n + THARQ + 3Nslotsubframe,µ + max{Tfirst_target-PL-RS1 + 4*Ttarget_PL-RS1 + 2ms, Tfirst_target-PL-RS2 + 4*Ttarget_PL-RS2 + 2ms} / NR slot length and onwards.

The rate of correct events observed during repeated tests shall be at least 90%.

=====End of change #17 =====