3GPP TSG-RAN4 Meeting #111	R4-2410400

# Fukuoka, JP, 20th May – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4567 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR to TS 38.133 on performance requirements for Further NR mobility enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-5-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN4#110bis and RAN4#111, the following draft CRs were endorsed for Rel-18 FeMob enh. Perf. parts. This document includes the endoresed draft CRs: Endorsed in RAN4#110bis: TDoc Endorsed CR; CR title; Source companies. R4-2406498; TC for RACH-less intra-frequency cell switch from FR2 to FR2; Qualcomm. R4-2406453; draft CR for TC of PDCCH-order RACH on NBR within active BWP in FR1 (A.6.5.x.1); MediaTek inc., Ericsson. R4-2406454; draft CR for TC of RACH based intra-frequency PCell switch from FR1 to FR1 with early TCI state activation; MediaTek inc., Ericsson, CATT. R4-2406455; draft CR for TC of LTM intra-frequency L1-RSRP measurement in FR1 (A.6.6.x.1); MediaTek inc., Ericsson. R4-2406456; Test case for Inter-f L1-RSRP measurement with MG in FR2; Apple. R4-2406457; Test case for Inter-f L1-RSRP measurement without gap in FR2; Apple. R4-2406458; Draft CR on intra-f L1-RSRP accuracy requirements for neighbour cell in FR1 for LTM; CATT. R4-2406459; Draft CR on test cases for intra-f RACH based PSCell Cell switch from FR1 to FR1 for LTM; CATT. R4-2406460; Draft CR on test cases for intra-f L1-RSRP measurement in FR2 for LTM; CATT, ZTE. R4-2406461; DraftCR on accuracy requirements and test case for inter-frequency L1-RSRP measurement in FR1; CMCC. R4-2406462; Draft CR for LTM cell switch delay test cases (A.6.3.x.6, A.6.3.y.2 and A.7.3.x.2); Nokia. R4-2406463; CR on Inter-frequency L1-RSRP accuracy requirements in FR2; Huawei, HiSilicon. R4-2406464; Test case for RACH based Cell switch from FR2 to FR2 Intra-frequency cell switch; Huawei, HiSilicon. R4-2406465; Test case for Inter-frequency L1-RSRP measurement with MG in FR1; Huawei, HiSilicon. R4-2406466; Test case for Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1; Huawei, HiSilicon. R4-2406495; Draft CR on test case for PScell Cell switch from FR2 to FR2; OPPO. R4-2406469; Draft CR for test case of PDCCH-order RACH on NBR within active BWP in FR2; OPPO. R4-2406470; CR for test cases of PDCCH-order RACH for inter-frequency and measurement accuracy requirements in FR2; MediaTek Inc.. R4-2406501; DarftCR for test configurations of R18 LTM; MediaTek Inc.. R4-2406471; draftCR on Intra-frequency RACH-less cell switch from FR1 to FR1; ZTE Corporation. R4-2406472; draftCR on Intra-frequency L1-RSRP accuracy requirements for neighbour cell in FR2; ZTE Corporation. R4-2406474; draft CR on inter-frequency RACH based Cell switch from FR1 to FR1 for R18 LTM; vivo. R4-2404323; Test case for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC; Apple. R4-2406348; Draft CR on test cases for intra-frequency CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC; CATT. R4-2406349; DraftCR on inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC; CMCC. R4-2406350; Draft CR for FR1-FR1 to FR1-FR1 CHO+CPC test cases; Nokia. R4-2406486; Test case for NR conditional handover including target MCG and target SCG from NR-DC to NR-DC; Huawei, HiSilicon. R4-2406351; Draft CR for test case of CHO including target MCG and candidate SCG in NR-DC; OPPO. R4-2406352; draft CR for test case on CHO with CPC for FR1-FR2 to FR1-FR2 NR-DC; vivo. R4-2406353; draftCR on Inter-frequency CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC; ZTE. Endorsed in RAN4#111: TDoc Endorsed CR; CR title; Source companies. R4-2410309; draft CR on TC for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for R18 LTM; vivo. R4-2410310; Correction on L1-RSRP accuracy requirements on neighbor cells; Huawei, HiSilicon. R4-2410311; Draft CR on performance requirements for R18 LTM; MediaTek Inc., Apple, Qualcom, Nokia, OPPO, Ericsson. R4-2410312; Draft CR to TS 38.133 on performance requirements for R18 LTM; Ericsson, MediaTek inc., Qualcomm Incorporated. R4-2407354; draft CR for performance part of R18 NR mobility further enhancement part 2; Apple. R4-2410282; DraftCR on inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC; CMCC. R4-2410283; Draft CR to TS 38.133 for Test case of improment on SCG_Scell setup delay for FR1; Ericsson, Nokia, Apple, Qualcomm. R4-2410284; Draft CR to TS 38.133 for Test case of improment on SCG_Scell setup delay for FR2; Ericsson, Apple, Nokia, Qualcomm. R4-2408591; Update on test case for NR conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC; Huawei, HiSilicon. R4-2410411; Correction on subsequent conditional PSCell addition/change; ZTE Corporation, Sanechips, Nokia.. |
| Summary of change: |  | - Specify LTM L1-RSRP accuracy requirements (change#1,2,3,4) - Add test configurations for R18 LTM (change#5,6,7,8,10) - Update FR1-FR2 test applicability table to include new FR1-FR2 test cases. (change#9) - Introduce TCs for PDCCH-order RACH in FR1 (change#11) A.6.3.2.x.1 PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP A.6.3.2.x.2 PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM A.6.3.2.x.3 PDCCH-order RACH on neighbor cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP - Introduce TCs for Enhanced CHO for FR1-FR1 NR-DC (change#12,13,14) A.6.3.3.x NR conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC A.6.3.3.X1 NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC A.6.3.3.X2 NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC with complementary conditional handover configuration - Introduce TCs for PCell switch in FR1 (change#15) A.6.3.x.1 RACH-based Intra-frequency PCell switch from FR1 to FR1 A.6.3.x.2 RACH based Inter-frequency LTM PCell switch from FR1 to FR1 A.6.3.X.3 RACH-less Intra-frequency PCell switch from FR1 to FR1 A.6.3.X.4 RACH-less Intra-frequency PCell switch from FR1 to FR1 with L1-RSRP measurement - Introduce TCs for PSCell switch in FR1 (change#16) A.6.3.y.1 RACH-based intra-frequency LTM PSCell switch from FR1 to FR1 - Introduce TCs for Subsequent conditional PSCell addition/change in FR1-FR1 NR-DC (change#17) A.6.5.X.1 Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC A.6.5.X.2 Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC - Fix event from A1 to A4. - Add the Rel-18 specified FR1 scenario for both EMR and non-EMR capable UE test case (change#18,19,20) - Introduce TCs for L1-RSRP measurement delay in FR1 (change#21,22,23) - Introduce TCs for L1-RSRP measurement accuracy requirements in FR1 (change#24) - Introduce TCs for PDCCH-order RACH in FR2 (change#25) A.7. 3.2.x.1 PDCCH-order RACH on neighbor cell in FR2 when RACH BW is within active BWP A.7.3.2.x.2 PDCCH-order RACH on inter-frequency neighbor cell in FR2 - Introduce TCs for Enhanced CHO for FR1-FR2 NR-DC (change#26,27) A.7.3.3.X NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC A.7.3.3.Y NR conditional Handover including target MCG and candidate SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC - Introduce TCs for PCell switch in FR2 (change#28) A.7.3.x.1 RACH based Intra-frequency PCell switch from FR2 to FR2 A.7.3.x.2 RACH-less Intra-frequency PCell switch from FR2 to FR2 A.7.3.x.3 RACH-less Inter-frequency LTM PCell switch from FR2 to FR2 - Introduce TCs for PSCell switch in FR2 (change#29) A.7.3.x1.1 RACH-based Intra-frequency LTM PSCell switch from FR2 to FR2 - Introduce TCs for Subsequent conditional PSCell addition/change in FR1-FR2 NR-DC (change#30) A.7.5.X.1 Intra-frequency subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC A.7.5.X.2 Inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC - Fix event from A1 to A4 - Introduce TCs for L1-RSRP measurement delay in FR2 (change#31,32,33) - Add the Rel-18 specified FR2 scenario for both EMR and non-EMR capable UE test case (change#34) - Introduce TCs for L1-RSRP measurement accuracy requirements in FR2 (change#35) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Accuray requirements and test case for R18 mobility would still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new)10.1.19X, (new)10.1.19y, (new)10.1.20X, (new)10.1.20Y, (new)A.3.8.2.5, (new)A.3.8.2.6, (new)A.3.8.3.5, (new)A.3.8.3.6, A.13A, (new)A.3.16B, (new)A.6.3.2.x, (new)A.6.3.3.x, (new)A.6.3.3.X1, (new)A.6.3.3.X2, (new)A.6.3.x, (new)A.6.3.y, (new)A.6.5.X, (new) A.6.6.9.2, (new) A.6.6.9.3, (new) A.6.6.9.4, (new)A.6.6.x, (new)A.6.6.y, (new)A.6.6.z, (new)A.6.7.x, (new)A.7.3.2.X, (new)A.7.3.3.X, (new)A.7.3.3.Y, (new)A.7.3.x, (new)A.7.3.x1, (new)A.7.5.X, (new)A.7.6.x, (new)A.7.6.Y, (new)A.7.6.Z, (new)A.7.6.X2, (new)A.7.7.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

10.1.19X LTM Intra-frequency L1-RSRP accuracy requirements for FR1

10.1.19X.1	SSB based intra-frequency L1-RSRP accuracy requirements

10.1.19X.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based intra-frequency L1-RSRP in this clause apply to all SSBs of candidate neighbour cell(s) on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.19X.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.19X.1.1-1: SSB based L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| ±5.0 | ±9.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| ±8.5 | ±11.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

10.1.19X.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell(s) on the same frequency in FR1.The accuracy requirements in Table 10.1.19X.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.19X.1.2-1: SSB based L1-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±3 | ±4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

End of Change 1

Start of Change 2

10.1.19y	LTM Inter-frequency L1-RSRP accuracy requirements for FR1

10.1.19y.1	SSB based Inter-frequency L1-RSRP accuracy requirements

10.1.19y.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-RSRP in this clause apply to all SSBs of candidate neighbour cell(s) on a frequency in FR1 that is on a different frequency than the serving cell.

The accuracy requirements in Table 10.1.19y.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.19y.1.1-1: Inter-frequency L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| ±5.0 | ±9.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| ±8.5 | ±11.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

10.1.19y.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB from one cell on a frequency in FR1 compared to the largest measured value of L1-RSRP among all SSBs measured from another cell on another frequency in FR1.

The accuracy requirements in Table 10.1.19y.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 10.1.19y.1.2-1: Inter-frequency L1-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| ±5.0 | ±9.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

End of Change 2

Start of Change 3

10.1.20X	 LTM Intra-frequency L1-RSRP accuracy requirements for FR2

10.1.20X.1	SSB based intra-frequency L1-RSRP accuracy requirements

10.1.20X.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based intra-frequency L1-RSRP in this clause apply to all SSBs of candidate neighbour cell(s) on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 10.1.20X.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20X.1.1-1: SSB based L1-RSRP absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±6.5 | ±9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8.5 | 11.5 | ≥-3 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

10.1.20X.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the cell(s) on the same frequency in FR2.

The accuracy requirements in Table 10.1.20X.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20X.1.2-1: SSB based L1-RSRP relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 3 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 6.5 | 9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

End of Change 3

Start of Change 4

10.1.20Y LTM Inter-frequency L1-RSRP accuracy requirements for FR2

10.1.20Y.1	SSB based inter-frequency L1-RSRP accuracy requirements

10.1.20Y.1.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-RSRP in this clause apply to all SSBs of candidate neighbour cell(s) on a frequency in FR2 that is on a different frequency than the serving cell.

The accuracy requirements in Table 10.1.20Y.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20Y.1.1-1: SSB based L1-RSRP absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±6.5 | ±9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | N/A | -70 |
| 8.5 | 11.5 | ≥-3 | N/A |  | -70 | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

10.1.20Y.1.2	Relative Accuracy

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB from one cell on a frequency in FR2 compared to the largest measured value of L1-RSRP among all SSBs measured from another cell on another frequency in FR2.

The accuracy requirements in Table 10.1.20Y.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.1 for a corresponding Band for each relevant SSB.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.20Y.1.2-1: SSB based L1-RSRP relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 3 |  | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |
| 6.5 | 9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |

End of Change 4

Start of Change 5

A.3.8.2.5	FR1 PRACH configuration 5

FR1 PRACH configuration 5 in this clause provides the typical PRACH configuration for LTM early UL synchronization on candidate cell in FR1.

Table A.3.8.2.5-1: Parameters for FR1 PRACH configuration 5

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| msg1-FrequencyStart | 0 |  |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 |  |
| powerRampingStep | dB2 |  |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ltm-prach-SubcarrierSpacing | Same as UL carrier SCS |  |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

End of Change 5

Start of Change 6

A.3.8.2.6	FR1 PRACH configuration 6

FR1 PRACH configuration 6 in this clause provides the typical PRACH configuration for SSB-based contention free random access in FR1.

Table A.3.8.2.6-1: Parameters for FR1 PRACH configuration 6

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 102 | 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-2 and table 6.3.3.2-3 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n16 | OneFourth: 1 SSB associated with 4 RACH occasions n16: 16 contention based preambles per SSB |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

End of Change 6

Start of Change 7

A.3.8.3.5	FR2 PRACH configuration 5

FR2 PRACH configuration 5 in this clause provides the typical PRACH configuration for LTM early UL synchronization on candidate cell in FR2.

Table A.3.8.3.5-1: Parameters for FR2 PRACH configuration 5

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6] |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| msg1-FrequencyStart | 0 |  |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 |  |
| powerRampingStep | dB2 |  |
| ssb-perRACH-Occasion | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ltm-prach-SubcarrierSpacing | Same as UL carrier SCS |  |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

End of Change 7

Start of Change 8

A.3.8.3.6	FR2 PRACH configuration 6

FR2 PRACH configuration 6 in this clause provides the typical PRACH configuration for SSB-based contention free random access in FR2.

Table A.3.8.3.6-1: Parameters for FR2 PRACH configuration 6

| Field | Value | Comment |
| --- | --- | --- |
| prach-ConfigurationIndex | 190 | Preamble Format C2, with 10ms PRACH periodicity, and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [6]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | No group B. |
| prach-RootSequenceIndex | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n16 | OneFourth: 1 SSB associated with 4 RACH occasions n16: 16 contention based preambles per SSB |
| msg1-FDM | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [2]. |
| ra-ContentionResolutionTimer | sf48 | 48 sub-frames |
| powerRampingStep | dB2 |  |
| preambleReceivedTargetPower | dBm-120 |  |
| preambleTransMax | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | 10 slots |
| zeroCorrelationZoneConfig | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 20 ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

End of Change 8

Start of Change 9

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers

A.3.13A.1	Introduction

The following applies to UE compliant to this version of the specification when undergoing tests with a mix of E-UTRA/NR FR1 and NR FR2 carriers in clauses A.5, A.7 and A.8.

A.3.13A.2	Principle of Testing in EN-DC

For test cases in clause A.5 listed in Table A.3.13A.2-1, the following applies:

-	UE does not have to pass the test case.

Table A.3.13A.2-1: Test cases UE does not have to pass in current version of specification (EN-DC)

| Clause | Test case slogan |
| --- | --- |
| A.5.5.2.7 | E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching |
| A.5.5.2.9 | E-UTRAN – NR FR2 interruptions during measurements on deactivated NR PSCell |
| A.5.5.3.2 | SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle |
| A.5.5.3.5 | SCell Activation and deactivation of SCell in FR2 |
| A.5.5.3.6 | Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2 |
| A.5.5.6.4.2 | E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time |
| A.5.7.1.3 | EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell |

A.3.13A.3	Principle of Testing in SA

For test cases in clause A.7 listed in Table A.3.13A.3-1, the following applies:

-	UE does not have to pass the test case.

Table A.3.13A.3-1: Test cases UE does not have to pass in current version of specification (SA)

| Clause | Test case slogan |
| --- | --- |
| A.7.5.3.2 | SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 |
| A.7.5.3.7 | PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell |
| A.7.5.6.1.2 | NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA |
| A.7.5.6.4.2 | NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time |
| A.7.6.2.5 | SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) |
| A.7.6.2.6 | SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) |
| A.7.6.2.7 | SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) |
| A.7.6.2.8 | SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) |
| A.7.7.1.3 | SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell |

A.3.13A.4	Principle of Testing in E-UTRA

For test cases in clause A.8 listed in Table A.3.13A.4-1, the following applies:

-	UE does not have to pass the test case.

Table A.3.13A.4-1: Test cases UE does not have to pass in current version of specification (E-UTRA)

| Clause | Test case slogan |
| --- | --- |
| A.8.2.2.2 | E-UTRA – NR Early Measurement Reporting for NR in FR2 |
| A.8.4.2.5 | NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used |
| A.8.4.2.6 | NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used |
| A.8.4.2.7 | NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used |
| A.8.4.2.8 | NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used |

A.3.13A.5	Principle of Testing in NR-DC

For test cases in clause A.6 listed in Table A.3.13A.5-1, the following applies:

-	UE does not have to pass the test case.

Table A.3.13A.5-1: Test cases UE does not have to pass in current version of specification (EN-DC)

| Clause | Test case slogan |
| --- | --- |
| A.6.3.X.2 | CHO including target MCG and target SCG in NR-DC (FR1-FR2 NR-DC to FR1-FR2 NR-DC) |
| A.5.3.X.4 | CHO including target MCG and candidate SCG in NR-DC (FR1-FR2 NR-DC to FR1-FR2 NR-DC) |
| A.6.5.X.3 | Intra-frequency CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC |
| A.6.5.X.4 | Inter-frequency CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC |

End of Change 9

Start of Change 10

A.3.16B	LTM Candidate TCI State Configuration

A.3.16B.1	Introduction

This clause provides the configurations for TCI states of LTM candidate cell(s) towards either SSB or TRS. The LTM candidate DLorJoint TCI states defined in this clause are configured in each test when applicable to indicate that certain DL (and UL, if joint DL/UL operation is configured) signals are QCL’ed with the referenceSignal configured in the TCI states. The UL TCI states defined in this clause are configured in each test when applicable to indicate that certain UL signals are QCL’ed with the referenceSignal configured in the TCI states.

A.3.16B.2	LTM candidate DLorJoint TCI states

Table A.3.16B.2-1: LTM candidate DLorJoint TCI States

| Parameter | Candidate DLorJoint TCI.State.0 | Candidate DLorJoint TCI.State.1 | Candidate DLorJoint TCI.State.2 | Candidate DLorJoint TCI.State.3 |
| --- | --- | --- | --- | --- |
| Candidatetci-StateId | Id0 | Id1 | Id2 | Id3 |
| qcl-Type1 | typeA | typeA | typeA | typeA |
| referenceSignal of qcl-Type 1 Note2 | SSB0 | Resource #4 in TRS resource set 1 Note3 | SSB0 | Resource #4 in TRS resource set 1 Note3 |
| qcl-Type2Note1 | typeD | typed | typeD | typeD |
| referenceSignal of qcl-Type2 Note2 | SSB0 | Resource #4 in TRS resource set 1 Note3 | SSB0 | Resource #4 in TRS resource set 1 Note3 |
| pathlossReferenceRS | SSB0 | SSB0 | N/A | N/A |
| additionalPCI | N/A | N/A | N/A | N/A |
| Note 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 Note 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 3: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured for a candidate cell in a test case, it is considered as resource set 1. The TCI state of the TRS is the CandidateDLorJoint TCI.State.2. |  |  |  |  |

A.3.16B.3	LTM candidate UL TCI states

Table A.3.16B.3-1: LTM candidate UL TCI States

| Parameter | Candidate UL TCI.State.0 |
| --- | --- |
| Candidateul-TCIState-Id | Id0 |
| referenceSignal Note1 | SSB0 |
| pathlossReferenceRS | SSB0 |
| additionalPCI | N/A |
| Note 1: referenceSignal configurations towards which the UL TCI states are configured are defined in a test-specific manner. |  |

End of Change 10

Start of Change 11

A.6.3.2.x	LTM PDCCH-order Random Access

A.6.3.2.x.1	PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP

A.6.3.2.x.1.1	Test Purpose and Environment

This test is to verify the requirement for PDCCH-order RACH on neighbour cell in FR1 when RACH BW is within active UL BWP specified in clause 8.1 in 38.213 [3] and UE transmit timing in clause 7.1 for UE supporting [RACH-based early TA acquisition].

A.6.3.2.x.1.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. Test configurations are given in table A.6.3.2.x.1.2-1. Both PDCCH order RACH delay, transmit timing requirement and the interruption requirements are tested by using the parameters in table A.6.3.2.x.1.2-2, and A.6.3.2.x.1.2-3.

This test contains 3 tests (test 1, 2, and 3) and UE may have to pass one of the tests based on the conditions defined in this clause.

In test 1, joint TCI state configuration as defined in Table A.6.3.2.x.1.2-2 is provided.

In test 2, separate TCI state configuration as defined in Table A.6.3.2.x.1.2-2 is provided.

In test 3, no candidate TCI state configurations are configured as in Table A.6.3.2.x.1.2-2.

If a UE supports ltm-MAC-CE-JointTCI-r18, it is only required to pass test 1. If a UE supports ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18, it is only required to pass test 2. If a UE supports neither ltm-MAC-CE-SeparateTCI-r18 nor ltm-MAC-CE-JointTCI-r18, it is only required to pass test 3.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. No gap patterns are configured in the test case.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	For tests 1, 2, 3, the UE has reported L3 measurement results and performed SSB based L1-RSRP measurement on Cell 2.

In test 1, 2 and 3, T1 starts from UE transmitting a valid L1 report on Cell 2.

In test 1 and test 2, after receiving the first L1 report on Cell 2 during T1, the test equipment sends TCI state activation MAC CE to active TCI state of Cell 2 no later than 100 ms after receiving the L1 report.

In test 1, CandidateTCI-State#1 is activated.

In test 2, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are activated.

In test 3, test equipment shall not send TCI state activation MAC CE to active TCI state of Cell 2.

The start of T2 is the instant when PDCCH order to trigger PRACH transmission on Cell 2 is sent to the UE.

Table A.6.3.2.x.1.2-1: PDCCH order RACH on Neighbor cell in FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Candidate cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Candidate cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Candidate: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.2.x.1.2-2: General test parameters for PDCCH order RACH in FR1

| Parameter |  | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 1 |  |  | After transmitting PRACH on Cell 2, UE shall be back to Cell 1. |
| A3-Offset |  | dB | -6 |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |
| Time To Trigger |  | ms | 0 |  |  |  |
| Filter coefficient |  |  | 0 |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  | DRX is not used |
| Time offset between cells |  |  | 2 s |  |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  | NR RF Channel Number 1 |  |  | Same as Cell 1 |
|  | PRACH configuration |  | FR1 PRACH configuration 5 |  |  | RACH bandwidth is within active UL BWP of Cell 1 |
|  | bwp-GenericParameters |  | ULBWP.0.1 |  |  |  |
|  | n-TimingAdvanceOffset | Tc | 25600 |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | #1 CandidateTCI-State |  | DlorJoint TCI.State.0 | DlorJoint TCI.State.2 | N/A | As specified in clause A.3.16B. Configured for early TCI state activation for test 1 and test 2. |
| Ltm-UL-TCI-StatesToAddModList | #1 CandidateTCI-UL-State#0 |  | N/A | UL TCI.State.0 | N/A | As specified in clause A.3.16B. Configured for early TCI state activation for test 2. |
| Ltm-ConfigComplete |  |  | True |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |  |  |
| T2 |  | s | 0.5 |  |  |  |

Table A.6.3.2.x.1.2-3: Cell specific test parameters for PDCCH order RACH test case

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  | N/A |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | N/A |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | N/A |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | N/A |  |  |
| CP length |  |  |  | Normal |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |
|  |  |  | dB | -0.64 | -0.64 |  | -0.64 | -0.64 |
|  |  |  | dB | 8 | 8 |  | 8 | 8 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -90 | -90 |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -87 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -58.7 | -58.7 |  | -58.7 | -58.7 |
|  | Config 3 |  | dBm/ 38.16MHz | -52.6 | -52.6 |  | -52.6 | -52.6 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

A.6.3.2.x.1.3 Test Requirements

The UE shall transmit the PRACH preamble to Cell 2 in the first available PRACH occasion after  NT,2 + 0.5ms + TSSB from the beginning of time period T2. After transmitting PRACH on Cell 2, UE shall retune back to Cell 1.

NOTE:	The PDCCH order RACH delay can be expressed as: NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation, where:

-	NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission and is specified in Table 6.4-1 in 38.214 [26].

-	TBWPswitchDelay= 0, Tswitch= 0, ∆RF/BB preparation= 0

-	∆Delay= 0.5ms

-	TSSB= 0 for UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18, otherwise TSSB is [the time to first SSB after the slot that UE receives PDCCH-order].

During T2, interruption on Cell 1 UL shall not happen outside [the overlapped slot to transmit PRACH] and N symbols before and after the PRACH occasion as defined in clause 8.1 in 38.213 [3], where N=2. During T2, interruption on Cell 1 DL shall not occur outside the overlapped slot to transmit PRACH.

The test equipment will verify that the timing of PRACH transmission on Cell 2 is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB of Cell 2.

a.	The NTA_offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1.

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.3.2.x.2	PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM

A.6.3.2.x.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM. The Te requirements are specified in 7.1. The interruption requirements specified in clause 8.2.2.2.20. This test is for UE supporting PDCCH-ordered RACH to an inter-frequency candidate cell, whose SSB is outside active BWPs of the UE.

A.6.3.2.x.2.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on a different frequency from the PCell. Test configurations are given in table A.6.3.2.x.2.2-1. Both interruption length and Te requirements for the first uplink transmission are tested by using the parameters in table A.6.3.2.x.2.2-2, A.6.3.2.x.2.2-3, and A.6.3.2.y.2-4.

The test consists of 2 tests, and UE is required to pass one among Test 1, Test 2.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

The test consists of two successive time periods, with time durations of T1 and T2, respectively. Measurement gap patterns are configured in the test case and the SSB of Cell2 is outside the active BWP of Cell 1. The PCell is continuously scheduled during the whole test.

Before T1, for Test 1, 2:

-	Cell 1 and Cell 2 on radio channel 1 are powered on.

-	UE establishes a connection with the Cell 1.

-	A measurement object is configured for the frequency of the Cell 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used. UE has reported an L3 measurement result of Cell 2 to Cell 1.

-	UE is provided with LTM-Candidate-r18 for Cell 2

-	Joint TCI state configuration as defined in Table A.6.3.x.5.2-2 for Test 1 is provided.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2, and UE has already reported a valid L1-RSRP result of Cell 2.

During T1, for Test 1:

-	At the start of T1, UE receives candidate cell TCI state activation MAC CE for Cell 2.

-	In Test 1, CandidateTCI-State#1 is activated.

-	T1 ends 100ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2, T1 is skipped.

During T2, for Test 1, 2:

-	At the start of T2, UE receives PDCCH order to trigger PRACH transmission on Cell 2.

-	T2 ends with UE back to Cell 1 after transmitting PRACH to Cell 2.

-	The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during the transmission of PDCCH ordered RACH.

-   The test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB.

a.	The NTA offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1

Table A.6.3.2.x.2.2-1: Inter-frequency PDCCH-ordered RACH from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.2.x.2.2-2: General test parameters for Inter-frequency PDCCH-ordered RACH test from FR1 to FR1 from FR1 to FR1

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |
|  | Neighbouring cell |  | Cell 2 |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 1 |  |  |
| A3-Offset |  | dB | -6 |  |  |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | s | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| Measurement gap periodicity |  | ms | 80 |  |  |
| DRX |  |  | OFF |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 2 s |  | RTD between cells is less than CP |
| reportQuantityRS-Indexes |  |  | Rsrp |  |  |
| maxNrofRS-IndexesToReport |  |  | 1 |  |  |
| includeBeamMeasurements |  |  | True |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  | NR RF Channel Number 2 |  | Cell 2 |
|  | PRACH configuration |  | FR2 PRACH configuration 5 |  |  |
|  | bwp-GenericParameters |  | ULBWP.0.1 |  |  |
|  | n-TimingAdvanceOffset | Tc | 25600 |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DLorJoint TCI.State.0 | N/A | As specified in clause A.3.16B. In test 1, CandidateTCI-State#1 is configured for early TCI state activation. In test 2, CandidateTCI-State#1 is configured for TCI state indication in cell switch command. |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.1 |  |  |
| T2 |  | s | 0.2 |  |  |

Table A.6.3.2.x.2.2-3: Cell specific test parameters for NR FR1-FR1 Inter-frequency PDCCH-ordered RACH test

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1-T2 | T1-T2 |
| NR RF Channel Number |  |  |  | 1 | 2 |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DRx Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  |
| TRS configuration for serving cell |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |
|  |  | Config 3 |  | 30 |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |
|  |  | Config 3 |  | 30 |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 6 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |
|  | Config 3 |  |  | -95 |  |
|  |  |  | dB | 8 | 4 |
|  |  |  | dB | 8 | 4 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -87 | -90 |
|  | Config 3 |  | dBm/SCS | -84 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -57.06 | -61.41 |
|  | Config 3 |  | dBm/ 9.36MHz | -50.96 | -55.31 |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

A.6.3.2.x.2.3 Test Requirements

The UE shall transmit the PRACH preamble to Cell 2 in the first available PRACH occasion after  NT,2 + 0.5ms + TSSB from the beginning of time period T2. After transmitting PRACH on Cell 2, UE shall retune back to Cell 1.

NOTE:	The PDCCH order RACH delay can be expressed as: NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation, where:

-	NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission and is specified in Table 6.4-1 in 38.214 [26].

-	TBWPswitchDelay= 0, Tswitch= 0

-	∆Delay= 0.5ms

-	∆RF/BB preparationand TSSB are defined in 6.2.2C

During T2, interruption on Cell 1 UL shall not happen outside [the overlapped slot to transmit PRACH] and N symbols before and after the PRACH occasion as defined in clause 8.1 in 38.213 [3], where N=2. During T2, interruption on Cell 1 DL shall not occur outside the overlapped slot to transmit PRACH.

The test equipment will verify that the timing of PRACH transmission on Cell 2 is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB of Cell 2.

a.	The NTA_offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1.

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.3.2.x.3	PDCCH-order RACH on neighbor cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP

A.6.3.2.x.3.1	Test Purpose and Environment

This test is to verify the requirement for PDCCH-order RACH on neighbour cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP specified in clause 8.1 in 38.213 [3] and UE transmit timing in clause 7.1 for UE supporting [RACH-based early TA acquisition].

A.6.3.2.x.3.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. Test configurations are given in table A.6.3.2.x.3.2-1. Both PDCCH order RACH delay, transmit timing requirement and the interruption requirements are tested by using the parameters in table A.6.3.2.x.3.2-2, and A.6.3.2.x.3.2-3.

This test contains 3 tests (test 1, 2 and 3) and UE may have to pass one of the tests based on the conditions defined in this clause.

In test 1, joint TCI state configuration as defined in Table A.6.3.2.x.3.2-2 is provided.

In test 2, separate TCI state configuration as defined in Table A.6.3.2.x.3.2-2 is provided.

In test 3, no candidate TCI state configurations are configured as in Table A.6.3.2.x.3.2-2.

If a UE supports ltm-MAC-CE-JointTCI-r18, it is only required to pass test 1. If a UE supports ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18, it is only required to pass test 2. If a UE supports neither ltm-MAC-CE-SeparateTCI-r18 nor ltm-MAC-CE-JointTCI-r18, it is only required to pass test 3. The test consists of two successive time periods, with time durations of T1 and T2 respectively. No gap patterns are configured in the test case.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

In test 1, 2 and 3, T1 starts from UE transmitting a L3 report on Cell 2. In test 1 and test 2, after receiving the L3 report on Cell 2 during T1, the test equipment sends TCI state activation MAC CE to active TCI state of Cell 2 no later than [100] ms after receiving the L3 report.

In test 1, CandidateTCI-State#1 is activated.

In test 2, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are activated.

In test 3, test equipment shall not send TCI state activation MAC CE to active TCI state of Cell 2.

The start of T2 is the instant when PDCCH order to trigger PRACH transmission on Cell 2 is sent to the UE.

Table A.6.3.2.x.3.2-1: PDCCH order RACH on Neighbor cell in FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Candidate cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Candidate cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Candidate: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.2.x.3.2-2: General test parameters for PDCCH order RACH in FR1

| Parameter |  | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 1 |  |  | After transmitting PRACH on Cell 2, UE shall be back to Cell 1. |
| A3-Offset |  | dB | -6 |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |
| Time To Trigger |  | ms | 0 |  |  |  |
| Filter coefficient |  |  | 0 |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  | DRX is not used |
| Time offset between cells |  |  | 2 s |  |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  | NR RF Channel Number 1 |  |  | Same as Cell 1 |
|  | PRACH configuration |  | FR1 PRACH configuration 5 |  |  | RACH bandwidth is within active UL BWP of Cell 1 |
|  | bwp-GenericParameters |  | ULBWP.0.1 |  |  |  |
|  | n-TimingAdvanceOffset | Tc | 25600 |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | #1 CandidateTCI-State |  | DlorJoint TCI.State.0 | DlorJoint TCI.State.2 | N/A | As specified in clause A.3.16B. Configured for early TCI state activation for test 1 and test 2. |
| Ltm-UL-TCI-StatesToAddModList | #1 CandidateTCI-UL-State#0 |  | N/A | UL TCI.State.0 | N/A | As specified in clause A.3.16B. Configured for early TCI state activation for test 2. |
| Ltm-ConfigComplete |  |  | True |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |  |  |
| T2 |  | s | 0.5 |  |  |  |

Table A.6.3.2.x.3.2-3: Cell specific test parameters for PDCCH order RACH test case

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  | N/A |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | N/A |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | N/A |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | N/A |  |  |
| CP length |  |  |  | Normal |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |
|  |  |  | dB | -0.64 | -0.64 |  | -0.64 | -0.64 |
|  |  |  | dB | -1 | -1 |  | -1 | -1 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -90 | -90 |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -87 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -58.7 | -58.7 |  | -58.7 | -58.7 |
|  | Config 3 |  | dBm/ 38.16MHz | -52.6 | -52.6 |  | -52.6 | -52.6 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

A.6.3.2.x.3.3 Test Requirements

The UE shall transmit the PRACH preamble to Cell 2 in the first available PRACH occasion after  NT,2 + 0.5ms + TSSB from the beginning of time period T2. After transmitting PRACH on Cell 2, UE shall retune back to Cell 1.

NOTE:	The PDCCH order RACH delay can be expressed as: NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation, where:

-	NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission and is specified in Table 6.4-1 in 38.214 [26].

-	TBWPswitchDelay= 0, Tswitch= 0, ∆RF/BB preparation= 0

-	∆Delay= 0.5ms

-	TSSB is the time to first SSB occasion, after [1slot from] the end of the slot of the PDCCH plus 2ms (SSB processing time) .

During T2, interruption on Cell 1 UL shall not happen outside the overlapped slot to transmit PRACH and N symbols before and after the PRACH occasion as defined in clause 8.1 in 38.213 [3], where N=2. During T2, interruption on Cell 1 DL shall not occur outside the overlapped slot to transmit PRACH.

The test equipment will verify that the timing of PRACH transmission on Cell 2 is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB of Cell 2.

a.	The NTA_offset value (in Tc units) is 25600

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1.

The rate of correct events observed during repeated tests shall be at least 90%.

End of Change 11

Start of Change 12

A.6.3.3.x	NR conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC

A.6.3.3.x.1	Test Purpose and Environment

The purpose of this test is to verify that UE can make correct NR-RC to NR-DC conditional handover including target MCG in FR1 and target SCG in FR1 as specified in clauses 6.1.6.1.

The supported test configurations are given in Table A.6.3.3.x.1-1. The test scenario comprises four NR cells, source PCell (Cell 1) and source PSCell (Cell 2), target PCell (Cell 3), and target PSCell (Cell 4). Cell 1 and Cell 3 are on radio channel 1 in FR1. Cell 2 and Cell 4 are on radio channel 2 in FR1.

Test parameters are given in Tables A.6.3.3.x.1-2, A.6.3.3.x.1-3, A.6.3.3.x.1-4 and A.6.3.3.x.1-5 below.

The test consists of two successive time periods, with time durations of T1, T2 respectively.

At the start of T1, the UE shall be connected to Cell 1 on radio channel 1 and Cell 2 on radio channel 2. UE is not aware of Cell 3 and Cell 4.  NR shall configure a condition implying handover to Cell 3 and Cell 4 during T1, at a time earlier than TRRC before the beginning of T2.

At the start of T2, Cell3 and Cell 4 become detectable. The condition for conditional PCell handover is met during T2.

Table A.6.3.3.x.1-1: Supported test configurations for NR conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Config | Description |
| --- | --- |
| 1 | Source PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Source PSCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target PSCell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Source PSCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target PSCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source PCell: FR1 NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target PCell: FR1 NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Source PSCell: FR1 NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target PSCell: FR1 NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.3.x.1-2: General test parameters for PCell handover at conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 3 |  |
| Final condition | Active cell |  | Cell 3 |  |
| A3-Offset in handover condition |  | dB | -4 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Cell 1 and Cell 3 are synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.6.3.3.x.1-3: Cell specific test parameters for PCell handover at conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter |  |  | Unit | Cell 1 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  | -98 |  |
|  | Config 3 |  |  | -95 |  | -95 |  |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -Infinity | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -Infinity | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -63.94 | -57.75 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.6.3.3.x.1-4: General test parameters for PSCell change at conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 2 |  |
|  | Neighbouring cell |  | Cell 4 |  |
| Final condition | Active cell |  | Cell 4 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Cell 2 and Cell 4 are Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

Table A.6.3.3.x.1-5: Cell specific test parameters for PSCell change at conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter |  |  | Unit | Cell 2 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 2 |  | 2 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  | -98 |  |
|  | Config 3 |  |  | -95 |  | -95 |  |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -Infinity | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -Infinity | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -63.94 | -57.75 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

A.6.3.3.x.2	Test Requirements

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 3 less than Tmeasure + Tinterrupt + TCHO_execution =920 +57 +10=987 ms from the start of T2, and the interruption during T2 shall not exceeed TIU + Tprocessing  + T∆ + Tmargin=10+25+20+2=57ms.

The UE shall start to transmit the PRACH to Cell 4 less than Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms =920+10+25+0+ (3*20)+ 20+2=1037ms from the start of T2, excluding any transmissions which do not occur due to measurement gaps.

The rate of correct conditional handovers observed during repeated tests shall be at least 90%.

Note1:	The PCell conditional handover delay can be expressed as specified in clause 6.1.6.1:

DCHOwithPSCell_PCell = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

The interruption time is the time between when the UE starts to execute the conditional handover to the target cell and the time the UE starts transmission of the new PRACH as specified in clause 6.1.6.1.1

Tinterrupt = TIU + Tprocessing  + T∆ + Tmargin ms

The PSCell conditional handover delay can be expressed as specified in clause 6.1.6.1.2:

DCHOwithPSCell_PSCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_DU + 2 ms

End of Change 12

Start of Change 13

A.6.3.3.X1	NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC

A.6.3.3.X1.1	Test Purpose and Environment

This test is to verify the requirement for conditional handover including target MCG and candidate SCG for CPC in FR1 NR-DC specified in clause 6.1.7.1. This test verifies the requirements for PCell conditional handover delay in section 6.1.7.1.1 and PSCell conditional change delay in section 6.1.7.1.2 for FR1-FR1 NR-DC to FR1-FR1 NR-DC CHO with CPC.

A.6.3.3.X1.2	Test Parameters

Supported test configurations are shown in table A.6.3.3.X1.2-1. Conditional handover delay and interruption length are tested by using the parameters in table A.6.3.3.X1.2-2 and A.6.3.3.X1.2-3. Conditional PSCell change delay and interruption length are tested by using the parameters in A.6.3.3.X1.2-2 and A.6.3.3.X1.2-4.

The test consists of three successive time periods, with time durations of T1, T2, and T3, respectively.

At the start of time duration T1, the UE is connected to cell 1 (source PCell) and cell 2 (source PSCell). The UE may not have any timing information of cell 3 (target PCell) and cell 4 (target PSCell).

TE shall configure a condition implying conditional handover to cell 3 with a condition implying conditional PSCell change to cell 4 during T1, at a time earlier than TRRC before the beginning of T2.

At the start of T2, Cell 3 becomes detectable. At the start of T3, Cell 4 becomes detectable. UE meets the handover condition and meets the PSCell change condition during T3.

Table A.6.3.3.X1.2-1: Intra-frequency conditional handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cells (PCell and PSCell): NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cells (PCell and PSCell): NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.3.X1.2-2: General test parameters Intra-frequency conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cells |  | PCell: Cell 1 PSCell: Cell 2 |  |
|  | Neighbouring cells |  | Cell 3, Cell 4 |  |
| Final condition | Active cells |  | PCell: Cell 3 PSCell: Cell 4 |  |
| A3-Offset |  | dBm | 0 | A3-offset is applied for conditions for both CHO and CPC |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | S | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  | FR1 PRACH configuration 1 | As specified in table Table 6.3.3.2-3 in TS 38.211 [6] |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | S | 5 |  |
| T2 |  | S | 1 |  |
| T3 |  | S | 2 |  |

Table A.6.3.3.X1.2-3: Cell specific test parameters for FR1-FR1 CHO (Cell 1 and Cell 3)

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |
|  |  |  | dB | 4 | 4 | 4 | -Infinity | 5 | 5 |
|  |  |  | dB | 4 | 4 | 4 | -Infinity | 5 | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -94 | -Infinity | -93 | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -91 | -Infinity | -90 | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -64.59 | -70.05 | -63.85 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -58.49 | -63.94 | -57.75 | -57.75 |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

Table A.6.3.3.X1.2-4: Cell specific test parameters for FR1-FR1 CPC (Cell 2 and Cell 4)

| Parameter |  |  | Unit | Cell 2 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  |  | 2 |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |
|  |  |  | dB | 4 | 4 | 4 | -Infinity | -Infinity | 5 |
|  |  |  | dB | 4 | 4 | 4 | -Infinity | -Infinity | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -94 | -Infinity | -Infinity | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -91 | -Infinity | -Infinity | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -64.59 | -70.05 | -70.05 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -58.49 | -63.94 | -63.94 | -57.75 |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

A.6.3.3.X1.3 Test Requirements

TRRC + TEvent_DU occurs during T1 and T2, as the conditional handover condition for cell 3 becomes satisfied at the start of T2, and the conditional PSCell change condition for cell 4 becomes satisfied at the start of T3. The test shall verify that there are no interruptions during T1 and T2. The UE shall not send PRACH to cell 3 before the end of T2.

The UE shall start to transmit PRACH to Cell 3 less than max (Tmeasure_PCell, Tmeasure_PSCell) + TUE_preparation + Tprocessing + T∆_PCell + TPCell_DU + 2 ms = (800 + 10 + 25 + 20 + 20 + 2) ms = 877 ms from the start of T3 and the interruption during T3 shall not exceeed Tprocessing + T∆_PCell + TPCell_DU + 2 ms = (25 + 20 + 20 + 2) ms = 67 ms

The UE shall start to transmit PRACH to Cell 4 less than max (Tmeasure_PCell, Tmeasure_PSCell) + TUE_preparation + Tprocessing + T∆_PSCell + TPSCell_DU + 2 ms = (800 + 10 + 25 + 20 + 20 + 2) ms = 877 ms from the start of T3. The interruption during T3 shall not exceed Tprocessing + T∆_PSCell + TPSCell_DU + 2 ms = 25 ms + 20 + 20 ms + 2 ms = 67 ms.

End of Change 13

Start of Change 14

A.6.3.3.X2	NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC with complementary conditional handover configuration

A.6.3.3.X2.1	Test Purpose and Environment

This test verifies the requirement for conditional handover when the UE is configured with target MCG and candidate SCG for CPC in FR1 NR-DC and additionally configured with a complementary CHO configuration in FR1. The test verifies that the UE makes the correct decision to proceed with CHO without CPC in case CHO condition is met without CPC condition being met, meanwhile verifying the conditional handover delay requirement in section 6.1.4.2.

For UE which can pass test case defined in clause A.6.3.3.X2, test case defined in clause A.6.3.3.1 can be skipped and corresponding test requirements are deemed to be fulfilled.

A.6.3.3.X2.2	Test Parameters

Supported test configurations are shown in table A.6.3.3.X2.2-1. Both conditional handover delay and interruption length are tested by using the parameters in table A.6.3.3.X2.2-2, and A.6.3.3.X2.2-3.

The test consists of two successive time periods, with time durations of T1 and T2 respectively.

At the start of time duration T1, the UE is connected to cell 1 (source PCell) and cell 2 (source PSCell). The UE may not have any timing information of cell 3 (target PCell) and cell 4 (target PSCell).

TE shall configure a condition implying conditional handover to cell 3 with a condition implying conditional PSCell change to cell 4 during T1, at a time earlier than TRRC before the beginning of T2. Additionally, the TE shall configure a complementary condition only implying conditional handover to cell 3.

At the start of T2, cell 3 becomes detectable. UE meets the handover condition during T2.

Table A.6.3.3.X2.2-1: Intra-frequency conditional handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cells (PCell and PSCell): NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cells (PCell and PSCell): NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cells (PCell and PSCell): NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.3.X2.2-2: General test parameters Intra-frequency conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cells |  | PCell: Cell 1 PSCell: Cell 2 |  |
|  | Neighbouring cells |  | Cell 3, Cell 4 |  |
| Final condition | Active cells |  | PCell: Cell 3 PSCell: Cell 4 |  |
| A3-Offset in condition for PCell |  | dB | -10 |  |
| A4-Offset in condition for PSCell |  | dB | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  | FR1 PRACH configuration 1 | As specified in table Table 6.3.3.2-3 in TS 38.211 [6] |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.6.3.3.X2.2-3: Cell specific test parameters for FR1-FR1 CHO (Cell 1 and Cell 3)

| Parameter |  |  | Unit | Cell 1 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -Infinity | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -Infinity | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -63.94 | -57.75 |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.6.3.3.X2.2-4: Cell specific test parameters for FR1-FR1 CPC (Cell 2 and Cell 4)

| Parameter |  |  | Unit | Cell 2 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 2 |  | 2 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -Infinity | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -Infinity | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -63.94 | -57.75 |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

A.6.3.3.X2.3 Test Requirements

TRRC + TEvent_DU occurs during T1 as the conditional handover condition for cell becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 3 less than Tmeasure + Tinterrupt + TCHO_execution = 800 +62 +10=872ms from the start of T2 and the interruption during T2 shall not exceeed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin =40+20+2 = 62ms.

The UE shall not send PRACH to Cell 4.

End of Change 14

Start of Change 15

A.6.3.x	LTM PCell Switch

A.6.3.x.1	RACH-based Intra-frequency PCell switch from FR1 to FR1

A.6.3.x.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 RACH-based intra frequency PCell switch specified in clause 6.3.1 for both with and without early TCI state activation.

A.6.3.x.1.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. Test configurations are given in table A.6.3.x.1.2-1. Both cell switch delay and interruption length are tested by using the parameters in table A.6.3.x.1.2-2, and A.6.3.x.1.2-3.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

The test consists of four successive time periods, with time durations of T1 to T4 respectively. No gap patterns are configured in the test case.

During T1, for Test 1A, 1B, 2A and 2B:

-	A measurement object is configured for the frequency of the Cell 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting an L3 measurement result of Cell 2 to Cell 1.

During T2, for Test 1A, 1B, 2A and 2B:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 2

-	In Test 1A and Test 2A, joint TCI state configurations as defined in Table A.6.3.x.1.2-2 are provided.

-	In Test 1B and Test 2B, separate TCI state configurations as defined in Table A.6.3.x.1.2-2 are provided.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	T2 ends with UE reporting a valid L1-RSRP result of Cell 2.

During T3, for Test 1A and 1B:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 2.

-	In Test 1A, CandidateTCI-State#1 is activated.

-	In Test 1B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 is activated.

-	T3 ends 50 ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2A and 2B, T3 is skipped.

During T4, for Test 1A, 1B, 2A and 2B:

-	The start of T4 is the last TTI containing LTM cell switch command MAC CE is sent by Cell 1 to the UE.

-	In the cell switch command, Cell 2 is the target cell. Contention-Free Random-Access Resources are indicated and the field of Timing Advance Command is set to FFF.

-	In test 1A, CandidateTCI-State#2 is indicated.

-	In test 1B, CandidateTCI-State#2 and CandidateTCI-UL-State#1 are indicated.

-	In test 2A, CandidateTCI-State#1 is indicated.

-	In test 2B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are indicated.

-	T4 ends upon the reception of PRACH at Cell 2.

Table A.6.3.x.1.2-1: Intra-frequency cell switch from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.x.1.2-2: General test parameters for Intra-frequency cell switch from FR1 to FR1

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 2 |  |  |  |  |
| A3-Offset |  | dB | -6 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 2 s |  |  |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DLorJoint TCI.State.0 | DLorJoint TCI.State.2 | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | #2 CandidateTCI-State#2 |  | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.1 |  |  |  |  |
| T4 |  | s | 0.1 |  |  |  |  |

Table A.6.3.x.1.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency cell switch test case

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1~T4 |  | T1~T4 |  |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |
| CP length |  |  |  | Normal |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 6 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |
|  |  |  | dB | -0.64 | -0.64 | -0.64 | -0.64 |
|  |  |  | dB | 8 | 8 | 8 | 8 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 | -90 | -90 |
|  | Config 3 |  | dBm/SCS | -87 | -87 | -87 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -58.7 | -58.7 | -58.7 | -58.7 |
|  | Config 3 |  | dBm/ 38.16MHz | -52.6 | -52.6 | -52.6 | -52.6 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

A.6.3.x.1.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 in no later than DLTM from the beginning of time period T4. The rate of correct cell switches observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-interrupt), where:

Tcmd = THARQ + 3ms and is specified in clause 6.3.1.2. TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms and is specified in clause 6.3.1.2.1

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B

-	TLTM-IU_=20ms

-	TLTM-RRC-processing =10 ms if UE does not support [Early processing of an LTM candidate cell RRC configuration], otherwise TLTM-RRC-processing =0ms

-	TLTM-processing =10 ms if the UE supports [faster LTM processing] capability and UE reports 10 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing =15 ms if the UE supports [faster LTM processing] capability and UE reports 15 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing =20 ms if the UE does not support [faster LTM processing] capability.

A.6.3.x.2	RACH based Inter-frequency LTM PCell switch from FR1 to FR1

A.6.3.x.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 inter-frequency LTM RACH based cell switch delay requirements specified in clause 6.3.1 for both with and without early TCI state activation.

A.6.3.x.2.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on a different frequency from the PCell. Test configurations are given in table A.6.3.x.2.2-1. Both cell switch delay and interruption length are tested by using the parameters in table A.6.3.x.2.2-2 and A.6.3.x.2.2-3.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

The test consists of four successive time periods, with time durations of T1, T2, T3 and T4, respectively.

During T1, for Test 1A, 1B,2A and 2B:

-	A measurement object is configured for the frequency of the Cell 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting an L3 measurement result of Cell 2 to Cell 1.

During T2, for Test 1A, 1B, 2A and 2B:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 2

-	In Test 1A and Test 2A, joint TCI state configurations as defined in Table A.6.3.x.2.2-2 are provided.

-	In Test 1B and Test 2B, separate TCI state configuration as defined in Table A.6.3.x.2.2-2 for are provided.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	T2 ends with UE reporting a valid L1-RSRP result of Cell 2.

During T3, for Test 1A and 1B:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 2.

-	In Test 1A, CandidateTCI-State#1 is activated.

-	In Test 1B, CandidateTCI-State#1 is activated.

-	T3 ends 100 ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2A and 2B, T3 is skipped.

During T4, for Test 1A, 1B and 2A and 2B:

-	The start of T4 is the instant when the last TTI containing LTM cell switch command MAC CE is sent by Cell 2 to the UE.

-	In the cell switch command, Cell 2 is the target cell for PCell switch. Contention-Free Random-Access Resources are indicated and the field of Timing Advance Command is set to FFF.

-	In test 1A, CandidateTCI-State#2 is indicated.

-	In test 1B, CandidateTCI-State#2 is indicated.

-	In test 2A, CandidateTCI-State#1 is indicated.

-	In test 2B, CandidateTCI-State#1 is indicated.

-	T4 ends upon the reception of PRACH at Cell 2.

Editors’ Note: FFS whether UL TCI State is configured in the FR1 test case.

Table A.6.3.x.2.2-1: Inter-frequency RACH based cell switch from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.x.2.2-2: General test parameters Inter-frequency RACH based cell switch from FR1 to FR1

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 2 |  |  |  |  |
| A3-Offset |  | dB | -6 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| maxNrofRS-IndexesToReport |  |  | 1 |  |  |  |  |
| includeBeamMeasurements |  |  | True |  |  |  |  |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Measurement gap pattern ID |  |  | gp0 |  |  |  | As specified in Table 9.1.2-1 |
| Measurement gap offset |  |  | 39 |  |  |  |  |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 2 s |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DLorJoint TCI.State.0 | DLorJoint TCI.State.2 | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | N/A | N/A |  |
| [ltm-UL-TCI-StatesToAddModList] | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.2 |  |  |  |  |
| T4 |  | s | 0.1 |  |  |  |  |

Table A.6.3.x.2.2-3: Cell specific test parameters for NR FR1-FR1 Inter frequency RACH-based cell switch test case

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1-T4 | T1-T4 |
| NR RF Channel Number |  |  |  | 1 | 2 |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DRx Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| TRS configuration for serving cell |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |
|  |  | Config 3 |  | 30 |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |
|  |  | Config 3 |  | 30 |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 6 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |
|  | Config 3 |  |  | -95 |  |
|  |  |  | dB | 8 | 8 |
|  |  |  | dB | 8 | 8 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |
|  | Config 3 |  | dBm/SCS | -87 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -61.41 |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -55.31 |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

A.6.3.x.2.3 Test Requirements

The UE shall start to transmit PRACH to Cell 2 in no later than DLTM from the beginning of time period T4.

The rate of correct cell switches observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-interrupt), where:

Tcmd = THARQ + 3ms and is specified in clause 6.3.1.2, TLTM-interrupt is defined in clause 6.3.1.3 as TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU,

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B,

-	TLTM-IU_= 20 ms.

-	TLTM-RRC-processing =10ms if UE does not support [Early processing of an LTM candidate cell RRC configuration], otherwise TLTM-RRC-processing = 0 ms

-	TLTM-processing = 10 ms if the UE supports [faster LTM processing] capability and UE reports 10 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing = 15 ms if the UE supports [faster LTM processing] capability and UE reports 15 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing = 20 ms if the UE does not support [faster LTM processing] capability.

A.6.3.X.3	RACH-less Intra-frequency PCell switch from FR1 to FR1

A.6.3.X.3.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 RACH-less intra frequency PCell switch specified in clause 6.3.1 for both with and without early TCI state activation.

A.6.3.X.3.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. Supported test configurations are shown in table A.6.3.X.3.2-1. Both cell switch delay and interruption length are tested by using the parameters in table A.6.3.X.3.2-2, and A.6.3.X.3.2-3.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18

The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5, respectively. No gap patterns are configured in the test case.

During T1, for Test 1A, 1B, 2A and 2B:

-	A measurement object is configured for the frequency of the Cell 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting an L3 measurement result of Cell 2 to Cell 1.

During T2, for Test 1A, 1B, 2A and 2B:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 2

-	Joint TCI state configuration as defined in Table A.6.3.x.3.2-2 for Test 1A and Test 2A are provided.

-	Separate TCI state configuration as defined in Table A.6.3.x.3.2-2 for Test 1B and Test 2B are provided.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	T2 ends with UE reporting a valid L1-RSRP result of Cell 2.

During T3, for Test 1A and 1B:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 2.

-	In Test 1A, CandidateTCI-State#1 is activated.

-	In Test 1B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 is activated.

-	T3 ends 50ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2A and 2B, T3 is skipped.

During T4, for Test 1A, 1B, 2A and 2B:

-	At the start of T4, UE receives PDCCH order to trigger PRACH transmission on Cell 2.

-	T4 ends 5ms after the UE transmits the PRACH to Cell 2.

-	For UE incapable of rach-EarlyTA-Measurement-r18, T4 is skipped.

During T5, for Test 1A, 1B, 2A and 2B:

-	The start of T5 is the last TTI containing LTM cell switch command MAC CE is sent by Cell 1 to the UE.

-	In the cell switch command, Cell 2 is the target cell and the field of Timing Advance Command is set to 0.

-	In test 1A, CandidateTCI-State#2 is indicated.

-	In test 1B, CandidateTCI-State#2 and CandidateTCI-UL-State#1 are indicated.

-	In test 2A, CandidateTCI-State#1 is indicated.

-	In test 2B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are indicated.

-	Cell 2 continuously schedules PUSCH for the UE.

-	T5 ends either at the UL slot of PUSCH scheduled by Cell 2 at the first DL slot not earlier than (Tcmd + TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc) after the beginning of T5 or upon the reception of PUSCH at Cell 2, whichever is earlier.

-	The values of Tcmd, TLTM-RRC-processing TLTM-processing,Tfirst-RS and TRS-proc are specified in A.6.3.x.3.3.

Table A.6.3.X.3.2-1: Intra-frequency cell switch from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.X.3.2-2: General test parameters Intra-frequency cell switch from FR1 to FR1

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 2 |  |  |  |  |
| A3-Offset |  | dB | -6 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 2 s |  |  |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  | NR RF Channel Number 1 |  |  |  | Same as Cell 1 |
|  | PRACH configuration |  | FR1 PRACH configuration 5 |  |  |  | RACH bandwidth is within active UL BWP of Cell 1 |
|  | bwp-GenericParameters |  | ULBWP.0.1 |  |  |  |  |
|  | n-TimingAdvanceOffset | Tc | 25600 |  |  |  |  |
| ltm-CSI-SSB-ResourceList |  |  | SSB 0 of Cell 2 |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DLorJoint TCI.State.0 | DLorJoint TCI.State.2 | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.1 |  |  |  |  |
| T4 |  | s | 0.2 |  |  |  |  |
| T5 |  | s | 0.1 |  |  |  |  |

Table A.6.3.X.3.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency cell switch test case

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1~T5 | T1~T5 |
| NR RF Channel Number |  |  |  | 1 | 1 |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DRx Cycle |  |  | Ms | Not Applicable |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |
|  |  | Config 3 |  | 30 |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |
|  |  | Config 3 |  | 30 |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 6 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |
|  | Config 3 |  |  | -95 |  |
|  |  |  | dB | [-0.64] | [-0.64] |
|  |  |  | dB | [8] | [8] |
| SSB_RP | Config 1,2 |  | dBm/SCS | [-90] | [-90] |
|  | Config 3 |  | dBm/SCS | [-87] | [-87] |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | [-58.7] | [-58.7] |
|  | Config 3 |  | dBm/ 38.16MHz | [-52.6] | [-52.6] |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

A.6.3.X.3.3 Test Requirements

The UE shall start to transmit PUSCH to Cell 2 in no later than DLTM from the beginning of time period T5.

The rate of correct cell switches observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-interrupt), where:

Tcmd = THARQ + 3ms and is specified in clause 6.3.1.2.

TLTM-interrupt is defined in clause 6.3.1.3 as TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. Tfirst-RS + TRS-proc=0 for Test 1A and 1B, Tfirst-RS + TRS-proc=22ms for Test 2A and 2B, and TLTM-IU_is the uncertainty on transmitting the first uplink transmission on Cell 2.

-	TLTM-RRC-processing = 10 ms if UE does not support [Early processing of an LTM candidate cell RRC configuration], otherwise TLTM-RRC-processing =0ms

-	TLTM-processing = 10 ms if the UE supports [faster LTM processing] capability and UE reports 10 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing = 15 ms if the UE supports [faster LTM processing] capability and UE reports 15 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing = 20 ms if the UE does not support [faster LTM processing] capability.

A.6.3.X.4	RACH-less Intra-frequency PCell switch from FR1 to FR1 with L1-RSRP measurement

A.6.3.X.4.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 RACH-less intra frequency PCell switch specified in clause 6.3.1 without L1-RSRP measurement and measurement reporting.

A.6.3.X.4.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. Supported test configurations are shown in table A.6.3.X.4.2-1. Both cell switch delay and interruption length are tested by using the parameters in table A.6.3.X.4.2-2, and A.6.3.X.4.2-3.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5, respectively. No gap patterns are configured in the test case.

During T1, for Test 1A, 1B, 2A and 2B

-	A measurement object is configured for the frequency of the Cell 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting an L3 measurement result of Cell 2 to Cell 1.

During T2, for Test 1A, 1B, 2A and 2B:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 2

-	Joint TCI state configuration as defined in Table A.6.3.x.4.2-2 for Test 1A and Test 2A are provided.

-	Separate TCI state configuration as defined in Table A.6.3.x.4.2-2 for Test 1B and Test 2B are provided.

During T3, for Test 1A and 1B:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 2.

-	In Test 1A, CandidateTCI-State#1 is activated.

-	In Test 1B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 is activated.

-	T3 ends 50ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2A and 2B, T3 is skipped.

During T4, for Test 1A, 1B, 2A and 2B:

-	At the start of T4, UE receives PDCCH order to trigger PRACH transmission on Cell 2.

-	T4 ends 5ms after the UE transmits the PRACH to Cell 2.

-	For UE incapable of rach-EarlyTA-Measurement-r18, T4 is skipped.

During T5, for Test 1A, 1B, 2A and 2B:

-	The start of T5 is the last TTI containing LTM cell switch command MAC CE is sent by Cell 1 to the UE.

-	In the cell switch command, Cell 2 is the target cell and the field of Timing Advance Command is set to 0.

-	In test 1A, CandidateTCI-State#2 is indicated.

-	In test 1B, CandidateTCI-State#2 and CandidateTCI-UL-State#1 are indicated.

-	In test 2A, CandidateTCI-State#1 is indicated.

-	In test 2B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are indicated.

-	Cell 2 continuously schedules PUSCH for the UE.

-	T5 ends either at the UL slot of PUSCH scheduled by Cell 2 at the first DL slot not earlier than (Tcmd + TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc) after the beginning of T5 or upon the reception of PUSCH at Cell 2, whichever is earlier.

-	The values of Tcmd, TLTM-RRC-processing TLTM-processing,Tfirst-RS and TRS-proc are specified in A.6.3.x.3.3.

Table A.6.3.X.3.2-1: Intra-frequency cell switch from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.X.3.2-2: General test parameters Intra-frequency cell switch from FR1 to FR1

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 2 |  |  |  |  |
| A3-Offset |  | dB | -6 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 2 s |  |  |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  | NR RF Channel Number 1 |  |  |  | Same as Cell 1 |
|  | PRACH configuration |  | FR1 PRACH configuration 5 |  |  |  | RACH bandwidth is within active UL BWP of Cell 1 |
|  | bwp-GenericParameters |  | ULBWP.0.1 |  |  |  |  |
|  | n-TimingAdvanceOffset | Tc | 25600 |  |  |  |  |
| ltm-CSI-SSB-ResourceList |  |  | SSB 0 of Cell 2 |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DLorJoint TCI.State.0 | DLorJoint TCI.State.2 | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.1 |  |  |  |  |
| T4 |  | s | 0.2 |  |  |  |  |
| T5 |  | s | 0.1 |  |  |  |  |

Table A.6.3.X.3.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency cell switch test case

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1~T5 | T1~T5 |
| NR RF Channel Number |  |  |  | 1 | 1 |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DRx Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |
|  |  | Config 3 |  | 30 |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |
|  |  | Config 3 |  | 30 |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 6 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |
|  | Config 3 |  |  | -95 |  |
|  |  |  | dB | -0.64 | -0.64 |
|  |  |  | dB | 8 | 8 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |
|  | Config 3 |  | dBm/SCS | -87 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -58.7 | -58.7 |
|  | Config 3 |  | dBm/ 38.16MHz | -52.6 | -52.6 |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

A.6.3.X.4.3 Test Requirements

The UE shall start to transmit PUSCH to Cell 2 in no later than DLTM from the beginning of time period T5.

The rate of correct cell switches observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-interrupt), where:

Tcmd = THARQ + 3ms and is specified in clause 6.3.1.2.

TLTM-interrupt is defined in clause 6.3.1.3 as TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. Tfirst-RS + TRS-proc=0 for Test 1A and 1B, Tfirst-RS + TRS-proc=22ms for Test 2A and 2B, and TLTM-IU_is the uncertainty on transmitting the first uplink transmission on Cell 2.

-	TLTM-RRC-processing = 10 ms if UE does not support [Early processing of an LTM candidate cell RRC configuration], otherwise TLTM-RRC-processing =0ms

-	TLTM-processing = 10 ms if the UE supports [faster LTM processing] capability and UE reports 10 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing = 15 ms if the UE supports [faster LTM processing] capability and UE reports 15 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing = 20 ms if the UE does not support [faster LTM processing] capability.

End of Change 15

Start of Change 16

A.6.3.y	LTM PSCell Switch

A.6.3.y.1 RACH-based intra-frequency LTM PSCell switch from FR1 to FR1

A.6.3.y.1.1	Test Purpose and Environment

This test is to verify the intra-frequency RACH based LTM PSCell switch requirements from NR FR1 to NR FR1 specified in clause 8.20 for both with and without early TCI state activation.

A.6.3.y.1.2	Test Parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on the same frequency as the PSCell. The test configurations of PCell and PSCell are given in Table A.6.3.y.1.2-1 and Table A.6.3.y.1.2-1A. Both cell switch delay and interruption length are tested by using the parameters in Table A.6.3.y.1.2-2 and Table A.6.3.y.1.2-3.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

The test consists of four successive time periods, with time durations of T1, T2, T3 and T4, respectively.

During T1, for Test 1A, 1B, 2A and 2B:

-	A measurement object is configured for the frequency of the Cell 3, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting an L3 measurement result of Cell 3 to Cell 2.

During T2, for Test 1A, 1B, 2A and 2B:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 3

-	Joint TCI state configuration as defined in Table A.6.3.y.1.2-2 for Test 1A and Test 2A are provided.

-	Separate TCI state configuration as defined in Table A.6.3.y.1.2-2 for Test 1B and Test 2B are provided.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports for candidate cell (Cell 3) in PUCCH format 2.

-	T2 ends with UE reporting a valid L1-RSRP result of Cell 3.

During T3, for Test 1A and 1B:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 3.

-	In Test 1A, CandidateTCI-State#1 is activated.

-	In Test 1B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 is activated.

-	T3 ends 50 ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2A and 2B, T3 is skipped.

During T4, for Test 1A, 1B, 2A and 2B:

-	The start of T4 is the instant when the last TTI containing LTM cell switch command MAC CE is sent by Cell 2 to the UE.

-	In the cell switch command, Cell 3 is the target cell for PSCell switch. Contention-Free Random-Access Resources are indicated and the field of Timing Advance Command is set to FFF.

-	In test 1A, CandidateTCI-State#2 is indicated.

-	In test 1B, CandidateTCI-State#2 and CandidateTCI-UL-State#1 are indicated.

-	In test 2A, CandidateTCI-State#1 is indicated.

-	In test 2B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are indicated.-	T4 ends upon the reception of PRACH at Cell 3.

Table A.6.3.y.1.2-1: Supported PCell test configurations for intra-frequency PSCell cell switch from FR1 to FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.y.1.2-1A: Supported PSCell test configurations for intra-frequency PSCell cell switch from FR1 to FR1

| Config | Description |
| --- | --- |
| 1A | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2A | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3A | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.y.1.2-2: General test parameters for Intra-frequency cell switch from FR1 to FR1

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cells |  | Cell 1 (PCell), Cell 2 (PSCell) |  |  |  |  |
|  | Neighbouring cell |  | Cell 3 |  |  |  | Cell 3 is the candidate cell |
| Final condition | Active cells |  | Cell 1 (PCell), Cell 3 (PSCell) |  |  |  |  |
| A3-Offset |  | dB | -6 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between Cell 2 and Cell 3 |  |  | 2 s |  |  |  | RTD between Cell 2 and Cell 3 is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 3) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DLorJoint TCI.State.0 | DLorJoint TCI.State.2 | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.1 |  |  |  |  |
| T4 |  | s | 0.1 |  |  |  |  |

Table A.6.3.y.1.2-3: Cell specific test parameters for NR FR1-FR1 Intra-frequency cell switch test case

| Parameter |  |  | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 ~ T4 | T1 ~ T4 | T1 ~ T4 |
| NR RF Channel Number |  |  |  | 1 | 2 | 2 |
| Duplex mode |  | Config 1, 1A |  | FDD |  |  |
|  |  | Config 2, 3, 2A, 3A |  | TDD |  |  |
| TDD configuration |  | Config 1, 1A |  | Not Applicable |  |  |
|  |  | Config 2, 2A |  | TDDConf.1.1 |  |  |
|  |  | Config 3, 3A |  | TDDConf.2.1 |  |  |
| BWchannel |  | Config 1, 1A | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2, 2A | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 3, 3A | MHz | 40: NRB,c = 106 |  |  |
| BWP BW |  | Config 1, 1A | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2, 2A | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 3, 3A | MHz | 40: NRB,c = 106 |  |  |
| PDSCH Reference |  | Config 1, 1A |  | SR.1.1 FDD |  |  |
|  |  | Config 2, 2A |  | SR.1.1 TDD |  |  |
|  |  | Config 3, 3A |  | SR.2.1 TDD |  |  |
| CORESET Reference Channel |  | Config 1, 1A |  | CR.1.1 FDD |  |  |
|  |  | Config 2, 2A |  | CR.1.1 TDD |  |  |
|  |  | Config 3, 3A |  | CR.2.1 TDD |  |  |
| CP length |  | Config 1, 2, 3, 1A, 2A, 3A |  | Normal |  |  |
| TRS configuration |  | Config 1, 1A |  | TRS.1.1 FDD |  |  |
|  |  | Config 2, 2A |  | TRS.1.1 TDD |  |  |
|  |  | Config 3, 3A |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  | Config 1, 2, 3, 1A, 2A, 3A |  | OP.1 |  |  |
| SMTC Configuration |  | Config 1, 2, 3, 1A, 2A, 3A |  | SMTC.1 |  |  |
| SSB Configuration |  | Config 1,2, 1A, 2A |  | SSB.1 FR1 |  |  |
|  |  | Config 3, 3A |  | SSB.2 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2, 1A, 2A | kHz | 15 |  |  |
|  |  | Config 3, 3A | kHz | 30 |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2, 1A, 2A | kHz | 15 |  |  |
|  |  | Config 3, 3A | kHz | 30 |  |  |
| PRACH configuration |  | Config 1, 2, 3, 1A, 2A, 3A |  | FR1 PRACH configuration 6 |  |  |
| BWP configuration | Initial DL BWP | Config 1, 2, 3, 1A, 2A, 3A |  | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP | Config 1, 2, 3, 1A, 2A, 3A |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP | Config 1, 2, 3, 1A, 2A, 3A |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP | Config 1, 2, 3, 1A, 2A, 3A |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS |  | Config 1, 2, 3, 1A, 2A, 3A | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  | Config 1, 2, 3, 1A, 2A, 3A |  |  |  |  |
| Note2 |  | Config 1, 2, 3, 1A, 2A, 3A | dBm/15kHz | -98 |  |  |
| Note2 |  | Config 1,2, 1A, 2A | dBm/SCS | -98 |  |  |
|  |  | Config 3, 3A |  | -95 |  |  |
|  |  | Config 1, 2, 3, 1A, 2A, 3A | dB | 11 | -0.64 | -0.64 |
|  |  | Config 1, 2, 3, 1A, 2A, 3A | dB | 11 | 8 | 8 |
| SSB_RP |  | Config 1,2, 1A, 2A | dBm/SCS | -87 | -90 | -90 |
|  |  | Config 3, 3A | dBm/SCS | -84 | -87 | -87 |
| IoNote3 |  | Config 1,2, 1A, 2A | dBm/ 9.36MHz | -58.7 | -58.7 | -58.7 |
|  |  | Config 3, 3A | dBm/ 38.16MHz | -52.62 | -52.6 | -52.6 |
| Propagation condition |  | Config 1, 2, 3, 1A, 2A, 3A | - | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.3.y.1.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 3 in no later than DLTM from the beginning of time period T4.

The rate of correct cell switches observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU), where:

Tcmd = THARQ + 3ms and is specified in clause 6.3.1.2

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B

-	TLTM-IU = 20 ms

-	TLTM-RRC-processing = 10 ms if UE does not support [Early processing of an LTM candidate cell RRC configuration], otherwise TLTM-RRC-processing =0ms

-	TLTM-processing = 10 ms if the UE supports [faster LTM processing] capability and UE reports 10 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing = 15 ms if the UE supports [faster LTM processing] capability and UE reports 15 ms for FR1-to-FR1 cell switch in the capability

-	TLTM-processing = 20 ms if the UE does not support [faster LTM processing] capability.

End of Change 16

Start of Change 17

A.6.5.X	Subsequent conditional PSCell addition/change

A.6.5.X.1	Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

A.6.5.X.1.1	Test purpose and environment

The purpose of this test is to verify that the subsequent conditional NR PSCell change under NR-DC is within the requirements stated in clause 8.11E.2.

For UE supporting subsequent conditional PSCell addition/change, UE only needs to pass either intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC defined in clause or intra-frequency CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC defined in clause A.7.5.X.1.

For UE which can pass this test, test of conditional PSCell addition and release delay defined in [A.6.5.10] can be skipped.

A.6.5.X.1.2	Test Parameters

Supported test configurations are shown in A.6.5.X.1.2-1. The test scenario comprises three NR cells, Cell 1, Cell 2 and Cell 3. Cell1 is on radio channel 1 in FR1. Cell 2 and 3 are on radio channel 2 in FR1. Test parameters are given in Tables A.6.5.X.1.2-2 and A.6.5.X.1.2-3 below.

The test consists of three successive time periods with duration of T1, T2, and T3 respectively. Before the test starts the UE is connected to Cell 1 (NR PCell) on radio channel 1 (PCC) but is not aware of Cell 2 (NR PSCell) on radio channel 2. The UE is only monitoring the PCC. During T1 only Cell1 is known to the UE.

At the start of time duration T1, the UE does not have any timing information of Cell 2. The TE shall configure subsequent conditional PSCell addition/change with cell 2 and cell 3 as target PSCells during T1, at a time earlier than TRRC_delay before the beginning of T2.

At the start of T2, cell 2 becomes detectable and meets the PSCell addition condition. UE shall be able to measure and detect that the condition is fulfilled, after which it will transmit the PRACH preamble to cell 2. Upon PSCell addition complete (UE transmits SN RRCReconfigurationcomplete message), T3 starts.

At the start of T3, cell 3 becomes detectable and meets the PSCell change condition. UE shall be able to measure and detect that the condition is fulfilled, after which it will transmit the PRACH preamble to cell 3.

Table A.6.5.X.1.2-1: Supported test configurations for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: FDD |
| 2 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode. PSCell: FDD |
| 3 | PCell: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode. PSCell: FDD |
| 4 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: TDD |
| 5 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode. PSCell: TDD |
| 6 | PCell: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode. PSCell: TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.5.X.1.2-2: General Test Parameters for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. One for PCell and second for NR PSCell |
| Initial | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2, Cell3 | Neighbour cells on RF channel number 2. |
| Final | Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Condition | active PSCell |  | Cell3 | PSCell on RF channel number 2. |
| A4 | Hysteresis | dB | 0 | Used to trigger conditional PSCell addition of Cell 2 |
|  | Threshold RSRP | dBm | -118 |  |
|  | Time to Trigger | S | 0 |  |
| A3 |  | dB | 0 | Used to trigger conditional PSCell change from Cell 2 to Cell 3 |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern Id |  |  | 0 | Gaps are configured during T1, T2 and released upon T3 starts. |
| PRACH configuration on cell2 and cell3 |  |  | FR1 PRACH configuration 1 | Captured in A.3.8.2.1 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| T1 |  | s | 1 | During this time the PCell is known and Cell 2 is unknown. |
| T2 |  | s | 5 | During this time Cell 2 meets the PSCell addition condition and UE adds this PSCell. |
| T3 |  | s | 5 | During this time Cell 3 meets the PSCell change condition and UE sends PRACH to Cell 3. |

Table A.6.5.X.1.2-3: Cell Specific Parameters for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter | Unit | Config | Cell 2 |  |  |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 |  | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1,2,3,4,5,6 | 1 |  |  |  |  | 1 |  |  |
| NR RF Channel Number |  | 1,2,3,4,5,6 | 1 |  |  |  |  | 1 |  |  |
| TDD |  | 1,4 | Not Applicable |  |  |  |  | Not Applicable |  |  |
| configuration |  | 2,5 | TDDConf.1.1 |  |  |  |  | TDDConf.1.1 |  |  |
|  |  | 3,6 | TDDConf.2.1 |  |  |  |  | TDDConf.2.1 |  |  |
| BWchannel | MHz | 1,4 | 10: NRB,c = 52 |  |  |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2,5 | 10: NRB,c = 52 |  |  |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3,6 | 40: NRB,c = 106 |  |  |  |  | 40: NRB,c = 106 |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| PDSCH Reference |  | 1,4 | SR.1.1 FDD |  |  |  |  | SR.1.1 FDD |  |  |
| measurement |  | 2,5 | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |
| channel |  | 3,6 | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference |  | 1,4 | CR.1.1 FDD |  |  |  |  | CR.1.1 FDD |  |  |
| Channel |  | 2,5 | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |
|  |  | 3,6 | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET Reference |  | 1,4 | CCR.1.1 FDD |  |  |  |  | CCR.1.1 FDD |  |  |
| Channel |  | 2,5 | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |
|  |  | 3,6 | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1,2,3,4,5,6 | OP.1 |  |  |  |  | OP.1 |  |  |
| SSB configuration |  | 1,2,4,5 | SSB.1 FR1 |  |  |  |  | SSB.1 FR1 |  |  |
|  |  | 3,6 | SSB.2 FR1 |  |  |  |  | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1,2,4,5 | SMTC.1 |  |  |  |  | SMTC.1 |  |  |
|  |  | 3,6 | SMTC.1 |  |  |  |  | SMTC.1 |  |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  |  |  |  | TRS.1.1 FDD |  |  |
|  |  | 2,5 | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |
|  |  | 3,6 | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |
| CSI-RS configuration for CSI reporting |  | 1,4 | CSI-RS.1.1 FDD |  |  |  |  | CSI-RS.1.1 FDD |  |  |
|  |  | 2,5 | CSI-RS.1.1 TDD |  |  |  |  | CSI-RS.1.1 TDD |  |  |
|  |  | 3,6 | CSI-RS.2.1 TDD |  |  |  |  | CSI-RS.2.1 TDD |  |  |
| reportConfigType |  | 1,2,3,4,5,6 | Periodic |  |  |  |  | periodic |  |  |
| reportQuantity |  | 1,2,3,4,5,6 | cri-RI-PMI-CQI |  |  |  |  | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity | slot | 1,2,4,5 | 5 |  |  |  |  | 5 |  |  |
|  |  | 3,6 | 10 |  |  |  |  | 10 |  |  |
| CSI reporting offset | slot | 1,2,4,5 | 2 |  |  |  |  | 2 |  |  |
|  |  | 3,6 | 4 |  |  |  |  | 4 |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 1,2,3,4,5,6 | 0 |  |  |  |  | 0 |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4,5,6 | -98 |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1,2,4,5 | -98 |  |  |  |  |  |  |  |
|  |  | 3,6 | -95 |  |  |  |  |  |  |  |
|  |  | 1,2,3,4,5,6 | -infinity | 8 |  | -3.3 |  | -infinity | -infinity | 2.36 |
|  |  | 1,2,3,4,5,6 | -infinity | 8 |  | 8 |  | -infinity | -infinity | 11 |
| SS-RSRPNote3 | dBm/SCS | 1,2,4,5 | -infinity | -90 |  | -90 |  | -infinity | -infinity | -87 |
|  |  | 3,6 | -infinity | -87 |  | -87 |  | -infinity | -infinity | -84 |
| IoNote3 | dBm/9.36MHz | 1,2,4,5 | -70.05 | -61.41 |  | -57.06 |  | -70.05 | -61.41 | -57.06 |
|  | dBm/38.1MHz | 3,6 | -63.94 | -55.31 |  | -50.96 |  | -63.94 | -55.31 | -50.96 |
| Propagation condition |  | 1,2,3,4,5,6 | AWGN |  |  |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

A.6.5.X.1.3	Test Requirements

TRRC_delay + TEvent_DU for PSCell addition (Cell 2) occurs during T1 as the addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 920+10+62ms=992 ms from the start of T2.

The UE shall start to transmit the PRACH to Cell 3 less than TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 0+920+10+62ms=992 ms from the start of T3.

All of the above test requirements shall be fulfilled in order for the observed conditional PSCell addition and release delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90%.

A.6.5.X.2	Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC

A.6.5.X.2.1	Test purpose and environment

The purpose of this test is to verify that the subsequent conditional NR PSCell addition under NR-DC is within the requirements stated in clause 8.9C.2.

For UE supporting subsequent conditional PSCell addition/change, UE only needs to pass either inter-frequency CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC defined in clause or inter-frequency CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC defined in clause A.7.5.X.2.

For UE which can pass this test, test of conditional PSCell addition and release delay defined in [A.6.5.10] can be skipped.

A.6.5.X.2.2	Test Parameters

Supported test configurations are shown in A.6.5.X.2.2-1.

The test parameters for NR Cell 2 and Cell 3 are given in Tables A.6.5.X.2.2-2 and cell-specific parameters in A.6.5.X.2.2-3 below.

The test consists of four successive time periods with duration of T1, T2, T3 and T4 respectively. Before the test starts the UE is connected to Cell 1 (NR PCell) on radio channel 1 (PCC) but is not aware of Cell 2 (NR PSCell) on radio channel 2. The UE is only monitoring the PCC. During T1 only Cell1 is known to the UE.

At the start of time duration T1, the UE does not have any timing information of Cell 2. The TE shall configure subsequent conditional PSCell addition with cell 2 and cell 3 as target PSCells during T1, at a time earlier than TRRC_delay before the beginning of T2.

At the start of T2, cell 2 becomes detectable and meets the PSCell addition condition. UE shall be able to measure and detect that the condition is fulfilled, after which it will transmit the PRACH preamble to cell 2. Upon PSCell addition complete (UE transmits SN RRCReconfigurationcomplete message), T3 starts.

At the start of T3, the test system shall send a RRCRconfiguration message to the UE to release PSCell (Cell 2) on radio channel 2. Upon PSCell release complete (UE transmits SN RRCReconfigurationcomplete message), T4 starts.

At the start of T4, cell 3 becomes detectable and meets the subsequent PSCell addition condition. UE shall be able to measure and detect that the condition is fulfilled during time Tmeasure, after which it will transmit the PRACH preamble to cell 3.

Table A.6.5.X.2.2-1: Supported test configurations for Inter-frequency Subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: FDD |
| 2 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode. PSCell: FDD |
| 3 | PCell: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode. PSCell: FDD |
| 4 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: TDD |
| 5 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode. PSCell: TDD |
| 6 | PCell: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode. PSCell: TDD |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Table A.6.5.X.2.2-2: General Test Parameters for Subsequent Conditional PSCell Addition and Release

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for NR Cells for this test. |
| Initial | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2 | Neighbour cell on RF channel number 2. |
|  |  |  | Cell3 | Neighbour cell on RF channel number 3. |
| Final | Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Condition | Active PSCell |  | Cell3 | PSCell on RF channel number 3 |
|  | Neighbour Cell |  | Cell2 | Neighbour cell on RF channel number 2 |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -118 | A4 event is used to trigger conditional PSCell addition of Cell 2. |
|  | Time to Trigger | s | 0 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern Id |  |  | 0 | Gaps are configured during T1, T2, T3 and T4. |
| PRACH configuration on cell2 |  |  | FR1 PRACH configuration 1 | Captured in A.3.8.2.1 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on carrier frequency of cell3. |
| T1 |  | s | 1 | During this time the PCell is known, PSCell 1 and PSCell 2 are unknown. |
| T2 |  | s | 7 | During this time PSCell 1 meets the PSCell addition condition and UE adds this PSCell. |
| T3 |  | s | 1 | During this time the UE releases the PSCell 1. |
| T4 |  | s | 7 | During this time PSCell 2 meets the addition condition and UE adds this PSCell. |

Table A.6.5.X.2.2-3: Cell Specific Parameters for Subsequent Conditional PSCell Addition and Release

| Parameter | Unit | Config | Cell 2 |  |  |  |  | Cell 3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 | T3 | T4 | T1 | T2 |  |  | T3 |  | T4 |
| NR RF Channel Number |  | 1,2,3,4,5,6 | 2 |  |  |  |  | 3 |  |  |  |  |  |  |
| TDD |  | 1,4 | Not Applicable |  |  |  |  | Not Applicable |  |  |  |  |  |  |
| configuration |  | 2,5 | TDDConf.1.1 |  |  |  |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | 3,6 | TDDConf.2.1 |  |  |  |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel | MHz | 1,4 | 10: NRB,c = 52 |  |  |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | 2,5 | 10: NRB,c = 52 |  |  |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | 3,6 | 40: NRB,c = 106 |  |  |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |  |
| PDSCH Reference |  | 1,4 | SR.1.1 FDD |  |  |  |  | SR.1.1 FDD |  |  |  |  |  |  |
| measurement |  | 2,5 | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  |  |  |  |
| channel |  | 3,6 | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET Reference |  | 1,4 | CR.1.1 FDD |  |  |  |  | CR.1.1 FDD |  |  |  |  |  |  |
| Channel |  | 2,5 | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | 3,6 | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  |  |  |  |
| Dedicated CORESET Reference |  | 1,4 | CCR.1.1 FDD |  |  |  |  | CCR.1.1 FDD |  |  |  |  |  |  |
| Channel |  | 2,5 | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |  |  |  |
|  |  | 3,6 | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  | 1,2,3,4,5,6 | OP.1 |  |  |  |  | OP.1 |  |  |  |  |  |  |
| SSB configuration |  | 1,2,4,5 | SSB.1 FR1 |  |  |  |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | 3,6 | SSB.2 FR1 |  |  |  |  | SSB.2 FR1 |  |  |  |  |  |  |
| SMTC configuration |  | 1,2,4,5 | SMTC.1 |  |  |  |  | SMTC.1 |  |  |  |  |  |  |
|  |  | 3,6 | SMTC.1 |  |  |  |  | SMTC.1 |  |  |  |  |  |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  |  |  |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | 2,5 | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | 3,6 | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting |  | 1,4 | CSI-RS.1.1 FDD |  |  |  |  | CSI-RS.1.1 FDD |  |  |  |  |  |  |
|  |  | 2,5 | CSI-RS.1.1 TDD |  |  |  |  | CSI-RS.1.1 TDD |  |  |  |  |  |  |
|  |  | 3,6 | CSI-RS.2.1 TDD |  |  |  |  | CSI-RS.2.1 TDD |  |  |  |  |  |  |
| reportConfigType |  | 1,2,3,4,5,6 | Periodic |  |  |  |  | periodic |  |  |  |  |  |  |
| reportQuantity |  | 1,2,3,4,5,6 | cri-RI-PMI-CQI |  |  |  |  | cri-RI-PMI-CQI |  |  |  |  |  |  |
| CSI reporting periodicity | slot | 1,2,4,5 | 5 |  |  |  |  | 5 |  |  |  |  |  |  |
|  |  | 3,6 | 10 |  |  |  |  | 10 |  |  |  |  |  |  |
| CSI reporting offset | slot | 1,2,4,5 | 2 |  |  |  |  | 2 |  |  |  |  |  |  |
|  |  | 3,6 | 4 |  |  |  |  | 4 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 1,2,3,4,5,6 | 0 |  |  |  |  | 0 |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4,5,6 | -85 |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1,2,4,5 | -85 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3,6 | -82 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1,2,3,4,5,6 | -∞ | 0 |  | -∞ | -∞ | -∞ |  | -∞ | -∞ |  | 0 |  |
|  |  | 1,2,3,4,5,6 | -∞ | 0 |  | -∞ | -∞ | -∞ |  | -∞ | -∞ |  | 0 |  |
| SS-RSRPNote3 | dBm/SCS | 1,2,4,5 | -∞ | -85 |  | -∞ | -∞ | -∞ |  | -∞ | -∞ |  | -85 |  |
|  |  | 3,6 | -∞ | -82 |  | -∞ | -∞ | -∞ |  | -∞ | -∞ |  | -82 |  |
| IoNote3 | dBm/9.36MHz | 1,2,4,5 | -57 | -54 |  | -57 | -57 | -57 |  | -57 | -57 |  | -54 |  |
|  | dBm/38.1MHz | 3,6 | -57 | -48 |  | -57 | -57 | -57 |  | -57 | -57 |  | -48 |  |
| Propagation condition |  | 1,2,3,4,5,6 | AWGN |  |  |  |  | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.6.5.X.2.3	Test Requirements

TRRC_delay + TEvent_DU for PSCell addition (Cell 2) occurs during T1 as the addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 1040+10+62ms=1112 ms from the start of T2.

The UE shall start to transmit the PRACH to Cell 3 less than TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms = 0+1040+10+62ms=1112 ms from the start of T4.

All of the above test requirements shall be fulfilled in order for the observed conditional PSCell addition and release delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90%.

End of Change 17

Start of Change 18

A.6.6.9.2	Test case for Idle mode fast CA/DC eEMR measurement for FR1

A.6.6.9.2.1	Test Purpose and Environment

The purpose of this test is to verify UE measurement reporting behaviour as specified in clause 4.7. This test will partly verify the fast CA/DC measurement reporting requirements in clause 4.7 when measIdleValidityDuration is configured for the test case when there are no measurement results to report at RRC connection setup.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as inter-frequency neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.9.2.1-1, A.6.6.9.2.1-2, A.6.6.9.2.1-3.

The test consists of 4 successive time periods, with time duration of T1, T2, T3 and T4 respectively.

During T1, the UE is connected to cell 1 only and shall not have any timing information of cell 2. UE is configured with early measurement reporting for cell 2 in measIdleCarrierListNR-r16. Beam level reporting for early measurements is not configured. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

During T2 and T3 the UE is in idle mode.

At the beginning of T2, cell 2 becomes detectable however cell reselection shall not be performed. Signal level of cell 2 is set to the value given in Table A.6.6.9.2.1-3. The time when T331 timer expires defines the ending point of T2.

At the beginning of T3, the signal level of the neibour cell is set to turned off.  The duration of the T3 equals to measIdleValidityDuration.

The time when TE sends the paging message is defined as the starting point of T4. During T4, in this test the UE shall not send measurement report.

Table A.6.6.9.2.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.9.2.1-2: General test parameters for Idle mode fast CA/DC eEMR measurement for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| T331 | s |  | 30 |  |
| measIdleValidityDuration | s |  | 10 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 30 |  |
| T3 | s | Config 1,2,3 | 10 |  |
| T4 | s |  | 1 |  |

Table A.6.6.9.2.1-3: Cell specific test parameters for Idle and connected mode for fast CA/DC eEMR measurement for FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |  |  |
| TDD configuration |  |  | N.A | N.A |  |  |  |  |  |  |  |  |
|  |  |  | TDDConf.1.1 | TDDConf.1.1 |  |  |  |  |  |  |  |  |
|  |  |  | TDDConf.2.1 | TDDConf.2.1 |  |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  |  | -98 |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  |  | -98 |  |  |  |  |
|  |  |  | Config 3 | -95 |  |  |  | -95 |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 |  |  |  | -infinity | -98 | infinity | infinity |
|  |  |  | Config 3 | -88 |  |  |  | -infinity | -95 | infinity | infinity |
|  |  | dB | Config 1,2,3,4, | 7 |  |  |  | - infinity | 0 | infinity | infinity |
|  |  | dB | Config 1,2,3 | 7 |  |  |  | infinity | 0 | infinity | infinity |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -62.26 |  |  |  | -70.05 | -67.04 | -70.05 | -70.05 |
|  |  | dBm/38.16MHz | Config 3 | -56.15 |  |  |  | -63.94 | -60.93 | -63.94 | -63.94 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |  |

A.6.6.9.2.2	Test Requirements

During the period T2 and T3, the UE shall not perform reselection.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for cell 2.

The UE shall NOT send early measurement report to the PCell in this test.

The rate of correct events observed during repeated tests shall be at least 90%.

End of Change 18

Start of Change 19

A.6.6.9.3	Test case for Idle mode fast CA/DC cell reselection measurement for FR1 without valid reporting

A.6.6.9.3.1	Test Purpose and Environment

The purpose of this test is to verify UE measurement reporting behaviour as specified in clause 4.7. This test will partly verify the fast CA/DC measurement reporting requirements in clause 4.7 when  measReselectionValidityDuration-r18 is configured for the test case when there are no measurement results to report at RRC connection setup.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as inter-frequency neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.9.3.1-1, A.6.6.9.3.1-2, A.6.6.9.3.1-3.

The test consists of 4 successive time periods, with time duration of T1, T2, T3 and T4 respectively.

During T1, the UE is connected to cell 1 only and shall not have any timing information of cell 2. UE is configured with inter-frequency measurement reporting for cell 2 in MeasReselectionCarrierListNR-r18. Beam level reporting for early measurements is not configured. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

During T2 and T3 the UE is in idle mode.

At the beginning of T2, cell 2 becomes detectable however cell reselection shall not be performed. Signal level of cell 2 is set to the value given in Table A.6.6.9.3.1-3. The duration of T2 is set to fixed value according to the Table A.6.6.9.3.1-2.

At the beginning of T3, the singal level of cell 2 is set to turned off. The duration of the T3 equals to measReselectionValidityDuration-r18.

The time when TE sends the paging message defined as the starting point of T4. During T4, in this test the UE shall not send measurement report.

Table A.6.6.9.3.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.9.3.1-2: General test parameters for Idle mode fast CA/DC cell-reselection measurement for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| measReselectionValidityDuration-r18 | s |  | 5 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 30 |  |
| T3 | s | Config 1,2,3 | 5 |  |
| T4 | s |  | 1 |  |

Table A.6.6.9.3.1-3: Cell specific test parameters for Idle and connected mode for fast CA/DC cell re-selection measurement for FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | N.A | N.A |  |  |  |  |  |  |  |
|  |  |  | TDDConf.1.1 | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  |  | TDDConf.2.1 | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  |  | -98 |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  |  | -98 |  |  |  |
|  |  |  | Config 3 | -95 |  |  |  | -95 |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 |  |  |  | -infinity | -98 | infinity | infinity |
|  |  |  | Config 3 | -88 |  |  |  | -infinity | -95 | infinity | infinity |
|  |  | dB | Config 1,2,3,4, | 7 |  |  |  | - infinity | 0 | infinity | infinity |
|  |  | dB | Config 1,2,3 | 7 |  |  |  | infinity | 0 | infinity | infinity |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -62.26 |  |  |  | -70.05 | -67.04 | -70.05 | -70.05 |
|  |  | dBm/38.16MHz | Config 3 | -56.15 |  |  |  | -63.94 | -60.93 | -63.94 | -63.94 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |

A.6.6.9.3.2	Test Requirements

During the period T2, the UE shall not perform reselection.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for cell 2.

The UE shall NOT send early measurement report to the PCell in this test.

The rate of correct events observed during repeated tests shall be at least 90%.

End of Change 19

Start of Change 20

A.6.6.9.4	Test case for Idle mode fast CA/DC cell reselection measurement for FR1 with valid reporting

A.6.6.9.4.1	Test Purpose and Environment

The purpose of this test is to verify UE measurement reporting behaviour as specified in clause 4.7. This test will partly verify the m fast CA/DC measurement reporting requirements in clause 4.7 when measReselectionValidityDuration-r18 is configured for the test case when there are measurement results to report at RRC connection setup.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.  The test parameters are given in Tables A.6.6.9.4.1-1, A.6.6.9.4.1-2, A.6.6.9.4.1-3.

The test consists of 4 successive time periods, with time duration of T1, T2, T3 and T4 respectively.

During T1, the UE is connected to cell 1 only and shall not have any timing information of cell 2. UE is configured with inter-frequency measurement reporting for cell 2 in MeasReselectionCarrierListNR-r18. Beam level reporting for early measurements is not configured. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

At the beginning of T2, cell 2 becomes detectable however cell reselection shall not be performed. Signal level of cell 2 is set to the value given in Table A.6.6.9.4.1-3. The duration of T2 is set to fixed value according to the Table A.6.6.9.4.1-2.

At the beginning of T3, the signal level of cell 2 is set to another value according to the Table A.6.6.9.4.1-3. The duration of T3 equals to measReselectionValidityDuration-r18 .

The time when TE sends the paging message is defined as the starting point of T4. During T4, in this test the UE shall send measurement report within the duration of T4.

Table A.6.6.9.4.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.6.6.9.4.1-2: General test parameters for Idle mode fast CA/DC cell-reselection measurement for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| measReselectionValidityDuration-r18 | s |  | 5 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 10 |  |
| T2 | s | Config 1,2,3 | 30 |  |
| T3 | s | Config 1,2,3 | 5 |  |
| T4 | s |  | 1 |  |

Table A.6.6.9.3.1-3: Cell specific test parameters for Idle and connected mode for fast CA/DC cell re-selection measurement for FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | N.A | N.A |  |  |  |  |  |  |  |
|  |  |  | TDDConf.1.1 | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  |  | TDDConf.2.1 | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  |  | -98 |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  |  | -98 |  |  |  |
|  |  |  | Config 3 | -95 |  |  |  | -95 |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -88 |  |  |  | -infinity | -91 | -100 | -100 |
|  |  |  | Config 3 | -85 |  |  |  | -infinity | -88 | -97 | -97 |
|  |  | dB | Config 1,2,3,4, | 10 |  |  |  | - infinity | 7 | -2 | -2 |
|  |  | dB | Config 1,2,3 | 10 |  |  |  | -infinity | 7 | -2 | -2 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -59.63 |  |  |  | -70.5 | -62.26 | -67.92 | -67.92 |
|  |  | dBm/38.16MHz | Config 3 | -53.53 |  |  |  | -63.94 | -56.15 | -61.82 | -61.82 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |

A.6.6.9.4.2	Test Requirements

During the period T2 and T3 the UE shall not perform reselection.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for cell 2.

The UE shall send early measurement report to the PCell with valid measurement results.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for cell 2 meets the requirements in Section 10.1.4B for SS-RSRP and in Section 10.1.8B for SS-RSRQ and test ends. In the test case, the reported measurements are considered valid if they fulfil measurement accuracy requirements according to cell 2 signal level during T3.

The rate of correct events observed during repeated tests shall be at least 90%.

End of Change 20

Start of Change 21

A.6.6.x	LTM Intra-frequency L1-RSRP measurement

A.6.6.x.1	Intra-frequency SSB based L1-RSRP measurement in FR1

A.6.6.x.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of SSB based intra-frequency L1-RSRP measurement on neighbor cell in FR1. This test will partly verify the requirements for SSB based intra-frequency L1-RSRP measurement on neighbor cell specified in clause 9.14, with the testing configurations for NR cells in Table A.6.6.x.1.1-1.

Table A.6.6.x.1.1-1: Applicable NR configurations for SSB based intra-frequency L1-RSRP LTM measurement test in FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

A.6.6.x.1.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. Measurement period [and measurement accuracy] is tested by using the parameters in table A.6.6.x.1.2-1, and A.6.6.x.1.2-2.

There are two tests in the test case, test 1 and test 2:

In test 1, time offset between cells is within CP length.

In test 2, time offset between cells is larger than CP length.

If a UE does not support [RTD>CP], it is only required to pass test 1. Otherwise, it is only required to pass test 2.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. SSB_RP of Cell 2 in T1 and T2 are different.  No gap patterns are configured in the test case.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC).

-	A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Before the start of the T1, event is triggered, and UE has sent a measurement report for the Cell 2 with SSB Index.

-    UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

At the beginning of T2, SSB_RP of Cell 2 changes to a different value from T1. T2 starts at the beginning of a frame with an odd SFN.

Table A.6.6.x.1.2-1: General test parameters for SSB based intra-frequency L1-RSRP LTM measurement test in FR1

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbouring cell |  |  | Cell 2 |  | Cell 2 is the candidate cell |
| A3-Offset |  | dB | -6 |  |  |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | ms | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| DRX |  |  | OFF |  | DRX is not used |
| Time offset between cells |  |  | 2 s | 20s | The timing of Cell 2 is later than the timing of Cell 1 |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | S | 0.3 |  |  |
| T2 |  | S | 0.5 |  |  |

Table A.6.6.x.1.2-2: Cell specific test parameters for SSB based intra-frequency L1-RSRP LTM measurement test in FR1

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  |  |
| SSB GSCN |  |  |  | freq1 |  |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  | N/A |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | N/A |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | N/A |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | N/A |  |  |
| CP length |  |  |  | Normal |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | KHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | KHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -101 |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -101 |  |  |  |  |
|  | Config 3 |  |  | -98 |  |  |  |  |
|  |  |  | dB | -1.76 | -0.19 |  | -1.76 | -0.19 |
|  |  |  | dB | 3 | 14.5 |  | 3 | 14.5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -98 | -87.5 |  | -98 | -87.5 |
|  | Config 3 |  | dBm/SCS | -95 | -84.5 |  | -95 | -84.5 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -66.07 | -56.44 |  | -66.07 | -56.44 |
|  | Config 3 |  | dBm/ 38.16MHz | -59.96 | -50.34 |  | -59.96 | -50.34 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

A.6.6.x.1.3 Test Requirements

The UE shall send L1-RSRP report every 80 slots. The UE shall start to report a larger L1-RSRP value of Cell 2 in no later than 20 ms plus 80 slots from the beginning of time period T2. UE shall send L1-RSRP report including results of Cell 2 while meeting the L1-RSRP absolute accuracy requirement in clause 10.1.19X.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

End of Change 21

Start of Change 22

A.6.6.y	LTM Inter-frequency L1-RSRP measurement with measurement gap

A.6.6.y.1	Inter-frequency SSB based L1-RSRP measurement with measurement gap

A.6.6.y.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of inter-frequency L1-RSRP measurement with measurement gap on candidate neighbour cell. This test will partly verify the L1-RSRP measurement requirements in clause 9.15.5, with the testing configurations for NR cells in Table A.6.6.y.1.1-1.

Table A.6.6.y.1.1-1: Applicable NR configurations for SSB based inter-frequency L1-RSRP LTM measurement with MG test in FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

A.6.6.y.1.2	Test parameters

There are two carriers and one cell on each carrier in the test, NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters for the Cell 1 and Cell 2 are given in Table A.6.6.y.1.2-1 and Table A.6.6.y.1.2-2 below.

In CSI measurement configuration, UE is indicated to perform inter-frequency L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively.

Measurement gap pattern configuration defined in Table A.6.6.y.1.2-1 is provided.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC).

-	A measurement object is configured for the RF channel 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Before the start of the T1, event is triggered, and UE has sent a measurement report for the Cell 2 with SSB Index.

-    UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

At the beginning of T2, SSB_RP of Cell 2 changes to a different value from T1.

Table A.6.6.y.1.2-1: General test parameters for SSB based inter-frequency L1-RSRP LTM measurement with MG test in FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 |  |
| Neighbouring cell |  |  | Cell 2 | Cell 2 is the candidate cell |
| A3-Offset |  | dB | -6 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | ms | 160 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| reportQuantityRS-Indexes |  |  | rsrp |  |
| maxNrofRS-IndexesToReport |  |  | 1 |  |
| includeBeamMeasurements |  |  | True |  |
| DRX |  |  | OFF | DRX is not used |
| Time offset between cells |  |  | 3 s | The timing of Cell 2 is later than the timing of Cell 1 |
| deriveSSB-IndexFromCell |  |  | Enabled | Not relevant to this test case |
| Gap Pattern Id |  |  | 0 |  |
| Measurement gap offset |  | ms | 39 |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |
|  | spCellInclusion |  | N/A |  |
| ltm-ConfigComplete |  |  | True | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |
| T2 |  | s | 0.5 |  |

Table A.6.6.y.1.2-2: Cell specific test parameters for SSB based inter-frequency L1-RSRP LTM measurement with MG test in FR1

| Parameter | Config | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB GSCN | 1~3 |  | freq1 |  | freq2 |  |
| Duplex mode | 1 |  | FDD |  |  |  |
|  | 2 |  | TDD |  |  |  |
|  | 3 |  | TDD |  |  |  |
| TDD Configuration | 1 |  | N/A |  |  |  |
|  | 2 |  | TDDConf.1.1 |  |  |  |
|  | 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  | 2 |  | 10: NRB,c = 52 |  |  |  |
|  | 3 |  | 40: NRB,c = 106 |  |  |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |  | N/A |  |
|  | 2 |  | SR.1.1 TDD |  | N/A |  |
|  | 3 |  | SR.2.1 TDD |  | N/A |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |  | N/A |  |
|  | 2 |  | CR.1.1 TDD |  | N/A |  |
|  | 3 |  | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |  | N/A |  |
|  | 2 |  | CCR.1.1 TDD |  | N/A |  |
|  | 3 |  | CCR.2.1 TDD |  | N/A |  |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  |  |
|  | 2 |  | SSB.1 FR1 |  |  |  |
|  | 3 |  | SSB.2 FR1 |  |  |  |
| OCNG Patterns | 1~3 |  | OP.1 |  | OP.1 |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| SMTC configuration | 1~3 |  | SMTC.1 |  |  |  |
| TRS Configuration | 1 |  | TRS.1.1 FDD |  | N/A |  |
|  | 2 |  | TRS.1.1 TDD |  | N/A |  |
|  | 3 |  | TRS.1.2 TDD |  | N/A |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | 0 | 8 |
|  | 1~3 | dB | 0 | 0 | 0 | 8 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -94.65 | -86.65 |
|  | 3 |  | -91.65 | -91.65 | -91.65 | -83.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -63.69 | -58.06 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -57.59 | -51.97 |
| Propagation condition | 1~3 |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves.. |  |  |  |  |  |  |

A.6.6.y.1.3	Test Requirements

During T1 The UE shall send inter-frequency L1-RSRP report every 80 slots. No later than 80 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report of Cell 2. The RSRP report during T2 shall be larger than that during T1. These reported measurement report shall meet the absolute accuracy requirement in clause 10.1.19y. The rate of correct events observed during repeated tests shall be at least 90%.

End of Change 22

Start of Change 23

A.6.6.z	LTM Inter-frequency L1-RSRP measurement without measurement gap

A.6.6.z.1	Inter-frequency SSB based L1-RSRP measurement without measurement gap

A.6.6.z.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting inter-frequency L1-RSRP measurements without gap makes correct reporting of inter-frequency L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.15.6, with the testing configurations for NR serving cells in Table A.6.6.z.1.1-1.

Table A.6.6.z.1.1-1: Applicable NR configurations for SSB based inter-frequency L1-RSRP measurement without measurement gap in test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

A.6.6.z.1.2	Test parameters

There are two cells in the test, the FR1 PCell (Cell 1) on NR RF channel 1 and Cell 2 as neighbour cell in FR1 on NR RF channel 2. The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from Cell 1 and Cell 2 should be different in frequency location within the cell bandwidth. The test parameters are given in Table A.6.6.z.1.2-1 and Table A.6.6.z.1.2-2 below.

There are two tests in the test case, test 1 and test 2:

In test 1, time offset between cells is within CP length.

In test 2, time offset between cells is larger than CP length.

UE not capable of [RTD>CP] is only required to pass test 1. Otherwise, it is only required to pass test 2.

The test consists of two successive time periods, with time duration of T1 and T2 respectively. There is no measurement gap configured in the test.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC).

-	A measurement object is configured for RF channel 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Before the start of the T1, event is triggered, and UE has sent a measurement report for the Cell 2 with SSB Index.

-    UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

At the beginning of T2, SSB_RP of Cell 2 changes to a different value from T1. T2 starts at the beginning of a frame with an even SFN.

Table A.6.6.z.1.2-1: General test parameters  for SSB based inter-frequency L1-RSRP measurement without measurement gap in test

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbour cell |  |  | Cell 2 |  | Cell 2 is candidate cell |
| RF Channel Number |  |  | 1: Cell 1 2: Cell 2 |  |  |
| A3-Offset |  | dB | -6 |  |  |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | s | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| maxNrofRS-IndexesToReport |  |  | 1 |  |  |
| includeBeamMeasurements |  |  | True |  |  |
| Time offset between cells |  | s | 2 | 20 |  |
| DRX |  |  | OFF |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |  |
| T2 |  | s | 0.5 |  |  |

Table A.6.6.z.1.2-2: Cell specific test parameters  for SSB based inter-frequency L1-RSRP measurement without measurement gap in test

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 2 |  |  |
| SSB GSCN |  |  |  | freq1 |  | freq2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  | N/A |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | N/A |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | N/A |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | N/A |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | N/A |  |  |
| CP length |  |  |  | Normal |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |
| SMTC Configuration |  | Config 1 |  | SMTC.2 |  | SMTC.5 |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  | SMTC.4 |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  | SSB.5 FR1 |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  | SSB.6 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  | NA |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  | NA |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  | NA |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  | NA |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -101 |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -101 |  |  |  |  |
|  | Config 3 |  |  | -98 |  |  |  |  |
|  |  |  | dB | -1.76 | -0.19 |  | -1.76 | -0.19 |
|  |  |  | dB | 3 | 14.5 |  | 3 | 14.5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -98 | -87.5 |  | -98 | -87.5 |
|  | Config 3 |  | dBm/SCS | -95 | -84.5 |  | -95 | -84.5 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -66.07 | -56.44 |  | -66.07 | -56.44 |
|  | Config 3 |  | dBm/ 38.16MHz | -59.96 | -50.34 |  | -59.96 | -50.34 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

A.6.6.z.1.3	Test Requirements

The UE shall send L1-RSRP report every 80 slots. No later than 20 ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report of Cell 2 while meeting the absolute accuracy requirement in clause 10.1.19.x.y and relative accuracy requirement in clause 10.1.19.x.y.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

End of Change 23

Start of Change 24

A.6.7.x	LTM L1-RSRP measurement

A.6.7.x.1	Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1

A.6.7.x.1.1	Test Purpose and Environment

The purpose of this test is to verify that the inter-frequency L1-RSRP measurement accuracy on neigbor cell is within the specified limits. This test will verify the requirements in clause 9.15.5 and clause [10.1.19Y] for inter-frequency L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.6.7.x.1.1-1.

Table A.6.7.x.1.1-1: Applicable NR configurations for FR1 SSB based inte-frequency L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations in each supported band Note 2: Target NR cell has the same SCS, BW and deplex mode as NR serving cell. |  |

A.6.7.x.1.2	Test parameters

In this set of test cases there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters for the Cell 2 are given in Table A.6.7.x.1.2-1 below. The absolute and relative accuracy of L1-RSRP measurements are tested by using the parameters in Table A.6.7.x.1.2-1.

Measurement gap pattern configuration defined in Table A.6.7.x.1.2-1 is provided. Before the test,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC)

-	UE is configured one SSB resource set with one SSB resource on Cell 2

-	A measurement object is configured for the RF channel 2, and it is indicated to the UE to reprort periodica reporting with with SSB Index.

-    UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

Table A.6.7.x.1.2-1: FR1 inter-frequency SSB based L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell1 | Cell 2 | Cell 1 | Cell 2 |
| SSB GSCN |  | 1~3 |  | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | 1 |  | FDD |  | FDD |  |
|  |  | 2 |  | TDD |  | TDD |  |
|  |  | 3 |  | TDD |  | TDD |  |
| Gap Pattern Id |  | 1~3 |  | 0 |  |  |  |
| Measurement gap offset |  | 1~3 |  | 39 |  |  |  |
| TDD Configuration |  | 1 |  | N/A |  | N/A |  |
|  |  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  |  | 2 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  |  | 3 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | 2 |  | SR.1.1 TDD | - | SR.1.1 TDD | - |
|  |  | 3 |  | SR.2.1 TDD | - | SR.2.1 TDD | - |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  |  | 3 |  | CR.2.1 TDD | - | CR.2.1 TDD | - |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  |  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  | OP.1 |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  |  | 2 |  | TRS.1.1 TDD | - | TRS.1.1 TDD | - |
|  |  | 3 |  | TRS.1.2 TDD | - | TRS.1.2 TDD | - |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  | 1~3 |  | SMTC.1 |  | SMTC.1 |  |
| reportConfigType |  | 1~3 |  | periodic |  | periodic |  |
| reportQuantity |  | 1~3 |  | ssb-Index-RSRP |  | ssb-Index-RSRP |  |
| Number of reported RS |  | 1~3 |  | 1 |  | 1 |  |
| L1-RSRP reporting period |  | 1~3 |  | slot80 |  | slot80 |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 |  | ( for Channel 2 +8dB) | -117 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -110.5 |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -94.65 |  | ( for Channel 2 +8dB) | -117 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -110.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -91.65 |  | ( for Channel 2 +8dB) | -114 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -113.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -111 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -110.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -107.5 |
|  |  | 1~3 | dB | 10 | 10 | 13 | -3 |
| SSB RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/SSB SCS | -84.65 |  | (RSRP for Cell 2 +24dB) | -120 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -119.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -119 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -118.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -118 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -117.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -117 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -116.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 |  | -81.65 |  | (RSRP for Cell 2 +24dB) | -117 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -116.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -116 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -115.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -115 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -114.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -110.5 |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2 | dBm/9.36 MHz | -56.28 |  | (Io for Channel 2 +19.75dB) | -87.28 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -86.78 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -86.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -85.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -85.28 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -84.78 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -84.28 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -83.78 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -80.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3 | dBm/38.16 MHz | -50.19 |  | (Io for Channel 2 +19.75dB) | -81.19 |
|  | NR_FDD_FR1_B |  |  |  |  |  | -80.69 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -80.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -79.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -79.19 |
|  | NR_FDD_FR1_F |  |  |  |  |  | -78.69 |
|  | NR_FDD_FR1_G |  |  |  |  |  | -78.19 |
|  | NR_FDD_FR1_H |  |  |  |  |  | -77.69 |
|  | NR_FDD_FR1_N |  |  |  |  |  | -74.69 |
|  |  | 1~3 | dB | 10 | 10 | 13 | -3 |
| Propagation condition |  | 1~3 |  | AWGN |  | AWGN |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

A.6.7.x.1.3	Test Requirements

The inter-frequency L1-RSRP measurement accuracy for SSB resource reported by UE in L1-RSRP report (SSB#0 of Cell 2) shall fulfil the requirements in clauses 10.1.19y.

End of Change 24

Start of Change 25

#### A.7.3.2.x	LTM PDCCH-order Random Access

##### A.7.3.2.x.1	PDCCH-order RACH on neighbor cell in FR2 when RACH BW is within active BWP

A.7.3.2.x.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 PDCCH-ordered RACH to an intra-frequency candidate cell in FR2 for LTM. The interruption requirements specified in clause 8.2.2.2.20. This test is for UE supporting PDCCH-ordered RACH to an intra-frequency candidate cell, whose SSB is within active BWPs of the UE.

A.7.3.2.x.1.2	Test Parameters

Two cells are deployed in the test, which are FR2 PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. Test configurations are given in table A.7.3.2.x.1.2-1. Both PDCCH order RACH delay and transmit timing requirement are tested by using the parameters in table A.7.3.2.x.1.2-2, and A.7.3.2.x.1.2-3.

There are two tests in the test case, test 1 and test 2:

In test 1, joint TCI state configuration as defined in Table A.7.3.2.x.1.2-2 is provided for UE that supports ltm-BeamIndicationJointTCI-r18.

In test 2, separate TCI state configuration as defined in Table A.7.3.2.x.1.2-2 for test 2 is provided for UE that supports ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

If a UE supports ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18, it is only required to pass test 2. Otherwise, it is only required to pass test 1.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. No gap patterns are configured in the test case.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	The UE has performed L3 measurement and SSB based L1-RSRP measurement on Cell 2.

T1 starts from UE transmitting a valid L1 report on Cell 2. After receiving the first L1 report on Cell 2 during T1, the network sends TCI state activation MAC CE to active TCI state of Cell 2 in no later than 100ms.

In test 1, CandidateTCI-State#1 is activated.

In test 2, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are activated.

For UE incapable of early TCI state activation, network shall not send TCI state activation MAC CE to active TCI state of Cell 2.

The start of T2 is the instant when PDCCH order to trigger PRACH transmission on Cell 2 is sent to the UE.

Table A.7.3.2.x.1.2-1: PDCCH order RACH on Neighbor cell in FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.x..2.2-2: General test parameters for PDCCH order RACH in FR2

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |
|  | Neighbouring cell |  | Cell 2 |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 1 |  | After transmitting PRACH on Cell 2, UE shall be back to Cell 1. |
| A3-Offset |  | dB | 0 |  |  |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | s | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| DRX |  |  | OFF |  | DRX is not used |
| Time offset between cells |  |  | 0.3 s |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  | NR RF Channel Number 1 |  | Same as Cell 1 |
|  | PRACH configuration |  | FR2 PRACH configuration 5 |  | RACH bandwidth is within active UL BWP of Cell 1 |
|  | bwp-GenericParameters |  | ULBWP.0.1 |  |  |
|  | n-TimingAdvanceOffset | Tc | N/A |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | #1 CandidateTCI-State |  | DLorJoint TCI.State.0 | DLorJoint TCI.State.2 | As specified in clause A.3.16B. Configured for early TCI state activation. |
| ltm-UL-TCI-StatesToAddModList | #1 CandidateTCI-UL-State#0 |  | N/A | UL TCI.State.0 | As specified in clause A.3.16B. Configured for early TCI state activation. |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |  |
| T2 |  | s | 0.5 |  |  |

Table A.7.3.2.x..2.2-3: Cell specific test parameters for PDCCH order RACH test case in FR2

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  | Rough |  |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |  |  |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  |
| Duplex mode |  |  |  | TDD |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |
| Data RBs allocated |  |  |  | 66 |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | O P. 1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 6 |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.2 |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  | -104.7 |  |
| Note2 |  |  | dBm/SCS | -95.7 |  | -95.7 |  |
|  |  |  | dB | -1.8 |  | 0 |  |
|  |  |  | dB | 6 |  | 7 |  |
| SSB_RP |  |  | dBm/SCS | -89.7 |  | -88.7 |  |
| IoNote3 |  |  | dBm/ 95.04MHz | -56.7 |  | -56.7 |  |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

A.7.3.2.x.1.3 Test Requirements

The UE shall transmit the PRACH to Cell 2 in the first available PRACH occasion after  NT,2 + 0.25ms + TSSB from the beginning of time period T2. After transmitting PRACH on Cell 2, UE shall be back to Cell 1.

NOTE:	The PDCCH order RACH delay can be expressed as: NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation, where:

-	NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission and is specified in Table 6.4-1 in 38.214 [26].

-	TBWPswitchDelay= 0, Tswitch= 0, ∆RF/BB preparation= 0

-	∆Delay= 0.25ms

-	[TSSB is the time to first gap occasion after [UE decoding PDCCH-order]].

During T2, interruption on Cell 1 UL shall not happen outside the overlapped slot to transmit PRACH and N symbols from the last or first symbol of PRACH occasion as defined in clause 8.1 in 38.213 [3], where N=4. During T2, interruption on Cell 1 DL shall not occur outside the overlapped slot to transmit PRACH.

The test equipment will verify that the timing of PRACH transmission on Cell 2 is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB of Cell 2.

a.	The NTA_offset value (in Tc units) is 13792.

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1.

The rate of correct events observed during repeated tests shall be at least 90%.

##### A.7.3.2.x.2	PDCCH-order RACH on inter-frequency neighbor cell in FR2

A.7.3.2.x.2.1	Test Purpose and Environment

This test is to verify the requirement for PDCCH-order RACH on neighbour cell in FR2 when RACH bandwidth is outside any configured UL BWP specified in clause 8.1 in 38.213 [3], UE transmit timing in clause 7.1 and interruption in clause for UE supporting [RACH-based early TA acquisition], [RF/BB preparation time for PDCCH-order RACH] and [Interruption due to RF retuning for PDCCH- ordered RACH].

A.7.3.2.x.2.2	Test Parameters

In this test, there are two cells: NR Cell 1 as PCell in FR2 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR2 on NR RF channel 2. Test configurations are given in table A.7.3.2.x.2.2-1. Both PDCCH order RACH delay, transmit timing requirement and the interruption requirements are tested by using the parameters in table A.7.3.2.x.2.2-2, and A.7.3.2.x.2.2-3.

The test consists of two successive time periods, with time durations of T1 and T2 respectively.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A4 is used.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	The UE has reported L3 measurement results and performed SSB based L1-RSRP measurement on Cell 2.

T1 starts from UE transmitting a valid L1 report on Cell 2. After T1, test equipment sends PDCCH order to trigger RACH transmission. The start of T2 is the instant when PDCCH order to trigger PRACH transmission on Cell 2 is received.

Table A.7.3.2.x.2.2-1: PDCCH order RACH on inter-frequency neighbor cell in FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Candidate cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.x.2.2-2: General test parameters for PDCCH order RACH in FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 1 | After transmitting PRACH on Cell 2, UE shall be back to Cell 1. |
| a4-Threshold |  | dBm | -110 | Cell 2 |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | ms | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| includeBeamMeasurements |  |  | True |  |
| Gap Pattern Id |  |  | 13 | As specified in Table 9.1.2-1. |
| Measurement gap offset |  |  | 39 |  |
| DRX |  |  | OFF | DRX is not used |
| Time offset between cells |  |  | 0.3 s |  |
| deriveSSB-IndexFromCell |  |  | Enabled |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  | NR RF Channel Number 2 | Cell 2 |
|  | PRACH configuration |  | FR2 PRACH configuration 5 |  |
|  | bwp-GenericParameters |  | ULBWP.0.1 |  |
|  | n-TimingAdvanceOffset | Tc | N/A |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |
|  | spCellInclusion |  | N/A |  |
| ltm-ConfigComplete |  |  | True | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |
| T2 |  | s | 0.5 |  |

Table A.7.3.2.x.2.2-3: Cell specific test parameters for PDCCH order RACH test case

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.3.15 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 66 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | N/A |  |
| Control Channel RMC |  |  | Config 1 | CCR.3.1 TDD |  | N/A |  |
| SMTC configuration |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note 3 |  | dB | Config 1 | 5 | 5 | 5 | 5 |
| Note2 |  | dBm/15 kHz | Config 1 | -104.7 | -104.7 | -104.7 | -104.7 |
| Note2 |  | dBm/SCS | Config 1 | -95.7 | -95.7 | -95.7 | -95.7 |
| SSB_RP Note 3 |  | dBm/SCS Note5 | Config 1 | -90.7 | -90.7 | -90.7 | -90.7 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -60.5 | -60.5 | -60.5 | -60.5 |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

A.7.3.2.x.2.3 Test Requirements

The UE shall transmit the PRACH to Cell 2 in the first available PRACH occasion after  NT,2 + 0.25ms + TSSB + ∆RF/BB preparation from the beginning of time period T2. After transmitting PRACH on Cell 2, UE shall be back to Cell 1.

NOTE:	The PDCCH order RACH delay can be expressed as: NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation, where:

-	NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission and is specified in Table 6.4-1 in 38.214 [26].

-	TBWPswitchDelay= 0, Tswitch= 0

-    ∆RF/BB preparation is reported in [UE capability of RF/BB preparation time for PDCCH-order RACH]

-	∆Delay= 0.25ms

-	[TSSB is the time to first gap occasion after [UE decoding PDCCH-order]].

During T2, interruption on Cell 1 shall not happen outside ceil (Y/NR Slot length) +1 slots before and after PRACH transmission and the same slot of PRACH, where Y as reported in [UE capability xx],

The test equipment will verify that the timing of PRACH transmission on Cell 2 is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB of Cell 2.

a.	The NTA_offset value (in Tc units) is 13792.

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 7.1.2-1.

The rate of correct events observed during repeated tests shall be at least 90%.

End of Change 25

Start of Change 26

A.7.3.3.X	NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC

A.7.3.3.X.1	Test Purpose and Environment

This test is to verify the requirement for the requirements of CHO including target MCG and target SCG in NR-DC requirements specified in clause 6.1.6.2. inter-frequency conditional handover from NR FR1 to NR FR1 and intra-frequency PSCell change from NR-FR2 to NR FR2 are tested independently in the same test, with different end points.

The supported test configurations are given in Table A.7.3.3.X.1-1. The test scenario comprises four NR cells, source PCell(Cell 1) and source PSCell(Cell 2), target PCell(Cell 3), target PSCell(Cell 4).

Cell 1 and Cell 3 are on radio channel 1 in FR1.Cell 2 and Cell 4 are on radio channel 2 in FR2. Test parameters are given in Tables A.7.3.3.X.1-2, A.7.3.3.X.1-3, A.7.3.3.X.1-4 and A.7.3.3.X.1-5 below. The test consists of two successive time periods, with time durations of T1, T2 respectively. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. NR shall configure a message implying conditional handover including target MCG in FR1 and target SCG in FR2 to Cell 3 during T1, at a time earlier than TRRC before the beginning of T2.  At the start of T2, cell 2 becomes detectable and meets the handover condition.

Table A.7.3.3.X.1-1: Supported test configurations for CHO with PSCell change from NR-DC to NR-DC

| Config | Description |
| --- | --- |
| 1 | Source PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Source PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode Target PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | Source PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Source PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode Target PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 3 | Source PCell: FR1 NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target PCell: FR1 NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Source PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode Target PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.3.3.X.1-2: General test parameters for PCell FR1-FR1 Inter frequency conditional handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 3 |  |
| Final condition | Active cell |  | Cell 3 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.7.3.3.X.1-3: Cell specific test parameters for PCell FR1-FR1 Inter frequency conditional handover

| Parameter |  |  | Unit | Cell 1 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 2 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  | -98 |  |
|  | Config 3 |  |  | -95 |  | -95 |  |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
|  |  |  | dB | 4 | 4 | -Infinity | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -Infinity | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -Infinity | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -63.94 | -57.75 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.7.3.3.X.1-4: General test parameters Intra-frequency FR2-FR2 PSCell change

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 2 |  |
|  | Neighbouring cell |  | Cell 4 |  |
| Final condition | Active cell |  | Cell 4 |  |
| A4-Offset |  | dBm | -120 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

Table A.7.3.3.X.1-5: Cell specific test parameters for Intra-frequency FR2-FR2 PSCell change

| Parameter |  |  | Unit | Cell 2 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  | Rough |  |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |  |  |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  |
| Duplex mode |  |  |  | TDD |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |
| Data RBs allocated |  |  |  | 66 |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | O P. 1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |
| SSB Configuration |  |  |  | SSB. 3 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.2 |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |  |  |
| Note2 |  |  | dBm/SCS | -95.7 |  |  |  |
|  |  |  | dB | 6 | -1.8 | -Infinity | 0 |
|  |  |  | dB | 6 | 6 | -Infinity | 7 |
| IoNote3 |  |  | dBm/ BW | -59.7 | -56.7 | -59.7 | -56.7 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

A.7.3.3.X.2	Test Requirements

A.7.3.3.X.2.1	Test Requirements for NR conditional handover

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 3 less than Tmeasure + Tinterrupt + TCHO_execution = 6720+47+10ms=6777 ms (power class 1) or 4160+47+10ms =4217ms (power classes 2,3 and 4) from the start of T2 and the interruption during T2 shall not exceeed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin =25+20+2 = 47ms excluding any transmissions which do not occur due to scheduling restrictions. excluding any transmissions which do not occur due to scheduling restrictions.

The rate of correct handovers observed during repeated tests shall be at least 90%.

A.7.3.3.X.2.1	Test Requirements for NR PSCell change

The UE shall start to transmit the PRACH to Cell 4 less than Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 =6720+10+25+20+10ms+2=6792 ms (power class 1) or 4160+25+20+10+2ms =4232ms (power classes 2,3 and 4) from the start of T2

End of Change 26

Start of Change 27

A.7.3.3.Y	NR conditional Handover including target MCG and candidate SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC

A.7.3.3.Y.1	Test Purpose and Environment

This test is to verify the requirements for conditional handover including target MCG and candidate SCG in NR-DC: from FR1-FR2 NR-DC to FR1-FR2 NR-DC specified in clause 6.1.7.2. This test verifies the requirements for inter-frequency FR1-FR1 conditional handover and intra-frequency FR2-FR2 conditional PSCell change.

A.7.3.3.Y.2	Test Parameters

The supported test configurations are given in Table A.7.3.3.Y.1-1. The test scenario comprises four NR cells, source PCell (Cell 1) and source PSCell (Cell 2), target PCell (Cell 3), and target PSCell (Cell 4).

Cell 1 is on radio channel 1 in FR1. Cell 3 is on radio channel 2 in FR1, Cell 2 and Cell 4 are on radio channel 3 in FR2. The event-triggered reporting with Event A3 is used for handover condition. Test parameters are given in Tables A.7.3.3.Y.1-2, A.7.3.3.Y.1-3, and A.7.3.3.Y.1-4 below. The test consists of three successive time periods, with time durations of T1, T2 and T3, respectively.

At the start of T1, the UE shall be connected to Cell 1 on radio channel 1 and Cell 2 on radio channel 3. UE is not aware of Cell 3 and Cell 4. TE shall configure a condition implying conditional handover to Cell 3 with a condition implying conditional PSCell change to cell 4 during T1, at a time earlier than TRRC before the beginning of T2.

At the start of T2, cell 3 becomes detectable. At the start of T3, cell 4 becomes detectable. The condition for conditional handover and the condition for conditional PSCell change is met during T3.

Table A.7.3.3.Y.1-1: Supported test configurations for Conditional Handover including target MCG and candidate SCG from NR-DC to NR-DC

| Config | Description |
| --- | --- |
| 1 | Source PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Source PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode Target PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | Source PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target PCell: FR1 NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Source PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode Target PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 3 | Source PCell: FR1 NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target PCell: FR1 NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Source PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode Target PSCell: FR2 NR 120kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.3.3.Y.1-2: General test parameters for FR1-FR1 inter frequency conditional handover with target MCG and FR2-FR2 intra-frequency conditional PSCell change with candidate SCG

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1, 2 |  |
|  | Target cell |  | Cell 3, 4 |  |
| Final condition | Active cell |  | Cell 3, 4 |  |
| A3-Offset for handover condition |  | dBm | -4 | Applied for both CHO and CPC. |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 1 |  |
| T2 |  | s | 2 | It is the time gap between target Pcell and target PScell become detectable |
| T3 |  | s | 2 |  |

Table A.7.3.3.Y.1-3: Cell specific test parameters for PCell FR1-FR1 Inter frequency handover with target MCG

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 |  | T2 | T3 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  | -95 |  |  |  |
|  |  |  | dB | 4 | 4 | 4 | -Infinity |  | 5 | 5 |
|  |  |  | dB | 4 | 4 | 4 | -Infinity |  | 5 | 5 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 | -94 | -Infinity |  | -93 | -93 |
|  | Config 3 |  | dBm/SCS | -91 | -91 | -91 | -Infinity |  | -90 | -90 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 | -64.59 | -70.05 |  | -63.85 | -63.85 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 | -58.49 | -63.94 |  | -57.75 | -57.75 |
| Propagation condition |  |  | - | AWGN |  |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

Table A.7.3.3.Y.1-4: Cell specific test parameters for conditionaly intra-frequency FR2-FR2 PSCell change with candidate SCG

| Parameter |  |  | Unit | Cell 2 |  |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 |  | T2 | T3 |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  |  |  | Rough |  |  |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |  |  |  |  |  |
| NR RF Channel Number |  |  |  | 3 |  |  |  | 3 |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |
| Data RBs allocated |  |  |  | 66 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |  |  |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | O P. 1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |  |
| SSB Configuration |  |  |  | SSB. 3 FR2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |  |  |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.2 |  |  |  |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |  |  |  |  |  |
| Note2 |  |  | dBm/SCS | -95.7 |  |  |  |  |  |  |
|  |  |  | dB | 5.03 | 5.03 | -5.41 | -Infinity |  | -Infinity | 3.81 |
|  |  |  | dB | 6 | 6 | 6 | -Infinity |  | -Infinity | 11 |
| IoNote3 |  |  | dBm/ BW | -59.7 | -59.7 | -54.2 | -59.7 |  | -59.7 | -54.2 |
| Propagation condition |  |  | - | AWGN |  |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |  |

A.7.3.3.Y.3 Test Requirements

TRRC + TEvent_DU occurs during T1 and T2, as the conditional handover condition for cell 3 becomes satisfied from the start of T2, and the conditional PSCell change condition for cell 4 becomes satisfied from the start of T3. The test shall verify that there are no interruptions during T1 and T2. The UE shall not start the transmission of the new uplink PRACH channel of the target PCell before T3.

In this test, the UE shall start to transmit the PRACH to Cell 3 less than 1677 ms (power class 1) or 1157 ms (power classes 2,3 and 4)Note1 from the beginning of time period T3.

The UE shall transmit the PRACH to Cell 4 at latest 1677 ms (power class 1) or 1157 ms (power classes 2,3 and 4)Note2 from the beginning of time period T3.

The rate of correct observed delay in conditional handover including target MCG and candidate SCG during repeated tests shall be at least 90%.

Note 1:	The PCell conditional handover delay can be expressed as specified in clause 6.1.7.2.1:

DCHOwithCPC_PCell = TRRC_delay + TEvent_DU + max (Tmeasure_PCell, Tmeasure_PSCell) + TUE_preparation + Tprocessing + TΔ_PCell + TPCell_DU + 2 ms,

Where:

max (Tmeasure_PCell, Tmeasure_PSCell) = 1600 ms (power class 1) or 1080 ms (power classes 2, 3 and 4)

TUE_preparation = 10ms

Tprocessing = 25ms

TΔ_PSCell = 20ms

TPCell_ DU = 20ms

Note 2: The PSCell conditional change delay can be expressed as follows as specified in clause 6.1.7.2.2:

DCHOwithCPC_PSCell = TRRC_delay + TEvent_DU + max (Tmeasure_PCell, Tmeasure_PSCell) + TUE_preparation + Tprocessing + TΔ_PSCell + TPSCell_DU + 2 ms

Where:

max (Tmeasure_PCell, Tmeasure_PSCell) = 1600 ms (power class 1) or 1080 ms (power classes 2, 3 and 4)

TUE_preparation = 10ms

Tprocessing = 25ms

TΔ_PSCell = 20ms

TPSCell_ DU = 20ms

End of Change 27

Start of Change 28

A.7.3.x	LTM PCell Switch

A.7.3.x.1	RACH based Intra-frequency PCell switch from FR2 to FR2

A.7.3.x.1.1	Test Purpose and Environment

This test is to verify the intra frequency RACH based LTM PCell switch requirements from NR FR2 to NR FR2 specified in clause 6.3.1 for both with and without early TCI state activation.

A.7.3.x.1.2	Test Parameters

Two cells are deployed in the test, which are FR2 PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. Test configurations are given in table A.7.3.x.1.2-1. Both cell switch delay and interruption length are tested by using the parameters in table A.7.3.x.1.2-2 and A.7.3.x.1.2-3.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

The test consists of four successive time periods, with time durations of T1, T2, T3 and T4, respectively. No gap patterns are configured in the test case.

During T1, for Test 1A, 1B, 2A and 2B:

-	A measurement object is configured for the frequency of the Cell 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting an L3 measurement result of Cell 2 to Cell 1.

During T2, for Test 1A, 1B, 2A and 2B:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 2

-	Joint TCI state configuration as defined in Table A.7.3.x.1.2-2 for Test 1A and Test 2A are provided.

-	Separate TCI state configuration as defined in Table A.7.3.x.1.2-2 for Test 1B and Test 2B are provided.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	T2 ends with UE reporting a valid L1-RSRP result of Cell 2.

During T3, for Test 1A and 1B:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 2.

-	In Test 1A, CandidateTCI-State#1 is activated.

-	In Test 1B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 is activated.

-	T3 ends 50 ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2A and 2B, T3 is skipped.

During T4, for Test 1A, 1B, 2A and 2B:

-	The start of T4 is the instant when the last TTI containing LTM cell switch command MAC CE is sent by Cell 1 to the UE.

-	In the cell switch command, Cell 2 is the target cell. Contention-Free Random-Access Resources are indicated. The field of Timing Advance Command is set to FFF.

-	In test 1A, CandidateTCI-State#2 is indicated.

-	In test 1B, CandidateTCI-State#2 and CandidateTCI-UL-State#1 are indicated.

-	In test 2A, CandidateTCI-State#1 is indicated.

-	In test 2B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are indicated.

-	T4 ends upon the reception of PRACH at Cell 2.

Table A.7.3.x.1.2-1: Intra-frequency cell switch from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.x.1.2-2: General test parameters for Intra-frequency cell switch from FR2 to FR2

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 2 |  |  |  |  |
| A3-Offset |  | dB | -30 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 0.3 s |  |  |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DlorJoint TCI.State.0 | DlorJoint TCI.State.2 | DlorJoint TCI.State.1 | DlorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | DlorJoint TCI.State.1 | DlorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.1 |  |  |  |  |
| T4 |  | s | 0.2 |  |  |  |  |

Table A.7.3.x.1.2-3: Cell specific test parameters for NR FR2-FR2 Intra frequency cell switch test case

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 ~ T4 | T1 ~ T4 |
| NR RF Channel Number |  |  |  | 1 | 1 |
| Assumption for UE beamsNote 6 |  |  |  | Rough | Rough |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |
| Duplex mode |  |  |  | TDD |  |
| TDD configuration |  |  |  | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |
| PDSCH Reference |  |  |  | SR3.1 TDD |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |
| CP length |  |  |  | Normal |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 6 |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.2 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |
| Note2 |  |  | dBm/SCS | -95.7 |  |
|  |  |  | dB | -1.8 | 0 |
|  |  |  | dB | 6 | 7 |
| SSB_RP |  |  | dBm/SCS | -89.7 | -88.7 |
| IoNote3 |  |  | dBm/ 95.04MHz | -56.7 | -56.7 |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

A.7.3.x.1.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 in no later than DLTM from the beginning of time period T4.

The rate of correct cell switches observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-interrupt), where:

Tcmd = THARQ + 3 ms and is specified in clause 6.3.1.2,

TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms, as stated in section 6.3.1.3

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B

-	TLTM-IU = 20 ms

-	TLTM-RRC-processing = 10 ms if UE does not support [Early processing of an LTM candidate cell RRC configuration], otherwise TLTM-RRC-processing =0ms

-	TLTM-processing = 10 ms if the UE supports [faster LTM processing] capability and UE reports 10 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing = 15 ms if the UE supports [faster LTM processing] capability and UE reports 15 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing = 20 ms if the UE does not support [faster LTM processing] capability.

A.7.3.x.2	RACH-less Intra-frequency PCell switch from FR2 to FR2

A.7.3.x.2.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 RACH-less intra frequency PCell switch specified in clause 6.3.1 for both with and without early TCI state activation.

A.7.3.x.2.2	Test Parameters

Two cells are deployed in the test, which are FR2 PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. Test configurations are given in table A.7.3.x.2.2-1. Both cell switch delay and interruption length are tested by using the parameters in table A.7.3.x.2.2-2 and A.7.3.x.2.2-3.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

The test consists of five successive time periods, with time durations of T1, T2, T3, T4 and T5, respectively. No gap patterns are configured in the test case.

During T1, for Test 1A, 1B, 2A and 2B:

-	A measurement object is configured for the frequency of the Cell 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting an L3 measurement result of Cell 2 to Cell 1.

During T2, for Test 1A, 1B, 2A and 2B:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 2

-	Joint TCI state configuration as defined in Table A.7.3.x.2.2-2 for Test 1A and Test 2A are provided.

-	Separate TCI state configuration as defined in Table A.7.3.x.2.2-2 for Test 1B and Test 2B are provided.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	T2 ends with UE reporting a valid L1-RSRP result of Cell 2.

During T3, for Test 1A and 1B:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 2.

-	In Test 1A, CandidateTCI-State#1 is activated.

-	In Test 1B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 is activated.

-	T3 ends 50ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2A and 2B, T3 is skipped.

During T4, for Test 1A, 1B, 2A and 2B:

-	At the start of T4, UE receives PDCCH order to trigger PRACH transmission on Cell 2.

-	T4 ends 5ms after the UE transmits the PRACH to Cell 2.

-	For UE incapable of rach-EarlyTA-Measurement-r18, T4 is skipped.

During T5, for Test 1A, 1B, 2A and 2B:

-	The start of T5 is the last TTI containing LTM cell switch command MAC CE is sent by Cell 1 to the UE.

-	In the cell switch command, Cell 2 is the target cell and the field of Timing Advance Command is set to 0.

-	In test 1A, CandidateTCI-State#2 is indicated.

-	In test 1B, CandidateTCI-State#2 and CandidateTCI-UL-State#1 are indicated.

-	In test 2A, CandidateTCI-State#1 is indicated.

-	In test 2B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are indicated.

-	Cell 2 continuously schedules PUSCH for the UE.

-	T5 ends either at the UL slot of PUSCH scheduled by Cell 2 at the fist DL slot not earlier than (Tcmd + TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc) after the beginning of T5 or upon the reception of PUSCH at Cell 2, whichever is earlier.

-	The values of Tcmd, TLTM-RRC-processing TLTM-processing, Tfirst-RS and TRS-proc are specified in A.7.3.x.2.3.

Table A.7.3.x.2.2-1: Intra-frequency cell switch from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.x.2.2-2: General test parameters for Intra-frequency cell switch from FR2 to FR2

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 2 |  |  |  |  |
| A3-Offset |  | dB | -30 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 0.3 s |  |  |  | RTD between cells is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| EarlyUL-SyncConfig | frequencyInfoUL |  | NR RF Channel Number 1 |  |  |  | Same as Cell 1 |
|  | PRACH configuration |  | FR2 PRACH configuration 5 |  |  |  | RACH bandwidth is within active UL BWP of Cell 1 |
|  | bwp-GenericParameters |  | ULBWP.0.1 |  |  |  |  |
|  | n-TimingAdvanceOffset | Tc | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DlorJoint TCI.State.0 | DlorJoint TCI.State.2 | DlorJoint TCI.State.1 | DlorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | DlorJoint TCI.State.1 | DlorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.1 |  |  |  |  |
| T4 |  | s | 0.2 |  |  |  |  |
| T5 |  | s | 0.1 |  |  |  |  |

Table A.7.3.x.2.2-3: Cell specific test parameters for NR FR2-FR2 Intra frequency cell switch test case

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 ~ T5 | T1 ~ T5 |
| NR RF Channel Number |  |  |  | 1 | 1 |
| Assumption for UE beamsNote 6 |  |  |  | Rough | Rough |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |
| Duplex mode |  |  |  | TDD |  |
| TDD configuration |  |  |  | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |
| PDSCH Reference |  |  |  | SR3.1 TDD |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |
| CP length |  |  |  | Normal |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 6 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |
| Note2 |  |  | dBm/SCS | -95.7 |  |
|  |  |  | dB | -1.8 | 0 |
|  |  |  | dB | 6 | 7 |
| SSB_RP |  |  | dBm/SCS | -88.7 | -89.7 |
| IoNote3 |  |  | dBm/ 95.04MHz | -56.7 | -56.7 |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

A.7.3.x.2.3 Test Requirements

The UE shall start to transmit PUSCH to Cell 2 in no later than DLTM from the beginning of time period T5.

The rate of correct cell switches observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-interrupt), where:

Tcmd = THARQ + 3 ms and is specified in clause 6.3.1.2, TLTM-interrupt is defined in clause 6.3.1.3 as TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU,

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B,

-	TLTM-IU is the uncertainty on transmitting the first uplink transmission on Cell 2.

-	TLTM-RRC-processing = 10 ms if UE does not support [Early processing of an LTM candidate cell RRC configuration], otherwise TLTM-RRC-processing =0ms

-	TLTM-processing = 10 ms if the UE supports [faster LTM processing] capability and UE reports 10 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing = 15 ms if the UE supports [faster LTM processing] capability and UE reports 15 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing = 20 ms if the UE does not support [faster LTM processing] capability.

A.7.3.x.3	RACH-based Inter-frequency LTM PCell switch from FR2 to FR2

A.7.3.x.3.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 RACH-based inter-frequency PCell switch specified in clause 6.3.1 for both with and without early TCI state activation.

A.7.3.x.3.2	Test Parameters

Two cells are deployed in the test, which are FR2 PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on a different frequency than the PCell. Test configurations are given in table A.7.3.x.3.2-1. Both cell switch delay and interruption length are tested by using the parameters in table A.7.3.x.3.2-2 and A.7.3.x.3.2-3.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

The test consists of five successive time periods, with time durations of T1, T2, T3, and T4, respectively. Measurement gap pattern gp0 is configured.

During T1, for Test 1A, 1B,2A and 2B:

-	A measurement object is configured for the frequency of the Cell 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting an L3 measurement result of Cell 2 to Cell 1.

During T2, for Test 1A, 1B, 2A and 2B:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 2

-	Joint TCI state configuration as defined in Table A.7.3.x.3.2-2 for Test 1A and Test 2A are provided.

-	Separate TCI state configuration as defined in Table A.7.3.x.3.2-2 for Test 1B and Test 2B are provided.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	T2 ends with UE reporting a valid L1-RSRP result of Cell 2.

During T3, for Test 1A and 1B:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 2.

-	In Test 1A, CandidateTCI-State#1 is activated.

-	In Test 1B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 is activated.

-	T3 ends 100 ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2A and 2B, T3 is skipped.

During T4, for Test 1A, 1B, 2A and 2B:

-	The start of T4 is the last TTI containing LTM cell switch command MAC CE is sent by Cell 1 to the UE.

-	In the cell switch command, Cell 2 is the target cell and the field of Timing Advance Command is set to 0.

-	In test 1A, CandidateTCI-State#2 is indicated.

-	In test 1B, CandidateTCI-State#2 and CandidateTCI-UL-State#1 are indicated.

-	In test 2A, CandidateTCI-State#1 is indicated.

-	In test 2B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are indicated.

-	Cell 2 continuously schedules PUSCH for the UE.

-	T4 ends upon the reception of PRACH at Cell 2.

Table A.7.3.x.3.2-1: Inter-frequency cell switch from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.x.3.2-2: General test parameters for Inter-frequency cell switch from FR2 to FR2

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cell |  | Cell 1 |  |  |  |  |
|  | Neighbouring cell |  | Cell 2 |  |  |  | Cell 2 is the candidate cell |
| Final condition | Active cell |  | Cell 2 |  |  |  |  |
| A3-Offset |  | dB | -30 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| includeBeamMeasurements |  |  | True |  |  |  |  |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Measurement gap pattern ID |  |  | gp0 |  |  |  | As specified in Table 9.1.2-1 |
| Measurement gap offset |  |  | 39 |  |  |  |  |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between cells |  |  | 0.3 s |  |  |  |  |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DlorJoint TCI.State.0 | DlorJoint TCI.State.2 | DlorJoint TCI.State.1 | DlorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | DlorJoint TCI.State.1 | DlorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.2 |  |  |  |  |
| T4 |  | s | 0.1 |  |  |  |  |

Table A.7.3.x.3.2-3: Cell specific test parameters for NR FR2-FR2 Inter frequency cell switch test case

| Parameter |  |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 ~ T5 | T1 ~ T5 |
| NR RF Channel Number |  |  |  | 1 | 2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough | Rough |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |
| Duplex mode |  |  |  | TDD |  |
| TDD configuration |  |  |  | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |
| PDSCH Reference |  |  |  | SR3.1 TDD |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |
| CP length |  |  |  | Normal |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 6 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |
| Note2 |  |  | dBm/SCS | -95.7 |  |
|  |  |  | dB | 5 | 5 |
|  |  |  | dB | 5 | 5 |
| SSB_RP |  |  | dBm/SCS | -90.7 | -90.7 |
| IoNote3 |  |  | dBm/ 95.04MHz | -60.5 | -60.5 |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

A.7.3.x.3.3 Test Requirements

The UE shall start to transmit PUSCH to Cell 2 in no later than DLTM from the beginning of time period T4.

The rate of correct cell switches observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-interrupt), where:

Tcmd = THARQ + 3 ms and is specified in clause 6.3.1.2, TLTM-interrupt is defined in clause 6.3.1.3 as TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU,

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B,

-	TLTM-IU = 20 ms.

-	TLTM-RRC-processing = 10 ms if UE does not support [Early processing of an LTM candidate cell RRC configuration], otherwise TLTM-RRC-processing =0ms

-	TLTM-processing = 10 ms if the UE supports [faster LTM processing] capability and UE reports 10 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing = 15 ms if the UE supports [faster LTM processing] capability and UE reports 15 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing = 20 ms if the UE does not support [faster LTM processing] capability.

End of Change 28

Start of Change 29

A.7.3.x1	LTM PSCell Switch

A.7.3.x1.1	RACH-based Intra-frequency LTM PSCell switch from FR2 to FR2

A.7.3.x1.1.1	Test Purpose and Environment

This test is to verify RACH-based LTM PSCell Switch requirements for the NR FR2-NR FR2 intra frequency cell switch specified in clause 8.20 for both with and without early TCI state activation.

A.7.3.x1.1.2	Test Parameters

Three cells are deployed in the test, which are FR1 PCell (Cell 1), source FR2 PSCell (Cell 2) and target FR2 PSCell (Cell 3) on the same frequency as the PSCell. Test configurations are given in table A.7.3.x1.1.2-1. Both PSCell cell switch delay and interruption length are tested by using the parameters in table A.7.3.x1.1.2-2, A.7.3.x1.1.2-3 and A.7.3.x1.1.2-4.

The test consists of 4 tests, and UE is required to pass one among Test 1A, Test 1B, Test 2A and Test 2B.

-	Test 1: for a UE supporting ltm-MAC-CE-JointTCI-r18 and/or ltm-MAC-CE-SeparateTCI-r18

-	Test 1A: for a UE supporting ltm-MAC-CE-JointTCI-r18.

-	Test 1B: for a UE supporting ltm-MAC-CE-SeparateTCI-r18 and does not support ltm-MAC-CE-JointTCI-r18.

-	Test 2: for a UE not supporting ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18

-	Test 2A: for a UE supporting ltm-BeamIndicationJointTCI-r18.

-	Test 2B: for a UE supporting ltm-BeamIndicationSeparateTCI-r18 and does not support ltm-BeamIndicationJointTCI-r18.

The test consists of four successive time periods, with time durations of T1, T2, T3 and T4, respectively. No gap patterns are configured in the test case.

During T1, for Test 1A, 1B, 2A and 2B:

-	A measurement object is configured for the frequency of the Cell 3, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

-	T1 ends with UE reporting an L3 measurement result of Cell 3 to Cell 1.

During T2, for Test 1A, 1B, 2A and 2B:

-	At the start of T2, UE is provided with LTM-Candidate-r18 for Cell 3

-	Joint TCI state configuration as defined in Table A.7.3.x1.1.2-2 for Test 1A and Test 2A are provided.

-	Separate TCI state configuration as defined in Table A.7.3.x1.1.2-2 for Test 1B and Test 2B are provided.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 3) in PUCCH format 2.

-	T2 ends with UE reporting a valid L1-RSRP result of Cell 3.

During T3, for Test 1A and 1B:

-	At the start of T3, UE receives candidate cell TCI state activation MAC CE for Cell 3.

-	In Test 1A, CandidateTCI-State#1 is activated.

-	In Test 1B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 is activated.

-	T3 ends 50 ms after the candidate cell TCI state activation MAC CE transmission.

-	In Test 2A and 2B, T3 is skipped.

During T4, for Test 1A, 1B, 2A and 2B:

-	The start of T4 is the instant when the last TTI containing LTM cell switch command MAC CE is sent by Cell 2 to the UE.

-	In the cell switch command, Cell 3 is the target cell for PSCell switch. Contention-Free Random-Access Resources are indicated and the field of Timing Advance Command is set to FFF.

-	In test 1A, CandidateTCI-State#2 is indicated.

-	In test 1B, CandidateTCI-State#2 and CandidateTCI-UL-State#1 are indicated.

-	In test 2A, CandidateTCI-State#1 is indicated.

-	In test 2B, CandidateTCI-State#1 and CandidateTCI-UL-State#1 are indicated.

-	T4 ends upon the reception of PRACH at Cell 3.

Table A.7.3.x1.1.2-1: Intra-frequency PSCell switch from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Pcell: FR1 NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode Source Pscell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target Pscell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | Pcell: FR1 NR 15kHz SSB SCS, 10MHz bandwidth, TDD duplex mode Source Pscell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target Pscell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | Pcell: FR1 NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Source Pscell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target Pscell: FR2 NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table A.7.3.x1.1.2-2: General test parameters for Intra-frequency cell switch from FR2 to FR2

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1A | Test 1B | Test 2A | Test 2B |  |
| Initial conditions | Active cell |  | Cell 1, Cell 2 |  |  |  |  |
|  | Neighbouring cell |  | Cell 3 |  |  |  | Cell 3 is the candidate cell |
| Final condition | Active cell |  | Cell 1, Cell 3 |  |  |  |  |
| A3-Offset |  | dB | -30 |  |  |  |  |
| Hysteresis |  | dB | 0 |  |  |  |  |
| Time To Trigger |  | s | 0 |  |  |  |  |
| Filter coefficient |  |  | 0 |  |  |  | L3 filtering is not used |
| DRX |  |  | OFF |  |  |  | DRX is not used |
| Access Barring Information |  | - | Not Sent |  |  |  | No additional delays in random access procedure. |
| Time offset between Cell 2 and Cell 3 |  |  | 0.3 s |  |  |  | RTD between Cell 2 and Cell 3 is less than CP |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 |  |  |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  |  |  | Report candidate cell’s (Cell 3) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |  |  |
|  | spCellInclusion |  | N/A |  |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList | CandidateTCI-State#1 |  | DlorJoint TCI.State.0 | DLorJoint TCI.State.2 | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | As specified in clause A.3.16B. In test 1A and 1B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for early TCI state activation. CandidateTCI-State#2 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. In test 2A and 2B, CandidateTCI-State#1 and/or CandidateTCI-UL-State#1 are configured for TCI state indication in cell switch command. |
|  | CandidateTCI-State#2 |  | DLorJoint TCI.State.1 | DLorJoint TCI.State.3 | N/A | N/A |  |
| ltm-UL-TCI-StatesToAddModList | CandidateTCI-UL-State#1 |  | N/A | UL TCI.State.0 | N/A | UL TCI.State.0 |  |
| ltm-ConfigComplete |  |  | True |  |  |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | <3 |  |  |  |  |
| T2 |  | s | 0.2 |  |  |  |  |
| T3 |  | s | 0.1 |  |  |  |  |
| T4 |  | s | 0.1 |  |  |  |  |

Table A.7.3.Y.1.2-3: Cell specific test parameters for PCell (Cell 1)

| Parameter |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- |
|  |  |  |  | T1~T4 |
| NR RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2,3 |  | TDD |
| TDD configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 40: NRB,c = 106 |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 40: NRB,c = 106 |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |
|  |  | Config 2 |  | TRS.1.1 TDD |
|  |  | Config 3 |  | TRS.1.2 TDD |
| DRx Cycle |  |  | ms | Not Applicable |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.2.1 TDD |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |
|  |  | Config 3 |  | 30 |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 |
|  |  | Config 3 |  | 30 |
| PRACH configuration |  |  |  | FR1 PRACH configuration 6 |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |
|  |  | Initial UL BWP |  | ULBWP.0.1 |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |
|  | Config 3 |  |  |  |
|  |  |  | dB | 4 |
|  |  |  | dB | 4 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 |
|  | Config 3 |  |  | -91 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 |
| Propagation condition |  |  | - | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.7.3.x.1.2-4: Cell specific test parameters for NR FR2-FR2 Intra frequency cell switch test case

| Parameter |  |  | Unit | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 ~ T4 | T1 ~ T4 |
| NR RF Channel Number |  |  |  | 2 | 2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough | Rough |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |
| Duplex mode |  |  |  | TDD |  |
| TDD configuration |  |  |  | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |
| PDSCH Reference |  |  |  | SR3.1 TDD |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |
| CP length |  |  |  | Normal |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 6 |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -104.7 |  |
| Note2 |  |  | dBm/SCS | -95.7 |  |
|  |  |  | dB | -1.8 | 0 |
|  |  |  | dB | 6 | 7 |
| SSB_RP |  |  | dBm/SCS | -88.7 | -89.7 |
| IoNote3 |  |  | dBm/ 95.04MHz | -56.7 | -56.7 |
| Propagation condition |  |  | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

A.7.3.x.1.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 3 in no later than DLTM from the beginning of time period T4.

The rate of correct cell switches observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as DLTM (= Tcmd + TLTM-interrupt), where:

Tcmd = THARQ + 3 ms and is specified in clause 6.3.1.2, TLTM-interrupt is defined in clause 8.20.3 as TLTM-interrupt  = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU ms.

-	Tfirst-RS + TRS-proc= 0 ms for Test 1A and 1B, Tfirst-RS + TRS-proc= 22 ms for Test 2A and 2B

-	TLTM-IU=20ms,

-	TLTM-RRC-processing = 10 ms if UE does not support [Early processing of an LTM candidate cell RRC configuration], otherwise TLTM-RRC-processing =0ms

-	TLTM-processing = 10 ms if the UE supports [faster LTM processing] capability and UE reports 10 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing = 15 ms if the UE supports [faster LTM processing] capability and UE reports 15 ms for FR2-to-FR2 cell switch in the capability

-	TLTM-processing = 20 ms if the UE does not support [faster LTM processing] capability.

End of Change 29

Start of Change 30

A.7.5.X	Subsequent conditional PSCell addition/change

A.7.5.X.1	Intra-frequency subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC

A.7.5.X.1.1	Test purpose and environment

The purpose of this test is to verify that the subsequent conditional NR PSCell change under NR-DC is within the requirements stated in clause 8.11E.2.

For UE supporting subsequent conditional PSCell addition/change, UE only needs to pass either intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC defined in clause or intra-frequency CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC defined in clause A.6.5.X.1.

For UE which can pass this test, test of conditional PSCell addition and release delay defined in [A.7.5.12] can be skipped.

Supported test configurations are shown in A.7.5.X.1.1-1. The test scenario comprises three NR cells, Cell 1, Cell 2 and Cell 3. Cell1 is on radio channel 1 in FR1. Cell 2 and Cell 3 are on radio channel 2 in FR2. The test parameters for the NR Cell1 are given in Table A.3.7A. The NR Cell 1 once set up is not changed across time. The test parameters for NR Cell 2 and Cell 3 are given in Tables A.7.5.X.1.1-2, cell-specific parameters in A.7.5.X.1.1-3 and OTA parameters in A.7.5.X.1.1-4 below.

The test consists of three successive time periods with duration of T1, T2, and T3 respectively. There are two carriers each with one cell. Before the test starts the UE is connected to Cell 1 (NR PCell) on radio channel 1 (PCC) but is not aware of Cell 2 (NR PSCell 1) and Cell 3 (NR PSCell 2) on radio channel 2. The UE is only monitoring the PCC. During T1 only Cell 1 is known to the UE.

At the start of time duration T1, the UE does not have any timing information of Cell 2. The TE shall configure subsequent conditional PSCell addition/change with Cell 2 and Cell 3 as target PSCells during T1, at a time earlier than TRRC_delay before the beginning of T2.

At the start of T2, Cell 2 becomes detectable and meets the PSCell addition condition. UE shall be able to measure and detect that the condition is fulfilled, after which it will transmit the PRACH preamble to Cell 2 during T2.

Upon PSCell addition complete (UE transmits SN RRCReconfigurationcomplete message), T3 starts. At the start of T3, Cell 3 becomes detectable and meets the PSCell change condition. UE shall be able to measure and detect that the condition is fulfilled, after which it will transmit the PRACH preamble to Cell 3 during T3.

Table A.7.5.X.1.1-1: Supported test configurations for Intra-frequency Subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC

| Configuration | Description |
| --- | --- |
| 1 | PCell (Cell 1): NR FDD, 15 kHz SSB SCS, 10 MHz bandwidth PSCell (Cell 2 and Cell 3): NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz bandwidth |
| 2 | PCell (Cell 1): NR TDD, 15 kHz SSB SCS, 10 MHz bandwidth PSCell (Cell 2 and Cell 3): NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz bandwidth |
| 3 | PCell (Cell 1): NR TDD, 30 kHz SSB SCS, 40 MHz bandwidth PSCell (Cell 2 and Cell 3): NR TDD, SSB SCS 240 kHz, data SCS 120 kHz, BW 100 MHz bandwidth |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.5.X.1.1-2: General Test Parameters for subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Three radio channels are used for this test. One for NR Cell1, second for NR Cell2 and Cell3 |
| Initial | Active PCell |  | Cell1, Cell2 | Cell1: NR PCell on RF channel number 1. Cell2: NR PSCell on RF channel number 2 |
| Condition | Neighbour cell |  | Cell3 | Neighbour cell on RF channel number 2. |
| Final | Active PCell |  | Cell1, Cell3 | Cell1: PCell on RF channel number 1. Cell3:NR PSCell on RF channel number 2 |
| Condition | Neighbour Cell |  | Cell2 | PSCell changed on RF channel number 2. |
| A4 | Hysteresis | dB | 0 | Used to trigger conditional PSCell addition of Cell 2 |
|  | Threshold RSRP | dBm | -118 |  |
|  | Time to Trigger | s | 0 |  |
| A3 |  | dB | 0 | Used to trigger conditional PSCell change from Cell 2 to Cell 3 |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Gap pattern ID |  |  | gp0 | Gaps are configured during T1, T2 and released upon T3 starts. |
| PRACH configuration on cell2 |  |  | FR2 configuration 2 | Captured in A.3.8.3.2 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on carrier frequency of cell3. |
| T1 |  | s | 1 | During this time the PCell is known and Cell 2 is unknown. |
| T2 |  | s | 5 | During this time Cell 2 meets the PSCell addition condition and UE adds this PSCell. |
| T3 |  | s | 5 | During this time Cell 3 meets the PSCell change condition and UE sends PRACH to Cell 3. |

Table A.7.5.X.1.1-3: Cell Specific Parameters for subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC (Cell 2, Cell 3)

| Parameter | Unit | Config | Test |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| NR Channel Number |  | 1,2,3 | 1 for Cell 1 |  |  |
| NR Channel Number |  | 1,2,3 | 2 for Cell 2 and Cell 3 |  |  |
| Duplex Mode |  | 1,2,3 | TDD |  |  |
| TDD configuration |  | 1,2,3 | TDDConf.3.1 |  |  |
| BWchannel | MHz | 1,2,3 | 100: NRB,c = 66 |  |  |
| Data RBs allocated |  | 1,2,3 | 48 |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |
| TRS Configuration |  | 1,2,3 | TRS.2.1 TDD |  |  |
| PDSCH/PDCCH TCI state |  | 1,2,3 | TCI.State.2 |  |  |
| PDSCH Reference measurement channel |  | 1,2,3 | SR.3.3 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1,2,3 | CR.3.2 TDD |  |  |
| Dedicated CORESET Reference Channel |  | 1,2,3 | CCR.3.7 TDD |  |  |
| OCNG Patterns |  | 1,2,3 | OP.3 |  |  |
| SSB configuration |  | 1,2,3 | SSB.2 FR2 |  |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2,3 | 120 |  |  |
| TRS Configuration |  | 1,2,3 | TRS.2.1 TDD |  |  |
| CSI-RS configuration for CSI reporting |  | 1,2,3 | CSI-RS.3.1 TDD |  |  |
| reportConfigType |  | 1,2,3 | periodic |  |  |
| reportQuantity |  | 1,2,3 | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity | slot | 1,2,3 | 40 |  |  |
| CSI reporting offset | slot | 1,2,3 | 4 |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Propagation condition |  | 1,2,3 | AWGN |  |  |

Table A.7.5.X.1.1-4: OTA related test parameters for subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC (Cell 2, Cell 3)

| Parameter | Unit | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  | Setup 2a according to clause A.3.15.2.1 |  |  | Setup 2a according to clause A.3.15.2.1 |  |  |
| Assumption for UE beamsNote 6 |  | Rough |  |  | Rough |  |  |
| Ês Note2 | dBm/SCS | -∞ | -81 | -81 | -∞ | -∞ | -78 |
| SSB_RPNote2, Note 4 | dBm/SCS | -∞ | -81 | -81 | -∞ | -∞ | -78 |
| BB Note 2, Note 7 | dB | -∞ | 4.88 | 4.88 | -∞ | -∞ | 4.88 |
| IoNote 2, Note 4 | dBm/95.04 MHz | N/A | -56.41 | -56.41 | N/A | N/A | -56.41 |
| Note 1: Void Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

A.7.5.X.1.2	Test Requirements

TRRC_delay +TEvent_DU for PSCell addition (Cell 2) occurs during T1 as the PSCell addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms from the start of T2.

The UE shall transmit the PRACH to Cell 3 less than Tconfig_PSCell_Subsequent_Change_Conditional Note1 from the start of T3.

All the above test requirements shall be fulfilled for the observed PSCell change delay to be counted as correct. The rate of correct observed PSCell change delay during repeated tests shall be at least 90%.

Note1:	The subsequent Conditional PSCell change delay during T3 can be expressed as follows:

Tconfig_PSCell_Subsequent_Change_Conditional = TEvent_DU +Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

Where:

TEvent_DU  = 0ms

Tmeasure = 6720ms for power class 1 or 4160 for power class 2/3/4

TUE_preparation = 10ms

Tprocessing = 40ms

T∆ = 20ms

TPSCell_ DU = 1*10+10 = 20 ms

A.7.5.X.2	Inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC

A.7.5.X.2.1	Test Purpose and Environment

The purpose of this test is to verify that the sunsequent conditional NR PSCell addition under NR-DC is within the requirements stated in clause 8.9C.2.

For UE supporting subsequent conditional PSCell addition/change, UE only needs to pass either inter-frequency CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC defined in clause or inter-frequency CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC defined in clause A.6.5.X.2.

For UE which can pass this test, test of conditional PSCell addition and release delay defined in [A.7.5.12] can be skipped.

Supported test configurations are shown in A.7.5.X.2.1-1. The test parameters for the NR cell 1 are given in Table A.3.7A. The NR cell 1 once set up is not changed across time.

The test parameters for NR cell 2, NR cell 3 are given in Tables A.7.5.X.2.1-2, cell-specific parameters in A.7.5.X.2.1-3 and OTA parameters in A.7.5.X.2.1-4 below.  The test comprises of three NR carrier. There are three cells and one cell on each carrier. Before the test starts the UE is connected to Cell 1 (NR PCell) on radio channel 1, but is not aware of Cell 2 (NR candidate NR PSCell 1) on radio channel 2 and Cell 3 (NR candidate PSCell 2) on radio channel 3. The test consists of  four successive time periods with duration of T1, T2, T3, T4.

During T1, the UE does not have any timing information of Cell 2 and Cell 3.  The TE shall configure subsequent conditional PSCell addition/change with Cell 2 and Cell 3 as target PSCells during T1, at a time earlier than TRRC_delay before the beginning of T2.

At the start of T2, Cell 2 becomes detectable and meets the PSCell addition condition. UE shall be able to measure and detect that the condition is fulfilled, after which the UE shall transmit the PRACH preamble to Cell 2. Upon PSCell addition complete (UE transmits SN RRCReconfigurationcomplete message), T3 starts.

During T3, the TE shall send a RRCRconfiguration message to the UE to release PSCell (Cell 2) on radio channel 2. Upon PSCell release complete (UE transmits SN RRCReconfigurationcomplete message), T4 starts.

At the start of T4, Cell 3 becomes detectable and meets the addition condition. UE shall be able to measure and detect that the condition is fulfilled, after which UE shall send PRACH to the PSCell (Cell 3).

Table A.7.5.X.2.1-1: Supported test configurations for Inter-frequency Subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC

| Configuration | Description |
| --- | --- |
| 1 | Cell 1: FDD15 kHz SSB SCS, 10 MHz bandwidth, Cell 2: TDD 120kHz SSB SCS, 100MHz bandwidth Cell 3: TDD 120kHz SSB SCS, 100MHz bandwidth |
| 2 | Cell 1: TDD 15 kHz SSB SCS, 10 MHz bandwidth, Cell 2: TDD 120kHz SSB SCS, 100MHz bandwidth Cell 3: TDD 120kHz SSB SCS, 100MHz bandwidth |
| 3 | Cell 1: TDD 30 kHz SSB SCS, 40 MHz bandwidth, Cell 2: TDD 120kHz SSB SCS, 100MHz bandwidth Cell 3: TDD 120kHz SSB SCS, 100MHz bandwidth |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.5.X.2.1-2: General Test Parameters for subsequent CPA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2, 3 | Three NR radio channels are used for this test. |
| Initial | Active serving cells |  | Cell1 | Cell1: NR PCell on RF channel number 1 |
| Condition | Neighbour cell |  | Cell2, Cell3 | Cell2:NR PSCell on RF channel number 2 Neighbour cell on RF channel number 3 |
| Final | Active serving cells |  | Cell1, Cell3 | Cell1: PCell on RF channel number 1 Cell3:NR PSCell on RF channel number 3 |
| Condition | Neighbour Cell |  | Cell2 | Neighbour cell on RF channel number 2. |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -118 | A4 event is used to trigger conditional PSCell addition of Cell 2 |
|  | Time to Trigger | s | 0 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Gap pattern ID |  |  | gp0 |  |
| PRACH configuration on cell3 |  |  | FR2 configuration 2 | Captured in A.3.8.3.2 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on carrier frequency of cell3. |
| T1 |  | s | 1 | UE is connected to Cell 1 (NR PCell) on radio channel 1, but is not aware of Cell 2 (NR PSCell 1) on radio channel 2 and Cell (NR PSCell 2) on radio channel 3 |
| T2 |  |  | <7 | During this time PSCell 1 meets the PSCell addition condition and the UE adds the PSCell (cell 2). |
| T3 |  | s | 1 | During this time the UE releases the PSCell (cell 2). |
| T4 |  | s | <7 | During this time PSCell 2 meets the PSCell addition condition and the UE adds the PSCell (cell 3). |

Table A.7.5.X.2.1-3: Cell Specific Parameters for subsequent CPA (cell 2, cell 3)

| Parameter | Unit | Config | Test |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 |
| NR Channel Number |  | 1,2,3 | 2 for cell 2, 3 for cell 3 |  |  |  |
| Duplex Mode |  | 1,2,3 | TDD |  |  |  |
| TDD configuration |  | 1,2,3 | TDDConf.3.1 |  |  |  |
| BWchannel | MHz | 1,2,3 | 100: NRB,c = 66 |  |  |  |
| Data RBs allocated |  | 1,2,3 | 48 |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| TRS Configuration |  | 1,2,3 | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  | 1,2,3 | TCI.State.2 |  |  |  |
| PDSCH Reference measurement channel |  | 1,2,3 | SR.3.3 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1,2,3 | CR.3.2 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | 1,2,3 | CCR.3.7 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3 | OP.3 |  |  |  |
| SSB configuration |  | 1,2,3 | SSB.2 FR2 |  |  |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1,2,3 | 120 |  |  |  |
| TRS Configuration |  | 1,2,3 | TRS.2.1 TDD |  |  |  |
| CSI-RS configuration for CSI reporting |  | 1,2,3 | CSI-RS.3.1 TDD |  |  |  |
| reportConfigType |  | 1,2,3 | periodic |  |  |  |
| reportQuantity |  | 1,2,3 | cri-RI-PMI-CQI |  |  |  |
| CSI reporting periodicity | slot | 1,2,3 | 40 |  |  |  |
| CSI reporting offset | slot | 1,2,3 | 4 |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Propagation condition |  | 1,2,3 | AWGN |  |  |  |

Table A.7.5.X.2.1-4: OTA related test parameters

| Parameter | Unit | Cell 2 |  |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| Angle of arrival configuration |  | Setup 2a according to clause A.3.15.2.1 |  |  |  |  | Setup 2a according to clause A.3.15.2.1 |  |  |  |
| Assumption for UE beamsNote 6 |  | Rough |  |  |  |  | Rough |  |  |  |
| Ês Note2 | dBm/SCS | -∞ | -81 |  | -∞ |  | -∞ |  |  | -81 |
| SSB_RPNote2, Note 4 | dBm/SCS | -∞ | -81 |  | -∞ |  | -∞ |  |  | -81 |
| BB Note 2, Note 7 | dB | -∞ | 4.88 |  | -∞ |  | -∞ |  |  | 4.88 |
| IoNote 2, Note 4 | dBm/95.04 MHz | N/A | -56.41 |  | N/A |  | N/A |  |  | -56.41 |
| Note 1: Void Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |  |  |

A.7.5.X.2.2	Test Requirements

TRRC_delay + TEvent_DU for PSCell addition (Cell 2) occurs during T1 as the PSCell addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall transmit the PRACH to PSCell (Cell 2) less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 msfrom the start of T2.

The UE shall transmit the PRACH to PSCell (Cell 3) less than Tconfig_PSCell_Addition_Conditional Note1 from the start of T4.

All the above test requirements shall be fulfilled for the observed PSCell addition delay and PSCell release delay to be counted as correct. The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90%.

Note 1:	The PSCell addition delay during T2 can be expressed as follows:

Tconfig_PSCell_Addition_Conditional = TEvent_DU +  Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms

Where:

TEvent_DU = 0

Tmeasure = 6720ms for power class 1 or 4160 for power class 2/3/4

TUE_preparation = 10ms

Tprocessing = 40ms

T∆ = 20ms

TPSCell_ DU = 1*10+10 = 20 ms

End of Change 30

Start of Change 31

A.7.6.x	LTM Intra-frequency L1-RSRP measurement

A.7.6.X.1	Intra-frequency SSB based L1-RSRP measurement in FR2

A.7.6.X.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of SSB based intra-frequency L1-RSRP measurement on neighbor cell in FR2. This test will partly verify the L1-RSRP measurement requirements in clause 9.14, with the testing configurations for NR cells in Table A.7.6.X.1.1-1.

The AoA setup of FR2 cell for this test is Setup 3 as defined in clause A.3.15.

Table A.7.6.X.1.1-1: Applicable NR configurations for SSB based Intra-frequency L1-RSRP LTM measurement with activated TCI state test in FR2

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

A.7.6.X.1.2	Test parameters

There are two cells in the test, which are FR2 PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell.

Measurement period [and measurement accuracy] is tested by using the parameters in Table A.7.6.X.1.2-1 and Table A.7.6.X.1.2-2.

There are two tests in the test case, test 1 and test 2:

In test 1, time offset between cells is within CP length.

In test 2, time offset between cells is larger than CP length.

If a UE does not support [RTD>CP], it is only required to pass test 1. Otherwise, it is only required to pass test 2.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSBs, and report measurement results periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. SSB_RP of Cell 2 in T1 and T2 are different. No gap patterns are configured in the test case.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC).

-	A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Before the start of the T1, event is triggered, and UE has sent a measurement report for the Cell 2 with SSB Index.

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	The UE has performed L3 measurement on Cell 2.

At the beginning of T2, SSB_RP of Cell 2 change to different values from T1. T2 starts at the beginning of a frame with an odd SFN.

Table A.7.6.X.1.2-1: General test parameters for SSB based intra-frequency L1-RSRP LTM measurement without activated TCI state test in FR2

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbouring cell |  |  | Cell 2 |  | Cell 2 is the candidate cell |
| A3-Offset |  | dB | -30 |  |  |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | ms | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| DRX |  |  | OFF |  | DRX is not used |
| Time offset between cells |  |  | 0.2 s | 2s | The timing of Cell 2 is later than the timing of Cell 1 |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 1.5 |  |  |
| T2 |  | s | 2 |  |  |

Table A.7.6.X.1.2-2: Cell specific test parameters for SSB based intra-frequency L1-RSRP LTM measurement without activated TCI state test in FR2

| Parameter | Config | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB GSCN | 1~2 |  | Freq1 |  |  |  |
| Duplex mode | 1~2 |  | TDD |  |  |  |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |  |  |  |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |  |  |  |
| Data RBs allocated | 1~2 |  | 66 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | 1~2 | KHz | 120 |  |  |  |
| PUCCH/PUSCH subcarrier spacing | 1~2 | KHz | 120 |  |  |  |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD |  | N/A |  |
|  | 2 |  | SR.3.3 TDD |  | N/A |  |
| RMSI CORESET Reference Channel | 1, |  | CR.3.1 TDD |  | N/A |  |
|  | 2 |  | CR.3.2 TDD |  | N/A |  |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |  | N/A |  |
|  | 2 |  | CCR.3.7 TDD |  | N/A |  |
| SSB configuration | 1 |  | SSB.3 FR2 |  | SSB.7 FR2 |  |
|  | 2 |  | SSB.8 FR2 |  | SSB.4 FR2 |  |
| OCNG Patterns | 1~2 |  | OP.4 |  | N/A |  |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |  |  |  |
| SMTC configuration | 1~2 |  | SMTC.1 |  |  |  |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |  | N/A |  |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |  | N/A |  |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Propagation condition | 1~2 |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |

Table A.7.6.X.1.2-3: NR OTA Cell specific test parameters for SSB based intra-frequency L1-RSRP LTM measurement without activated TCI state test in FR2

| Parameter | Config | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration | 1~2 |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam AssumptionNote 4 | 1~2 |  | Rough |  | Rough |  |
| BBNote 5 | 1~2 | dB | -1.5 | 16.5 | -1.5 | 16.5 |
| SSB_RP Note3 | 1 | dBm/SSB SCS | -103 | -85 | -103 | -85 |
|  | 2 |  | -100 | -82 | -100 | -82 |
| Io Note3 | 1 | dBm/95.04MHz | -74 | -56 | -74 | -56 |
|  | 2 |  | -74 | -56 | -74 | -56 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

A.7.6.X.1.3	Test Requirements

The UE shall send L1-RSRP report every 320 slots in T2. The UE shall start to report a larger L1-RSRP value of Cell 2 in no later than 960 ms plus 320 slots from the beginning of time period T2, UE shall send L1-RSRP report including the valid results for Cell 2 while meeting the accuracy requirements defined in clause 10.1.20X.The reported L1-RSRP value shall include the Rx antenna gain in the range of -10 to +20 dB.

The rate of correct events observed during repeated tests shall be at least 90%.

End of Change 31

Start of Change 32

A.7.6.Y	LTM Inter-frequency L1-RSRP measurement with measurement gap

A.7.6.Y.1	Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM

A.7.6.Y.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of inter-frequency L1-RSRP measurement with MG for LTM. This test will partly verify the L1-RSRP measurement requirements in clause 9.15.5, with the testing configurations in Tables A.7.6.Y.1.1-1, A.7.6.Y.1.2-1, A.7.6.Y.1.2-2 and A.7.6.Y.1.2-3.

The AoA setup of FR2 cell for this test is Setup 1 as defined in clause A.3.15.

Table A.7.6.Y.1.1-1: Applicable NR configurations for inter-cell L1-RSRP measurement with measurement gapfor LTM

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

A.7.6.Y.1.2	Test parameters

There are two cells in the test, Cell 1 is the PCell on NR RF channel number 1 and Cell 2 is a neighbor cell NR RF channel number 2. The test parameters for Cell 1 and Cell 2 are given in Table A.7.6.Y.1.2-2 and Table A.7.6.Y.1.2-3.

The test consists of two successive time periods, with time duration of T1 and T2 respectively. SSB_RP of Cell 2 in T1 and T2 are different. Measurement gap is configured in the test.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC).

-	A measurement object is configured for the RF channel 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Before the start of the T1, event is triggered, and UE has sent a measurement report for the Cell 2 with SSB Index.

-    UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

At the beginning of T2, SSB_RP of Cell 2 changes to a different value from T1.

Table A.7.6.y.1.2-1: General test parameters for SSB based inter-frequency L1-RSRP LTM measurement with measurement gap test in FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 |  |
| Neighbouring cell |  |  | Cell 2 | Cell 2 is the candidate cell |
| A3-Offset |  | dB | -30 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | ms | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| maxNrofRS-IndexesToReport |  |  | 1 |  |
| includeBeamMeasurements |  |  | True |  |
| DRX |  |  | OFF | DRX is not used |
| Time offset between cells |  |  | 3 s | The timing of Cell 2 is later than the timing of Cell 1 |
| deriveSSB-IndexFromCell |  |  | Enabled | Not relevant to this test case |
| Gap Pattern Id |  |  | 13 |  |
| Measurement gap offset |  | ms | 39 |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |
|  | spCellInclusion |  | N/A |  |
| ltm-ConfigComplete |  |  | True | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 2 |  |
| T2 |  | s | 2 |  |

Table A.7.6.Y.1.2-1: Cell specific test parameters for SSB based inter-frequency L1-RSRP LTM measurement with measurement gap test in FR2

| Parameter | Config | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 | freq 2 |
| Duplex mode | 1~2 |  | TDD |  |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |  |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated | 1~2 |  | 66 |  |
| PDSCH/PDCCH subcarrier spacing | 1~2 | KHz | 120 |  |
| PUCCH/PUSCH subcarrier spacing | 1~2 | KHz | 120 |  |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD | N/A |
|  | 2 |  | SR.3.3 TDD | N/A |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | N/A |
|  | 2 |  | CR.3.2 TDD | N/A |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | N/A |
|  | 2 |  | CCR.3.7 TDD | N/A |
| SSB configuration | 1 |  | SSB.3 FR2 | SSB.3 FR2 |
|  | 2 |  | SSB.4 FR2 | SSB.4 FR2 |
| OCNG Patterns | 1~2 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |  |
| SMTC configuration | 1~2 |  | SMTC.1 | SMTC.1 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD | N/A |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 | N/A |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition | 1~2 |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table A.7.6.Y.1.2-3: SSB specific test parameters for neighbor cell

| Parameter | Config | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration | 1~2 |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam AssumptionNote 4 | 1~2 |  | Rough |  | Rough |  |
| BBNote 5 | 1~2 | dB | 16.5 | 16.5 | -1.5 | 16.5 |
| SSB_RP Note3 | 1 | dBm/SSB SCS | -85 | -85 | -103 | -85 |
|  | 2 |  | -82 | -82 | -100 | -82 |
| Io Note3 | 1 | dBm/95.04MHz | -56 | -56 | -74 | -56 |
|  | 2 |  | -56 | -56 | -74 | -56 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

A.7.6.Y.1.3	Test Requirements

The UE shall send L1-RSRP report every 320 slots in T2. The UE shall start to report a larger L1-RSRP value of Cell 2 in no later than 1280 ms plus 320 slots from the beginning of time period T2. UE shall send L1-RSRP report including the valid results for Cell 2 while meeting the accuracy requirements defined in clause 10.1.20Y.

The reported L1-RSRP value shall include the Rx antenna gain in the range of -10 to +20 dB.

The rate of correct events observed during repeated tests shall be at least 90%.

End of Change 32

Start of Change 33

A.7.6.Z	LTM Inter-frequency L1-RSRP measurement without measurement gap

A.7.6.Z.1	Inter-frequency SSB based L1-RSRP measurement without measurement gap in FR2

A.7.6.Z.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting [FG 39-2: SSB based inter-frequency L1-RSRP measurements without measurement gaps] makes correct reporting of inter-frequency L1-RSRP measurement without gap for LTM. This test will partly verify the L1-RSRP measurement requirements in clause 9.15.6, with the testing configurations in Tables A.7.6.Z.1.1-1, A.7.6.Z.1.2-1, A.7.6.Z.1.2-2, A.7.6.Z.1.2-3 and A.7.6.Z.1.2-4.

The AoA setup of FR2 cell for this test is Setup 1 as defined in clause A.3.15.

Table A.7.6.Z.1.1-1: Applicable NR configurations for SSB based inter-frequency L1-RSRP LTM measurement without gap in FR2

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

A.7.6.Z.1.2	Test parameters

There are two cells in the test, Cell 1 is the PCell on NR RF channel number 1 and Cell 2 is a neighbor cell NR RF channel number 2. The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from Cell 1 and Cell 2 should be different in frequency location within the cell bandwidth. The test parameters for Cell 1 are given in Table A.7.6.Z.1.2-1. The test parameters for Cell 2 are given in Table A.7.6.Z.1.2-2 and Table A.7.6.Z.1.2-3.

There are two tests in the test case, test 1 and test 2:

In test 1, time offset between cells is within CP length.

In test 2, time offset between cells is larger than CP length.

UE not capable of [RTD>CP] is only required to pass test 1. Otherwise, it is only required to pass test 2.

The test consists of two successive time periods, with time duration of T1 and T2 respectively. SSB_RP of Cell 2 in T1 and T2 are different. No measurement gap is configured in the test.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 (PCell) on RF channel 1 (PCC).

-	A measurement object is configured for the RF channel 2, and it is indicated to the UE that event-triggered reporting with Event A3 is used. Before the start of the T1, event is triggered, and UE has sent a measurement report for the Cell 2 with SSB Index.

-    UE is provided with LTM-Candidate-r18 for Cell 2.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

At the beginning of T2, SSB_RP of Cell 2 changes to a different value from T1. T2 starts at the beginning of a frame with an odd SFN.

Table A.7.6.Z.1.2-1: General test parameters for SSB based inter-frequency L1-RSRP LTM measurement test in FR2

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbouring cell |  |  | Cell 2 |  | Cell 2 is the candidate cell |
| RF Channel Number |  |  | 1: Cell 1 2: Cell 2 |  |  |
| A3-Offset |  | dB | -30 |  |  |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | ms | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| maxNrofRS-IndexesToReport |  |  | 1 |  |  |
| includeBeamMeasurements |  |  | True |  |  |
| DRX |  |  | OFF |  | DRX is not used |
| Time offset between cells |  |  | 0.2 s | 2s | The timing of Cell 2 is later than the timing of Cell 1 |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 1.5 |  |  |
| T2 |  | s | 2 |  |  |

Table A.7.6.Z.1.2-2: Cell specific test parameters for for SSB based inter-frequency L1-RSRP LTM measurement test in FR2

| Parameter | Config | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 | freq2 |
| Duplex mode | 1~2 |  | TDD |  |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |  |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated | 1~2 |  | 66 |  |
| PDSCH/PDCCH subcarrier spacing | 1~2 | KHz | 120 |  |
| PUCCH/PUSCH subcarrier spacing | 1~2 | KHz | 120 |  |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD | N/A |
|  | 2 |  | SR.3.3 TDD | N/A |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | N/A |
|  | 2 |  | CR.3.2 TDD | N/A |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | N/A |
|  | 2 |  | CCR.3.7 TDD | N/A |
| SSB configuration | 1 |  | SSB.3 FR2 |  |
|  | 2 |  | SSB.4 FR2 |  |
| OCNG Patterns | 1~2 |  | OP.4 | N/A |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |  |
| SMTC configuration | 1~2 |  | SMTC.1 |  |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD | N/A |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 | N/A |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition | 1~2 |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table A.7.6.Z.1.2-3: NR OTA Cell specific test parameters for SSB based inter-frequency L1-RSRP LTM measurement without measurement gap in FR2

| Parameter | Config | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration | 1~2 |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam AssumptionNote 3 | 1~2 |  | Rough |  | Rough |  |
| BBNote 4 | 1~2 | dB | -1.5 | 16.5 | -1.5 | 16.5 |
| SSB_RP Note2 | 1 | dBm/SSB SCS | -103 | -85 | -103 | -85 |
|  | 2 |  | -100 | -82 | -100 | -82 |
| Io Note2 | 1 | dBm/95.04MHz | -74 | -56 | -74 | -56 |
|  | 2 |  | -74 | -56 | -74 | -56 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

A.7.6.Z.1.3	Test Requirements

The UE shall send L1-RSRP report every 320 slots in T2. No later than 960 ms plus 320 slots from the beginning of time period T2, UE shall send L1-RSRP report including the valid results for Cell 2 while meeting the accuracy requirements defined in clause 10.1.20Y.

The reported L1-RSRP value shall include the Rx antenna gain in the range of -10 to +20 dB.

The rate of correct events observed during repeated tests shall be at least 90%.

End of Change 33

Start of Change 34

A.7.6.X2	Idle Mode CA/DC Measurements

A.7.6.X2.1	Test case for Idle mode fast CA/DC eEMR measurement for FR2

A.7.6.X2.1.1	Test Purpose and Environment

The purpose of this test is to verify UE measurement reporting behaviour as specified in clause 4.7. This test will partly verify the fast CA/DC measurement reporting requirements in clause 4.7 when measIdleValidityDuration is configured for the test case when there are no measurement results to report at RRC connection setup.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as inter-frequency neighbour cell in FR2 on NR RF channel 2.  The test parameters are given in Tables A.7.6.X2.1.1-1, A.7.6.X2.1.1-2, A.7.6.X2.1.1-3 and A.7.6.X2.1.1-4.

The test consists of 4 successive time periods, with time duration of T1, T2, T3 and T4 respectively.

Before T1, the UE is connected to cell 1 and configured with inter-frequency measurement on cell 2 with periodic reporting. The time when a valid measurement report is received at TE defines the starting point of T1.

During T1, the UE is configured with early measurement reporting for cell 2 in measIdleCarrierListNR-r16. Beam level reporting for early measurements is not configured. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

During T2 and T3 the UE is in idle mode. The time when T331 timer expires defines the ending point of T2.

At the beginning of T3, the signal level of the neibour cell is set to turned off.  The duration of the T3 equals to measIdleValidityDuration.

The time when TE sends the paging message is defined as the starting point of T4. During T4, in this test the UE shall not send measurement report.

Table A.7.6.X2.1.1-1: supported test configuration

| Config | Description for serving cell | Description for neighbor cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |  |

Table A.7.6.X2.1.1-2: General test parameters for Idle mode fast CA/DC eEMR measurement for FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | One FR1 and one FR2 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | Cell1: SSB.1 FR1 Cell2: SSB.1 FR2 |  |
|  |  | Config 2 | Cell1: SSB.1 FR1 Cell2: SSB.1 FR2 |  |
|  |  | Config 3 | Cell1: SSB.2 FR1 Cell2: SSB.2 FR2 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| T331 | s |  | 30 |  |
| measIdleValidityDuration | s |  | 50 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 1 |  |
| T2 | s | Config 1,2,3 | 30 |  |
| T3 | s | Config 1,2,3 | 50 |  |
| T4 | s |  | 1 |  |

Table A.7.6.X2.1.1-3: Cell specific test parameters in Idle and Connected mode for Idle mode fast CA/DC eEMR measurement for FR2

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  | TDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  | TDD |  |  |  |
| TDD configuration |  |  | N.A | N.A |  |  |  | TDDConf.3.1 |  |  |  |
|  |  |  | TDDConf.1.1 | TDDConf.1.1 |  |  |  | TDDConf.3.1 |  |  |  |
|  |  |  | TDDConf.2.1 | TDDConf.2.1 |  |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  | 100: NRB,c = 66 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  | 100: NRB,c = 66 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | SR.3.1 TDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | SR.3.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | SR.3.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | CR.3.1 TDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | CR.3.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | CR.3.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.1 FR2 |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.1 FR2 |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.1 FR2 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.1 |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  | 120 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  | 120 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  |  | See Table A.7.6.X2.1.1-4 |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | -95 |  |  |  |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | -88 |  |  |  |  |  |  |  |
|  |  | dB | Config 1,2,3,4, | 7 |  |  |  |  |  |  |  |
|  |  | dB | Config 1,2,3 | 7 |  |  |  |  |  |  |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -62.26 |  |  |  |  |  |  |  |
|  |  | dBm/38.16MHz | Config 3 | -56.15 |  |  |  |  |  |  |  |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.6.X2.1.1-4: OTA related test parameters for Test case Idle mode fast CA/DC eEMR measurement for FR2

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| Angle of arrival configuration |  |  | N/A |  |  |  | Setup 1 defined in clause A.3.15.1 |  |  |  |
| Assumption for UE beams Note 7 |  |  | N/A |  |  |  | Rough |  |  |  |
| Note 1 | Config 1,2,3 | dBm/15kHz | N/A |  |  |  | -104.7 | -104.7 | -104.7 | -104.7 |
| Note 1 | Config 1,2,3 | dBm/SCS |  |  |  |  | -95.7 | -95.7 | -95.7 | -95.7 |
|  | Config 1,2,3 | dB |  |  |  |  | 7 | 7 | -infinity | -infinity |
|  | Config 1,2,3 | dB |  |  |  |  | 7 | 7 | -infinity | -infinity |
| SSB_RPNote 2, Note 4 | Config 1,2,3 | dBm/SCS |  |  |  |  | -88.7 | -88.7 | -infinity | -infinity |
| IoNote 2, Note 4 | Config 1,2,3 | dBm/95.04 MHz |  |  |  |  | -58.92 | -58.92 | -66.68 | -66.68 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not imit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |  |

A.7.6.X2.1.2	Test Requirements

During the period T2 and T3, the UE shall not perform reselection.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for cell 2.

The UE shall NOT send early measurement report to the PCell in this test.

The rate of correct events observed during repeated tests shall be at least 90%.

A.7.6.X2.2	Test case for Idle mode fast CA/DC cell reselection measurement for FR2 without valid reporting

A.7.6.X2.2.1	Test Purpose and Environment

The purpose of this test is to verify UE measurement reporting behaviour as specified in clause 4.7. This test will partly verify the fast CA/DC measurement reporting requirements in clause 4.7 when measReselectionValidityDuration-r18 is configured for te test case when there are no measurement results to report at RRC connection setup.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as inter-frequency neighbour cell in FR2 on NR RF channel 2.  The test parameters are given in Tables A.7.6.X2.2-1, A.7.6.X2.2-2, A.7.6.X2.2-3 and A.7.6.X2.2-4.

The test consists of 4 successive time periods, with time duration of T1, T2, T3 and T4 respectively.

Before T1, the UE is connected to cell 1 and configured with inter-frequency measurement on cell 2 with periodic reporting. The time when a valid measurement report is received at TE defines the starting point of T1.

During T1, the UE is configured with early measurement reporting for cell 2 in MeasReselectionCarrierListNR-r18. Beam level reporting for early measurements is not configured. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

During T2 and T3 the UE is in idle mode.

At the beginning of T2, cell 2 becomes detectable however no cell reselection is being performed. Signal level of cell 2 is set to the level according to Table A.7.6.X2.2-3. The duration of T2 is set to fixed value according to the Table A.7.6.X2.2-2.

At the beginning of T3, the signal level of the neibour cell is set to turned off.  The duration of the T3 equals to measReselectionValidityDuration-r18.

During T3, the signal level of the neighbour cell shall be set to turned off. The time when TE sends the paging message defined as the starting point of T4.

During T4, UE shall not send measurement report.

Table A.7.6.X2.2-1: supported test configuration

| Config | Description for serving cell | Description for neighbor cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |  |

Table A.7.6.X2.2-2: General test parameters for Idle mode fast CA/DC cell reselection measurement for FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | One FR1 and one FR2 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | Cell1: SSB.1 FR1 Cell2: SSB.1 FR2 |  |
|  |  | Config 2 | Cell1: SSB.1 FR1 Cell2: SSB.1 FR2 |  |
|  |  | Config 3 | Cell1: SSB.2 FR1 Cell2: SSB.2 FR2 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| T331 | s |  | 30 |  |
| measIdleValidityDuration | s |  | 50 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 1 |  |
| T2 | s | Config 1,2,3 | 30 |  |
| T3 | s | Config 1,2,3 | 50 |  |
| T4 | s |  | 1 |  |

Table A.7.6.X2.2-3: Cell specific test parameters in Idle and Connected mode for Idle mode fast CA/DC cell-reselection measurement for FR2

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  | TDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  | TDD |  |  |  |
| TDD configuration |  |  | N.A | N.A |  |  |  | TDDConf.3.1 |  |  |  |
|  |  |  | TDDConf.1.1 | TDDConf.1.1 |  |  |  | TDDConf.3.1 |  |  |  |
|  |  |  | TDDConf.2.1 | TDDConf.2.1 |  |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  | 100: NRB,c = 66 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  | 100: NRB,c = 66 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | SR.3.1 TDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | SR.3.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | SR.3.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | CR.3.1 TDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | CR.3.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | CR.3.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.1 FR2 |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.1 FR2 |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.1 FR2 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.1 |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  | 120 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  | 120 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  |  | See Table A.7.6.X2.2-4 |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | -95 |  |  |  |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | -88 |  |  |  |  |  |  |  |
|  |  | dB | Config 1,2,3,4, | 7 |  |  |  |  |  |  |  |
|  |  | dB | Config 1,2,3 | 7 |  |  |  |  |  |  |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -62.26 |  |  |  |  |  |  |  |
|  |  | dBm/38.16MHz | Config 3 | -56.15 |  |  |  |  |  |  |  |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.6.X2.2-4: OTA related test parameters for Test case Idle mode fast CA/DC cell-reselection measurement for FR2

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| Angle of arrival configuration |  |  | N/A |  |  |  | Setup 1 defined in clause A.3.15.1 |  |  |  |
| Assumption for UE beams Note 7 |  |  | N/A |  |  |  | Rough |  |  |  |
| Note 1 | Config 1,2,3 | dBm/15kHz | N/A |  |  |  | -104.7 | -104.7 | -104.7 | -104.7 |
| Note 1 | Config 1,2,3 | dBm/SCS |  |  |  |  | -95.7 | -95.7 | -95.7 | -95.7 |
|  | Config 1,2,3 | dB |  |  |  |  | 7 | 7 | -infinity | -infinity |
|  | Config 1,2,3 | dB |  |  |  |  | 7 | 7 | -infinity | -infinity |
| SSB_RPNote 2, Note 4 | Config 1,2,3 | dBm/SCS |  |  |  |  | -88.7 | -88.7 | -infinity | -infinity |
| IoNote 2, Note 4 | Config 1,2,3 | dBm/95.04 MHz |  |  |  |  | -58.92 | -58.92 | -66.68 | -66.68 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not imit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |  |

A.7.6.X2.2.2	Test Requirements

During the period T2 and T3, the UE shall not perform reselection.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for cell 2.

The UE shall NOT send early measurement report to the PCell in this test.

The rate of correct events observed during repeated tests shall be at least 90%.

A.7.6.X2.3 Test case for Idle mode fast CA/DC cell reselection measurement for FR2 with valid reporting

A.7.6.X2.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE performs the required measurements on the serving cell and the configured inter-frequency carrier for idle mode measurement reporting after the UE has entered Idle mode. This test will partly verify the fast CA/DC measurement reporting requirements in clause 4.7 when measReselectionValidityDuration-r18 is configured for the test case when there are measurement results to report at RRC connection setup.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters are given in Tables A.7.6.X2.3.1-1, A.7.6.X2.3.1-2, A.7.6.X2.3.1-3 A.7.6.X2.3.1-4.

The test consists of 4 successive time periods, with time duration of T1, T2, T3 and T4 respectively.

During T1, the UE is connected to cell 1 only and shall not have any timing information of cell 2. UE is configured with early measurement reporting for cell 2 in MeasReselectionCarrierListNR-r18. Beam level reporting for early measurements is not configured. The time point when UE receives RRC_Release message from the TE defines as the starting point of T2.

During T2, cell 2 becomes detectable however no cell reselection is being performed. Signal level of cell 2 is set to level 1. The duration of T2 is set to fixed value according to the Table A.7.6.X2.3-2.

The duration of the T3 equals to the configured value measReselectionValidityDuration-r18.

During T3, the singal level of cell 2 is set to another value according to the Table A.7.6.X2.3-2. The time when TE sends the paging message as the starting point of T4.

During T4, UE shall send measurement report within the duration of T4.

Table A.7.6.X2.3-1: supported test configuration

| Config | Description for serving cell | Description for neighbor cell |
| --- | --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |  |

Table A.7.6.X2.3-2: General test parameters for Idle mode fast CA/DC cell reselection measurement for FR2

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | One FR1 and one FR2 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | Cell1: SSB.1 FR1 Cell2: SSB.1 FR2 |  |
|  |  | Config 2 | Cell1: SSB.1 FR1 Cell2: SSB.1 FR2 |  |
|  |  | Config 3 | Cell1: SSB.2 FR1 Cell2: SSB.2 FR2 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| T331 | s |  | 30 |  |
| measIdleValidityDuration | s |  | 50 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 1 |  |
| T2 | s | Config 1,2,3 | 30 |  |
| T3 | s | Config 1,2,3 | 50 |  |
| T4 | s |  | 1 |  |

Table A.7.6.X2.3-3: Cell specific test parameters in Idle and Connected mode for Idle mode fast CA/DC cell-reselection measurement for FR2

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  | TDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  | TDD |  |  |  |
| TDD configuration |  |  | N.A | N.A |  |  |  | TDDConf.3.1 |  |  |  |
|  |  |  | TDDConf.1.1 | TDDConf.1.1 |  |  |  | TDDConf.3.1 |  |  |  |
|  |  |  | TDDConf.2.1 | TDDConf.2.1 |  |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  | 100: NRB,c = 66 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  | 100: NRB,c = 66 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  | 100: NRB,c = 66 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | SR.3.1 TDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | SR.3.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | SR.3.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | CR.3.1 TDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | CR.3.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | CR.3.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.1 FR2 |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.1 FR2 |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.1 FR2 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.1 |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  | 120 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  | 120 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  |  | See Table A.7.6.X2.3-4 |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | -95 |  |  |  |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | -88 |  |  |  |  |  |  |  |
|  |  | dB | Config 1,2,3,4, | 7 |  |  |  |  |  |  |  |
|  |  | dB | Config 1,2,3 | 7 |  |  |  |  |  |  |  |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -62.26 |  |  |  |  |  |  |  |
|  |  | dBm/38.16MHz | Config 3 | -56.15 |  |  |  |  |  |  |  |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.6.X2.3-4: OTA related test parameters for Test case Idle mode fast CA/DC cell-reselection measurement for FR2

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| Angle of arrival configuration |  |  | N/A |  |  |  | Setup 1 defined in clause A.3.15.1 |  |  |  |
| Assumption for UE beams Note 7 |  |  | N/A |  |  |  | Rough |  |  |  |
| Note 1 | Config 1,2,3 | dBm/15kHz | N/A |  |  |  | -104.7 | -104.7 | -104.7 | -104.7 |
| Note 1 | Config 1,2,3 | dBm/SCS |  |  |  |  | -95.7 | -95.7 | -95.7 | -95.7 |
|  | Config 1,2,3 | dB |  |  |  |  | 7 | 7 | -0.5 | - 0.5 |
|  | Config 1,2,3 | dB |  |  |  |  | 7 | 7 | -0.5 | -0.5 |
| SSB_RPNote 2, Note 4 | Config 1,2,3 | dBm/SCS |  |  |  |  | -88.7 | -88.7 | -96.5 | -96.5 |
| IoNote 2, Note 4 | Config 1,2,3 | dBm/95.04 MHz |  |  |  |  | -58.92 | -58.92 | -69.3 | -69.3 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not imit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |  |

A.7.6.X2.3.2	Test Requirements

During the period T2 and T3 the UE shall not perform reselection.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for cell 2.

The UE shall send early measurement report to the PCell with valid measurement results.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for cell 2 meets the requirements in Section 10.1.4B for SS-RSRP and in Section 10.1.8B for SS-RSRQ and test ends. In the test case, the reported measurements are considered valid if they fulfil measurement accuracy requirements according to cell 2 signal level during T3.

The rate of correct events observed during repeated tests shall be at least 90%.

End of Change 34

Start of Change 35

A.7.7.x	LTM L1-RSRP measurement

A.7.7.x.1	SSB based inter-frequency L1-RSRP measurement

A.7.7.x.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause [10.1.20.x] for inter-frequency L1-RSRP measurements based on SSB with the testing configurations for NR cells in Table A.7.7.x.1.1-1.

Prior to the start of the test,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is provided with LTM-Candidate-r18 for Cell 2.

-	A measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A4 is used.

-	UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports on candidate cell (Cell 2) in PUCCH format 2.

-	The UE has reported L3 measurement results and performed SSB based L1-RSRP measurement on Cell 2.

Table A.7.7.x.1.1-1: Applicable NR configurations for FR2 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

A.7.7.x.1.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on a different frequency than the PCell. The test parameters for the Cell 1 and Cell 2 are given in Table A.7.7.x.1.2-1 and Table A.7.7.x.1.2-2 below. The absolute accuracy of inter-frequency L1-RSRP measurements are tested by using the parameters in Table A.7.7.X.1.2-1 and Table A.7.7.X.1.2-2. The inter-frequency L1-RSRP measurements are supported by a measurement gap.

Before the test, UE is configured L1-RSRP measurement on SSB0 of Cell 2.

Table A.7.7.x.1.2-1: FR2 SSB based inter-frequency L1-RSRP general test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN | 1~2 |  | freq1 | freq2 | freq1 | freq2 |
| BWchannel | 1~2 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated | 1~2 |  | 66 |  | 66 |  |
| Gap pattern ID | 1~2 |  | 13 |  | 13 |  |
| Measurement gap offset | 1~2 |  | 39 |  | 39 |  |
| Duplex mode | 1~2 |  | TDD |  | TDD |  |
| TDD configuration | 1~2 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.3. 2 TDD | - | SR.3. 2 TDD | - |
|  | 2 |  | SR.3.3 TDD |  | SR.3.3 TDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | - | CR.3.1 TDD | - |
|  | 2 |  | CR.3.2 TDD |  | CR.3.2 TDD |  |
| Control Channel RMC | 1 |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
|  | 2 |  | CCR.3.7 TDD |  | CCR.3.7 TDD |  |
| SSB configuration | 1 |  | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  | 2 |  | SSB.4 FR2 |  | SSB.4 FR2 |  |
| PDSCH/PDCCH subcarrier spacing | 1~2 | kHz | 120 |  | 120 |  |
| OCNG Patterns | 1~2 |  | OP.1 |  | OP.1 |  |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |  | DLBWP.1.3 ULBWP.1.3 |  |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |  | TCI.State.2 |  |
| SMTC configuration | 1~2 |  | SMTC.1 |  | SMTC.1 |  |
| LTM reportConfigType | 1~2 |  | periodic | - | periodic | - |
| ltm-ResourcesForChannelMeasurement | 1~2 |  | SSB0 of Cell 2 | - | SSB0 of Cell 2 | - |
| LTM L1-RSRP reporting period | 1~2 |  | slot320 | - | slot320 | - |
| nrOfReportedCells | 1~2 |  | 1 | - | 1 | - |
| nrOfReportedRS-PerCell | 1~2 |  | 1 | - | 1 | - |
| spCellInclusion | 1~2 |  | N/A | - | N/A |  |
| Time offset between Cell 2 and Cell 1 | 1~2 | s | 3 |  | 3 |  |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Propagation condition | 1~2 | - | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration | 1~2 | - | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |  |

Table A.7.7.X.1.2-2: FR2 SSB based inter-frequency L1-RSRP OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration | 1~2 |  | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 4 | 1~2 |  | Rough |  | Rough |  |
| Note1 | 1 | dBm/15kHzNote4 | -100 |  | n.a. |  |
|  | 2 |  |  |  |  |  |
| Note1 | 1 | dBm/SCSNote4 | -91 |  | n.a. |  |
|  | 2 |  | -88 |  | n.a. |  |
|  | 1~2 | dB | 10 | -2 | n.a. |  |
| ÊsIot Note2 | 1~2 | dB | 10 | -2 | n.a. |  |
| SSB_RPNote2 | 1 | dBm/SCS | -81 | -93 | As in Table B.2.4-2 |  |
|  | 2 |  | -78 | -90 | As in Table B.2.4-2 |  |
| IoNote2 | 1 | dBm/95.04 MHz Note3 | -51.57 | -59.86 | SSB_RP+28.98 |  |
|  | 2 |  | -51.57 | -59.86 | SSB_RP+28.98 |  |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot, Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

A.7.7.X.1.3	Test Requirements

The L1-RSRP measurement accuracy for Cell 2 shall fulfil the absolute requirements in clause 10.1.20Y.

Test 1:

Absolute accuracy of SSB0 in Cell 2. The UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in Table A.7.7.x.1.3-1.

Test 2:

Absolute accuracy of SSB0 in Cell 2. The UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in Table A.7.7.x.1.3-1.

Table A.7.7.X.1.3-1: L1-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| SSB0 | SSB_RP0 -δ + Gmin ≤ Reported RSRP(dBm) ≤ SSB_RP0 +δ + Gmax |
| Note 1: SSB_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the SSB n under consideration Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.20.1.1-1, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

End of Change 35