3GPP TSG-RAN WG4 Meeting #111	R4-2408495

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0530 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on applicability of requirements for FR2 multi-Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRx_DL-Perf |  |  |  |  |  | Date: |  |  | 2024-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some problems of the applicability of FR2 multi-Rx requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of changes in this CR as below: - update requirements in Clause 7.1.1.3 7.1.1.4 7.1.1.9 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The applicability of FR2 multi-Rx requirements is not right. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3 7.1.1.4 7.1.1.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

7.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 7.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 7.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |  |
| --- | --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 3-1) |  |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 7.5A.1 | 1) Up to 16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |  |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR2-1 TDD | PDSCH | Clause 7.2.2.2.2 |  |  |
| Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR2 TDD | PDSCH | Clause 7.2.2.2.2 |  |  |
| DRX Adaptation (drx-Adaptation-r16) | FR2-1 TDD | PDCCH | Clause 7.3.2.2.3 | If the Test 3-1 in Clause 7.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 7.3.2.2.1. |  |
| 256QAM for PDSCH (pdsch-256QAM-FR2) | FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 1-4) |  |  |
| 256QAM for PDSCH (pdsch-256QAM-FR2) | FR2-1 TDD | SDR | Clause 7.5A.1 | For UE capable of pdsch-256QAM-FR2 for certain band(s), mcs-Table is configured to ‘64QAM’ for SDR test. |  |
| Support of FR2 HST operation [(FR2 UE power class PC6 signalling is used to indicate support of feature group)] | FR2-1 TDD | PDSCH | [Clause 7.2.2.2.4] |  |  |
| Support of Single Carrier operations with 120kHz SCS for FR2-2 (initialAccessSSB-120kHz-r17) | FR2-2 TDD | PDSCH | Clause 7.2.2.2.1 (Table 7.2.2.2.1-6: Test 4-1, 4-2, 4-3, 4-4) |  |  |
|  |  | PDCCH | Clause 7.3.2.2 (Table 7.3.2.2.1-2: Test 1a-1, 1a-2, 1a-3) (Table 7.3.2.2.2-2, Test 3-1, 3-2) |  |  |
|  |  | PBCH | Clause 7.4.2.2 (Table 7.4.2.2-2: Test 3) |  |  |
| Support of 480kHz SCS for FR2-2 (ul-FR2-2-SCS-480kHz-r17 and initialAccessSSB-480kHz-r17) | FR2-2 TDD | PDSCH | Clause 7.2.2.2.1 (Table 7.2.2.2.1-6: Test 4-5, 4-6) |  |  |
|  |  | PDCCH | Clause 7.3.2.2 (Table 7.3.2.2.1-2: Test 1a-4) (Table 7.3.2.2.2-2, Test 3-3) |  |  |
|  |  | PBCH | Clause 7.4.2.2 (Table 7.4.2.2-2: Test 4) |  |  |
| Support simultaneous reception with different QCL Type-D RSs (simultaneousReceptionDiffTypeD-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.5 Clause 7.2.2.2.6 Clause 7.2.2.2.7 |  |  |
| Single DCI based SDM transmission for simultaneous reception support (singleDCI-SDM-scheme-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.7 |  |  |
| Multi DCI based simultaneous reception non-overlapping support (multiDCI-MultiTRP-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.5 |  |  |
| Multi DCI based simultaneous reception fully-overlapping support (overlapPDSCHsFullyFreqTime-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.6 |  |  |
| Support of 2-port DL PTRS (supportTwoPortDL-PTRS-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.7 Test 1-2 |  |  |

#### 7.1.1.4	Applicability of requirements for mandatory UE features with capability signalling

The performance requirements in Table 7.1.1.4-1 shall apply for UEs which support mandatory UE features with capability signalling only.

Table 7.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR2 TDD | PDSCH | Clause 7.2.2.2.1 (Tests from 2-1 to 2-6) | The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed UE PDSCH MIMO layers capability |
| Support of PT-RS with one antenna port for DL reception (onePortsPTRS) | FR2 TDD | PDSCH | Clause 7.2 |  |
|  |  | SDR | Clause 7.5.1 Clause 7.5A.1 |  |
| PCell operation on FR2 (pCell-FR2) | FR2 TDD | SDR | Clause 7.5A.1 |  |
| PDSCH mapping type B (pdsch-MappingTypeB) | FR2 TDD | PDSCH | Clause 7.2.2.2.3 |  |
| Support number of active TCI states per BWP per CC, including control and data (maxNumberActiveTCI-PerBWP) | FR2 TDD | PDSCH | Clause 7.2.2.2.4 (Test 1-2) | The requirements apply only when maxNumberActiveTCI-PerBWP is other than n1. |
| Support for maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC) | FR2 TDD | PDSCH | Clause 7.2.2.2.5 Clause 7.2.2.2.6 Clause 7.2.2.2.7 | The requirements apply only when maxSimultaneousResourceSetsPerCC ≥ 2 |

<< Unchanged sections omitted >>

#### 7.1.1.9	Applicability of different requirements with simultaneous reception

The applicability rules for different requirements with simultaneous reception transmission schemes in section 7 are specified in Table 7.1.1.9-1.

Table 7.1.1.9-1: Applicability of different requirements with simultaneous reception

| If UE has passed |  |  | UE can skip |  |  | Applicability notes |
| --- | --- | --- | --- | --- | --- | --- |
| Test type |  | Test list | Test type |  | Test list |  |
| FR2 TDD | PDSCH | Clause 7.2.2.2.7 (Test 1-1) | FR2 TDD | PDSCH | Clause 7.2.2.2.7 (Test 1-2) | If UE supports one PTRS port per TRxP configuration, UE is not tested for one PTRS port across TRxPs; |

<End of changes>