3GPP TSG-RAN WG4 Meeting #111	R4-2407117

# Fukuoka, Japan, 20-24 May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.878 | CR | 0004 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the required information analysis for advanced receiver for MU-MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh3-Perf |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN1#114 makes the following conclusion for CDM groups for DMRS: Conclusion The following specification in TS 38.214 is interpreted as the UE may assume that “CDM groups without data” are not used for data transmission for any co-scheduled user in the same serving cell. When receiving PDSCH scheduled by DCI format 1_1, the UE shall assume that the CDM groups indicated in the configured index from Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, 7.3.1.2.2-4 of [5, TS. 38.212] contain potential co-scheduled downlink DM-RS and are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) in Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, 7.3.1.2.2-4 of [5, TS. 38.212] correspond to CDM group 0, {0,1}, {0,1,2}, respectively.. The previous RAN4 default assumptions and required RRC signaling on ‘DMRS power boosting for the co-scheduled UE’ should be removed from the TR38.878, if RAN2#126 finally decided not to introduce RRC assistant signalling on ‘DMRS power boosting for the co-scheduled UE’. |
| Summary of change: |  | Removes the required RAN4 default assumption and RRC signalling for ‘DMRS power boosting for the co-scheduled UE’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There remains misaliment between RAN4 analysis outcome and RAN2 signaling design. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

4.3	Analysis on the required information

The agreed list of required information for advanced receiver is captured below:

| Information | RAN4 Default assumption (If N/A, how could be obtained by the UE) | Signalling if RAN4 default assumption not valid |
| --- | --- | --- |
| The DMRS port information for the co-scheduled UE | N/A (Obtained by UE blind detection) | N/A |
| PRB bundling size for the co-scheduled UE Frequency domain resource allocation for the co-UE within each PRG of the target UE | For the target and any co-scheduled UEs in different CDM groups and with the same DMRS sequence, the target UE assumes the precoding and resource allocation of the co-scheduled UE are the same in the PRG-level grid configured to the target UE when PRG=2 or 4. | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not |
| Time domain resource allocation information of the co-scheduled UE | Same as target UE | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not |
| Frequency domain resource allocation for the co-UE across different PRGs of the target UE: | N/A (Obtained by UE blind detection) | N/A |
| CSI-RS location of co-scheduled UE (Only required for R-ML) | UE assumes the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE | No RRC signalling is needed |
| Modulation order of co-scheduled UE | N/A Obtained by DCI based network assistance information or UE blind detection |  |

<End of change>