3GPP TSG-RAN WG4 Meeting #111  	R4-2407658

# Fukuoka, Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.761 | CR | 0007 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.761 on n28 lab alignment campaign |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT, SAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_OTA_enh |  |  |  |  |  | Date: |  |  | 2024-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 has successfully concluded the Rel-18 FR1 MIMO OTA lab alignment activity for bands < 1GHz. The outcome should be captured in the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Rel-18 FR1 MIMO OTA lab alignment activity measurement results and outcome. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TR would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of change 1 >

## 7.4	Lab alignment campaign for frequency bands < 1GHz

### 7.4.1	Framework

This clause defines the working procedure on how to proceed the lab alignment campaign for frequency bands < 1GHz.

1.	Test labs are invited to participate in the lab alignment activity, the following conditions should be fulfilled:

-	At least 3 participating labs are required.

-	Participating labs shall complete channel model validation.

-	Participating labs should have sufficient test resource to provide on-time measurement results without delay.

-	Participating labs should first examine and exclude the impact of noise before submitting PAD measurement results.

2.	Test methodology:

-	Test plan: 3GPP TS 38.151

-	Test system: MPAC

3.	Test cases for Lab Alignment Activity:

-	Test band: n28

-	Number of test cases: 3 PADs

-	Operation mode: NR Standalone (SA)

-	Use scenario: Free space

4.	Test results submission:

-	Use the same worksheet template in R4-2316308 to submit the measurement results. Report any systematic offsets that have been applied to the TRMS results. The lab alignment outcome should be based on the measurement results without power validation systematic offsets.

-	The measurement results should be submitted to RAN4 by anonymous approach (the UE model shall not be disclosed publicly)

-	Results shall not be shared between labs before submitting to RAN4 meetings or sharing in the RAN4 reflector. Comparison and lab alignment analysis should only be done in RAN4 meetings/discussions

5.	Lab alignment criteria:

-	The pass/fail criteria are defined as the maximum deviation between the measurement result and the reference value

-	The reference value will be derived based on the averaging approach (linear average in dBm) of lab alignment data pool from ≥ 3 labs

-	Apparent outliers (if identified) will not be considered in the average process for reference value. The PAD measurement result deviates over 1.5* MU (i.e., 3.92 dB for band n28) from all the other labs’ results should be identified as apparent outlier.

-	Pass/fail limit for lab alignment should be defined as 0.75*MU (+/- 1.96 dB for band n28).

6.	Volunteer lab procedures:

-	PAD delivery scheme: Decide PAD delivery scheme after all the volunteer labs and PADs information being confirmed.

-	PAD measurement time in each volunteer lab: finalize PAD measurement within [7] workdays, and deliver to the next lab ASAP with PAD delivery In/Out information shared in email-reflector.

### 7.4.2	Measurement results

RAN4 carried out the lab alignment campaign for frequency bands < 1GHz during Rel-18. RAN4 decided to select three smartphones (named as PADs, Performance Alignment Devices) as reference DUTs, and select band n28, to perform FR1 MIMO OTA lab alignment activity for frequencies bands < 1GHz.

There are 6 test labs participated in the lab alignment campaign, the reference value of each PAD at each band is derived based on linear average (with dBm) of PAD measurement results submitted by all labs. Channel model validation results and noise impact evaluations performed by the 6 labs are presented in Clauses 6.2.2 and 7.2. The summary of the lab alignment results is shown in Table 7.4.2-1 and Figure 7.4.2-1. The deviation between each TRMS measurement result and reference value is shown in Table 7.4.2-2 and Figure 7.4.2-1.

Table 7.4.2-1: Summary of FR1 MIMO OTA lab alignment results for bands < 1GHz

| Device | Band | TRMS measurement result [dBm/15kHz] |  |  |  |  |  | Average approach | Average value | Max-Min deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Lab 1 | Lab 2 | Lab 4 | Lab 5 | Lab 7 | Lab 6 |  |  |  |
| PAD_1 | n28 | -89.34 | -90.12 | -90.43 | -88.26 | -89.22 | -91.75 | Linear average | -89.85 | 3.49 |
| PAD_2 | n28 | -86.46 | -87.93 | -87.91 | -85.50 | -85.86 | -87.74 |  | -86.90 | 2.43 |
| PAD_3 | n28 | -86.26 | -89.34 | -89.23 | -86.19 | -86.49 | -88.36 |  | -87.65 | 3.15 |

Table 7.4.2-2: Deviations between TRMS measurement results and reference values

| Device | Band | TRMS offset [dBm/15kHz] |  |  |  |  |  | Pass/fail limit |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Lab 1 | Lab 2 | Lab 4 | Lab 5 | Lab 7 | Lab 6 |  |  |  |
| PAD_1 | n28 | 0.52 | -0.27 | -0.58 | 1.59 | 0.63 | -1.90 | +/- 0.75 MU, i.e., +/- 1.96 dB |  |  |
| PAD_2 | n28 | 0.44 | -1.03 | -1.01 | 1.40 | 1.04 | -0.84 |  |  |  |
| PAD_3 | n28 | 1.38 | -1.69 | -1.59 | 1.46 | 1.15 | -0.71 |  |  |  |
| Lab alignment conclusion |  | Pass | Pass | Pass | Pass | Pass | Pass |  |  |  |

Figure 7.4.2-1: Measurement results of FR1 MIMO OTA lab alignment for bands < 1GHz

Figure 7.4.2-2: Deviation between each measurement result and reference value

The maximum deviation between measurement results and reference values is -1.9 dB.

### 7.4.3	Pass/fail limits

Based on the MU assessment of FR1 MPAC system in Annex A.2.5 of TS 38.151 V18.0.0, and lab alignment measurement results in Clause 7.4.2, RAN4 decided the final pass/fail limits for FR1 MIMO OTA lab alignment activity for bands < 1GHz as 0.75* MU, i.e., +/-1.96 dB for band n28.

### 7.4.4	Conclusion

RAN4 concluded the successful FR1 MIMO OTA lab alignment activity for bands < 1GHz, all the 6 test labs are aligned. FR1 MIMO OTA performance requirements for bands < 1GHz are specified based on the measurement results submitted by the above aligned test labs.

< end of change 1 >