3GPP TSG RAN meeting #104										RP-241496

Shanghai, China, June 17-20, 2024

## Status Report to TSG

Agenda item:			9.3.4.4

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FR1_lessthan_5MHz_BW_Ph2 |  |  |  |  |
| Unique ID | 1030081 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-240832 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: Dec./2024 | Performance part: June/2025 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 25% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | R4 |
| --- | --- | --- |
| Rapporteur | Name | Zhang, Meng |
|  | Company | Intel Corporation |
|  | Email | meng.zhang@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	Decisions during RAN4#110bis

- The agreements for UE RF requirements for inter-band NR CA/DC with less than 5MHz CBW are shown below (R4-2406623):

  - NR CA configuration:

    - Define new inter-band CA operating band combination:

| NR CA Band | NR Band |
| --- | --- |
| CA_n100-n101 | n100, n101 |

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n100A-n101A | CA_n100A-n101A | n100 | 3 | 0 |
|  |  | n101 | 5, 10 |  |

    - 5MHz bandwidth can be added for n100 in the band combination after RAN plenary agrees to add it in the WID.

  - NR DC configuration:

    - Define new inter-band DC configuration:

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n100A-n101A | DC_n100A-n101A |

  - Transmission bandwidth configuration for 3MHz and 5MHz CBW for band n100:

    - Define RF requirements agnostic of transmission bandwidth configurations for 3MHz CBW.

  - Power class for UL inter-band CA:

    - Define PC3 inter-band CA/DC combinations.

    - FFS on whether to define PC1 inter-band CA/DC combinations.

  - Signalling aspects:

    - FFS whether to introduce new UE capability and/or other RRC signalling to support NR CA/DC with less than 5MHz CBW and support of 12 or 20 RB transmission bandwidth.

  - UE RF specification impact for inter-band CA (CA_n100A-n101A) for TS 38.101-1:

    - Define the following requirements for NR CA operation for CA_n100A-n101A:

| Requirement | Required change for NR CA |
| --- | --- |
| General |  |
| 5.2A Operating bands | Specify inter-band CA operating band |
| 5.3 UE channel bandwidth | Remove the restriction that limited 3MHz CBW to single carrier case |
| 5.5A Configurations for CA | Specify new NR CA configurations and bandwidth combinations sets for inter-band CA |
| TX requirements |  |
| 6.2A.1 UE maximum output power for CA | Specify UE Power Class for uplink inter-band CA combination (Table 6.2A.1.3-1) |
| 6.2A.4 Configured transmitted power for CA | Specify ΔTIB,c for uplink inter-band CA combination (Table 6.2A.4.2.3-1) |
| 6.5A Output RF spectrum emissions for CA | Specify spurious emissions for UE coexistence for uplink inter-band CA combination (Table 6.5A.3.2.3-1) |
| RX requirements |  |
| 7.3A.2 Reference sensitivity for CA | Specify ΔRIB,c for inter-band CA combination (Table 7.3A.3.2.1-1) |

    - For the following requirements there is no impact for CA_n100A-n101A, but they may need to be assessed for other combinations involving CBW with less than 5MHz:

| Requirement | Required change for NR CA |
| --- | --- |
| RX requirements |  |
| 7.6A Blocking characteristics for CA |  |

  - UE RF specification impact for NR DC (DC_n100A-n101A) for TS 38.101-1:

    - Define the following requirements for NR DC operation for DC_n100A-n101A.

| Requirement | Required change for NR DC |
| --- | --- |
| General |  |
| 5.5B Configurations for DC | Specify new NR DC configurations |
| TX requirements |  |
| 6.2B.1 UE maximum output power for NR-DC | Specify UE Power Class for uplink inter-band DC combination (Table 6.2B.1.3-1) |

    - Other requirements for DC_n100A-n101A are same as for CA_n100-n101.

  - Configured transmitted power for Inter-band CA: ΔTIB,c:

    - ΔTIB,c  is FFS:

      - Option 1: ΔTIB,c = [0.3] dB.

      - Other options are not precluded.

  - Reference sensitivity: ΔRIB,c:

    - ΔRIB,c  = 0 dB.

  - Reference sensitivity: MSD requirements:

    - FFS on MSD requirements.

- The way forward on clarification of multiple carrier operation for n100/n101 are shown below (R4-2406624):

  - During RAN4#111, RAN4 to reflect the “multiple carriers using wideband technologies” wording and related considerations on “coordination procedure or other mitigation measures” from Decides 4 in ECC(20)02, into Rel-17 versions of TS 38.104 and TS 38.141-1, where the exact wording to be implemented in related CRs is FFS.

  - Further inputs on multi-carrier/CA aspects of ECC(20)02 (and ECC report 313, ECC report 314) are encouraged.

##### 2.4.1.2	Decisions during RAN4#111

- The agreements for UE RF requirements for inter-band NR CA/DC with less than 5MHz CBW are shown below (R4-2410602):

  - Support of 5MHz CBW in band n100 for CA_n100-n101 and NR DC_n100-n101:

    - In RAN4 common understanding, the support of 5MHz CBW on band n100 and 5MHz/10MHz on n101 in the band combination with band n100 and n101 can be included in Rel-19 WID.

  - ΔTIB,c  for CA_n100-n101 and and NR DC_n100-n101:

    - ΔTIB,c = 0.3 dB for CA_n100-n101 and NR DC_n100-n101

| Inter-band CA combination | ΔTIB,c for NR bands (dB) |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration |  |
| CA_n100-n101 | 0.3 | 0.3 |

  - MSD requirements:

    - Do not specify MSD requirements for CA_n100-n101 and NR DC_n100-n101.

    - MSD requirements for other band combinations for BW less than 5 MHz can be discussed in the basket WI(s).

  - Spurious emissions for UE co-existence for CA_n100-n101 and and NR DC_n100-n101:

    - Spurious emissions for UE co-existence for CA_n100-n101

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n100-n101 | Frequency range | 758 | - | 788 | -50 | 1 |  |

  - Power class for UL inter-band CA for CA_n100-n101:

    - Whether to define PC1 for UL inter-band CA for CA_n100-n101 can be discussed in RANP.

  - UE capability and network signalling:

    - FFS whether any updates to Rel-18 UE capability signalling are needed to enable less than 5 MHz CBW operation for CA/DC.

    - Further potential enhancements can be further discussed for SCell with less than 5MHz.

      - FFS whether UEs needs to be informed that SCell has 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW, respectively.

  - Sync raster applicability for SCell with < 5MHz CBW for CA_n100-n101:

    - FFS whether any restrictions on ARFCN for SSB locations for SCell SSB need to be defined

      - Option 1: Require less than 5 MHz SCell to be associated with the new sync raster points for less than 5MHz, similar as PCell.

      - Other options are not precluded.

  - CR handling:

    - Rapporteur (Intel) will prepare a single draft CR for RAN4 #112 covering the agreements in RAN4 #110bis and #111 and share before the RAN4 #112 with interested companies.

#### 2.4.2	Remaining Open issues

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band.

  - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101.

- Define RRM requirements for inter-band CA and DC for combinations introduced in RF part.

- Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

###### RAN4 #110bis

- R4-2404257 DC CA UE Requirements in the case of 3MHz CBW	Huawei Technologies France

- R4-2404288 UE RF specification impact for inter-band NR CA/DC with 3MHz CBW	Nokia

- R4-2404453 On UE RF requirements for inter-band NR CA/DC with 3MHz CBW	CATT

- R4-2404500 draftCR for inter-band NR CA/DC with 3MHz CBW (System parameter part)	CATT

- R4-2404735 Workplan for Rel-19 NR channel BW less than 5MHz for TN work item	Intel Corporation

- R4-2404736 Discussion on the RF spec scope of Rel-19 less than 5MHz work item for TN	Intel Corporation, UIC

- R4-2404784 Discussion for UE RF requirements for inter-band NR CA/DC with 3MHz CBW	LG Electronics France

- R4-2405062 Discussion on UE RF requirements inter-band NR CA/DC with 3MHz CBW	ZTE Corporation

- R4-2405287 Topic summary for [110bis][134] NR_FR1_5MHz_BW_Ph2	Moderator(Intel)

- R4-2405714 UE RF impacts from NR less than 5 MHz phase 2	Qualcomm Inc.

- R4-2405765 TP adding 3MHz example combination CA_n100-n101	Ericsson

- R4-2405766 draft CR adding 3MHz example combination CA_n100-n101	Ericsson

- R4-2405912 Analysis of the ECC regulation for multi-carrier operation in n100/n101	Huawei, HiSilicon, Vodafone, Orange, Telia Company, Telefónica, Deutsche Telekom

- R4-2405913 Draft CR to TS 38.104: Clarification on multiple carrier operation for n100/n101, Rel-17 Huawei, HiSilicon

- R4-2405914 Draft CR to TS 38.104: Clarification on multiple carrier operation for n100/n101, Rel-18 Huawei, HiSilicon

- R4-2405915 Draft CR to TS 38.141-1: Clarification on multiple carrier operation for n100/n101, Rel-17 Huawei, HiSilicon

- R4-2405916 Draft CR to TS 38.141-1: Clarification on multiple carrier operation for n100/n101, Rel-18 Huawei, HiSilicon

- R4-2406622 Analysis of the ECC regulation for multi-carrier operation in n100/n101 Huawei, HiSilicon, Vodafone, Orange, Telia Company, Telefónica, Deutsche Telekom

- R4-2406623 WF on RF requirements for less than 5MHz band combinations	Intel

- R4-2406624 WF on clarification of multiple carrier operation for n100/n101	Huawei

###### RAN4 #111

- R4-2407028 Clarification of WID RP-240832 - NR_FR1_5MHz_BW_Ph2	Union Inter. Chemins de Fer

- R4-2407439 UE RF requirements for inter-band NR CA/DC with 3MHz CBW	Nokia

- R4-2407548 Further discussion on UE RF requirements for inter-band NR CA/DC with 3MHz CBW	CATT

- R4-2407549 draftCR for inter-band NR CA&DC with 3MHz CBW (System parameter part)	CATT

- R4-2407991 Discussion for UE RF requirements for inter-band NR CA/DC with 3MHz CBW	LG Electronics France

- R4-2408480 Discussion on the RF requirements for Rel-19 less than 5MHz work item for TN phase 2	Intel Corporation

- R4-2408797 Discussion on UE RF requirements for inter-band NR CADC with 3MHz CBW	ZTE Corporation, Sanechips

- R4-2408798 draftCR for TS 38.101-1: Introduction of inter-band NR CA/DC with less than 5MHz CBW	ZTE Corporation, Sanechips

- R4-2408814 draft CR to TS 38.101-1: Introduction of CA_n100-n101	Qualcomm Inc.

- R4-2408815 Scell bandwidth and sync raster	Qualcomm Inc.

- R4-2408943 Topic summary for [111][132] NR_FR1_5MHz_BW_Ph2	Moderator(Intel)

- R4-2409152 Remaining RF requirements for CA_n100-n101 and  DC_n100-n101 with 3MHz CBW	Huawei, HiSilicon

- R4-2409153 Remaining RF requirements for CA_n100-n101 and  DC_n100-n101 with 3MHz CBW	Huawei, HiSilicon

- R4-2410602 WF on UE RF requirements for Rel-19 less than 5MHz	Intel

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template