3GPP TSG-RAN WG2 Meeting #126	 R2-2405981

# Fukuoka, Japan , May 20th – 24th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0503 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections on LPP for Rel-18 positioning UE capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To capture following agreements made in RAN2#125bis: Move the two fields posSRS-BWA-RRC-Connected-r18 and posSRS-BWA-IndependentCA-RRC-Connected-r18 from IE SRS-CapabilityPerBand-r16 to SRS-PosResourcesPerBand-r16. To update FG 41-4-6, FG 41-4-7 and FG41-4-8 according to R1-2403703 updated RAN1 UE feature list for Rel-18 NR after RAN1 116bis. The featue ‘symbolTimeStampSupport-r18’ is not in the RAN1 UE feature list. To capture the following agreements made in RAN2#126: - Replace the field name supportOfLegacyMeasurementInTimeWindow by supportOfMeasurementInTimeWindowForDL-TDOA and similarly for other methods. - Delete nr-DL-AoD-OnDemandPRS-ForBWA-Support-r18 IE from NR-DL-AoD-ProvideCapabilities. - Delete the words “within a MG” from the field descriptions for the IEs prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleAndInactive and prs-BWA-ThreeContiguousIntrabandInMG-RRC-IdleAndInactive. - In LPP CR revise the field descriptions of reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-Connected and reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive to make it condition to the support of component 1 of R1 FG 41-4-1. - In LPP CR fix the editorial issues in the field descriptions of posSRS-Preconfigured-RRC-InactiveInitialUL-BWP, posSRS-Preconfigured-RRC-InactiveOutsideInitialUL-BWP, posSRS-ValidityAreaRRC-InactiveInitialUL-BWP, posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP and posSRS-TxFH-RRC-Inactive. To update the prerequisite feature groups of RAN4 UE features according to the R4-2410748 LS on RAN4 UE feature list for Rel-18 (version 5). To update the UE capabilities according to RAN1-2405566 LS on Rel-18 RAN1 UE features list for NR after RAN1#117. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 Move the two fields posSRS-BWA-RRC-Connected-r18 and posSRS-BWA-IndependentCA-RRC-Connected-r18 from IE SRS-CapabilityPerBand-r16 to SRS-PosResourcesPerBand-r16. 2 Update component 2 and notes of FG41-4-6. 3 Update component 2, component 9 and notes of FG 41-4-7. 4 Update component 2, component 9 and notes of FG 41-4-8. 5.Remove the symbolTimeStampSupport-r18. 6 Update supportOfLegacyMeasurementInTimeWindow. 7 Delete nr-DL-AoD-OnDemandPRS-ForBWA-Support-r18 IE from NR-DL-AoD-ProvideCapabilities. 8 Update the name of prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleAndInactive and prs-BWA-ThreeContiguousIntrabandInMG-RRC-IdleAndInactive, and update the corresponding field description. 9 Update the prerequisite feature groups of reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-Connected-r18, reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive-r18, reducedNumOfSampleForMeasurementWithFH-RRC-Connected-r18 and reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive-r18. 10 Fix the editorial issues in the field descriptions of posSRS-Preconfigured-RRC-InactiveInitialUL-BWP, posSRS-Preconfigured-RRC-InactiveOutsideInitialUL-BWP, posSRS-ValidityAreaRRC-InactiveInitialUL-BWP, posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP and posSRS-TxFH-RRC-Inactive. 11 Move the supportOfPRS-MeasurementRRC-Idle-r18 from NR-DL-PRS-ProcessingCapability to NR-DL-TDOA-MeasurementCapability and NR-DL-AoD-MeasurementCapability. 12 Update the ASN.1 and corresponding field description of PosSRS-BWA-RRC-Connected-r18, PosSRS-BWA-IndependentCA-RRC-Connected-r18 and PosSRS-BWA-RRC-Inactive. 13. Move the supportOfRSCPD-MeasurementInTimeWindow-r18 and supportOfRSCP-MeasurementInTimeWindow-r18 into supportOfMeasurementsInTimeWindow-r18 respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE capabilities for Rel-18 positioning are not captured correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3, 6.5.10.6, 6.5.10.6a, 6.5.11.6, 6.5.11.6a, 6.5.12.6, 6.5.12.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R2-2404595 |  |  |  |  |  |  |  |  |

START OF CHANGE

6.4.3	Common NR Positioning Information Elements

–	NR-DL-PRS-ProcessingCapability

The IE NR-DL-PRS-ProcessingCapability defines the common DL-PRS Processing capability. In the case of capabilities for multiple NR positioning methods are provided, the IE NR-DL-PRS-ProcessingCapability applies across the NR positioning methods and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities.

The PRS-ProcessingCapabilityPerBand is defined for a single positioning frequency layer on a certain band (i.e., a target device supporting multiple positioning frequency layers is expected to process one frequency layer at a time).

-- ASN1START

NR-DL-PRS-ProcessingCapability-r16 ::= SEQUENCE {

prs-ProcessingCapabilityBandList-r16	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

PRS-ProcessingCapabilityPerBand-r16,

maxSupportedFreqLayers-r16				INTEGER (1..4),

simulLTE-NR-PRS-r16						ENUMERATED { supported }		OPTIONAL,

...,

[[

dummy									ENUMERATED { m1, m2, ... }		OPTIONAL

]]

}

PRS-ProcessingCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16				FreqBandIndicatorNR-r16,

supportedBandwidthPRS-r16			CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400},

...

},

dl-PRS-BufferType-r16				ENUMERATED {type1, type2, ...},

durationOfPRS-Processing-r16		SEQUENCE {

durationOfPRS-ProcessingSymbols-r16	ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r16

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280},

...

},

maxNumOfDL-PRS-ResProcessedPerSlot-r16	SEQUENCE {

scs15-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs30-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs60-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs120-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

...,

[[

scs15-v1690								ENUMERATED {n6, n12}					OPTIONAL,

scs30-v1690								ENUMERATED {n6, n12}					OPTIONAL,

scs60-v1690								ENUMERATED {n6, n12}					OPTIONAL,

scs120-v1690							ENUMERATED {n6, n12}					OPTIONAL

]]

},

...,

[[

supportedDL-PRS-ProcessingSamples-RRC-CONNECTED-r17	ENUMERATED { supported }		OPTIONAL,

prs-ProcessingWindowType1A-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingWindowType1B-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingWindowType2-r17			ENUMERATED { option1, option2, option3}		OPTIONAL,

prs-ProcessingCapabilityOutsideMGinPPW-r17

SEQUENCE (SIZE(1..3)) OF

PRS-ProcessingCapabilityOutsideMGinPPWperType-r17

OPTIONAL,

dl-PRS-BufferType-RRC-Inactive-r17		ENUMERATED { type1, type2, ... }			OPTIONAL,

durationOfPRS-Processing-RRC-Inactive-r17	SEQUENCE {

durationOfPRS-ProcessingSymbols-r17			ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r17

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280},

...

}																					OPTIONAL,

maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17	SEQUENCE {

scs15-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs30-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs60-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs120-r17								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

...

}																					OPTIONAL,

supportedLowerRxBeamSweepingFactor-FR2-r17	ENUMERATED { n1, n2, n4, n6 }			OPTIONAL

]],

[[

supportedDL-PRS-ProcessingSamples-RRC-Inactive-r17	ENUMERATED { supported }		OPTIONAL

]],

[[

prs-MeasurementWithoutMG-r17				ENUMERATED {cp, symbolDot25, symbolDot5,

slotDot5}					OPTIONAL

]],

[[

maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Inactive-r18	SEQUENCE {

scs15-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs30-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs60-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs120-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

...

}																					OPTIONAL,

maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Connected-r18	SEQUENCE {

scs15-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs30-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs60-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs120-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

...

}																					OPTIONAL,

ppw-maxNumOfOneSymbolPRS-ResProcessedPerSlot-r18	SEQUENCE {

scs15-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs30-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs60-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

scs120-r18								ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24,

n32, n48, n64}				OPTIONAL,

...

}																					OPTIONAL,

prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected-r18

PRS-BWA-TwoContiguousIntrabandInMG-r18		OPTIONAL,

prs-BWA-ThreeContiguousIntrabandInMG-RRC-Connected-r18

PRS-BWA-ThreeContiguousIntrabandInMG-r18	OPTIONAL,

prs-BWA-TwoContiguousIntraband-RRC-IdleAndInactive-r18

PRS-BWA-TwoContiguousIntrabandInMG-r18		OPTIONAL,

prs-BWA-ThreeContiguousIntraband-RRC-IdleAndInactive-r18

PRS-BWA-ThreeContiguousIntrabandInMG-r18	OPTIONAL,

reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-Connected-r18		ENUMERATED { supported }

OPTIONAL,

reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive-r18

ENUMERATED { supported }			OPTIONAL,

dl-PRS-MeasurementWithRxFH-RRC-Inactive-r18		ENUMERATED { supported }			OPTIONAL,

dl-PRS-MeasurementWithRxFH-RRC-Idle-r18			ENUMERATED { supported }			OPTIONAL,

reducedNumOfSampleForMeasurementWithFH-RRC-Connected-r18	ENUMERATED { supported }

OPTIONAL,

reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive-r18	ENUMERATED { supported }

OPTIONAL,

supportOfPRS-BWA-WithTwoPFL-Combination-r18			ENUMERATED { supported }	OPTIONAL,

dl-PRS-MeasurementWithRxFH-RRC-Connected-r18	DL-PRS-MeasurementWithRxFH-RRC-Connected-r18																						OPTIONAL

]]

}

PRS-ProcessingCapabilityOutsideMGinPPWperType-r17 ::= SEQUENCE {

prsProcessingType-r17						ENUMERATED { type1A, type1B, type2 },

ppw-dl-PRS-BufferType-r17					ENUMERATED { type1, type2, ... },

ppw-durationOfPRS-Processing1-r17			SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms4,

ms6, ms8, ms12, ms16, ms20, ms25, ms30, ms32, ms35,

ms40, ms45, ms50 },

ppw-durationOfPRS-ProcessingSymbolsT-r17

ENUMERATED { ms1, ms2, ms4, ms8, ms16, ms20, ms30, ms40, ms80,

ms160, ms320, ms640, ms1280 }

}																					OPTIONAL,

ppw-durationOfPRS-Processing2-r17			SEQUENCE {

ppw-durationOfPRS-ProcessingSymbolsN2-r17

ENUMERATED { msDot125, msDot25, msDot5, ms1, ms2, ms3, ms4, ms5,

ms6, ms8, ms12 },

ppw-durationOfPRS-ProcessingSymbolsT2-r17

ENUMERATED { ms4, ms5, ms6, ms8 }

}																					OPTIONAL,

ppw-maxNumOfDL-PRS-ResProcessedPerSlot-r17	SEQUENCE {

scs15-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs30-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs60-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

scs120-r17									ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }

OPTIONAL,

...

},

...,

[[

ppw-maxNumOfDL-Bandwidth-r17		CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}																					OPTIONAL

]]

}

PRS-BWA-TwoContiguousIntrabandInMG-r18 ::= SEQUENCE {

maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1-r18	ENUMERATED {mhz10, mhz20, mhz40, mhz50,

mhz80, mhz100, mhz160, mhz200}

OPTIONAL,

maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2-r18	ENUMERATED {mhz100, mhz200, mhz400, mhz800}

OPTIONAL,

maximumOfDL-PRS-BandwidthPerPFL-FR1-r18			ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100}	OPTIONAL,

maximumOfDL-PRS-BandwidthPerPFL-FR2-r18			ENUMERATED {mhz50, mhz100, mhz200, mhz400}

OPTIONAL,

dl-PRS-BufferTypeOfBWA-r18						ENUMERATED {type1, type2},

prs-durationOfTwoPRS-BWA-Processing-r18			SEQUENCE {

prs-durationOfTwoPRS-BWA-ProcessingSymbolsN-r18

ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms4, ms6, ms8, ms12,

ms16, ms20, ms25, ms30, ms32, ms35, ms40, ms45, ms50},

prs-durationOfTwoPRS-BWA-ProcessingSymbolsT-r18

ENUMERATED {ms8, ms16, ms20, ms30, ms40, ms80, ms160, ms320, ms640, ms1280}

}																					OPTIONAL,

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18	SEQUENCE {

scs15-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs30-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs60-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL

},

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18	SEQUENCE {

scs60-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs120-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL

}

}

PRS-BWA-ThreeContiguousIntrabandInMG-r18 ::= SEQUENCE {

maximumOfThreeAggregatedDL-PRS-Bandwidth-FR1-r18

ENUMERATED {mhz15, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz120,

mhz140, mhz150, mhz180, mhz200, mhz240, mhz300}		OPTIONAL,

maximumOfThreeAggregatedDL-PRS-Bandwidth-FR2-r18

ENUMERATED {mhz150, mhz200, mhz300, mhz400, mhz600, mhz800, mhz1000,

mhz1200}											OPTIONAL,

maximumOfDL-PRS-BandwidthPerPFL-FR1-r18

ENUMERATED {mhz5, mhz10, mhz20, mhz40, mhz50, mhz80, mhz100}	OPTIONAL,

maximumOfDL-PRS-BandwidthPerPFL-FR2-r18

ENUMERATED {mhz50, mhz100, mhz200, mhz400}						OPTIONAL,

dl-PRS-BufferTypeOfBWA-r18							ENUMERATED {type1, type2},

prs-durationOfThreePRS-BWA-Processing-r18			SEQUENCE {

prs-durationOfThreePRS-BWA-ProcessingSymbolsN-r18

ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2, ms4, ms6, ms8, ms12,

ms16, ms20, ms25, ms30, ms32, ms35, ms40, ms45, ms50},

prs-durationOfThreePRS-BWA-ProcessingSymbolsT-r18

ENUMERATED {ms8, ms16, ms20, ms30, ms40, ms80, ms160,

ms320, ms640, ms1280}

}																					OPTIONAL,

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18	SEQUENCE {

scs15-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs30-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs60-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL

},

maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18	SEQUENCE {

scs60-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL,

scs120-r18						ENUMERATED {n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }		OPTIONAL

}

}

DL-PRS-MeasurementWithRxFH-RRC-Connected-r18 ::=SEQUENCE {

maximumPRS-BandwidthAcrossAllHopsFR1-r18	ENUMERATED {mhz40, mhz50, mhz80, mhz100}

OPTIONAL,

maximumPRS-BandwidthAcrossAllHopsFR2-r18	ENUMERATED {mhz100, mhz200, mhz400}		OPTIONAL,

maximumFH-Hops-r18							ENUMERATED {n2, n3, n4, n5, n6}		OPTIONAL,

processingDuration-r18	SEQUENCE {

processingPRS-SymbolsDurationN3-r18		ENUMERATED {msDot125, msDot25, msDot5, ms1, ms2,

ms4, ms6, ms8, ms12,ms16, ms20, ms25,

ms30, ms32, ms35, ms40, ms45, ms50},

processingDurationT3-r18				ENUMERATED {ms8, ms16, ms20, ms30, ms40, ms80,

ms160, ms320, ms640, ms1280}

}																					OPTIONAL,

rf-RxRetunTimeFR1-r18						ENUMERATED {n70,n140,n210}				OPTIONAL,

rf-RxRetunTimeFR2-r18						ENUMERATED {n35,n70,n140}				OPTIONAL,

numOfOverlappingPRB-r18						ENUMERATED {n0,n1,n2,n4}				OPTIONAL,

...

}

-- ASN1STOP

NR-DL-PRS-ProcessingCapability field descriptions

maxSupportedFreqLayers Indicates the maximum number of positioning frequency layers supported by UE.

simulLTE-NR-PRS Indicates whether the UE supports parallel processing of LTE PRS and NR DL-PRS.

dummy This field is not used in the specification. If received it shall be ignored by the receiver.

supportedBandwidthPRS Indicates the maximum number of DL-PRS bandwidth in MHz, which is supported and reported by UE.

dl-PRS-BufferType Indicates DL-PRS buffering capability. Value type1 indicates sub-slot/symbol level buffering and value type2 indicates slot level buffering.

durationOfPRS-Processing Indicates the duration N of DL-PRS symbols in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in supportedBandwidthPRS and comprises the following subfields: - durationOfPRS-ProcessingSymbols: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - durationOfPRS-ProcessingSymbolsInEveryTms: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. See NOTE 9.

maxNumOfDL-PRS-ResProcessedPerSlot Indicates the maximum number of DL-PRS Resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands.

supportedDL-PRS-ProcessingSamples-RRC-CONNECTED Indicates the UE capability for support of measurements based on measuring M=1 or M=2 (instances) of a DL-PRS Resource Set. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 1: This feature is supported for both UE-assisted and UE based positioning.

prs-ProcessingWindowType1A Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1A refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the DL-PRS Processing Window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR. Enumerated value indicates supported priority handing options of DL-PRS: - option1: Support of "st1" and "st3" defined in clause 5.1.6.5 of TS 38.214 [45]. - option2: Support of "st1", "st2", and "st3" defined in clause 5.1.6.5 of TS 38.214 [45]. - option3: Support of "st1" only defined in clause 5.1.6.5 of TS 38.214 [45]. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 2: Within a DL-PRS processing window, UE measurement is inside the active DL BWP with DL-PRS having the same numerology as the active DL BWP. NOTE 2a: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS.

prs-ProcessingWindowType1B Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1B refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the DL-PRS processing window. The DL signals/channels from a certain band are affected. Enumerated value indicates supported priority handing options of DL-PRS (see prs-ProcessingWindowType1A). The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 3: Within a DL-PRS processing window, UE measurement is inside the active DL BWP with DL-PRS having the same numerology as the active DL BWP. NOTE 3a: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS.

prs-ProcessingWindowType2 Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 2 refers to the determination of prioritization between DL-PRS and other DL signals/channels only in DL-PRS symbols within the DL-PRS processing window. Enumerated value indicates supported priority handing options of DL-PRS (see prs-ProcessingWindowType1A). The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. NOTE 4: Within a DL-PRS processing window, UE measurement is inside the active DL BWP with DL-PRS having the same numerology as the active DL BWP. NOTE 4a: When the UE determines higher priority for other DL signals/channels over the DL-PRS measurement/processing, the UE is not expected to measure/process DL-PRS.

prs-ProcessingCapabilityOutsideMGinPPW Indicates the DL-PRS Processing Capability outside MG of each of the supported PPW Type in the case the UE supports multiple PPW Types in a band and comprises the following subfields: - prsProcessingType: Indicates the DL-PRS Processing Window Type for which the prs-ProcessingCapabilityOutsideMGinPPW are provided. - ppw-dl-PRS-BufferType: Indicates DL-PRS buffering capability. Value 'type1' indicates sub-slot/symbol level buffering and value 'type2' indicates slot level buffering. - ppw-durationOfPRS-Processing1: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth and comprises the following subfields: - ppw-durationOfPRS-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - ppw-durationOfPRS-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. - ppw-durationOfPRS-Processing2: Indicates the duration of DL-PRS symbols N2 in units of ms a UE can process inT2 ms assuming maximum DL-PRS bandwidth provided in ppw-maxNumOfDL-Bandwidth and comprises the following subfields: - ppw-durationOfPRS-ProcessingSymbolsN2: This field specifies the values for N2. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12 ms. - ppw-durationOfPRS-ProcessingSymbolsT2: This field specifies the values for T2. Enumerated values indicate 4, 5, 6, 8 ms. - ppw-maxNumOfDL-PRS-ResProcessedPerSlot: Indicates the maximum number of DL-PRS resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. - ppw-maxNumOfDL-Bandwidth: Indicates the maximum number of DL-PRS bandwidth in MHz, which is supported and reported by UE for DL-PRS measurement outside MG within the PPW. The UE can include this field only if the UE supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B and prs-ProcessingWindowType2. Otherwise, the UE does not include this field. NOTE 5: A UE that supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B or prs-ProcessingWindowType2 shall always include the prs-ProcessingCapabilityOutsideMGinPPW. NOTE 6: The (N, T) UE capability in ppw-durationOfPRS-Processing1 is interpreted as in NOTE 9, and the UE is expected to receive the DL-PRS within the DL-PRS processing window but the processing of the received DL-PRS may be outside a DL-PRS processing window. NOTE 7: The (N2, T2) UE capability in ppw-durationOfPRS-Processing2 is interpreted such that the UE is capable of measuring up to N2 ms DL-PRS within a PPW and is capable of completing the DL-PRS processing within the PPW, e.g., if the time duration from the last symbol of the measured DL-PRS Resource(s) inside the PPW to the end of PPW is not smaller than T2 ms. NOTE 8: A UE which supports prs-ProcessingCapabilityOutsideMGinPPW shall support either ppw-durationOfPRS-Processing1 or ppw-durationOfPRS-Processing2, but not both for each supported type in a band.

dl-PRS-BufferType-RRC-Inactive Indicates DL-PRS buffering capability in RRC_INACTIVE state. Value 'type1' indicates sub-slot/symbol level buffering and value 'type2' indicates slot level buffering.

durationOfPRS-Processing-RRC-Inactive Indicates the duration N of DL-PRS symbols in units of ms a UE can process every T ms in RRC_INACTIVE state assuming maximum DL-PRS bandwidth provided in supportedBandwidthPRS and comprises the following subfields: - durationOfPRS-ProcessingSymbols: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - durationOfPRS-ProcessingSymbolsInEveryTms: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. See NOTE 9.

maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive Indicates the maximum number of DL-PRS Resources a UE can process in a slot in RRC_INACTIVE state. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands.

supportedLowerRxBeamSweepingFactor-FR2 Indicates support of the lower Rx beam sweeping factor than 8 for FR2. Enumerated value indicates the number of Rx beam sweeping factors supported.

supportedDL-PRS-ProcessingSamples-RRC-Inactive Indicates the UE capability for support of reduced number of samples for DL-PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports prs-ProcessingRRC-Inactive defined in TS 38.331 [35]. Otherwise, the UE does not include this field.

maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Inactive Indicates the maximum number of single-symbol DL-PRS Resources that UE can process in a slot in RRC_INACTIVE. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. A UE which supports maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Inactive-r18 shall support single-symbol DL-PRS with the comb sizes from {2,4,6,12}. The UE can include this field only if the UE supports one of dl-PRS-BufferType-RRC-Inactive, durationOfPRS-Processing-RRC-Inactive, and maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive. Otherwise, the UE does not include this field.

maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Connected Indicates the maximum number of single-symbol DL-PRS Resources that UE can process in a slot inside a measurement gap in RRC_CONNECTED. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. A UE which supports maxNumOfOneSymbolPRS-ResProcessedPerSlot-RRC-Connected-r18 shall support single-symbol DL-PRS with the comb sizes from {2,4,6,12}. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field.

ppw-maxNumOfOneSymbolPRS-ResProcessedPerSlot Indicates the maximum number of single-symbol DL-PRS Resources that UE can process in a slot outside a measurement gap in RRC_CONNECTED. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands. A UE which supports ppw-maxNumOfOneSymbolPRS-ResProcessedPerSlot-r18 shall support single-symbol DL-PRS with the comb sizes from {2,4,6,12}. The UE can include this field only if the UE supports prs-ProcessingCapabilityOutsideMGinPPW. Otherwise, the UE does not include this field.

prs-MeasurementWithoutMG Indicates the UE capability for support of Rx timing difference between the serving cell and non-serving cell for DL-PRS measurement within a PPW. Value 'cp' indicates one CP length, value 'symbolDot25' indicates 0.25 symbol length, value 'symbolDot5' indicates 0.5 symbol length and value 'slotDot5' indicates 0.5 slot length. The UE can include this field only if the UE supports one of prs-ProcessingWindowType1A, prs-ProcessingWindowType1B and prs-ProcessingWindowType2. Otherwise, the UE does not include this field.

prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected Indicates the UE capability for support of DL-PRS processing capabilities for aggregated DL-PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_CONNECTED state and and comprises the following subfields: - maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR1, which is supported and reported by UE. - maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR2, which is supported and reported by UE. - maximumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL-PRS bandwidth in MHz for FR1, per PFL. - maximumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL-PRS bandwidth in MHz for FR2, per PFL. - dl-PRS-BufferTypeOfBWA: Indicates the DL-PRS buffering capability. - prs-durationOfTwoPRS-BWA-Processing: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL-PRS bandwidth in MHz, which is supported and reported by UE. - prs-durationOfTwoPRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - prs-durationOfTwoPRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 2560 ms. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR1. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR2. The UE can include this field only if the UE supports supportedBandwidthPRS, dl-PRS-BufferType, durationOfPRS-Processing and maxNumOfDL-PRS-ResProcessedPerSlot. Otherwise, the UE does not include this field. NOTE 10: dl-PRS-BufferTypeOfBWA follows buffering capability type reported in dl-PRS-BufferType. NOTE 11: The value N should be equal or smaller than the value N reported by durationOfPRS-ProcessingSymbols, or this value T should be equal or larger than the value T reported by durationOfPRS-ProcessingSymbolsInEveryTms. NOTE 12: Each two linked DL-PRS Resources are counted as 1 resource. NOTE 13: maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by maxNumOfDL-PRS-ResProcessedPerSlot. NOTE 14: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30%.

prs-BWA-ThreeContiguousIntrabandInMG-RRC-Connected Indicates the UE capability for support of DL-PRS processing capabilities for aggregated DL-PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_CONNECTED state and comprises the following subfields: - maximumOfThreeAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL-PRS bandwidth in MHz of for FR1, which is supported and reported by UE. - maximumOfThreeAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR2, which is supported and reported by UE. - maximumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL-PRS bandwidth in MHz for FR1, per PFL - maximumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL-PRS bandwidth in MHz for FR2, per PFL - dl-PRS-BufferTypeOfBWA: Indicates the DL-PRS buffering capability. - prs-durationOfThreePRS-BWA-Processing: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL-PRS bandwidth in MHz, which is supported and reported by UE. - prs-durationOfThreePRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - prs-durationOfThreePRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 3840 ms. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR1. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR2. The UE can include this field only if the UE supports prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field. NOTE15: dl-PRS-BufferTypeOfBWA follows buffering capability type reported in dl-PRS-BufferType. NOTE16: The value N should be equal or smaller than the value N reported by durationOfPRS-ProcessingSymbols, or this value T should be equal or larger than the value T reported by durationOfPRS-ProcessingSymbolsInEveryTms. NOTE17: Each three linked DL-PRS Resources are counted as 1 resource. NOTE18: maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by maxNumOfDL-PRS-ResProcessedPerSlot. NOTE19: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30%.

prs-BWA-TwoContiguousIntraband-RRC-IdleAndInactive Indicates the UE capability for support of DL-PRS processing capabilities for aggregated DL-PRS processing of 2 PFLs in intra-band contiguous for RRC_INACTIVE and RRC_IDLE state. The UE can include this field only if the UE supports dl-PRS-BufferType-RRC-Inactive, durationOfPRS-Processing-RRC-Inactive and maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR1, which is supported and reported by UE. - maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR2, which is supported and reported by UE. - maximumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL-PRS bandwidth in MHz for FR1, per PFL. - maximumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL-PRS bandwidth in MHz for FR2, per PFL. - dl-PRS-BufferTypeOfBWA: Indicates the DL PRS buffering capability. - prs-durationOfTwoPRS-BWA-Processing: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. - prs-durationOfTwoPRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - prs-durationOfTwoPRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 2560 ms. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR1. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR2. NOTE 20: dl-PRS-BufferTypeOfBWA follows buffering capability type reported in dl-PRS-BufferType. NOTE 21: The value N should be equal or smaller than the value N reported by durationOfPRS-ProcessingSymbols, or this value T should be equal or larger than the value T reported by durationOfPRS-ProcessingSymbolsInEveryTms. NOTE 22: Each two linked PRS resources are counted as 1 resource NOTE 23: maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive.

prs-BWA-ThreeContiguousIntraband-RRC-IdleAndInactive Indicates the UE capability for support of DL-PRS processing capabilities for aggregated DL-PRS processing of 3 PFLs in intra-band contiguous for RRC_INACTIVE and RRC_IDLE state. The UE can include this field only if the UE supports prs-BWA-TwoContiguousIntraband-RRC-IdleAndInactive. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maximumOfThreeAggregatedDL-PRS-Bandwidth-FR1: Indicates the maximum aggregated DL-PRS bandwidth in MHz of for FR1, which is supported and reported by UE. - maximumOfThreeAggregatedDL-PRS-Bandwidth-FR2: Indicates the maximum aggregated DL-PRS bandwidth in MHz for FR2, which is supported and reported by UE. - maximumOfDL-PRS-BandwidthPerPFL-FR1: Indicates the maximum DL-PRS bandwidth in MHz for FR1, per PFL - maximumOfDL-PRS-BandwidthPerPFL-FR2: Indicates the maximum DL-PRS bandwidth in MHz for FR2, per PFL - dl-PRS-BufferTypeOfBWA: Indicates the DL-PRS buffering capability. - prs-durationOfThreePRS-BWA-Processing: Indicates the duration of DL-PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. - prs-durationOfThreePRS-BWA-ProcessingSymbolsN: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - prs-durationOfThreePRS-BWA-ProcessingSymbolsT: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280, 3840 ms. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR1. - maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2: Indicates the Maximum number of aggregated DL-PRS Resources across aggregated PFLs that UE can process in a slot for FR2. NOTE 24: dl-PRS-BufferTypeOfBWA follows buffering capability type reported in dl-PRS-BufferType. NOTE 25: The value N should be equal or smaller than the value N reported by durationOfPRS-ProcessingSymbols, or this value T should be equal or larger than the value T reported by durationOfPRS-ProcessingSymbolsInEveryTms. NOTE 26: Each two linked PRS resources are counted as 1 resource NOTE 27: maxNumOfAggregatedDL-PRS-ResourcePerSlot should be equal or smaller than the value reported by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive.

reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-Connected Indicates whether the UE supports reduced number of samples in positioning measurements with DL-PRS bandwidth aggregation for RRC_CONNECTED. The UE can include this field only if the UE indicates the capability of maximum aggregated DL PRS bandwidth for the supported FR1 or FR2 bands by using maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1 or maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2 of prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field.

reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive Indicates whether the UE supports reduced number of samples in positioning measurements with DL-PRS bandwidth aggregation for RRC_IDLE and RRC_INACTIVE. The UE can include this field only if the UE indicates the capability of maximum aggregated DL PRS bandwidth for the supported FR1 or FR2 bands by using maximumOfTwoAggregatedDL-PRS-Bandwidth-FR1 or maximumOfTwoAggregatedDL-PRS-Bandwidth-FR2 of prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field.

dl-PRS-MeasurementWithRxFH-RRC-Inactive Indicates the UE capability for support of DL-PRS measurement with Rx frequency hopping in RRC_INACTIVE for RedCap UEs. The UE can include this field only if the UE supports dl-PRS-MeasurementWithRxFH-RRC-Connected and prs-ProcessingRRC-Inactive defined in TS 38.331 [35]. Otherwise, the UE does not include this field.

dl-PRS-MeasurementWithRxFH-RRC-Idle Indicates the UE capability for support of DL-PRS measurement with Rx frequency hopping in RRC_IDLE for RedCap UEs. The UE can include this field only if the UE supports dl-PRS-MeasurementWithRxFH-RRC-Connected. Otherwise, the UE does not include this field.

reducedNumOfSampleForMeasurementWithFH-RRC-Connected Indicates whether the UE supports reduced number of samples for DL-PRS based positioning measurements with frequency hopping for RRC_CONNECTED. The UE can include this field only if the UE supports supportOfRedCap or supportOfERedCap defined in TS 38.331 [35], supportedDL-PRS-ProcessingSamples-RRC-CONNECTED and dl-PRS-MeasurementWithRxFH-RRC-Connected. Otherwise, the UE does not include this field.

reducedNumOfSampleForMeasurementWithFH-RRC-IdleAndInactive Indicates whether the UE supports reduced number of samples for DL-PRS based positioning measurements with frequency hopping for RRC_IDLE and RRC_INACTIVE. The UE can include this field only if the UE supports supportOfRedCap or supportOfERedCap defined in TS 38.331 [35], supportedDL-PRS-ProcessingSamples-RRC-CONNECTED and dl-PRS-MeasurementWithRxFH-RRC-Connected. Otherwise, the UE does not include this field.

supportOfPRS-BWA-WithTwoPFL-Combination Indicates whether the UE supports DL-PRS bandwidth aggregation with two PFL combinations. The UE can include this field only if the UE supports prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field.

dl-PRS-MeasurementWithRxFH-RRC-Connected Indicates the UE capability for DL-PRS measurement with Rx frequency hopping within a MG and measurement reporting in RRC_CONNECTED for RedCap UEs. The UE can include this field only if the UE supports supportedBandwidthPRS, dl-PRS-BufferType, durationOfPRS-Processing, maxNumOfDL-PRS-ResProcessedPerSlot and one of supportOfRedCap and supportOfERedCap defined in TS 38.331 [35]. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maximumPRS-BandwidthAcrossAllHopsFR1: Indicates the maximum DL-PRS bandwidth across all hops in MHz for FR1, which is supported and reported by UE. - maximumPRS-BandwidthAcrossAllHopsFR2: Indicates the maximum DL-PRS bandwidth across all hops in MHz for FR2, which is supported and reported by UE. - maximumFH-Hops: Indicates the maximum number of hops, which is supported and reported by UE. - processingDuration: Indicates the duration of DL-PRS symbols N3 in units of ms a UE can process every T3 ms. - processingPRS-SymbolsDurationN3: This field specifies the values for N3. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50 ms. - processingDurationT3: This field specifies the values for T3. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280ms. - rf-RxRetunTimeFR1: Indicates the RF Rx retune times between consecutive hops for FR1. Enumerated values indicate 70, 140, 210us. - rf-RxRetunTimeFR2: Indicates the RF Rx retune times between consecutive hops for FR2. Enumerated values indicate 35, 70, 140us. - numOfOverlappingPRB: Indicates the overlapping PRB(s) between adjacent hops. Enumerated values indicate 0,1,2,4 PRBs. NOTE 28: The maximum DL-PRS bandwidth per hop follows supportedBandwidthPRS. NOTE 29: DL-PRS buffering capability follows dl-PRS-BufferType.

NOTE 9: When the target device provides the durationOfPRS-Processing capability (N, T) for any P(≥T) time window defined in TS 38.214 [45] clause 5.1.6.5, the target device should be capable of processing all DL-PRS Resources within P, if - N≥K where K is defined in the TS 38.214 [45] clause 5.1.6.5, and - the number of DL-PRS Resources in each slot does not exceed the maxNumOfDL-PRS-ResProcessedPerSlot, and - the configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) is as specified in TS 38.133 [46].

–	NR-UL-SRS-Capability

The IE NR-UL-SRS-Capability defines the UE uplink SRS capability.

-- ASN1START

NR-UL-SRS-Capability-r16 ::= SEQUENCE {

srs-CapabilityBandList-r16					SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

SRS-CapabilityPerBand-r16,

srs-PosResourceConfigCA-BandList-r16		SEQUENCE (SIZE (1..nrMaxConfiguredBands-r16)) OF

SRS-PosResourcesPerBand-r16			OPTIONAL,

maxNumberSRS-PosPathLossEstimateAllServingCells-r16

ENUMERATED {n1, n4, n8, n16}			OPTIONAL,

maxNumberSRS-PosSpatialRelationsAllServingCells-r16

ENUMERATED {n0, n1, n2, n4, n8, n16}	OPTIONAL,

...

}

SRS-CapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16			FreqBandIndicatorNR-r16,

olpc-SRS-Pos-r16				OLPC-SRS-Pos-r16									OPTIONAL,

spatialRelationsSRS-Pos-r16		SpatialRelationsSRS-Pos-r16							OPTIONAL,

...,

[[

posSRS-RRC-Inactive-InInitialUL-BWP-r17		PosSRS-RRC-Inactive-InInitialUL-BWP-r17	OPTIONAL,

posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17

PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17

OPTIONAL,

olpc-SRS-PosRRC-Inactive-r17				OLPC-SRS-Pos-r16						OPTIONAL,

spatialRelationsSRS-PosRRC-Inactive-r17		SpatialRelationsSRS-Pos-r16				OPTIONAL

]],

[[

posSRS-SP-RRC-Inactive-InInitialUL-BWP-r17	PosSRS-SP-RRC-Inactive-InInitialUL-BWP-r17

OPTIONAL

]],

[[

posSRS-Preconfigured-RRC-InactiveInitialUL-BWP-r18		ENUMERATED {supported}	OPTIONAL,

posSRS-Preconfigured-RRC-InactiveOutsideInitialUL-BWP-r18	ENUMERATED {supported}	OPTIONAL,

posSRS-ValidityAreaRRC-InactiveInitialUL-BWP-r18		ENUMERATED {supported}	OPTIONAL,

posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP-r18		ENUMERATED {supported}	OPTIONAL,

posSRS-TxFH-RRC-Connected-r18	PosSRS-TxFrequencyHoppingRRC-Connected-r18			OPTIONAL,

posSRS-TxFH-RRC-Inactive-r18	PosSRS-TxFrequencyHoppingRRC-Inactive-r18			OPTIONAL,

posSRS-TxFH-WithTimeWindow-r18						ENUMERATED {supported}			OPTIONAL,

posSRS-BWA-RRC-Inactive-r18						PosSRS-BWA-RRC-Inactive-r18			OPTIONAL

]]

}

OLPC-SRS-Pos-r16 ::= SEQUENCE {

olpc-SRS-PosBasedOnPRS-Serving-r16		ENUMERATED {supported}						OPTIONAL,

olpc-SRS-PosBasedOnSSB-Neigh-r16		ENUMERATED {supported}						OPTIONAL,

olpc-SRS-PosBasedOnPRS-Neigh-r16		ENUMERATED {supported}						OPTIONAL,

maxNumberPathLossEstimatePerServing-r16	ENUMERATED {n1, n4, n8, n16}				OPTIONAL,

...

}

SpatialRelationsSRS-Pos-r16 ::=	SEQUENCE {

spatialRelation-SRS-PosBasedOnSSB-Serving-r16		ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16	ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Serving-r16		ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnSRS-r16				ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnSSB-Neigh-r16			ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Neigh-r16			ENUMERATED {supported}			OPTIONAL,

...

}

SRS-PosResourcesPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16							FreqBandIndicatorNR-r16,

maxNumberSRS-PosResourceSetsPerBWP-r16			ENUMERATED {n1, n2, n4, n8, n12, n16},

maxNumberSRS-PosResourcesPerBWP-r16				ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPeriodicSRS-PosResourcesPerBWP-r16		ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberAP-SRS-PosResourcesPerBWP-r16			ENUMERATED {n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

maxNumberSP-SRS-PosResourcesPerBWP-r16			ENUMERATED {n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

...,

[[

posSRS-BWA-RRC-Connected-r18					PosSRS-BWA-RRC-Connected-r18		OPTIONAL,

posSRS-BWA-IndependentCA-RRC-Connected-r18	PosSRS-BWA-IndependentCA-RRC-Connected-r18

OPTIONAL

]]

}

PosSRS-RRC-Inactive-InInitialUL-BWP-r17 ::= SEQUENCE {

maxNumOfSRSposResourceSets-r17			ENUMERATED {n1, n2, n4, n8, n12, n16 }		OPTIONAL,

maxNumOfPeriodicAndSemiPersistentSRSposResources-r17

ENUMERATED {n1, n2, n4, n8, n16, n32, n64 }

OPTIONAL,

maxNumOfPeriodicAndSemiPersistentSRSposResourcesPerSlot-r17

ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

OPTIONAL,

maxNumOfPeriodicSRSposResources-r17

ENUMERATED {n1, n2, n4, n8, n16, n32, n64 }

OPTIONAL,

maxNumOfPeriodicSRSposResourcesPerSlot-r17

ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

OPTIONAL,

dummy1									ENUMERATED {n1, n2, n4, n8, n16, n32, n64}	OPTIONAL,

dummy2									ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10, n12, n14 }

OPTIONAL,

...

}

PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 ::= SEQUENCE {

maxSRSposBandwidthForEachSCS-withinCC-FR1-r17

ENUMERATED { mhz5, mhz10, mhz15, mhz20, mhz25, mhz30,
														mhz35, mhz40, mhz45, mhz50, mhz60, mhz70,

mhz80, mhz90, mhz100 }			OPTIONAL,

maxSRSposBandwidthForEachSCS-withinCC-FR2-r17

ENUMERATED {mhz50, mhz100, mhz200, mhz400}	OPTIONAL,

maxNumOfSRSposResourceSets-r17			ENUMERATED { n1, n2, n4, n8, n12, n16 }		OPTIONAL,

maxNumOfPeriodicSRSposResources-r17		ENUMERATED { n1, n2, n4, n8, n16, n32, n64 }

OPTIONAL,

maxNumOfPeriodicSRSposResourcesPerSlot-r17

ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10, n12, n14 }

OPTIONAL,

differentNumerologyBetweenSRSposAndInitialBWP-r17

ENUMERATED { supported }					OPTIONAL,

srsPosWithoutRestrictionOnBWP-r17

ENUMERATED { supported }					OPTIONAL,

maxNumOfPeriodicAndSemiPersistentSRSposResources-r17

ENUMERATED {n1, n2, n4, n8, n16, n32, n64}	OPTIONAL,

maxNumOfPeriodicAndSemiPersistentSRSposResourcesPerSlot-r17

ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14 }						OPTIONAL,

differentCenterFreqBetweenSRSposAndInitialBWP-r17

ENUMERATED { supported }					OPTIONAL,

maxNumOfSemiPersistentSRSposResources-r17

ENUMERATED { n1, n2, n4, n8, n16, n32, n64 }

OPTIONAL,

maxNumOfSemiPersistentSRSposResourcesPerSlot-r17

ENUMERATED { n1, n2, n3, n4, n5, n6, n8, n10,

n12, n14 }						OPTIONAL,

switchingTimeSRS-TX-OtherTX-r17			ENUMERATED { us100, us140, us200, us300, us500 }

OPTIONAL,

...

}

PosSRS-SP-RRC-Inactive-InInitialUL-BWP-r17 ::= SEQUENCE {

maxNumOfSemiPersistentSRSposResources-r17

ENUMERATED {n1, n2, n4, n8, n16, n32, n64}		OPTIONAL,

maxNumOfSemiPersistentSRSposResourcesPerSlot-r17

ENUMERATED {n1, n2, n3, n4, n5, n6, n8, n10, n12, n14}

OPTIONAL,

...

}

PosSRS-TxFrequencyHoppingRRC-Connected-r18 ::=SEQUENCE {

maximumSRS-BandwidthAcrossAllHopsFR1-r18	ENUMERATED {mhz40, mhz50, mhz80, mhz100}

OPTIONAL,

maximumSRS-BandwidthAcrossAllHopsFR2-r18	ENUMERATED {mhz100, mhz200, mhz400}	OPTIONAL,

maximumTxFH-Hops-r18							ENUMERATED {n2, n3, n4, n5, n6}		OPTIONAL,

rf-TxRetunTimeFR1-r18	ENUMERATED {n70, n140, n210}								OPTIONAL,

rf-TxRetunTimeFR2-r18	ENUMERATED {n35, n70, n140}									OPTIONAL,

switchTimeBetweenActiveBWP-FrequencyHop-r18	ENUMERATED {n100, n140,n200,n300,n500}	OPTIONAL,

numOfOverlappingPRB-r18	ENUMERATED {n0, n1, n2, n4}									OPTIONAL,

maximumSRS-ResourcePeriodic-r18	ENUMERATED {n1, n2, n4, n8, n16, n32, n64}		OPTIONAL,

maximumSRS-ResourceAperiodic-r18	ENUMERATED {n0,n1, n2, n4, n8, n16, n32, n64}		OPTIONAL,

maximumSRS-ResourceSemipersistent-r18	ENUMERATED {n0,n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

...

}

PosSRS-TxFrequencyHoppingRRC-Inactive-r18 ::=SEQUENCE {

maximumSRS-BandwidthAcrossAllHopsFR1-r18	ENUMERATED {mhz40, mhz50, mhz80, mhz100}

OPTIONAL,

maximumSRS-BandwidthAcrossAllHopsFR2-r18	ENUMERATED {mhz100, mhz200, mhz400}	OPTIONAL,

maximumTxFH-Hops-r18							ENUMERATED {n2, n3, n4, n5, n6}		OPTIONAL,

rf-TxRetunTimeFR1-r18	ENUMERATED {n70, n140, n210}								OPTIONAL,

rf-TxRetunTimeFR2-r18	ENUMERATED {n35, n70, n140}									OPTIONAL,

switchTimeBetweenActiveBWP-FrequencyHop-r18	ENUMERATED {n100, n140,n200,n300,n500}	OPTIONAL,

numOfOverlappingPRB-r18	ENUMERATED {n0, n1, n2, n4}									OPTIONAL,

maximumSRS-ResourcePeriodic-r18	ENUMERATED {n1, n2, n4, n8, n16, n32, n64}		OPTIONAL,

maximumSRS-ResourceSemipersistent-r18	ENUMERATED {n0,n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

...

}

PosSRS-BWA-RRC-Connected-r18 ::=SEQUENCE {

numOfCarriersIntraBandContiguous-r18	ENUMERATED {two, three, twoandthree}		,

maximumAggregatedBW-TwoCarriersFR1-r18	ENUMERATED {mhz20, mhz40, mhz50, mhz80, mhz100, mhz160, mhz180, mhz190, mhz200}																OPTIONAL,

maximumAggregatedBW-TwoCarriersFR2-r18	ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz600, mhz800}																					OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR1-r18	ENUMERATED {mhz80, mhz100, mhz160, mhz200, mhz240, mhz300}																					OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR2-r18	ENUMERATED {mhz50, mhz100, mhz200, mhz300, mhz400, mhz600, mhz800, mhz1000, mhz1200}														OPTIONAL,

maximumAggregatedResourceSet-r18	ENUMERATED {n1, n2, n4, n8, n12, n16}			,

maximumAggregatedResourcePeriodic-r18		ENUMERATED {n1, n2, n4, n8, n16, n32, n64}																									,

maximumAggregatedResourceAperiodic-r18		ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64}																						,

maximumAggregatedResourceSemi-r18			ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64}

,

maximumAggregatedResourcePeriodicPerSlot-r18		ENUMERATED {n1, n2, n3, n4, n5, n6,

n8, n10, n12, n14}				,

maximumAggregatedResourceAperiodicPerSlot-r18		ENUMERATED {n0, n1, n2, n3, n4,

n5, n6, n8, n10, n12, n14}		,	maximumAggregatedResourceSemiPerSlot-r18			ENUMERATED {n0, n1, n2, n3, n4,

n5, n6, n8, n10, n12, n14}																	,

...

}

PosSRS-BWA-IndependentCA-RRC-Connected-r18 ::=SEQUENCE {

numOfCarriersIntraBandContiguous-r18	ENUMERATED {two, three, twoandthree}		,

maximumAggregatedBW-TwoCarriersFR1-r18	ENUMERATED {mhz20, mhz40, mhz50, mhz80, mhz100, mhz160, mhz180, mhz190, mhz200}																OPTIONAL,

maximumAggregatedBW-TwoCarriersFR2-r18	ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz600, mhz800}																					OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR1-r18	ENUMERATED {mhz80, mhz100, mhz160, mhz200, mhz240, mhz300}																					OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR2-r18	ENUMERATED {mhz50, mhz100, mhz200, mhz300, mhz400, mhz600, mhz800, mhz1000, mhz1200}														OPTIONAL,

maximumAggregatedResourceSet-r18	ENUMERATED {n1, n2, n4, n8, n12, n16}			,

maximumAggregatedResourcePeriodic-r18		ENUMERATED {n1, n2, n4, n8, n16, n32, n64}																									,

maximumAggregatedResourceAperiodic-r18		ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64}																						,

maximumAggregatedResourceSemi-r18			ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64}

,

maximumAggregatedResourcePeriodicPerSlot-r18		ENUMERATED {n1, n2, n3, n4, n5,

n6, n8, n10, n12, n14}																			,

maximumAggregatedResourceAperiodicPerSlot-r18		ENUMERATED {n0, n1, n2, n3, n4,

n5, n6, n8, n10, n12, n14}																		,

maximumAggregatedResourceSemiPerSlot-r18			ENUMERATED {n0, n1, n2, n3, n4,

n5, n6, n8, n10, n12, n14}																		,

guardPeriod-r18					ENUMERATED {n0, n30, n100, n140, n200}		,

powerClassForTwoAggregatedCarriers-r18			ENUMERATED {pc2, pc3}			OPTIONAL,

powerClassForThreeAggregatedCarriers-r18		ENUMERATED {pc2, pc3}			OPTIONAL,

...

}

PosSRS-BWA-RRC-Inactive-r18 ::=SEQUENCE {

numOfCarriersIntraBandContiguous-r18	ENUMERATED {two, three, twoandthree}		,

maximumAggregatedBW-TwoCarriersFR1-r18	ENUMERATED {mhz20, mhz40, mhz50, mhz80, mhz100, mhz160, mhz180, mhz190, mhz200}																OPTIONAL,

maximumAggregatedBW-TwoCarriersFR2-r18	ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz600, mhz800}																					OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR1-r18	ENUMERATED {mhz80, mhz100, mhz160, mhz200, mhz240, mhz300}																					OPTIONAL,

maximumAggregatedBW-ThreeCarriersFR2-r18	ENUMERATED {mhz50, mhz100, mhz200, mhz300, mhz400, mhz600, mhz800, mhz1000, mhz1200}														OPTIONAL,

maximumAggregatedResourceSet-r18	ENUMERATED {n1, n2, n4, n8, n12, n16}			,

maximumAggregatedResourcePeriodic-r18		ENUMERATED {n1, n2, n4, n8, n16, n32, n64}																									,

maximumAggregatedResourceSemi-r18			ENUMERATED {n0, n1, n2, n4, n8, n16, n32, n64}

,

maximumAggregatedResourcePeriodicPerSlot-r18		ENUMERATED {n1, n2, n3, n4,

n5, n6, n8, n10, n12, n14}																			,

maximumAggregatedResourceSemiPerSlot-r18			ENUMERATED {n0, n1, n2, n3, n4,

n5, n6, n8, n10, n12, n14}																	,

guardPeriod-r18				ENUMERATED {n0, n30, n100, n140, n200}		,

powerClassForTwoAggregatedCarriers-r18			ENUMERATED {pc2, pc3}			OPTIONAL,

powerClassForThreeAggregatedCarriers-r18		ENUMERATED {pc2, pc3}			OPTIONAL,

...

}

-- ASN1STOP

NR-UL-SRS-Capability field descriptions

srs-PosResourceConfigCA-BandList This field indicates the number of SRS for positioning resources supported by the target device. The target device includes this field for each band which belongs to the srs-CapabilityBandList for the current configured CA band combination. The capability signalling comprises the following parameters: - freqBandIndicatorNR indicates the current configured NR band of the target device. - maxNumberSRS-PosResourceSetsPerBWP indicates the maximum number of SRS Resource Sets for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n12, n16 correspond to 1, 2, 4, 8, 12, 16 SRS Resource Sets for positioning, respectively. - maxNumberSRS-PosResourcesPerBWP indicates the maximum number of periodic, semi-persistent, and aperiodic SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 SRS Resources for positioning, respectively. - maxNumberPeriodicSRS-PosResourcesPerBWP indicates the maximum number of periodic SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 periodic SRS Resources for positioning, respectively. - maxNumberAP-SRS-PosResourcesPerBWP indicates the maximum number of aperiodic SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 aperiodic SRS Resources for positioning, respectively. - maxNumberSP-SRS-PosResourcesPerBWP indicates the maximum number of semi-persistent SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 semi-persistent SRS Resources for positioning, respectively.

maxNumberSRS-PosPathLossEstimateAllServingCells Indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning across all cells in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. The UE shall include this field if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving, olpc-SRS-PosBasedOnSSB-Neigh and olpc-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field.

maxNumberSRS-PosSpatialRelationsAllServingCells indicates the maximum number of maintained spatial relations for all the SRS resource sets for positioning across all serving cells in addition to the spatial relations maintained spatial relations per serving cell for the PUSCH/PUCCH/SRS transmissions. It is only applied for FR2. The UE can include this field only if the UE supports any of spatialRelation-SRS-PosBasedOnSSB-Serving, spatialRelation-SRS-PosBasedOnCSI-RS-Serving, spatialRelation-SRS-PosBasedOnPRS-Serving, spatialRelation-SRS-PosBasedOnSSB-Neigh or spatialRelation-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field.

olpc-SRS-Pos Indicates whether the UE supports open-loop power control for SRS for positioning. The capability signalling comprises the following parameters: - olpc-SRS-PosBasedOnPRS-Serving indicates whether the UE supports OLPC for SRS for positioning based on DL-PRS from the serving cell in the same band. The UE can include this field only if the UE supports NR-DL-PRS-ProcessingCapability and srs-PosResources TS38.331 [35] Otherwise, the UE does not include this field. - olpc-SRS-PosBasedOnSSB-Neigh indicates whether the UE supports OLPC for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - olpc-SRS-PosBasedOnPRS-Neigh indicates whether the UE supports OLPC for SRS for positioning based on DL-PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports olpc-SRS-PosBasedOnPRS-Serving. Otherwise, the UE does not include this field. Note: A DL-PRS from a PRS-only TP is treated as DL-PRS from a non-serving cell. - maxNumberPathLossEstimatePerServing indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. The UE shall include this field if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving, olpc-SRS-PosBasedOnSSB-Neigh and olpc-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field.

spatialRelationsSRS-Pos Indicates whether the UE supports spatial relations for SRS for positioning. It is only applicable for FR2. The capability signalling comprises the following parameters: - spatialRelation-SRS-PosBasedOnSSB-Serving indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the serving cell in the same band. The UE can include this field only if the UE supports srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnCSI-RS-Serving indicates whether the UE supports spatial relation for SRS for positioning based on CSI-RS from the serving cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnPRS-Serving indicates whether the UE supports spatial relation for SRS for positioning based on DL-PRS from the serving cell in the same band. The UE can include this field only if the UE supports any of DL-PRS Resources for DL-AoD, DL-PRS Resources for DL-TDOA or DL-PRS Resources for Multi-RTT, or srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnSRS indicates whether the UE supports spatial relation for SRS for positioning based on SRS in the same band. The UE can include this field only if the UE supports srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnSSB-Neigh indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnPRS-Neigh indicates whether the UE supports spatial relation for SRS for positioning based on DL-PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnPRS-Serving. Otherwise, the UE does not include this field. Note: A DL-PRS from a PRS-only TP is treated as DL-PRS from a non-serving cell.

posSRS-RRC-Inactive-InInitialUL-BWP Indicates whether the UE supports positioning SRS transmission in RRC_INACTIVE state for initial UL BWP. The capability signalling comprises the following parameters: - maxNumOfSRSposResourceSets indicates the maximum number of SRS Resource Sets for positioning supported by the UE. - maxNumOfPeriodicAndSemiPersistentSRSposResources indicates the maximum number of periodic and semi-persistent SRS Resources for positioning supported by the UE. - maxNumOfPeriodicAndSemiPersistentSRSposResourcesPerSlot indicates the maximum number of periodic and semi-persistent SRS Resources for positioning per slot supported by the UE. - maxNumOfPeriodicSRSposResources indicates the maximum number of periodic SRS Resources for positioning supported by the UE. - maxNumOfPeriodicSRSposResourcesPerSlot indicates the maximum number of periodic SRS Resources for positioning per slot supported by the UE. - dummy1, dummy2 are not used in the specification. If received they shall be ignored by the receiver.

posSRS-RRC-Inactive-OutsideInitialUL-BWP Indicates whether the UE supports positioning SRS transmission in RRC_INACTIVE state outside initial UL BWP. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maxSRSposBandwidthForEachSCS-withinCC-FR1 indicates the maximum SRS bandwidth in MHz supported for each SCS that UE supports within a single CC for FR1. - maxSRSposBandwidthForEachSCS-withinCC-FR2 indicates the maximum SRS bandwidth in MHz supported for each SCS that UE supports within a single CC for FR2. - maxNumOfSRSposResourceSets indicates the maximum number of SRS Resource Sets for positioning supported by the UE. - maxNumOfPeriodicSRSposResources indicates the maximum number of periodic SRS Resources for positioning supported by the UE. - maxNumOfPeriodicSRSposResourcesPerSlot indicates the maximum number of periodic SRS Resources for positioning per slot supported by the UE. - differentNumerologyBetweenSRSposAndInitialBWP indicates whether different numerology between the SRS and the initial UL BWP is supported by the UE. If the field is absent, the UE only supports same numerology between the SRS and the initial UL BWP. - srsPosWithoutRestrictionOnBWP indicates whether SRS operation without restriction on the BW is supported by the UE; BW of the SRS may not include BW of the CORESET#0 and SSB. If the field is absent, the UE supports only SRS BW that includes the BW of the CORESET #0 and SSB. - maxNumOfPeriodicAndSemiPersistentSRSposResources indicates the maximum number of periodic and semi-persistent SRS Resources for positioning supported by the UE. - maxNumOfPeriodicAndSemiPersistentSRSposResourcesPerSlot indicates the maximum number of periodic and semi-persistent SRS Resources for positioning per slot supported by the UE. - differentCenterFreqBetweenSRSposAndInitialBWP indicates whether different center frequency between the SRS for positioning and the initial UL BWP is supported by the UE. If the field is absent, the UE only supports same center frequency between the SRS for positioning and initial UL BWP. - maxNumOfSemiPersistentSRSposResources indicates the maximum number of semi-persistent SRS Resources for positioning supported by the UE. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field. - maxNumOfSemiPersistentSRSposResourcesPerSlot indicates the maximum number of semi-persistent SRS Resources for positioning per slot supported by the UE. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field. - switchingTimeSRS-TX-OtherTX indicates the switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP.

olpc-SRS-PosRRC-Inactive Indicates whether the UE supports open-loop power control for SRS for positioning in RRC_INACTIVE state. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field.

spatialRelationsSRS-PosRRC-Inactive Indicates whether the UE supports spatial relations for SRS for positioning in RRC_INACTIVE state on FR2. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field.

posSRS-SP-RRC-Inactive-InInitialUL-BWP Indicates whether the UE supports positioning SRS transmission in RRC_INACTIVE state for initial UL BWP with semi-persistent SRS. The UE can include this field only if the UE supports posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maxNumOfSemiPersistentSRSposResources indicates the maximum number of semi-persistent SRS Resources for positioning supported by the UE. - maxNumOfSemiPersistentSRSposResourcesPerSlot indicates the maximum number of semi-persistent SRS Resources for positioning per slot supported by the UE.

posSRS-Preconfigured-RRC-InactiveInitialUL-BWP Indicates whether the UE supports pre-configured SRS with validity area in RRC_INACTIVE for initial UL BWP. The UE can include this field only if the UE supports posSRS-ValidityAreaRRC-InactiveInitialUL-BWP. Otherwise, the UE does not include this field.

posSRS-Preconfigured-RRC-InactiveOutsideInitialUL-BWP Indicates whether the UE supports pre-configured SRS with validity area in RRC_INACTIVE outside initial UL BWP. The UE can include this field only if the UE supports posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP. Otherwise, the UE does not include this field.

posSRS-ValidityAreaRRC-InactiveInitialUL-BWP Indicates whether the UE supports SRS for positioning configuration in multi cells in RRC_INACTIVE for initial UL BWP. The UE can include this field only if the UE support posSRS-RRC-Inactive-InInitialUL-BWP. Otherwise, the UE does not include this field.

posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP Indicates whether the UE supports SRS for positioning configuration in multi cells in RRC_INACTIVE outside initial UL BWP. The UE can include this field only if the UE supports posSRS-RRC-Inactive-OutsideInitialUL-BWP and posSRS-ValidityAreaRRC-InactiveInitialUL-BWP. Otherwise, the UE does not include this field.

posSRS-TxFH-RRC-Connected Indicates the UE capability for support of positioning SRS with Tx frequency hopping in RRC_CONNECTED for RedCap UEs. The UE can include this field only if the UE supports SRS-AllPosResources and one of supportOfRedCap and supportOfERedCap defined in TS 38.331 [35]. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maximumSRS-BandwidthAcrossAllHopsFR1: Indicates the maximum positioning SRS bandwidth across all hops in MHz for FR1, which is supported and reported by UE. - maximumSRS-BandwidthAcrossAllHopsFR2: Indicates the maximum positioning SRS bandwidth across all hops in MHz for FR2, which is supported and reported by UE. - maximumTxFH-Hops: Indicates the maximum number of transmission hops, which is supported and reported by UE. - rf-TxRetunTimeFR1: Indicates the RF Tx retune times between consecutive hops for FR1. Enumerated values indicate 70, 140, 210us. - rf-TxRetunTimeFR2: Indicates the RF Tx retune times between consecutive hops for FR2. Enumerated values indicate 35, 70, 140us. - switchTimeBetweenActiveBWP-FrequencyHop: Indicates the switching time between active BWP and frequency hop. Enumerated values indicate 100, 140, 200, 300, 500µs. - numOverlappingPRB: Indicates the overlapping PRB(s) between adjacent hops. Enumerated values indicate 0,1,2,4 PRBs. - maximumSRS-ResourcePeriodic: Indicates the maximum number of periodic positioning SRS resources with Tx frequency hopping. - maximumSRS-ResourceAperiodic: Indicates the maximum number of aperiodic positioning SRS resources with Tx frequency hopping. - maximumSRS-ResourceSemipersistent: Indicates the maximum number of Semi-persistent positioning SRS resources with Tx frequency hopping. NOTE 1: No additional UE requirements shall be specified for the case of Tx hopping with non-overlapping hops compared to the case of Tx hopping with overlapping hops, e.g., a UE is not responsible for keeping phase continuity across the hops in either case of overlapping or non-overlapping hops.

posSRS-TxFH-RRC-Inactive Indicates the UE capability for support of positioning SRS with Tx frequency hopping in RRC_INACTIVE for RedCap UEs. The UE can include this field only if the UE supports posSRS-RRC-Inactive-OutsideInitialUL-BWP and one of supportOfRedCap and supportOfERedCap defined in TS 38.331 [35]. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maximumSRS-BandwidthAcrossAllHopsFR1: Indicates the maximum positioning SRS bandwidth across all hops in MHz for FR1, which is supported and reported by UE. - maximumSRS-BandwidthAcrossAllHopsFR2: Indicates the maximum positioning SRS bandwidth across all hops in MHz for FR2, which is supported and reported by UE. - maximumTxFH-Hops: Indicates the maximum number of transmission hops, which is supported and reported by UE. - rf-TxRetunTimeFR1: Indicates the RF Tx retune times between consecutive hops for FR1. Enumerated values indicate 70, 140, 210µs. - rf-TxRetunTimeFR2: Indicates the RF Tx retune times between consecutive hops for FR2. Enumerated values indicate 35, 70, 140µs. - switchTimeBetweenActiveBWP-FrequencyHop: Indicates the switching time between active BWP and frequency hop. Enumerated values indicate 100, 140, 200, 300, 500µs. - numOfOverlappingPRB: Indicates the overlapping PRB(s) between adjacent hops. Enumerated values indicate 0,1,2,4 PRBs. - maximumSRS-ResourcePeriodic indicates the maximum number of periodic positioning SRS resources with Tx frequency hopping. - maximumSRS-ResourceSemipersistent indicates the maximum number of Semi-persistent positioning SRS resources with Tx frequency hopping. NOTE 2: No additional UE requirements shall be specified for the case of Tx hopping with non-overlapping hops compared to the case of Tx hopping with overlapping hops, e.g., a UE is not responsible for keeping phase continuity across the hops in either case of overlapping or non-overlapping hops.

posSRS-TxFH-WithTimeWindow Indicates the UE capability for support of UL time window and transmission of SRS for positioning with Tx Frequency hopping within the window. The UE can include this field only if the UE supports posSRS-TxFH-RRC-Connected. Otherwise, the UE does not include this field.

posSRS-BWA-RRC-Connected Indicates the UE capability for support of positioning SRS bandwidth aggregation in RRC_CONNECTED and comprises the support of the same SRS power reduction across aggregated carriers. The UE can include this field only if the UE supports SRS-AllPosResources and supportedBandCombinationList defined in TS 38.331 [35]. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - numOfCarriersIntraBandContiguous: Indicates the number of supported aggregated carriers in intra band contiguous carriers, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR1: Indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR2: Indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR1: Indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR2: Indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedResourceSet: Indicates the max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodic: Indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourceAperiodic: Indicates the maximum number of aggregated aperiodic SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourceSemi: Indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodicPerSlot: Indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - maximumAggregatedResourceAperiodicPerSlot: Indicates the maximum number of aggregated aperiodic SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - maximumAggregatedResourceSemiPerSlot: Indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. NOTE 3: The UE supports the simultaneous transmission in a coherent manner of 2 or 3 SRS resources in 2 or 3 intra-band contiguous CCs. NOTE 4: Each two or three linked SRS resources are counted as 1 resource NOTE 5: A UE that support SRS-PosResourceAP defined in TS 38.331 [35] must signal a non-zero value for maximumAggregatedResourceAperiodic and maximumAggregatedResourceAperiodicPerSlot; NOTE 6: UE only reports the number on bands for the current configured CA band combination. NOTE 7: For numOfCarriersIntraBandContiguous, it shall be less than or equal to the maximum number of the component carrier associated with ca-BandwidthClassUL-NR in TS 38.331 [35]. NOTE 8: For maximum aggregated UL SRS bandwidth, it shall be less than or equal to the maximum aggregated transmission bandwidth associated with ca-BandwidthClassUL-NR in TS 38.331 [35]. Additionally, it shall be less than or equal to the maximum aggregated bandwidth for the supported CA configuration in Table 5.5A.1-1 in TS 38.101-1 [37] for FR1 bands or Table 5.5A.1-1 in TS 38.101-2 [34] for FR2 bands for the band where aggregated SRS CCs is configured.

posSRS-BWA-IndependentCA-RRC-Connected Indicates the UE capability for support of positioning SRS bandwidth aggregation independent from UL communication CA in RRC_CONNECTED and comprises the support of the same SRS power reduction across aggregated carriers. The UE can include this field only if the UE supports SRS-AllPosResources defined in TS 38.331 [35]. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - numOfCarriersIntraBandContiguous: Indicates the number of supported aggregated carriers in intra band contiguous carriers, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR1: Indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR2: Indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR1: Indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR2: Indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedResourceSet: Indicates the max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodic: Indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourceAperiodic: Indicates the maximum number of aggregated aperiodic SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourceSemi: Indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodicPerSlot: Indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - maximumAggregatedResourceAperiodicPerSlot: Indicates the maximum number of aggregated aperiodic SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - maximumAggregatedResourceSemiPerSlot: Indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - guardPeriod: Indicates the guard period in microseconds before and after aggregated SRS transmission. - powerClassForTwoAggregatedCarriers: Indicates the power class of supported two aggregated carriers in intra band contiguous carries. - powerClassForThreeAggregatedCarriers: Indicates the power class of supported three aggregated carriers in intra band contiguous carries. NOTE 9: The UE supports the simultaneous transmission in a coherent manner of 2 or 3 SRS resources in 2 or 3 intra-band contiguous CCs. NOTE 10: Each two or three linked SRS resources are counted as 1 resource NOTE 11: UE only reports the number on bands for the current configured CA band combination. NOTE 12: Guard period is needed before and after the aggregated SRS transmissions when SRS resource is configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC. NOTE 13: For a given band, independent of the band combination, the UE must signal the same guard period. NOTE 14: The power class is only applicable for FR1 bands.

posSRS-BWA-RRC-Inactive Indicates the UE capability for support of positioning SRS bandwidth aggregation in RRC_INACTIVE and comprises the support of the same SRS power reduction across aggregated carriers. The UE can include this field only if the UE supports posSRS-RRC-Inactive-OutsideInitialUL-BWP. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - numOfCarriersIntraBandContiguous: Indicates the number of supported aggregated carriers in intra band contiguous carriers, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR1: Indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-TwoCarriersFR2: Indicates the maximum aggregated SRS bandwidth in MHz for two aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR1: Indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR1, which is supported and reported by UE. - maximumAggregatedBW-ThreeCarriersFR2: Indicates the maximum aggregated SRS bandwidth in MHz for three aggregated carriers for FR2, which is supported and reported by UE. - maximumAggregatedResourceSet: Indicates the max number of aggregated SRS resource sets for positioning supported by UE for SRS bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodic: Indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourceSemi: Indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation, which is supported and reported by UE. - maximumAggregatedResourcePeriodicPerSlot: Indicates the maximum number of aggregated periodic SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - maximumAggregatedResourceSemiPerSlot: Indicates the maximum number of aggregated semi-persistent SRS resources for bandwidth aggregation per slot, which is supported and reported by UE. - guardPeriod: Indicates the guard period in microseconds before and after aggregated SRS transmission. - powerClassForTwoAggregatedCarriers: Indicates the power class of supported two aggregated carriers in intra band contiguous carries. - powerClassForThreeAggregatedCarriers: Indicates the power class of supported three aggregated carriers in intra band contiguous carries. NOTE 15: The power class is only applicable for FR1 bands.

6.5.10.6	NR DL-TDOA Capability Information

–	NR-DL-TDOA-ProvideCapabilities

The IE NR-DL-TDOA-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-TDOA and to provide its NR DL-TDOA positioning capabilities to the location server.

-- ASN1START

NR-DL-TDOA-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16						PositioningModes,

nr-DL-TDOA-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-TDOA-MeasurementCapability-r16	NR-DL-TDOA-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

additionalPathsReport-r16				ENUMERATED { supported }					OPTIONAL,

periodicalReporting-r16					PositioningModes							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

trpTEG-InfoSup	(3),

nr-IntegritySup-r18 (4),

pruInfoSup-r18	(5)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,

nr-DL-TDOA-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

additionalPathsExtSupport-r17			ENUMERATED { n4, n6, n8 }					OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17		OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,								...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]],

[[

posMeasGapSupport-r17					ENUMERATED { supported }					OPTIONAL

]],

[[

multiLocationEstimateInSameMeasReport-r17	ENUMERATED { supported }				OPTIONAL

]],

[[

locationCoordinateTypes-r18				LocationCoordinateTypes						OPTIONAL,

periodicAssistanceData-r18				BIT STRING { solicited	 (0),

unsolicited (1)} (SIZE (1..8))	OPTIONAL,

nr-IntegrityAssistanceSupport-r18		BIT STRING {

serviceParametersSup		(0),

serviceAlertSup				(1),

riskParametersSup			(2),

integrityParaTRP-LocSup		(3),

integrityParaBeamInfoSup	(4),

integrityParaRTD-InfoSup	(5)

} (SIZE (1..8)) 					OPTIONAL,

nr-DL-TDOA-OnDemandPRS-ForBWA-Support-r18

ENUMERATED { supported }					OPTIONAL

]]

}

-- ASN1STOP

NR-DL-TDOA-ProvideCapabilities field descriptions

nr-DL-TDOA-Mode This field specifies the NR DL-TDOA mode(s) supported by the target device.

periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-TDOA. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is supported or not. The UE can indicate this bit only if the UE supports prs-ProcessingCapabilityBandList and any of maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer, maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field; - bit 4 together with bit 0 indicates whether the fields nr-IntegrityTRP-LocationBounds, nr-IntegrityDL-PRS-ResourceSetARP-LocationBounds, nr-IntegrityDL-PRS-ResourceARP-LocationBounds in IE NR-TRP-LocationInfo are supported or not; bit 4 together with bit 1 indicates whether the field nr-IntegrityBeamInfoBounds in IE NR-DL-PRS-BeamInfo is supported or not; bit 4 together with the bit 2 indicates whether the field nr-IntegrityRTD-InfoBounds in IE NR-RTD-Info is supported or not; - bit 5 indicates whether the field nr-PRU-DL-Info in IE NR-PositionCalculationAssistance is supported or not.

nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance 'per-trp', 'per-resource', or both. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1, maxDL-PRS-RSRP-MeasurementFR2, dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1 and supportOfRSRP-MeasFR2. Otherwise, the UE does not include this field.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-DL-TDOA-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-TDOA-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both. NOTE: A single value is reported when both Multi-RTT and DL-TDOA are supported.

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-DL-TDOA-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-DL-TDOA-MeasurementCapability also applies to the additional paths.

scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

mg-ActivationRequest This field, if present, indicates that the target device supports UL MAC CE for positioning measurement gap activation/deactivation request for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationRequestPRS-Meas and mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

posMeasGapSupport This field, if present, indicates that the target device supports pre-configured positioning measurement gap for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

multiLocationEstimateInSameMeasReport This field, if present, indicates that the target device supports multiple location estimate instances in a single measurement report.

locationCoordinateTypes This field indicates the geographical location coordinate types that a target device supports for UE-based DL-TDOA. TRUE indicates that a location coordinate type is supported and FALSE that it is not.

periodicAssistanceData This field identifies the periodic NR assistance data delivery procedures supported by the target device. This is represented by a bit string, with a one value at the bit position means the periodic NR assistance data delivery procedure is supported; a zero value means not supported. Bit 0 (solicited) represents the procedure according to clause 5.2.1a; bit (1) (unsolicited) represents the procedure according to clause 5.2.2a.

nr-IntegrityAssistanceSupport This field indicates the Integrity Assistance Data supported. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-IntegrityServiceParameters in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-IntegrityServiceAlert in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-IntegrityRiskParameters in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-IntegrityParametersTRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 4 indicates whether the field nr-IntegrityParametersDL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 5 indicates whether the field nr-IntegrityParametersRTD-Info in IE NR-PositionCalculationAssistance is supported or not.

nr-DL-TDOA-OnDemandPRS-ForBWA-Support This field, if present, indicates that the target device supports on-demand DL-PRS request for bandwidth aggregation.

6.5.10.6a	NR DL-TDOA Capability Information Elements

–	NR-DL-TDOA-MeasurementCapability

The IE NR-DL-TDOA-MeasurementCapability defines the DL-TDOA measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for DL-TDOA. Otherwise, the UE does not include this IE.

-- ASN1START

NR-DL-TDOA-MeasurementCapability-r16 ::= SEQUENCE {

dl-RSTD-MeasurementPerPairOfTRP-FR1-r16			INTEGER (1..4),

dl-RSTD-MeasurementPerPairOfTRP-FR2-r16			INTEGER (1..4),

supportOfDL-PRS-RSRP-MeasFR1-r16				ENUMERATED { supported}	OPTIONAL,

supportOfDL-PRS-RSRP-MeasFR2-r16				ENUMERATED { supported}	OPTIONAL,

...,

[[

nr-UE-TEG-Capability-r17						NR-UE-TEG-Capability-r17			OPTIONAL,

dl-tdoa-MeasCapabilityBandList-r17				SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

DL-TDOA-MeasCapabilityPerBand-r17	OPTIONAL

]]

}

DL-TDOA-MeasCapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17				FreqBandIndicatorNR-r16,

supportOfDL-PRS-FirstPathRSRP-r17	ENUMERATED { supported }			OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17			ENUMERATED { supported }			OPTIONAL,

...,

[[

supportOfDL-PRS-BWA-RRC-Connected-r18			ENUMERATED { supported }		OPTIONAL,

supportOfDL-PRS-BWA-RRC-Inactive-r18			ENUMERATED { supported }		OPTIONAL,

supportOfDL-PRS-BWA-RRC-Idle-r18				ENUMERATED { supported }		OPTIONAL,

nr-DL-PRS-RSCPD-ReportingRRC-Connected-r18			ENUMERATED { supported }		OPTIONAL,

assocSingleRSTD-WithUpToNsampleRSCPD-r18		ENUMERATED { supported }		OPTIONAL,

nr-DL-PRS-RSCPD-MeasurementRRC-Idle-r18			ENUMERATED { supported }		OPTIONAL,

supportOfUE-basedCarrierPhasePositioning-r18	ENUMERATED { supported }		OPTIONAL,

supportOfSymbolTimeStampForRSCPD-r18			ENUMERATED { supported }		OPTIONAL,

supportOfFinerTimingReportGranularityForPRS-Meas-r18	ENUMERATED { minus1, minus2,

minus3, minus4, minus5, minus6}

OPTIONAL,

supportOfMeasurementsInTimeWindow-r18			BIT STRING {	rstd	(0),

rsrp	(1),

rsrpp	(2),

rscpd	(3)

}	(SIZE (1..8))	OPTIONAL,

supportOfPRS-MeasurementRRC-Idle-r18			ENUMERATED { supported }		OPTIONAL

]]

}

-- ASN1STOP

NR-DL-TDOA-MeasurementCapability field descriptions

dl-RSTD-MeasurementPerPairOfTRP-FR1 Indicates number of DL RSTD measurements per pair of TRPs on FR1.

dl-RSTD-MeasurementPerPairOfTRP-FR2 Indicates number of DL RSTD measurements per pair of TRPs on FR2.

supportOfDL-PRS-RSRP-MeasFR1 Indicates whether the UE supports DL-PRS RSRP measurement for DL-TDOA on FR1.

supportOfDL-PRS-RSRP-MeasFR2 Indicates whether the UE supports DL-PRS RSRP measurement for DL-TDOA on FR2.

nr-UE-TEG-Capability Indicates the UE TEG capability.

supportOfDL-PRS-FirstPathRSRP Indicates whether the target device supports DL-PRS RSRPP of first path measurement for DL-TDOA. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. The UE supporting additionalPathsReport and supportOfDL-PRS-FirstPathRSRP shall support RSRPP reporting for K=1 or 2 additional paths.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and dl-PRS-BufferType-RRC-Inactive. Otherwise, the UE does not include this field. NOTE 1: This capability is applicable to both, UE-assisted and UE-based DL-TDOA. NOTE 2: The capabilities NR-DL-PRS-ResourcesCapability, dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, supportOfDL-PRS-RSRP-MeasFR1, supportOfDL-PRS-RSRP-MeasFR2, simul-NR-DL-AoD-DL-TDOA are the same in RRC_INACTIVE state.

supportOfDL-PRS-BWA-RRC-Connected Indicates whether the target device supports DL-PRS bandwidth aggregation in RRC_CONNECTED for DL-TDOA. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field.

supportOfDL-PRS-BWA-RRC-Inactive Indicates whether the target device supports DL-PRS bandwidth aggregation in RRC_INACTIVE for DL-TDOA. The UE can include this field only if the UE supports dl-PRS-MeasRRC-Inactive and prs-BWA-TwoContiguousIntraband-RRC-IdleAndInactive. Otherwise, the UE does not include this field.

supportOfDL-PRS-BWA-RRC-Idle Indicates whether the target device supports DL-PRS bandwidth aggregation in RRC_IDLE for DL-TDOA. The UE can include this field only if the UE supports supportOfPRS-MeasurementRRC-Idle and prs-BWA-TwoContiguousIntraband-RRC-IdleAndInactive. Otherwise, the UE does not include this field.

nr-DL-PRS-RSCPD-ReportingRRC-Connected This field, if present, indicates that the target device supports reporting RSCPD in RRC CONNECTED. The UE can include this field only if the UE supports dl-RSTD-MeasurementPerPairOfTRP-FR1 and dl-RSTD-MeasurementPerPairOfTRP-FR2. Otherwise, the UE does not include this field. NOTE 3: RSCPD is reported together with RSTD measurement.

nr-DL-PRS-RSCPD-ReportingRRC-Inactive This field, if present, indicates that the target device supports reporting RSCPD in RRC INACTIVE. The UE can include this field only if the UE supports dl-PRS-MeasRRC-Inactive. Otherwise, the UE does not include this field. NOTE 4: RSCPD is reported together with RSTD measurement.

assocSingleRSTD-WithUpToNsampleRSCPD This field, if present, indicates that the target device supports associating a single RSTD measurement with up to N_sample RSCPD measurement. The UE can include this field only if the UE supports one of nr-DL-PRS-RSCPD-ReportingRRC-Connected and nr-DL-PRS-RSCPD-ReportingRRC-Inactive. Otherwise, the UE does not include this field.

nr-DL-PRS-RSCPD-MeasurementRRC-Idle This field, if present, indicates that the target device supports DL RSCPD measurement based on DL-PRS measurement in RRC_IDLE. The UE can include this field only if the UE supports supportOfPRS-MeasurementRRC-Idle. Otherwise, the UE does not include this field. NOTE 5: DL RSCPD is reported along with measurement report for DL-RSTD

supportOfLegacyMeasurementInTimeWindow This field, if present, indicates that the target device supports performing legacy measurements inside the indicated time window only for DL TDoA. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field.

supportOfUE-basedCarrierPhasePositioning This field, if present, indicates that the target device supports carrier phase measurement for UE-based positioning and support of assistance data for UE-based carrier phase positioning.

supportOfSymbolTimeStampForRSCPD This field, if present, indicates that the target device supports reporting timestamp with OFDM symbol index associated with RSCPD measurement. The UE can include this field only if the UE supports one of nr-DL-PRS-RSCPD-ReportingRRC-Connected and nr-DL-PRS-RSCPD-ReportingRRC-Inactive. Otherwise, the UE does not include this field.

supportOfFinerTimingReportGranularityForPRS-Meas This field, if present, indicates that the target device supports of finer timing reporting granularity for DL-PRS measurement.

supportOfMeasurementsInTimeWindow This field, if present, indicates that the target device supports performing measurements inside the indicated time window only for DL TDOA. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. - bit 0 indicates whether performing DL RSTD is supported or not; - bit 1 indicates whether performing DL PRS-RSRP is supported or not; - bit 2 indicates whether performing DL PRS-RSRPP is supported or not; - bit 3 indicates whether performing DL PRS-RSCPD is supported or not; The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field.

supportOfPRS-MeasurementRRC-Idle Indicates whether the UE supports DL-PRS measurement in RRC_IDLE for DL-TDOA. The UE can include this field only if the UE supports supportedBandwidthPRS, dl-PRS-BufferType, durationOfPRS-Processing, maxNumOfDL-PRS-ResProcessedPerSlot, dl-PRS-BufferType-RRC-Inactive, durationOfPRS-Processing-RRC-Inactive, maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive and dl-PRS-MeasRRC-Inactive. Otherwise, the UE does not include this field.

6.5.11.6	NR DL-AoD Capability Information

–	NR-DL-AoD-ProvideCapabilities

The IE NR-DL-AoD-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-AoD and to provide its NR DL-AoD positioning capabilities to the location server.

-- ASN1START

NR-DL-AoD-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-AoD-Mode-r16						PositioningModes,

nr-DL-AoD-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-AoD-MeasurementCapability-r16		NR-DL-AoD-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

periodicalReporting-r16					PositioningModes							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

beamAntInfoSup	(3),

nr-IntegrityBoundsSup-r18	(4)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,

dl-PRS-ResourcePrioritySubset-Sup-r17	ENUMERATED { sameSet, differentSet, sameOrDifferentSet }

OPTIONAL,

nr-DL-PRS-BeamInfoSup-r17				ENUMERATED { supported }					OPTIONAL,

nr-DL-AoD-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		LOS-NLOS-IndicatorType2-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity2-r17,

...

}											OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupportPerMode-r17

OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)			OPTIONAL,

...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]],

[[

posMeasGapSupport-r17					ENUMERATED { supported }					OPTIONAL

]],

[[

multiLocationEstimateInSameMeasReport-r17	ENUMERATED { supported }				OPTIONAL

]],

[[

locationCoordinateTypes-r18				LocationCoordinateTypes						OPTIONAL,

nr-IntegrityAssistanceSupport-r18		BIT STRING {

serviceParametersSup		(0),

serviceAlertSup				(1),

riskParametersSup			(2),

integrityParaTRP-LocSup		(3),

integrityParaBeamInfoSup	(4),

integrityParaRTD-InfoSup	(5),

integrityBeamAntInfoSup		(6)

} (SIZE (1..8)) 					OPTIONAL

]]

}

-- ASN1STOP

| NR-DL-AoD-ProvideCapabilities field descriptions |
| --- |
| nr-DL-AoD-Mode This field specifies the NR DL-AoD mode(s) supported by the target device. |
| periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation. |
| ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation. |
| nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-AoD. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not. The UE can indicate this bit only if the UE supports prs-ProcessingCapabilityBandList and any of maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer, maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field; - bit 3 indicates whether the field nr-TRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 4 indicates whether the target service supports the range of integrity risk (IR) for which the integrity assistance data are valid; - bit 5 together with bit 0 indicates whether the fields nr-IntegrityTRP-LocationBounds, nr-IntegrityDL-PRS-ResourceSetARP-LocationBounds, nr-IntegrityDL-PRS-ResourceARP-LocationBounds in IE NR-TRP-LocationInfo are supported or not; bit 5 together with bit 1 indicates whether the field nr-IntegrityBeamInfoBounds in IE NR-DL-PRS-BeamInfo is supported or not; bit 5 together with bit 2 indicates whether the field nr-IntegrityRTD-InfoBounds in IE NR-RTD-Info is supported or not; bit 5 together with bit 3 indicates whether the field nr-integrityBeamPowerBounds in IE NR-TRP-BeamAntennaInfo is supported or not. |
| nr-los-nlos-AssistanceDataSupport This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedLOS-NLOS-Assistance in IE NR-PositionCalculationAssistance: - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in LOS-NLOS-Indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-ExpectedLOS-NLOS-Assistance 'per-trp', 'per-resource', or both. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1, maxDL-PRS-RSRP-MeasurementFR2,dl-RSTD-MeasurementPerPairOfTRP-FR1, dl-RSTD-MeasurementPerPairOfTRP-FR2, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1 and supportOfRSRP-MeasFR2 . Otherwise, the UE does not include this field. |
| nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData. |
| dl-PRS-ResourcePrioritySubset-Sup This field, if present, indicates that the target device supports the DL-PRS-ResourcePrioritySubset in IE NR-DL-PRS-Info. Enumerated value indicates the supported resource set relationship for the target DL-PRS Resource and the associated subset. |  |
| nr-DL-PRS-BeamInfoSup This field, if present, indicates that the target device supports the NR-DL-PRS-BeamInfo in IE NR-DL-AoD-ProvideAssistanceData. |
| nr-DL-AoD-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests. |
| nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-AoD-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both. |
| scheduledLocationRequestSupported This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests. |
| nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports. |
| multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report. |
| mg-ActivationRequest This field, if present, indicates that the target device supports UL MAC CE for positioning measurement gap activation/deactivation request for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationRequestPRS-Meas and mg-ActivationCommPRS-Meas defined in TS 38.331 [35]. |
| posMeasGapSupport This field, if present, indicates that the target device supports pre-configured positioning measurement gap for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationCommPRS-Meas defined in TS 38.331 [35]. |
| multiLocationEstimateInSameMeasReport This field, if present, indicates that the target device supports multiple location estimate instances in a single measurement report. |
| locationCoordinateTypes This field indicates the geographical location coordinate types that a target device supports for UE-based DL-AoD. TRUE indicates that a location coordinate type is supported and FALSE that it is not. |
| nr-IntegrityAssistanceSupport This field indicates the Integrity Assistance Data supported. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-IntegrityServiceParameters in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-IntegrityServiceAlert in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-IntegrityRiskParameters in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-IntegrityParametersTRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 4 indicates whether the field nr-IntegrityParametersDL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 5 indicates whether the field nr-IntegrityParametersRTD-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 6 indicates whether the field nr-IntegrityParametersTRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is supported or not. |

6.5.11.6a	NR DL-AoD Capability Information Elements

–	NR-DL-AoD-MeasurementCapability

The IE NR-DL-AoD-MeasurementCapability defines the DL-AoD measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for DL-AoD. Otherwise, the UE does not include this IE;

-- ASN1START

NR-DL-AoD-MeasurementCapability-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementFR1-r16		INTEGER (1..8),

maxDL-PRS-RSRP-MeasurementFR2-r16		INTEGER (1..8),

dl-AoD-MeasCapabilityBandList-r16		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

DL-AoD-MeasCapabilityPerBand-r16,

...,

[[

maxDL-PRS-RSRP-MeasurementFR1-v1730		ENUMERATED { n16, n24 }					OPTIONAL,

maxDL-PRS-RSRP-MeasurementFR2-v1730		ENUMERATED { n16, n24 }					OPTIONAL

]]

}

DL-AoD-MeasCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16					FreqBandIndicatorNR-r16,

simul-NR-DL-AoD-DL-TDOA-r16				ENUMERATED { supported}					OPTIONAL,

simul-NR-DL-AoD-Multi-RTT-r16			ENUMERATED { supported}					OPTIONAL,

...,

[[

maxDL-PRS-FirstPathRSRP-MeasPerTRP-r17	ENUMERATED { n1, n2, n4, n8, n16, n24 }	OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17				ENUMERATED { supported }				OPTIONAL

]],

[[

supportOfMeasurementsInTimeWindow-r18	BIT STRING {

rsrp		(0),

rsrpp		(1)

}	(SIZE (1..8))		OPTIONAL,

supportOfPRS-MeasurementRRC-Idle-r18	ENUMERATED { supported }		OPTIONAL

]]

}

-- ASN1STOP

NR-DL-AoD-MeasurementCapability field descriptions

maxDL-PRS-RSRP-MeasurementFR1 Indicates the maximum number of DL-PRS RSRP measurements on different DL-PRS Resources from the same TRP supported by the UE on FR1. If this field with suffix -v1730 is present, the target device should set the field with suffix -r16 to value '8'.

maxDL-PRS-RSRP-MeasurementFR2 Indicates the maximum number of DL-PRS RSRP measurements on different DL-PRS Resources from the same TRP supported by the UE on FR2. If this field with suffix -v1730 is present, the target device should set the field with suffix -r16 to value '8'.

simul-NR-DL-AoD-DL-TDOA Indicates whether the UE supports simultaneous processing for DL-AoD and DL-TDOA measurements. The UE can include this field only if the UE supports DL-TDOA and DL-AoD. Otherwise, the UE does not include this field.

simul-NR-DL-AoD-Multi-RTT Indicates whether the UE supports simultaneous processing for DL-AoD and UE Multi-RTT measurements. The UE can include this field only if the UE supports Multi-RTT and DL-AoD. Otherwise, the UE does not include this field.

maxDL-PRS-FirstPathRSRP-MeasPerTRP This field, if present, indicates that the target device supports measuring and reporting the DL-PRS RSRPP of the first path. The enumerated value indicates the maximum number of RSRPP of first path per TRP supported. The UE can include this field only if the UE supports one of maxDL-PRS-RSRP-MeasurementFR1 and maxDL-PRS-RSRP-MeasurementFR2. Otherwise, the UE does not include this field. NOTE 1: The maximum number of first path DL-PRS RSRP per TRP should be less than or equal to the maximum number of DL-PRS RSRP defined in maxDL-PRS-RSRP-MeasurementFR1 and maxDL-PRS-RSRP-MeasurementFR2.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and dl-PRS-BufferType-RRC-Inactive. Otherwise, the UE does not include this field. NOTE 1: This capability is applicable to both, UE-assisted and UE-based DL-AoD. NOTE 2: The capabilities NR-DL-PRS-ResourcesCapability, simul-NR-DL-AoD-DL-TDOA are the same in RRC_INACTIVE state.

supportOfMeasurementsInTimeWindow This field, if present, indicates that the target device supports performing measurements inside the indicated time window only for DL AoD. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. - bit 0 indicates whether performing DL PRS-RSRP is supported or not; - bit 1 indicates whether performing DL PRS-RSRPP is supported or not; The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field.

supportOfPRS-MeasurementRRC-Idle Indicates whether the UE supports DL-PRS measurement in RRC_IDLE for DL-AoD. The UE can include this field only if the UE supports supportedBandwidthPRS, dl-PRS-BufferType, durationOfPRS-Processing, maxNumOfDL-PRS-ResProcessedPerSlot, dl-PRS-BufferType-RRC-Inactive, durationOfPRS-Processing-RRC-Inactive, maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive and dl-PRS-MeasRRC-Inactive. Otherwise, the UE does not include this field.

6.5.12.6	NR Multi-RTT Capability Information

–	NR-Multi-RTT-ProvideCapabilities

The IE NR-Multi-RTT-ProvideCapabilities is used by the target device to indicate its capability to support NR Multi-RTT and to provide its NR Multi-RTT positioning capabilities to the location server.

-- ASN1START

NR-Multi-RTT-ProvideCapabilities-r16 ::= SEQUENCE {

nr-Multi-RTT-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-Multi-RTT-MeasurementCapability-r16	NR-Multi-RTT-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

nr-UL-SRS-Capability-r16				NR-UL-SRS-Capability-r16,

additionalPathsReport-r16				ENUMERATED { supported }					OPTIONAL,

periodicalReporting-r16					ENUMERATED { supported }					OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			ENUMERATED { supported }					OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,	nr-Multi-RTT-On-Demand-DL-PRS-Support-r17

NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-UE-RxTx-TEG-ID-ReportingSupport-r17	BIT STRING {	case1	(0),

case2	(1),

case3	(2)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17	SEQUENCE {

type-r17			LOS-NLOS-IndicatorType2-r17,

granularity-r17		LOS-NLOS-IndicatorGranularity2-r17,

...

}												OPTIONAL,

additionalPathsExtSupport-r17			ENUMERATED { n4, n6, n8 }					OPTIONAL,

scheduledLocationRequestSupported-r17	ScheduledLocationTimeSupport-r17			OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxNrOfAreas-r17)		OPTIONAL,

...

}															OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL

]],

[[

posMeasGapSupport-r17					ENUMERATED { supported }					OPTIONAL

]],

[[

nr-MultiRTT-OnDemandPRS-ForBWA-Support-r18	ENUMERATED { supported }				OPTIONAL

]]

}

-- ASN1STOP

NR-Multi-RTT-ProvideCapabilities field descriptions

ten-ms-unit-ResponseTime This field, if present, indicates that the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-Multi-RTT-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-UE-RxTx-TEG-ID-ReportingSupport This field, if present, indicates that the target device supports nr-UE-RxTx-TEG-Info reporting in IE NR-Multi-RTT-SignalMeasurementInformation. This is represented by a bit string, with a one-value at the bit position means the particular case is supported; a zero-value means not supported: - bit 0 indicates that the target device supports the 'case1' choice in NR-UE-RxTx-TEG-Info. - bit 1 indicates that the target device supports the 'case2' choice in NR-UE-RxTx-TEG-Info. - bit 2 indicates that the target device supports the 'case3' choice in NR-UE-RxTx-TEG-Info.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-Multi-RTT-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value or 'hard' and 'soft' value in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both. NOTE: A single value is reported when both Multi-RTT and DL-TDOA are supported.

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-Multi-RTT-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device. NOTE: The supportOfDL-PRS-FirstPathRSRP in IE NR-Multi-RTT-MeasurementCapability also applies to the additional paths.

scheduledLocationRequestSupported This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationTime in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of areas the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

mg-ActivationRequest This field, if present, indicates that the target device supports UL MAC CE for positioning measurement gap activation/deactivation request for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationRequestPRS-Meas and mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

posMeasGapSupport This field, if present, indicates that the target device supports pre-configured positioning measurement gap for DL-PRS measurements. The UE can include this field only if the UE supports mg-ActivationCommPRS-Meas defined in TS 38.331 [35].

nr-MultiRTT-OnDemandPRS-ForBWA-Support This field, if present, indicates that the target device supports on-demand DL-PRS request for bandwidth aggregation.

6.5.12.6a	NR Multi-RTT Capability Information Elements

–	NR-Multi-RTT-MeasurementCapability

The IE NR-Multi-RTT-MeasurementCapability defines the Multi-RTT measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for Multi-RTT. Otherwise, the UE does not include this IE;

-- ASN1START

NR-Multi-RTT-MeasurementCapability-r16 ::= SEQUENCE {

maxNrOfRx-TX-MeasFR1-r16					INTEGER (1..4)						OPTIONAL,

maxNrOfRx-TX-MeasFR2-r16					INTEGER (1..4)						OPTIONAL,

supportOfRSRP-MeasFR1-r16					ENUMERATED { supported }			OPTIONAL,

supportOfRSRP-MeasFR2-r16					ENUMERATED { supported }			OPTIONAL,

srs-AssocPRS-MultiLayersFR1-r16				ENUMERATED { supported }			OPTIONAL,

srs-AssocPRS-MultiLayersFR2-r16				ENUMERATED { supported }			OPTIONAL,

...,

[[

nr-UE-TEG-Capability-r17					NR-UE-TEG-Capability-r17			OPTIONAL,

multi-RTT-MeasCapabilityBandList-r17		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

Multi-RTT-MeasCapabilityPerBand-r17

OPTIONAL

]]

}

Multi-RTT-MeasCapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17					FreqBandIndicatorNR-r16,

supportOfDL-PRS-FirstPathRSRP-r17		ENUMERATED { supported }			OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17				ENUMERATED { supported }			OPTIONAL,

...,

[[

supportOfDL-PRS-BWA-RRC-Connected-r18		ENUMERATED { supported }				OPTIONAL,

supportOfDL-PRS-BWA-RRC-Inactive-r18		ENUMERATED { supported }				OPTIONAL,

nr-NTN-MeasAndReport-r18					ENUMERATED { supported }				OPTIONAL,

nr-DL-PRS-RSCP-ReportingRRC-Connected-r18	ENUMERATED { supported }				OPTIONAL,

nr-DL-PRS-RSCP-ReportingRRC-Inactive-r18	ENUMERATED { supported }				OPTIONAL,

assocSingleRx-Tx-WithUpToNsampleRSCP-r18	ENUMERATED { supported }				OPTIONAL,

supportOfSymbolTimeStampForRSCP-r18			ENUMERATED { supported }				OPTIONAL,

supportOfFinerTimingReportGranularityForPRS-Meas-r18	ENUMERATED { minus1, minus2,

minus3, minus4, minus5, minus6}		OPTIONAL,

supportOfMeasurementsInTimeWindow-r18		BIT STRING {	rx-tx	(0),

rsrp	(1),

rsrpp	(2),

rscp	(3)

}	(SIZE (1..8))		OPTIONAL

]]

}

-- ASN1STOP

NR-Multi-RTT-MeasurementCapability field descriptions

maxNrOfRx-TX-MeasFR1 Indicates the maximum number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL-PRS Resource/Resource Set on FR1.

maxNrOfRx-TX-MeasFR2 Indicates the maximum number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL-PRS Resource/Resource Set on FR2.

srs-AssocPRS-MultiLayersFR1 Indicates whether the UE supports measurements derived on one or more DL-PRS Resource/Resource Sets which may be in different positioning frequency layers for SRS transmitted in a single CC. DL-PRS and SRS may be on different bands. This is for FR1 only.

srs-AssocPRS-MultiLayersFR2 Indicates whether the UE supports measurements derived on one or more DL-PRS Resource/Resource Sets which may be in different positioning frequency layers for SRS transmitted in a single CC. DL-PRS and SRS may be on different bands. This is for FR2 only.

supportOfRSRP-MeasFR1 Indicates whether the UE supports RSRP measurement for Multi-RTT on FR1.

supportOfRSRP-MeasFR2 Indicates whether the UE supports RSRP measurement for Multi-RTT on FR2.

nr-UE-TEG-Capability Indicates the UE TEG capability.

supportOfDL-PRS-FirstPathRSRP Indicates whether the target device supports DL-PRS RSRPP of first path measurement for Multi-RTT. The UE can include this field only if the UE supports prs-ProcessingCapabilityBandList. Otherwise, the UE does not include this field. The UE supporting additionalPathsReport and supportOfDL-PRS-FirstPathRSRP shall support RSRPP reporting for K=1 or 2 additional paths.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state. The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and dl-PRS-BufferType-RRC-Inactive. Otherwise, the UE does not include this field. NOTE 1: The capabilities NR-DL-PRS-ResourcesCapability, maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1, supportOfRSRP-MeasFR2, srs-AssocPRS-MultiLayersFR1, srs-AssocPRS-MultiLayersFR2, simul-NR-DL-AoD-Multi-RTT are the same in RRC_INACTIVE state.

supportOfDL-PRS-BWA-RRC-Connected Indicates whether the target device supports DL-PRS bandwidth aggregation in RRC_CONNECTED for Multi-RTT. The target device can include this field only if the target device supports maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer, maxNrOfTRP-AcrossFreqs, maxNrOfPosLayer and prs-BWA-TwoContiguousIntrabandInMG-RRC-Connected. Otherwise, the UE does not include this field.

supportOfDL-PRS-BWA-RRC-Inactive Indicates whether the target device supports DL-PRS bandwidth aggregation in RRC_INACTIVE for Multi-RTT. The target device can include this field only if the target device supports dl-PRS-MeasRRC-Inactive and prs-BWA-TwoContiguousIntraband-RRC-IdleandInactive. Otherwise, the target device does not include this field.

nr-NTN-MeasAndReport This field, if present, indicates that the UE supports UE Rx-Tx Measurement and Report for Multi-RTT with single satellite in NTN with the following capabilities: - UE Rx-Tx time difference and UE Rx-Tx time difference offset measurement and report for Multi-RTT positioning; - Reporting DL timing drift due to Doppler over the service link associated with the UE Rx-Tx time difference measurement period. NOTE 2: This field is only present, if freqBandIndicatorNR indicates the bands in Table 5.2.2-1 in TS 38.101-5 [54].

nr-DL-PRS-RSCP-ReportingRRC-Connected This field, if present, indicates that the target device supports reporting RSCP in RRC CONNECTED. The UE can include this field only if the UE supports maxNrOfRx-TX-MeasFR1, maxNrOfRx-TX-MeasFR2, supportOfRSRP-MeasFR1 and supportOfRSRP-MeasFR2. Otherwise, the UE does not include this field. NOTE 3: RSCP is reported together with UE Rx-Tx time difference measurement.

nr-DL-PRS-RSCP-ReportingRRC-Inactive This field, if present, indicates that the target device supports reporting RSCP in RRC INACTIVE. The UE can include this field only if the UE supports dl-PRS-MeasRRC-Inactive. Otherwise, the UE does not include this field. NOTE 4: RSCP is reported together with UE Rx-Tx time difference measurement.

assocSingleRx-Tx-WithUpToNsampleRSCP This field, if present, indicates that the target device supports associating a single Rx-Tx measurement with up to N_sample RSCP measurement. The UE can include this field only if the UE supports one of nr-DL-PRS-RSCP-ReportingRRC-Connected and nr-DL-PRS-RSCP-ReportingRRC-Inactive. Otherwise, the UE does not include this field.

supportOfSymbolTimeStampForRSCP This field, if present, indicates that the target device supports reporting timestamp with OFDM symbol index associated with RSCP measurement. The UE can include this field only if the UE supports one of nr-DL-PRS-RSCP-ReportingRRC-Connected and nr-DL-PRS-RSCP-ReportingRRC-Inactive. Otherwise, the UE does not include this field.

supportOfFinerTimingReportGranularityForPRS-Meas This field, if present, indicates that the target device supports finer timing reporting granularity for DL-PRS measurement.

supportOfMeasurementsInTimeWindow This field, if present, indicates that the target device supports performing measurements inside the indicated time window only for Multi RTT. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. - bit 0 indicates whether performing UE Rx-Tx time difference is supported or not; - bit 1 indicates whether performing DL PRS-RSRP is supported or not; - bit 2 indicates whether performing DL PRS-RSRPP is supported or not; - bit 3 indicates whether performing DL PRS-RSCP is supported or not; The UE can include this field only if the UE supports maxNrOfDL-PRS-ResourcesPerResourceSet and maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer. Otherwise, the UE does not include this field.

End of the change