3GPP TSG-RAN WG2 Meeting #126	R2-2406061

# Fukuoka, Japan, 20th – 24th May 2024	was R3-243845

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0397 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Handover Cancel in CHO with SN for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (Samsung, ZTE, Ericsson, Huawei) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In CHO with SN in EN-DC, multiple candidate target MNs with candidate target SNs are prepared. When the UE selects one target MN and one target SN, the source MN sends the Handover Cancel message(s) to the other candidate MN(s) and may send it to the target MN if the target MN configures the other candidate PCell. Then the candidate MN(s) and the target MN trigger the SN Release procedure to the corresponding SNs. In case of both the other candidate MN(s) and the target MN configure the PSCell in the target SN, the behaviour of the other candidate MN(s) and the target MN are different upon receiving the Handover Cancel message from the source MN: - For the target MN, the target MN doesn’t send the SgNB Release Request message to the target SN since the target MN knows this SN is selected as the target SN by the UE. - For the other candidate MN(s), the other candidate MN(s) needs to send the SgNB Release Request message(s) to the candidate SN(s) including the target SN, since the other candidate MN(s) doesn’t know which SN is selected as the target SN by the UE. However, in the current figure 10.19.1-1, the other potential target MN configures SCG in the target SN, but it misses to send the SgNB Release Request message to the target SN to release the SCG after receiving the Handover Cancel message from the source MN. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the Figure 10.19.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Imply the other candidate MN(s) knows which candidate SN is selected by the UE as the target SN which is not true. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was endorsed by RAN3 in R3-243845. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

## 10.19	Conditional Handover with Secondary Node

### 10.19.1	EN-DC

The Conditional Handover with Secondary Node procedure is used for configuration and execution of CHO with SN. This procedure includes the cases where the SN is kept, changed or added. If the SN is kept, the UE context at the SN is kept. If the SN is changed, the UE context at the source SN is moved to the target SN.

Figure 10.19.1-1: Conditional Handover with Secondary Node procedure

<<<<<<<<<<<<<<<<<<<< End of the change >>>>>>>>>>>>>>>>>>>>