3GPP TSG-RAN WG2 Meeting #126	R2-2405852

# Fukuoka, Japan, 20th – 24th May 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1086 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The 80ms offset for posSI scheduling is used in NR. This feature is mandatory when UE acquires positioning SI. The mandatory capability information is missing in TS 38.306. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of 80ms scheduling offset for positioning SI acquisition feature as Conditionally mandatory features without UE radio access capability parameters Impact Analysis Impacted functionality: Positioning SI Acquisition Inter-operability: - No inter-operability issue is seen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing mandatory capability information which may lead UE to not implement it and thus unable to receive the desired positioning SIBs and Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of Changes

# 6	Conditionally mandatory features without UE radio access capability parameters

| Features | Condition |
| --- | --- |
| Acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList | It is mandatory to support acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList for UEs which support the acquisition of the posSIB types in posSchedulingInfoList as specified in TS 38.331 [9]. |
| Acquisition of SI messages with explicit SI window positions | It is mandatory to support acquisition of SI messages with explicit SI window positions for UEs which support the SIB types in schedulingInfoList2 as specified in TS 38.331 [9]. |
| AS layer memory size for QoE paused measurement reports | It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. |
| AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE | It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports" |
| ATG specific P-max | It is mandatory to support the ATG specific P-max configured by network for UEs supporting airToGroundNetwork-r18. |
| Downlink SDAP header | Either NAS reflective QoS or as-ReflectiveQoS is supported. |
| Extended values for drx-HARQ-RTT-TimerDL/UL | It is mandatory for UEs which support FR2-2 bands with SCS 480kHz and/or 960kHz. |
| IMS emergency call | It is mandatory to support IMS emergency call over PLMN for UEs which are IMS voice capable in NR. It is mandatory to support IMS emergency call over SNPN for UEs that are SNPN capable and IMS voice capable over SNPNs. |
| Logged measurements suspension due to IDC interference | It is mandatory to support Logged measurements suspension due to IDC interference for UEs which are supporting logged measurements in RRC_IDLE and RRC_INACTIVE upon request from the network and in-device coexistence indication as specified in TS 38.331 [9]. |
| MAC subheaders with LX field | It is mandatory to support MAC subheaders with LX field for UEs supporting MAC SDU(s) using the LCID value(s) as specified in Table 6.2.1-2c in TS 38.321 [8]. |
| MAC subheaders with one-octet eLCID field | It is mandatory to support MAC subheaders with one-octet eLCID field for UEs/IAB-MTs supporting MAC CEs using extended LCID values as specified in TS 38.321 [8]. |
| Paging cause in RAN paging message | It is mandatory for a UE to support paging cause in RAN paging if UE supports paging cause in CN paging. |
| SON report in PNI-NPN | It is mandatory for a UE to support a SON report in PNI-NPN if UE supports PNI-NPN and supports the SON report in PLMN. |
| Skipping UL configured grant if no data to transmit, as specified in release-15 version of TS 38.321 [8]. | Either configuredUL-GrantType1 or configuredUL-GrantType1-v1650 or configuredUL-GrantType2 or configuredUL-GrantType2-v1650 is supported. |
| TA reporting during initial access | It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 or uplinkTA-ReportingATG-r18 as specified in TS 38.321 [8]. |

End of Change