3GPP TSG-RAN WG2 Meeting #126	R2-2406120

# Fukuoka, Japan, May 20th – 24th 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1128 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE capability for misclassification of RLF reports as Too Early HO failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-21 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR is to add a capability without signalling for the UE when the UE is not to log some of the fields in the RLF report upon radio link failure after successful fast MCG recovery. In Rel-18 RRC TS, the procedural text for logging RLF report leads to wrong classification of the HO failure and hence leads to wrong tuning of mobility control parameters. An excerpt from the RRC TS 38.331 (version 17.7.0) is shown in the following. 2> if an RRCReconfiguration message including the reconfigurationWithSync was received before the connection failure: 3> if the last successfully executed RRCReconfiguration message including the reconfigurationWithSync concerned an intra NR handover and it was received while connected to the previous PCell to which the UE was connected before connecting to the PCell where radio link failure is detected; and 3> if T311 was not running before entering the PCell in which the radio link failure was detected: 4> include the nrPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the PCell where the last executed RRCReconfiguration message including reconfigurationWithSync was received; 4> if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a DAPS handover: 5> set lastHO-Type to daps; 4> else if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover: 5> set lastHO-Type to cho; 4> set the timeConnFailure to the elapsed time since the execution of the last RRCReconfiguration message including the reconfigurationWithSync; As shown above, the procedural text for the radio link failure is written to avoid logging the previousPCellId and timeConnFailure for the cases that RLF is detected shortly after the following scenarios: - RLF after a successful re-establishment - RLF after a successful CHO recovery - RLF after RRC connection (transition from IDLE mode to connected mode) However, there is still one scenario missing in the current procedural text that is RLF shortly after fast MCG recovery. According to the current procedural text the UE logs previousPCellId and timeConnFailure when RLF occurs shortly after a fast MCG recovery procedure. This creates problem at the network side when analysing the RLF-report as the above information is logged to detect the Too Early HO failure. In other words, the above information creates confusion for the network in adjusting the HO parameters, which should be avoided. Therefore, similar to the other scenarios mentioned above the procedural text is corrected to avoid logging previousPCellId and timeConnFailure in the mentioned scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A capability without signaling is considered for the feature of avoiding to log the previous PCell ID and timeConnFailure in case the RLF occurred after a successful fast MCG recovery. Impact analysis Impacted functionality: RLF-report |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature of UE avoiding to log the previousPcellId and timeConnFailure in the RLF report will remain mandatory for Rel-18 UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.331 CR 4814 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Coversheet update by MCC: wrong WI code (should use the same WI as the cat F CR). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

# 5	Optional features without UE radio access capability parameters

<text omitted>

## 5.7	MDT and SON features

Definitions for feature

Cross RAT RLF Report It is optional for UE to support the delivery of EUTRA RLF report to an NR node upon request from the network.

Mobility history information storage It is optional for UE to support the storage of PCell mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [9].

Radio Link Failure Report for inter-RAT MRO EUTRA It is optional for UE to support: - Inclusion of EUTRA CGI and associated TAC, if available, and otherwise to include the physical cell identity and carrier frequency of the target PCell of the failed handover as failedPCellId in RLF-Report upon request from the network as specified in TS 38.331 [9]. - Inclusion of EUTRA CGI and associated TAC as previousPCellId in RLF-Report as specified in TS 38.331 [9]. - Inclusion of eutraReconnectCellId in reconnectCellId in the RLF-Report as specified in TS 38.331 [9] upon UE has radio link failure or handover failure and successfully re-connected to an E-UTRA cell.

RACH Partitioning Information It is optional for UE to support the delivery of RACH partitioning related information via RACH report procedure, upon request from the network.

RLF Report for Fast MCG Recovery It is optional for UE to support the delivery of the Fast MCG recovery related information in the RLF-Report.

RLF Report for Inter-system HO for Voice Fallback It is optional for UE to support the delivery of an explicit indication in the RLF-report when mobility from NR due to voice fallback fails.

RLF report after successful fast MCG recovery It is optional for UE to support logging previousPCellId, lastHO-Type, and timeConnFailure when T316 was not running before entering the PCell in which the radio link failure was detected.

SCG Failure Report for CPAC It is optional for UE to support the delivery of the CPAC related parameters for MRO in SCGFailureInformation message to the network.

SCG Failure Report for MRO It is optional for UE to support the delivery of the SCG failure related parameters for MRO in SCGFailureInformation message to the network.

SON enhancements for NR-U It is optional for UE to support the delivery of NR-U related information (FR1 only) in RA-report/SHR/RLF report, upon request from the network.

SON Report in SNPN It is optional for UE to support collection and delivery of SON reports in SNPN. UE is not required to support all SON reports if it supports collection and delivery of the SON reports in SNPN, it may support one or more SON report for SNPN.

SpCell ID indication It is optional for UE to support the delivery of the spCellID-r17 in the RA-Report, if the RA procedure is performed in a SCell of the MCG/SCG.

Uplink PDCP delay measurements upon MO update It is optional for UE to support not resetting the UL PDCP Packet Average Delay measurement or UL PDCP excess packet delay measurement when the associated measurement object is modified. A UE supporting this feature shall also indicate the support of at least one of ulPDCP-Delay-r16 and excessPacketDelay-r17.

<text omitted>

End of the change