3GPP TSG-RAN WG2 Meeting #126	R2-2405771

# Fukuoka, Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1842 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MAC reset for multicast reception in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Samsung, CATT, Huawei, HiSilicon, Nokia, Sharp, Qualcomm Incorporated, Ericsson, Langbo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For change 1: During RRC Resume procedure, Upon receiving the RRCResume or RRCSetup message, UE will reset MAC if UE is configured with multicast reception in RRC_INACTIVE. The description in 38.331 #### 5.3.13.4 Reception of the RRCResume by the UE The UE shall: …… 1> if the UE is configured with multicast reception in RRC_INACTIVE: 2> reset MAC; ……. #### 5.3.13.7 Reception of the RRCSetup by the UE The UE shall: - perform the RRC connection setup procedure as specified in 5.3.3.4. #### 5.3.3.4 Reception of the RRCSetup by the UE The UE shall perform the following actions upon reception of the RRCSetup: 1> if the RRCSetup is received in response to an RRCReestablishmentRequest; or 1> if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1: …… 2> if the UE is configured with multicast reception in RRC_INACTIVE: 3> reset MAC; #### ……. The intention to introduce the MAC reset is to reset the MAC timers and reset the HARQ variables used for the multicast reception in RRC_INACTIVE. But according to current UE operation in MAC reset, UE will stop the TATimer and cancel the TAR reporting (for NTN), which will impact the legacy behavior. To avoid the impact on legacy UE behavior, the MAC reset during RRC resumption procedure is proposed to be described separately. For change 2: Currently, the MAC entity would accept data transmitted for a multicast MRB that is not configured for MBS multicast reception in RRC_INACTIVE. This is because an (e)LCID associated with a multicast MRB that is not configured for MBS multicast reception in RRC_INACTIVE and an (e)LCID associated with a multicast MRB that is configured for MBS multicast reception in RRC_INACTIVE can be received by a same G-RNTI. Hence, this should be accounted as unknown, unforeseen and erroneous protocol data and be discarded. And RAN2 made the following agreement in RAN2#126 meeting. - The MAC entity shall discard the received subPDU containing an LCID or eLCID value associated with a suspended multicast MRB. |
| Summary of change: |  | Change 1: Add the description for the MAC reset upon receiving the RRCResume or RRCSetup message separately, which includes the following UE operations: - stop the MBS multicast DRX timers; - flush the soft buffers for all DL HARQ processes used for MBS multicast; - for each DL HARQ process used for MBS multicast, consider the next received transmission for a TB as the very first transmission. Change 2: Add a condition that when this MAC entity receives a MAC PDU for the MAC entity's G-RNTI, containing an LCID or eLCID value associated with a suspended RB as specified in TS 38.331 [5], the MAC entity shall at least discard the received subPDU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. UE will stop TATimer and cancel TAR procedure upon receiving RRCResume or RRCSetup message. 2. MAC entity of the UE would receive data transmitted on a multicast MRB while such bearer was not configured for multicast reception in RRC_INACTIVE. This could lead to error in the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.12, 5.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.12	MAC Reset

If a reset of the MAC entity is requested by upper layers upon receiving RRCResume or RRCSetup, the MAC entity shall:

1>	stop the MBS multicast DRX timers;

1> flush the soft buffers for all DL HARQ processes used for MBS multicast;

1>	for each DL HARQ process used for MBS multicast, consider the next received transmission for a TB as the very first transmission.

Otherwise, if a reset of the MAC entity is requested by upper layers or the reset of the MAC entity is triggered due to SCG deactivation as defined in clause 5.29, the MAC entity shall:

1>	if the MAC reset is not due to SCG deactivation:

2>	initialize Bj for each logical channel to zero;

1>	initialize SBj for each logical channel to zero if Sidelink resource allocation mode 1 is configured by RRC;

1>	if upper layers indicate SCG deactivation and bfd-and-RLM with value true is configured for the deactivated SCG:

2>	stop (if running) all timers except beamFailureDetectionTimer associated with PSCell and timeAlignmentTimers.

1>	else:

2>	stop (if running) all timers, except MBS broadcast DRX timers;

2>	consider all timeAlignmentTimers, inactivePosSRS-TimeAlignmentTimer, and cg-SDT-TimeAlignmentTimer, if configured, as expired and perform the corresponding actions in clause 5.2;

1>	set the NDIs for all uplink HARQ processes to the value 0;

1>	sets the NDIs for all HARQ process IDs to the value 0 for monitoring PDCCH in Sidelink resource allocation mode 1;

1>	stop, if any, ongoing Random Access procedure;

1>	discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any;

1>	flush Msg3 buffer;

1>	flush MSGA buffer;

1>	cancel, if any, triggered Scheduling Request procedure;

1>	cancel, if any, triggered Buffer Status Reporting procedure;

1>	cancel, if any, triggered Delay Status Reporting procedure;

1>	cancel, if any, triggered Power Headroom Reporting procedure;

1>	cancel, if any, triggered consistent LBT failure;

1>	cancel, if any, triggered Sidelink consistent LBT failure;

1>	cancel, if any, triggered BFR;

1>	cancel, if any, triggered Sidelink Buffer Status Reporting procedure;

1>	cancel, if any, triggered Pre-emptive Buffer Status Reporting procedure;

1>	cancel, if any, triggered Timing Advance Reporting procedure;

1>	cancel, if any, triggered Recommended bit rate query procedure;

1>	cancel, if any, triggered Configured uplink grant confirmation;

1>	cancel, if any, triggered configured sidelink grant confirmation;

1>	clear, if any, configured sidelink grants;

1>	cancel, if any, triggered Desired Guard Symbol query;

1>	cancel, if any, triggered Positioning Measurement Gap Activation/Deactivation Request procedure;

1>	cancel, if any, triggered SDT procedure;

1>	cancel, if any, triggered IAB-MT Recommended Beam Indication query;

1>	cancel, if any, triggered Desired DL TX Power Adjustment query;

1>	cancel, if any, triggered Desired IAB-MT PSD range query;

1>	cancel, if any, triggered Case-6 Timing Request query;

1>	cancel, if any, triggered SL-PRS resource request;

1>	flush the soft buffers for all DL HARQ processes, except for the DL HARQ process being used for MBS broadcast;

1>	for each DL HARQ process, except for the DL HARQ process being used for MBS broadcast, consider the next received transmission for a TB as the very first transmission;

1>	release, if any, Temporary C-RNTI;

1>	clear, if any, Differential Koffset;

1>	if upper layers indicate SCG deactivation and bfd-and-RLM with value true is not configured; or

1>	if the MAC reset is not due to SCG deactivation:

2>	reset all BFI_COUNTERs;

1>	reset all LBT_COUNTERs.

If a Sidelink specific reset of the MAC entity is requested for a PC5-RRC connection by upper layers, the MAC entity shall:

1>	flush the soft buffers for all Sidelink processes for all TB(s) associated to the PC5-RRC connection;

1>	consider all Sidelink processes for all TB(s) associated to the PC5-RRC connection as unoccupied;

1>	cancel, if any, triggered Scheduling Request procedure only associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink Buffer Status Reporting procedure only associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink CSI Reporting procedure associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink DRX Command MAC CE associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink IUC-Request transmission procedure associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink IUC-Information Reporting procedure associated to the PC5-RRC connection;

1>	stop (if running) all timers associated to the PC5-RRC connection;

1>	reset the numConsecutiveDTX associated to the PC5-RRC connection;

1>	initialize SBj for each logical channel associated to the PC5-RRC connection to zero.

## 5.13	Handling of unknown, unforeseen and erroneous protocol data

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI or by the configured downlink assignment, containing a Reserved LCID or eLCID value, or an LCID or eLCID value the MAC Entity does not support, the MAC entity shall at least:

1>	discard the received subPDU and any remaining subPDUs in the MAC PDU.

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI, CS-RNTI or G-RNTI, or by the configured downlink assignment, containing an LCID or eLCID value which is not configured, or an LCID or eLCID value associated with a suspended RB as specified in TS 38.331 [5], the MAC entity shall at least:

1>	discard the received subPDU.

When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value for broadcast or groupcast, or an LCID value which is not configured, the MAC entity shall:

1>	discard the received subPDU.

When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value for unicast, the MAC entity shall:

1>	discard the received subPDU and any remaining subPDUs in the MAC PDU.