3GPP TSG RAN #105	RP-241760

Melbourne, Australia, September 9 - 12, 2024

Agenda Item:	9.3.1.5

Source:	Futurewei

Title:	R19 NES normative work for on-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion and decision

# Introduction

For the Work Item on Enhancements of Network Energy Savings for NR [1], it includes the following objective:

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3]

  - Triggering method by uplink wake-up-signal using an existing signal/channel.

  - Wake-up-signal configuration provisioning to UE

    - Note: No modification of SSB will be discussed under this objective

  - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary.

  - Checkpoint for normative work in RAN#105

In this contribution, our views will be provided regarding the normative work for on-demand SIB1 for idle/inactive mode UEs.

# Discussion

In the last RAN1 meetings, several agreements focused on defining the on-demand SIB1 potential scenarios.  The scenarios consider at least two cells in the network, called Cell A and NES Cell. Here is our understanding of these terms and their relationship:

Cell A

Cell A periodically transmits at least its own legacy SSB and SIBs (including SIB1), and operates according to legacy specifications without the NES feature of turning off its SSB and/or SIBs. Therefore, Cell A can be used for initial access for legacy / new UEs.

Cell A may provide wake-up signal (WUS) configuration or assistance (such as timing) to UEs for accessing other cells (e.g., NES Cell).

NES Cell

NES Cell may transmit SIB1 transmission in response to UL WUS from a UE. It does not transmit at least the legacy periodic SIBs, including SIB1. In other words, NES Cell can turn off its SIBs, for, e.g., NES purposes. NES Cell cannot be used for initial access for legacy UEs.

Relationship between Cell A and NES Cell

Cell A and NES Cell can be collocated and share certain components, and they may be intra-band CA carriers or inter-band CA carriers, or they can be non-collocated as neighbor cells. Generally, the coverage area of the NES Cell may be partially or fully covered by that of Cell A.

They may share certain properties, such as the same timing, same large-scale fading, etc., or not share common properties.

The potential enhancements can be different based on different relationships between Cell A and NES Cell.

Three cases were identified by RAN1.

Agreement

For the further study of on-demand SIB1 for idle/inactive mode UE, RAN1 focuses its studies on the following cases:

- Case 1: Option 1+A+X

- Case 2: Option 1+B+X

- Case 3: Option 2+B+Y

Where the options 1/2/A/B/X/Y are defined below:

- On target cell of UL WUS transmission:

  - Option 1: UE transmits UL WUS to NES Cell

  - Option 2: UE transmits UL WUS to Cell A

- On configuration provision for UL WUS transmission

  - Option A: UE obtains the UL WUS configuration from NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

- On receiving of SIB1

  - Option X: UE receives on-demand SIB1 from NES Cell

  - Option Y: UE receives on-demand SIB1 from Cell A

- In RAN1 #117, companies agreed to give higher priority to the Case 2 and Case 3 in RAN1.

Agreement

For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3

- Case 2 (Option 1+B+X) is feasible from RAN1 perspective.

- Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

In RAN1 #118, companies agreed to support normative work for Case 2.

Agreement

RAN1 recommends specifying on-demand SIB1 only for Case 2 (Option 1+B+X) in Rel-19.

- Note: RAN1 strive to minimize impact to legacy UE.

- Note: RAN1 specification impact to support this feature should be minimized.

A similar conclusion was reached in RAN2 #127 meeting and RAN3 #124 meeting.

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

[Agreement] Focus on case2 in RAN3 first.

Proposal 1: Specify on-demand SIB1 for Case 2 (Option 1+B+X) in Rel-19.

- In the RAN1 #118 meeting, multiple companies supported Case 3 as a second option for on-demand SIB1 scenarios. This case may require minimal specs changes with respect to Case 2. However, companies did not reach an agreement to recommend specification support for this scenario.

Agreement

For on-demand SIB1 in idle/inactive mode, Case 3 (Option 2+B+Y) is feasible from RAN1 perspective for some scenarios. Case 3 (Option 2+B+Y) is lower priority compared to Case 2 from RAN1 perspective.

- RAN1 is inconclusive on whether the required specification support is justified by the observed NES gain for the Case 3

- A similar outcome was also decided in RAN2 #127.

Agreements on OD-SIB1

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

- Nevertheless, in the study phase, multiple companies show that Case 3 can provide similar or additional NES benefits compared to Case 2, primarily due to the turning-off of on-demand SIB1 transmission on the NES cell so that the NES cell may save more energy at least in some scenarios (e.g., no or low load). RAN1 had the following agreements on its potential energy saving gains (the agreements below are abbreviated to focus on only Case 3 results):

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, empty load

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - the NES gain is 30.43% - 45.79% from the following 4 sources

    - R1-2404463, R1-2404859, R1-2405363, R1-2404625

  - One source (R1-2405162) reports the following NES gain with PRACH configuration index = 152

    - 25% NES gain with Case 3 (Options 2+B+Y) assuming the UE can camp on the NES cell and the SIB1 is only transmitted in a single beam corresponding to the UL WUS beam

    - 23% with Case 2 (Options 1+B+X)

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 160ms SIB1 period (Case C), FR1, empty load

- For Cat 1 BS, 4 beams, 0% on-demand SIB1 transmission rate,

  - the NES gain is 1.7% - 6.23% from the following 5 sources

    - R1-2403979, R1-2405341, R1-2405363, R1-2404625, R1-2404758

  - One source (R1-2404758) reports 28.9% NES gain with Case 3 where no PRACH and paging is activated for the NES cell

Agreement

For the evaluation of NES loss on cell A, the following is observed

  - NES loss of 0%~0.98% from the following 15 sources with UL WUS configuration transmitted in legacy SIB on Cell A (with different assumptions on SIB periodicity and cell loading):

    - R1-2404408, R1-2404463, R1-2403942, R1-2403979, R1-2405341, R1-2404224, R1-2404859, R1-2404758, R1-2405162, R1-2404122, R1-2404507, R1-2404033, R1-2405363, R1-2404625, R1-2404561 (for Case 2)

  - One source (R1-2405106) reports for the case with UL WUS configuration transmitted in separated SIB on Cell A where the separated SIB is TDMed with legacy SIB and transmitted in a 320ms period using dedicated resource.

    - 0.62%~2.65% NES loss for Case 2 (Options 1+B+X) with empty load

    - 0.73%~3.20% NES loss for Case 3 (Options 2+B+Y) with empty load

From these results, one can observe that Case 3 can provide similar or higher energy saving gains compared to Case 2.

- Observation 1: RAN1 evaluations show that Case 3 can provide similar or higher energy saving gains compared to Case 2.

- During the meeting it was recognized that supporting Case 3 would require relatively small changes to the specs in addition to those required by Case 2; this is because the UE transmission of WUS and reception of the on-demand SIB1 is similar regardless of which cell (Cell A or the NES Cell) is involved. Thus, we prefer that, if the time permits, Case 3 should be considered for normative work.

- Observation 2: There is very little additional specification impact for Case 3 in addition to Case 2.

- Proposal 2: Consider starting the normative work for Case 3 in R19 if time permits.

- Finally, regarding Case 1, RAN1 had the following conclusion on its technical feasibility and agreements on its potential energy saving gains (the agreements below are abbreviated to focus on only Case 1 results):

Conclusion

For on-demand SIB1 in idle/inactive mode, RAN1 was not able to achieve consensus to support Case 1 (Option 1+A+X) in Rel-19, while Case 1 is technically possible from RAN1’s perspective.

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, empty load

- For Cat 1 BS, 8 beams, 0% on-demand SIB1 transmission rate,

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 9.27%~18.94% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~9.66% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 13.25% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, low load

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 4.73%~9.64% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~4.8% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 7% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

Agreement

For the evaluation of achievable NES gain on NES cell with on-demand SIB1 in idle/inactive mode, the following is observed with 20ms SSB period and 20ms SIB1 period (Case A), FR1, medium load

- For Cat 1 BS, 8 beams, 0% < on-demand SIB1 transmission rate <= 30%,

  - One source (R1-2404122) reports the following with FDMed SSB and PDSCH of SIB1

    - 1.94%~4% NES gain with UL WUS configuration transmitted in legacy SIB on Cell A

    - 0%~1.94% NES gain with UL WUS configuration transmitted in separated SIB on Cell A. The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource.

    - 0% NES gain with Case 1 (Options 1+A+X) and WUS configuration transmitted in separate SIB on the NES cell

    - 2.64% NES gain with Case 1 (Options 1+A+X) and pre-configured for fixed WUS configuration

      - NES cell does not transmit the WUS configuration

Agreement

For the evaluation of the energy consumption to transmit WUS configuration on NES Cell:

- One source (R1-2405595) reports that for broadcasting WUS configuration using DCI occupying 2 symbols per SSB beam) on NES cell every 80 ms on CORESET#0 (48 RBs):

  - For BS category 1, empty/low load, the energy consumption is increased by 6.26%~8.51% over SSB transmission only. The energy saving gain with Case 1 is 40.52% / 32.41% for empty/low load over the baseline with 8 beams, SIB-1 period 20ms (baseline), 0% SIB-1 transmission rate (for on demand SIB1).

  - For BS category 2, empty/low load, the energy consumption is increased by 1.55%~2.11% over SSB transmission only. The energy saving gain with Case 1 is 28.25% / 23.03% for empty/low load over the baseline with 8 beams, SIB-1 period 20ms (baseline), 0% SIB-1 transmission rate (for on demand SIB1).

- From these results, one can observe that for Case 1, depending on various parameters of the network configurations and traffic load conditions, it can provide a range of energy saving gains, which may be moderate or even high in some scenarios.

- Observation 3: Depending on the various parameters of the network configurations and traffic load conditions, Case 1 can provide a range of energy saving gains, which may be moderate or even high in some scenarios.

- Furthermore, the application scenarios for Case 2 and Case 3 require the presence of Cell A, or even complete coverage of the NES Cell by Cell A. If only Case 2 and Case 3 are supported, a cell not fully covered by another cell cannot benefit from R19 on-demand SIB1 NES techniques since otherwise it creates SIB1 coverage holes in the network. In other words, if a cell is not fully covered by another cell or other cells, the only way that it may turn off its periodic SIB1 transmissions and rely on on-demand SIB1 is via Case 1. For overall network-wise energy saving benefits and to provide potentially useful features to operators, it is suggested to support Case 1 if time permits.

- Observation 4: Case 1 can provide NES gains when an assistant cell is not available, e.g., the NES Cell is not fully covered by another cell or other cells, in which scenarios Case 2 and Case 3 are not applicable.

- Proposal 3: Consider starting the normative work for Case 1 in R19 if time permits.

# Conclusions

In this contribution, we present our views on on-demand SIB1 for idle/inactive mode UEs

Proposal 1: Specify on-demand SIB1 for Case 2 (Option 1+B+X) in Rel-19.

- Observation 1: RAN1 evaluations show that Case 3 can provide similar or higher energy saving gains compared to Case 2.

- Observation 2: There is very little additional specification impact for Case 3 in addition to Case 2.

- Proposal 2: Consider starting the normative work for Case 3 in R19 if time permits.

Observation 3: Depending on the various parameters of the network configurations and traffic load conditions, Case 1 can provide a range of energy saving gains, which may be moderate or even high in some scenarios.

- Observation 4: Case 1 can provide NES gains when an assistant cell is not available, e.g., the NES Cell is not fully covered by another cell or other cells, in which scenarios Case 2 and Case 3 are not applicable.

Proposal 3: Consider starting the normative work for Case 1 in R19 if time permits.

# References

RP-240170, Enhancements of network energy savings for NR, Ericsson (Moderator).

R1-2405812, Discussion of on-demand SIB1 SCell for idle inactive mode UEs, Futurewei.

Chair Notes RAN1 #117.

Chair Notes RAN1 #118.

R2-2407571, RAN2 Report from session on V2X/SL, R18/19 MOB, and R19 NES.

RP-241182, Status report for WI - Enhancements of network energy savings for NR, Source: RAN1, Ericsson, Apple.