3GPP TSG-RAN Meeting #106	RP-242653

Madrid, Spain, December 9-12, 2024

Agenda item:	9.3.1.5

Source:	Nokia

Title:	On-demand SIB1 for CONNECTED mode UEs

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Discussion

The release 19 work item on network energy saving (RP-242354) includes the following objective:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

As can be seen from the description OD-SIB1 (On demand SIB1) is limited in WGs to IDLE/INACTIVE UEs.

Observation 1: WI only covers IDLE/INACTIVE UEs for OD-SIB1 objective

In RAN2#128 there was discussion regardless also on RRC_CONNECTED mode UE handling – especially concerning radio link failure (RLF) scenarios where from RAN2 perspective UE is in RRC_CONNECTED although it requires to perform cell selection and acquire SIB1 when re-establishing connection.

Observation 2: In order to reestablish connection UE stays technically in RRC_CONNECTED and needs to acquire SIB1

In case UE selects a cell using on demand SIB1 but if UE cannot acquire SIB1 from cells utilizing OD-SIB1 it would delay reestablishment procedure before UE can find a cell providing SIB1 in normal release 15 broadcast manner as it could take multiple cell selections before such a cell is found – leading to increased amount of failed re-establishments.

Observation 3: If UE cannot reestablish connection in OD-SIB1 cell it would increase amount of failed re-stablishments

In our understanding motivation to limit the OD-SIB1 operation in WI discussions was to limit new mechanisms for OD-SIB1 operation in dedicated signaling and/or when UE is actually in RRC_CONNECTED and reachable by network as then NW can by implementation update SIB1 (if needed) via existing RRC ASN.1 procedures, namely dedicated dedicatedSIB1-Delivery.

In RAN2#128 the decision concerning new delivery mechanism was:

Wait one cycle of RAN discussion (i.e. to see whether WID is updated or not to handle RRC connected UEs)

In RAN2 we have agreed to provide information for OD-SIB1 request for serving and neighbour cells in system information, namely SIBX. UE in RRC_CONNECTED in RAN2 discussion has been seen able to obtain such a SIBX as well (by reading SIB or NW providing it with dedicatedSystemInformationDelivery (to the UE with an active BWP with no common search space configured). One reason for not agreeing adding the SIBX information in RRCRelease in RAN2 was that UE will have this from SIBX in RRC_CONNECTED as well.

Observation  4: RAN2 understanding is that UE can acquire SIBX in RRC_CONNECTED (being one reason not to include this information in RRCRelease message)

Now question is that is the understanding that UE can utilize this SIBX in the RRC_CONNECTED when RLF occurs in order to reestablish connection in a cell using OD-SIB1 operation. This seems to be trivial but we want to ensure that the WI does not motivate to not allow such a behaviour in the specification as technically UE is in RRC_CONNECTED according to RAN2 specifications. So in stage-3 in order to allow re-establishing on cell using OD-SIB1 on would need to ensure following:

    - SIBX is acquired in RRC_CONNECTED by UE (by reading SIBX or NW providing it with dedicatedSystemInformationDelivery to the UE with an active BWP with no common search space configured)

    - UE can use SIBX information in case of radio link failure (RLF) to request SIB1 transmission i.e. at RLF UE performs cell selection and if it selects to OD-SIB1 utilizing cell for which SIB1 request information was in the acquired SIBX

Proposal 1: To discuss whether above behaviour is allowed to be written in stage-3 RAN2 specifications by current WID and if not then update the WI to allow it

# 2	Conclusions

In this paper we discussed on demand SIB1 usage in case of radio link failure i.e. in following scenario:

- UE is in RRC_CONNECTED

- Radio link failure occurs

- UE performs cell selection

- Selected cell is using on demand SIB1

In order not to deliberately increase amount of failed RRC re-establishment we want UE to be able to use existing RAN2 framework of using SIBX in the above scenario in order to re-establish connection.

It should be noted that in RAN2 discussion not to include SIBX information in RRCRelease was taken with assumption UE acquires SIBX in RRC_CONNECTED.

We want to ensure above operation is not outscoped and if it does then update the WI accordingly to allow following to be specified in stage-3:

    - SIBX is acquired in RRC_CONNECTED by UE (by reading SIB or NW providing it with dedicatedSystemInformationDelivery to the UE with an active BWP with no common search space configured)

    - UE can use SIBX information in case of radio link failure (RLF) to request SIB1 transmission i.e. at RLF UE performs cell selection and if it selects to OD-SIB1 utilizing cell for which SIB1 request information was in the acquired SIBX

Proposal 1: To discuss whether above behaviour is allowed to be written in stage-3 RAN2 specifications by current WID and if not then update the WI to allow it