3GPP TSG-RAN WG1 Meeting #78 	R1-143489

# Dresden, Germany, August 18 – 22, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.213 | CR | 0117 | rev | 2 | Current version: | 11.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Enhanced Broadcast of System Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia Networks, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | UTRA_SIBenh-Core |  |  |  |  |  |  | Date: |  | 2014-08-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Introduction of Enhanced Broadcast of System Information |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mapping of BCH to S-CCPCH is introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No introduction of Enhanced Broadcast of System Information |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.1.1, 5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | 25.211, 25.212 25.300, 25.301, 25.302, 25.304, 25.306, 25.321, 25.331 25.433 25.101, 25.133 |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[---------------------------------------------------------- Text Start --------------------------------------------------------------------]

--- Unchanged text omitted ---

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

16QAM	16 Quadrature Amplitude Modulation

4PAM	4 Pulse Amplitude Modulation

64QAM	64 Quadrature Amplitude Modulation

8PAM	8 Pulse Amplitude Modulation

AICH	Acquisition Indicator Channel

BCH	Broadcast Channel

CCPCH	Common Control Physical Channel

CLTD	Closed Loop Transmit Diversity

CPICH	Common Pilot Channel

DCH	Dedicated Channel

DPCH	Dedicated Physical Channel

DPCCH	Dedicated Physical Control Channel

DPDCH	Dedicated Physical Data Channel

E-AGCH	E-DCH Absolute Grant Channel

E-DPCCH	E-DCH Dedicated Physical Control Channel

E-DPDCH	E-DCH Dedicated Physical Data Channel

E-HICH	E-DCH Hybrid ARQ Indicator Channel

E-RGCH	E-DCH Relative Grant Channel

E-ROCH	E-DCH Rank and Offset Channel

FDD	Frequency Division Duplex

F-DPCH	Fractional Dedicated Physical Channel

F-TPICH	Fractional Transmitted Precoding Indicator Channel

HS-DPCCH	Dedicated Physical Control Channel (uplink) for HS-DSCH

HS-DPCCH2	Secondary Dedicated Physical Control Channel (uplink) for HS-DSCH, when Secondary_Cell_Enabled is greater than 3

HS-DSCH	High Speed Downlink Shared Channel

HS-PDSCH	High Speed Physical Downlink Shared Channel

HS-SCCH	Shared Control Physical Channel for HS-DSCH

MBSFN	MBMS over a Single Frequency Network

Mcps	Mega Chip Per Second

MICH	MBMS Indication Channel

OVSF	Orthogonal Variable Spreading Factor (codes)

TPI	Transmitted Precoding Indicator

PICH	Page Indication Channel

PRACH	Physical Random Access Channel

PSC	Primary Synchronisation Code

RACH	Random Access Channel

SCH	Synchronisation Channel

S-DPCCH	Secondary Dedicated Physical Control Channel

S-E-DPCCH	Secondary Dedicated Physical Control Channel for E-DCH

S-E-DPDCH	Secondary Dedicated Physical Data Channel for E-DCH

SSC	Secondary Synchronisation Code

SF	Spreading Factor

UE	User Equipment

--- Unchanged text omitted ---

### 5.1.1	Modulation mapper

Table 3A defines which of the IQ mapping specified in subclauses 5.1.1.1 and 5.1.1.2 may be used for the physical channel being processed.

Table 3A: IQ mapping

| Physical channel | IQ mapping |
| --- | --- |
| HS-PDSCH | QPSK, 16QAM or 64QAM |
| S-CCPCH used for MBSFN | QPSK or 16QAM |
| All other channels (except the SCH) | QPSK |

--- Unchanged text omitted ---

### 5.2.2	Scrambling code

A total of 218-1 = 262,143 scrambling codes, numbered 0…262,142 can be generated. However not all the scrambling codes are used. The scrambling codes are divided into 512 sets each of a primary scrambling code and 15 secondary scrambling codes.

The primary scrambling codes consist of scrambling codes n=16*i where i=0…511. The i:th set of secondary scrambling codes consists of scrambling codes 16*i+k, where k=1…15.

There is a one-to-one mapping between each primary scrambling code and 15 secondary scrambling codes in a set such that i:th primary scrambling code corresponds to i:th set of secondary scrambling codes.

Hence, according to the above, scrambling codes k = 0, 1, …, 8191 are used. Each of these codes are associated with a left alternative scrambling code and a right alternative scrambling code, that may be used for compressed frames. The left alternative scrambling code corresponding to scrambling code k is scrambling code number k + 8192, while the right alternative scrambling code corresponding to scrambling code k is scrambling code number k + 16384. The alternative scrambling codes can be used for compressed frames. In this case, the left alternative scrambling code is used if n<SF/2 and the right alternative scrambling code is used if nSF/2, where cch,SF,n is the channelisation code used for non-compressed frames. The usage of alternative scrambling code for compressed frames is signalled by higher layers for each physical channel respectively.

In case F-DPCH is configured in the downlink, the same scrambling code and OVSF code shall be used in F-DPCH compressed frames and normal frames.

In the case that F-TPICH is configured in the downlink, the same scrambling code and OVSF code shall be used in F-TPICH compressed frames and normal frames.

The set of primary scrambling codes is further divided into 64 scrambling code groups, each consisting of 8 primary scrambling codes. The j:th scrambling code group consists of primary scrambling codes 16*8*j+16*k, where j=0..63 and k=0..7.

Each cell is allocated one and only one primary scrambling code. The primary CCPCH, primary CPICH, PICH, MICH, AICH and S-CCPCH carrying PCH or BCH shall always be transmitted using the primary scrambling code. The other downlink physical channels may be transmitted with either the primary scrambling code or a secondary scrambling code from the set associated with the primary scrambling code of the cell.

In MBSFN operations, the S-CCPCH carries FACH only and shall always be transmitted using the primary scrambling code. The same primary CCPCH, primary CPICH, MICH and S-CCPCH may be transmitted from multiple cells using the same primary scrambling code when part of MBSFN operations.

The mixture of primary scrambling code and no more than one secondary scrambling code for one CCTrCH is allowable. In compressed mode during compressed frames, these can be changed to the associated left or right scrambling codes as described above, i.e. in these frames, the total number of different scrambling codes may exceed two.

In the case of CCTrCH of type of HS-DSCH then all the HS-PDSCH channelisation codes and HS-SCCH that a single UE may receive from a single cell shall be under a single scrambling code (either the primary or a secondary scrambling code).

In each cell, the F-DPCH, E-RGCH, E-HICH, E-AGCH, E-ROCH and F-TPICH assigned to a UE shall be configured with same scrambling code as the assigned phase reference (primary or secondary CPICH).

In each cell the UE may be configured simultaneously with at most two scrambling codes.

The scrambling code sequences are constructed by combining two real sequences into a complex sequence. Each of the two real sequences are constructed as the position wise modulo 2 sum of 38400 chip segments of two binary m-sequences generated by means of two generator polynomials of degree 18. The resulting sequences thus constitute segments of a set of Gold sequences. The scrambling codes are repeated for every 10 ms radio frame. Let x and y be the two sequences respectively. The x sequence is constructed using the primitive (over GF(2)) polynomial 1+X7+X18 . The y sequence is constructed using the polynomial 1+X5+X7+ X10+X18 .

The sequence depending on the chosen scrambling code number n is denoted zn, in the sequel. Furthermore, let x(i), y(i) and zn(i) denote the i:th symbol of the sequence x, y, and zn, respectively.

The m-sequences xand y are constructed as:

Initial conditions:

-	x is constructed with x (0)=1, x(1)= x(2)=...= x (16)= x (17)=0.

-	y(0)=y(1)= … =y(16)= y(17)=1.

Recursive definition of subsequent symbols:

-	x(i+18) =x(i+7) + x(i) modulo 2, i=0,…,218-20.

-	y(i+18) = y(i+10)+y(i+7)+y(i+5)+y(i)  modulo 2, i=0,…, 218-20.

The n:th Gold code sequence zn, n=0,1,2,…,218-2, is then defined as:

-	zn(i) = x((i+n) modulo (218 - 1)) + y(i) modulo 2, i=0,…, 218-2.

These binary sequences are converted to real valued sequences Zn by the following transformation:

Finally, the n:th complex scrambling code sequence Sdl,n is defined as:

-	Sdl,n(i) = Zn(i) + j Zn((i+131072) modulo (218-1)), i=0,1,…,38399.

Note that the pattern from phase 0 up to the phase of 38399 is repeated.

Figure 10: Configuration of downlink scrambling code generator

--- Unchanged text omitted ---

[---------------------------------------------------------- Text End ---------------------------------------------------------------------]