3GPP TSG-RAN4 Meeting #74bis 	R4-151284

# Rio de Janeiro, Brazil, 20 – 24 April 2015

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 0647 | rev | - | Current version: | 12.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of parameter P for emission requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11 |  |  |  |  |  |  | Date: |  | 2015-04-24 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | 1) The parameter P is used for the emission requirements both in subclause 6.6.3.2B for Home BS and subclause 6.6.3.2C for Medium Range BS. But it is only defined for Home BS in NOTE 8 at the end of subclause 6.6.3.3. 2) It is not clear whether the parameter P for Home BS is defined per carrier or for all carriers of all transmit antenna connectors. The parameter P was copied from TS 25.104, which state ‘BS maximum output power P’, which in turn is defined as ‘Maximum output power, Pmax, of the base station is the mean power level per carrier measured at the antenna connector in specified reference condition’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Define the parameter P for Medium Range BS as the maximum output power (i.e. the mean power level per carrier measured at the antenna connector). 2) Clarify the parameter P for Home BS as the aggregated maximum output power (i.e. per carrier) of all transmit antenna connectors. Isolated impact analysis: This is an isolated impact change. Would not affect implementations behaving like indicated in the CR, would affect implementations supporting the corrected functionality otherwise. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguities remain and different interpretations may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3.2C, 6.6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.141 CR 0721 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

#### 6.6.3.2C	Minimum requirements for Medium Range BS (Category A and B)

For Medium Range BS, emissions shall not exceed the maximum levels specified in Tables 6.6.3.2C-1 to 6.6.3.2C-6.

Table 6.6.3.2C-1: Medium Range BS operating band unwanted emission limits for 1.4 MHz channel bandwidth, 31 < Pmax,c   38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | Pmax,c-55dB | 100 kHz |
| 2.8 MHz f fmax | 2.85 MHz f_offset < f_offsetmax | -25dBm | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -25dBm/100kHz. NOTE 2: For BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the minimum requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the inter RF bandwidth gap. |  |  |  |

Table 6.6.3.2C-2: Medium Range BS operating band unwanted emission limits for 1.4 MHz channel bandwidth, Pmax,c  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -24 dBm | 100 kHz |
| 2.8 MHz f fmax | 2.85 MHz f_offset < f_offsetmax | -25dBm | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -25dBm/100kHz. NOTE 2: For BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the minimum requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the inter RF bandwidth gap. |  |  |  |

Table 6.6.3.2C-3: Medium Range BS operating band unwanted emission limits for 3 MHz channel bandwidth, 31 < Pmax,c   38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | Pmax,c-59dB | 100 kHz |
| 6 MHz f fmax | 6.05 MHz f_offset < f_offsetmax | Min(Pmax,c-59dB, -25dBm) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be Min(Pmax,c-59dB, -25dBm)/100kHz. NOTE 2: For BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the minimum requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the inter RF bandwidth gap. |  |  |  |

Table 6.6.3.2C-4: Medium Range BS operating band unwanted emission limits for 3 MHz channel bandwidth, Pmax,c  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 0 MHz f < 3 MHz | 0.05 MHz f_offset < 3.05 MHz |  | 100 kHz |
| 3 MHz f < 6 MHz | 3.05 MHz f_offset < 6.05 MHz | -28 dBm | 100 kHz |
| 6 MHz f fmax | 6.05 MHz f_offset < f_offsetmax | -28 dBm | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -28dBm/100kHz. NOTE 2: For BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the minimum requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the inter RF bandwidth gap. |  |  |  |

Table 6.6.3.2C-5: Medium Range BS operating band unwanted emission limits for 5, 10, 15 and 20 MHz channel bandwidth, 31< Pmax,c   38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | Pmax,c-60dB | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | Min(Pmax,c-60dB, -25dBm) (Note 6) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be Min(Pmax,c-60dB, -25dBm)/100kHz. NOTE 2: For BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the minimum requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the inter RF bandwidth gap. |  |  |  |

Table 6.6.3.2C-6: Medium Range BS operating band unwanted emission limits for 5, 10, 15 and 20 MHz channel bandwidth, Pmax,c  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -29 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -29 dBm (Note 6) | 100 kHz |
| NOTE 1: For a BS supporting non-contiguous spectrum operation within any operating band the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -29dBm/100kHz. NOTE 2: For BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the minimum requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks on each side of the inter RF bandwidth gap. |  |  |  |

#### 6.6.3.3	Additional requirements

These requirements may be applied for the protection of other systems operating inside or near each supported E-UTRA BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the E-UTRA BS, or they may be set by local or regional regulation as a mandatory requirement for an E-UTRA operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in subclause 4.3.

In certain regions the following requirement may apply. For E-UTRA BS operating in Bands 5, 26, 27 or 28, emissions shall not exceed the maximum levels specified in Tables 6.6.3.3-1.

Table 6.6.3.3-1: Additional operating band unwanted emission limits for E-UTRA bands <1GHz

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 0 MHz f < 1 MHz | 0.005 MHz f_offset < 0.995 MHz | -14 dBm | 10 kHz |
| 3 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -13 dBm | 30 kHz |
| 5 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -15 dBm | 30 kHz |
| 10 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13 dBm | 100 kHz |
| 15 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13 dBm | 100 kHz |
| 20 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13 dBm | 100 kHz |
| All | 1 MHz f < fmax | 1.05 MHz f_offset < f_offsetmax | -13 dBm | 100 kHz |

In certain regions the following requirement may apply. For E-UTRA BS operating in Bands 2, 4, 10, 23, 25, 30, 35, 36, 41, emissions shall not exceed the maximum levels specified in Table 6.6.3.3-2.

Table 6.6.3.3-2: Additional operating band unwanted emission limits for E-UTRA bands>1GHz

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 0 MHz f < 1 MHz | 0.005 MHz f_offset < 0.995 MHz | -14 dBm | 10 kHz |
| 3 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -13 dBm | 30 kHz |
| 5 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -15 dBm | 30 kHz |
| 10 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13 dBm | 100 kHz |
| 15 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -15 dBm | 100 kHz |
| 20 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -16 dBm | 100 kHz |
| All | 1 MHz f < fmax | 1.5 MHz f_offset < f_offsetmax | -13 dBm | 1 MHz |

In certain regions the following requirement may apply. For E-UTRA BS operating in Bands 12, 13, 14, 17, 29 emissions shall not exceed the maximum levels specified in Table 6.6.3.3-3.

Table 6.6.3.3-3: Additional operating band unwanted emission limits for E-UTRA (bands 12, 13, 14, 17 and 29)

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- | --- |
| All | 0 MHz f < 100 kHz | 0.015 MHz f_offset < 0.085 MHz | -13 dBm | 30 kHz |
| All | 100 kHz f < fmax | 150 kHz f_offset < f_offsetmax | -13 dBm | 100 kHz |

In certain regions, the following requirements may apply to an E-UTRA TDD BS operating in the same geographic area and in the same operating band as another E-UTRA TDD system without synchronisation. For this case the emissions shall not exceed -52 dBm/MHz in each supported downlink operating band except in:

- The frequency range from 10 MHz below the lower channel edge to the frequency 10 MHz above the upper channel edge of each supported band.

In certain regions the following requirement may apply for protection of DTT. For E-UTRA BS operating in Band 20, the level of emissions in the band 470-790 MHz, measured in an 8MHz filter bandwidth on centre frequencies Ffilter according to Table 6.6.3.3-4, shall not exceed the  maximum emission level PEM,N declared by the manufacturer.  This requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain.

Table 6.6.3.3-4: Declared emissions levels for protection of DTT

| Filter centre frequency, Ffilter | Measurement bandwidth | Declared emission level [dBm] |
| --- | --- | --- |
| Ffilter = 8*N + 306 (MHz); 21 ≤ N ≤ 60 | 8 MHz | PEM,N |

Note:	The regional requirement is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in Annex G.

In certain regions the following requirement may apply for the protection of systems operating in frequency bands adjacent to band 1 as defined in clause 5.5, in geographic areas in which both an adjacent band service E-UTRA are deployed.

The power of any spurious emission shall not exceed:

Table 6.6.3.3-5: Emissions limits for protection of adjacent band services

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth |
| --- | --- | --- | --- |
| 1 | 2100-2105 MHz | -30 + 3.4 (f - 2100 MHz) dBm | 1 MHz |
|  | 2175-2180 MHz | -30 + 3.4 (2180 MHz - f) dBm | 1 MHz |

In regions where FCC regulation applies, requirements for protection of GPS according to FCC Order DA 10-534 applies for operation in Band 24. The following normative requirement covers the base station, to be used together with other information about the site installation to verify compliance with the requirement in FCC Order DA 10-534. The requirement applies to BS operating in Band 24 to ensure that appropriate interference protection is provided to the 1559 – 1610 MHz band. This requirement applies to the frequency range 1559-1610 MHz, even though part of this range falls within the spurious domain.

The level of emissions in the 1559 – 1610 MHz band, measured in measurement bandwidth according to Table 6.6.3.3-6 shall not exceed the maximum emission levels PE_1MHz and PE_1kHz declared by the manufacturer.

Table 6.6.3.3-6: Declared emissions levels for protection of the 1559-1610 MHz band

| Operating Band | Frequency range | Declared emission level [dBW] (Measurement bandwidth = 1 MHz) | Declared emission level [dBW] of discrete emissions of less than 700 Hz bandwidth (Measurement bandwidth = 1 kHz) |
| --- | --- | --- | --- |
| 24 | 1559 - 1610 MHz | PE_1MHz | PE_1kHz |

Note:	The regional requirement in FCC Order DA 10-534 is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The EIRP level is calculated using: PEIRP = PE + Gant where PE denotes the BS unwanted emission level at the antenna connector, Gant equals the BS antenna gain minus feeder loss. The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement.

The following requirement may apply to E-UTRA BS operating in Band 41 in certain regions. Emissions shall not exceed the maximum levels specified in Table 6.6.3.3-7.

Table 6.6.3.3-7: Additional operating band unwanted emission limits for Band 41

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- | --- |
| 10 MHz | 10 MHz f < 20 MHz | 10.5 MHz f_offset < 19.5 MHz | -22 dBm | 1 MHz |
| 20 MHz | 20 MHz f < 40 MHz | 20.5 MHz f_offset < 39.5 MHz | -22 dBm | 1 MHz |
| NOTE: This requirement applies for carriers allocated within 2545-2575MHz or 2595-2645MHz. |  |  |  |  |

In certain regions, the following requirements may apply to E-UTRA BS operating in Band 32 within 1452-1492 MHz. The level of operating band unwanted emissions, measured on centre frequencies f_offset with filter bandwidth, according to Table 6.6.3.3-8, shall neither exceed the maximum emission level PEM,B32,a ,  PEM,B32,b nor PEM,B32,c declared by the manufacturer.

Table 6.6.3.3-8: Declared operating band 32 unwanted emission within 1452-1492 MHz

| Frequency offset of measurement filter centre frequency, f_offset | Declared emission level [dBm] | Measurement bandwidth |
| --- | --- | --- |
| 2.5 MHz | PEM,B32,a | 5 MHz |
| 7.5 MHz | PEM,B32,b | 5 MHz |
| 12.5 MHz ≤ f_offset ≤ f_offsetmax,B32 | PEM,B32,c | 5 MHz |
| NOTE: f_offsetmax,B32 denotes the frequency difference between the lower channel edge and 1454.5 MHz, and the frequency difference between the upper channel edge and 1489.5 MHz for the set channel position. |  |  |

NOTE:	The regional requirement, included in [16], is defined in terms of EIRP per antenna, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in Annex H.

In certain regions, the following requirement may apply to E-UTRA BS operating in Band 32 within 1452-1492 MHz for the protection of services in spectrum adjacent to the frequency range 1452-1492 MHz. The level of emissions, measured on centre frequencies Ffilter with filter bandwidth according to Table 6.6.3.3-9, shall neither exceed the maximum emission level PEM,B32,d nor PEM,B32,e declared by the manufacturer. This requirement applies in the frequency range 1429-1518MHz even though part of the range falls in the spurious domain.

Table 6.6.3.3-9: Operating band 32 declared emission outside 1452-1492 MHz

| Filter centre frequency, Ffilter | Declared emission level [dBm] | Measurement bandwidth |
| --- | --- | --- |
| 1429.5 MHz ≤ Ffilter ≤ 1448.5 MHz | PEM,B32,d | 1 MHz |
| Ffilter = 1450.5 MHz | PEM,B32,e | 3 MHz |
| Ffilter = 1493.5 MHz | PEM,B32,e | 3 MHz |
| 1495.5 MHz ≤ Ffilter ≤ 1517.5 MHz | PEM,B32,d | 1 MHz |

NOTE:	The regional requirement, included in [16], is defined in terms of EIRP, which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement. The assessment of the EIRP level is described in Annex H.

The following notes are common to all subclauses in 6.6.3:

NOTE 3: 	Local or regional regulations may specify another excluded frequency range, which may include frequencies where synchronised E-UTRA TDD systems operate.

NOTE 4:	E-UTRA TDD base stations that are synchronized can transmit without these additional co-existence requirements.

NOTE 5:	As a general rule for the requirements in subclause 6.6.3, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 6:	This frequency range ensures that the range of values of f_offset is continuous.

NOTE 7:	The requirement is not applicable when fmax < 10 MHz.

NOTE 8:	For Home BS, the parameter P is defined as the aggregated maximum output power of all transmit antenna connectors of Home BS.

<End of change>