3GPP TSG RAN Meeting #69                                                                         RP-151544

Phoenix, USA, June 14 – 16, 2015

Title:	Channel modeling for higher frequency bands

Source:	NTT DOCOMO, CMCC, Ericsson, Huawei, HiSillicon, Intel, KT Corporation, Nokia, Qualcomm and Samsung

Agenda Item:	14.1.1

Document for:	Discussion and decision

- Introduction

Extensive research is ongoing worldwide, with the goal to define the next generation “beyond LTE” system. It is envisioned that this next generation standard, typically referred to as 5G, will enable operation at frequency bands as high as 100GHz and will allow the support of a diverse set of services and in a number of different environments and scenarios.

The 3GPP Spatial Channel Model (SCM) [1] and the ITU based model [2,3] have been instrumental in the development and evaluation of technologies for LTE. They provide modelling support for a number of important channel attributes and for operating a number of important technologies including MIMO. The recently developed 3D-UMi and 3D-UMa channel models have provided additional flexibility to include the elevation dimension. This is because there is a new requirement that the two dimensional arrange antenna system will be deployed. These channel models, however, were designed and evaluated for operation at frequencies only as high as 6GHz. Consequently, it is important for 3GPP to develop a new channel model that will be validated for operation at frequencies up to, e.g., 100GHz, and will allow accurate performance evaluation of possible future technical specifications at higher frequency bands and over a representative set of possible environments and scenarios of interest.

This contribution highlights the efforts that are necessary in order to establish a new channel model that can be utilized for 5G evaluation for 3GPP evolution. This contribution also proposes to start channel modeling study in 3GPP. As part of these efforts, 3GPP needs to identify important scenarios and channel modelling requirements for 5G. The contribution also encourages member companies to conduct the measurement campaigns that are needed for developing a channel model that would meet these requirements. This contribution is also supported by Aalto University, Beijing University of Posts and Telecommunications, New York University (NYU WIRELESS), Univ. of Bristol and University of Southern California.

- Current channel modelling activities for 5G

In its effort to develop a new channel model for 5G, it is important for 3GPP to leverage the findings of all the efforts that are conducted outside 3GPP. There are many existing and ongoing campaign efforts worldwide targeting 5G channel measurements and modelling. They include METIS2020 [4], COST2100/COST IC1004 [5,6], ETSI mmWave SIG, 5G mmWave Channel Model Alliance [7], MiWEBA [8], mmMagic [9], and NYU WIRELESS [10,11]. METIS2020, for instance, has focused on 5G technologies and has contributed extensive studies in terms of channel modelling. Their target requirements include a wide range of frequency bands (up to 86GHz), very large bandwidths (hundreds of MHz), fully three dimensional and accurate polarization modelling, spherical wave modelling, and high spatial resolution. The METIS channel models consist of a map-based model, stochastic model, and a hybrid model which can meet requirement of flexibility and scalability. The COST2100 channel model is a geometry-based stochastic channel model (GSCM) that can reproduce the stochastic properties of MIMO channels over time, frequency, and space. On the other hand, the 5G mmWave Channel Model Alliance is newly established and will establish guidelines for measurement calibration and methodology, modeling methodology, as well as parameterization in various environments and a database for channel measurement campaigns. NYU WIRELESS has conducted and openly published extensive urban propagation measurements at 28, 38 and 73 GHz for both outdoor and indoor channels, and has created large-scale and small-scale channel models using physically-based properties and the concept of spatial lobes to model multiple multipath time clusters that are seen to arrive in particular directions [12].

Additionally, we are aware that many parties have started and continue to conduct measurement campaigns, with the goal to collect various measurement data in various scenario and frequency bands and explore propagation characteristics for higher frequency. An overview of some of the recent channel measurements and ray tracing that have already been conducted and planned to be conducted is included in an accompanying addendum. It includes measurement for 0.8-86 GHz and ray tracing activities for the scenarios described in the annex. These measurements have as their goal the characterization of pertinent channel model parameters in the typical scenarios of interest, including pathloss, LOS-NLOS transition, O/I penetration loss, geometry-induced blockage, geometry-induced shadow fading, temporal shadow fading, and additional losses (atmosphere losses, small objects, foliage, etc.) and angular and delay characteristics.

- Channel modelling approaches in 3GPP

The new 5G channel model should provide realistic and high accuracy radio propagation models with low computational complexity in terms of future simulation campaigns. Since 5G channel modelling has been studied in many organizations, universities, and industrial alliances, it is possible to utilize these results to meet our 3GPP requirements. In addition, the new channel model should preferably be based on the existing 3GPP 3D channel model [13] but with extensions to cater for additional 5G modeling requirements and scenarios.

As a starting point, our proposed requirements and scenarios are described in the annex.

- Summary

In this contribution, we have advocated the need for a new channel model that will support 5G activities across frequency bands up to 100GHz and their evaluation in 3GPP. In support of our proposal, we have provided a set of preliminary 5G scenarios and channel modelling requirements in the annex.

We encourage all 3GPP members to conduct measurement campaigns at frequency bands, environment and scenarios that would enable the development of a new model which would meet the 3GPP 5G channel modeling requirements.

# References

[1] 3GPP TR 25.996 (V.12.0.0), “Spatial channel model for Multiple Input Multiple Output (MIMO) simulations,” Sep. 2014.

[2] 3GPP TR 36.814 (V9.0.0), “Further advancements for E-UTRA physical layer aspects ,” Mar. 2010.

[3] ITU-R Rep M.2135-1, ”Guidelines for evaluation of radio interface technologies for IMT-Advanced.”

[4] METIS202, Deliverable D1.4 “METIS Channel Model,” Feb. 2015. https://www.metis2020.com/wp-content/uploads/deliverables/METIS_D1.4_v1.0.pdf

[5] http://www.cost2100.org/

[6] http://www.ic1004.org/

[7] http://www.nist.gov/ctl/wireless-networks/5gmillimeterwavechannelmodel.cfm

[8] MiWEBA, Deliverable D5.1 “Channel modeling and characterization,” June, 2014.

http://www.miweba.eu/wp-content/uploads/2014/07/MiWEBA_D5.1_v1.011.pdf

[9] https://5g-ppp.eu/mmmagic/

[10] M. K. Samimi, et. al, “Probabilistic Omnidirectional Path Loss Models for

Millimeter-Wave Outdoor Communications,” IEEE Trans. Wireless Letters, August 2015, Vol. 4, No. 4, pp. 357-360.  http://ieeexplore.ieee.org/xpl/articleDetails.jsp?arnumber=7070688&sortType%3Dasc_p_Sequence%26filter%3DAND(p_Publication_Number%3A5962382)%26pageNumber%3D2%26rowsPerPage%3D50

[11]  G. MacCartney, Jr., T. S. Rappaport, M.K. Samimi, S. Sun, “Millimeter-Wave Omnidirectional Path Loss Data for Small Cell 5G Channel Modeling,”  IEEE Access Journal,  Volume PP, Issue 99, pp. 1-7, DOI: 10.1109/ACCESS.2015.2465848. http://ieeexplore.ieee.org/xpl/articleDetails.jsp?arnumber=7181638&filter%3DAND(p_IS_Number%3A6514899)

[12] T. S. Rappaport, G. R. MacCartney, M.K. Samimi, S. Sun, “Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design (invited),”  IEEE Transactions on Communications, Vol. 63, Issue 9, pp. 3029 - 3056, DOI: 10.1109/TCOMM.2015.2434384, http://ieeexplore.ieee.org/xpl/articleDetails.jsp?reload=true&arnumber=7109864

[13] 3GPP, TR 36.873 (V12.2.0), “Study on 3D channel model for LTE,” July 2015.

# Annex

Our proposed requirements and scenarios can serve as a starting point for discussion and towards defining 3GPP’s 5G radio channel and propagation modelling requirements:

- Deployment scenarios

  - The new channel model should be developed for a small set of scenarios representing typical 5G practical usage scenarios. Potential scenarios may include the following:

    - Urban Micro (UMi) Street Canyon with O2O (outdoor to outdoor) and O2I (outdoor to indoor)

    - Urban Micro (UMi) Open Square with O2O and O2I

    - Indoor – Office

    - Indoor – Shopping Mall

    - Urban Macro with O2O and O2I

- Wide frequency range up to 100GHz

  - The joint propagation characteristics over different frequency bands can be addressed in multi-band operation, e.g., lowband and highband carrier aggregation evaluations.

- Large bandwidth (up to 2GHz)

- Large arrays antenna configuration

  - Large antenna arrays with very high directivity with angular resolution down to 1 degree.

- Mobility (up to T.B.D km/h)

  - The channel model structure should be prepared for small-scale mobility of both ends of the link in order to support scenarios such as D2D or V2V.

- Spatial/Temporal/Frequency consistency

  - The model should provide spatial/temporal/frequency consistencies, characterized via e.g. cluster spatial consistence, inter-site correlation, correlation among frequency bands. This model should ensure that the channel states such as LOS/NLOS or outdoor/indoor locations and the second order statistics of the channel as well as the channel realizations change smoothly as a function of time, position, or frequency changes. The spatial/temporal/frequency consistencies should be supported for simulations where the channel consistency impacts the results (e.g. massive MIMO, mobility and beam tracking, etc.) but could possibly be optional for simpler studies.

- Computational complexity

  - The model should be suitable for implementation in single-link simulation tools and in multi-cell, multi-link radio network simulation tools. Computational complexity and memory requirements should not be excessive. The 3GPP 3D channel model [13] is seen as the model for an acceptable level of complexity.