3GPP TSG-RAN Meeting #72          													          RP-160983

Busan, Korea, June 13 - 16, 2016

Source: 	Huawei, HiSilicon

Title: 	Text proposal to TR 38.913 to add a KPI for signalling overhead

Agenda Item:	9.2.2

Document for:	Approval

Introduction

At RAN#71 meeting in March 2016, requirements on diverse services including enhanced mobile broadband (eMBB), massive machine type communications (mMTC) and ultra-reliable and low latency communications (URLLC) were discussed, and several key performance indicators (KPIs) were given. Several discussions on requirement of signalling overhead were also provided (see e.g., [1][2]). On the other hand, at RAN1#84bis meeting in April 2016, it was agreed that for multiple access study for new radio (NR), signalling overhead would be one of the performance metrics for its study in eMBB and mMTC scenarios (see [3][4]).

Based on the above observations, it would be good that TR38.913 provides the definitions and requirements on signalling overhead, to guide future study for NR in TSG RAN WGs. This document proposes KPI for signalling overhead to be captured in TR38.913.

Signalling overhead

In 5G timeframe, the small packet services will be increasingly frequent in, e.g., mMTC scenarios, as well as eMBB and URLLC scenarios. In this case, the signalling overhead of small packets would be a great concern due to the fact that large overhead would occupy a large amount of radio resources when many small-packet users concurrently request data transfer. This would especially be the case for social network applications that send heartbeat periodically or TCP ACK for downlink applications like video etc. For eMBB scenarios, signalling overhead would also decrease the spectrum efficiency due to considerable portion of radio resource occupied by signalling messages. However, in the evaluation of spectrum efficiency, the signalling overhead is less considered. If signalling overhead is not controlled, there is a risk that the evaluation results would be biased compared to the reality.

Therefore, it would be important to define signalling overhead as one of the technical performance requirements. It is proposed to define the signalling overhead as follows.

- Signalling overhead is defined as the occupied radio resource that is required by the signalling divided by the total occupied radio resource that is used to complete a transmission of a packet. The signalling includes following 2 types:

- Necessary messages exchanged in DL and UL directions during a signalling mechanism, according to the protocol design, including L1 signalling (e.g. PDCCH, PRACH preamble), L2 signalling (e.g. BSR) and L3 signalling (e.g. RRC messages together with layer 2 protocol header)

- L2 protocol header for the data packet, including PDCP/RLC/MAC protocol header

Conclusion

This contribution discusses the consideration on signalling overhead. The text proposals based on these discussions are included in Annex of this contribution. It is proposed to incorporate them into the TR of scenarios and requirement for next generation access technologies.

Reference

[1] RP-160323, “Text proposal to TR 38.913 to add a KPI for signalling overhead”, Huawei, HiSilicon, RAN#71, March 2016.

[2] RP-160153, “pCR on Section “Security and Privacy related requirement relevant for Radio Access” of TR38.913 – Requirements on radio signaling messages”, Telecom Italia, RAN#71, March 2016.

[3] R1-163961, “Final Report of 3GPP TSG RAN WG1 #84bis v1.0.0”, MCC Support, RAN1#85, May 2016.

[4] R1-163560, “Way forward on assumptions for multiple access evaluation”, Huawei, HiSilicon, RAN1#84bis, April 2016.

Annex

Text proposal to TR38.913

-------------Start of text proposal-----------------

- 7.20		Signalling overhead

Signalling overhead is defined as the total occupied radio resource that is required by the signalling messages divided by the total occupied radio resource that is used to complete a transmission of a data session. The signalling message includes:

- Necessary messages exchanged in DL and UL directions during a signalling mechanism, according to the protocol design, including L1, L2 and L3 signalling

For small packet applications, the signalling overhead should be minimized to improve the transmission efficiency as well as to reduce the total occupied radio resource for completing the data session transmission.

-------------End of text proposal-----------------