3GPP TSG-RAN WG1 Meeting #84bis 	R1-163434

# Busan, Korea 11th - 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 0624 | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections SRS dropping in CA in 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13, LTE-Phys |  |  |  |  |  |  | Date: |  | 2016-04-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Currently, when the UE is configured with CA under a single TAG, SRS on a CC is dropped when there is overlapped PUCCH or PUSCH transmission on a different CCAs summarized in R1-160866, when multiple TAGs are not configured for a UE (a single TAG), the SRS dropping behavior is rather unclear and inconsistent: - The last SRS symbol in a regular UL subframe with ECP may collide PUCCH cell with NCP, but SRS is NOT dropped. - The last SRS symbol in UpPTS with ECP may collide PUCCH cell with NCP, SRS is dropped - It may also depend on how the specification is interpreted w.r.t. the wording “in the same symbol”. If the wording is strictly interpreted, SRS is NOT dropped - The last SRS symbol in regular UL subframe or in UpPTS with ECP may collide PUSCH cell with NCP, and SRS is always dropped. - It may also depend on how the specification is interpreted w.r.t the wording “in the same symbol”. If the wording is strictly interpreted, SRS is NOT dropped. - The 2nd last SRS symbol in UpPTS is always dropped when FDD cell has PUCCH or PUSCH due to overlapping, but NOT dropped otherwise (unless the UE is power limited). - Note also that the 6-symbol UpPTS configuration is introduced in Rel-13 for more SRS transmission opportunities. Based on the same rule, SRS not in the last symbol is dropped as long as FDD is the PUCCH cell, but not dropped otherwise (unless the UE is power limited) It was concluded in RAN1#84 that: - Regarding the SRS dropping under CA/DC with CCs of different CPs when a single TAG is configured as listed in R1-160866: - For UEs of Rel-12 or earlier, no clarification is done - For UEs of Rel-13 and onwards, clarify one UE behavior, details to be discussed in RAN1#84bis Based on contribution in R1-163001, further discussion was carried out the RAN1. It was concluded that the proposals in R1-163001 are agreed in principle for Rel-13, i.e., not to drop SRS unnecessarily when the UE is configured with a single TAG, and CA of different CPs or additional SRS transmission opportunities introduced in Rel-13. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that if the UE is configured with a single TAG, but is configured with CA of different CPs, or is configured with additional SRS transmission opportunities via parameter srs-UpPtsAdd, SRS is not unnecessarily dropped |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behaviour, and unintended/unnecessary SRS dropping |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 8.2	UE sounding procedure

If the UE is configured with a PUCCH-SCell, the UE shall apply the procedures described in this clause for both primary PUCCH group and secondary PUCCH group unless stated otherwise

- When the procedures are applied for the primary PUCCH group, the terms ‘secondary cell’, ‘secondary cells’, ‘serving cell’, and ‘serving cells’ in this clause refer to secondary cell, secondary cells, serving cell or serving cells belonging to the primary PUCCH group respectively unless stated otherwise.

- When the procedures are applied for secondary PUCCH group, the terms ‘secondary cell’, ‘secondary cells’, ‘serving cell’ and ‘serving cells’ in this clause refer to secondary cell, secondary cells (not including the PUCCH-SCell), serving cell, serving cells belonging to the secondary PUCCH group respectively unless stated otherwise. The term ‘primary cell’ in this clause refers to the PUCCH-SCell of the secondary PUCCH group.

A UE shall transmit Sounding Reference Symbol (SRS) on per serving cell SRS resources based on two trigger types:

-	trigger type 0: higher layer signalling

-	trigger type 1: DCI formats 0/4/1A/6-0A/6-1A for FDD and TDD and DCI formats 2B/2C/2D for TDD.

In case both trigger type 0 and trigger type 1 SRS transmissions would occur in the same subframe in the same serving cell, the UE shall only transmit the trigger type 1 SRS transmission.

A UE may be configured with SRS parameters for trigger type 0 and trigger type 1 on each serving cell. A BL/CE UE configured with CEModeB is not expected to be configured with SRS parameters for trigger type 0 and trigger type 1. The following SRS parameters are serving cell specific and semi-statically configurable by higher layers for trigger type 0 and for trigger type 1.

-	Number of combs  as defined in subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1, if configured

-	srs-UpPtsAdd: two or four additional SC-FDMA symbols in UpPTS as defined in [11] for trigger type 0 and trigger type 1, if configured

-	Transmission comb , as defined in subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1

-	Starting physical resource block assignment , as defined in subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1

-	duration: single or indefinite (until disabled), as defined in [11] for trigger type 0

-	srs-ConfigIndex ISRS for SRS periodicity  and SRS subframe offset , as defined in Table 8.2-1 and Table 8.2-2 for trigger type 0 and SRS periodicity  and SRS subframe offset , as defined in Table 8.2-4 and Table 8.2-5 trigger type 1

-	SRS bandwidth , as defined in subclause 5.5.3.2 of [3] for trigger type 0 and each configuration of trigger type 1

-	Frequency hopping bandwidth, , as defined in subclause 5.5.3.2 of [3] for trigger type 0

-	Cyclic shift , as defined in subclause 5.5.3.1 of [3] for trigger type 0 and each configuration of trigger type 1

-	Number of antenna ports  for trigger type 0 and each configuration of trigger type 1

For trigger type 0, if SoundingRS-UL-ConfigDedicatedUpPTsExt is configured, the SRS parameters in SoundingRS-UL-ConfigDedicatedUpPTsExt shall be used; otherwise, SoundingRS-UL-ConfigDedicated shall be used. For trigger type 1, if SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt is configured, the SRS parameters in SoundingRS-UL-ConfigDedicatedAperiodic shall be used; otherwise, SoundingRS-UL-ConfigDedicated shall be used.

For trigger type 1 and DCI format 4 three sets of SRS parameters, srs-ConfigApDCI-Format4, are configured by higher layer signalling. The 2-bit SRS request field [4] in DCI format 4 indicates the SRS parameter set given in Table 8.1-1. For trigger type 1 and DCI format 0/6-0A, a single set of SRS parameters, srs-ConfigApDCI-Format0, is configured by higher layer signalling. For trigger type 1 and DCI formats 1A/2B/2C/2D/6-1A, a single common set of SRS parameters, srs-ConfigApDCI-Format1a2b2c, is configured by higher layer signalling. The SRS request field is 1 bit [4] for DCI formats 0/1A/2B/2C/2D/6-0A/6-1A, with a type 1 SRS triggered if the value of the SRS request field is set to '1'. 
A 1-bit SRS request field shall be included in DCI formats 0/1A/6-0A/6-1A for frame structure type 1 and 0/1A/2B/2C/2D/6-0A/6-1A for frame structure type 2 if the UE is configured with SRS parameters for DCI formats 0/1A/2B/2C/2D/6-0A/6-1A by higher-layer signalling.

Table 8.1-1: SRS request value for trigger type 1 in DCI format 4

| Value of SRS request field | Description |
| --- | --- |
| '00' | No type 1 SRS trigger |
| '01' | The 1st SRS parameter set configured by higher layers |
| '10' | The 2nd SRS parameter set configured by higher layers |
| '11' | The 3rd SRS parameter set configured by higher layers |

The serving cell specific SRS transmission bandwidths  are configured by higher layers. The allowable values are given in subclause 5.5.3.2 of [3].

The serving cell specific SRS transmission sub-frames are configured by higher layers. The allowable values are given in subclause 5.5.3.3 of [3].

For a TDD serving cell, SRS transmissions can occur in UpPTS and uplink subframes of the UL/DL configuration indicated by the higher layer parameter subframeAssignment for the serving cell.

When closed-loop UE transmit antenna selection is enabled for a given serving cell for a UE that supports transmit antenna selection, the index , of the UE antenna that transmits the SRS at time nSRS is given by

, for both partial and full sounding bandwidth, and when frequency hopping is disabled (i.e.,  ),

,

when frequency hopping is enabled (i.e., ),

where values BSRS, bhop, Nb, and nSRS are given in subclause 5.5.3.2 of [3], and (where  regardless of the  value), except when a single SRS transmission is configured for the UE. If a UE is configured with more than one serving cell, the UE is not expected to transmit SRS on different antenna ports simultaneously.

A UE may be configured to transmit SRS on  antenna ports of a serving cell where may be configured by higher layer signalling. For PUSCH transmission mode 1 and for PUSCH transmission mode 2  with two antenna ports configured for PUSCH and  with 4 antenna ports configured for PUSCH. A UE configured for SRS transmission on multiple antenna ports of a serving cell shall transmit SRS for all the configured transmit antenna ports within one SC-FDMA symbol of the same subframe of the serving cell.  
The SRS transmission bandwidth and starting physical resource block assignment are the same for all the configured antenna ports of a given serving cell. The UE does not support a value of  set to ‘4’, if the UE is configured for SRS transmission on 4 antenna ports of a serving cell.

If a UE is not configured with multiple TAGs and the UE not is configured with the parameter srs-UpPtsAdd for trigger type 1, or if a UE is not configured with multiple TAGs and the UE is not configured with more than one serving cell of different CPs, the UE shall not transmit SRS in a symbol whenever SRS and PUSCH transmissions happen to overlap in the same symbol.

For TDD serving cell, and UE not configured with additional SC-FDMA symbols in UpPTS, when one SC-FDMA symbol exists in UpPTS of the given serving cell, it can be used for SRS transmission, when two SC-FDMA symbols exist in UpPTS of the given serving cell, both can be used for SRS transmission and for trigger type 0 SRS both can be assigned to the same UE. For TDD serving cell, and if the UE is configured with two or four additional SC-FDMA symbols in UpPTS of the given serving cell, all can be used for SRS transmission and for trigger type 0 SRS at most two SC-FDMA symbols out of the configured additional SC-FDMA symbols in UpPTS can be assigned to the same UE.

If a UE is not configured with multiple TAGs and the UE not is configured with the parameter srs-UpPtsAdd for trigger type 1, or if a UE is not configured with multiple TAGs and the UE is not configured with more than one serving cell of different CPs, or if a UE is configured with multiple TAGs and SRS and PUCCH format 2/2a/2b happen to coincide in the same subframe in the same serving cell,

-	The UE shall not transmit type 0 triggered SRS whenever type 0 triggered SRS and PUCCH format 2/2a/2b transmissions happen to coincide in the same subframe;

-	The UE shall not transmit type 1 triggered SRS whenever type 1 triggered SRS and PUCCH format 2a/2b or format 2 with HARQ-ACK transmissions happen to coincide in the same subframe;

-	The UE shall not transmit PUCCH format 2 without HARQ-ACK whenever type 1 triggered SRS and PUCCH format 2 without HARQ-ACK transmissions happen to coincide in the same subframe.

If a UE is not configured with multiple TAGs and the UE not is configured with the parameter srs-UpPtsAdd for trigger type 1, or if a UE is not configured with multiple TAGs and the UE is not configured with more than one serving cell of different CPs, or if a UE is configured with multiple TAGs and SRS and PUCCH happen to coincide in the same subframe in the same serving cell,

-	The UE shall not transmit SRS whenever SRS transmission and PUCCH transmission carrying HARQ-ACK and/or positive SR happen to coincide in the same subframe if the parameter ackNackSRS-SimultaneousTransmission is FALSE;

-	For FDD-TDD and primary cell frame structure 1, the UE shall not transmit SRS in a symbol whenever SRS transmission and PUCCH transmission carrying HARQ-ACK and/or positive SR using shortened format as defined in subclauses 5.4.1, 5.4.2A, 5.4.2B, and 5.4.2C of [3] happen to overlap in the same symbol if the parameter ackNackSRS-SimultaneousTransmission is TRUE.

-	Unless otherwise prohibited, the UE shall transmit SRS whenever SRS transmission and PUCCH transmission carrying HARQ-ACK and/or positive SR using shortened format as defined in subclauses 5.4.1 and 5.4.2A of [3] happen to coincide in the same subframe if the parameter ackNackSRS-SimultaneousTransmission is TRUE.

If a UE is not configured with multiple TAGs and the UE not is configured with the parameter srs-UpPtsAdd for trigger type 1, or if a UE is not configured with multiple TAGs and the UE is not configured with more than one serving cell of different CPs, the UE shall not transmit SRS whenever SRS transmission on any serving cells and PUCCH transmission carrying HARQ-ACK and/or positive SR using normal PUCCH format as defined in subclauses 5.4.1 and 5.4.2A of [3] happen to coincide in the same subframe.

In UpPTS, whenever SRS transmission instance overlaps with the PRACH region for preamble format 4 or exceeds the range of uplink system bandwidth configured in the serving cell, the UE shall not transmit SRS.

The parameter ackNackSRS-SimultaneousTransmission provided by higher layers determines if a UE is configured to support the transmission of HARQ-ACK on PUCCH and SRS in one subframe. If it is configured to support the transmission of HARQ-ACK on PUCCH and SRS in one subframe, then in the cell specific SRS subframes of the primary cell,

-	if the UE transmits PUCCH format 1/1a/1b/3, the UE shall transmit HARQ-ACK and SR using the shortened PUCCH format as defined in subclauses 5.4.1 and 5.4.2A of [3], where the HARQ-ACK or the SR symbol corresponding to the SRS location is punctured.

-	If the UE transmits PUCCH format 4/5 partly or fully overlapping with the cell specific SRS bandwidth in the cell specific SRS subframes of the primary cell, then UE shall transmit UCI using the shortened PUCCH format as defined in subclauses 5.4.2B and 5.4.2C of [3].

For PUCCH format 1/1a/1b/3, this shortened PUCCH format shall be used in a cell specific SRS subframe of the primary cell even if the UE does not transmit SRS in that subframe. For PUCCH format 4/5, this shortened PUCCH format shall be used if the PUCCH transmission partly or fully overlaps with the cell-specific SRS bandwidth in the cell specific SRS subframes of the primary cell even if the UE does not transmit SRS in that subframe.The cell specific SRS subframes are defined in subclause 5.5.3.3 of [3]. Otherwise, the UE shall use the normal PUCCH format 1/1a/1b as defined in subclause 5.4.1 of [3] or normal PUCCH format 3 as defined in subclause 5.4.2A or normal PUCCH format 4 as defined in subclause 5.4.2B or normal PUCCH format 5 as defined in subclause 5.4.2C of [3].

For a BL/CE UE, for a SRS transmission in subframe n and if the UE transmits PUSCH/PUCCH in subframe n and/or n+1, the UE shall not transmit the SRS in subframe n if the narrowband of PUSCH/PUCCH in subframe n and/or n+1 is different from the narrowband of the SRS in subframe n.

A BL/CE UE shall not transmit SRS in UpPTS if SRS frequency location is different from DwPTS reception narrowband in the same special subframe.

Trigger type 0 SRS configuration of a UE in a serving cell for SRS periodicity,, and SRS subframe offset,, is defined in Table 8.2-1 and Table 8.2-2, for FDD and TDD serving cell, respectively. The periodicity  of the SRS transmission is serving cell specific and is selected from the set {2, 5, 10, 20, 40, 80, 160, 320} ms or subframes. 
For the SRS periodicity  of 2 ms in TDD serving cell, two SRS resources are configured in a half frame containing UL subframe(s) of the given serving cell.

Type 0 triggered SRS transmission instances in a given serving cell for TDD serving cell with  and for FDD serving cell are the subframes satisfying  , where for FDD  is the subframe index within the frame, for TDD serving cell, if the UE is configured with the parameter srs-UpPtsAdd for trigger type 0,  is defined in Table 8.2-6; otherwise  is defined in Table 8.2-3. The SRS transmission instances for TDD serving cell with  are the subframes satisfying .

For TDD serving cell, and a UE configured for type 0 triggered SRS transmission in serving cell c, and the UE configured with the parameter EIMTA-MainConfigServCell-r12 for serving cell c, if the UE does not detect an UL/DL configuration indication for radio frame m (as described in section 13.1), the UE shall not transmit trigger type 0 SRS in a subframe of radio frame m that is indicated by the parameter eimta-HARQ-ReferenceConfig-r12 as a downlink subframe unless the UE transmits PUSCH in the same subframe.

Trigger type 1 SRS configuration of a UE in a serving cell for SRS periodicity,, and SRS subframe offset,, is defined in Table 8.2-4 and Table 8.2-5, for FDD and TDD serving cell, respectively. The periodicity  of the SRS transmission is serving cell specific and is selected from the set {2, 5, 10} ms or subframes. 
For the SRS periodicity  of 2 ms in TDD serving cell, two SRS resources are configured in a half frame containing UL subframe(s) of the given serving cell.

For TDD serving cell, and a UE configured for type 1 triggered SRS transmission in serving cell c and configured with the parameter srs-UpPtsAdd, the UE is not expected to receive trigger type 1 SRS configurations with SRS periodicity  of 2 ms.

A UE configured for type 1 triggered SRS transmission in serving cell c and not configured with a carrier indicator field shall transmit SRS on serving cell c upon detection of a positive SRS request in PDCCH/EPDCCH/MPDCCH scheduling PUSCH/PDSCH on serving cell c.

A UE configured for type 1 triggered SRS transmission in serving cell c and configured with a carrier indicator field shall transmit SRS on serving cell c upon detection of a positive SRS request in PDCCH/EPDCCH scheduling PUSCH/PDSCH with the value of carrier indicator field corresponding to serving cell c.

A non-BL/CE UE configured for type 1 triggered SRS transmission on serving cell c upon detection of a positive SRS request in subframe n of serving cell c shall commence SRS transmission in the first subframe satisfying  and

for TDD serving cell c with  and for FDD serving cell c,

for TDD serving cell c with

where for FDD serving cell c  is the subframe index within the frame , for TDD serving cell c, if the UE is configured with the parameter srs-UpPtsAdd for trigger type 1,  is defined in Table 8.2-6; otherwise  is defined in Table 8.2-3.

A BL/CE UE configured for type 1 triggered SRS transmission on serving cell c upon detection of a positive SRS request of serving cell c shall commence SRS transmission in the first subframe satisfying , where subframe n is the last subframe in which the DCI format 6-0A/6-1A with the positive SRS request is transmitted, and

for TDD serving cell c with  and for FDD serving cell c,

for TDD serving cell c with  where for FDD serving cell c  is the subframe index within the frame , for TDD serving cell c  is defined in Table 8.2-3.

A UE configured for type 1 triggered SRS transmission is not expected to receive type 1 SRS triggering events associated with different values of trigger type 1 SRS transmission parameters, as configured by higher layer signalling, for the same subframe and the same serving cell.

For TDD serving cell c, and a UE configured with EIMTA-MainConfigServCell-r12 for a serving cell c, the UE shall not transmit SRS in a subframe of a radio frame that is indicated by the corresponding eIMTA-UL/DL-configuration as a downlink subframe.

A UE shall not transmit SRS whenever SRS and a PUSCH transmission corresponding to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure coincide in the same subframe.

Table 8.2-1: UE Specific SRS Periodicity  and Subframe Offset Configuration 
 for trigger type 0, FDD

| SRS Configuration Index ISRS | SRS Periodicity (ms) | SRS Subframe Offset |
| --- | --- | --- |
| 0 – 1 | 2 | ISRS |
| 2 – 6 | 5 | ISRS – 2 |
| 7 – 16 | 10 | ISRS – 7 |
| 17 – 36 | 20 | ISRS – 17 |
| 37 – 76 | 40 | ISRS – 37 |
| 77 – 156 | 80 | ISRS – 77 |
| 157 – 316 | 160 | ISRS – 157 |
| 317 – 636 | 320 | ISRS – 317 |
| 637 – 1023 | reserved | reserved |

Table 8.2-2: UE Specific SRS Periodicity  and Subframe Offset Configuration 
 for trigger type 0, TDD

| SRS Configuration Index ISRS | SRS Periodicity (ms) | SRS Subframe Offset |
| --- | --- | --- |
| 0 | 2 | 0, 1 |
| 1 | 2 | 0, 2 |
| 2 | 2 | 1, 2 |
| 3 | 2 | 0, 3 |
| 4 | 2 | 1, 3 |
| 5 | 2 | 0, 4 |
| 6 | 2 | 1, 4 |
| 7 | 2 | 2, 3 |
| 8 | 2 | 2, 4 |
| 9 | 2 | 3, 4 |
| 10 – 14 | 5 | ISRS – 10 |
| 15 – 24 | 10 | ISRS – 15 |
| 25 – 44 | 20 | ISRS – 25 |
| 45 – 84 | 40 | ISRS – 45 |
| 85 – 164 | 80 | ISRS – 85 |
| 165 – 324 | 160 | ISRS – 165 |
| 325 – 644 | 320 | ISRS – 325 |
| 645 – 1023 | reserved | reserved |

Table 8.2-3:  for TDD

|  | subframe index n |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 |  | 2 | 3 | 4 | 5 | 6 |  | 7 | 8 | 9 |
|  |  | 1st symbol of UpPTS | 2nd symbol of UpPTS |  |  |  |  | 1st symbol of UpPTS | 2nd symbol of UpPTS |  |  |  |
| in case UpPTS length of 2 symbols |  | 0 | 1 | 2 | 3 | 4 |  | 5 | 6 | 7 | 8 | 9 |
| in case UpPTS length of 1 symbol |  | 1 |  | 2 | 3 | 4 |  | 6 |  | 7 | 8 | 9 |

Table 8.2-4: UE Specific SRS Periodicity  and Subframe Offset Configuration 
 for trigger type 1, FDD

| SRS Configuration Index ISRS | SRS Periodicity (ms) | SRS Subframe Offset |
| --- | --- | --- |
| 0 – 1 | 2 | ISRS |
| 2 – 6 | 5 | ISRS – 2 |
| 7 – 16 | 10 | ISRS – 7 |
| 17 – 31 | reserved | reserved |

Table 8.2-5: UE Specific SRS Periodicity  and Subframe Offset Configuration 
 for trigger type 1, TDD

| SRS Configuration Index ISRS | SRS Periodicity (ms) | SRS Subframe Offset |
| --- | --- | --- |
| 0 | reserved | reserved |
| 1 | 2 | 0, 2 |
| 2 | 2 | 1, 2 |
| 3 | 2 | 0, 3 |
| 4 | 2 | 1, 3 |
| 5 | 2 | 0, 4 |
| 6 | 2 | 1, 4 |
| 7 | 2 | 2, 3 |
| 8 | 2 | 2, 4 |
| 9 | 2 | 3, 4 |
| 10 – 14 | 5 | ISRS – 10 |
| 15 – 24 | 10 | ISRS – 15 |
| 25 – 31 | reserved | reserved |

Table 8.2-6:  for TDD and UE configured with two or four additional SC-FDMA symbols in UpPTS

|  | subframe index n |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 |  |  |  | 2 | 3 | 4 | 5 | 6 |  |  |  | 7 | 8 | 9 |
|  |  | 1st symbol of UpPTS | 2nd symbol of UpPTS | 3rd symbol of UpPTS | 4th symbol of UpPTS |  |  |  |  | 1st symbol of UpPTS | 2nd symbol of UpPTS | 3rd symbol of UpPTS | 4th symbol of UpPTS |  |  |  |
| in case UpPTS length of 4 symbols |  | 0 | 1 | 2 | 3 |  |  |  |  | 5 | 6 | 7 | 8 |  |  |  |
| in case UpPTS length of 2 symbols |  | 2 | 3 |  |  |  |  |  |  | 7 | 8 |  |  |  |  |  |