3GPP TSG-RAN WG4 Meeting #79 	R4-164663

Nanjing, China, 23 – 27 May, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.141 | CR | 0771 | rev | 1 | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on definition of multi-band definition and blocking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Ericssson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MB_MSR_RF-Core |  |  |  |  |  |  | Date: |  | 2016-05-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | A BS supports no overlapping frequency range in the two bands in common active RF components should be counted as Multi-band base station. In current specification, it is excluded for the case there is overlapping of two bands but the allocted frequency ranges are not overlapping in some region, e.g. Band 28+Band 20 in Europe. The clarifications in the definitions of Multi-band base station regarding non-overlapping operating bands leads to an adjustment of the note for blocking requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The definition of Multi-band base station/Multi-band transmitter/Multi-band receiver is updated. - A note in sub-clause 7.5 is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Part of Multi-band base station is excluded in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 25.104 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

Ancillary RF amplifier: piece of equipment, which when connected by RF coaxial cables to the BS, has the primary function to provide amplification between the transmit and/or receive antenna connector of a BS and an antenna without requiring any control signal to fulfil its amplifying function.

Bit Error Ratio: ratio of the bits wrongly received to all data bits sent.

NOTE:	The bits are the data bits above the convolutional/turbo decoder. The BER is the overall BER independent of frame erasures or when erased frames are not defined.

Block Error Ratio: ratio of the number of erroneous blocks received to the total number of blocks sent.

NOTE:	An erroneous block is a Transport Block whose cyclic redundancy check (CRC) is wrong.

Mean power: power (transmitted or received) in a bandwidth of at least (1+  times the chip rate of the radio access mode.

NOTE1:	α = 0,22 is the roll off factor of the WCDMA signal. The roll-off factor α factor is defined in [1] section 6.8.1.

NOTE2:	The period of measurement shall be at least one timeslot unless otherwise stated.

RRC filtered mean power: mean power as measured through a root raised cosine filter with roll-off factor  and a bandwidth equal to the chip rate of the radio access mode.

NOTE 1:	The RRC filtered mean power of a perfectly modulated W-CDMA signal is 0,246 dB lower than the mean power of the same signal.

NOTE 2:	The roll-off factor  factor is defined in [1] section 6.8.1.

Code domain power: part of the mean power which correlates with a particular (OVSF) code channel.

NOTE:	The sum of all powers in the code domain equals the mean power in a bandwidth of (1+  times the chip rate of the radio access mode. See Annex E.2.5.1.

Output power: mean power of one carrier of the base station, delivered to a load with resistance equal to the nominal load impedance of the transmitter.

Rated output power: mean power level per carrier that the manufacturer has declared to be available at the antenna connector.

Rated total output power: mean power level that the manufacturer has declared to be available at the antenna connector.

Maximum output power: mean power level per carrier of the base station measured at the antenna connector in a specified reference condition.

MIMO mode: downlink MIMO configuration with two transmit antennas.

MIMO mode with four transmit antennas: downlink MIMO configuration with four transmit antennas.

Power control dynamic range: difference between the maximum and the minimum code domain power of a code channel for a specified reference condition.

Total power dynamic range: difference between the maximum and the minimum total power for a specified reference condition.

Secondary serving HS-DSCH cell(s): set of cells where the UE is configured to simultaneously monitor an HS-SCCH set and receive the HS-DSCH if it is scheduled in that cell.

NOTE:	There can be up to 7 secondary serving HS-DSCH cells, in addition to the serving HS-DSCH cell.

1st secondary serving HS-DSCH cell: first secondary serving HS-DSCH cell.

NOTE:	If the UE is configured with two uplink frequencies, the 1st secondary serving HS-DSCH cell is the secondary serving HS-DSCH cell that is associated with the secondary uplink frequency.  If the UE is configured with a single uplink frequency, the 1st secondary serving HS-DSCH cell is a secondary serving HS-DSCH cell whose index is indicated by higher layers.

Channel bandwidth: RF bandwidth supporting a single UTRA RF carrier.

NOTE:	the channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Channel edge: lowest or highest frequency of the UTRA carrier, separated by the channel bandwidth.

Base Station RF Bandwidth: bandwidth in which a base station transmits and/or receives single or multiple carriers simultaneously within each supported operating band.

NOTE:	in single carrier operation the channel bandwidth is equal to Base Station RF Bandwidth.

Maximum Base Station RF Bandwidth: maximum Base Station RF Bandwidth supported by a BS within each supported operating band.

Maximum Radio Bandwidth: maximum frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier.

Base Station RF Bandwidth edge: frequency of one of the edges of the Base Station RF Bandwidth.

Contiguous spectrum: spectrum consisting of a contiguous block of spectrum with no sub-block gap(s).

Non-contiguous spectrum: spectrum consisting of two or more sub-blocks separated by sub-block gap(s).

Sub-block: one contiguous allocated block of spectrum for use by the same base station.

NOTE:	There may be multiple instances of sub-blocks within an Base Station RF Bandwidth.

Sub-block bandwidth: bandwidth of one sub-block.

Sub-block gap: frequency gap between two consecutive sub-blocks within a Base Station RF Bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation.

Lower sub-block edge: frequency at the lower edge of one sub-block.

NOTE:	It is used as a frequency reference point for both transmitter and receiver requirements.

Upper sub-block edge: frequency at the higher edge of one sub-block.

NOTE:	It is used as a frequency reference point for both transmitter and receiver requirements.

Multi-band Base Station: base station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not a sub-band or superseding-band of another supported operating band) than the other carrier(s).

Multi-carrier transmission configuration: set of one or more contiguous or non-contiguous carriers that a BS is able to transmit simultaneously according to the manufacturer’s specification.

Multi-band transmitter: transmitter characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not a sub-band or superseding-band of another supported operating band) than the other carrier(s).

Multi-band receiver: receiver characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not a sub-band or superseding-band of another supported operating band) than the other carrier(s).

Inter-band gap: The frequency gap between two supported consecutive operating bands.

Inter RF Bandwidth gap: frequency gap between two consecutive Base Station RF Bandwidths that are placed within two supported operating bands.

Sub-band: A sub-band of an operating band contains a part of the uplink and downlink frequency range of the operating band.

Superseding-band: A superseding-band of an operating band includes the whole of the uplink and downlink frequency range of the operating band.

Total RF Bandwidth: maximum sum of Base Station RF Bandwidths in all supported operating bands.

<Next change>

## 7.5	Blocking characteristics

<Text removed> 

For a BS capable of multi-band operation, the narrowband blocking requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least 400kHz or 600kHz, depending on the operating band. The interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -200kHz/+200kHz or -300kHz/+300kHz, respectively.

Table 7.4K: Blocking characteristics for Wide Area BS

| Operating Band | Center Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| I | 1920 - 1980 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1900 - 1920 MHz 1980 - 2000 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -1900 MHz 2000 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| II | 1850 - 1910 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1830 - 1850 MHz 1910 - 1930 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| III | 1710 - 1785 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1785 - 1805 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1805 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| IV | 1710 - 1755 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1755 - 1775 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1775 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| V | 824-849 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 804-824 MHz 849-869 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 804 MHz 869 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| VI | 810 - 830 MHz 840 - 860 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 810 MHz 860 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| VII | 2500 - 2570 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 2480 - 2500 MHz 2570 - 2590 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -2480 MHz 2590 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| VIII | 880 - 915 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 860 - 880 MHz 915 - 925 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -860 MHz 925 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| IX | 1749.9 - 1784.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1729.9 - 1749.9 MHz 1784.9 - 1804.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1729.9 MHz 1804.9 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| X | 1710 - 1770 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1770 - 1790 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1790 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XI | 1427.9 - 1447.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1407.9 - 1427.9 MHz 1447.9 - 1467.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1407.9 MHz 1467.9 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XII | 699 - 716 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 679 - 699 MHz 716 - 729 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 679 MHz 729 MHz – 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XIII | 777 - 787 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 757 - 777 MHz 787 - 807 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 - 757 MHz 807 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XIV | 788 - 798 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 768 - 788 MHz 798 - 818 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 - 768 MHz 818 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XIX | 830 - 845 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 810 - 830 MHz 845 - 865 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 810 MHz 865 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XX | 832 - 862 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal * |
|  | 821 - 832 MHz 862 - 882 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 821 MHz 882 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XXI | 1447.9 - 1462.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1427.9 - 1447.9 MHz 1462.9 - 1482.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1427.9 MHz 1482.9 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XXII | 3410 - 3490 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 3390 - 3410 MHz 3490 - 3510 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 3390 MHz 3510 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XXV | 1850 - 1915 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal * |
|  | 1830 - 1850 MHz 1915 - 1930 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XXVI | 814-849 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal * |
|  | 794-814 MHz 849-859 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 794 MHz 859 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| Note *: The characteristics of the W-CDMA interference signal are specified in Annex I. NOTE**: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to -119.6 dBm. |  |  |  |  |  |

NOTE:	Table 7.4K assumes that two operating bands, where the downlink frequencies (see Table 3.0) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.4L: Blocking characteristics for Medium Range BS

| Operating Band | Center Frequency of Interfering Signal | Interfering Signal Level | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| I | 1920 - 1980 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1900 - 1920 MHz 1980 - 2000 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -1900 MHz 2000 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| II | 1850 - 1910 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1830 - 1850 MHz 1910 - 1930 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| III | 1710 - 1785 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1785 - 1805 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1805 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| IV | 1710 - 1755 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1755 - 1775 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1775 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| V | 824-849 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 804-824 MHz 849-869 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 804 MHz 869 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| VI | 810 - 830 MHz 840 - 860 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 810 MHz 860 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| VII | 2500 - 2570 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 2480 - 2500 MHz 2570 - 2590 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -2480 MHz 2590 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| VIII | 880 - 915 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 860 - 880 MHz 915 - 925 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -860 MHz 925 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| IX | 1749.9 - 1784.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1729.9 - 1749.9 MHz 1784.9 - 1804.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1729.9 MHz 1804.9 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| X | 1710 - 1770 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1770 - 1790 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1790 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XI | 1427.9 - 1447.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1407.9 - 1427.9 MHz 1447.9 - 1467.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1407.9 MHz 1467.9 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XII | 699 - 716 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 679 - 699 MHz 716 - 729 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 679 MHz 729 MHz – 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XIII | 777 - 787 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 757 - 777 MHz 787 - 807 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 - 757 MHz 807 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XIV | 788 - 798 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 768 - 788 MHz 798 - 818 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 - 768 MHz 818 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XIX | 830 - 845 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 810 - 830 MHz 845 - 865 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 810 MHz 865 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XX | 832 - 862 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 821 - 832 MHz 862 - 882 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 821 MHz 882 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XXI | 1447.9 - 1462.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1427.9 - 1447.9 MHz 1462.9 - 1482.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1427.9 MHz 1482.9 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XXII | 3410 - 3490 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 3390 - 3410 MHz 3490 - 3510 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 3390 MHz 3510 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XXV | 1850 - 1915 MHz | -35 dBm | -105 dBm | ±10 MHz | WCDMA signal * |
|  | 1830 - 1850 MHz 1915 - 1930 MHz | -35 dBm | -105 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XXVI | 814-849 MHz | -35 dBm | -105 dBm | ±10 MHz | WCDMA signal * |
|  | 794-814 MHz 849-859 MHz | -35 dBm | -105 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 794 MHz 859 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| Note *: The characteristics of the W-CDMA interference signal are specified in Annex I. NOTE**: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to -109.6 dBm. |  |  |  |  |  |

NOTE:	Table 7.4L assumes that two operating bands, where the downlink frequencies (see Table 3.0) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.4M: Blocking characteristics for Local Area / Home BS

| Operating Band | Center Frequency of Interfering Signal | Interfering Signal Level | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| I | 1920 - 1980 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1900 - 1920 MHz 1980 - 2000 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -1900 MHz 2000 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| II | 1850 - 1910 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1830 - 1850 MHz 1910 - 1930 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| III | 1710 - 1785 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1785 - 1805 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1805 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| IV | 1710 - 1755 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1755 - 1775 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1775 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| V | 824-849 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 804-824 MHz 849-869 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 804 MHz 869 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| VI | 810 - 830 MHz 840 - 860 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 810 MHz 860 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| VII | 2500 - 2570 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 2480 - 2500 MHz 2570 - 2590 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -2480 MHz 2590 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| VIII | 880 - 915 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 860 - 880 MHz 915 - 925 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -860 MHz 925 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| IX | 1749.9 - 1784.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1729.9 - 1749.9 MHz 1784.9 - 1804.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1729.9 MHz 1804.9 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| X | 1710 - 1770 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1770 - 1790 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1790 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XI | 1427.9 - 1447.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1407.9 - 1427.9 MHz 1447.9 - 1467.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1407.9 MHz 1467.9 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XII | 699 - 716 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 679 - 699 MHz 716 - 729 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 679 MHz 729 MHz – 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XIII | 777 - 787 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 757 - 777 MHz 787 - 807 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 - 757 MHz 807 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XIV | 788 - 798 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 768 - 788 MHz 798 - 818 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 - 768 MHz 818 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XIX | 830 - 845 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 810 - 830 MHz 845 - 865 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 810 MHz 865 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XX | 832 - 862 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 821 - 832 MHz 862 - 882 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 821 MHz 882 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XXI | 1447.9 - 1462.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1427.9 - 1447.9 MHz 1462.9 - 1482.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1427.9 MHz 1482.9 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XXII | 3410 - 3490 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 3390 - 3410 MHz 3490 - 3510 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 3390 MHz 3510 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XXV | 1850 - 1915 MHz | -30 dBm | -101 dBm | ±10 MHz | WCDMA signal * |
|  | 1830 - 1850 MHz 1915 - 1930 MHz | -30 dBm | -101 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XXVI | 814-849 MHz | -30 dBm | -101 dBm | ±10 MHz | WCDMA signal * |
|  | 794-814 MHz 849-859 MHz | -30 dBm | -101 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 794 MHz 859 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| Note *: The characteristics of the W-CDMA interference signal are specified in Annex I. NOTE**: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to -105.6 dBm. |  |  |  |  |  |

NOTE:	Table 7.4M assumes that two operating bands, where the downlink frequencies (see Table 3.0) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

<End of change>