3GPP TSG RAN WG1 Meeting #87						R1-1613259

Reno, USA, November 14-18, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 0791 | rev | 2 | Current version: | 13.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the Table 16.6-2: Type 1- NPDCCH common search space candidates for NB-IoT in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  |  | Date: |  | 2016-11-17 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | 1. The maximum value of R under Rmax of 2048 in Table 16.6-2 should be 2048 not 2056. 2. The notation {x}, {y} is not used in Table 16.6-2 and Table 16.6-3. It should be removed. 3. There are change marks that affect the text colour in the current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the maximum value of R from 2056 to 2048 when Rmax=2048. - Remove Note 1 from Table 16.6-2 and Table 16.6-3 - Correct the text colour and formatting in the current spec |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Unsupported parameter value in specification. - Risk of implementation misinterpretation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.5.1.2, 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 16.5.1.2	Modulation order, redundancy version and transport block size determination

To determine the modulation order, redundancy version and transport block size for the NPUSCH, the UE shall first

-	read the “modulation and coding scheme” field () in the DCI, and

-	read the “redundancy version” field () in the DCI, and

-	read the "resource assignment" field () in the DCI, and

-	compute the total number of allocated subcarriers (), number of resource units (), and repetition number () according to subclause 16.5.1.1.

The UE shall use modulation order, = 2 if . The UE shall useand Table 16.5.1.2-1 to determine the modulation order to use for NPUSCH if .

Table 16.5.1.2-1: Modulation and TBS index table for NPUSCH with .

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 2 |
| 2 | 2 | 1 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |

NPUSCH is transmitted in N consecutive NB-IoT UL slots, ni , i=0,1,…,N-1. The redundancy version  of the NPUSCH transmission in jth block of B consecutive NB-IoT UL slots ni , is determined by, , where  if ,  otherwise. Portion of NPUSCH codeword with  as defined in clause 6.3.2 in [4] mapped to slot  of allocated  resource unit(s) is transmitted in NB-IoT UL slots ni for and   for .

The UE shall use (,) and Table 16.5.1.2-2 to determine the TBS to use for the NPUSCH. is given in Table 16.5.1.2.1-1 if ,  otherwise.

Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 |  |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 |  |  |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 |  |  |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 |  |  |
| 11 | 176 | 376 | 584 | 776 | 1000 |  |  |  |
| 12 | 208 | 440 | 680 | 1000 |  |  |  |  |

The NDI as signalled on NPDCCH, and the RV and TBS, as determined above, shall be delivered to higher layers.

#### < Unchanged parts are omitted >

## 16.6	Narrowband physical downlink control channel related procedures

#### < Unchanged parts are omitted >

Table 16.6-2: Type 1- NPDCCH common search space candidates

|  |  |  |  |  |  |  |  |  | NCCE indices of monitored NPDCCH candidates |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | L’=1 | L’=2 |
| 1 | 1 | - | - | - | - | - | - | - | - | {0,1} |
| 2 | 1 | 2 | - | - | - | - | - | - | - | {0,1} |
| 4 | 1 | 2 | 4 | - | - | - | - | - | - | {0,1} |
| 8 | 1 | 2 | 4 | 8 | - | - | - | - | - | {0,1} |
| 16 | 1 | 2 | 4 | 8 | 16 | - | - | - | -- | {0,1} |
| 32 | 1 | 2 | 4 | 8 | 16 | 32 | - | - | - | {0,1} |
| 64 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | - | - | {0,1} |
| 128 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | - | {0,1} |
| 256 | 1 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | - | {0,1} |
| 512 | 1 | 4 | 16 | 32 | 64 | 128 | 256 | 512 | - | {0,1} |
| 1024 | 1 | 8 | 32 | 64 | 128 | 256 | 512 | 1024 | - | {0,1} |
| 2048 | 1 | 8 | 64 | 128 | 256 | 512 | 1024 | 2048 | - | {0,1} |
| DCI subframe repetition number | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |  |  |
| Note 1: {x,y} denotes NPDCCH Format1 candidate corresponding to NCCEs ‘x’ and ‘y’ is monitored. |  |  |  |  |  |  |  |  |  |  |

Table 16.6-3: Type 2- NPDCCH common search space candidates

|  |  | DCI subframe repetition number | NCCE indices of monitored NPDCCH candidates |  |
| --- | --- | --- | --- | --- |
|  |  |  | L’=1 | L’=2 |
| 1 | 1 | 00 | - | {0,1} |
| 2 | 1 | 00 | - | {0,1} |
|  | 2 | 01 | - | {0,1} |
| 4 | 1 | 00 | - | {0,1} |
|  | 2 | 01 | - | {0,1} |
|  | 4 | 10 | - | {0,1} |
| >=8 |  | 00 | - | {0,1} |
|  |  | 01 | - | {0,1} |
|  |  | 10 | - | {0,1} |
|  |  | 11 | - | {0,1} |
| Note 1: {x,y} denotes NPDCCH Format1 candidate corresponding to NCCEs ‘x’ and ‘y’ is monitored. |  |  |  |  |

#### < Unchanged parts are omitted >