3GPP TSG-RAN1 Meeting #86	R1-1609824

# Lisbon, Portugal, 10th Oct 2016 - 14th Oct 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.214 | CR | 0040 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SSTD definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_SC_enh_dualC-Core |  |  |  |  |  | Date: |  |  | 2016-09-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Based on LS R1-1608579 from RAN4, the definition of Frame boundary offset in the SFN and subframe timing difference (SSTD) measurement in 36.214 is not completely correct. It might lead to ambiguity in determining the subframe pair in some DC scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the formula defined in 5.1.25. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNB could not make the correct decision on the subframe pair in some DC scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 5.1.25	SFN and subframe timing difference (SSTD)

| Definition | The observed SFN and subframe timing difference (SSTD) between a PCell and a PSCell is defined as consisting of the following three components; - SFN offset = (SFNPCell - SFNPSCell) mod 1024, where SFNPCell is the SFN of a PCell radio frame and SFNPSCell is the SFN of the PSCell radio frame of which the UE receives the start closest in time to the time when it receives the start of the PCell radio frame. - Frame boundary offset =, where TFrameBoundaryPCell is the time when the UE receives the start of a radio frame from the PCell and TFrameBoundaryPSCell is the time when the UE receives the start of the radio frame of PSCell that is closest in time to the radio frame received from the PCell. The unit of (TFrameBoundaryPCell - TFrameBoundaryPSCell) is [µs]. - Subframe boundary offset = TSubframePCell - TSubframePSCell, where TSubframePCell is the time when the UE receives the start of a subframe from the PCell and TSubframePSCell is the time when the UE receives the start of the subframe from the PSCell that is closest in time to the subframe received from the PCell. The reference point for the observed SFN and subframe time difference shall be the antenna connector of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency |