3GPP TSG-RAN WG4 Meeting #80bis 	R4-168078

# Ljubljana, Slovenia, 10 – 14 October, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 0875 | rev | - | Current version: | 13.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interfering signal bandwidth for NB-IoT BS receiver dynamic range requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  |  | Date: |  | 2016-09-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The interfering signal bandwidth for NB-IoT BS receiver dynamic range requirements should be BWChannel as stated in TR36.802, while it is specified in TS36.104 as BWConfig receiver for NB-IoT stand-alone operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the interfering signal bandwidth for NB-IoT BS receiver dynamic range requirements from BWConfig to BWChannel for NB-IoT stand-alone operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error remains in the specification, and would lead to different interpretations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

### 7.3.1	Minimum requirement

For E-UTRA, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A with parameters specified in Table 7.3.1-1 for Wide Area BS, in Table 7.3.1-2 for Local Area BS, in Table 7.3.1-3 for Home BS and in Table 7.3.1-4 for Medium Range BS.

Table 7.3.1-1: Wide Area BS dynamic range for E-UTRA carrier

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | FRC A2-1 in Annex A.2 | -76.3 | -88.7 | AWGN |
| 3 | FRC A2-2 in Annex A.2 | -72.4 | -84.7 | AWGN |
| 5 | FRC A2-3 in Annex A.2 | -70.2 | -82.5 | AWGN |
| 10 | FRC A2-3 in Annex A.2* | -70.2 | -79.5 | AWGN |
| 15 | FRC A2-3 in Annex A.2* | -70.2 | -77.7 | AWGN |
| 20 | FRC A2-3 in Annex A.2* | -70.2 | -76.4 | AWGN |

Table 7.3.1-2: Local Area BS dynamic range for E-UTRA carrier

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | FRC A2-1 in Annex A.2 | -68.3 | -80.7 | AWGN |
| 3 | FRC A2-2 in Annex A.2 | -64.4 | -76.7 | AWGN |
| 5 | FRC A2-3 in Annex A.2 | -62.2 | -74.5 | AWGN |
| 10 | FRC A2-3 in Annex A.2* | -62.2 | -71.5 | AWGN |
| 15 | FRC A2-3 in Annex A.2* | -62.2 | -69.7 | AWGN |
| 20 | FRC A2-3 in Annex A.2* | -62.2 | -68.4 | AWGN |

Table 7.3.1-3: Home BS dynamic range for E-UTRA carrier

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | FRC A2-1 in Annex A.2 | -31.8 | -44.2 | AWGN |
| 3 | FRC A2-2 in Annex A.2 | -27.9 | -40.2 | AWGN |
| 5 | FRC A2-3 in Annex A.2 | -25.7 | -38 | AWGN |
| 10 | FRC A2-3 in Annex A.2* | -25.7 | -35 | AWGN |
| 15 | FRC A2-3 in Annex A.2* | -25.7 | -33.2 | AWGN |
| 20 | FRC A2-3 in Annex A.2* | -25.7 | -31.9 | AWGN |

# Table 7.3.1-4: Medium Range BS dynamic range for E-UTRA carrier

| E-UTRA channel bandwidth [MHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4 | FRC A2-1 in Annex A.2 | -71.3 | -83.7 | AWGN |
| 3 | FRC A2-2 in Annex A.2 | -67.4 | -79.7 | AWGN |
| 5 | FRC A2-3 in Annex A.2 | -65.2 | -77.5 | AWGN |
| 10 | FRC A2-3 in Annex A.2* | -65.2 | -74.5 | AWGN |
| 15 | FRC A2-3 in Annex A.2* | -65.2 | -72.7 | AWGN |
| 20 | FRC A2-3 in Annex A.2* | -65.2 | -71.4 | AWGN |

For NB-IoT standalone operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A with parameters specified in Table 7.3.1-5 for Wide Area BS.

# Table 7.3.1-5: Wide Area BS dynamic range for NB-IoT standalone operation

| NB-IoT channel bandwidth [kHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | FRC A15-1 in Annex A.15 | -99.7 | -96 | AWGN |
| 200 | FRC A15-2 in Annex A.15 | -105.6 | -96 | AWGN |

For NB-IoT in-band or guard band operation, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A with parameters specified in Table 7.3.1-6 for Wide Area BS.

# Table 7.3.1-6: Wide Area BS dynamic range for NB-IoT in-band or guard band operation

| NB-IoT channel bandwidth [MHz] | Reference measurement channel | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] / BWChannel | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 1.4* | FRC A15-1 in Annex A.15 | -99.7 | -87.5 | AWGN |
|  | FRC A15-2 in Annex A.15 | -105.6 |  |  |
| 3* | FRC A15-1 in Annex A.15 | -99.7 | -84.2 | AWGN |
|  | FRC A15-2 in Annex A.15 | -105.6 |  |  |
| 5 | FRC A15-1 in Annex A.15 | -99.7 | -82.0 | AWGN |
|  | FRC A15-2 in Annex A.15 | -105.6 |  |  |
| 10 | FRC A15-1 in Annex A.15 | -99.7 | -79.0 | AWGN |
|  | FRC A15-2 in Annex A.15 | -105.6 |  |  |
| 15 | FRC A15-1 in Annex A.15 | -99.7 | -77.2 | AWGN |
|  | FRC A15-2 in Annex A.15 | -105.6 |  |  |
| 20 | FRC A15-1 in Annex A.15 | -99.7 | -76.0 | AWGN |
|  | FRC A15-2 in Annex A.15 | -105.6 |  |  |
| Note*: 1.4 MHz and 3 MHz channel bandwidth is not applicable to guard band operation. |  |  |  |  |

<End of change>