3GPP TSG-RAN WG4 Meeting #81 	R4-1610144

# Reno, Nevada, USA, 14 - 18 November, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 0903 | rev | - | Current version: | 13.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Fixed Reference Channels for NB-IOT reference sensitivity and dynamic range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  |  | Date: |  | 2016-11-04 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | For NB-IoT reference sensitivity and dynamic range, Channel estimation length included in the Fixed Reference Channel is an implementation dependent parameter. It is neither configurable, nor derived from configured parameters during the test. Thus, it should be indicated that it is included in Fixed Reference Channels requirements for information only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Indicate the Channel estimation length in NPUSCH Fixed Reference Channels is for information only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An implementation dependent parameter is wrongly included in FRCs as a mandate parameter. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.14, A.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

# A.14	Fixed Reference Channels for NB-IOT reference sensitivity (π/2 BPSK, R=1/3)

The parameters for the reference measurement channels are specified in Table A.14-1 for reference sensitivity and repetitions sensitivity.

Table A.14-1 FRC parameters for reference sensitivity, repetitions sensitivity and in-channel selectivity

| Reference channel | A14-1 | A14-2 | A14-3 | A14-4 |
| --- | --- | --- | --- | --- |
| Sub-carrier spacing (kHz) | 15 | 3.75 | 15 | 3.75 |
| Number of tone | 1 | 1 | 1 | 1 |
| Diversity | No | No | No | No |
| Modulation | π/2 BPSK | π/2 BPSK | π/2 BPSK | π/2 BPSK |
| Frequency offset | 0 | 0 | 0 | 0 |
| Channel estimation length (ms) Note 1 | 4 | 16 | 4 | 16 |
| Number of NPUSCH repetition | 1 | 1 | TBD | TBD |
| IMCS / TBS | 0 / 0 | 0 / 0 | 0 / 0 | 0 / 0 |
| Payload size (bits) | 32 | 32 | 32 | 32 |
| Allocated resource unit | 2 | 2 | 2 | 2 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.29 | 0.29 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 96 | 96 |
| Total symbols per resource unit | 96 | 96 | 96 | 96 |
| Tx time (ms) | 16 | 64 | TBD | TBD |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |  |  |

# A.15	Fixed Reference Channels for NB-IoT dynamic range (π/4 QPSK, R=2/3)

The parameters for the reference measurement channels are specified in Table A.15-1 for NB-IoT dynamic range.

Table A.15-1 FRC parameters for NB-IoT dynamic range

| Reference channel | A15-1 | A15-2 |
| --- | --- | --- |
| Sub carrier spacing (kHz) | 15 | 3.75 |
| Number of tone | 1 | 1 |
| Modulation | π/4 QPSK | π/4 QPSK |
| Diversity | No | No |
| Frequency offset | 0 | 0 |
| IMCS / ITBS | 7 / 7 | 7 / 7 |
| Payload size (bits) | 104 | 104 |
| Allocated resource units | 1 | 1 |
| Transport block CRC (bits) | 24 | 24 |
| Coding rate (target) | 2/3 | 2/3 |
| Coding Rate | 0.67 | 0.67 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks – C | 1 | 1 |
| Total symbols per resource unit | 96 | 96 |
| Total number of bits per resource unit | 192 | 192 |
| Tx time (ms) | 8 | 32 |
| Frequency offset | 0 | 0 |
| Channel estimation length (ms) Note 1 | 4 | 16 |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |

<End of change>