3GPP TSG RAN#76		RP-170968

West Palm Beach (FL), USA

05th – 08th June 2017

Agenda item:	9.2.1

Source:	Deutsche Telekom, T-Mobile USA, BT, KPN, KT, Orange, SK Telecom, Telecom Italia, Telefonica, Telekom R&D Sdn. Bhd., Telstra, TELUS, Swisscom, Nokia, Alcatel-Lucent Shanghai Bell Samsung

Title:	Definition of termination point(s) for NG, Xn interfaces in RAN3 stage 2 – 38.401

Document for:	Approval

# Introduction:

During discussions in RAN3#96 based on [1], it was suggested to clarify the termination points of gNB external interfaces, in case of a gNB split architecture, in TS 38.401. This clarification is required to support the requirements on standardisation of open interfaces as part of the NR WI [2].

As there was no agreement reached in RAN3#96, the co-signing companies kindly request RAN plenary to clarify:

In a disaggregated gNB deployment (i.e. gNB’s deployed as gNB-CU and gNB-DU split components), the gNB-CU terminate NG, S1-U, Xn and X2 interfaces.

This clarification is reflected in the TP below based on the last version of TS 38.401 in R3-171307 [3].

RAN#76 is requested to approve this TP.

# References

- R3-171921, Deutsche Telekom, Verizon, AT&T, T-Mobile USA, Telecom Italia, China Unicom, SK Telecom, KT; “Discussion on Termination point(s) of NG, Xn interfaces”

- RP-170847

- R3-171307, NEC, TS 384.01 NG-RAN;  Architecture description (Release 15)

# Text Proposal to TS 38.401

Beginning of Text Proposal to TS 38.401

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

gNB Central Unit (gNB-CU): a logical node hosting RRC, SDAP and PDCP protocols, and controls the operation of one or more gNB-DUs. The gNB-CU terminates F1 interface connected with the gNB-DU. The gNB-CU also terminates NG, S1-U, Xn and X2 interfaces.

Editor’s note: Definition and naming of gNB-CU are FFS.

Editor’s note: How SDAP is related to EN-DC is FFS.

Editor’s note: How many gNB-DUs can be operated by one gNB-CU is FFS.

gNB Distributed Unit (gNB-DU): a logical node hosting RLC, MAC and PHY layers, and its operation is partly controlled by gNB-CU. One gNB-DU supports one or multiple cells. The gNB-DU terminates F1 interface connected with the gNB-CU.

Editor’s note: Definition and naming of gNB-DU are FFS.

Editor’s note: Whether one gNB-DU can be connected with multiple gNB-CUs is FFS (e.g. by pooling concept).

Editor’s note: Whether one cell can be supported by multiple gNB-DUs is FFS.

gNB: as defined in TS 38.300 [2].

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

gNB-CU	gNB Central Unit

gNB-DU	gNB Distributed Unit

GTP-U	GPRS Tunnelling Protocol

IP	Internet Protocol

O&M	Operation and Maintenance

QoS	Quality of Service

RNL	Radio Network Layer

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

SCTP	Stream Control Transmission Protocol

TNL	Transport Network Layer

End of Text Proposal to TS 38.401