3GPP TSG RAN Meeting #77	RP-171776

Sapporo, Japan, 11 – 14 September 2017

Agenda item:	10.6.8

Source:	Huawei, HiSilicon

Title:	Details of specification handling of 3GPP-based access to unlicensed spectrum

Document for:	Discussion

# Introduction:

RAN#76 endorsed the proposal in RP-171128 for handling of 3GPP-based access to unlicensed spectrum:

- From Rel-15 onwards, standard impact specific to 3GPP-based access to unlicensed spectrum will be specified in a different set of specifications.

- No use of unlicensed spectrum will be assumed in the IMT-2020 self-evaluations from 3GPP, and the corresponding specifications will not be submitted to ITU-R as part of the IMT-2020 submission.

This paper discusses the details of the practical handling of RAN1 and RAN4 specifications for unlicensed spectrum, with the goal to achieve a simple solution targeting only the essential aspects of channel access mechanisms (e.g. LBT).

# Handling of RAN1 specifications

LTE-based LAA has impact across several RAN1 specifications, and is tightly related to the use of frame structure 3. The channel access mechanism specified in section 15 of TS36.213 is exclusively relevant to band 46. It would thus be straightforward to move this essential part of the use of unlicensed spectrum to a new RAN1 specification focusing on channel access mechanisms for unlicensed spectrum (covering Rel-13/14/15), while other impacts which are affecting more commonly shared functions with licensed spectrum would remain in legacy RAN1 specifications and refer as appropriate to the new TS. It is not proposed to move the definition of frame structure 3 to the new TS.

Considering that 3GPP-based access to unlicensed spectrum will soon be under study for NR with possible specification in Rel-16, it may be useful to consider the forward compatibility of the new specification and its applicability to NR from Rel-16. Therefore a TS in 37 series may be appropriate. Candidate names for the new RAN1 TS could be “channel access mechanisms for unlicensed spectrum” or “channel access mechanisms for shared spectrum” or “channel access mechanisms for unlicensed and shared spectrum”.

Proposal 1: create a new RAN1-led technical specification to specify channel access mechanisms for unlicensed and shared spectrum, taking the Rel-13/14/15 contents of section 15 of 36.213. Consider the 37 spec series.

# Handling of RAN4 specifications

LTE-based LAA also has impacts across RAN4 specifications. From the aspect of the relation with LBT mechanism, all the requirements can be categorized into three types:

- Type 1: Requirements tightly related to LBT mechanism, including channel access procedure, demodulation performance, RRM requirements, which verify the particular UE implementation or behavior under LBT mechanism.

- Type 2: Requirements defined based on reference channel with LBT model but not tightly related to LBT mechanism, like the reference sensitivity and other receiver RF requirements, which mainly verify receiver RF channel performance.

- Type 3: Requirements for Band 46 but not tightly related to LBT mechanism, including UE transmitter requirements for eLAA and BS transmitter requirements for LAA, which verify whether UE can meet the regulatory requirements for the unwanted signal emission or have good signal quality. Those requirements would be kept unchanged, if a new licensed band was specified in the same spectrum range.

There would be two alternative ways to separate the unlicensed RAN4 specifications. One is to move both Type 1 and Type 2 requirements to new specifications. The other is to move only Type 1 to new specifications. In our view, the latter is more preferable since it is easy and has less impact on the existing specifications. Besides, some of Type 1 requirements are specific for Band 46. So we also need the unlicensed band definition in the new specifications.

Based on the above analysis, the following requirements can be moved to the new RAN4 specifications dedicated to unlicensed spectrum:

- Unlicensed band definition including the frequency range, channel bandwidth and channel arrangement specified in Section 5 of 36.101 and 36.104.

- Channel access procedure for eLAA UE in Section 13 of 36.101.

- Channel access procedure for LAA BS in Section 9 of 36.104 and corresponding conformance tests in 36.141.

- All the RRM requirements defined for Frame structure 3 in 36.133, including SCell activation/deactivation delay, interruptions, timing, discovery signal based measurement delay and measurement accuracy.

- UE demodulation performance and CSI reporting requirements specified for LAA (or Frame structure type 3) in Section 8 and 9 of 36.101.

- BS demodulation performance requirements for eLAA specified in Section 8 of 36.104 and the corresponding conformance tests specified in 36.141.

Proposal 2: create new RAN4 specifications for including the unlicensed band definition, i.e. band 46, and all the requirements related to LBT mechanism for both BS and UE, including channel access procedures, RRM for frame structure 3, and demodulation tests for LAA/eLAA. Consider the 37 spec series.

# Conclusion

This paper discusses practical handling of the principle to separate aspects exclusively specific to the use of unlicensed spectrum from RAN1 and RAN4 specifications. The proposed principle is to create new RAN1-led TS and RAN4-led TS to specify channel access mechanisms for unlicensed or shared spectrum, the related requirements and the associated unlicensed or shared bands definition, starting in Rel-15.

Proposal 1: create a new RAN1-led technical specification to specify channel access mechanisms for unlicensed and shared spectrum, taking the Rel-13/14/15 contents of section 15 of 36.213. Consider the 37 spec series.

Proposal 2: create new RAN4 specifications for including the unlicensed band definition, i.e. band 46, and all the requirements related to LBT mechanism for both BS and UE, including channel access procedures, RRM for frame structure 3, and demodulation tests for LAA/eLAA. Consider the 37 spec series.

It is proposed that RAN makes a final decision at RAN#78 in December 2017 and updates the relevant LTE Rel-15 WIs with the new and impacted technical specifications. The relevant LTE WIs are “Enhancements to LTE operation in unlicensed spectrum” and “LAA/eLAA for the CBRS 3.5GHz band in the United States”.