3GPP TSG RAN Meeting #78	RP-172274

Lisboa, Portugal, 18th – 21st December 2017

Source: 	Thales

Title:	NR-NTN: Link budget analysis

TDOC Type:	pCR

Agenda Item:	9.3.1  (sFS_NR_nonterr_nw)

Document for:	Approval

Release:	Rel-15

# Abstract

The objective of this document is to propose a link budget analysis for an appendix to the TR 38.811 “Study on NR to support Non-Terrestrial Networks”.

# Proposed text for approval

It is proposed to add the following text to TR 38.811 “Study on NR to support Non-Terrestrial Networks”.

* * * Start of changes * * * * (new text)

# Annex A: Link budget analysis for the different deployment scenarios

This annex provides examples of Link Budgets associated to the different deployment scenarios.

## A.1 Introduction

This section provides an short overview of the formula used in the satellite link budgets.

The general formula for link budgets in free space between a transmitter and a receiver is provided here below, expressed in dB. It is composed of two terms:

The first term is the received carrier power over noise density ratio in dBHz called C/No, and the second term is the C/Io ratio which the carrier power to interference nose density ratio.

For convenience this term includes all the potential sources of interference: other beams interference, other system interference, and intermodulation product interference.

The first term is the received power carrier to noise density ratio expressed in  dBHz

The second is C/Io is the carrier power over interference noise density, also in dBHz.

We have

- C/No : carrier power over noise density ratio in dBHz

- EIRP: useful transmitter EIRP

- G/T : figure of merit of the receiver, where G is the receiver antenna gain,a nd T the equivalent system temperature. The unit is dB/K

- Loss: free space loss (negative according to the definition)

- : propagation margins (here positive counted); these are provisioned for shadowing margins for instance when dealing with satellite systems that can suffer from shaded conditions (trees)

- Aloes:  atmospheric losses : due to gases and rain fades: in S or L bands atmospheric losses are very small typically between 0.5 and 1 dB, but they can be very high in Ka band. They depend on the location of the terminal with respect to the satellite, the elevation angle. In this document we provide only clear sky conditions Link Budgets, with a typical atmospheric attenuation of 1 dB.

- : Additional losses, like degradation due to feeder links in case of non-regenerative systems. Typically we provision 0.5 dB feeder link degradation

- k: Boltzmann Constant: -10*LOG(k) =-228.6 dBW/K/Hz

In fact there are interference noise coming from other spot beams, other system interference, and intermodulation product interference, and so we must considerer at the entrance of the receiver the C/Io ratio which the carrier power to interference nose density ratio

The relationship between C/No and C/(No+Io) in dBHz is the following:

CN0+I0=-10×Log(10-0.1×CN0+10-0.1×CI0)

And the relationship between C/N and C/No (and all equivalent parameters like C/I or C/(N+I) is

C/N =C/No -10xLOG (BW), where BWis the signal bandwidth expressed in Hz, and C/N is in dB.

Slant range : this is the distance between the satellite and the earth station.

We define

- RT: Earth Radius

- Hs: Satellite Altitude at nadir

- Α: elevation angle of the considered point with respect to the satellite.

D=-RT×sinα+(RT2×sinα2+HS2+2×RT×Hs)

Free space loss : if we have a waveform of wavelength λ, and R the distance between with transmitter and the receiver, the free space loss is the quantity:

, and in dB . The sign is minus.

In the link budgets presented all over this document, the required C/N is the nearest value of C/N compared to the obtained C/(N+I). Required C/N≤ Obtained C/(N+I)

Note 1: the relationship  between G/T and the minimum Rx level and receiver’s antenna is:

Min_Rx_Level_At_antenna = - (G/T )+10*LOG(k)+10*LOG (BW)+ (C/N)required

Where  BW is the signal bandwidth.

In satellite world G/T is commonly used.

Note 2 : in the link budgets  Symbol rate is defined as the ratio  Ncarrier/ Ts the Time Symbol is including CP length. It is expressed in Msymb/s.

## A.1	Deployment scenario D1: GEO in Ka band

The following tables present the downlink and uplink link budgets, for fixed terminal featuring an antenna with 60 cm equivalent aperture (dish or phased array antenna) and xx W Transmit power.

The radio interface configuration is provided in the different Link Budgets Tables.

On both directions, the impact of feeder links is supposed to be around 0,5 dB degradation.

## A.1.1 Downlink performance

An example of the downlink link budgets is presented here below, and the graph presents the spectral efficiency versus the EIRP density.

Table 1: Example of GEO satellite Ka band Downlink link budgets for fixed terminal

| System performance target |  |  |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | 2 |
| Information data rate (after decoding) | Mb/s | 370 |
| Radio interface configuration | Unit | Value |
| Down-link Carrier Frequency | GHz) | 20,0 |
| Channel bandwidth | MHz | 198 |
| SCS | kHz | 60 |
| Cyclic Prefix | µs | 1,2 |
| Total Symbol duration (µs) including CP | µs | 17,84 |
| Modulation Symbol rate | Msymb/s | 185,0 |
| Terminal RF characteristics |  |  |
| G/T ( figure of merit see Note above | dB/K | 15,7 |
| Antenna polarization (Circular or Linear) |  | C |
| System assumptions |  |  |
| Elevation angle to satellite as seen from the UE | ° | 30,0 |
| Corresponding slant Range (distance between UE and satellite for the considered elevation angle) | km | 38613,7 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Satellite nominal altitude | km | 35788,0 |
| Typical average C/I within a satellite beam | dB | 16,0 |
| Satellite beam polarization |  | C |
| Link budget |  |  |
| Free space propagation loss | dB | -210,2 |
| Atmospheric loss (in clear sky conditions) | dB | 1 |
| # Required C/(N+I) for the targeted data rate | # dB | # 6,700 |
| Required useful EIRP per channel ( | dBW | 57,7 |
| Required useful satellite EIRP density | dBW/MHz | 37,5 |

The figure below provides the required useful satellite EIRP density for a range of targeted spectral efficiency. This range of performances is typically supported by current and future satellites.

Figure 1: Spectral Efficiency versus the required useful satellite EIRP density – GEO in Ka band

## A.1.1 Uplink performance

An example of the uplink link budgets is presented here below, and the graph  presents the spectral efficiency versus the satellite G/T.

The return link can use CP-OFDM or DFT-SOFDM.The first table is the results with DFT-SOFDM, the second with CP-OFDM.

Table 2: Example of GEO satellite Ka band Uplink Link budgets with DFT-SOFDM

| System performance target |  | Fixed |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | # 2,3750 |
| Information data rate (after decoding) | Mb/s | 19,17 |
| Radio interface configuration |  |  |
| Up-link carrier frequency | GHz | 30,00000 |
| Channel Bandwidth | Mhz | 9,072 |
| Cyclic Prefix | µs | 2,3 |
| Total Symbol duration (µs) | µs | 35,68 |
| Modulation symbol rate | Msymb/s | 8,1 |
| Terminal RF characteristics |  |  |
| Max EIRP (corresponding to 2 W Transmission power) | dBW | 45 |
| System assumptions |  |  |
| Elevation angle to satellite | ° | 30,0 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 38613,7 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Nominal altitude | km | 35788,0 |
| Typical average C/I in a beam | dB | 16,0 |
| Link budget |  |  |
| Output back off in the terminal | dB | 1,0 |
| Terminal useful EIRP per channel | dBW | 44,0 |
| Free space propagation loss | dB | -213,7 |
| Atmospheric propagation loss (in clear sky conditions) | dB | 1,0 |
| # Required C/(N+I) | # dB | # 8,488 |
| # Required C/N | # dB | # 8,2 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | 22,0 |

Table 3: Example of GEO satellite Ka band Uplink Link budget with CP-OFDM

| System performance target |  | Fixed |
| --- | --- | --- |
| Spectral efficiency | bits/s/Hz | # 2,0 |
| Information data rate (after decoding) | Mb/s | 16,4 |
| Radio interface configuration |  |  |
| Up-link carrier frequency | GHz | 30,00000 |
| Channel Bandwidth | Mhz | 9,072 |
| (Cyclic Prefix | µs | 2,3 |
| Total Symbol duration (µs) | µs | 35,68 |
| Modulation symbol rate | Msymb/s | 8,1 |
| Terminal RF characteristics |  |  |
| Max EIRP (corresponding to 2 W Transmission power) | dBW | 45 |
| System assumptions |  |  |
| Elevation angle to satellite | ° | 30,0 |
| Slant Range (distance between UE and satellite for the considered elevation angle) | km | 38613,7 |
| Clear sky conditions |  |  |
| Satellite RF characteristics |  |  |
| Nominal altitude | km | 35788,0 |
| Typical average C/I in a beam | dB | 16,0 |
| Link budget |  |  |
| Output back off in the terminal | dB | 2,5 |
| Terminal useful EIRP per channel | dBW | 42,5 |
| Free space propagation loss | dB | -213,7 |
| Atmospheric propagation loss (in clear sky conditions) | dB | 1,0 |
| # Required C/(N+I) | # dB | # 7,228 |
| # Required C/N | # dB | # 6,7 |
| Required satellite G/T (corresponding to Rx sensitivity) | dB/K | 22,0 |

The figure below provides the required satellite G/T for a range of targeted spectral efficiency. This range of performances is typically supported by current and future satellites.

Figure 2: Spectral Efficiency versus the required satellite G/T – GEO in Ka band for DFTS-OFDM

## A.2	Deployment scenario D2 : GEO satellite in S band

## A.3	Deployment scenario D3 : LEO satellite at 600 km in S band

## A.4  Scenario D4 : LEO at 600 km in Ka band

## A.5 : Scenario D5: HAPS at 20 km in S band

## A.6 : Performances assessment summary

## A.6.1 Ka band deployment scenarios: D1 and D4

For the Ka band deployment scenarios (GEO and LEO at 600 km), we summarise here under the minimum satellite payload performance requirements (EIRP density and figure of merit (G/T) for a spectral efficiency of 2.

It assumes for UE, a terminal equiped with 60 cm equivalent aperture antenna (dish or phased array antenna) and 45 dBW maximum EIRP corresponding to approximately 2 W transmit power.

Table 13:Minimum Required Useful Satellite EIRP density (dBW/MHz) for a Spectral Efficiency of 2 bit/s/Hz

|  | GEO: 35786km | LEO: 600 km |
| --- | --- | --- |
| Minimum EIRP density (dBW/MHz) | 37 dBW/MHz | TBD |

Table 14:Minimum Required Useful Satellite G/T (dB/K) for a Spectral Efficiency of 2 bit/s/Hz

|  | GEO: 35786km | LEO: 600 km |
| --- | --- | --- |
| Minimum G/T (dB/K) | 22 dB/K | TBD |

## A.6.2 S band deployment scenarios: D2, D3 and D5

For the S band deployment scenarios (GEO, LEO at 600 km and HAPS), we summarise here under the minimum space or airborne payload performance requirements (EIRP density and figure of merit, G/T) for various targeted data rates on respectively down and uplink.

It assumes for UE, the 3GPP power class 3 performance.

Table 11: Minimum Required Useful Satellite EIRP (dBW) per 180 kHz carrier

| Downlink Data rate per 180 KHz channel | GEO: 35786km | LEO: 600 km | HAPS: 20 km |
| --- | --- | --- | --- |
| TBD kb/s | TBD dBW | TBD dBW | TBD dBW |

Table 12 :Minimum Required space and airborne G/T (in dB/K)

| Uplink data rate per UE | HAPS: 20 km | LEO: 600 km | GEO: 36000 km |
| --- | --- | --- | --- |
| TBD kbps | N/A | TBD dB/K | TBD dB/K |

* * * End of Changes * * * *