3GPP TSG RAN Meeting #78	RP-172391

# Lisbon, Portugal, Dec 18 - 21, 2017

# Source:	ZTE Corp.

# Title:	On handling of band combinations for NR

# Document for:	Discussion

Agenda Item:	9.2.1

1	Introduction

As NR standardization is approaching to its first release in Jun 2018, a boom of band and band combinations for NR is observed which is mainly driven by operators’ demands. According to the latest update of band combination list for NR, there are 506 DC and CA band combinations proposed and included in the first release [1]. This is not touched or discussed in the latest agreements on down selection at RAN plenary [2].

In this document, we discuss how to handle band combinations for NR, aiming to prioritization of the current band combinations on table.

2	Discussion

2.1	Overview

Some generic principles were agreed in RAN4#85 on introducing band and band combinations for NR [3]:

- For RAN4 NR core specification(s) by December 2017, only the bands or band combinations that meet the following criteria shall be introduced:

  - The bands or band combinations have been captured in the latest version of Rel-15 NR WID approved in RAN#77.

  - The band or band combinations have been discussed by RAN4 #85, i.e., RAN4 contributions or TPs for such bands and band combinations have been submitted and discussed in RAN4 by RAN4 #85 (including RAN4 #85).

- By June 2018, band and band combinations whose specific requirements remain still TBD shall be removed from Rel-15 specification(s).

- Those bands or band combinations that are not included in Rel-15 specifications can be supported by UEs in a release independent manner.

  - Note which features can be supported in a release independent manner may be affected by RAN4 and the other WG’s progress in Rel15.

But the generic principles seem not enough for the sake of NR standardization process, given that only 6 months are left until the formal first release of NR in Jun 2018.

To ensure the completion of the first release in time, we look through the overall progress of band combination and propose a prioritized way of handling these band combinations.

2.2	Progress of the current band combinations

According to the current procedure, a band or band combination may be included into NR by one proponent together with 3 supporting companies. Interests in the band and band combinations for every company are guaranteed and respected in this way, but of course, it also accounts for the booming of the band and band combinations on the table.

Below we check the overall progress of band combinations after the latest RAN4#85 which is shown in Annex in details.

2.2.1 	Band combinations for LTE-NR dual connectivity

There are nine groups of band combinations for LTE-NR dual connectivity by considering different number of LTE CCs and NR CCs:

- LTE 1 CC + NR 1 CC

- LTE 2 CC + NR 1 CC

- LTE 3 CC + NR 1 CC

- LTE 4 CC + NR 1 CC

- LTE 5 CC + NR 1 CC

- LTE 1 CC + NR 2 CC

- LTE 2 CC + NR 2 CC

- LTE 3 CC + NR 2 CC

- LTE 4 CC + NR 2 CC

LTE 1 CC + NR 1 CC

There are 97 band combinations in total for this group, and 25 of them are completed.

LTE 2 CC + NR 1 CC

There are 141 band combinations in total for this group, and 34 of them are completed.

LTE 3 CC + NR 1 CC

There are 116 band combinations in total for this group, and 26 of them are completed.

LTE 4 CC + NR 1 CC

There are 58 band combinations in total for this group, and 7 of them are completed.

LTE 5 CC + NR 1 CC

There are 9 band combinations in total for this group, and none of them are completed.

LTE 1 CC + NR 2 CC

There are 38 band combinations in total for this group, and none of them are completed.

LTE 2 CC + NR 2 CC

There are 9 band combinations in total for this group, and none of them are completed.

LTE  3 CC + NR 2 CC

There are 8 band combinations in total for this group, and none of them are completed.

LTE 4 CC + NR 2 CC

There are 1 band combinations in total for this group, and it is not completed yet.

The following table summarizes the progress on band combinations for LTE-NR DC:

Table-1 Progress on band combinations for LTE-NR dual connectivity

| LTE – NR Band combination | Number (completed) | Number (Total) | Progress (%) |
| --- | --- | --- | --- |
| LTE 1 CC + NR 1 CC | 25 | 97 | 26 |
| LTE 2 CC + NR 1 CC | 34 | 141 | 24 |
| LTE 3 CC + NR 1 CC | 26 | 116 | 22 |
| LTE 4 CC + NR 1 CC | 7 | 58 | 12 |
| LTE 5 CC + NR 1 CC | 0 | 9 | 0 |
| LTE 1 CC + NR 2 CC | 0 | 38 | 0 |
| LTE 2 CC + NR 2 CC | 0 | 9 | 0 |
| LTE 3 CC + NR 2 CC | 0 | 8 | 0 |
| LTE 4 CC + NR 2 CC | 0 | 1 | 0 |
| Overall progress | 19% |  |  |

Observation 1: Only 19% of the band combinations for LTE-NR dual connectivity are completed up to now.

2.2.2 	Band combinations for NR CA

There are two types of band combinations for NR CA introduced for the time being:

- Intra-band CA with xDL/1UL

- Inter-band CA with 2DL/1UL

NR intra-band CA with xDL/1UL

There are 9 band combinations introduced for NR intra-band CA with  xDL/1UL and none of them is completed.

NR inter-band CA with 2DL/1UL

There are 20 band combinations introduced for NR intra-band CA with  2DL/1UL and one of them is completed.

The following table summarizes the progress on band combinations for NR CA:

Table-2 Progress on band combinations for NR CA

| LTE – NR Band combination | Number (completed) | Number (Total) | Progress (%) |
| --- | --- | --- | --- |
| NR intra-band CA with xDL/1UL | 0 | 9 | 0 |
| NR inter-band CA with 2DL/1UL | 1 | 20 | 5 |
| Overall progress | 3% |  |  |

Observation 2: Only 3% of the band combinations for NR CA are completed up to now.

According to Rel-15 timeline, part of the Rel-15 specs should be finished by Dec 2017, and the rest part by Jun 2018. With such a tight timeline and the amount of workload, some kind of prioritization on band combinations should be considered in order to ensure the completion of Rel-15 in time.

Proposal 1: Prioritization on band combinations should be considered in order to ensure the completion of Rel-15 in time.

2.3	Prioritizing the current band combinations

For the current band combinations, it could be grouped further according to how strong the demand for each band combination. For instance, those of  the band combinations with a substantiate deployment plan in 3 years, they can be labelled as “strong”, otherwise they are regarded as “weak demand”.

And global harmonization is also important to determine the priority of the band combinations. Obviously, these globally harmonized band combinations should have a higher priority than those for regional band combinations.

As shown in Fig. 1, the band combinations on the table can be categorized as:

- Global strong demand

- Regional strong demand

- Global weak demand

- Regional weak demand

This will act as basis for prioritizing the works on band combinations, and it is natural that the “global strong demand” band combinations have the higher priority.

Proposal 2: Prioritizing the global strong band combinations.

Fig. 1, Prioritization of band combinations

Considering the possibility of early deployment of NR systems, for those operators who are interested in starting with NSA mode, the band combination of LTE 1 CC + NR 1 CC remains on top of the demands globally. Similarly for those operators who are interested in starting with SA mode directly, the worldwide demand for the band combinations of NR intra-band and inter-band CA is also strong. Therefore these band combinations should have the highest priority.

Proposal 3: Band combinations for LTE 1 CC + NR 1CC and NR intra-band and inter-band CA should have the highest priority.

3	Conclusion

This document provides our main observations and proposals on handling of band combinations for NR:

Observation 1: Only 19% of the band combinations for LTE-NR dual connectivity are completed up to now.

Observation 2: Only 3% of the band combinations for NR CA are completed up to now.

Proposal 1: Prioritization on band combinations should be considered in order to ensure the completion of Rel-15 in time.

Proposal 2: Prioritizing the global strong band combinations.

Proposal 3: Band combinations for LTE 1 CC + NR 1CC and NR intra-band and inter-band CA should have the highest priority.

Reference

- 3GPP, R4-1713190, Update of NR band combination list,  NTT DoCoMo

- 3GPP, RP-172108, Updated workplan for RAN1 and RAN4 NR, NTT DOCOMO & drafting group

- 3GPP, R4-1714088, Principle on introducing bands/band combinations in Rel-15 Specifications for NR by December 2017,

Annex

Note: Highlighted band combinations are included in RAN4 #85.

Table-3 Progress on band combinations for LTE 1 CC + NR 1 CC

| Combination |  | contact name, company | contact email | other supporting companies (min. 3) | Status (After #85) |
| --- | --- | --- | --- | --- | --- |
| LTE bands | NR |  |  |  |  |
| 1 | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| 1 | n78 | ZHANG Meng, China Telecom | zhangmeng.bri@chinatelecom.cn | Vodafone, Orange, Deutsche Telekom, Telia, Qorvo, KDDI, LG Uplus, BT, Huawei, HiSilicon, ZTE, Nokia, SK telecom, China Unicom | completed |
| 3 | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu, Deutsche Telekom, Vodafone, Orange, Telia, Qorvo | completed |
| 3 | n78 | ZHANG Meng, China Telecom | zhangmeng.bri@chinatelecom.cn | Telecom Italia, Telstra, BT, Deutsche Telekom, Huawei, HiSilicon, ZTE, Nokia, SK telecom, China Unicom | completed |
| 7 | n78 | Suhwan Lim, LG Electronics | suhwan.lim@lge.com | LG Uplus, SK Telecom, Telecom Italia, Telstra, Deutsche Telekom, Vodafone, Orange, Deutsche Telekom, Telia, Qorvo, BT, Huawei, Nokia | completed |
| 5 | n78 | ZHANG Meng, China Telecom | zhangmeng.bri@chinatelecom.cn | ZTE, Huawei, HiSilicon, CATT, LG Uplus, SK telecom | Ongoing |
| 8 | n78 | Shengxiang Guo, ZTE | guo.shengxiang@zte.com.cn | CMCC, CATT, Nubia, SoftBank, China Unicom | Ongoing |
| 8 | n77 | Kenichi Kihara，SoftBank | kenichi.kihara@g.softbank.co.jp | ZTE, Sanechips, Huawei, HiSilicon | Ongoing |
| 19 | n77 | Suguru Okuyama，NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| 19 | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| 20 | n78 | Liu Bo, Huawei | brod.liubo@huawei.com | Orange, Hisilicon, Telecom Italia, Deutsche Telekom, Vodafone, Telia, Qorvo | completed |
| 21 | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| 21 | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| 26 | n77 | Masaaki Obara, KDDI | ms-obara@kddi.com | Ericsson, Nokia, Samsung, Sumitomo | Ongoing |
| 26 | n78 | Masaaki Obara, KDDI | ms-obara@kddi.com | Ericsson, Nokia, Samsung, Sumitomo | Ongoing |
| 28 | n78 | Liu Bo, Huawei | brod.liubo@huawei.com | Orange, Hisilicon, Telstra, Telefonica, Swisscom, Deutsche Telekom, NTT DOCOMO, Nokia, NEC, Fujitsu | completed |
| 28 | n77 | Liu Bo, Huawei | brod.liubo@huawei.com | NTT DOCOMO, KDDI, Softbank | completed |
| 38 | n78 | Alper Ucar, Vodafone | alper.ucar@vodafone.com | Huawei, Ericsson, Nokia, Deutsche Telekom | No further progress |
| 39 | n78 | Yuexia Song, CATT | songyuexia @catt.cn | CATT, CMCC, Huawei, Hisilicon, ZTE | Ongoing |
| 41 | n78 | Yuexia Song, CATT | songyuexia @catt.cn | CMCC, China Telecom, Huawei, Hisilicon, ZTE | Ongoing |
| 41 | n77 | Kenichi Kihara, SoftBank | Kenichi.kihara@g.softbank.co.jp | KDDI, Ericsson, Nokia | Ongoing in #85 |
| 42 | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | ETISALAT, KDDI, Softbank, NEC, Nokia | completed |
| 42 | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | ETISALAT, KDDI, Softbank, NEC, Nokia | completed |
| 1 | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| 3 | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu, China Unicom | completed |
| 8 | n79 | Shengxiang Guo, ZTE | guo.shengxiang@zte.com.cn | CMCC, CATT, Nubia | Ongoing |
| 19 | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| 21 | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| 26 | n79 | Masaaki Obara, KDDI | ms-obara@kddi.com | Ericsson, Nokia, Samsung, Sumitomo | Ongoing |
| 28 | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| 39 | n79 | Yuexia Song, CATT | songyuexia @catt.cn | CMCC, Huawei, Hisilicon, ZTE | Ongoing |
| 41 | n79 | Yuexia Song, CATT | songyuexia @catt.cn | CMCC, Huawei, Hisilicon, ZTE, KDDI | Ongoing |
| 42 | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | KDDI, Softbank, NEC, Nokia | Ongoing |
| 1 | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu, SK telecom | Ongoing |
| 3 | n258 | Nick Tompson, Telstra | nick.a.tompson@team.telstra.com | Telecom Italia, Ericsson, Nokia, Samsung | Ongoing |
| 3 | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu, Telecom Italia, Deutsche Telekom, SK telecom | Ongoing |
| 5 | n257 | Yunsung Kim, LG Uplus | kimy0011@lguplus.co.kr | LG Electronics, SK Telecom, Samsung, Ericsson-LG, Huawei, HiSilicon | completed |
| 7 | n258 | Nick Tompson, Telstra | nick.a.tompson@team.telstra.com | Telecom Italia, Ericsson, Nokia, Samsung | Ongoing |
| 7 | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG U+, LG Electronics, Telecom Italia, [Deutsche Telekom] | completed |
| 8 | n258 | Shengxiang Guo, ZTE | guo.shengxiang@zte.com.cn | CMCC, CATT, Nubia | Ongoing |
| 8 | n257 | Kenichi Kihara, SoftBank | kenichi.kihara@g.softbank.co.jp | ZTE, Sanechips, Huawei, HiSilicon | Ongoing |
| 19 | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | Ongoing |
| 20 | n258 | Liu Ye, Huawei | leo.liuye@huawei.com | Telecom Italia, Deutsche Telekom, Orange | No progress |
| 21 | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | Ongoing |
| 26 | n257 | Masaaki Obara, KDDI | ms-obara@kddi.com | Ericsson, Nokia, Samsung, Sumitomo | Ongoing |
| 28 | n258 | Nick Tompson, Telstra | nick.a.tompson@team.telstra.com | Ericsson, Nokia, Samsung | Ongoing |
| 28 | n257 | Kenichi Kihara, SoftBank Corp. | kenichi.kihara@g.softbank.co.jp | Nokia, NEC, Fujitsu, NTT DOCOMO | completed |
| 39 | n258 | Yuexia Song, CATT | songyuexia @catt.cn | CATT, CMCC, Huawei, Hisilicon, ZTE | Ongoing |
| 41 | n258 | Yuexia Song, CATT | songyuexia @catt.cn | CATT, CMCC, Huawei, Hisilicon, ZTE | Ongoing |
| 41 | n257 | Masaki Obara, KDDI | ms-obara@kddi.com | Ericsson, Samsung, Sumitomo, SOFTBANK | Ongoing |
| 42 | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | KDDI, Softbank, NEC, Nokia | completed |
| 66 | n257 | Nelson Ueng, T-Mobile USA | neng-tsann.ueng@t-mobile.com | Ericsson, Nokia, DISH Network, Deutsche Telekom, Qorvo, Skyworks | Ongoing |
| 66 | n260 | Nelson Ueng, T-Mobile USA | neng-tsann.ueng@t-mobile.com | Ericsson, Nokia, Deutsche Telekom , Qorvo, Skyworks | No progress |
| 1 | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| 3 | 7 | Bo-Han Hsieh, CHTTL | pohanhsieh@cht.com.tw | British Telecom, Huawei, Mediatek, Nokia | completed |
| 3 | 8 | Alper Ucar, Vodafone | Alper.ucar@vodafone.com | Ericsson, Huawei, BT | No progress |
| 3 | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| 7 | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| 25 | 41 | George Cummings, Sprint | george.cummings@sprint.com | Nokia, C-Spire, Ericsson, Samsung Qorvo, Skyworks | Ongoing |
| 26 | 41 | George Cummings, Sprint | george.cummings@sprint.com | Nokia, C-Spire, Ericsson, Samsung Qorvo, Skyworks | Ongoing |
| 41 | 41 | George Cummings, Sprint | george.cummings@sprint.com | C-Spire, Ericsson, Samsung Qorvo, Skyworks | Ongoing |
| 18 | n77 | Masaki Obara, KDDI | ms-obara@kddi.com | Nokia, Samsung, Sumitomo | Ongoing |
| 18 | n78 | Masaki Obara, KDDI | ms-obara@kddi.com | Nokia, Samsung, Sumitomo | Ongoing |
| 18 | n79 | Masaki Obara, KDDI | ms-obara@kddi.com | Nokia, Samsung, Sumitomo | Ongoing |
| 18 | n257 | Masaki Obara, KDDI | ms-obara@kddi.com | Nokia, Samsung, Sumitomo | Ongoing |
| 11 | n77 | Masaki Obara, KDDI | ms-obara@kddi.com | Nokia, Samsung, Sumitomo | Ongoing |
| 11 | n78 | Masaki Obara, KDDI | ms-obara@kddi.com | Nokia, Samsung, Sumitomo, SOFTBANK | Ongoing |
| 11 | n79 | Masaki Obara, KDDI | ms-obara@kddi.com | Nokia, Samsung, Sumitomo SOFTBANK | Ongoing |
| 11 | n257 | Masaki Obara, KDDI | ms-obara@kddi.com | Nokia, Samsung, Sumitomo SOFTBANK | Ongoing |
| 20 | 28 | Alper Ucar, Vodafone | Alper.ucar@vodafone.com | Huawei, Ericson, Nokia | completed |
| 71 | n71 | Nelson Ueng, T-Mobile USA | neng-tsann.ueng@t-mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | ongoing |
| 66 | n71 | Nelson Ueng, T-Mobile USA | neng-tsann.ueng@t-mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| 2 | n71 | Nelson Ueng, T-Mobile USA | neng-tsann.ueng@t-mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| 1 | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| 3 | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| 7 | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| 20 | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| 28 | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| 42 | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| 1 | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| 3 | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| 7 | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| 20 | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| 28 | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| 42 | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| 28 | n8 | Nick Tompson, Telstra | nick.a.tompson@team.telstra.com | Ericsson, Nokia, [Samsung], Cohere Technologies | No progress |
| 42 | n258 | Per Lindell, Ericsson | per.lindell@ericsson.com | ZTE, Huawei, MediaTek |  |
| 2 | n260 | Marc Grant, AT&T | Marc.grant@att.com | Nokia, Ericsson, Qualcomm |  |
| 12 | n260 | Marc Grant, AT&T | Marc.grant@att.com | Nokia, Ericsson, Qualcomm |  |
| 1 | 40 | Satish Jamadagni, RJIO | Satish.Jamadagni@ril.com | Airtel, RJIO, Optus, Telkomsel, Globe, Ericsson, Qualcomm, Intel, Huawei, Nokia, |  |
| 3 | 40 | Satish Jamadagni, RJIO | Satish.Jamadagni@ril.com | Airtel, RJIO, Optus, Telkomsel, Globe, Ericsson, Qualcomm, Intel, Huawei, Nokia, |  |
| 5 | 40 | Satish Jamadagni, RJIO | Satish.Jamadagni@ril.com | Airtel, RJIO, Optus, Telkomsel, Globe, Ericsson, Qualcomm, Intel, Huawei, Nokia, |  |
| 8 | 40 | Satish Jamadagni, RJIO | Satish.Jamadagni@ril.com | Airtel, RJIO, Optus, Telkomsel, Globe, Ericsson, Qualcomm, Intel, Huawei, Nokia, |  |
| 40 | 40 | Satish Jamadagni, RJIO | Satish.Jamadagni@ril.com | Airtel, RJIO, Optus, Telkomsel, Globe, Ericsson, Qualcomm, Intel, Huawei, Nokia, |  |
| 40 | n78 | Satish Jamadagni, RJIO | Satish.Jamadagni@ril.com | Airtel, RJIO, Optus, Telkomsel, Globe, Ericsson, Qualcomm, Intel, Huawei, Nokia, |  |
| 8 | 20 | Alper Ucar, Vodafone | Alper.ucar@vodafone.com | Ericsson, Huawei, Skyworks, Qorvo, Nokia |  |
| 20 | 8 | Alper Ucar, Vodafone | Alper.ucar@vodafone.com | Orange, Ericsson, Huawei, Skyworks, Qorvo, Nokia |  |

Table-4 Progress on band combinations for LTE 2 CC + NR 1 CC

| Combination |  | contact name, company | contact email | other supporting companies (min. 3) | Status (After #85) |
| --- | --- | --- | --- | --- | --- |
| LTE band combinations (2CC) | NR1 |  |  |  |  |
| CA_41C | n41 | george cummings, Sprint | george.cummings@sprint.com | Ericsson, Qorvo, Nokia | ongoing |
| CA_66A-71A | n71 | Nelson Ueng, T-Mobile USA | neng-tsann.ueng@t-mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| CA_2A-66A | n71 | Nelson Ueng, T-Mobile USA | neng-tsann.ueng@t-mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| CA_2A-71A | n71 | Nelson Ueng, T-Mobile USA | neng-tsann.ueng@t-mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| CA_3A-7A | n78 | Nick Tompson, Telstra | nick.tompson@team.telstra.com | Nokia, ZTE, Ericsson, Samsung, Cohere Technologies, BT, Deutsche Telekom, Huawei, Vodafone, Orange, Telia, Qorvo, SK Telecom | No further progress |
| CA_3A-28A | n78 | Nick Tompson, Telstra | nick.tompson@team.telstra.com | Nokia, ZTE, Ericsson, Samsung, Cohere Technologies | No progress |
| CA_7A-28A | n78 | Nick Tompson, Telstra | nick.tompson@team.telstra.com | Nokia, ZTE, Ericsson, Samsung, Cohere Technologies | No progress |
| CA_7C | n78 | Nick Tompson, Telstra | nick.tompson@team.telstra.com | Nokia, ZTE, Ericsson, Samsung, Cohere Technologies | No progress |
| CA_1A-18A | n78 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-28A | n78 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_18A-28A | n78 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-18A | n77 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-28A | n77 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_18A-28A | n77 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-41A | n79 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-42A | n79 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_41A-42A | n79 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_42C | n79 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-41A | n257 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-42A | n257 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_41A-42A | n257 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_42C | n257 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_5A-7A | n78 | Yunsung Kim, LG Upus | kimy0011@lguplus.co.kr | LG Electronics, SK Telecom, Samsung | completed |
| CA_1A-3A | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-19A | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-19A | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-21A | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-21A | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_3A-19A | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_3A-19A | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_3A-21A | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_3A-21A | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_19A-21A | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_19A-21A | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-19A | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-21A | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_3A-19A | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_3A-21A | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_19A-21A | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-19A | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-21A | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_3A-19A | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_3A-21A | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_19A-21A | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A | n78 | Hongliang Bian, China Telecom | bianhl.bri@chinatelecom.cn | China Unicom, Huawei, Hisilicon, ZTE, Xiaomi, Vodafone, Orange, Deutsche Telekom, Telia, Qorvo, SK telecom, BT, Nokia | Ongoing |
| CA_1A-5A | n78 | Hongliang Bian, China Telecom | bianhl.bri@chinatelecom.cn | Huawei, Hisilicon, ZTE, Xiaomi, LG Uplus, LG Electronics, SK telecom | completed |
| CA_1A-41A | n78 | Hongliang Bian, China Telecom | bianhl.bri@chinatelecom.cn | Huawei, Hisilicon, ZTE | No progress |
| CA_5A-41A | n78 | Hongliang Bian, China Telecom | bianhl.bri@chinatelecom.cn | Huawei, Hisilicon, ZTE | No progress |
| CA-3C | n78 | Naveen Khapali, BT | Naveen.khapali@ee.co.uk | Deutsche Telekom, Huawei, Nokia, China Unicom | No progress |
| CA_7C | n78 | Naveen Khapali, BT | Naveen.khapali@ee.co.uk | Deutsche Telekom, Huawei, Nokia | No progress |
| CA_1A-7A | n78 | Naveen Khapali, BT | Naveen.khapali@ee.co.uk | Deutsche Telekom, Huawei, Nokia, Vodafone, Orange, Telia, Qorvo, SK telecom | completed |
| CA_1A-3A | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| CA_1A-7A | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| CA_1A-20A | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| CA_3A-7A | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| CA_3A-20A | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| CA_7A-20A | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| CA_3A-5A | n78 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung, China Telecom, Huawei, Hisilicon, ZTE, Xiaomi | Ongoing in #85 |
| CA_7A-7A | n78 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_1A-3A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_1A-5A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_1A-7A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_3A-5A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_3A-7A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_5A-7A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_7A-7A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_1A-20A | n78 | Alper Ucar, Vodafone | alper.ucar@vodafone.com | Orange, Deutsche Telekom, Telia, Qorvo | Ongoing |
| CA_3A-20A | n78 | Alper Ucar, Vodafone | alper.ucar@vodafone.com | Orange, Deutsche Telekom, Telia, Qorvo | Ongoing |
| CA_7A-20A | n78 | Alper Ucar, Vodafone | alper.ucar@vodafone.com | Orange, Deutsche Telekom, Telia, Qorvo | Ongoing |
| CA_3C | n78 | Alper Ucar, Vodafone | alper.ucar@vodafone.com | Huawei, Ericsson, Nokia, Deutsche Telekom | No progress |
| CA_3A-38A | n78 | Alper Ucar, Vodafone | alper.ucar@vodafone.com | Huawei, Ericsson, Nokia, Deutsche Telekom | No further progress |
| CA_1A-3A | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-20A | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-28A | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_3A-20A | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_3A-28A | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_7C | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-7A | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_3A-7A | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_20A-28A | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_42C | n50 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-3A | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-20A | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-28A | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_3A-20A | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_3A-28A | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_7C | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-7A | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_3A-7A | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_20A-28A | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_42C | n51 | Abdulhadi AbouAlmal, Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_3A-41A | n78 | Hu Rongyi, China Unicom | hury3@chinaunicom.cn | Huawei, Hisilicon, ZTE | No progress |
| CA_1A-8A | n78 | Hu Rongyi, China Unicom | hury3@chinaunicom.cn | Huawei, Hisilicon, ZTE | No progress |
| CA_3A-8A | n78 | Hu Rongyi, China Unicom | hury3@chinaunicom.cn | Huawei, Hisilicon, ZTE | No progress |
| CA_2A-12A | n260 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_2A-5A | n260 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_2A-66A | n260 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_2A-30A | n260 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_12A-66A | n260 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_12A-30A | n260 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_30A-66A | n260 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_66A-66A | n260 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | ongoing |
| CA_5A-30A | n260 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_5A-66A | n260 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_2A-12A | n5 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_2A-66A | n5 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_2A-30A | n5 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_12A-30A | n5 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_12A-66A | n5 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_30A-66A | n5 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_66A-66A | n5 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_2A-12A | n66 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_2A-5A | n66 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_2A-30A | n66 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_12A-30A | n66 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_5A-30A | n66 | Marc Grant, AT&T | Marc.grant@att.com | Qualcomm, Nokia, Ericsson | No progress |
| CA_1A-42A | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-42A | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-42A | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_21A-42A | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| 42C | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-42A | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-42A | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-42A | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_21A-42A | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| 42C | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-42A | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-42A | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-42A | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_21A-42A | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| 42C | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-42A | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-42A | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-42A | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_21A-42A | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| 42C | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_41C | n41 | george cummings, Sprint | george.cummings@sprint.com | Ericsson, Qorvo, Nokia, Skyworks |  |
| CA_42C | n258 | Per Lindell, Ericsson | per.lindell@ericsson.com | ZTE, Huawei, MediaTek |  |
| CA_42A-42A | n258 | Per Lindell, Ericsson | per.lindell@ericsson.com | ZTE, Huawei, MediaTek |  |

Table-5 Progress on band combinations for LTE 3 CC + NR 1 CC

| Combination |  | contact name, company | contact email | other supporting companies (min. 3) | Status (After #85) |
| --- | --- | --- | --- | --- | --- |
| LTE band combinations (3CC) | NR1 (Frequency range/LTE bands) |  |  |  |  |
| CA_1A-3A-5A | n78 | Hongliang Bian, China Telecom | bianhl.bri@chinatelecom.cn | Huawei, Hisilicon, ZTE, Xiaomi | No further progress |
| CA_1A-3A-41A | n78 | Hongliang Bian, China Telecom | bianhl.bri@chinatelecom.cn | Huawei, Hisilicon, ZTE | No progress |
| CA_1A-5A-41A | n78 | Hongliang Bian, China Telecom | bianhl.bri@chinatelecom.cn | Huawei, Hisilicon, ZTE | No progress |
| CA_1A-3A-5A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_1A-3C | n78 | Naveen Khapali, BT plc | Naveen.khapali@ee.co.uk | Deutsche Telekom, Nokia, Huawei | No progress |
| CA_1A-3C | n50 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-3C | n51 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-3A-7A | n78 | Alper Ucar, Vodafone | alper.ucar@VODAFONE.COM | Orange, Deutsche Telekom, Telia, Qorvo, Etisalat | Ongoing |
| CA_1A-3A-7A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung, | completed |
| CA_1A-3A-7A | n50 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-3A-7A | n51 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-3A-7A | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| CA_1A-3A-20A | n78 | Alper Ucar, Vodafone | alper.ucar@VODAFONE.COM | Orange, Deutsche Telekom, Telia, Qorvo, Etisalat | Ongoing |
| CA_1A-3A-20A | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.comOrange | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| CA_1A-3A-20A | n50 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-3A-28A | n50 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-20A-28A | n50 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-7A-20A | n50 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_3A-7A-20A | n50 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_3A-7A-28A | n50 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-3A-20A | n51 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-3A-28A | n51 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-20A-28A | n51 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-7A-20A | n51 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_3A-7A-20A | n51 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_3A-7A-28A | n51 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-7A-20A | n78 | Alper Ucar, Vodafone | alper.ucar@VODAFONE.COM | Orange, Deutsche Telekom, Telia, Qorvo, Etisalat | Ongoing |
| CA_1A-7A-20A | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.comOrange | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| CA_3A-7A-20A | n78 | Alper Ucar, Vodafone | alper.ucar@VODAFONE.COM | Orange, Deutsche Telekom, Telia, Qorvo, Etisalat | Ongoing |
| CA_3A-7A-20A | 28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| CA_1A-3A-19A | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A-21A | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-19A-21A | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_3A-19A-21A | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A-19A | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A-21A | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-19A-21A | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_3A-19A-21A | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A-19A | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A-21A | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-19A-21A | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-41A-42A | n79 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-42C | n79 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_41A-42C | n79 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-41A-42A | n257 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-42C | n257 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_41A-42C | n257 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_3A-7C | n78 | Naveen Khapali, BT plc | Naveen.khapali@ee.co.uk | Deutsche Telekom, Nokia, Huawei, Telstra, Etisalat | No progress |
| CA_3A-19A-21A | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A-19A | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A-21A | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-19A-21A | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_3A-19A-21A | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_3A-5A-7A | n78 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_3A-5A-7A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_3A-7A-7A | n78 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_3A-7A-7A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_5A-7A-7A | n78 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_5A-7A-7A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_3C-20A | n78 | Alper Ucar, Vodafone | Alper.ucar@vodafone.com | Huawei, Ericsson, Nokia, Deutsche Telekom | No progress |
| CA_3C-7A | n78 | Alper Ucar, Vodafone | Alper.ucar@vodafone.com | Huawei, Ericsson, Nokia | No progress |
| CA_3A-7A-28A | n78 | Nick Tompson, Telstra | nick.tompson@team.telstra.com | Nokia, Ericsson, Samsung, Cohere Technologies, ZTE | No progress |
| CA_7C-28A | n78 | Nick Tompson, Telstra | nick.tompson@team.telstra.com | Nokia, Ericsson, Samsung, Cohere Technologies, ZTE | No progress |
| CA_7A-46C | n78 | Liu Bo, Huawei | Brod.liubo@huawei.com | Deutsche Telekom, T-Mobile, Telecom Italia | No progress |
| CA_46D | n78 | Liu Bo, Huawei | Brod.liubo@huawei.com | Deutsche Telekom, T-Mobile, Telecom Italia | No progress |
| CA_41D | 41 | george cummings, Sprint | george.cummings@sprint.com | Qorvo, Ericsson, SouthernLinc | Ongoing |
| CA_41C-41A | 41 | george cummings, Sprint | george.cummings@sprint.com | Ericsson, Qorvo, Nokia | Ongoing |
| CA_1A-18A-28A | n78 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-18A-28A | n77 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-5A-7A | n78 | Yunsung Kim, LG Uplus | kimy0011@lguplus.co.kr | LG Electronics, SK Telecom, Samsung | completed |
| CA_1A-5A-7A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_2A-66A-71A | n71 | Nelson Ueng, T-Mobile USA | neng-tsann.ueng@t-mobile.com | Ericsson, Samsung, Deutsche Telekom, Nokia, Qorvo, Skyworks | No progress |
| CA_3C-41A | n78 | Hu Rongyi, China Unicom | hury3@chinaunicom.cn | Huawei, Hisilicon, ZTE | No progress |
| CA_1A-3A-8A | n78 | Hu Rongyi, China Unicom | hury3@chinaunicom.cn | Huawei, Hisilicon, ZTE | No progress |
| CA_1A-3A-42A | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-21A-42A | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-21A-42A | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-42C | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-42C | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_21A-42C | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-42A | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-21A-42A | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-42C | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-42A | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-21A-42A | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-21A-42A | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-42C | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-42C | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_21A-42C | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-42A | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-21A-42A | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-42C | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-42A | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-21A-42A | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-21A-42A | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-42C | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-42C | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_21A-42C | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-42A | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-21A-42A | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-42C | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-42A | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-21A-42A | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-21A-42A | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-42C | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-42C | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_21A-42C | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-42A | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-21A-42A | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-42C | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_41D | 41 | george cummings, Sprint | george.cummings@sprint.com | Qorvo, Ericsson, SouthernLinc, Skyworks |  |
| CA_41C-41A | 41 | george cummings, Sprint | george.cummings@sprint.com | Ericsson, Qorvo, Nokia, Skyworks |  |
| 3A-7A-20A | n1A | Christoph Maeder, Swisscom | Christoph.Maeder@swisscom.com | Ericsson, HTC, Vodafone |  |
| 1A-7A-20A | n3A | Christoph Maeder, Swisscom | Christoph.Maeder@swisscom.com | Ericsson, HTC, Vodafone |  |
| CA_42D | n258 | Per Lindell, Ericsson | per.lindell@ericsson.com | ZTE, Huawei, MediaTek |  |
| CA_42A-42C | n258 | Per Lindell, Ericsson | per.lindell@ericsson.com | ZTE, Huawei, MediaTek |  |

Table-6 Progress on band combinations for LTE 4 CC + NR 1 CC

| Combination |  | contact name, company | contact email | other supporting companies (min. 3) | Status |
| --- | --- | --- | --- | --- | --- |
| LTE band combinations (4CC) | NR1, (Frequency range/LTE bands) |  |  |  |  |
| CA_1A-3A-7A-20A | n78 | Alper Ucar, Vodafone | alper.ucar@VODAFONE.COM | Orange, Deutsche Telekom, Telia, Qorvo | Ongoing |
| CA_1A-3A-19A-21A | n77 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A-19A-21A | n78 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A-19A-21A | n79 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | completed |
| CA_1A-3A-19A-21A | n257 | Suguru Okuyama, NTT DOCOMO | suguru.okuyama.zw@nttdocomo.com | Nokia, NEC, Fujitsu | Ongoing in #85 |
| CA_1A-3C-7A | n78 | Naveen Khapali, BT plc | Naveen.khapali@ee.co.uk | Deutsche Telekom, Nokia, Huawei | Ongoing in #85 |
| CA_3C-7C | n78 | Naveen Khapali, BT plc | Naveen.khapali@ee.co.uk | Deutsche Telekom, Nokia, Huawei | No progress |
| CA_3A-7C-28A | n78 | Nick Tompson, Telstra | nick.tompson@team.telstra.com | Nokia, Ericsson, Samsung, Cohere Technologies, ZTE | No progress |
| CA_41E | 41 | george cummings, Sprint | george.cummings@sprint.com | Ericsson, Qorvo, Nokia | Ongoing |
| CA_41A-41D | 41 | george cummings, Sprint | george.cummings@sprint.com | Ericsson, Qorvo, Nokia | Ongoing |
| CA_41C-41C | 41 | george cummings, Sprint | george.cummings@sprint.com | Ericsson, Qorvo, Nokia | Ongoing |
| CA_1A-41A-42C | n79 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_1A-41A-42C | n257 | Masaki Obara, KDDI | ms-obara@kddi.com | Huawei, Hisilicon, Sumitomo | No progress |
| CA_7A-46D | n78 | Liu Bo, Huawei | Brod.liubo@huawei.com | Deutsche Telekom, T-Mobile Telecom Italia | No progress |
| CA_46E | n78 | Liu Bo, Huawei | Brod.liubo@huawei.com | Deutsche Telekom, T-Mobile, Telecom Italia | No progress |
| CA_1A-3A-5A-7A | n78 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_3A-5A-7A-7A | n78 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_1A-3A-5A-7A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_3A-5A-7A-7A | n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson, Ericsson-LG, Samsung | completed |
| CA_1A-3A-7A-20A | n28 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | Ongoing |
| CA_1A-3A-7A-20A | n50 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-3A-20A-28A | n50 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-3A-7A-20A | n51 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-3A-20A-28A | n51 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| CA_1A-3A-5A-41A | n78 | Hongliang Bian, China Telecom | bianhl.bri@chinatelecom.cn | Huawei, Hisilicon, ZTE | No progress |
| CA_1A-3A-21A-42A | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-42C | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-21A-42C | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-21A-42C | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-21A-42A | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-42C | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-21A-42C | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-21A-42A | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-42C | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-21A-42C | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-21A-42C | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-21A-42A | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-42C | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-21A-42C | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-21A-42A | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-42C | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-21A-42C | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-21A-42C | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-21A-42A | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-42C | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-21A-42C | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-21A-42A | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-42C | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-21A-42C | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_3A-21A-42C | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-21A-42A | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-42C | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_19A-21A-42C | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_41E | 41 | george cummings, Sprint | george.cummings@sprint.com | Ericsson, Qorvo, Nokia, Skyworks |  |
| CA_41A-41D | 41 | george cummings, Sprint | george.cummings@sprint.com | Ericsson, Qorvo, Nokia, Skyworks |  |
| CA_41C-41C | 41 | george cummings, Sprint | george.cummings@sprint.com | Ericsson, Qorvo, Nokia, Skyworks |  |
| CA_42A-42D | n258 | Per Lindell, Ericsson | per.lindell@ericsson.com | ZTE, Huawei, MediaTek |  |
| CA_42C-42C | n258 | Per Lindell, Ericsson | per.lindell@ericsson.com | ZTE, Huawei, MediaTek |  |

Table-7 Progress on band combinations for LTE 5 CC + NR 1 CC

| Combination |  | contact name, company | contact email | other supporting companies (min. 3) | Status |
| --- | --- | --- | --- | --- | --- |
| NR1, (Frequency range/LTE bands) | NR1, (Frequency range/LTE bands) |  |  |  |  |
| CA_7A-46E | n78 | Liu Bo, Huawei | Brod.liubo@huawei.com | Deutsche Telekom, T-Mobile, Telecom Italia | No progress |
| CA_1A-3A-21A-42C | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-21A-42C | n77 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-21A-42C | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-21A-42C | n78 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-21A-42C | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-21A-42C | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-3A-21A-42C | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_1A-19A-21A-42C | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |

Table-8 Progress on band combinations for LTE 1 CC + NR 2 CC

| Combination |  | contact name, company | contact email | other supporting companies (min. 3) | Status |
| --- | --- | --- | --- | --- | --- |
| LTE bands | NR band combinations (2CC) |  |  |  |  |
| 1 | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia |  |
| 3 | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia |  |
| 7 | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia |  |
| 20 | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia |  |
| 20 | CA_n75/76-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Telecom Italia, Deutsche Telekom, Vodafone, TeliaCompany, Ericsson |  |
| 25 | n41C | George Cummings,, Sprint | george.cummings@sprint.com | Ericsson, Nokia, Qorvo |  |
| 26 | n41C | George Cummings, Sprint | george.cummings@sprint.com | Ericsson, Nokia, Qorvo |  |
| 41 | n41C | George Cummings, Sprint | george.cummings@sprint.com | Ericsson, Nokia, Qorvo |  |
| 3 | CA_n258-n258C | Nick Tompson, Telstra | nick.a.tompson@team.telstra.com | Ericsson, Nokia, [Samsung], Cohere Technologies |  |
| 7 | CA_n258-n258C | Nick Tompson, Telstra | nick.a.tompson@team.telstra.com | Ericsson, Nokia, [Samsung], Cohere Technologies |  |
| 28 | CA_n258-n258C | Nick Tompson, Telstra | nick.a.tompson@team.telstra.com | Ericsson, Nokia, [Samsung], Cohere Technologies |  |
| 28 | CA_n8-n78 | Nick Tompson, Telstra | nick.a.tompson@team.telstra.com | Ericsson, Nokia, [Samsung], Cohere Technologies |  |
| 28 | CA_n8-n258 | Nick Tompson, Telstra | nick.a.tompson@team.telstra.com | Ericsson, Nokia, [Samsung], Cohere Technologies |  |
| Band 1 | CA_n77-n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 1 | CA_n78-n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 1 | CA_n77-n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 1 | CA_n78-n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu, SK telecom |  |
| Band 1 | CA_n79-n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 3 | CA_n77-n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 3 | CA_n78-n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 3 | CA_n77-n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 3 | CA_n78-n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu, SK telecom |  |
| Band 3 | CA_n79-n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 19 | CA_n77-n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 19 | CA_n78-n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 19 | CA_n77-n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 19 | CA_n78-n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 19 | CA_n79-n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 21 | CA_n77-n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 21 | CA_n78-n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 21 | CA_n77-n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 21 | CA_n78-n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| Band 21 | CA_n79-n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| 25 | n41C | George Cummings,, Sprint | george.cummings@sprint.com | Ericsson, Nokia, Qorvo, Skyworks |  |
| 26 | n41C | George Cummings, Sprint | george.cummings@sprint.com | Ericsson, Nokia, Qorvo, Skyworks |  |
| 41 | n41C | George Cummings, Sprint | george.cummings@sprint.com | Ericsson, Nokia, Qorvo, Skyworks |  |
| Band 5 | CA_n78-n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson LG, Ericsson, LG Uplus |  |
| Band 7 | CA_n78-n257 | Joonyoung Shin, SK telecom | joon0.sin@sk.com | LG Electronics, Ericsson LG, Ericsson, LG Uplus |  |

Table-9 Progress on band combinations for LTE 2 CC + NR 2 CC

| Combination |  | contact name, company | contact email | other supporting companies (min. 3) | supported next level fallback modes, (in DL and UL) | status |
| --- | --- | --- | --- | --- | --- | --- |
| LTE band combinations (2CC) | NR band combinations (2CC) |  |  |  |  |  |
| CA_1A-3A | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | DC_1_n28-n78, DC_3_n28-n78, DC_1-3_n28, DC_1-3_n78 |  |
| CA_1A-7A | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | DC_1_n28-n78, DC_7_n28-n78, DC_1-7_n28, DC_1-7_n78 |  |
| CA_1A-20A | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | DC_1_n28-n78, DC_20_n28-n78, DC_1-20_n28, DC_1-20_n78 |  |
| CA_3A-7A | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | DC_3_n28-n78, DC_7_n28-n78, DC_3-7_n28, DC_3-7_n78 |  |
| CA_3A-20A | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | DC_3_n28-n78, DC_20_n28-n78, DC_3-20_n28, DC_3-20_n78 |  |
| CA_7A-20A | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia | DC_7_n28-n78, DC_20_n28-n78, DC_7-20_n28, DC_7-20_n78 |  |
| CA_41C | CA_n41C | George Cummings, Sprint | George.cummings@sprint.com | Ericsson, Nokia, Qorvo | DC_41C-n41C, DC_41A-n41C, DC_41C-n41 |  |
| CA_41C | CA_n41C | George Cummings, Sprint | George.cummings@sprint.com | Ericsson, Nokia, Qorvo, Skyworks | DC_41C-n41C, DC_41A-n41C, DC_41C-n41 |  |
| CA_7A-20A | CA_n1A-n3A | Christoph Maeder, Swisscom | Christoph.Maeder@swisscom.com | Ericsson, HTC, Vodafone | DC_7A-20A_n1A, DC_7A-20A_n3A, DC_7A_n1A-n3A, DC_20A-n1A-n3A |  |

Table-10 Progress on band combinations for LTE 3 CC + NR 2 CC

| Combination |  | contact name, company | contact email | other supporting companies (min. 3) | status |
| --- | --- | --- | --- | --- | --- |
| LTE band combinations (3CC) | NR band combinations (2CC) |  |  |  |  |
| CA_1A-3A-7A | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia |  |
| CA_1A-3A-20A | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia |  |
| CA_1A-7A-20A | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia |  |
| CA_3A-7A-20A | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia |  |
| CA_3A-7A-20A | CA_n1A-n78A | Christoph Maeder, Swisscom | Christoph.Maeder@swisscom.com | Ericsson, HTC, Vodafone |  |
| CA_3A-7A-20A | CA_n1A-n78A | Christoph Maeder, Swisscom | Christoph.Maeder@swisscom.com | Ericsson, HTC, Vodafone |  |
| CA_1A-7A-20A | CA_n3A-n78A | Christoph Maeder, Swisscom | Christoph.Maeder@swisscom.com | Ericsson, HTC, Vodafone |  |
| CA_1A-7A-20A | CA_n3A-n78A | Christoph Maeder, Swisscom | Christoph.Maeder@swisscom.com | Ericsson, HTC, Vodafone |  |

Table-11 Progress on band combinations for LTE 4 CC + NR 2 CC

| Combination |  | contact name, company | contact email | other supporting companies (min. 3) | status |
| --- | --- | --- | --- | --- | --- |
| LTE band combinations (4CC) | NR band combinations (2CC) |  |  |  |  |
| CA_1A-3A-7A-20A | CA_n28-n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Qorvo, Huawei, Nokia |  |

Table-12 Progress on band combinations for NR intra-band CA with xDL/1UL

| Combination | contact name, company | contact email | other supporting companies (min. 3) | Status |
| --- | --- | --- | --- | --- |
| NR1, (Frequency range/LTE bands) |  |  |  |  |
| n257, Non-contiguous | Nelson Ueng, T-Mobile USA | Nelson.ueng@T-Mobile.com | Samsung , Ericsson, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| n257, Contiguous | Ilwhan Kim, KT | Ilwhan.kim@kt.com | Samsung ,LG electronics, Intel | No progress |
| n260, Non-contiguous | Nelson Ueng, T-Mobile USA | Nelson.ueng@T-Mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| n41, Contiguous | George Cummings, Sprint | George.cummings@sprint.com | Ericsson, Nokia, Qorvo | No progress |
| CA_n258, Contiguous | Nick Tompson, Telstra | nick.a.tompson@team.telstra.com | Ericsson, Nokia, [Samsung], Cohere Technologies | No progress |
| CA_n260A- n260A-n260A-n260A-n260A-n260A-n260A-n260A, Non-contiguous | Marc Grant, AT&T | Marc.grant@att.com | Nokia, Ericsson, Qualcomm |  |
| n77 contiguous | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| n78 contiguous | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu, SK telecom |  |
| n79 contiguous | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| CA_n41C, Contiguous | George Cummings, Sprint | George.cummings@sprint.com | Ericsson, Nokia, Qorvo, Skyworks |  |

Table-13 Progress on band combinations for NR inter-band CA with 2DL/1UL

| Combination |  | contact name, company | contact email | other supporting companies (min. 3) | Status |
| --- | --- | --- | --- | --- | --- |
| NR1, (Frequency range/LTE bands) | NR2, (Frequency range/LTE bands) |  |  |  |  |
| 71 | n257 | Nelson Ueng, T-Mobile USA | Nelson.ueng@T-Mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | completed |
| 71 | n260 | Nelson Ueng, T-Mobile USA | Nelson.ueng@T-Mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| 66 | n257 | Nelson Ueng, T-Mobile USA | Nelson.ueng@T-Mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| 66 | n260 | Nelson Ueng, T-Mobile USA | Nelson.ueng@T-Mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| 2 | n257 | Nelson Ueng, T-Mobile USA | Nelson.ueng@T-Mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| 2 | n260 | Nelson Ueng, T-Mobile USA | Nelson.ueng@T-Mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| 66 | 71 | Nelson Ueng, T-Mobile USA | Nelson.ueng@T-Mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| 2 | 71 | Nelson Ueng, T-Mobile USA | Nelson.ueng@T-Mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | No progress |
| n28 | n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Ericsson, Deutsche Telekom, Telecom Italia, Qorvo, Huawei, Nokia | No progress |
| n75 | n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Telecom Italia, Deutsche Telekom, Vodafone, TeliaCompany, Ericsson | No progress |
| n76 | n78 | Atoosa Hatefi, Orange | atoosa.hatefi@orange.com | Telecom Italia, Deutsche Telekom, Vodafone, TeliaCompany, Ericsson | No progress |
| n50 | n78 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| n51 | n78 | Abdulhadi AbouAlmal Etisalat | aalmal@etisalat.ae | Huawei, Hisilicon, CATT | No progress |
| n8 | n78 | Nick Tompson, Telstra | nick.a.tompson@team.telstra.com | Ericsson, Nokia, [Samsung], Cohere Technologies | No progress |
| n8 | n258 | Nick Tompson, Telstra | nick.a.tompson@team.telstra.com | Ericsson, Nokia, [Samsung], Cohere Technologies | No progress |
| n77 | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| n78 | n79 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| n77 | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |
| n78 | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu, SK telecom |  |
| n79 | n257 | Naoto Iizasa, NTT DOCOMO | naoto.iizasa.kw@nttdocomo.com | Nokia, NEC, Fujitsu |  |

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"