3GPP TSG-RAN4 Meeting #85	R4-1712167

# Reno, Nevada, United States, 27th Nov 2017 - 1st Dec 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.106 | CR | 0053 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Band 72 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Andrew Wireless Systems GmbH, ArgoNET GmbH, Airbus DS SLC, 450Connect GmbH; Nokia Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE450_Europe_PPDR-Core |  |  |  |  |  | Date: |  |  | 2017-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Requirements are missing for operating band 72 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant requirements are introduced for operting band 72 operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No relevant requirements for operating band 72 operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5, 5.7.3, 9.1.1.1, 9.1.2.1.1, 9.2.2.1, 9.2.3.1, 11.2.1, 11.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.143 CR 0058 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 5.5	Operating bands

E-UTRA FDD is designed to operate in the operating bands defined in Table 5.5-1.

Table 5.5-1 E-UTRA operating bands

| E-UTRA operating band |  | Uplink (UL) operating band |  |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |  |
| 1 |  | 1920 MHz | – | 1980 MHz |  | 2110 MHz | – | 2170 MHz |  | FDD |  |
| 2 |  | 1850 MHz | – | 1910 MHz |  | 1930 MHz | – | 1990 MHz |  | FDD |  |
| 3 |  | 1710 MHz | – | 1785 MHz |  | 1805 MHz | – | 1880 MHz |  | FDD |  |
| 4 |  | 1710 MHz | – | 1755 MHz |  | 2110 MHz | – | 2155 MHz |  | FDD |  |
| 5 |  | 824 MHz | – | 849 MHz |  | 869 MHz | – | 894MHz |  | FDD |  |
| 6 (NOTE 1) |  | 830 MHz | – | 840 MHz |  | 875 MHz | – | 885 MHz |  | FDD |  |
| 7 |  | 2500 MHz | – | 2570 MHz |  | 2620 MHz | – | 2690 MHz |  | FDD |  |
| 8 |  | 880 MHz | – | 915 MHz |  | 925 MHz | – | 960 MHz |  | FDD |  |
| 9 |  | 1749.9 MHz | – | 1784.9 MHz |  | 1844.9 MHz | – | 1879.9 MHz |  | FDD |  |
| 10 |  | 1710 MHz | – | 1770 MHz |  | 2110 MHz | – | 2170 MHz |  | FDD |  |
| 11 |  | 1427.9 MHz | – | 1447.9 MHz |  | 1475.9 MHz | – | 1495.9 MHz |  | FDD |  |
| 12 |  | 698 MHz | – | 716 MHz |  | 728 MHz | – | 746 MHz |  | FDD |  |
| 13 |  | 777 MHz | – | 787 MHz |  | 746 MHz | – | 756 MHz |  | FDD |  |
| 14 |  | 788 MHz | – | 798 MHz |  | 758 MHz | – | 768 MHz |  | FDD |  |
| 15 |  | Reserved |  |  |  | Reserved |  |  |  |  |  |
| 16 |  | Reserved |  |  |  | Reserved |  |  |  |  |  |
| 17 |  | 704 MHz | – | 716 MHz |  | 734 MHz | – | 746 MHz |  | FDD |  |
| 18 |  | 815 MHz | – | 830 MHz |  | 860 MHz | – | 875 MHz |  | FDD |  |
| 19 |  | 830 MHz | – | 845 MHz |  | 875 MHz | – | 890 MHz |  | FDD |  |
| 20 |  | 832 MHz | – | 862 MHz |  | 791 MHz | – | 821 MHz |  | FDD |  |
| 21 |  | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
| 22 |  | 3410 MHz | – | 3490 MHz |  | 3510 MHz | – | 3590 MHz |  | FDD |  |
| 23 |  | 2000 MHz | – | 2020 MHz |  | 2180 MHz | – | 2200 MHz |  | FDD |  |
| 24 |  | 1626.5 MHz | – | 1660.5 MHz |  | 1525 MHz | – | 1559 MHz |  | FDD |  |
| 25 |  | 1850 MHz | – | 1915 MHz |  | 1930 MHz | – | 1995 MHz |  | FDD |  |
| 26 |  | 814 MHz | – | 849 MHz |  | 859 MHz | – | 894 MHz |  | FDD |  |
| 27 |  | 807 MHz | – | 824 MHz |  | 852 MHz | – | 869 MHz |  | FDD |  |
| 28 |  | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
| 29 |  | N/A |  |  |  | 717 MHz | – | 728 MHz |  | FDD |  |
| 30 |  | 2305 MHz | – | 2315 MHz |  | 2350 MHz | – | 2360 MHz |  | FDD |  |
| 31 |  | 452.5 MHz | – | 457.5 MHz |  | 462.5 MHz | – | 467.5 MHz |  | FDD |  |
| 32 |  | N/A |  |  |  | 1452 MHz | – | 1496 MHz |  | FDD |  |
| 72 |  | 451 MHz | – | 456 MHz |  | 461 MHz | – | 466 MHz |  | FDD |  |
| Note 1: Band 6 is not applicable. |  |  |  |  |  |  |  |  |  |  |  |

## --------------------------------------- NEXT SECTION ---------------------------

### 5.7.3	Carrier frequency and EARFCN

The carrier frequency in the uplink and downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 - 65535. The relation between EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL_low and NOffs-DL are given in table 5.7.3-1 and NDL is the downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL)

The relation between EARFCN and the carrier frequency in MHz for the uplink is given by the following equation where FUL_low and NOffs-UL are given in table 5.7.3-1 and NUL is the uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL)

Table 5.7.3-1 E-UTRA channel numbers

| E-UTRA operating band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low [MHz] | NOffs-DL | Range of NDL | FUL_low [MHz] | NOffs-UL | Range of NUL |
| 1 | 2110 | 0 | 0 – 599 | 1920 | 18000 | 18000 – 18599 |
| 2 | 1930 | 600 | 6001199 | 1850 | 18600 | 18600 – 19199 |
| 3 | 1805 | 1200 | 1200 – 1949 | 1710 | 19200 | 19200 – 19949 |
| 4 | 2110 | 1950 | 1950 – 2399 | 1710 | 19950 | 19950 – 20399 |
| 5 | 869 | 2400 | 2400 – 2649 | 824 | 20400 | 20400 – 20649 |
| 6 | 875 | 2650 | 2650 – 2749 | 830 | 20650 | 20650 – 20749 |
| 7 | 2620 | 2750 | 2750 – 3449 | 2500 | 20750 | 20750 – 21449 |
| 8 | 925 | 3450 | 3450 – 3799 | 880 | 21450 | 21450 – 21799 |
| 9 | 1844.9 | 3800 | 3800 – 4149 | 1749.9 | 21800 | 21800 – 22149 |
| 10 | 2110 | 4150 | 4150 – 4749 | 1710 | 22150 | 22150 – 22749 |
| 11 | 1475.9 | 4750 | 4750 – 4999 | 1427.9 | 22750 | 22750 – 22999 |
| 12 | 728 | 5000 | 5000 – 5179 | 698 | 23000 | 23000 – 23179 |
| 13 | 746 | 5180 | 5180 – 5279 | 777 | 23180 | 23180 – 23279 |
| 14 | 758 | 5280 | 5280 – 5379 | 788 | 23280 | 23280 – 23379 |
| 15 | Reserved |  |  | Reserved |  |  |
| 16 | Reserved |  |  | Reserved |  |  |
| 17 | 734 | 5730 | 5730 – 5849 | 704 | 23730 | 23730 – 23849 |
| 18 | 860 | 5850 | 5850 – 5999 | 815 | 23850 | 23850 – 23999 |
| 19 | 875 | 6000 | 6000 – 6149 | 830 | 24000 | 24000 – 24149 |
| 20 | 791 | 6150 | 6150 - 6449 | 832 | 24150 | 24150 - 24449 |
| 21 | 1495.9 | 6450 | 6450 – 6599 | 1447.9 | 24450 | 24450 – 24599 |
| 22 | 3510 | 6600 | 6600 – 7399 | 3410 | 24600 | 24600 – 25399 |
| 23 | 2180 | 7500 | 7500 – 7699 | 2000 | 25500 | 25500 – 25699 |
| 24 | 1525 | 7700 | 7700 – 8039 | 1626.5 | 25700 | 25700 – 26039 |
| 25 | 1930 | 8040 | 8040 – 8689 | 1850 | 26040 | 26040 – 26689 |
| 26 | 859 | 8690 | 8690 – 9039 | 814 | 26690 | 26690 - 27039 |
| 27 | 852 | 9040 | 9040 – 9209 | 807 | 27040 | 27040 – 27209 |
| 28 | 758 | 9210 | 9210 – 9659 | 703 | 27210 | 27210 – 27659 |
| 29 | 717 | 9660 | 9660 – 9769 | N/A |  |  |
| 30 | 2350 | 9770 | 9770 – 9869 | 2305 | 27660 | 27660 – 27759 |
| 31 | 462.5 | 9870 | 9870 – 9919 | 452.5 | 27760 | 27760 – 27809 |
| 32 | 1452 | 9920 | 9920 – 10359 | N/A |  |  |
| 72 | 461 | 68936 | 68936-68985 | 451 | 133472 | 133472-133521 |
| NOTE: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7, 15, 25, 50, 75 and 100 channel numbers at the lower operating band edge and the last 6, 14, 24, 49, 74 and 99 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 1.4, 3, 5, 10, 15 and 20 MHz respectively. |  |  |  |  |  |  |

## --------------------------------------- NEXT SECTION ---------------------------

#### 9.1.1.1	Minimum Requirements

This requirement applies to the uplink and downlink of the repeater, at maximum gain, and with the following input signals:

-	without E-UTRA input signal

-	with E-UTRA input signals in the pass band of the repeater, at levels that produce the maximum rated power output per channel

-	with 10 dB increased E-UTRA input signals in all channels in the pass band, compared to the input level producing the maximum rated output power.

For E-UTRA FDD repeater operating in Bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 26, 27, 28, 29, 31 and 72 emissions shall not exceed the maximum levels specified in Tables 9.1.1.1-1 and 9.1.1.1-2.

Table 9.1.1.1-1: General operating band unwanted emission limits for repeater pass band bandwidth lower than 5 MHz (E-UTRA bands <1GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measure-ment bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < BWPass band | BWMeas/2 f_offset < BWPass band + BWMeas/2 |  | 100 kHz |
| BWPass band f < 2*BWPass band | BWPass band + BWMeas/2 f_offset < 2* BWPass band + BWMeas/2 |  | 100 kHz |
| 2*BWPass band f fmax | 2* BWPass band + BWMeas/2 f_offset < f_offsetmax | -13 dBm | 100 kHz |
| Note: Frequencies and bandwidth are given in MHz. |  |  |  |

Table 9.1.1.1-2: General operating band unwanted emission limits for repeater pass band bandwidth 5 MHz and above (E-UTRA bands <1GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < 10 MHz | 5.05 MHz f_offset < 10.05 MHz | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -13 dBm | 100 kHz |
| Note: Frequencies and bandwidth are given in MHz. |  |  |  |

## --------------------------------------- NEXT SECTION ---------------------------

##### 9.1.2.1.1	Category B requirements (Option 1)

For E-UTRA FDD repeater operating in Bands 5, 6, 8, 12, 13, 14, 17, 20, 26, 27, 28, 29, 31 and 72 emissions shall not exceed the maximum levels specified in Tables 9.1.2.1.1-1 and 9.1.2.1.1-2:

Table 9.1.2.1.1-1: General operating band unwanted emission limits for repeater pass band bandwidth lower than 5 MHz (E-UTRA bands <1GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measure-ment bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < BWPass band | BWMeas/2 f_offset < BWPass band + BWMeas/2 |  | 100 kHz |
| BWPass band f < 2*BWPass band | BWPass band + BWMeas/2 f_offset < 2* BWPass band + BWMeas/2 |  | 100 kHz |
| 2*BWPass band f fmax | 2* BWPass band + BWMeas/2 f_offset < f_offsetmax | -16 dBm | 100 kHz |
| NOTE 1: Frequencies and bandwidth are given in MHz. NOTE 2: If the repeater input signal consists of E-UTRA signals with a channel bandwidth of 1.4 MHz placed so that the channel edge is less than 200 kHz from the pass band edge, the requirements in Table 9.1.2.1.1-3 supersedes Table 9.1.2.1.1-1 and Table 9.1.2.1.1-2 for applicable frequency offsets. NOTE 3: If the repeater input signal consists of E-UTRA signals with a channel bandwidth of 3 MHz placed so that the channel edge is less than 200 kHz from the pass band edge, the requirements in Table 9.1.2.1.1-4 supersedes Table 9.1.2.1.1-1 and Table 9.1.2.1.1-2 for applicable frequency offsets. |  |  |  |

Table 9.1.2.1.1-2: General operating band unwanted emission limits for repeater pass band bandwidth 5 MHz and above (E-UTRA bands <1GHz) for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 1) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < 10 MHz | 5.05 MHz f_offset < 10.05 MHz | -14 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -16 dBm | 100 kHz |
| NOTE 1: Frequencies and bandwidth are given in MHz. NOTE 2: If the repeater input signal consists of E-UTRA signals with a channel bandwidth of 1.4 MHz placed so that the channel edge is less than 200 kHz from the pass band edge, the requirements in Table 9.1.2.1.1-3 supersedes Table 9.1.2.1.1-1 and Table 9.1.2.1.1-2 for applicable frequency offsets. NOTE 3: If the repeater input signal consists of E-UTRA signals with a channel bandwidth of 3 MHz placed so that the channel edge is less than 200 kHz from the pass band edge, the requirements in Table 9.1.2.1.1-4 supersedes Table 9.1.2.1.1-1 and Table 9.1.2.1.1-2 for applicable frequency offsets. |  |  |  |

## --------------------------------------- NEXT SECTION ---------------------------

#### 9.2.2.1	Minimum requirement

Unless otherwise stated this requirement applies to the uplink and downlink of the repeater, at maximum gain.

The power of any spurious emission shall not exceed the limits of Table 9.2.2.1-1 for an E-UTRA Repeater where requirements for co-existence with the system listed in the first column apply.

Table 9.2.2.1-1: Spurious emissions limits for E-UTRA-FDD repeater in geographic coverage area of systems operating in other frequency bands

| System type operating in the same geographical area | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| GSM900 | 921 - 960 MHz | -57 dBm | 100 kHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 8. |
|  | 876 - 915 MHz | -61 dBm | 100 kHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 8, since it is already covered by the requirement in sub-clause 9.1.4 |
| DCS1800 | 1805 - 1880 MHz | -47 dBm | 100 kHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 3. |
|  | 1710 - 1785 MHz | -61 dBm | 100 kHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 3, since it is already covered by the requirement in sub-clause 9.1.4. |
| PCS1900 | 1930 - 1990 MHz | -47 dBm | 100 kHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25. |
|  | 1850 - 1910 MHz | -61 dBm | 100 kHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 9.1.4. |
| GSM850 or CDMA850 | 869 - 894 MHz | -57 dBm | 100 kHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 5 or 26. This requirement applies to E-UTRA Repeater operating in Band 27 for the frequency range 879-894 MHz. |
|  | 824 - 849 MHz | -61 dBm | 100 kHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5 or 26, since it is already covered by the requirement in sub-clause 9.1.4. For E-UTRA Repeater operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| UTRA FDD Band I or E-UTRA Band 1 | 2110 - 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 1 |
|  | 1920 - 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, since it is already covered by the requirement in sub-clause 9.1.4. |
| UTRA FDD Band II or E-UTRA Band 2 | 1930 - 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25. |
|  | 1850 - 1910 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 9.1.4 |
| UTRA FDD Band III or E-UTRA Band 3 | 1805 - 1880 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 3 or band 9. |
|  | 1710 - 1785 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 3, since it is already covered by the requirement in sub-clause 9.1.4. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 9 in the frequency Range from 1749,9 MHz to 1784,9 MHz, since it is already covered by the requirement in clause 9.1.4. |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 4 or band 10. |
|  | 1710 - 1755 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 4 or band 10, since it is already covered by the requirement in sub-clause 9.1.4. |
| UTRA FDD Band V or E-UTRA Band 5 | 869 – 894 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 5 or 26. This requirement applies for the frequency range 879-894 MHz to E-UTRA FDD repeater operating in Band 27. |
|  | 824 – 849 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5 or 26, since it is already covered by the requirement in sub-clause 9.1.4. For E-UTRA FDD repeater operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| UTRA FDD Band VI, XIX or E-UTRA Band 6, 18, 19 | 860 – 890 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 5, 6, 18, 19, 20, 26 or 27. |
|  | 815 – 830 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 18, since it is already covered by the requirement in sub-clause 9.1.4. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5, 6, 19, 20, 26 or 27. |
|  | 830 – 845 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 619, since it is already covered by the requirement in sub-clause 9.1.4. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5, 18, 20, 26 or 27. |
| UTRA FDD Band VII or E-UTRA Band 7 | 2620 - 2690 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 7. |
|  | 2500 - 2570 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 7, since it is already covered by the requirement in sub-clause 9.1.4. |
| UTRA FDD Band VIII or E-UTRA Band 8 | 925 – 960 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 8. |
|  | 880 – 915 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 8, since it is already covered by the requirement in sub-clause 9.1.4. |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 3 or band 9. |
|  | 1749.9 - 1784.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 3 or band 9, since it is already covered by the requirement in sub-clause 9.1.4. |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 - 2170 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 4 or band 10 |
|  | 1710 - 1770 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 10, since it is already covered by the requirement in sub-clause 9.1.4. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 4 in the frequency Range from 1710 MHz to 1755 MHz, since it is already covered by the requirement in clause 9.1.4. |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1475.9 - 1510.9 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 11, 21, or 32 |
|  | 1427.9 - 1447.9 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 11, since it is already covered by the requirement in sub-clause 9.1.4. For E-UTRA FDD repeater operating in band 32, this requirement applies for operation within 1475.9MHz and 1495.9MHz. |
|  | 1447.9 - 1462.9 MHz | -49 dBm | 1MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 21, since it is already covered by the requirement in sub-clause 9.1.4. For E-UTRA FDD repeater operating in band 32, this requirement applies for operation within 1475.9MHz and 1495.9MHz. |
| UTRA FDD Band XII or E-UTRA Band 12 | 728 - 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 12, 28 or 29. |
|  | 698 - 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 12 or 28, since it is already covered by the requirement in sub-clause 9.1.4. For E-UTRA BS operating in Band 29, it applies 1 MHz below the Band 29 downlink operating band. |
| UTRA FDD Band XIII or E-UTRA Band 13 | 746 - 756 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 13. |
|  | 777 - 787 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 13, since it is already covered by the requirement in sub-clause 9.1.4. |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 14. |
|  | 788 - 798 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 14, since it is already covered by the requirement in sub-clause 9.1.4. |
| E-UTRA Band 17 | 734 – 746 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 17. |
|  | 704 - 716 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 17, since it is already covered by the requirement in sub-clause 9.1.4. |
| UTRA FDD Band XX or E-UTRA Band 20 | 791 – 821 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 20. |
|  | 832 - 862 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 20, since it is already covered by the requirement in sub-clause 9.1.4. |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510-3590 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 22. |
|  | 3410 – 3490 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 22 since it is already covered by the requirement in sub-clause 9.1.4. |
| E-UTRA Band 23 | 2180 – 2200 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 23. |
|  | 2000 - 2020 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 23 since it is already covered by the requirement in sub-clause 9.1.4. This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25, where the limits are defined separately. |
|  | 2000 – 2010 MHz | -30 dBm | 1 MHz | This requirement only applies to to the uplink E-UTRA FDD Repeater operating in band 2 or band 25. This requirement applies starting 5 MHz above the band 25 downlink operating band. |
|  | 2010 – 2020 MHz | -49 dBm | 1 MHz |  |
| E-UTRA Band 24 | 1525 – 1559 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA Repeaters operating in band 24. |
|  | 1626.5 – 1660.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA Repeater operating in band 24, since it is already covered by the requirement in subclause 9.1.4. |
| UTRA FDD Band XXV or E-UTRA Band 25 | 1930 – 1995 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25. |
|  | 1850 – 1915 MHz | -49 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 25 since it is already covered by the requirement in sub-clause 9.1.4. For E-UTRA FDD Repeater operating in band 2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in sub-clause 9.1.4 |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 859 - 894 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 5 or 26. This requirement applies to E-UTRA FDD repeater operating in Band 27 for the frequency range 879-894 MHz. |
|  | 814 - 849 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 26, since it is already covered by the requirement in subclause 9.1.4. For E-UTRA FDD repater operating in Band 5, it applies for 814 MHz to 824 MHz, while the rest is covered in sub-clause 9.1.4. For E-UTRA FDD repeater operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |
| E-UTRA Band 27 | 852 - 869 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in Band 5, 26 or 27. |
|  | 807 - 824 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in Band 27, since it is already covered by the requirement in subclause 9.1.4. For E-UTRA FDD repeater operating in Band 26, it applies for 807 MHz to 814 MHz, while the rest is covered in sub-clause 9.1.4. This requirement also applies to E-UTRA FDD repeater operating in Band 28, starting 4 MHz above the Band 28 downlink operating band . |
| E-UTRA Band 28 | 758 - 803 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 28. . This requirement also applies to E-UTRA FDD repeater operating in Band 27, starting 4 MHz below the Band 27 uplink operating band . |
|  | 703 - 748 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 28, since it is already covered by the requirement in subclause 9.1.4. |
| E-UTRA Band 29 | 717 – 728 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in Band 29. |
| E-UTRA Band 30 | 2350 - 2360 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 30. |
|  | 2305 - 2315 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 30, since it is already covered by the requirement in subclause 9.1.4. |
| E-UTRA Band 31 | 462.5 – 467.5 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 31 or 72. |
|  | 452.5 – 457.5 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 31 or 72, since it is already covered by the requirement in subclause 9.1.4. |
| UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 – 1496 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 11, 21 or 32. |
| UTRA TDD in Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -52 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |
|  |  | -53 dBm | 100 kHz | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |
| UTRA TDD in Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | -52 dBm | 1 MHz |  |
| UTRA TDD in Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -52 dBm | 1 MHz | This requirement does not apply to the downlink of E-UTRA FDD Repeater operating in band 3 or band 9. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25. |
|  |  | -53 dBm | 100 kHz | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25. |
| UTRA TDD in Band b) or E-UTRA Band 36 | 1930 - 1990 MHz | -52 dBm | 1 MHz | This requirement does not apply to the downlink of E-UTRA FDD Repeater operating in band 2 or band 25. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. |
|  |  | -53 dBm | 100 kHz | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1. |
| UTRA TDD in Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
|  |  | -53 dBm | 100 kHz | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |
| UTRA TDD in Band d) or E-UTRA Band 38 | 2570 – 2620 MHz | -52 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 7. |
|  |  | -53 dBm | 100 kHz | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 7. |
| UTRA TDD Band f) or E-UTRA Band 39 | 1880 – 1920 MHz | -52 dBm | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |
|  |  | -53 dBm | 100 kHz | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |
| UTRA TDD Band e) or E-UTRA Band 40 | 2300 – 2400 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 30. |
| E-UTRA Band 41 | 2496 – 2690 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 7. |
| E-UTRA Band 42 | 3400 – 3600 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 22. |
| E-UTRA Band 43 | 3600 – 3800 MHz | -52 dBm | 1 MHz |  |
| E-UTRA Band 44 | 703 - 803 MHz | -52 dBm | 1 MHz | This is not applicable to E-UTRA FDD repeater operating in Band 28. |
| E-UTRA Band 72 | 461 - 466 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 31 or 72. |
|  | 451 - 456 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA FDD repeater operating in band 31 or 72, since it is already covered by the requirement in subclause 9.1.4. |

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-existence requirements in Table 9.2.2.1-1 do not apply for the 10 MHz frequency range immediately outside the repeaters operating band frequency range of an operating band (see Table 5.5-1). This is also the case when the repeaters operating band frequency range is adjacent to the band for the co-existence requirement in the Table 9.2.2.1-1. Emission limits for this excluded frequency range may also be covered by local or regional requirements.

NOTE 2:	The Table 9.2.2.1-1 assumes that two operating bands, where the frequency ranges in Table 5.5-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	The requirements of -53dBm/100kHz in Table 9.2.2.1-1 for the up link direction of the Repeater reflect what can be achieved with present state of the art technology and are based on a coupling loss of 73 dB between a Repeater and a UTRA TDD BS receiver.

NOTE 4:	The requirements of -53dBm/100kHz in Table 9.2.2.1-1 shall be reconsidered when the state of the art technology progresses.

## --------------------------------------- NEXT SECTION ---------------------------

#### 9.2.3.1	Minimum Requirements

Unless otherwise stated this requirement applies to the uplink and downlink of the repeater, at maximum gain.

The power of any spurious emission shall not exceed the limits of Table 9.2.3.1-1 for an E-UTRA FDD Repeater where requirements for co-location with a Base Station listed in the first column apply.

Table 9.2.3.1-1: Spurious emissions limits for E-UTRA-FDD Repeater co-located with Base Stations

| Type of co-located Base Station |  |  |  | Frequency range for co-location requirement |  |  |  | Maximum Level |  | Measurement Bandwidth |  |  |  |  | Note |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GSM900 |  |  |  | 876 - 915 MHz |  |  |  | -98 dBm |  | 100 kHz |  |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 8, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 75dB coupling loss between base station and the repeater UL transmit port. |  |  |  |  |
| DCS1800 |  |  |  | 1710 - 1785 MHz |  |  |  | -98 dBm |  | 100 kHz |  |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 3, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 75dB coupling loss between base station and the repeater UL transmit port. |  |  |  |  |
| PCS1900 |  |  |  | 1850 - 1910 MHz |  |  |  | -98 dBm |  |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 75dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| GSM850 or CDMA850 |  |  |  | 824 - 849 MHz |  |  |  | -98 dBm |  |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5 or 26, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 75dB coupling loss between base station and the repeater UL transmit port. For E-UTRA Repeater operating in Band 27, it applies 3 MHz below the Band 27 downlink operating band. |  |  |  |
| UTRA FDD Band I or E-UTRA Band 1 |  |  |  | 1920 - 1980 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band II or E-UTRA Band 2 |  |  |  | 1850 - 1910 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band III or E-UTRA Band 3 |  |  |  | 1710 - 1785 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 3, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 9 in the frequency Range from 1749,9 MHz to 1784,9 MHz, since it is already covered by the requirement in clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band IV or E-UTRA Band 4 |  |  |  | 1710 - 1755 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 4 or band 10, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band V or E-UTRA Band 5 |  |  |  | 824 - 849 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5 or 26, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band VI or XIX or E-UTRA Band 6, 18 or 19 |  |  |  | 815 - 830 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 18, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5, 6, 19, 20 26 or 27. |  |  |  |
|  |  |  |  | 830 - 845 MHz |  |  |  |  | -96 dBm |  |  | 100 kHz |  |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 6 or 19, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5, 18, 20, 26 or 27. |  |  |  |
| UTRA FDD Band VII or E-UTRA Band 7 |  |  |  | 2500 - 2570 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 7, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band VIII or E-UTRA Band 8 |  |  |  | 880 - 915 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 8, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band IX or E-UTRA Band 9 |  |  |  | 1749.9 - 1784.9 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 3 or band 9, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band X or E-UTRA Band 10 |  |  |  | 1710 - 1770 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 10, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 4 in the frequency Range from 1710 MHz to 1755 MHz, since it is already covered by the requirement in clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 |  |  |  | 1427.9 - 1447.9 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 11, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. This requirement applies only for operation between 1475.9 MHz and 1495.9 MHz for E-UTRA FDD repeater operating in band 32. |  |  |  |
|  |  |  |  | 1447.9 - 1462.9 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 21, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. This requirement applies only for operation between 1475.9 MHz and 1495.9 MHz for E-UTRA FDD repeater operating in band 32. |  |  |  |
| UTRA FDD Band XII or E-UTRA Band 12 |  |  |  | 698 - 716 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 12 or 28, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XIII or E-UTRA Band 13 |  |  |  | 777 - 787 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 13, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XIV or E-UTRA Band 14 |  |  |  | 788 - 798 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 14, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 17 |  |  |  | 704 - 716 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 17, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XX or E-UTRA Band 20 |  |  |  | 832 - 862 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 20, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XXII or E-UTRA Band 22 |  |  |  | 3410 - 3490 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 22, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 23 |  |  |  | 2000 - 2020 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 23, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 24 |  |  |  | 1626.5 – 1660.5 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 24, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XXV or E-UTRA Band 25 |  |  |  | 1850 - 1915 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 25, since it is already covered by the requirement in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. For the uplink of E-UTRA FDD Repeater operating in band 2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in sub-clause 9.1.4, but requires a 73dB coupling loss between base station and the repeater UL transmit port. |  |  |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 |  |  |  | 814 – 849 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the UL of E-UTRA FDD Repeater operating in band 26. The sub-clause 9.1.4 requirement applies, but requires a 73dB coupling loss between BS and the repeater UL transmit port. For E-UTRA FDD Repeater operating in band 5, it applies from 814 MHz to 824MHz, while the rest is covered in sub-clause 9.1.4, but requires a 73dB coupling loss between BS and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 27 |  |  |  | 807 – 824 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the UL of E-UTRA FDD Repeater operating in band 27. The sub-clause 9.1.4 requirement applies, but requires a 73dB coupling loss between BS and the repeater UL transmit port. For E-UTRA FDD Repeater operating in band 26, this requirement applies from 807 MHz to 814MHz, while the rest is covered in sub-clause 9.1.4, but requires a 73dB coupling loss between BS and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 28 |  |  |  | 703 – 748 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the UL of E-UTRA FDD Repeater operating in band 28. The sub-clause 9.1.4 requirement applies, but requires a 73dB coupling loss between BS and the repeater UL transmit port. |  |  |  |
| E-UTRA Band 30 |  |  |  | 2305 – 2315 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the UL of E-UTRA FDD Repeater operating in band 30. The sub-clause 9.1.4 requirement applies, but requires a 73dB coupling loss between BS and the repeater UL transmit port. |  |  |  |
| E.UTRA Band 31 |  |  |  | 452.5 – 457.5 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the UL of E-UTRA FDD Repeater operating in band 31 or 72. The sub-clause 9.1.4 requirement applies, but requires a 73dB coupling loss between BS and the repeater UL transmit port. |  |  |  |
| UTRA TDD in Band a) or E-UTRA Band 33 |  |  |  | 1900 - 1920 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |  |  |
|  |  |  |  |  |  |  |  |  | -53 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |  |  |
| UTRA TDD in Band a) or E-UTRA Band 34 |  |  |  | 2010 - 2025 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. |  |  |  |
|  |  |  |  |  |  |  |  |  | -83 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1. |  |  |  |
| UTRA TDD in Band b) or E-UTRA Band 35 |  |  |  | 1850 – 1910 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the downlink of E-UTRA FDD Repeater operating in band 3 or band 9. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25. |  |  |  |
|  |  |  |  |  |  |  |  |  | -53 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25. |  |  |  |
| UTRA TDD in Band b) or E-UTRA Band 36 |  |  |  | 1930 – 1990 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This is not applicable to the downlink of E-UTRA-FDD Repeater operating in band 2 or band 25. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. |  |  |  |
|  |  |  |  |  |  |  |  |  | -53 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1. |  |  |  |
| UTRA TDD in Band c) or E-UTRA Band 37 |  |  |  | 1910 - 1930 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |  |  |  |
|  |  |  |  |  |  |  |  |  | -53 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |  |  |
| UTRA TDD in Band d) or E-UTRA Band 38 |  |  |  | 2570 – 2620 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 7. |  |  |  |
|  |  |  |  |  |  |  |  |  | -53 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 7. |  |  |  |
| UTRA TDD Band f) or E-UTRA Band 39 |  |  |  | 1880 – 1920 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |  |  |
|  |  |  |  |  |  |  |  |  | -53 dBm |  |  |  | 100 kHz |  |  | This requirement is applied only to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |  |  |  |
| UTRA TDD Band e) or E-UTRA Band 40 |  |  |  | 2300 – 2400 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 30. |  |  |  |
| E-UTRA Band 41 |  |  |  | 2496 – 2690 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 7. |  |  |  |
| E-UTRA Band 42 |  |  |  | 3400 – 3600 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 22. |  |  |  |
| E-UTRA Band 43 |  |  |  | 3600 – 3800 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  |  |  |  |  |
| E-UTRA Band 44 |  |  |  | 703 – 803 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to E-UTRA FDD Repeater operating in band 28. |  |  |  |
| E-UTRA Band 72 |  |  |  | 451 – 456 MHz |  |  |  |  | -96 dBm |  |  |  | 100 kHz |  |  | This requirement does not apply to the UL of E-UTRA FDD Repeater operating in band 31 or 72. The sub-clause 9.1.4 requirement applies, but requires a 73dB coupling loss between BS and the repeater UL transmit port. |  |  |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-location requirements in Table 9.2.3.1-1 do not apply for the 10 MHz frequency range immediately outside the repeaters operating band frequency range of an operating band (see Table 5.5-1). This is also the case when the repeaters operating band frequency range is adjacent to the frequency range of the co-location requirement in the Table 9.2.3.1-1. The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB Repeater-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [5].

NOTE 2:	The Table 9.2.3.1-1 assumes that two operating bands, where the corresponding eNode B transmit and receive frequency ranges in Table 5.5-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	The requirements of -53dBm/100kHz in Table 9.2.3.1-1 for the up link direction of the Repeater reflect what can be achieved with present state of the art technology and are based on a coupling loss of 73 dB between a Repeater and a UTRA TDD BS receiver.

NOTE 4:	The requirements of -83dBm/100kHz in Table 9.2.3.1-1 for the up link direction of the Repeater reflect what can be achieved with present state of the art technology and are based on a coupling loss of 43 dB between a Repeater and a UTRA TDD BS receiver.

NOTE 5:	The requirements of -53dBm/100kHz and -83dBm/100kHz in Table 9.2.3.1-1shall be reconsidered when the state of the art technology progresses.

## --------------------------------------- NEXT SECTION ---------------------------

### 11.2.1	Minimum requirement

For the parameters specified in table 11.2.1-1, the power in the pass band shall not increase with more than 10 dB at the output of the repeater as measured in the centre of the pass band, compared to the level obtained without interfering signals applied.

The frequency separation between the two interfering signals shall be adjusted so that the lowest order intermodulation product is positioned in the centre of the pass band.

NOTE 1:	The lowest intermodulation products correspond to the 4th and 3rd order for the GSM 900 and DCS 1800 bands, respectively.

Table 11.2.1-1: Input intermodulation requirements for interfering signals in co-located other systems

| Co-located other systems | Frequency of interfering signals | Interfering Signal Levels | Type of signals | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- | --- |
| GSM900 | 921 - 960 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 8, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| DCS1800 | 1805 - 1880 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 3, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| PCS1900 | 1930 - 1990 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| GSM850 or CDMA850 | 869 - 894 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 5 or 26, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA-FDD Band I or E-UTRA Band 1 | 2110 - 2170 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 1, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA-FDD Band II or E-UTRA Band 2 | 1930 - 1990 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA-FDD Band III or E-UTRA Band 3 | 1805 - 1880 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 3 or band 9, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA-FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 4 or band 10, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA-FDD Band V or E-UTRA Band 5 | 869 - 894 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 5 or 26, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA-FDD Band VI,XIX or E-UTRA Band 6, 18, 19 | 860 - 890 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 6, 18, 19, or 26 since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA-FDD Band VII or E-UTRA Band 7 | 2620 - 2690 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 7, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA-FDD Band VIII or E-UTRA Band 8 | 925 - 960 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 8, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA-FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 3 or band 9, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA-FDD Band X or E-UTRA Band 10 | 2110 - 2170 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 4 or band 10, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA-FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1475.9 - 1510.9 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 11 or band 21, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. This requirement does not apply to E-UTRA FDD Repeater operating in band 32, between 1475.9 MHz and 1496 MHz, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA FDD Band XII or E-UTRA Band 12 | 728 - 746 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 12, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA FDD Band XIII or E-UTRA Band 13 | 746 - 756 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 13, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 14, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| E-UTRA Band 17 | 734 - 746 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 17, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA FDD Band XX or E-UTRA Band 20 | 791 - 821 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 20, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA Band XXII or E-UTRA Band 22 | 3510 - 3590 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 22, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| E-UTRA Band 23 | 2180 – 2200 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 23, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| E-UTRA Band 24 | 1525 - 1559 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 24, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA FDD Band XXV or E-UTRA Band 25 | 1930 – 1995 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between base station and the repeater DL receive port. |
| UTRA-FDD Band XXVI or E-UTRA Band 26 | 859 - 894 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 26, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between BS and the repeater DL receive port. For E-UTRA FDD Repeater operating in band 5, it applies from 859 MHz to 869 MHz, while the rest is covered in sub-clause 11.1, but requires a 86dB coupling loss between BS and the repeater DL receive port. |
| E-UTRA Band 27 | 852 - 869 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 27, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between BS and the repeater DL receive port. For E-UTRA FDD Repeater operating in band 26, it applies from 852 MHz to 859 MHz, while the rest is covered in sub-clause 11.1, but requires a 86dB coupling loss between BS and the repeater DL receive port. |
| E-UTRA Band 28 | 758 - 803 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 28, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between BS and the repeater DL receive port. |
| E-UTRA Band 29 | 717 - 728 MHz | +16 dBm | 2 CW carriers | 1 MHz |  |
| E-UTRA Band 30 | 2350 - 2360 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 30, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between BS and the repeater DL receive port. |
| E-UTRA Band 31 | 462.5 – 467.5 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 31 or band 72, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between BS and the repeater DL receive port. |
| UTRA-FDD Band XXXII or E-UTRA Band 32 | 1452 - 1496 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 11, 21 or 32, since it is already covered by the requirement in sub-clause 11. 1, but requires a 86dB coupling loss between BS and the repeater DL receive port. |
| UTRA TDD in Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |
| UTRA TDD in Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | +16 dBm | 2 CW carriers | 1 MHz |  |
| UTRA TDD in Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to the downlink of E-UTRA FDD Repeater operating in band 3 or band 9. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25. |
| UTRA TDD in Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to the downlink of E-UTRA FDD Repeater operating in band 2 or band 25. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. |
| UTRA TDD in Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD in Band d) or E-UTRA Band 38 | 2570 – 2620 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 7. |
| UTRA TDD Band f) or E-UTRA Band 39 | 1880 – 1920 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |
| UTRA TDD Band e) or E-UTRA Band 40 | 2300 – 2400 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 30. |
| E-UTRA Band 41 | 2496 – 2690 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 7. |
| E-UTRA Band 42 | 3400 – 3600 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 22. |
| E-UTRA Band 43 | 3600 – 3800 MHz | +16 dBm | 2 CW carriers | 1 MHz |  |
| E-UTRA Band 44 | 703 - 803 MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 28. |
| E-UTRA Band 72 | 461 - 466MHz | +16 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 31 or band 72, since it is already covered by the requirement in sub-clause 11.1, but requires a 86dB coupling loss between BS and the repeater DL receive port. |

NOTE 1:	The co-location requirements in Table 11.2.1-1 do not apply when the repeaters pass band frequency range is adjacent to the frequency range of the co-location requirement in the Table 11.2.1-1. The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB Repeater-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [5].

NOTE 2:	The Table 11.2.1-1 assumes that two operating bands, where the corresponding eNode B transmit and receive frequency ranges in Table 5.5-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

## --------------------------------------- NEXT SECTION ---------------------------

### 11.3.1	Minimum requirement

For the parameters specified in table 11.3.1-1, the power in the pass band shall not increase with more than 10 dB at the output of the repeater as measured in the centre of the pass band, compared to the level obtained without interfering signals applied.

The frequency separation between the two interfering signals shall be adjusted so that the lowest order intermodulation product is positioned in the centre of the pass band.

NOTE 1:	The lowest intermodulation products correspond to the 4th and 3rd order for the GSM 900 and DCS 1800 bands, respectively.

Table 11.3.1-1: Input intermodulation requirements for interfering signals in co-existing other systems

| Co-existence with other systems | Frequency of interfering signals | Interfering Signal Levels | Type of signals | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- | --- |
| GSM900 | 876 - 915 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 8, since it is already covered by the requirement in sub-clause 11.1. |
| DCS1800 | 1710 - 1785 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 3, since it is already covered by the requirement in sub-clause 11.1. |
| PCS1900 | 1850 - 1910 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 11.1. |
| GSM850 or CDMA850 | 824 - 849 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 5 or 26, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band I or E-UTRA Band 1 | 1920 - 1980 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 1, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band II or E-UTRA Band 2 | 1850 - 1910 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band III or E-UTRA Band 3 | 1710 - 1785 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 3 or band 9, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 4 or band 10, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band V or E-UTRA Band 5 | 824 - 849 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 5 or 26, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band VI or XIX or E-UTRA Band 6, 18 or 19 | 815 - 830 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 18, since it is already covered by the requirement in sub-clause 11.1. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5, 6, 19 or 20. |
|  | 830 - 845 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 6 or band 19, since it is already covered by the requirement in sub-clause 11.1. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 5, 18 or 20. |
| UTRA FDD Band VII or E-UTRA Band 7 | 2500 - 2570 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 7, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band VIII or E-UTRA Band 8 | 880 - 915 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 8, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 3 or band 9, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 4 or band 10, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band XI or XXI or E-UTRA Band 11 or 21 | 1427.9 - 1447.9 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 11, since it is already covered by the requirement in sub-clause 11.1. |
|  | 1447.9 - 1462.9 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 21, since it is already covered by the requirement in sub-clause 11.1. For E-UTRA FDD Repeater operating in band 32, it applies from 1447.9 MHz to 1452 MHz, while the rest is covered in sub-clause 11.1. |
| UTRA FDD Band XII or E-UTRA Band 12 | 698 - 716 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 12, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 13, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band XIV or E-UTRA Band 14 | 788 - 798 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 14, since it is already covered by the requirement in sub-clause 11.1. |
| E-UTRA Band 17 | 704 - 716 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 17, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band XX or E-UTRA Band 20 | 832 - 862 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 20, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3410 - 3490 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 22, since it is already covered by the requirement in sub-clause 11.1. |
| E-UTRA Band 23 | 2000 - 2020 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 23, since it is already covered by the requirement in sub-clause 11.1. |
| E-UTRA Band 24 | 1626.5 –1660.5 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 24, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA FDD Band XXV or E-UTRA Band 25 | 1850 - 1915 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 25, since it is already covered by the requirement in sub-clause 11.1. For E-UTRA FDD Repeater operating in band 2, it applies for 1910 MHz to 1915 MHz, while the rest is covered in sub-clause 11.1. |
| UTRA-FDD Band XXVI or E-UTRA Band 26 | 814 - 849 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 26, since it is already covered by the requirement in sub-clause 11.1. For E-UTRA FDD Repeater operating in band 5, it applies from 814MHz to 824 MHz, while the rest is covered in sub-clause 11.1. For E-UTRA FDD Repeater operating in band 27, it applies from 824MHz to 849 MHz, while the rest is covered in sub-clause 11.5.1 |
| E-UTRA Band 27 | 807 - 824 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 27, since it is already covered by the requirement in sub-clause 11.1. For E-UTRA FDD Repeater operating in band 26, this requirement applies from 807MHz to 814 MHz, while the rest is covered in sub-clause 11.1. |
| E-UTRA Band 28 | 703 - 748 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 28, since it is already covered by the requirement in sub-clause 11.1. |
| E-UTRA Band 30 | 2305 - 2315 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 30, since it is already covered by the requirement in sub-clause 11.1. |
| E-UTRA Band 31 | 452.5 – 457.5 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 31 or band 72, since it is already covered by the requirement in sub-clause 11.1. |
| UTRA TDD in Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |
| UTRA TDD in Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | -15 dBm | 2 CW carriers | 1 MHz |  |
| UTRA TDD in Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to the downlink of E-UTRA FDD Repeater operating in band 3 or band 9. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 2 or band 25. |
| UTRA TDD in Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to the downlink of E-UTRA FDD Repeater operating in band 2 or band 25. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. |
| UTRA TDD in Band c) or E-UTRA Band 37 | 1910 - 1930 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 2 or band 25. This requirement does not apply to the uplink of E-UTRA FDD Repeater operating in band 1. This unpaired band is defined in ITU-R M.1036, but is pending any future deployment. |
| UTRA TDD in Band d) or E-UTRA Band 38 | 2570 – 2620 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to the uplink E-UTRA FDD Repeater operating in band 7. |
| UTRA TDD Band f) or E-UTRA Band 39 | 1880 – 1920 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 1, band 2 or band 25. |
| UTRA TDD Band e) or E-UTRA Band 40 | 2300 – 2400 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 30. |
| E-UTRA Band 41 | 2496 – 2690 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 7. |
| E-UTRA Band 42 | 3400 – 3600 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 22. |
| E-UTRA Band 43 | 3600 – 3800 MHz | -15 dBm | 2 CW carriers | 1 MHz |  |
| E-UTRA Band 44 | 703 - 803 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 28. |
| E-UTRA Band 72 | 451 – 456 MHz | -15 dBm | 2 CW carriers | 1 MHz | This requirement does not apply to E-UTRA FDD Repeater operating in band 31 or band 72, since it is already covered by the requirement in sub-clause 11.1. |

NOTE 1:	The co-existence requirements in Table 11.3.1-1 do not apply when the repeaters pass band frequency range is adjacent to the frequency range of the co-existence requirement in the Table 11.3.1-1. The current state-of-the-art technology does not allow a single generic solution for co-existence.

NOTE 2:	The Table 11.3.1-1 assumes that two operating bands, where the frequency ranges in Table 5.5-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

## --------------------------------------- End of changes ---------------------------