3GPP TSG RAN WG1 Meeting #90bis 	R1-1717900

# Prague, Czechia, 9th – 13th, October 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.901 | CR | 0022 | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on 3D-InH channel model in TR 38.901 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_newRAT |  |  |  |  |  |  | Date: |  | 2017-10-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TR38.901 has an error in 3D-InH channel model specification. - In Table 7.4.1-1: Pathloss model of 3D-InH, the working distance range can not satisfy the topology distance to calculate interference in the indoor hotspot. Especially for 12TRPs in the network layout, the distance between UE and the farmost TRP is larger than 100m. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Extend the largest distance of d3D from 100m to 150m for the LOS state of 3D-InH in Table 7.4.1-1. - Extend the largest distance of d3D from 86m to 150m for the NLOS state of 3D-InH in Table 7.4.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Users of channel model do not know how to model the 3D-InH when the working distance is larger than 100m for the LOS state and 86m for the NLOS state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Sections 7.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# Text Proposal

# < Unchanged parts are omitted >

Table 7.4.1-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 6 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| RMa | LOS | , see note 5 |  | h = avg. building height W = avg. street width The applicability ranges: |
|  | NLOS | for |  |  |
| UMa | LOS | , see note 1 |  |  |
|  | NLOS | for |  | Explanations: see note 3 |
|  |  | Optional |  |  |
| UMi - Street Canyon | LOS | , see note 1 |  |  |
|  | NLOS | for |  | Explanations: see note 4 |
|  |  | Optional |  |  |
| InH - Office | LOS |  |  |  |
|  | NLOS |  |  |  |
|  |  | Optional |  |  |
| Note 1: Breakpoint distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and hE is the effective environment height. For UMi hE = 1.0m. For UMa hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. With C(d2D, hUT) given by , where . Note that hE depends on d2D and hUT and thus needs to be independently determined for every link between BS sites and UTs. A BS site may be a single BS or multiple co-located BSs. Note 2: The applicable frequency range of the PL formula in this table is 0.5 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified format and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |