3GPP TSG RAN Meeting #79																		RP-180211

Chennai, India, March 19 - 22, 2018

Agenda Item: 	9.2.1.2

Source:	Apple

Title:	Single UL transmission support for Intra-band LTE NR EN-DC deployment

Document for:		Decision

# 1	Introduction

A key enabler for early commercialization of 5G NR is the non-stand-alone (NSA) deployment, which is expected to be adopted in many of the initial deployment world-wide. The NSA mode, which is based on LTE-NR dual connectivity, requires UEs to support both LTE and NR uplink transmissions.

In EN-DC, based on whether UE is required to transmit LTE UL and NR UL simultaneously, UE can have two different UL operation modes

- Dual UL transmission: in this mode, UE is required to transmit LTE UL and NR UL simultaneously

- Single UL transmission: in this mode, at any given time, UE is required to transmit uplink in at most one RAT, i.e. either LTE or NR.

Compared to dual UL transmission, single UL transmission has many performance benefits especially for the UE in power limited scenario, i.e. cell edge.

- Single UL transmission solves the DL desense problem: as explained in [1] [2] [3], depending on the band combinations, sending two concurrent uplinks in two different frequency bands can produce undesirable intermodulation products that could fall right into the NR or LTE receiving bands, causing degradation of UE’s receiver sensitivity. This is especially problematic for band combinations for both LTE and NR bands below 6 GHz, the impacted receiver bands due to MSD can be virtually not usable in practice, and it creates extreme challenges in RF implementations.

- Single UL transmission removes the UE additional maximum transmit power backoff requirement i.e. A-MPR: As discussed in [4], simultaneous LTE and NR UL transmission can cause intermodulation products outside the desired transmission bandwidth, which can violate the regulatory emission requirement. To meet the regulatory emission requirement, UE will be required to apply additional maximum transmit power backoff, i.e. A-MPR. A-MPR as well as the requirement for sharing the power between LTE and NR can significantly impact the UL coverage.

- Single UL transmission ensures LTE coverage for UE not capable of dynamic power sharing: For EN-DC, total maximum transmit power budget has to be shared between LTE and NR in case of dual UL transmission. However, the initial NR UE may not support dynamic power sharing between LTE and NR, i.e. the UE cannot share power control information on per slot/subframe basis. As a result, maximum transmission power budget has to be split semi-statically between LTE and NR. For example, a 23dBm total maximum transmit power limit can be split such that both LTE and NR have 20dB maximum transmit power per RAT. Compared to single UL transmission where each RAT can transmit up to 23dBm, this reduces the coverage by 3dB.

In 3GPP RAN#77 [5], it was agreed to introduce the indication of whether UE supports 2 simultaneous UL transmission.

- Define in RAN4 specs for which band combinations and channel allocations (to the extent specified by RAN4) within that band combination the UE is allowed to indicate that it does not support 2 simultaneous UL tx (this is not implying the granularity of the signalling) - Signalling to be defined to support the 'red text' from RAN4 part of the single tx discussion (i.e. as in RP-172064): - UE capability indicates that the UE does not allow 2 simultaneous UL transmission for the RAN4 specified channel allocations in a given band combination. If the network chooses to operate the UE in a way that is not consistent with this capability indication then the UE behavior is not specified and the UE might not meet the performance criteria.

In 3GPP RAN1#90 in August [6], specific TDM patterns were agreed to enable the single UE transmission operation:

Agreements: - When the UE is configured with multiple UL carriers on different frequencies (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency), but the UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers - For LTE carrier, UE can be configured with - Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell - For scheduling/HARQ timing of LTE FDD carrier, DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell is applied - UE is allowed to transmit NR UL signals at least in the subframe(s) where LTE UL transmission is not allowed according to the DL-reference UL/DL configuration - FFS whether or not a UE-specific subframe offset for the DL-reference UL/DL configuration can be configured considering system resource utilization and potential spec impact - Case 2: Release 15 LTE-FDD HARQ timing - No impact on LTE RAN1 specifications - Note: it doesn’t necessarily imply that UE has to support both cases

In 3GPP RAN2#100 [7], a capability indication is agreed for UE to inform the NW whether it supports dual UL transmission per band combination.

Agreements - RAN2 to adopt per-BC capability bit for SUO. The bit applies to all fallback BCs that are defined in RAN4 as problematic BCs. The bit does not apply to fallback BCs that are defined in RAN4 as not problematic BCs. Hence the UE does not explicitly list the fallback BCs. - UE is only allowed to set the per-BC capability bit for those band combinations that are defined in RAN4 as problematic BCs

In 3GPP RAN4 NR-AH#180 [8], three band combinations are agreed in which UE is allowed to indicate to NW not supporting dual UL transmission.

EN-DC band; E-UTRA Band; NR Band; Single UL allowed. DC_1_n28; 1; n28; No. DC_1_n77; 1; n77; DC_1_n77. DC_1_n78; 1; n78; No. …. DC_1_n257; 1; n257; No. DC_3_n77; 3; n77; DC_3_n77. DC_3_n78; 3; n78; DC_3_n78. ….

Note that the current agreed three difficult band combinations in RAN4 are all for inter-band LTE NR EN-DC deployment. In this contribution, we discuss the intra-band LTE NR EN-DC deployment and the need for single UL transmission. We discuss two issues that arise from the intra-band EN-DC, and the need for single UL transmission

- Intra-band EN-DC in FDD band (e.g. band 71): With a single PA reference architecture, but to lower extent also dual PA architectures, even for intra-band continuous EN-DC deployment, intermodulation products created by simultaneous LTE and NR UL transmission can still significantly desense the DL reception because LTE and NR resource allocation may not be contiguous. Without single UL transmission, either large transmit power backoff is needed (more than 20dB), which significantly impacts the UL coverage, or Rx performance has to suffer. Single UL transmission solves the DL desense problem without requiring UE transmit power backoff, i.e. UL coverage impact

- Intra-band EN-DC in TDD or FDD bands (e.g. band 41): The simultaneous LTE and NR UL transmission can cause intermodulation that violates the emission requirement. To meet the emission requirement, without single UL transmission, large additional maximum transmit power backoff, A-MPR, is required which reduces the UL coverage significantly. Single UL transmission ensures that no A-MPR is needed for intra-band EN-DC, and therefore, ensures the UL coverage.

# 2	Single UL transmission for intra-band LTE NR EN-DC deployment in paired spectrum (FDD band, e.g. band71)

Contribution [9] provides the analysis of the power backoff requirement for intra-band EN-DC deployment in FDD band 71. The assumption is single PA and the channel allocation for NR and LTE are contiguous, however, also a dual PA architecture will generate these IM products due to reverse IMD.

Even though channel allocation for NR and LTE are contiguous, as illustrate in Fig. 1, NR and LTE resource allocation can still be non-contiguous. Fig.1 shows the narrow band resource allocation for both LTE and NR, i.e. single RB. Based on the resource allocation, it is easy to see that UE UL transmission is actually non-contiguous between LTE and NR. The intermodulation product between LTE and NR UL transmission can fall into the NR or LTE DL and cause large Rx reception desense. It was shown in the contribution that the worst backoff needed is on the order of 20dB. This means that UE may needs to reduce its maximum transmit power by 20dB in order to avoid the worst-case DL desense scenario. Clearly, UL coverage will be significantly impacted if UE applies 20dB maximum transmit power backoff.

Figure 1 Intra-band EN-DC in FDD band: DL Desense problem due to intermodulation

Single UL transmission can solve the problem by allowing UE to transmit in at most one RAT, either LTE or NR, at any given time. Therefore, we have the following proposal

Proposal 1: For intra-band LTE-NR EN-DC in FDD bands (e.g. band 71), allow UE to operate in single UL transmission mode in order to avoid DL desense problem without impacting UL coverage

# 3	Single UL transmission for intra-band LTE NR EN-DC deployment in unpaired spectrum (TDD band, e.g. band41, but also FDD bands)

Contribution [4] provides the analysis of the A-MPR requirement for intra-band EN-DC in a TDD band (e.g. band 41). Similar as discussion in the previous section, Fig.1 illustrates the narrow band resource allocation for both LTE and NR, i.e. single RB. The intermodulation products between LTE and NR UL transmissions can cause (spurious) emissions. This is not limited to TDD bands, also for FDD bands the intra-band general emissions limits can be violated and require power reduction. In order to meet the regulatory emission requirement, UE maximum transmit power has to be reduced by A-MPR. Analysis shows that, even with 2 PA architectures, A-MPR can be more than 10dB.

Figure 2 Intra-band EN-DC in TDD band: Emission due to IMD

Additionally, simultaneous LTE and NR UL transmission, especially for the UEs that do not support dynamic power showing, requires the total maximum transmit power to be split between LTE and NR. As a result, the maximum transmit power is further reduced for LTE and NR, in addition to A-MPR. Combing these two factors, UL coverage can be significantly reduced for LTE and NR.

Single UL transmission removes both link budget impact factors: i.e. it does not require A-MPR and it gives all the transmit power to each RAT. However, we still need to consider details of the single UL operation for TDD band. Below we list a few considerations

- When LTE and NR are deployed in the same band, it is very nature for them to be deployed synchronously in terms of TDD pattern, i.e. deployment uses one of the 7 LTE TDD configurations

- Given the LTE TDD pattern, if we do not want to limit the LTE DL scheduling, then we need to guarantee the PUCCH for DL ACK/NAK feedback. Therefore, all the U subframes in TDD configurations need to be used for LTE, unless new HARQ timing is introduced in LTE for DL.

- As all the U subframes are used for LTE, NR could use the special subframe for uplink. NR could transmit its uplink in either GAP or UpPTS or combination of both. NR supports very flexible PUCCH configurations, including short PUCCH (1-2 symbols) and long PUCCH (4-14 symbols). In addition, scalable numerology can be used as well.

Compared to 14 symbols LTE PUCCH, NR PUCCH with less than 14 symbols may suffer coverage loss due to short time duration, we provide some quantitative analysis below. Table 1 lists the assumptions for link budget analysis for single and dual UL transmission. Note that we assume A-MPR of 9dB for dual UL transmission with two PA architectures

Table 1 Link budget analysis for single and dual UL transmission

|  | 2PA, Dual Tx | Single Tx |
| --- | --- | --- |
| AMPR | 9dB | 0dB |
| LTE Effective Tx Power (Link Budget) | 20 - 9 = 11 | 23 |
| NR Effective Tx Power (Link Budget) | 20 - 9 = 11 | 23 + 10*log10(#NR_PUCCH Symbol/14) |

Figure 4 illustrates the link budget analysis of dual UL transmission and single UL transmission, for both LTE and NR. Figure 4 clearly demonstrates the link budget improvement from single UL transmission, especially LTE RAT.

Figure 4 Link budget comparison between dual and single UL transmission (top: LTE, bottom NR)

Based on the above discussion and analysis, it is clear that, compared to dual UL transmission, single UL transmission has benefit in terms of link budget. On the other hand, when UE is closer to the cell, even with large A-MPR, UE performance may not be impacted since UE anyway does not transmit at its maximum power. In our view, single RF architecture can be used to support this deployment, i.e. single Tx transmission can be used to ensure coverage at cell edge while single RF dual Tx transmission can be used when UE is closer enough to the cell. We have the following proposal

Proposal 2: For intra-band LTE-NR EN-DC (e.g. band 41), allow UE to operate in single UL transmission mode in order to meet emission requirement without requiring UE to additionally backoff its transmit power, i.e. A-MPR

# 4	Conclusion

In this contribution, we discuss the intra-band LTE NR EN-DC deployment and the need for single UL transmission. We discuss two issues that arise from the intra-band EN-DC, and the need for single UL transmission

- Intra-band EN-DC in FDD band (e.g. band 71): With a single PA reference architecture, even for intra-band continuous EN-DC deployment, intermodulation products created by simultaneously LTE and NR UL transmission can still significantly desense the DL reception because LTE and NR resource allocations may not be contiguous. Without single UL transmission, either large transmit power backoff is needed (more than 20dB), which significantly impacts the UL coverage, or Rx performance has to suffer. Single UL transmission solves the DL desense problem without requiring UE transmit power backoff, i.e. UL coverage impact

- Intra-band EN-DC in FDD or TDD bands (e.g. band 41): The simultaneous LTE and NR UL transmission can cause intermodulation that violates the emission requirement. To meet the emission requirement, without single UL transmission, large additional maximum transmit power backoff, A-MPR, is required which reduces the UL coverage significantly. Single UL transmission ensures that no A-MPR is needed for intra-band EN-DC, and therefore, ensures the UL coverage.

Based on the above discussion, we have the following proposals

Proposal 1: For intra-band LTE-NR EN-DC in FDD bands (e.g. band 71), allow UE to operate in single UL transmission mode in order to avoid DL desense problem without impacting UL coverage

Proposal 2: For intra-band LTE-NR EN-DC (e.g. band 41), allow UE to operate in single UL transmission mode in order to meet emission requirement without requiring UE to additionally backoff its transmit power, i.e. A-MPR

# 5	Reference

- R1-1708276, “Uplink sharing in NSA mode”, 3GPP RAN1#89, Hangzhou, China, May 15th-19th, 2017.

- R4-1704759, “Co-existence analysis on LTE-NR band combinations”, 3GPP RAN4 #83, Hangzhou, China, May 15th-19th, 2017.

- R4-1703999, “European LTE-NR band combinations, and coexistence analysis”, RAN4#82bis, Spokane, US, April 3-7, 2017

- R4-1803062, “A-MPR for DC_41A_n41A”, RAN4#86, Athens, Greece, Feb. 26-March 2, 2017

- RP-172085, “Signalling for Single/Dual UL Tx”, RAN#77, Sapporo, Japan, Sept 11-15, 2017

- Chairman Note, RAN1#90, Prague, Czech Rep, 9th – 13th October 2017

- Chairman Note, RAN2#100, Reno, Nevada, USA Nov. 27 – Dec. 1, 2017

- R4-1800404, “38.101-3 Implementation of agreed draft CRs”, RAN4 NR-AH#1801, San Diego, CA, Jan. 18-21, 2018

- R4-1802243, “Measurements of power backoff requirements for DC_(n)71B with single PA”, Athens, Greece, Feb. 26th ‒ March 2nd, 2018