3GPP TSG RAN Meeting #79	RP-180401

Chennai, India, March 19 - 22, 2018

Source:	vivo

Title:	Interference avoidance for in-device coexistence

Agenda Item:	9.2.1.2

Document for:	Discussion and Decision

# Introduction

In LTE, the scenarios of interference of in-device coexistence can be found in 36.816 [1]. The solutions to mitigate the interference of in-device coexistence (IDC) are specified as given in 36.331 [2] and 36.300 [3]. During the discussion of EN-DC, some issues on in-device coexistence (e.g. harmonic/inter-modulation frequency interference) are founded, and the TDM solutions which are specified can be found in [4]. According to the LS from RAN4 [6], “RAN4 has confirmed that IDC problems not only for other systems already identified in LTE CA but also for mmWave ISM bands are expected with EN-DC”. In this paper, we represent the potential interference scenarios of in-device coexistence in NR, and give our initial considerations on the challenges brought by the potential interference scenarios.

# Discussion

## Potential interference scenarios

Figure 1: Coexistence interference within the same UE

The NR UE could have the interference on adjacent frequencies or sub-harmonic frequencies from/to NR frequencies to/from BT (Blue Tooth)/WiFi/GNSS (Global Navigation Satellite System). The potential interference scenarios are also illustrated in Figure 1.

Observation 1: The UE could have the following interference scenarios of in-device coexistence in NR:

- Interference from/to NR to/from other non-3GPP RAT (e.g. BT/WiFi/GNSS)

According to the previous test data, the interference of in-device coexistence includes diverse wave types, e.g. side-lobe, harmonic (mostly second/third harmonic order) and intermodulation (e.g. LTE+WiFi inter-modulation). Thus, the potential victim frequency range are given in Annex A.

Observation 2: The potential victim frequency range(s) as given in Annex A include both FR1 and FR2.

## Challenges of in-device coexistence

According to the LS from RAN4 [6], RAN4’s preference is to reuse the framework of the LTE IDC solutions. Regarding the LTE IDC solutions [2] [3], the UE encountering IDC issue can report some assistance information of TDM and/or FDM via RRC message, and the UE is also allowed to autonomously deny LTE uplink transmission according to the network configuration.  Then the solutions can be divided into the following two categories:

- Cat.1 (FDM): UE reporting of the victim frequency and system

- Cat.2 (TDM)

  - Cat.2.1: UE reporting for the desired DRX/subframe pattern

  - Cat.2.2: Allowed subframe pattern in which the UE is allowed to deny any UL transmission

According to the analysis given in [7] [8], to re-use the framework of the LTE IDC solutions, some further analysis/discussion/modifications are required for both FDM and TDM of the IDC solutions. For example, in order to allow the UE reporting for the NR band, the NR should be included as a victim system, and the reporting of the NR frequency should be allowed. For the TDM solutions, due to the flexible numerology (e.g. flexible time range of slot/OFDM symbol) configured for the UE, more discussions are required on how to allow the TDM solutions.

Observation 3: Some further analysis/discussion/modifications for both FDM and TDM solutions are required, in order to reuse the LTD IDC solution framework.

According the discussion in NR, a UE could be configured with several different BWPs of small bandwidth (e.g. 20MHz) in a cell of wide bandwidth (e.g. 400MHz), and a UE could also be equipped with several antenna panels. Thus the UE may have different interference in different BWPs or different antenna panels, as shown in Figure 1. Furthermore the SUL frequency could also bring some challenges in the in-device coexistence. The current interference mitigation solutions (i.e. TDM) [4] for EN-DC only considers the tight inter-working architecture (i.e. DC) between LTE and NR. According to the LS [5] from SA2, the UE could have dual registrations to both NR and LTE, without any coordination between RAN nodes. More information related to the challenges and scenarios of IDC can also be found in [9]. Then the diverse protocol architectures and various new technologies in NR could bring more challenges to the IDC solutions (e.g. the signaling procedures for UE reporting and for MN and SN coordination).

Observation 4: The following diverse protocol architectures and various new technologies in NR can also impact the IDC solutions:

- Tight interworking architecture (e.g. DC) between LTE and NR, and amongst NR frequencies

- Loose interworking architecture (e.g. dual registration) between LTE and NR

- Multiple BWPs

- Flexible numerologies

- Multiple antenna panels

- SUL frequency

Due to the limited remaining time in Rel-15, we consider that the IDC issue of EN-DC as identify by RAN4 [6] can be studied in RAN2, and a simplified FDM solution as given by [7] [8] can be specified for the EN-DC in Rel-15. More enhancements can be considered in the future release.

Proposal: To specify a simplified solution (i.e. FDM) for the IDC issue of EN-DC in Rel-15.

# Conclusion

According to the analysis given above, we have the following observations and proposal for the IDC issue in NR Rel-15:

Observation 1: The UE could have the following interference scenarios of in-device coexistence in NR:

- Interference from/to NR to/from other non-3GPP RAT (e.g. BT/WiFi/GNSS)

Observation 2: The potential victim/attacking frequency range(s) as given in Annex A include both FR1 and FR2.

Observation 3: Some further analysis/discussion/modifications for both FDM and TDM solutions are required, in order to reuse the LTD IDC solution framework.

Observation 4: The following diverse protocol architectures and various new technologies in NR can also impact the IDC solutions:

- Tight interworking architecture (e.g. DC) between LTE and NR, and amongst NR frequencies

- Loose interworking architecture (e.g. dual registration) between LTE and NR

- Multiple BWPs

- Flexible numerologies

- Multiple antenna panels

- SUL frequency

Proposal: To specify a simplified solution (i.e. FDM) for the IDC issue of EN-DC in Rel-15.

# Reference

- 3GPP TR 38.816, “Study on signalling and procedure for interference avoidance for in-device coexistence”

- 3GPP TS 36.331, “Radio Resource Control (RRC); Protocol specification”

- 3GPP TS 36.300, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2”

- R2-1713684, TDM pattern coordination for 1Tx and Harmonic scenario	vivo

- S2-176689, LSin, From SA2, To RAN1, RAN2, RAN4, “LS on simultaneous transmission and/or reception over EPC/E-UTRAN and 5GC/NR”.

- R4-1803254, LSout, From RAN4, To RAN2, “LS reply on In-Device Coexistence solution for EN-DC”.

- R2-1802609, IDC indication for EN-DC, Qualcomm Incorporated	discussion

- R2-1802101, Discussion on the IDC report of NR frequency in EN-DC, vivo	discussion

- R2-1802100, Challenges for in-device coexistence in NR, vivo, Potevio, CATR, Xiaomi, Spreadtrum Communications, China Telecom, TCL		discussion

- 3GPP TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

- 3GPP TS 38.101-2, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

# Annex A

The following table gives the potential victim frequency range between NR band(s) and other systems:

| Victim Systems | Frequency range [MHz] |  |  | Regions | Side-lobe | NR second harmonic order | NR third harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  | n50(UL)/n74(UL) |  |  |
| Galileo | 1559 | - | 1591 |  | n50(UL)/n74(UL) |  |  |
| GLONASS | 1591 | - | 1610 |  | n50(UL)/n70(UL) |  |  |
| GPS | 1563 | - | 1587 |  | n50(UL)/n74(UL) |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | US/Europe | n7/n38/n41 |  | n5/n20/n28/n82 |
|  | 2400 | - | 2494 | Asia | n7/n38/n41 |  | n5/n20/n28/n82 |
| ISM band (5GHz) | 5150 | - | 5925 | US | n79 | n7/n38/n41 | n1/n2/n3/n66/n80/n84 |
|  | 5150 | - | 5350 | Europe | n79 | n7/n38/n41 | n1/n2/n3/n66/n80/n84 |
|  | 5470 | - | 5725 |  |  |  |  |
|  | 5150 | - | 5825 | Asia | n79 | n7/n38/n41 | n1/n2/n3/n66/n80/n84 |
| ISM band (45GHz) | 42300 | - | 47000 | China |  | n258 |  |
|  | 47200 | - | 48400 |  |  |  |  |
| ISM band (60GHz) | 57000 | - | 66000 | Europe |  | n257 |  |
|  | 57050 | - | 64000 | USA - Canada |  | n257 |  |
|  | 57000 | - | 64000 | South Korea |  | n257 |  |
|  | 59000 | - | 66000 | Japan |  | n257 |  |
|  | 59000 | - | 64000 | China |  | n257 |  |
|  | 59400 | - | 62900 | Australia |  | n257 |  |

# Annex B

The following table for FR1 is extracted from 38.101-1 [10]:

Table 5.2-1: NR operating bands in FR1

| NR Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n50 | 1432 MHz | – | 1517 MHz | 1432 MHz | – | 1517 MHz | TDD |
| n51 | 1427 MHz | – | 1432 MHz | 1427 MHz | – | 1432 MHz | TDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
| n70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| n74 | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz | FDD |
| n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
| n76 | N/A |  |  | 1427 MHz | – | 1432 MHz | SDL |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
| n80 | 1710 MHz | – | 1785 MHz | N/A |  |  | SUL |
| n81 | 880 MHz | – | 915 MHz | N/A |  |  | SUL |
| n82 | 832 MHz | – | 862 MHz | N/A |  |  | SUL |
| n83 | 703 MHz | – | 748 MHz | N/A |  |  | SUL |
| n84 | 1920 MHz | – | 1980 MHz | N/A |  |  | SUL |

# Annex C

The following table for FR2 is extracted from 38.101-2 [11]:

Table 5.2-1: NR operating bands in FR2

| NR Operating Band |  | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz |  | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz |  | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n260 | 37000 MHz |  | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |