3GPP TSG-RAN #79	RP-180535

Chennai, India, 19 – 22 March 2018

Agenda item:	9.2.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Way forward on secured signalling only connection

Document for:	Discussion and Decision

# 1	Introduction

In RP-180509 stated some views on secured signalling only connection. This paper provides further discussion and proposes the way forward.

# 2	Description

RP-180509 argues SA3 LS, and concluded the following.

Conclusion 1: There is no confirmed requirement on securing signalling connections, as stated in S3-180986

For this part, as already clarified in RP-180429, SA3 has just sent questions about these scenarios and we expect SA2 to clarify them as per the annex. The SA3 LS should not be interpreted as challenging SA2 work. Furthermore, SA2 specifications require support for “signalling only connection” and this is the default mode supported as 5GS Registration procedure does not support piggy backing of PDU session request to establish user plane connections. During SA3 Conference call (eMeeting held recently), they have also agreed to LS out in S3c0011 that refers to 2 solution options:

- Solution 1: Calculate a NAS key from the NAS context and further calculate an integrity key at the eNB using this key to protect the redirection parameters. The solution is described in the attachment S3-17c0006.

- Solution 2: Establish regular AS context between the UE and the eNB and then issue the secured redirect command to the UE.

Depending on their agreement, we believe that solution should be adopted also for 5GS to avoid insecure redirection. Therefore, conclusion 1 above is not justified.

RP-180509 argues to use Initial RRC Message Transfer instead of UE Context Setup Request.

Conclusion 2: The F1: UE Context Setup Request is not needed to transfer SRB1 configuration from gNB-DU to gNB-CU. Instead, the F1: Initial RRC Message Transfer can be used

This has not been discussed in RAN3 at all. There are many open issues to use Initial RRC message transfer in this manner. Secured siginalling connection is different from legacy S1 messages like NAS message transfer. This is a “connection” and using RRC message transfer is not justified. In addition, there is no confirmed way to release such connection without using UE Context Setup procedure.

Therefore, conclusion 2 above is not justified.

Conclusion 3: there is no need to run the F1: UE context Setup procedures to secure SRB1 communication. This can be done by means of RRC Security Mode procedures and F1: RRC UL/DL Message Transfer procedures

[…]

It should be pointed out that the functions assigned to SRB2 (signalling RRC messages including logged measurement information, signalling NAS messages) are supported by SRB1. Namely, there is nothing that SRB2 does, which SRB1 cannot do.

Therefore, there is no use case that can be identified for the establishment of SRB2 for signalling connections.

Conclusion 4: There is no scenario that can be foreseen for establishment of signalling connections. This is proven by the fact that in LTE NB-IoT, Data over NAS (the most message-intense form of signalling connection) does not use SRB2.

RAN3 agreed to convey Security Mode over UE Context Setup procedure as well. Therefore, the argument on Conclusion 3 is not relevant. Furthermore, if we argue SRB1 can do everything, then this will break the general principle, i.e., what is the purpose to have multiple bearer type in the first place. We can do everything over only one bearer type and we don’t need SRB2 (even other type) at all. There are certain usecase to use SRB2. Therefore, it must allow to use SRB2 once AS security is activated. For example,

a) SRB2 is conventionally used for NAS signaling and SRB1 for RAN signaling. There is no need to break this.

b) If the UE has to send/receive SMS, it should be done over SRB2, not SRB1. There is no need to break this.

There was also argument about SRB2 in RAN3 reflector. SRB2 is for RRC messages which include logged measurement information as well as for NAS messages, all using DCCH logical channel. SRB2 has a lower-priority than SRB1 and is always configured by E-UTRAN after security activation. SRB2 is not applicable for NB-IoT.

Therefore, if AMF allows to send Initial UE Context Setup Request message to NG-RAN without PDU session, but included the Security information, then the CU shall trigger the UE Context Setup Request message towards the DU in order to finalize the AS security activation and SRB2 establishment. This should be ensured according to RRC specification. There is no reason to have limitation in the network side that SRB2 is not allowed to be established after security activation. Furthermore, there has the case for registered UE without UP data still needs to perform mobility based on measurement report after AS security activation, which is different from NB-IoT (no AS security activation for CP solution).

Therefore, conclusions 3 and 4 above are not justified.

As discussed in RP-180429, SA2 already supports this scenario, RAN should support this scenario in alignment with SA2.

Then we propose the following way forward.

Proposal: It is proposed to agree on the following way forward. Plenary should either

approve the CR proposal RP-180493

OR

RAN Plenary should confirm:

- The requirement to support secured signaling only connection in Rel-15

- Task RAN3 to decide on the solution by June plenary

- Explicitly allow non-backwards compatible change for this issue (i.e. backwards compatibility shall not be a criterion when selecting the best solution)

# 3	Proposal

Proposal: It is proposed to agree on the following way forward. Plenary should either

approve the CR proposal RP-180493

OR

RAN Plenary should confirm:

- The requirement to support secured signaling only connection in Rel-15

- Task RAN3 to decide on the solution by June plenary

- Explicitly allow non-backwards compatible change for this issue (i.e. backwards compatibility shall not be a criterion when selecting the best solution)