##### RAN1

- RAN1 NR-Adhoc#1801 (January 2018)

General

R1-1800562 Correction to RSTD measurement states Intel Corporation (UK) Ltd The CR is endorsed in principle. To be further consolidated with any other 215 TPs R1-1800778 Draft CR 36.214 Inter-RAT measurements of NR Ericsson The CR is endorsed in principle. To be further checked in RAN1#92 R1-1800794 Draft CR to 38.211 Ericsson LM The CR is endorsed in principle (to fix the typo “Genric”). To be further consolidated with any other TPs R1-1801276 is endorsed R1-1801281 is approved.

Initial access and mobility

Agreements: - For signal generation for both DL and UL, adopt the following: - Transmitter generates signals for all symbols using a common reference tone - FFS: Which tone to use as the common reference tone - FFS applicability to PRACH Agreements: - Have independent power scaling of SSS, PBCH data, and PBCH DMRS in 36.211 as shown in TP below. - Update text of section 4.1 of 38.213 on UE assumption for power scaling as shown in TP below <<<< TP to 38.211 start >>>> 7.4.3.1.2 Mapping of SSS within an SS/PBCH block The UE shall assume the sequence of symbols constituting the secondary synchronization signal to be scaled by a factor and mapped to resource elements in increasing order of where and are given by Table 7.4.3.1-1 and represent the frequency and time indices, respectively, within one SS/PBCH block. 7.4.3.1.3 Mapping of PBCH and DM-RS within an SS/PBCH block The UE shall assume the sequence of complex-valued symbols constituting the physical broadcast channel to be scaled by a factor to conform to the PBCH power allocation specified in [5, TS 38.213] and mapped in sequence starting with to resource elements which meet all the following criteria: - they are not used for PBCH demodulation reference signals The mapping to resource elements not reserved for PBCH DM-RS shall be in increasing order of first the index and then the index , where and represent the frequency and time indices, respectively, within one SS/PBCH block and are given by Table 7.4.3.1-1. The UE shall assume the sequence of complex-valued symbols constituting the demodulation reference signals for the SS/PBCH block to be scaled by a factor of to conform to the PBCH power allocation specified in [5, TS 38.213] and to be mapped to resource elements in increasing order of first and then where and are given by Table 7.4.3.1-1 and represent the frequency and time indices, respectively, within one SS/PBCH block. <<<< TP to 38.211 end >>>> ------------------------------------- Begin of Text Proposal for Section 4.1 in TS38.213 ------------------------------ A UE shall assume that reception occasions of a physical broadcast channel (PBCH), PSS, and SSS are in consecutive symbols, as defined in [4, TS 38.211], and form a SS/PBCH block. The UE shall assume that SSS, PBCH DM-RS and PBCH data have the same EPRE. ------------------------------------- End of Text Proposal for 4.1 in TS38.213------------------------------ Agreements: - The value of M (as in 38.101) for sync raster definition (i.e., 0, ±1) for FR1 is informed to UE - Up to RAN4 to decide the set of offset values for FR1 - As a working assumption, the indication is in RMSI - If the minimum distance between adjacent sync rasters is large enough (w.r.t the intial frequency offset tolerance), the indication is no longer necessary - Send an LS to RAN2 and RAN4 – R1-1801109, which is approved and final LS is in R1-1801182 Agreements: - Send an LS to RAN4 regarding the following working assumption. Note that there are also other alternative(s) discussed in RAN1. - RAN1 asks RAN4 would especially appreciate if RAN4 can progress on the frequency offset associated with the value of M. RAN1 aims to make a decision in the early week of the next Feb. meeting. - (Working assumption) For signal generation: - Agree to option 3a (unquantized) - Baseband signal generation remains unchanged - Change upconversion formula for all channels/signals expect PRACH to: - where - Upconversion formula for PRACH remains unchanged - Draft LS to be prepared in R1-1801245, which is approved and final LS in R1-1801279 Agreements: - To adopt the following TP for Section 5.1 of 38.214: If the UE has received no SSB-transmitted through higher layer signalling about SS/PBCH block transmissions in the serving cell, the UE assumes SS/PBCH block transmission according to SSB-transmitted-SIB1, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall determine that PDSCH is rate matched around these PRBs. The UE assumes the periodicity of the SS/PBCH block transmission resources based on SSB-periodicity-serving-cell. If the UE has received a SSB-transmitted through higher layer signalling about SS/PBCH block transmissions in the serving cell, the UE assumes SS/PBCH block transmission according to the SSB-transmitted, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that PDSCH is rate matched around these PRBs. The UE assumes the periodicity of the SS/PBCH block transmission resources based on SSB-periodicity-serving-cell. When receiving the PDSCH conveying [SystemInformationBlockType1], a UE may assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH conveying [RAR, OSI, Paging, Msg4], the UE assumes SS/PBCH block transmission according to SSB-transmitted-SIB1, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that PDSCH is rate matched around these PRBs. Conclusion: Regarding signaling the valid search raster: - No additional signaling entries in the PBCH (for example cellBarred, intraFreqReselection) are to be used to inform the UE of the valid raster - The indication is only for the same frequency band Agreements: To agree the following TP to 38.211, section 7.4.1.4.1: - for , is the number of the half-frame in which the PBCH is transmitted in frame , corresponds to the first half-frame, corresponds to the second half-frame, and is the two least significant bits of the SS/PBCH index as defined in [5] Agreements: To agree the following change to section 7.1.1 of 38.212: - are the 6th, 5th, and 4th LSB bit of SS/PBCH block index, respectively. Conclusion: - Further offline discussion on the complete tables regarding the grid offset for FR1 and FR2 - No change of the 4-bit as per RRC by using some existing reserved/spare bit - After the table is completed, consider sending (to be checked offline) an LS to RAN2 to inform the number of spare bits for FR1/FR2 No LS is necessary, to be reflected in RRC parameter update Agreements: - To adopt the following 38.211 and 38.212 TPs: <<<< text proposal 38.211 for 7.4.3.1 start >>>> In the frequency domain, an SS/PBCH block consists of 240 contiguous subcarriers with the subcarriers numbered in increasing order from 0 to 239 within the SS/PBCH block. The quantities and represent the frequency and time indices, respectively, within one SS/PBCH block. The UE may assume resource elements denoted as 'Set to 0' in Table 7.4.3.1-1 are set to zero. Subcarrier 0 in an SS/PBCH block corresponds to subcarrier in common resource block is the subcarrier offset from subcarrier 0 of common resource block to subcarrier 0 of the SS/PBCH block, where is obtained from the higher-layer parameter offset-ref-low-scs-ref-PRB and where the 4 LSB of is given by higher-layer parameter ssb-subcarrierOffset in MIB and for SS/PBCH block type A the MSB of is given by in the PBCH payload as defined in Subclause 7.1.1 of [4, TS 38.212]. <<<< text proposal 38.211 end >>>> <<<< text proposal 38.212 start >>>> 7.1.1 PBCH payload generation Denote the bits in a transport block delivered to layer 1 by , where is the payload size generated by higher layers. The lowest order information bit is mapped to the most significant bit of the transport block as defined in Subclause [6.1.4] of [8, TS 38.321]. Generate the following additional timing related PBCH payload bits , where: - are the 4th, 3rd, 2nd, and 1st LSB of SFN, respectively; - is the half radio frame bit ; - If L=64 - - are the 6th, 5th, and 4th LSB bit of SS/PBCH block index, respectively. Else - is the MSB of as defined in Subclause 7.4.3.1 of [4, TS 38.211] - are reserved End if <<<< text proposal 38.212 end >>>> Agreements: - To adopt the following TP in 38.211, Section 7.4.3.1: For an SS/PBCH block, the UE shall assume - antenna port is used for transmission of PSS, SSS and PBCH, - the same cyclic prefix length and subcarrier spacing for the PSS, SSS, and PBCH, - for SS/PBCH block type A, , , and is are expressed in terms of 15 kHz subcarrier spacing, and - for SS/PBCH block type B, , is expressed in terms of the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon, , and is expressed in terms of the subcarrier spacing 60 kHz . The UE may assume that SS/PBCH blocks transmitted with the same block index are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters. The UE shall not assume quasi co-location for any other SS/PBCH block transmissions. ------------------end of TP--------------------------------- Agreements: - To adopt the following update to 38.213: Table 13-6: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {120, 120} kHz Index; SS/PBCH block and control resource set multiplexing pattern; Number of RBs; Number of Symbols; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 14. 3; 1; 48; 2; 14. 4; 3; 24; 2; -20 if condition A -21 if condition B. 5; 3; 24; 2; 24. 6; 3; 48; 2; -20 if condition A -21 if condition B. 7; 3; 48; 2; 48. 8; Reserved. 9; Reserved. 10; Reserved. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved. Table 13-9: Parameters for PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing pattern 1 and carrier frequencies smaller than or equal to 6 GHz Table 13-10: Parameters for PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing pattern 1 and carrier frequencies above 6 GHz Table 13-11: PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing pattern 2 and {SS/PBCH block, PDCCH} subcarrier spacing {120, 60} kHz Table 13-12: PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing pattern 2 and {SS/PBCH block, PDCCH} subcarrier spacing {240, 120} kHz Table 13-13: PDCCH monitoring occasions for Type0-PDCCH common search space - SS/PBCH block and control resource set multiplexing pattern 3 and {SS/PBCH block, PDCCH} subcarrier spacing {120, 120} kHz ---------end of TP--------------- Agreements: - Make the clarification that the existing Tables 13-1 and 13-2 in TS 38.213 are used for supporting the offset configurations for the bands with minimum channel bandwidths of 5MHz with SSB SCS = 15kHz; - The corresponding TP (38.213) - Table 13-1: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 15} kHz with minimum channel bandwidth 5MHz - Table 13-2: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 30} kHz with minimum channel bandwidth 5MHz - Make the clarification that the existing Tables 13-3 and 13-4 in TS 38.213 are for supporting the offset configurations for the bands with minimum channel bandwidths of 5MHz and 10MHz with SSB SCS = 30kHz; - The corresponding TP (38.213): - Table 13-3: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 15} kHz with minimum channel bandwidth 5MHz and 10MHz - Table 13-4: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 30} kHz with minimum channel bandwidth 5MHz and 10MHz - For the bands with minimum channel bandwidths of 40MHz, add new configuration tables for the following cases - One for {SSB SCS, RMSI SCS} = {30, 15}kHz - One for {SSB SCS, RMSI SCS} = {30, 30}kHz Agreements: - Adopt the following TP for Section 10.1 of 38.213: For each control resource set in a DL BWP of a serving cell, a respective higher layer parameter CORESET-freq-dom provides a bitmap. The bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 PRBs, in ascending order of the PRB index in the DL BWP bandwidth of PRBs with the starting position where the first PRB of the first group of 6 PRBs has index . Agreements: - To adopt the first TP in Section 2 of R1-1801122 regarding change to section 13 of 38.213 (de-highlight the yellow part) - To adopt the second TP in section 2 of R1-1801122 regarding change to Table 13-13 of 38.213 Agreements: - To adopt the TP in Section 4.1 regarding new tables 13-X3 and 13-X4 of 38.213 - Editors may update the table indexing Agreements: - Send an LS to RAN4 for the revised/new RMSI CORESET configuration tables – to be prepared in R1-1801188, which is approved by updating “would like to information” and final LS is in R1-1801246 - Note: Including Ref. R1-1801168 for the references on the generation of the offsets. - Agreements: - The TPs in section 2 of R1-1801192 for 38.213 are endorsed Agreements: - For the broadcast OSI PDCCH and paging PDCCH configured by RMSI, the corresponding number of candidates per AL for broadcast OSI PDCCH and for paging PDCCH is fixed and the same as the ones for RMSI in TS38.213. - Inform RAN2 of above update – LS to be prepared R1-1801113 (including updating the table header), which is approved and final LS is R1-1801189 Conclusion: - Regarding the max payload size for OSI/RMSI/paging, RAN1 expects RAN2 to finalize the necessary information fields in OSI/RMSI/paging - RAN1 may discuss the max payload size once the set of information fields in OSI/RMSI/paging become mature - Note: there should a limit on the max payload size for each of OSI/RMSI/paging (e.g., in LTE, the max # of payload size is about 2000 bits) Agreements: - The TP in section 3.3 in R1-1801142 impacting Section 10.1 of 38.213 is adopted Agreements: - NR supports a 1-bit in paging DCI to indicate whether the short message only or scheduling information only is carried in the Paging DCI - The TP is endorsed as follows (7.3.1.2.1, 38.212) - PUCCH resource indicator – [2] bits as defined in section x.x of [5, TS38.213] - PDSCH-to-HARQ_feedback timing indicator – [3] bits as defined in section x.x of [5, TS38.213] If scrambled by P-RNTI, the following information is transmitted by means of the DCI format 1_0: - Short Messages Indicator – 1 bit. This bit is used to indicate whether the short message only or scheduling information only is carried in the Paging DCI. Agreements: - To adopt the TP for Section 5.1 of TS38.214: When receiving PDSCH for RMSI or broadcasted Other System Info or paging the UE may assume that the DM-RS port of PDSCH is quasi co-located with the associated SS/PBCH block with respect to [spatial RX parameters]. Agreements: - The association between actual transmitted SSB and the monitoring window of PDCCH containing the Paging DCI and the broadcast OSI DCI can be respectively configured via RMSI. - It is up to RAN2 on how to do the above configuration. Send an LS to RAN2 (R1-1801248, which is approved and final LS is in R1-1801280) - The default association between SSB index and monitoring window of PDCCH containing a Paging DCI and a broadcast OSI DCI is same as that between SSB index and its RMSI monitoring window. Agreements: - The TP in section 2.2 (for section 10.1 of 38.213) of R1-1801261 is endorsed Agreements: Confirm the following working assumptions with the following update: - Number of time domain RACH occasions within a RACH slot - For preamble format (working assumption), at least the following # of occasions: - A1: 6 - A2: 3 - A3: 2 - B4: 1 - B1: 6 or 7 - C0: 4 6 or 7 - C2: 2 - A1/B1: 6 or 7 - A2/B2: 3 - A3/B3: 2 - The value is not applicable (N/A) for format 0-3 Agreements: - For the separate configuration of the number of PRACH transmission occasions FDMed in one time instance for initial access, support: - The value range of the 2-bit RRC parameter prach-FDM is {1,2,4,8} Agreements: - For RACH configuration table headings, update as follows: - Number of time domain PRACH occasions within a RACH slot Working assumption: For Random access configurations for FR1 and paired spectrum/SUL in case of long sequence PRACH, add the following configuration entries (shown with change marks in red): - The actual PRACH configuration index is up to the editor: Table 6.3.3.2-2: Random access configurations for FR1 and paired spectrum/SUL PRACH Configuration Index; Preamble format; Subframe number; Starting symbol; Number of PRACH slots within a subframe; Number of time domain PRACH occasions within a RACH slot. 0; 4; 1; 1; -; -; -. 0; 4; 1; 4; -; -; -. 0; 4; 1; 7; -; -; -. 0; 4; 1; 9; -; -; -. 0; 8; 1; 1; -; -; -. 0; 8; 1; 4; -; -; -. 0; 8; 1; 7; -; -; -. 0; 8; 1; 9; -; -; -. 0; 16; 1; 1; -; -; -. 0; 16; 1; 4; -; -; -. 0; 16; 1; 7; -; -; -. 0; 16; 1; 9; -; -; -. 1; 4; 1; 1; -; -; -. 1; 4; 1; 4; -; -; -. 1; 4; 1; 7; -; -; -. 1; 4; 1; 9; -; -; -. 1; 8; 1; 1; -; -; -. 1; 8; 1; 4; -; -; -. 1; 8; 1; 7; -; -; -. 1; 8; 1; 9; -; -; -. 1; 16; 1; 1; -; -; -. 1; 16; 1; 4; -; -; -. 1; 16; 1; 7; -; -; -. 1; 16; 1; 9; -; -; -. 3; 4; 1; 1; -; -; -. 3; 4; 1; 4; -; -; -. 3; 4; 1; 7; -; -; -. 3; 4; 1; 9; -; -; -. 3; 8; 1; 1; -; -; -. 3; 8; 1; 4; -; -; -. 3; 8; 1; 7; -; -; -. 3; 16; 1; 1; -; -; -. 3; 16; 1; 4; -; -; -. 3; 16; 1; 7; -; -; -. 3; 16; 1; 9; -; -; -. 2; 8; 1; 1; 0; -; -. 2; 16; 1; 1; 0; -; -. Agreements: For 38.211, Section 6.3.3.1, update the spec as: for - for Agreements: In 38.211, 6.3.3.2, update the spec as follows: - Random access preambles can only be transmitted in the time and frequency resources given by the higher-layer parameter PRACHConfigurationIndex according to Table 6.3.3.2-2 depends on FR1 or FR2 and the spectrum type as defined in [TS38.101]. Agreements: For RACH configuration for FR1 and paired spectrum: - More priority to Ax and Bx formats compared to Ax/Bx - Change from subframe 5 to subframe 4 for RACH configuration period 40,80,160ms - Regarding the number of RACH occasions within one RACH configuration period. - FDM is used to achieve 8 RACH occasions within one RACH configuration period - In addition, for preamble formats A1, A2 and for periodicities 80ms, 160ms at least one configuration supports 8 RACH occasions. - Note: This can be done by either have more subframes allocated or to have more frames allocated Agreements: - Adopt the following TP to Section 5.3 of 38.211: The time-continuous signal on antenna port for PRACH is defined by where and is given by clause 6.3.3. The value of is obtained from the subcarrier spacing configuration of the initial active uplink BWP. The value of is obtained from the higher-layer parameter k0 and is such that the lowest numbered subcarrier in a common resource block for subcarrier spacing configuration coincides with the lowest numbered subcarrier in a common resource block for any subcarrier spacing configuration less than . The value is the lowest numbered RB of initial active uplink BWP based on common resource block indexing and is derived by the higher-layer parameter initial-Uplink-Bandwidth-Part. The value is the frequency offset of lowest PRACH transmission occasion in frequency domain with respect to PRB 0 of initial active UL BWP(s) given by the higher-layer parameter prach-frequency-start, the value is the PRACH transmission occasion index in frequency domain for a given PRACH transmission occasion in one time instance as given by clause 6.3.3.2, and the value is the number of resource blocks occupied and is given by the parameter allocation expressed in number of RBs for PUSCH in Table 6.3.3.2-1. +++++++++++++++++++++ end text proposal +++++++++++++++++++++++++++++++++++++++++ Agreements: - The values of in table 6.3.3.2-1 of 38.211 are confirmed with the following updates: ------------- text proposal, 38.211 section 6.3.3.2 ----------------------------------------------- Table 6.3.3.2-1: Supported combinations of and , and the corresponding value of . for PRACH; for PUSCH; Allocation expressed in number of RBs for PUSCH. 839; 1.25; 15; 6; 712. 839; 1.25; 30; 3; 112. 839; 1.25; 60; 2; 133156. 839; 5; 15; 24; 12. 839; 5; 30; 12; 1012. 839; 5; 60; 6; 712. 139; 15; 15; 12; 2. 139; 15; 30; 6; 2. 139; 15; 60; 3; 2. 139; 30; 15; 24; 2. 139; 30; 30; 12; 2. 139; 30; 60; 6; 2. 139; 60; 60; 12; 2. 139; 60; 120; 6; 2. 139; 120; 60; 24; 2. 139; 120; 120; 12; 2. ------------- end text proposal, 38.211 section 6.3.3.2 ----------------------------------------------- Agreements: - To adopt the following for section 6.3.3.2 of 38.211 ------------- text proposal, 38.211 section 6.3.3.2 ----------------------------------------------- Table 6.3.3.2-2: Random access configurations for FR1 and paired spectrum/SUL (short sequence) PRACH Configuration Index; Preamble format; Subframe number; Starting symbol; Number of PRACH slots within a subframe; Number of PRACH occasions within a RACH slot. A1; 16; 0; 4,9; 0; 1; 6. A1; 16; 1; 4; 0; 2; 6. A1; 8; 0; 4,9; 0; 1; 6. A1; 8; 1; 4; 0; 2; 6. A1; 4; 0; 4,9; 0; 1; 6. A1; 4; 0; 4; 0; 2; 6. A1; 4; 1; 4,9; 0; 1; 6. A1; 2; 0; 4,9; 0; 1; 6. A1; 2; 0; 1; 0; 2; 6. A1; 2; 0; 4; 0; 2; 6. A1; 2; 0; 7; 0; 2; 6. A1; 1; 0; 1; 0; 2; 6. A1; 1; 0; 4; 0; 1; 6. A1; 1; 0; 7; 0; 2; 6. A1; 1; 0; 1,6; 0; 1; 6. A1; 1; 0; 2,7; 0; 2; 6. A1; 1; 0; 4,9; 0; 1; 6. A1; 1; 0; 1,4,7; 0; 2; 6. A1; 1; 0; 0,2,4,6,8; 0; 2; 6. A1; 1; 0; 0,1,2,3,4,5,6,7,8,9; 0; 2; 6. A1; 1; 0; 1,3,5,7,9; 0; 2; 6. A2; 16; 1; 2,6,9; 0; 1; 3. A2; 16; 1; 4; 0; 2; 3. A2; 8; 1; 2,6,9; 0; 1; 3. A2; 8; 1; 4; 0; 2; 3. A2; 4; 0; 2,6,9; 0; 1; 3. A2; 4; 0; 4; 0; 2; 3. A2; 2; 1; 2,6,9; 0; 1; 3. A2; 2; 0; 1; 0; 2; 3. A2; 2; 0; 4; 0; 2; 3. A2; 2; 0; 7; 0; 2; 3. A2; 1; 0; 1; 0; 2; 3. A2; 1; 0; 4; 0; 1; 3. A2; 1; 0; 7; 0; 2; 3. A2; 1; 0; 1,6; 0; 1; 3. A2; 1; 0; 2,7; 0; 2; 3. A2; 1; 0; 4,9; 0; 1; 3. A2; 1; 0; 1,4,7; 0; 2; 3. A2; 1; 0; 0,2,4,6,8; 0; 2; 3. A2; 1; 0; 0,1,2,3,4,5,6,7,8,9; 0; 2; 3. A2; 1; 0; 1,3,5,7,9; 0; 2; 3. A3; 16; 1; 4,9; 0; 1; 2. A3; 8; 1; 4,9; 0; 1; 2. A3; 4; 0; 4,9; 0; 1; 2. A3; 16; 1; 4; 0; 2; 2. A3; 8; 1; 4; 0; 2; 2. A3; 4; 0; 4; 0; 2; 2. A3; 2; 1; 2,6,9; 0; 2; 2. A3; 2; 0; 1; 0; 2; 2. A3; 2; 0; 4; 0; 2; 2. A3; 2; 0; 7; 0; 2; 2. A3; 1; 0; 1; 0; 2; 2. A3; 1; 0; 4; 0; 1; 2. A3; 1; 0; 7; 0; 2; 2. A3; 1; 0; 1,6; 0; 1; 2. A3; 1; 0; 2,7; 0; 2; 2. A3; 1; 0; 4,9; 0; 1; 2. A3; 1; 0; 1,4,7; 0; 2; 2. A3; 1; 0; 0,2,4,6,8; 0; 2; 2. A3; 1; 0; 0,1,2,3,4,5,6,7,8,9; 0; 2; 2. A3; 1; 0; 1,3,5,7,9; 0; 2; 2. B1; 16; 0; 4,9; 0; 1; 7. B1; 16; 1; 4; 0; 2; 7. B1; 8; 0; 4,9; 0; 1; 7. B1; 8; 1; 4; 0; 2; 7. B1; 4; 0; 4,9; 0; 1; 7. B1; 4; 0; 4; 0; 2; 7. B1; 4; 1; 4,9; 0; 1; 7. B1; 2; 0; 4,9; 0; 1; 7. B1; 2; 0; 1; 0; 2; 7. B1; 2; 0; 4; 0; 2; 7. B1; 2; 0; 7; 0; 2; 7. B1; 1; 0; 1; 0; 2; 7. B1; 1; 0; 4; 0; 1; 7. B1; 1; 0; 7; 0; 2; 7. B1; 1; 0; 1,6; 0; 1; 7. B1; 1; 0; 2,7; 0; 2; 7. B1; 1; 0; 4,9; 0; 1; 7. B1; 1; 0; 1,4,7; 0; 2; 7. B1; 1; 0; 0,2,4,6,8; 0; 2; 7. B1; 1; 0; 0,1,2,3,4,5,6,7,8,9; 0; 2; 7. B1; 1; 0; 1,3,5,7,9; 0; 2; 7. B4; 16; 0; 4,9; 0; 2; 1. B4; 16; 1; 4; 0; 2; 1. B4; 8; 0; 4,9; 0; 2; 1. B4; 8; 1; 4; 0; 2; 1. B4; 4; 0; 4,9; 0; 2; 1. B4; 4; 0; 4; 0; 2; 1. B4; 4; 1; 4,9; 0; 2; 1. B4; 2; 0; 4,9; 0; 2; 1. B4; 2; 0; 1; 0; 2; 1. B4; 2; 0; 4; 0; 2; 1. B4; 2; 0; 7; 0; 2; 1. B4; 1; 0; 1; 0; 2; 1. B4; 1; 0; 4; 0; 2; 1. B4; 1; 0; 7; 0; 2; 1. B4; 1; 0; 1,6; 0; 2; 1. B4; 1; 0; 2,7; 0; 2; 1. B4; 1; 0; 4,9; 0; 2; 1. B4; 1; 0; 1,4,7; 0; 2; 1. B4; 1; 0; 0,2,4,6,8; 0; 2; 1. B4; 1; 0; 0,1,2,3,4,5,6,7,8,9; 0; 2; 1. B4; 1; 0; 1,3,5,7,9; 0; 2; 1. C0; 8; 1; 4; 0; 2; 7. C0; 4; 0; 4; 0; 2; 7. C0; 4; 1; 4,9; 0; 1; 7. C0; 2; 0; 4,9; 0; 1; 7. C0; 2; 0; 1; 0; 2; 7. C0; 2; 0; 4; 0; 2; 7. C0; 2; 0; 7; 0; 2; 7. C0; 1; 0; 1; 0; 2; 7. C0; 1; 0; 4; 0; 1; 7. C0; 1; 0; 7; 0; 2; 7. C0; 1; 0; 1,6; 0; 1; 7. C0; 1; 0; 2,7; 0; 2; 7. C0; 1; 0; 4,9; 0; 1; 7. C0; 1; 0; 1,4,7; 0; 2; 7. C0; 1; 0; 0,2,4,6,8; 0; 2; 7. C0; 1; 0; 0,1,2,3,4,5,6,7,8,9; 0; 2; 7. C0; 1; 0; 1,3,5,7,9; 0; 2; 7. C2; 16; 1; 4,9; 0; 1; 2. C2; 8; 1; 4,9; 0; 1; 2. C2; 4; 0; 4,9; 0; 1; 2. C2; 16; 1; 4; 0; 2; 2. C2; 8; 1; 4; 0; 2; 2. C2; 4; 0; 4; 0; 2; 2. C2; 2; 1; 2,6,9; 0; 2; 2. C2; 2; 0; 1; 0; 2; 2. C2; 2; 0; 4; 0; 2; 2. C2; 2; 0; 7; 0; 2; 2. C2; 1; 0; 1; 0; 2; 2. C2; 1; 0; 4; 0; 1; 2. C2; 1; 0; 7; 0; 2; 2. C2; 1; 0; 1,6; 0; 1; 2. C2; 1; 0; 2,7; 0; 2; 2. C2; 1; 0; 4,9; 0; 1; 2. C2; 1; 0; 1,4,7; 0; 2; 2. C2; 1; 0; 0,2,4,6,8; 0; 2; 2. C2; 1; 0; 0,1,2,3,4,5,6,7,8,9; 0; 2; 2. C2; 1; 0; 1,3,5,7,9; 0; 2; 2. A1/B1; 2; 0; 4,9; 0; 1; 7. A1/B1; 2; 0; 4; 0; 2; 7. A1/B1; 1; 0; 1; 0; 2; 7. A1/B1; 1; 0; 4; 0; 1; 7. A1/B1; 1; 0; 7; 0; 2; 7. A1/B1; 1; 0; 1,6; 0; 1; 7. A1/B1; 1; 0; 4,9; 0; 1; 7. A1/B1; 1; 0; 1,4,7; 0; 2; 7. A1/B1; 1; 0; 0,2,4,6,8; 0; 2; 7. A1/B1; 1; 0; 0,1,2,3,4,5,6,7,8,9; 0; 2; 7. A2/B2; 2; 1; 2,6,9; 0; 1; 3. A2/B2; 2; 0; 4; 0; 2; 3. A2/B2; 1; 0; 1; 0; 2; 3. A2/B2; 1; 0; 4; 0; 1; 3. A2/B2; 1; 0; 7; 0; 2; 3. A2/B2; 1; 0; 1,6; 0; 1; 3. A2/B2; 1; 0; 4,9; 0; 1; 3. A2/B2; 1; 0; 1,4,7; 0; 2; 3. A2/B2; 1; 0; 0,2,4,6,8; 0; 2; 3. A2/B2; 1; 0; 0,1,2,3,4,5,6,7,8,9; 0; 2; 3. A3/B3; 2; 1; 2,6,9; 0; 2; 2. A3/B3; 2; 0; 4; 0; 2; 2. A3/B3; 1; 0; 1; 0; 2; 2. A3/B3; 1; 0; 4; 0; 1; 2. A3/B3; 1; 0; 7; 0; 2; 2. A3/B3; 1; 0; 1,6; 0; 1; 2. A3/B3; 1; 0; 4,9; 0; 1; 2. A3/B3; 1; 0; 1,4,7; 0; 2; 2. A3/B3; 1; 0; 0,2,4,6,8; 0; 2; 2. A3/B3; 1; 0; 0,1,2,3,4,5,6,7,8,9; 0; 2; 2. ------------- end text proposal, 38.211 section 6.3.3.2 ----------------------------------------------- Agreements: - NR supports PRACH format 0-3 for unpaired spectrum FR1. Agreements: - Revisit the agreement made in RAN1#91 regarding the start symbol index for preamble format 2 with the following change: - Start symbol Index (in Msg1 SCS for short sequence and 15kHz for long sequence): - Values {0,76} for format 2 Agreements: - To adopt the following TP to section 6.3.3.2 of 38.211 - The ones in red color are working assumption ------------- text proposal, 38.211 section 6.3.3.2 ----------------------------------------------- - Table 6.3.3.2-3: Random access configurations for FR1 and unpaired spectrum (long sequence) PRACH Configuration Index; Preamble format; Subframe number; Starting symbol; Number of PRACH slots within a subframe; Number of PRACH occasions within a RACH slot. 0; 0; 16; 1; 9; 0; -; -. 0; 8; 1; 9; 0; -; -. 0; 4; 1; 9; 0; -; -. 0; 2; 0; 9; 0; -; -. 0; 2; 1; 9; 0; -; -. 0; 2; 0; 4; 0; -; -. 0; 2; 1; 4; 0; -; -. 0; 1; 0; 9; 0; -; -. 0; 1; 0; 8; 0; -; -. 0; 1; 0; 7; 0; -; -. 10; 0; 1; 0; 6; 0; -; -. 0; 1; 0; 5; 0; -; -. 0; 1; 0; 4; 0; -; -. 0; 1; 0; 3; 0; -; -. 0; 1; 0; 2; 0; -; -. 0; 1; 0; 1; 0; -; -. 0; 1; 0; 4,9; 0; -; -. 0; 1; 0; 3,8; 0; -; -. 0; 1; 0; 2,7; 0; -; -. 0; 1; 0; 8,9; 0; -; -. 20; 0; 1; 0; 4,8,9; 0; -; -. 0; 1; 0; 3,4,9; 0; -; -. 0; 1; 0; 3,4,8; 0; -; -. 0; 1; 0; 7,8,9; 0; -; -. 0; 1; 0; 3,4,8,9; 0; -; -. 0; 1; 0; 6,7,8,9; 0; -; -. 0; 1; 0; 1,4,6,9; 0; -; -. 0; 1; 0; 1,6; 0. 0; 1; 0; 1,6; 7; -; -. 0; 1; 0; 1,3,5,7,9; 0; -; -. 30; 1; 16; 1; 7; 0; -; -. 1; 8; 1; 7; 0; -; -. 1; 4; 1; 7; 0; -; -. 1; 2; 0; 7; 0; -; -. 1; 2; 1; 7; 0; -; -. 1; 1; 0; 7; 0; -; -. 2; 16; 1; 6; 0; -; -. 2; 8; 1; 6; 0; -; -. 2; 4; 1; 6; 0; -; -. 2; 2; 0; 6; 7; -; -. 40; 2; 2; 1; 6; 7; -; -. 2; 1; 0; 6; 7; -; -. 3; 16; 1; 9; 0; -; -. 3; 8; 1; 9; 0; -; -. 3; 4; 1; 9; 0; -; -. 3; 2; 0; 9; 0; -; -. 3; 2; 1; 9; 0; -; -. 3; 2; 0; 4; 0; -; -. 3; 2; 1; 4; 0; -; -. 3; 1; 0; 9; 0; -; -. 50; 3; 1; 0; 8; 0; -; -. 3; 1; 0; 7; 0; -; -. 3; 1; 0; 6; 0; -; -. 3; 1; 0; 5; 0; -; -. 3; 1; 0; 4; 0; -; -. 3; 1; 0; 3; 0; -; -. 3; 1; 0; 2; 0; -; -. 3; 1; 0; 1; 0; -; -. 3; 1; 0; 1,6; 0; -; -. 3; 1; 0; 1,6; 7; -; -. 60; 3; 1; 0; 4,9; 0; -; -. 3; 1; 0; 3,8; 0; -; -. 3; 1; 0; 2,7; 0; -; -. 3; 1; 0; 8,9; 0; -; -. 3; 1; 0; 4,8,9; 0; -; -. 3; 1; 0; 3,4,9; 0; -; -. 3; 1; 0; 7,8,9; 0; -; -. 3; 1; 0; 3,4,8,9; 0; -; -. 3; 1; 0; 6,7,8,9; 0; -; -. 3; 1; 0; 1,4,6,9; 0; -; -. 70; 3; 1; 0; 1,3,5,7,9; 0; -; -. ------------- end text proposal, 38.211 section 6.3.3.2 ----------------------------------------------- Agreements: - Support cyclically mapping the actually transmitted SS/PBCH blocks and ROs within a period - FFS the definition of the period - FFS whether or not there a case of ROs that are not enough for a complete association with all the actually transmitted SS/PBCH blocks and if so, how to handle Agreements: - For the cyclic mapping of association between ROs and all the actually transmitted SS/PBCH blocks, if there are leftover ROs after an integer number of cycles within the defined period, - no SS/PBCH blocks are mapped to these leftover ROs Agreements: - Confirm the following working assumption - Working assumption: - When multiple SS block are associated with one RACH transmission occasion, the preamble indices for CBRA for each SS block are mapped consecutively Agreements: - NR supports maximum TA command value of 3846 for RAR Agreements: - Confirm the working assumption that L2 = 500us as the MAC processing latency during RACH procedure. Agreements: - To add the following text in Section 8.1 of 38.213: Actually transmitted SS/PBCH blocks are associated to RACH transmission according in the following order: - First in increasing preamble indices within a single RACH occasion. - Then in increasing number of frequency multiplexed RACH occasion. - Then in increasing number of time-domain RACH occasion within a RACH slot. - Then in increasing number of RACH slots. The number of SS/PBCH blocks associated to one RACH occasion is determined by parameter SSB-per-rach-occasion. If SSB-per-rach-occasion is less than one, one SS/PBCH is mapped to 1/SSB-per-rach-occasion consecutive RACH occasions. Agreements: - NR supports the following number of SSBs per RACH occasion: {1/8, 1/4, 1/2, 1, 2, 4, 8, 16} - Note: This agreement includes the possibility that FDMed RACH occasions are configured to support configurations where all FDMed RACH occasions get mapped to one set of SSBs Agreements: - For long sequence preamble formats, the parameter DELTA_PREAMBLE is given by the following table: Preamble Format; DELTA_PREAMBLE values. 0; 0 dB. 1; -3 dB. 2; -6 dB. 3; 0 dB. Agreements: - The indexing order of FDMed RACH transmissions occasions within the initial active UL BWP is from the lowest one to the highest one in frequency domain - To adopt the following TP in Section 6.3.3.2 of 38.211: Random access preambles can only be transmitted in the time and frequency resources given by the higher-layer parameter PRACHConfigurationIndex according to Table 6.3.3.2-2 depends on FR1 or FR2 and the spectrum type as defined in [TS38.101]. The random access preambles can be transmitted in the frequency resources given by parameter prach-FDM. The indexing order of the FDMed RACH transmission occasions within the initial active UL BWP, is from the lowest one to the highest one in frequency domain. The frequency multiplexed RACH occasions within the initial active UL BWP in one time instance are indexed according to: Agreements: - For SSB-per-rach-occasion {1/8, 1/4, ½, 1, 2}: CB-preambles-per-SSB is selected from the set of values 4*N, with N=1, …, floor(16/max(1, SSB-per-rach-occasion)) - For SSB-per-rach-occasion {4, 8, 16}: CB-preambles-per-SSB is selected from the set of values N, with N=1, …, floor(64/SSB-per-rach-occasion) Agreements: To adopt the following TP for Section 8.1 of 38.213: - Editors to update/define the termilogy of “UL Tx beam” as necessary Prior to initiation of the physical random access procedure, Layer 1 shall receive from higher layers a set of SS/PBCH block indexes and shall provide to higher layers a corresponding set of RSRP measurements. Prior to retransmission of a PRACH preamble, if a UE plans to switch its UL TX beam, layer 1 shall notify upper layer to suspend power ramping counter. Prior to initiation of the physical random access procedure, Layer 1 shall receive the following information from the higher layers Agreements: DELTA_PREAMBLE for short sequence preambles is given by: A1; 8 + 3∙μ dB. A2; 5 + 3∙μ dB. A3; 3 + 3∙μ dB. B1; 8 + 3∙μ dB. B2; 5 + 3∙μ dB. B3; 3 + 3∙μ dB. B4; 3∙μ dB. C0; 11 + 3∙μ dB. C2; 5 + 3∙μ dB. Where μ is the sub-carrier spacing configuration. Conclusion: - RAN1 does not see the need to specify a shorter TA field in the RAR that is different for short preamble sequences and for long preamble sequences in Rel-15 Agreements: - RAN1 agrees that N_TA_offset is the same with and without NR-LTE co-existence for TDD - LS reply R1-1801260, which is approved and final LS in R1-1801282 Agreements: - To adopt the following TP for 5.1.1 of 38.215 ================= start of text proposal ================== Definition; SS reference signal received power (SS -RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals (SS). The measurement time resource(s) for SS-RSRP are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. If SS-RSRP is used for L1-RSRP as configured by reporting configurations as defined in 38.214 [6], the measurement time resources(s) restriction by SMTC window duration is not applicable. For SS-RSRP determination demodulation reference signals for physical broadcast channel (PBCH) and, if indicated by higher layers, CSI reference signals in addition to secondary synchronization signals may be used. SS-RSRP using demodulation reference signal for PBCH or CSI reference signal shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in 3GPP TS 38.213 [5]. If SS-RSRP is not used for L1-RSRP, the additional use of CSI reference signals for SS-RSRP determination demodulation reference signals is not applicable. SS-RSRP shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. If SS-RSRP is not used for L1-RSRP and higher-layers indicate certain SS/PBCH blocks for performing SS-RSRP measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s). For frequency range 1, the reference point for the SS-RSRP shall be the antenna connector of the UE. For frequency range 2, SS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRP value shall not be lower than the corresponding SS-RSRP of any of the individual receiver branches.. Applicable for; If SS-RSRP is used for L1-RSRP, RRC_CONNECTED intra-frequency. Otherwise, RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency.. NOTE 1: The number of resource elements within the measurement period that are used by the UE to determine SS-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled. NOTE 2: The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP. ================= End of text proposal ================== Agreements: - To include the following correction (in red) in the RRC parameter update to RAN2: Measurement-BW; Allowed mMeasurement BW for CSI-RS; UE specific; 38.331; To be included in CSI-RS-Config-Mobility. Contains: CSI-RS-measurementBW-size and CSI-RS-measurement-BW-start. Configured per carriercell, where a common set of values are assigned across all the resources for a given cell.. Common-PRB-Grid-ofset; Information to define common PRB grid for CSI-RS sequence generation; [0,1,2, …,276*4]; NA; UE specific; 38.331; Contained in CSI-RS-Config-Mobility. Information to define common PRB grid for CSI-RS sequence generation. Configured per carriercell, where a common set of values are assigned across all the resources for a given cell. Corresponds to an offset (in terms of number of subcarriers in CSI-RS numerology) between PRB 0 for common PRB indexing and a reference location – the reference location is the lowest PRB of the cell-defining SSB. Agreements: In 38.211, Section 7.4.3.1, update the specs as follows: - The UE may assume that SS/PBCH blocks transmitted with the same block index on the same center frequency location are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters. The UE shall not assume quasi co-location for any other SS/PBCH block transmissions. Agreements: - If associated SSB is not configured: - UE is required to perform measurements based on CSI-RS(s), even if SS/PBCH block(s) with corresponding cell ID are not detected. - UE shall base the timing of the CSI-RS resource(s) to the timing of the serving cell. - If associated SSB is configured: - UE may base the timing of the CSI-RS resource(s) to the timing of the cell, given by the Cell_ID of the CSI-RS resource configuration. - Additionally, if the associated SS/PBCH block is configured, the higher layer parameter qcled-SSB further indicates whether or not the associated SS/PBCH block and the CSI-RS resource(s) are quasi co-located with the associated SS/PBCH block with respect to [spatial RX parameters]. - Send LS (to be prepared in R1-1801118, which is approved with the update “The value of N2 is still under discussion in RAN1” to “N2=64”. The final LS is in R1-1801190) to RAN4 regarding the above agreements (including the context of the above agreements). RAN1 recommends RAN4 to capture the agreements accordingly in 38.133. Conclusion: - For inter-frequency measurement, associated SSB may or may not be configured for CSI-RS resources. - No spec impact Agreements: - To adopt the following TP for 5.1.3 of 38.215: If higher-layers indicate certain measurement time resource(s), then NR Carrier RSSI is measured from the indicated OFDM symbols of the indicated slots. If indicated by higher-layers, for a half-frame with SS/PBCH blocks the NR Carrier RSSI is measured from OFDM symbols of the indication slots, and the OFDM symbol are indicated by Table 5.1.3-1. Table 5.1.3-1: NR Carrier RSSI measurement symbols OFDM signal indication SS-RSSI-MeasurementSymbolConfig; Symbol indexes. 0; {0,1}. 1; {0,1,2,..,10,11}. 2; {0,1,2,…, 5}. 3; {0,1,2,…, 7}. If higher-layers indicate certain SS/PBCH blocks for performing SS-RSRQ measurements, then SS-RSRPRSRQ is measured only from the indicated set of SS/PBCH block(s). Agreements: - Remove the parameter value 240 kHz SCS for CSI-RS for L3 mobility from the RRC parameter table. - No change on the bitwidth of the RRC parameter Agreements: - The same default SS-RSSI measurement time resources are applicable for all SS-RSRQ measurements in the same carrier. To continue discussion till next meeting. Aim to conclude in early week of next RAN1 meeting - For FDD: - Accept RAN4 recommended default SS-RSSI measurement time resources - FFS: whether or not the UE is required to measure all SS-RSSI measurement time resources. - For TDD - FFS: whether or not the UE is required to measure all SS-RSSI measurement time resources. - Down-select from the following alternatives: - Alt 1: a subset of OFDM symbol of the slots that contain detected SSB. - The subset of OFDM symbols are: - Option 1) first 12 OFDM symbols - Option 2) first 5 OFDM symbol in the half slot that SSB was detected - Option 3) all OFDM symbols before a detected SSB in each half slot that contains the detected SSB - Option 4) 2 OFDM symbols - Option 5) First 1 OFDM symbol for Case A & B, first 2 symbol for Case C & D, first 4 symbol for Case E (case A,B,C,D,E are SS/PBCH patterns for different bands) - Option 6) All symbols excluding the OFDM symbol corresponding to the detected SSB - Alt 2: a set of OFDM symbols relative to the detected SSB - Alt 3: RAN4 recommendation - Alt 4: RAN4 recommendation with the exclusion of resources where slots that do not carry detected SSB Agreements: - Agree to the following changes to RRC parameter, CSI-RS-measurementBW-size: CSI-RS-measurementBW-size; New; Allowed CSI-RS-measurementBW-size; Size of the measurement BW in PRBs; 24, 48, 96, 192, 268264; NA; UE specific; 38.331; Contained in Measurement-BW. PRBs in CSI-RS numerology. Agreements: - Adopt the following corrections to the Common-PRB-Grid-offset parameter description and down-select on the value range of common-prb-grid-offset depending on the correct intent. Common-PRB-Grid-offset; Information to define common PRB grid for CSI-RS sequence generation; [0,1,2, …,276*4*12]; NA; UE specific; 38.331; Contained in CSI-RS-Config-Mobility. Information to define common PRB grid for CSI-RS sequence generation. Configured per cell, where a common set of values are assigned across all the resources for a given cell. Corresponds to an offset (in terms of number of subcarriers in CSI-RS numerology) between PRB 0 for common PRB indexing and a reference location – the reference location is the lowest PRB of the cell-defining SSB. Agreements: - Regarding previous agreements: - If associated SSBs are not configured for CSI-RS, maximum N2>=1 number of CSI-RS resources can be configured per frequency layer - In this case, UE may assume that the carrier is synchronized with the serving cell. - FFS UE is not required to perform measurement based on CSI-RS if the corresponding cell ID is not detected - Adopt: - N2 = 64 - FFS whether CSI-RS resources associated with and without SSB be configured in the same frequency layer. Agreements: - Confirm the working assumption of 2-port CSI-RS is not supported for RLM. - The corresponding TP is adopted (section 5 of 38.213) ============= start of text proposal ============== A UE can be configured for each SpCell [11, TS 38.321] with a set of resource indexes for radio link monitoring by higher layer parameter RLM-RS-List. The UE is provided by higher layer parameter RLM-RS an association between a resource index, from the set of resource indexes, with either a CSI-RS resource configuration or a SS/PBCH block. For a CSI-RS resource configuration, the UE is provided a corresponding index by higher layer parameter RLM-CSIRS. 2 port CSI-RS configuration is not applicable for RLM. For a SS/PBCH block, the UE is provided a corresponding index by higher layer parameter RLM-SSB. ============= End of text proposal ============== Agreements: - In Section 5 of 38.213, update the specs as follows: - If the UE is configured with a SCG, as described in [12, TS 38.331], and the parameter rlf-TimersAndConstantsSCG is provided by the higher layers and is not set to release, the downlink radio link quality of the PSCell of the SCG shall be monitored by the UE for the purpose of indicating out-of-sync/in-sync status to higher layers. The UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP on the PSCell. Agreements: - In addition to agreements made in RAN1 #91, the following parameters of CSI-RS configuration is not applicable for CSI-RS for RLM - CDM-pattern - Pc - Pc_SS Conclusion: - UE behavior for re-configuration of RLM-RS is up to RAN2 Agreements: - The IS/OOS indication interval when the periodicities are different for different RLM-RS resources is determined by the shortest periodicity of the different RLM-RS resources, which is also lower bounded by 10ms - Send an LS to RAN4 regarding the above (to be prepared in R1-1801123, which is approved and final LS is in R1-1801191) Agreements: - Confirm working assumption of maximum of 8 RLM-RSs for FR2 that can be configured for a UE Agreements: - Remove “FFS: QCL-info-CSI-RS” from agreement in RAN1 #91 regarding applicability of CSI-RS parameters to CSI-RS based RLM. - Agree on the following text proposal for section 5 of 38.213 ======== start of text proposal ======= For a CSI-RS resource configuration, the UE is provided a corresponding index by higher layer parameter RLM-CSIRS. The following parameters, CSI-IM-RE-pattern, CSI-IM-Resource, CSI-IM-ResourceId, CSI-IM-timeConfig, CSI-IM-FreqBand, CSI-IM-ResourceMapping, CDM-pattern, CDM-pattern, Pc, and Pc_SS, in the CSI-RS configuration are not applicable. ======== End of text proposal ======= Agreements: - At most X RLM RS resources can be configured per each BWP. - Note: X is the maximum number of RLM-RS resources and RAN1 has already agreed on this value.

MIMO

Draft LS is endorsed in R1-1801272 with the following changes: ------start------ 7. Latency of the MAC-CE Messages RAN1 discussed about specification support of application timing of MAC- CE. To avoid ambiguity during the transition, RAN1 kindly asks RAN2’s opinion about following candidates of latency values of MAC- CE indication. These are candidate values that RAN1 is considering: - Downlink-related: - Time between the ACK transmission for the PDSCH carrying the MAC-CE message and the time that the UE applies the MAC-CE message. Note some of the MAC CE messages may carry QCL information. Following is based on assumption of 120 kHz SCS for slot duration. RAN1 will down select a single value per SCS among the following candidate values, which may be based on UE capability. - For PDSCH - Candidate values (slots): 8, 10, 20, 40, 80 (considering the minimum 10 ms periodicity for TRS in case of 80 slots) - For PDCCH: - Candidate values (slots): 8, 10, 20, 40, 80 - For SP-CSI-RS: - Candidate values (slots): 8, 10, 20, 40, 80 - For aperiodic CSI trigger state subselection: - Candidate values (slots): 8, 10, 20, 40, 80 - Uplink-related: - Time between the ACK transmission for the PDSCH carrying the MAC-CE activation message and the time that the UE applies the MAC-CE message. Following is based on assumption of 120 kHz SCS for slot duration. RAN1 will down select a single value per SCS among the following candidate values, which may be based on UE capability. - For semi-persistent SRS: - Candidate values (slots): 8, 10, 20, 40, 80 - For semi-persistent CSI reporting (on PUCCH): - Candidate values (slots): 8, 10, 20, 40, 80 - For PUCCH-SpatialRelationInfo: - Candidate values (slots): 8, 10, 20, 40, 80 ------end------ Agreement: Agree to the following text proposal for section 6.3.1.1 of 38.211 Section 6.3.1.1 of TS38.211: … where x and y are tags defined in [4, TS38.212] and where the scrambling sequence is given by clause 5.2.1. The scrambling sequence generator shall be initialized with where - equals the higher-layer parameter Data-scrambling-Identity if configured, - otherwise and where corresponds to the RNTI associated with the PUSCH transmission as described in clause 6.1 3GPP TS 38.214 [6]. … Agreement: Agree to the following text proposal for section 6.4.1.1 of 38.211 Section 6.4.1.1 of TS38.211: … where , , and are given by Tables 6.4.1.1.3-1 and 6.4.1.1.3-2 and the following conditions are fulfilled: - the resource elements are within the common resrouce resource blocks allocated for PUSCH transmission … Agreement: Agree to the following text proposal for section 7.3.1.1 of 38.211 ----start---- … Up to two codewords can be transmitted, . In case of single-codeword transmission, . For each codeword , the UE shall assume the block of bits , where is the number of bits in codeword transmitted on the physical channel, are scrambled prior to modulation, resulting in a block of scrambled bits according to where the scrambling sequence is given by clause 5.2.1. The scrambling sequence generator shall be initialized with where - equals the higher-layer parameter Data-scrambling-Identity if configured and the RNTI equals the C-RNTI, - otherwise and where corresponds to the RNTI associated with the PDSCH transmission as described in clause 5.1 3GPP TS 38.214 [6]. ------end------ Agreement: - Correct the name of the RRC parameter for codebook subset restriction. - fullyCoherent is changed into fullAndPartialAndNonCoherent - partialCoherent is changed into partialAndNonCoherent - change the related spec in 38.212 and 38.214 - RRC parameters such that maxRank and codebookSubset are configured for PUSCH, rather than for each SRS resource - Send the above agreement as part of LS to RAN2 Agreement For codebook based UL transmission, - If multiple SRS resources are configured, the number of SRS ports for different SRS resources should be the same Agreement - For codebook based uplink transmission, SRS resources in the same resource set should have the same time domain behavior on periodic, aperiodic and semi-persistent SRS - Send the above as part of LS to RAN2 Agreement For the TPMI/TRI table for 2 antenna ports, support the following spec changes for Table 7.3.1.1.2-4 and Table 7.3.1.1.2-5 for 38.212. Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if PUSCH-tp=Disabled and ULmaxRank = 2 Bit field mapped to index; Fully coherent; Bit field mapped to index; Non-coherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1; 1; 1 layer: TPMI=1. 2; 1 layer: TPMI=2; 2; 1 layer: TPMI=2. 3; 1 layer: TPMI=3; 3; 1 layer: TPMI=3. 42; 2 layers: TPMI=0; 42; 2 layers: TPMI=0. 5; 2 layers: TPMI=1; 5; 2 layers: TPMI=1. 63; 1 layer: TPMI=42; 6-73; Reserved. 74; 1 layer: TPMI=53. 5; 1 layer: TPMI=4. 6; 1 layer: TPMI=5. 7; 2 layers: TPMI=1. 8; 2 layers: TPMI=2. 9-15; reserved. Table 7.3.1.1.2-5: Precoding information and number of layers, for 2 antenna ports, if PUSCH-tp=Disabled, or if PUSCH-tp=Enabled and ULmaxRank = 1 Bit field mapped to index; Fully coherent; Bit field mapped to index; Non-coherent. 0; 1 layer: TPMI=0; 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1; 1; 1 layer: TPMI=1. 2; 1 layer: TPMI=2; 2; 1 layer: TPMI=2. 3; 1 layer: TPMI=3; 3; 1 layer: TPMI=3. 4; 1 layer: TPMI=4. 5; 1 layer: TPMI=5. 6-7; reserved. Agreement: For UL codebook, agree to the following text proposal to add a notation to clarify the waveform for Table 6.3.1.5-4 to Table 6.3.1.5-7 in 38.211. - Table 6.3.1.5-4: Precoding matrix for two-layer transmission using two antenna ports with transform precoding disabled. - Table 6.3.1.5-5: Precoding matrix for two-layer transmission using four antenna ports with transform precoding disabled. - Table 6.3.1.5.4-6: Precoding matrix for three-layer transmission using four antenna ports with transform precoding disabled. - Table 6.3.1.5-7: Precoding matrix for four-layer transmission using four antenna ports with transform precoding disabled. Conclusion: Explicit support of subband precoding for codebook based uplink transmission is not supported in NR Rel-15. Agreement: UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the AP-CSI-RS resource and the first symbol of the AP-SRS transmission is less than 42 OFDM symbols Agreement: Agree to the following text proposal for 7.3.1.1.2 in 38.212. The name of the SRI(s) field header can confirmed and modified if needed by the 38.212 spec editor. ----start---- - SRS resource indicator – or bits, where is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter SRS-SetUse of value 'CodeBook' or 'NonCodeBook', and is the maximum number of supported layers for the PUSCH. - bits for non-codebook based PUSCH transmission according to Table 7.3.1.1.2-a – 7.3.1.1.2-d, where is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter SRS-SetUse of value 'NonCodeBook'; - bits for codebook based PUSCH transmission, where is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter SRS-SetUse of value 'CodeBook'. Table 7.3.1.1.2-a: SRI indication for non-codebook based PUSCH transmission, Bit field mapped to index; SRI(s),; Bit field mapped to index; SRI(s),; Bit field mapped to index; SRI(s),. 0; 0; 0; 0; 0; 0. 1; 1; 1; 1; 1; 1. 2; 2; 2; 2. 3; reserved; 3; 3. Table 7.3.1.1.2-b: SRI indication for non-codebook based PUSCH transmission, Bit field mapped to index; SRI(s),; Bit field mapped to index; SRI(s),; Bit field mapped to index; SRI(s),. 0; 0; 0; 0; 0; 0. 1; 1; 1; 1; 1; 1. 2; 0,1; 2; 2; 2; 2. 3; reserved; 3; 0,1; 3; 3. 4; 0,2; 4; 0,1. 5; 1,2; 5; 0,2. 6-7; reserved; 6; 0,3. 7; 1,2. 8; 1,3. 9; 2,3. 10-15; reserved. Table 7.3.1.1.2-c: SRI indication for non-codebook based PUSCH transmission, Bit field mapped to index; SRI(s),; Bit field mapped to index; SRI(s),; Bit field mapped to index; SRI(s),. 0; 0; 0; 0; 0; 0. 1; 1; 1; 1; 1; 1. 2; 0,1; 2; 2; 2; 2. 3; reserved; 3; 0,1; 3; 3. 4; 0,2; 4; 0,1. 5; 1,2; 5; 0,2. 6; 0,1,2; 6; 0,3. 7; reserved; 7; 1,2. 8; 1,3. 9; 2,3. 10; 0,1,2. 11; 0,1,3. 12; 0,2,3. 13; 1,2,3. 14-15; reserved. Table 7.3.1.1.2-d: SRI indication for non-codebook based PUSCH transmission, Bit field mapped to index; SRI(s),; Bit field mapped to index; SRI(s),; Bit field mapped to index; SRI(s),. 0; 0; 0; 0; 0; 0. 1; 1; 1; 1; 1; 1. 2; 0,1; 2; 2; 2; 2. 3; reserved; 3; 0,1; 3; 3. 4; 0,2; 4; 0,1. 5; 1,2; 5; 0,2. 6; 0,1,2; 6; 0,3. 7; reserved; 7; 1,2. 8; 1,3. 9; 2,3. 10; 0,1,2. 11; 0,1,3. 12; 0,2,3. 13; 1,2,3. 14; 0,1,2,3. 15; reserved. ----end---- Agreement: Agree to the following text proposal for section 5.1.2.3 in 38.214 ----start---- Precoding Resource Block Group (PRGs) of size partition the carrier bandwidth part i and each PRG consists of one or multiple consecutive PRBs., which PRG can be one of the values among {2, 4, scheduled bandwidth}. < Unchanged parts are omitted > When the bundling size is configured as one of the values among {2, 4}, the first PRG size is given by and the last PRG size given by if≠0，and the last PRG size is if =0. ----end---- Agreement: Agree to the following text proposal for section 5.1.2.3 in 38.214 ----start---- If the higher layer parameter prbBundling is set to ‘ON’, the higher-layer parameter pdsch-BundleSize configures two sets of PRG values, the first set includes one or two PRG values among {2,4, scheduled bandwidth}, and the second set includes one PRG value. - If the PRB bundling size indicator signalled in DCI format 1_1 as defined in Subclause 7.3.1.2.2 of [2, TS 38.212] is set to ‘0’, the UE shall use the PRG value from the second set of PRG values. - If the PRB bundling size indicator signalled in DCI format 1_1 as defined in Subclause 7.3.1.2.2 of [2, TS 38.212] is set to ‘1’ and one value is configured for the first set of PRG values, the UE shall use this PRG value. - If the PRB bundling size indicator signalled in DCI format 1_1 as defined in Subclause 7.3.1.2.2 of [2, TS 38.212] is set to ‘1’ and two values are configured for the first set of PRG values as (2, scheduled BW) or (4, scheduled BW), the PRG is determined based on the minimum number of contiguous scheduled PRBs as follows: - If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than , the PRG is the same as the scheduled bandwidth, otherwise, the PRG is set to the remaining configured value of 2 or 4, respectively. - The UE is not expected to be configured with (2, 4). If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than , the PRG is the same as the scheduled bandwidth, otherwise, the PRG is set to the remaining configured value of 2 or 4, respectively. ----end---- Agreement: Agree to the following text proposal for section 7.3.1.6 in 38.211 ----start---- 7.3.1.6 Mapping from virtual to physical resource blocks The UE may assume that the same precoding in the frequency domain is used across a bundle of common resource blocks numbered where and the bundle size is determined by the procedures in Subclause 5.1.2.3 of [38.214].given by the higher-layer parameter PDSCH-bundle-size if configured, otherwise by the DCI scheduling the transmission. The UE shall not make any assumption that the same precoding is used for different bundles of common resource blocks. ----end---- Agreement: PRG for PDSCH carrying RMSI is partitioned from the lowest-numbered resource block in the CORESET configured by the PBCH. Agreement: When the UE is configured with resource block bundle size 2 for VRB to PRB mapping, the UE is not expected to be configured with PRG=4. Agreement: Add the following text proposal to TS 38.214 section 5.2.1.4 ----start---- When a UE is configured with the higher layer parameter ReportQuantity set to ‘CRI/RI/PMI/CQI’, ‘CRI/RI/i1’, ‘CRI/RI/i1/CQI’, ‘CRI/RI/CQI’ or ‘CRI/RI/LI/PMI/CQI’, and multiple resources are configured in the corresponding resource set for channel measurement, the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI. ----end---- Agreement: For the details of CSI-IM periodicity, the candidate values of periodicity and slot offset of CSI-IM are the same as those of NZP CSI-RS - The above agreement applies for both periodic and semi-persistent CSI-IM Agreement: For CSI-IM-ResourceMapping, Include parameters to capture OFDM symbol and subcarrier occupancy of the CSI-IM resource within a slot - Symbol locations: - For RE pattern (2, 2) : [0..12] - For RE pattern (4,1) : [0..13] - Subcarrier locations : - For RE pattern (4,1), the candidate starting subcarriers are: [0 4 8] - For RE pattern (2,2), the candidate starting subcarriers are [0 2 4 6 8 10] Send as part of LS to RAN2 Agreement: Include the following table as part of the LS sent to RAN2. Parameter; Value range; Description. CSI-IM-RE-pattern; Pattern0 or Pattern1; Pattern0: (2,2) Pattern1: (4,1). CSI-IM-FreqBand; Starting RB index in units of 4 RBs Number of spanned RBs in units of 4 RBs Minimum BW = min(24, BWP for data) RBs; Frequency-occupancy of CSI-IM. CSI-IM-ResourceMapping; Subcarrier location : - For RE Pattern0 : {0,2,4,6,8, 10} - For RE Pattern1 : {0,4,8} Symbol location : - For RE Pattern0 : {0,1,...,12} - For RE Pattern1 : {0,1,...,13}; Define symbol/subcarrier location of CSI-IM within a PRB. Agreement: Value range of CSI-IM freqBand is same as that of NZP CSI-RS Send as part of LS to RAN2 Agreement: Agree to the following text proposal for 38.214 in section 5.2.2.4 ----start---- #### 5.2.2.4 Channel State Information – Interference Measurement (CSI-IM) The UE can be configured with a single or more CSI-IM resource configuration(s). The following parameters for which the UE shall assume zero transmission power are configured via higher layer parameter CSI-IM-ResourceConfig for each CSI-IM resource configuration: - CSI-IM-ResourceId determines CSI-IM resource configuration identity - CSI-IM-ResourceMapping defines subcarrier and symbol occupancy of the CSI-IM resource within a slot. - CSI-IM-RE-Pattern defines CSI-IM resource element pattern - CSI-IM-timeConfig defines the CSI-IM periodicity and slot offset for periodic/semi-persistent CSI-IM according to the Table 7.4.1.5.2-6 of [4, TS 38.211]. - CSI-IM-FreqBand includes parameters to enable configuration of frequency-occupancy of CSI-IM. In each of the PRBs configured by CSI-IM-FreqBand, the UE shall assume each CSI-IM resource is located in - resource elements , ,and , if CSI-IM-RE-Pattern is set to ‘Pattern0’; - resource elements , ,and , if CSI-IM-RE-Pattern is set to ‘Pattern1’; where and are the configured frequency-domain location and time-domain location, respectively, by higher layer parameter CSI-IM-ResourceMapping. ----end---- Agreement: Density of CSI-IM resource measured in RE/PRB is 4 Agreement: CSI-IM REs are not rate matched around unless they are covered by ZP CSI-RS resources. Note that the above agreement has no additional specification impact Agreement: Triggering A-IMR without associated CMR(s) is not supported. Agreement: Semi-persistent ZP CSI-RS is supported, and its activation/deactivation is informed by MAC CE. Include as part of LS to RAN2 Agreement: For both channel measurement restriction and interference restriction (RRC parameters MeasRestrictionConfig-time-channel, MeasRestrictionConfig-time-interference), value range of MR is a Boolean to indicate ‘ON’ or ‘OFF’. - When UE is configured with ‘ON’, UE measurement window is 1 slot Include as part of LS to RAN2 Agreement: Agree to the following text proposal for 38.214 in section 5.2.2.1: ----start---- If a UE is not configured with higher layer parameter MeasRestrictionConfig-time-channel, the UE shall derive the channel measurements for computing CSICQI value reported in uplink slot n and corresponding to the MeasLinkConfig, based on only the non-zero power CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) within a configured MeasLinkConfig associated with the CSI resource setting. If a UE is configured with higher layer parameter MeasRestrictionConfig-time-channel, the UE shall derive the channel measurements for computing CQI value reported in uplink slot n and corresponding to the MeasLinkConfig, based on only the most recent, no later than the CSI reference resource, occasion of non-zero power CSI-RS (defined in [4, TS 38.211]) within a configured MeasLinkConfig associated with the CSI resource setting. If a UE is not configured with higher layer parameter MeasRestrictionConfig-time-interference, the UE shall derive the interference measurements for computing CQI value reported in uplink slot n and corresponding to the MeasLinkConfig in the CSI measurement setting, based on only the configured CSI resource setting, no later than the CSI reference resource, within a configured MeasLinkConfig associated with the CSI resource setting. If a UE is configured with higher layer parameter MeasRestrictionConfig-time-interference the UE shall derive the interference measurements for computing the CQI value reported in uplink slot n and corresponding to the MeasLinkConfig in the CSI measurement setting, based on the most recent, no later than the CSI reference resource, occasion of CSI-IM and/or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) within configured MeasLinkConfig associated with the CSI resource setting. ----end---- Agreement: Aperiodic CSI-RS cannot be transmitted before the OFDM symbol carrying its triggering DCI. Following text proposal for 38.214 in section 5.2.1.5.1 is agreed: ----start---- When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set in the higher layer parameter AperiodicNZP-CSI-RS-TriggeringOffset. The CSI-RS triggering offset X is measured in slots. The UE does not expect that aperiodic CSI-RS is transmitted before the OFDM symbol(s) carrying its triggering DCI. ----end---- Agreement: Agree to the following text change for TS 38.214 section 5.2.1.5.2 (Semi-persistent CSI): ----start---- For a UE configured with the higher layer parameter ResourceConfigType set to ‘semi-persistent’. - when a UE receives an activation command [10, TS 38.321] for CSI-RS resource(s) for channel measurement and CSI-IM/NZP CSI-RS resource(s) for interference measurement associated with configured CSI resource setting(s) in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions (including quasi-co-location assumptions provided by a reference to a TCI-RS-SetConfig) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied no later than the minimum requirement defined in [10, TS 38.133]. - when a UE receives a deactivation command [10, TS 38.321] for activated CSI-RS/CSI-IM resource(s) associated with configured CSI resource setting(s) in slot n, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of CSI-RS/CSI-IM transmission corresponding to the deactivated CSI-RS/CSI-IM resource(s) shall apply no later than the minimum requirement defined in [10, TS 38.133]. - ----end---- Agreement: Agree to the following text change for TS 38.214 section 5.2.1.2: ----start---- 5.2.1.2 Resource settings The following are configured via higher layer signaling for one or more CSI resource settings for channel and interference measurement: - CSI-IM resource for interference measurement as described in Subclause 5.2.2.43.3. - Non-zero power CSI-RS resource for interference measurement as described in Subclause 5.2.2.3.1. [see Table [TBD] in [4, TS 38.211] ] - Non-zero power CSI-RS resource for channel measurement as described in Subclause 5.2.2.3.1. [see Table [TBD] in [4, TS 38.211] ] - Non-zero power CSI-RS resource for channel measurement as described in Subclause 5.2.2.3.1. [see Table [TBD] in [4, TS 38.211] ] The UE may assume that the CSI-RS resource(s) for channel measurement and the CSI-IM/NZP CSI-RS resource(s) for interference measurement configured for one CSI reporting are quasi co-located with respect to spatial Rx parameters. ----end---- Agreement Delete the following text from 38.214 section 5.2.2.1 For the CQI derivation the UE assumes the same quasi co-location properties between the CSI-IM and/or non-zero power CSI-RS configured for interference measurements and the non-zero power CSI-RS configured for channel measurements. Agreement: Agree to the following text change in 38.214 section 5.2.2.4 - CSI-IM RE pattern configured by csi-IM-ResourceElementPattern as given in Subclause TBD7.4.1.5.3 of [4, TS 38.211]. Agreement Agree to the following text proposal for 38.214 section 5.2.1.5.1 ----start---- For Resource Sets CSI-RS resource sets associated with Resource Settings configured with the higher layer parameter ResourceConfigType set to 'aperiodic', trigger states for Reporting Setting(s) and/or Resource SettingSet(s) for channel and/or interference measurement on one or more component carriers are configured using the higher layer parameter AperiodicReportTrigger. For aperiodic CSI report triggering, a single set of CSI triggering states are higher layer configured, wherein the CSI triggering states can be associated with either candidate DL BWP. A UE is not expected to be triggered with a CSI report for a non-active DL BWP. A trigger state is initiated using the DCI CSI request field. - When the value of the DCI CSI request field is zero, no CSI is requested. - When the number of configured CSI triggering states in AperiodicReportTrigger is greater than , where is the number of bits in the DCI CSI request field, the UE receives a selection command [10, TS 38.321] used to map up to trigger states to the codepoints of the DCI CSI request field. is configured by the higher layer parameter ReportTriggerSize and . - When the number of CSI triggering states in AperiodicReportTrigger is less than or equal to , the DCI CSI request field directly indicates the triggering state and the UE's quasi-colocation assumption. - For each aperiodic CSI-RS resource associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in subclause 5.1.5, through higher layer signaling of QCL-Info-aPeriodicReportingTrigger which contains a list of references to TCI-RS-SetConfig's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a TCI-RS-SetConfig in the list is configured with a reference to an RS associated with QCL-TypeD, that RS may be an SS/PBCH block or a CSI-RS resource configured as periodic or semi-persistent. For a UE configured with the higher layer parameter AperiodicReportTrigger, if a resource setting Resource Setting linked to a ReportConfig has multiple aperiodic resource sets and only a subset of the aperiodic CSI-RS resource sets is associated with the trigger state, a higher layer configured bitmap ResourceSetBitmap is configured per trigger state per resource setting Resource Setting to select the CSI-IM/NZP CSI-RS resource set(s) from the resource setting Resource Setting. … ----nd---- Agreement Agree to the following text proposal for 38.214 section 5.2.3 ----start---- ### 5.2.3 CSI reporting using PUSCH A UE shall perform aperiodic CSI reporting using PUSCH in slot n+Y on serving cell c upon successful decoding in slot n of an uplink DCI format for serving cell c, where Y is indicated in the decoded uplink DCI. The higher layer parameter AperiodicReportSlotOffset contains the allowed values of Y for a given Reporting Setting. When reports are scheduled, let be the ith allowed value for Report Setting j (). Then the ith codepoint of the DCI field corresponds to the allowed value . The number of reports is configured for each trigger state (section 5.2.1.5.1) and corresponds to the number of Reporting Settings for the trigger state. … ----end---- Agreement: The following CSI priority rules apply: - Reports are first prioritized according to Rule #1, then according to Rule #2 if they have the same priority according to Rule #1, and so forth… - Rule #1: Time-domain behaviour / channel (AP-CSI > SP-CSI on PUSCH > SP-CSI on PUCCH > P-CSI) - Rule #2: CSI content (Beam reports > CSI) - Applies only for all periodic reports and semi-persistent reports intended for PUCCH - Rule #3: cellID (PCell > PSCell > other cell IDs in increasing order) - Applies only for all periodic reports and semi-persistent reports intended for PUCCH - Rule #4: csiReportID (in increasing order) - Applies only for all periodic reports and semi-persistent reports intended for PUCCH Agreement: - The terminologies for Low Complexity CSI and High Complexity CSI are changed to Low Latency CSI and High Latency CSI, respectively - For CSI computation capability, - Two CSI latency classes are supported. - Low Latency CSI class is defined as WB CSI including maximum 4 ports - Only applicable for Type-I codebook or when PMI is not configured - High Latency CSI class is defined as the superset of all CSI that is supported by the UE - Above does not apply for L1 RSRP. Agreement: - When CSI is transmitted on PUSCH, SLIV and PUSCH mapping type is determined from pusch-symbolAllocation in the same way as for PUSCH without CSI - PUSCH slot offset when CSI is multiplexed with UL-SCH on PUSCH is determined only from the indicated K2 value from pusch-symbolAllocation, not from aperiodicReportSlotOffset - The above only applies for the case where CSI is multiplexed with data - Note: The number of candidate values for aperiodicReportSlotOffset and K2 are the same Agreement: Support additional periodicities of {4, 8, 16} slots for periodic on PUCCH and semi-persistent CSI reporting on PUCCH and PUSCH. Include the above agreement as part of LS to RAN2 Agreement: - Clarify the frequency-granularities as follows: - Remove mentioning of “partial band” CSI reporting - Define “wideband CSI” as a CSI report with either wideband PMI+wideband CQI, RSRP, or wideband CQI without PMI - Other cases are “subband CSI” - For the first sub-bullet, it is up to the 38.212, 38.214 editors to make the correction Agreement: Agree to the following text proposal for 38.212 section 6.3.1.1.2 Table 6.3.1.1.2-8: Mapping order of CSI fields of one report for CRI/RSRP or SSB/RSRP reporting CSI report number; CSI fields. CSI report #n; CRI or SSB index#1 as in Table 6.3.1.1.2-6, if reported. CRI or SSB index#2 as in Table 6.3.1.1.2-6, if reported. CRI or SSB index#3 as in Table 6.3.1.1.2-6, if reported. CRI or SSB index#4 as in Table 6.3.1.1.2-6, if reported. RSRP#1 as in Table 6.3.1.1.2-6, if reported. RSRP#2 as in Table 6.3.1.1.2-6, if reported. Differential RSRP#2 as in Table 6.3.1.1.2-6, if reported. Differential RSRP#3 as in Table 6.3.1.1.2-6, if reported. Differential RSRP#4 as in Table 6.3.1.1.2-6, if reported. Agreement: - For CRI/RI/i1 reporting mode, the selected precoders for CQI calculation are within the set of precoders indicated by i1 to be reported Agreement: Make the following text change: Replace “RI/CRI” with “RI (if reported), CRI (if reported)” throughout 38.214 Agreement: When CSI reports originally configured for PUCCH reporting is piggybacked on PUSCH, - The CSI encoding scheme and omission follows that of PUCCH Agreement: Make the following text change: Align terminology for PTRS strongest layer indicator - Change “SLI” to “LI” in 38.214 Agreement: Type II CSI Part 1 on long PUCCH is supported for - Only SP-CSI reporting Agreement: Define explicit rule to calculate the number of padding bits in section 6.3.1.1 of TS 38.212 Agreement: - Clarify in 38.214 that the following dependencies apply for CSI parameters (if they are reported) - LI is calculated conditioned on the reported CQI, PMI, RI and CRI - CQI is calculated conditioned on the reported PMI, RI and CRI - PMI is calculated conditioned on the reported RI and CRI - RI is calculated conditioned on the reported CRI Agreement: Capture the following sentence in the section 5.2.1.5.1 of TS 38.214: A UE is not expected to receive more than one aperiodic CSI report request for a given slot. - Note: 38.214 editor may change the exact wording to reflect the above agreement Agreement: - For each PUCCH-based CSI Report, a PUCCH resource for each candidate UL BWP is configured, where the configuration of a resource contains PUCCH format (2,3, or 4), and the necessary parameters to define the PUCCH resource - The parametrization follows that of each PUCCH Format Include as part of LS to RAN2 Agreement: - Support configuring the UE with J>=1 PUCCH resource configuration per UL BWP candidate used for carrying multiple CSI reports (associated with a PUCCH resource config (Format 2/3/4 and its Maximum Code rate)) - In case the PUCCH resources for two or more PUCCH-based CSI reports collide (at least partially overlap in time), the colliding CSI reports with the highest priorities are carried in a multi-CSI PUCCH resource and remaining CSI reports are dropped - The number of included CSI reports is determined by the configured maximum code rate of the multi-CSI PUCCH resource - This applies to CSI only transmission on PUCCH, i.e. not multiplexed with HARQ-ACK - The PUCCH resource for carrying multiple CSI reports does not need to be configured to a UE - FFS if periodicity of multi-CSI resource needs to be defined - FFS value of J - Exact mechanism TBD in RAN1#92 Include as part of LS to RAN2 Agreement: - Subband boundaries are aligned with PRG boundaries in the CRB grid - The first subband size is given by and the last subband size given by if and S if , where - S is the configured subband size for the BWP - is the BWP starting position in the CRB grid - is the BWP bandwidth - The value range of csi-ReportingBand is a bitmap of size where is the bandwidth of the associated DL CC and S is the subband size. - Configuration of csi-ReportingBand with subbands which lie outside the associated BWP is not allowed - Note: RAN2 may decide to make this optional and if this field is not present, the reporting band is the same as the CSI-RS bandwidth Include as part of LS to RAN2 Agreement: - For SS block + CSI-RS independent L1 RSRP reporting - Reports are separately configured in different report settings - No new value for the RRC parameter ReportQuantity is defined Agreement: - Maximum number of candidate TCI states is M_max. Down-select to one of the following two alternatives: - Alt-1: M_max = 64 - Note that the value M_max is for configuration of TCI states only - Relationship between RRC configuration of TCI states and bandwidth parts is decided by RAN2 Include as part of LS to RAN2 Agreement: QCL source for a target semi-persistent CSI-RS resource set is provided by TCI states in the same MAC-CE at resource level - Include as part of LS to RAN2 FFS: Check whether text proposal is needed in RAN1 specification: Steve (Ericsson) Agreement Spatial relationship for a target semi-persistent SRS resource set is provided by SSB-ID/SRS resource ID/CSI-RS resource ID in the same MAC-CE at resource level - Include as part of LS to RAN2 FFS: Check whether text proposal is needed in RAN1 specification: Steve (Ericsson) Agreement: The same priority rules are adopted for PUCCH/SRS when PUCCH carries beam management RSRP reports as for PUCCH carrying CSI reports Agreement: - For beam management, triggering of N>1 aperiodic CSI-RS resource sets with different slot offsets with a single DCI message is NOT supported Agreement: - Maximum number of candidate TCI states configured for a CORESET is K_max - K_max = M - Note that the value M is for configuration of TCI states only - Note: UE is not expected to track the K configured TCI states. The value of K is for configuration of TCI states only. Include as part of LS to RAN2 Agreement: Include in LS to RAN2 in response to R1-1800003: “These are candidate values that RAN1 is considering.” Downlink-related: - Time between the ACK transmission for the PDSCH carrying the MAC-CE activation message and the time that the UE is able to apply the QCL assumption carried by the MAC-CE message. Following is based on assumption of 120 kHz SCS for slot duration. RAN1 will down select a single value per SCS among the following candidate values which may be based on UE capability. - For PDSCH - Candidate values (slots): 8, 10, 20, 40, 80 (considering the minimum 10 ms periodicity for TRS in case of 80 slots) - For PDCCH: - Candidate values (slots): 8, 10, 20, 40, 80 - For SP-CSI-RS: - Candidate values (slots): 8, 10, 20, 40, 80 Uplink-related: - FFS Agreement: For beam failure detection model, PHY performs detection of beam failure instances, and indicates a flag to higher layer if a beam failure instance is detected - FFS: When/Whether PHY needs to report candidate beam list and beam failure instance to MAC - FFS: Whether non-beam failure instance is defined or is needed Include as part of LS to RAN2 Agreement: Change candidate beam selection model to the following alternatives: - PHY performs L1-RSRP evaluation of each candidate new beam, provides to higher layer the subset of {beam RS index, L1-RSRP measurements} that satisfies the L1-RSRP threshold - RAN 1 expects higher layer to perform new candidate beam selection based on the subset of {beam RS index, RSRP measurements} - Note: The mapping between beam RS index(es) to PRACH resource(s)/sequence(s) is done in MAC - Support for candidate beam selection model is specified in the RAN2 specifications Include as part of LS to RAN2 Agreement: Behavior of Beam-failure-recovery-Timer - Start Beam-failure-recovery-Timer upon beam failure detection event declared by UE - Stop Beam-failure-recovery-Timer upon reception of gNB response for beam failure recovery request transmission Include as part of LS to RAN2 Agreement: - From RAN1 perspective, contention-free PRACH-based beam failure recovery is considered unsuccessful when one of the following conditions is met - Upon expiry of Beam-failure-recovery-Timer - Upon reaching max. # of BFRQ transmissions Include as part of LS to RAN2 Agreement: - Indication of beam failure instance to higher layer is periodic and indication interval is determined by the shortest periodicity of BFD RS , which is also lower bounded by [10] ms. - Note: if the evaluation is below beam failure instance BLER threshold, there is no indication to higher layer. - PHY provides to higher layer one or more sets of {beam RS index, L1-RSRP measurement} that satisfies the L1-RSRP threshold upon higher layer request. Agreement: The following text in sub-band differential CQI should be changed in TS 38.214 section 5.2.2.1: For each sub-band index s, a 2-bit sub-band differential CQI is defined as: - Sub-band Offset level (s) = wideband CQI index – sub-band CQI index (s) The mapping from the 2-bit wideband differential CQI values to the offset level is shown in Table 5.2.2.1-1 Table 5.2.2.1-1: Mapping spatial sub-band differential CQI value to offset level spatial sub-band differential CQI value; Offset level. 0; 0. 1; 1. 2; ≥ 2. 3; ≤-1. Agreement: Make the following text change for CQI assumption definition to 38.214 “The bandwidth as configured for the PDSCH reception the corresponding CQI report”. Agreement: Agree to the following text proposal for 38.214 section 5.2.2.1.1 In the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, PMI and RI: - The first 2 OFDM symbols are occupied by control signaling - The number of PDSCH and DM-RS symbols is equal to 12. Agreement: Agree to the following text proposal for 38.214 section 5.2.2.1.1 regarding the CQI assumption: Assume PRB bundling size of 2 PRB Assume no PTRS overhead Agreement: The work assumption of reusing the LTE CQI table for maximum modulation order of 256QAM for eMBB is confirmed Agreement: The following text proposal is agreed for Section 5.2.2.3.1 of 38.214 The UE may be configured the CSI-RS in same OFDM symbols as SS/PBCH block, but not in the same PRBs. The UE may be configured the CSI-RS in same OFDM symbols as SS/PBCH block, but not in the same PRBs, if the higher layer parameter NrofPorts is configured as 1 or 2. Agreement: The following text proposal is agreed for Section 5.1.6.1.2 of 38.214: The UE may be configured to use the same OFDM symbols for the CSI-RS and SSB/PBCH when those are spatially quasi co-located and resource elements PRBs associated with CSI-RS are the outside of PRBs configured for SSB/PBCH. Agreement: The following text proposal is agreed for Section 5.2.2.3.1 of 38.214: The UE may be configured the CSI-RS in same OFDM symbols as CORESET, but not in the same PRBs. The following text proposal is agreed for Section 5.1.6.1.2 of 38.214: The UE may be configured to use the same OFDM symbols for the CSI-RS and CORESET when those are spatially quasi co-located and PRBs associated with CSI-RS are the outside of PRBs configured for CORESET. Agreement: - For multiplexing between CSI-RS and CORESET, follow the current description in TS38.214 and remove the corresponding parts from TS38.211, which means that the bandwidths of CSI-RS and CORESET/SSB shall be configured exclusively (i.e. no assumptions on CSI-RS RE puncturing at UE side). Text proposal: in Section 7.4.1.5.3 of TS38.211 ##### 7.4.1.5.3 Mapping to physical resources For each CSI-RS component configured, the UE shall assume the sequence being mapped to physical resources according to Resource elements overlapping with a configured CORESET or declared as 'reserved' according to clause 4.4.3 shall be counted in the mapping process but not assumed to be used for transmission of CSI-RS. Agreement: Accept the following modification of Section 7.4.1.5.3 in 38.211: The UE is not expected to receive a CSI-RS and DMRS on the same resource elements. in resource elements overlapping with configured DM-RS Conclusion: On the support of indicating two starting OFDM symbol positions for non-contiguous NZP CSI-RS mapping, include as part of an LS to RAN2 to inform them that the RRC signalling should support indication of up to two starting OFDM symbol positions and it is up to RAN2 to decide on the detailed signalling (relevant RRC parameter: CSI_RS_resourcemapping) Agreement: Remove Pc-PDCCH from RRC parameter list for NZP-CSI-RS and add clarification on 0dB power offset between CSI-RS for beam failure detection and PDCCH DMRS R1-1801096 Feature lead summary #2 of CSI-RS Huawei, HiSilicon Agreement: - Indication mechanism of aperiodic ZP CSI-RS is RRC + DCI - Each triggering state of “ZP CSI-RS trigger” in DCI is to trigger one ZP CSI-RS resource set - A state is reserved for not triggering aperiodic ZP CSI-RS - All the resources in a ZP CSI-RS resource set are configured with the same ResourceConfigType (periodic or aperiodic) - The bit-length of DCI field “ZP CSI-RS trigger” depends on the number of aperiodic ZP CSI-RS resource sets configured (up to 2 bits) - The maximum number of aperiodic ZP CSI-RS resource sets configured per BWP is 3 Include as part of LS to RAN2 Agreement: - Add the following RRC parameters for ZP CSI-RS PDSCH; 38.214; ZP-CSI-RS-ResourceSetConfigList; New; ZP-CSI-RS-ResourceSetConfigList; Resource set configuration for ZP CSI-RS; NA; UE specific; 38.331; Y. PDSCH; 38.214; ZP-CSI-RS-ResourceSetConfigId; New; ZP-CSI-RS-ResourceSetConfigId; ZP CSI-RS Resource set configuration ID; 0 .. ZP-CSI-RS-ResourceSetMax-1; NA; UE specific; 38.331; Contained in ZP-CSI-RS-ResourceSetConfig; Y. - Update the RRC parameter Aperiodic-ZP-CSI-RS-Resource-List as follows Aperiodic-ZP-CSI-RS-Resource-List; Contains list of ZP-CSI-RS resource set IDs for aperiodic triggering; Up to 3 ZP-CSI-RS resource set IDs per BWP. Include as part of LS to RAN2 Agreement: - Introduce additional periodicities of {4,8,16,32,64} slots and the corresponding slot offsets to at least the following periodic/semi-persistent RS: - CSI-RS (includes ZP-CSI-RS and NZP-CSI-RS) - SRS - FFS on whether and how to align to additional periodicities as agreed in the frame structure agenda - “The two periodicities form X ms + Y ms total periodicity, where X, and Y are from {0.5, 0.625, 1, 1.25, 2, 2.5, 5, 10} ms” - FFS on CSI-IM periodicity of 1 slot Include as part of LS to RAN2 Working Assumption Agree to the following text for TS.38.211 with the understanding that the 1 port rate=1/2 case needs to be checked. If there is an issue with the working assumption for 1 port rate=1/2, technical details will be modified. 7.4.1.5.3 Mapping to physical resources For each CSI-RS component configured, the UE shall assume the sequence being mapped to physical resources according to under the condition that the resource elements indexed by are within the resource blocks occupied by the CSI-RS for which the UE is configured. The reference point for is subcarrier 0 in common resource block 0. The value of is given by the higher-layer parameter CSI-RS-Density. Agreement: - By default, UE does not perform rate matching on REs overlapped with at least CSI-RS for mobility, which is not configured with serving cell ID - UE shall perform rate matching on REs overlapped with a CSI-RS for mobility only if ZP-CSI-RS covers the REs overlapped with the CSI-RS for mobility. - FFS, rate matching on REs overlapped with CSI-RS for mobility configured with serving cell ID Conclusion: Update the RRC parameters of DMRS as follows: Specify two scrambling IDs for CP-OFDM PDSCH’s and PUSCH’s DMRS, namely: DL-DMRS-Scrambling-ID-1 and DL-DMRS-Scrambling-ID-2, UL-DMRS-Scrambling-ID-1 and UL-DMRS-Scrambling-ID-2, for DL and UL, respectively. Send the above as part of LS to RAN2 Agreement: Remove from the equation in Section 5.2.2 in 38.211. adding to the corresponding formula similar to CP-OFDM. Agreement: The following working assumption is confirmed: For DMRS sequence of PDSCH/PUSCH CP-OFDM: - for PDSCH carrying RMSI, the DMRS sequence generation - uses the lowest PRB of CORESET signaled in the PBCH - for PDSCH/PUSCH before RRC configuration, and for PDSCH/PUSCH after RRC configuration, the DMRS sequence generation - uses the lowest subcarrier of the reference PRB [point A] (i.e. PRB0 in previous agreements) - Note: this implements the previous agreement “resource specific w.r.t to a wideband CC from network perspective”. Agreement: Agree on the following text proposal in Sect 6.4.1.1.3, in 38.211: “If transform precoding is not enabled, the reference point for is subcarrier 0 in common resource block 0, otherwise the subcarrier 0 of the lowest-numbered resource block of the scheduled PUSCH allocation.” Agreement: Capture the following description in the last paragraph of section 6.2.2 in 38.214. - If a UE transmitting PUSCH is configured with the higher layer parameter UL-PTRS-present, the UE may assume that the following configurations are not occurring simultaneously for the transmitted PUSCH. - any DM-RS ports among 1004-1007 or 1006-1011 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and PT-RS is transmitted from the UE. Agreement: Agree to the following text proposal: For section 7.3.1.1.2 in 38.212: - DMRS sequence initialization – 0 if the higher layer parameter PUSCH-tp= Enabled or 1 bit if the higher layer parameter PUSCH-tp= Disabled for selection defined in section 7.4.1.1.1 of TS38.211. For section 7.3.1.2.2 in 38.212: - DMRS sequence initialization – 1 bit for selection defined in section 7.4.1.1.1 of TS38.211. Agreement: Agree to the following text proposal for Sect 7.4.1.1.2, in 38.211: For PDSCH mapping type B with duration of 2, 4, or 7 OFDM symbols, and the PDSCH allocation collides with resources reserved for a CORESET, shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET. - If PDSCH duration is 4 symbols, UE does not expect to receive a DM-RS symbol beyond the third symbol, - If PDSCH duration is 7 symbols, UE does not expect to receive the front-load DM-RS beyond the forth symbol - If one additional DM-RS is configured, it may only be transmitted in the 5th or 6th symbol with respect to the front-load DMRS symbol, otherwise it is not transmitted. For PDSCH mapping type B, if the PDSCH duration is 2 or 4 OFDM symbols, only single-symbol DM-RS is supported. Agreement: Agree to the following text proposal for Sect 5.1.6.2 in 38.214: The UE is not expected to assume co-scheduled UE(s) with different DMRS configuration with respect to the actual number of front-loaded DMRS symbol(s), the number of additional DMRS, the DMRS symbol location, and DMRS configuration type as described in Subclause 7.4.1.1 of [4, TS 38.211]. Agreement: Agree to the following text proposal and the agreement should be part of LS to RAN2: For Section 6.4.1.1.1 in 38.211: If transform precoding for PUSCH is enabled, the reference-signal sequence shall be generated according to where is given by clause 5.2.2 with . The sequence group , where is , if is given by higher layer parameter nPUSCH-Identity-Transform-precoding, otherwise . - If neither group hopping, nor sequence hopping shall be used, , . - If group hopping but not sequence hopping shall be used, , where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized with at the beginning of each radio frame. - If sequence hopping but not group hopping shall be used, for otherwise where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized with at the beginning of each radio frame. Agreement: TP for section 64.1.1.3: Update the formula of mapping the sequence in Sect 6.4.1.1.3, in 38.211 when transform precoding is enabled by adding to the corresponding formula similar to CP-OFDM. Conclusion Specification should be revised to place the UL DMRS precoding after physical resource mapping and update UL DMRS port definition Relevant tdoc# R1-1801242, R1-1801035, and R1-1801093 Working Assumption Text proposal for Section 6.2.2 of 38.214 6.2.2 UE DM-RS transmission procedure < Unchanged parts are omitted > If frequency hopping flag as defined in Subclause 7.3.1.1.2 of [5, TS 38.212] is enabled for PUSCH, the UE shall transmit either one single-symbol front-load DM-RS symbol when the higher layer parameters UL-DMRS-add-pos=0 in each hop, otherwise single-symbol front-load DM-RS symbol with one single-symbol additional DM-RS symbol in the 5th symbol with respect to the front-load DMRS symbol in each hop if within the PUSCH allocation of each hop. < Unchanged parts are omitted > Agreement: Value range for MCS thresholds (for time density) are 0-28 for MCS table 1 and 0-27 for MCS table 2 - The above applies for CP-OFDM only Note: the above agreement should be part of LS to RAN2 Note: Above agreement has been revised below Agreement: Revise the above agreement to: Value range for MCS thresholds (for time density) are 0-29 for MCS table 1 and 0-28 for MCS table 2 Note: The above agreement should be part of LS to RAN2 Agreement: Value range for Scheduled BW thresholds (for frequency density for CP-OFDM and sample density for DFTS-OFDM) is between minimum value of 1 and maximum value of 276 FFS: Compression method for reducing the RRC overhead Note: the above agreement should be part of LS to RAN2 Agreement: The RRC parameter epre-RATIO is assigned per PT-RS port for downlink Note: the above agreement should be part of LS to RAN2 Agreement: Revise the first row of Table 6.4.1.2.2.2-1 in TS 38.211 as follows Number of PT-RS groups; Number of samples per PT-RS group; Index of PT-RS samples in OFDM symbol prior to transform precoding. 2; 2; where. Agreement: Revise the row of Table 6.4.1.2.2.2-1 in TS 38.211 as follows 4; 2; where. Agreement: Remove PTRS multiplication by factor in clause 6.4.1.2.2.2 in 38.211 Note: It is up to the editor to check and remove/keep in other parts of the specification Agreement: In the PT-RS symbol mapping for DFT-s-OFDM, K=1,X=16 is not supported in NR Release 15. Agreement: Agree to the following text proposal in Table 4.1-2 of TS 38.214: Change the header “The number of PDSCH layers within the DMRS group associated with the PT-RS” to “The number of PDSCH layers within the DMRS port group containing DMRS port associated with the PT-RS port”. Agreement: For PTRS for CP-OFDM in both DL and UL, ptrs-MCS4 equals 29 for MCS table 1 and 28 for MCS table 2. Remove ptrs-MCS4 in related RRC parameters Include in LS to RAN2 Agreement: For DFT-s-OFDM, if PT-RS is enabled by UL-PTRS-present-transform-precoding, then the UE also expect configuration of the parameters (UL-PTRS-pre-DFT-density). Hence, there is no default sample density table for DFT-s-OFDM as in the CP-OFDM case. Agreement: The text in 7.4.1.2.2 of 38.211 is changed as below. Note: the text in 6.4.1.2.2.1 of 38.211 should also be changed in the same manner. The set of time indices defined relative to the start of the PDSCH allocation is defined by - set and 2. if any symbol from to overlaps with a symbol used for DM-RS according to clause 7.4.1.1.2 - set - set to the number of the last DM-RS symbol in a sequence of time-contiguous DM-RS occasions - repeat from step 2 above as long as is inside the PDSCH allocation. 3. add to the set of time indices for PT-RS 4. increment by one 5. repeat from step 2 above as long as is inside the PDSCH allocation where . Agreement: Remove the sentence “Resource elements declared as 'reserved' according to clause 4.4.3 shall be counted in the mapping process but not used for transmission.” in Section 6.4.1.2.2.1 Agreement: Agree to the following text proposal for Section 7.4.1.2.2 If present, the UE shall assume the PDSCH PT-RS being mapped to resource elements according to when all the following conditions are fulfilled - is within the OFDM symbols allocated for the PDSCH transmission - resource element is not used for DM-RS, CSI-RS, SS/PBCH or by a configured CORESET Agreement Agree to the following text proposal for section 6.4.1.2.1.2 of TS38.211 - If transform precoding is enabled, the phase-tracking reference signal rm(m’) to be mapped in position m before transform precoding, where m depends on the number of PT-RS groups NgroupPTRS, the number of samples per PT-RS group Nsampgroup, and MscPUSCH according to Table 6.4.1.2.2.2-1, shall be generated according to where the pseudo-random sequence c(i) is defined in clause 5.2.1, and w(i) is given by Table 6.4.1.2.1.2-1. - Note: The editor is respectfully asked to crosscheck and harmonize the PUSCH pi/2 BPSK generation in clause 5.1.1 to ensure that pi/2 BPSK property is maintained in transitions between PUSCH and PTRS samples Agreement: Agree to the following text proposal for section 6.4.1.2.2.2 of TS38.211 - The set of time indices l defined relative to the start of the PUSCH allocation is defined by 1. set i=0 and lref=0 2. if any symbol from max(lref+(i-1)LPTRS+1, lref) to lref+iLPTRS overlaps with a symbol used for DM-RS according to clause 6.4.1.1.2 - set i=1 - set lref to the number of the last DM-RS symbol in a sequence of time-contiguous DM-RS occasions - repeat from step 2 above as long as lref+iLPTRS is inside the PUSCH allocation. 3. add lref+iLPTRS to the set of time indices for PT-RS 4. increment i by one 5. repeat from step 2 above as long as lref+iLPTRS is inside the PUSCH allocation where LPTRS is given by the higher-layer parameter UL-PTRS-time-density-transform-precoding. Agreement Agree to the following text proposal for section 6.2.3 in TS38.214. The text proposal to update for the non-coherent case, partial coherent case, non-codebook case if needed. For PT-RS, the transmit power of PTRS is derived from , which is the power ratio between power of PUSCH and power of PTRS per port. For codebook based coherent uplink transmission, when the UE is scheduled with one PTRS port in uplink and the number of scheduled layers is , - If the UE is configured with higher layer parameter UL-PTRS-power, the PUSCH to PT-RS power ratio per layer per RE is given by [dB] Where is shown in the Table 6.2.3-5 according to the higher layer parameter UL-PTRS-power. - The UE shall assume UL-PTRS-power is set to state “00” in Table 6.2.3-5 if not configured. - Table 6.2.3-5: Factor related to PUSCH to PTRS power ratio per layer per RE () UL-PTRS-power /; The number of PUSCH layers ( ). 1; 2; 3; 4. 00; 0; 3; 4.77; 6. 01; reserved. 10; reserved. 11; reserved. Conclusion Make decision on the following issue in RAN1#92: For UCI on PUSCH without UL-SCH, the PT-RS time domain density for CP-OFDM Agreement If a UE is configured with the higher layer parameter Downlink-PTRS-Config, set to ‘ON’, if the additional higher layer parameters timeDensity and frequencyDensity are both configured, the UE shall assume the PT-RS antenna ports’ presence and pattern are a function of the scheduled MCS of the corresponding CW and scheduled bandwidth in corresponding bandwidth part as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2. Agreement If the TCI-PresentinDCI is set as “Disabled”, the scheduled number of PT-RS ports for a UE PDSCH transmission is indicated by the TCI state applied for the CORESET used for the PDCCH transmission that schedules the PDSCH. If the offset is less than a threshold, the scheduled number of PT-RS ports for a UE PDSCH transmission is indicated by the TCI state applied for the CORESET with the lowest CORESET-ID in the latest slot in which one or more CORESETs are configured for the UE. Agreement: The bitwidth of IE in DCI for indicating PT-RS to DMRS port association for UL in CP-OFDM is 0,1 or 2 bits, taking into account the number of SRS ports, maximum ranks supported, and number of PT-RS ports. Agreement Agree to the following text proposal for 38.211 Section 6.4.1.4.2 - if SRS-GroupSequenceHopping equals ‘sequenceHopping’, sequence hopping but not group hopping shall be used and where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized with . Agreement: Agree to the following text proposal for TS 38.211 Section 6.4.1.4.2 The sounding reference signal sequence for an SRS resource shall be generated according to where is given by cause 6.4.1.4.3, is given by clause 5.2.2 and the transmission comb number is contained in the higher-layer parameter SRS-TransmissionComb. The cyclic shift for antenna port is given as , where is given by the higher layer parameter SRS-CyclicShiftConfig. The maximum number of cyclic shifts is if and if . Agreement: Agree on the following text proposal for TS 38.214 Section 6.2.1 The UE may be configured by the higher layer parameter SRS-ResourceMapping with an SRS resource occupying a location within the last 6 symbols of the slot. When PUSCH and SRS are transmitted in the same slot, the UE may be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS. A UE may be configured to transmit one or more precoded SRS on configured SRS resource(s), where the transmission of precoded SRS is based on precoder determination computed on the reference signals indicated by the higher layer parameters SRS-SpatialRelationInfo. Agreement: Agree to the following text proposal for TS 38.214 Section 6.2.1.2 …. and a guard period where UE does not transmit any other signal, not used for any other UE transmission, of Y symbols in-between the SRS resources is used in case the SRS resources are transmitted in the same slot. The value of Y is defined by Table 6.2.1.2-1. Table 6.2.1.2-1 The minimum guard period between two SRS resources of an SRS resource set for antenna switching Y(symbol). 0; 15; 1. 1; 30; 1. 2; 60; 1. 3; 120; 2. Agreement: Agree to the following text proposal for section 5.1.6.1.1 in 38.214 ----start---- A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a CSI-RS resource set for tracking, and receive the higher layer parameter TRS-INFO set as ‘ON’. If a UE is configured with the higher layer parameter TRS-INFO set as 'ON', the UE shall assume the antenna port with the same port index of the configured NZP CSI-RS resources in the CSI-RS resource set is same. For frequency range 1, the UE may be configured with a CSI-RS resource set of four periodic CSI-RS resources in two consecutive slots with two periodic CSI-RS resources in each slot, f. For frequency range 2 the UE may be configured with a CSI-RS resource set of two periodic CSI-RS resources in one slot or with a CSI-RS resource set of four periodic CSI-RS resources in two consecutive slots with two periodic CSI-RS resources in each slot. The periodic CSI-RS resources in the CSI-RS resource set with higher layer parameter TRS-INFO set as ‘ON’ have the same periodicity, bandwidth and subcarrier location. Each CSI-RS resource, defined in Subclause 7.4.1.5 of [4, TS 38.211], is configured by the higher-layer parameter NZP-CSI-RS-ResourceConfig CSI-RS-ResourceMapping with the following restrictions: - the time-domain locations of the two periodic CSI-RS resources in a slot, or of the four periodic CSI-RS resources in two consecutive slots, as defined by higher layer parameter CSI-RS-ResourceMapping, is given by one of , , - a single port CSI-RS resource with density given by Table 7.4.1.5.2-12 and higher layer parameter csi-RS-Density - the bandwidth of the CSI-RS resource, as given by the higher layer parameter csi-RS-FreqBand, is the minimum of 50 or resource blocks, or is equal to resource blocks when the bandwidth of active BWP is greater than 50 resource blocks and when the periodicity of the CSI-RS resource is greater than 10ms - the periodicity, as given by the higher layer parameter csi-RS-timeConfig, is one of slots where 10, 20, 40, or 80 ms and where µ is defined in [4.2, TS 38.211]. 10, 20, 40, or 80 ms - same Pc and Pc_SS value across all resources -----end----- Agreement: Following working assumption is confirmed: - For above-6GHz, the following configurations are supported - Same configuration as below-6GHz: X=2 and N=2+2 - For X=1, use the same OFDM symbols as X=2 case within a slot Agreement Agree to the following text proposal for section 5.1.6.1.1 in 38.214 the time-domain locations of the two periodic CSI-RS resources in a slot, or of the four periodic CSI-RS resources in two consecutive slots, as defined by higher layer parameter resourceMapping, is given by one of , , . The time domain locations are the same across the two consecutive slots. Conclusion: For further discussion in RAN1#92: Clarification on whether TRS for SCell is enabled during deactivated state. Companies are encourage to check the summary of feature lead. Agreement: Agree to the following text proposal in 5.2.1.5.2 of 38.214 For a UE configured with the higher layer parameter ResourceConfigType set to 'semi-persistent'. - when a UE receives an activation command [10, TS 38.321] for CSI-RS resource(s) for channel measurement and CSI-IM/NZP CSI-RS resource(s) for interference measurement associated with configured CSI resource setting(s) in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions (including quasi-co-location assumptions provided by a reference to a TCI-RS-SetConfig) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied no later than the minimum requirement defined in [10, TS 38.133]. If a TCI-RS-SetConfig in the list is configured with a reference to an RS associated with QCL-TypeD, that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/BWP. Agreement: Agree to the following text change for 38.213 section 10.1: ---- Unchanged parts omitted ---- A UE may assume that the DM-RS antenna port associated with PDCCH reception in the Type0A-PDCCH common search space and associated PDSCH are quasi co-located with the associated SS/PBCH block with respect to average gain delay spread, Doppler spread, Doppler shift, average delay, and spatial Rx parameters when applicable. A UE may assume that the DM-RS antenna port associated with PDCCH reception and associated PDSCH reception in the Type1-PDCCH common search space are quasi co-located with the SS/PBCH block identified in initial access procedure with respect to delay spread, Doppler spread, Doppler shift, average delay, and spatial Rx parameters when applicable. ---- Unchanged parts omitted ---- If the UE has received initial RRC configuration of TCI-StatesPDCCH containing more than one TCI states but not MAC CE activation of one of those states, the UE assumes that the DM-RS antenna port associated with PDCCH reception in the UE-specific search space is quasi co-located with the SS/PBCH block the UE identified during initial access procedure with respect to delay spread, Doppler spread, Doppler shift, average delay, and spatial Rx parameters when applicable. If the UE has received RRC configuration of TCI-StatesPDCCH containing a single TCIstate, the UE assumes that the DM-RS antenna port associated with PDCCH reception in the UE-specific search space is quasi co-located with the DL RS(s) configured by the TCI state. Agreement: Agree to the following text change for 38.214 section 5.1.5: ---- Unchanged parts omitted ---- The UE receives a selection command [10, TS 38.321] used to map up to 8 TCI states to the codepoints of the DCI field Transmission configuration indication. Until a UE receives higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the SSB determined in the initial access procedure with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters where applicable. ---- Unchanged parts omitted ---- If the TCI-PresentinDCI is set as 'Enabled', the UE shall use the TCI-States according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the RS(s) in the RS set with respect to the QCL type parameter(s) given by the indicated TCI state if the offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold Threshold-Sched-Offset, where the threshold is FFS. For both the case when TCI-PresentInDCI = 'Enabled' and TCI-PresentInDCI = 'Disabled' if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold Threshold-Sched-Offset, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located based on the TCI state used for PDCCH quasi-colocation indication of the lowest CORESET-ID in the latest slot in which one or more CORESETs are configured for the UE.

Scheduling /HARQ aspects

Agreements: - Adopt following text proposal for PDCCH scrambling (TS38.211 Section 7.3.2.3) == The scrambling sequence generator shall be initialized with where - equals the higher-layer parameter Control-scrambling-Identity if configured and RNTI is equal to C-RNTI, otherwise, and - is C-RNTI for PDCCH in USS, while is 0 for PDCCH in CSS. Companies can check whether the above agreements are consistent with previous coding discussion/agreements or not Agreements: - DM-RS sequence for PDCCH is initialized with an equation with at least symbol index, slot index, and the ID. - FFS: further randomization to avoid consistent collisions over frames. - DMRS sequence for PDCCH is generated per symbol. Agreements: - The TP in R1-1801278 for 38.211 is endorsed - The last sentence is subject to editor’s refinement Agreements: - The maximum number of CORESETs per BWP per cell is 3 - For the 3rd CORESET, i.e., for p=2, Ap=39839. - Confirm 10 as the maximum number of search space sets per BWP per cell. Agreements: - CORESET ID of the CORESET configured by PBCH is 0. - Search space ID of the search space configured by PBCH is 0. Agreements: - A UE can be configured with a search space configuration by UE-specific RRC signaling which includes following: - CORESET ID (range: 0-11, to indicate which CORESET the search space is mapped to) - The search space can be associated with any CORESET configuration - When the CORSET ID is UE-specifically configured to be 0, it is mapped to the one configured by PBCH - Search space ID (range: 0-39) - When the search space ID is UE-specifically configured to be 0, it is mapped to the one configured by PBCH Agreements: - DCI format 0_1 and 1_1 are monitored only in USS. - DCI format 0_0 and 1_0 are monitored in CSS. - DCI format 0_0 and 1_0 can be monitored in USS. - They have the same DCI payload size. - One of the following is configured by RRC signaling for the USS: - Monitoring DCI format 0_1 and 1_1 only - Monitoring DCI format 0_0 and 1_0 only Offline discussion on the set of RNTIs associated with CSS & USS, especially taking into account the respective set of DCI formats/sizes monitored for CSS & USS Discuss till next meeting Agreements: - NR supports a DCI format having the same size as the DCI format 1_0 to be used for scheduling RMSI/OSI, for Paging, and for random access. Agreements: - For each search space configuration configured by UE-specific RRC signaling, the UE is informed whether the search space configuration is CSS or USS, together with the following information, as part of the search space configuration: - Which DCI format(s) to monitor - For a CSS, - DCI format 0_0 and DCI format 1_0 - In which case, the UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), SP-CSI-RNTI (if configured), RA-RNTI, TC-RNTI, P-RNTI, SI-RNTI - DCI format 2_0 - In which case, the UE monitors the DCI format with CRC scrambled by SFI-RNTI, and the SFI-related parameters SFI-PDCCH is provided as part of the search space configuration - FFS: how to select one or two decoding candidates if the configured PDCCH candidates are larger than the value - DCI format 2_1 - In which case, the UE monitors the DCI format with CRC scrambled by INT-RNTI, and the PI-related parameters Preemp-DL is provided as part of the search space configuration - DCI format 2_2 - In which case, the UE monitors the DCI format with CRC scrambled by TPC-PUSCH-RNTI or TPC-PUCCH-RNTI - DCI format 2_3 - In which case, the UE monitors the DCI format with CRC scrambled by TPC-SRS-RNTI - Monitoring of multiple DCI formats can be configured for one CSS - For USS, - A UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), TC-RNTI (if a certain condition is met), and SP-CSI-RNTI (if configured) - Further discussion offline the association of the RNTIs with DCI formats - Monitoring of multiple DCI formats can be configured for one USS Agreements: - For a search space configuration, monitoring periodicity of slot(s) is updated as follows: - For all SCS, {1, 2, 4, 5, 8, 10, 16, 20} slots - For INT-RNTI, a subset of {1,2,4} slots is applied - FFS: the case when concatenated semi-static DL/UL assignments is configured Working assumption: - At least for case 1-1 and case 1-2, all UE supports channel estimation capability for 48 CCEs for a given slot per scheduled cell - FFS: cross-carrier scheduling - FFS: wideband RS - FFS: overbooking and/or nested structure - FFS: exceptional case of CCE counting - FFS: for case 2 Agreements - For the following previous agreement, N=4 Agreements: - For CA with up to N CCs, maximum number of PDCCH blind decodes per slot for a UE depends on the number of configured CCs. - All UEs supporting CA with the same set of CCs supports the same maximum number of PDCCH blind decodes. - No explicit UE capability signaling to inform the maximum number of PDCCH blind decodes is reported. - For CA with more than N CCs, maximum number of PDCCH blind decodes for a UE depends on the explicit UE capability. - All UEs supporting CA with the same set of CCs supports at least the same number of PDCCH blind decodes. - FFS: the value of N (no more than 8). Agreements: - For SFI table - Remove entries 46, 47, 48, 49, 50, 51, 52, 53 - Discuss further offline additional entries 46; D; D; D; D; D; D; X; D; D; D; D; D; D; X. 47; D; D; D; D; D; X; X; D; D; D; D; D; X; X. 48; D; D; X; X; X; X; X; D; D; X; X; X; X; X. 49; D; X; X; X; X; X; X; D; X; X; X; X; X; X. 50; X; U; U; U; U; U; U; X; U; U; U; U; U; U. 51; X; X; U; U; U; U; U; X; X; U; U; U; U; U. 52; X; X; X; U; U; U; U; X; X; X; U; U; U; U. 53; X; X; X; X; U; U; U; X; X; X; X; U; U; U. Agreements: - On the action time for GC-PDCCH carrying SFI, - For RRC configured DL reception cancellation, same slot cancellation is supported - For RRC configured UL transmission cancellation, N2 timeline is followed. - Further discussion offline on the detailed conditions for DL/UL cancellation (related to the overwriting rules) Agreements: - Explicitly add reference SCS field in UE-specific SFI table configuration - The UE does not expect the reference SCS to have larger SCS than any of the configured BWP the GC-PDCCH is configured for - The reference SCS is UE-specifically configured per BWP per cell (new RRC parameter) - For FR1: 15kHz/30kHz/60kHz - For FR2: 60kHz/120kHz Agreements: Confirm the following working assumption with updates: - For FDD SFI support, use multi-slot SFI configuration to achieve FDD SFI support - RRC configures reference SCS for DL BWP and reference SCS for UL BWP (new RRC parameters) - The SFI for one FDD slot is configured with multiple values when configuring the slot format for one slot in each entry in the UE-specific SFI table - If the DL and UL reference SCSs are the same, for each pair of values in the configuration for an SFI entry, even location value is for DL BWP, and odd location value is for UL BWP - If DL reference SCS is higher than the UL reference SCS, K is the SCS ratio between DL reference SCS and UL reference SCS (K>1), use a (K+1) values for the SFI configuration for each reference UL slot (or K DL reference slots), with the first K values in the (K+1) values being the SFI for the K DL reference slots, and the last value for the one UL reference slot - If DL reference SCS is lower than the UL reference SCS, K is the SCS ratio bewtween UL reference SCS and DL reference SCS (K>1), use a (K+1) values for the SFI configuration for each DL reference slot (or K UL reference slots), with the first value in the (K+1) values being the SFI for the DL reference slot, and the last K values for the K UL reference slots - Same mechanism can be applied to SUL case - For TDD non-SUL carrier - RRC configures reference SCS for non-SUL carrier and reference SCS for SUL carrier (new RRC parameter) - K is the SCS ratio bewtween non-SUL reference SCS and SUL reference SCS (K>=1), use a (K+1) values for the SFI configuration for each SUL reference slot (or K non-SUL reference slots), with the first K values in the (K+1) values being the SFI for the K reference slots in non-SUL carrier, and the last value for the one reference slot of the SUL carrier Agreements: - UE-specific SFI table configuration (including reference SCS(s)) is per cell Agreements: - Support XXXXXXXDDDDDDD in the slot format table - (working assumption) Support DDXXXUUUDDDDDD in the slot format table Agreements: - Update the previous agreements as follows: - For DCI granted multi-slot transmission (PDSCH/PUSCH) vs semi-static DL/UL assignment - If semi-static DL/UL assignment configuration of a slot has no direction confliction with scheduled PDSCH/PUSCH assigned symbols, the PDSCH/PUSCH in that slot is received/transmitted - If semi-static DL/UL assignment configuration of a slot has direction confliction with scheduled PDSCH/PUSCH assigned symbols, the PDSCH/PUSCH transmission in that slot is not received/transmitted - For DCI granted multi-slot transmission (PDSCH/PUSCH) vs dynamic SFI, when there is no semi-static DL/UL assignment or the semi-static DL/UL assignment indicates unknown - UE is not expected to receive a dynamic SFI indicating a conflicting direction from DCI grant The TPs in R1-1801266 for 38.211 is endorsed The TPs in R1-1801267 for 38.213 is endorsed Agreements: For one carrier: - (working assumption) Payload sizes for 2-2 and 2-3 are padded (if needed) to match the size of formats 0-0/1-0 as defined by the initial BWP - (working assumption) At most 4 different DCI sizes are monitored by the UE per slot - At most 3 different DCI sizes are monitored per C-RNTI per slot - Payload size for formats 0-1 and 1-1 may differ Working assumption: - The number of bits in the resource allocation field for format 0-0 and 1-0 depends on search space: - In CSS(s) in CORESET 0, use initial DL BWP for DCI size determination and RB numbering - FFS If a UE monitors 0-0/1-0 in CSS in CORESET 0 in a slot, it does not monitor formats 0-0 or 1-0 (or 2-x family in case they have a size aligned with 0-0/1-0) in any other search space - Otherwise, use active BWP for DCI size determination and RB numbering Agreements: - The TPs in R1-1801208 for 38.214 is endorsed Agreements: - The TPs in R1-1801207 for 38.212 is endorsed (to fix typos if any) Agreements: - A single RRC parameter PUCCH-GroupHopping configures enable/disable of group and sequence hopping for all the PUCCH formats 0, 1, 3 and 4. - Note: This implies that PUCCH-GroupHopping replaces the already introduced parameters “PUCCH-F0-Base-sequence-hopping” and “PUCCH-F1-Base-sequence-hopping”. - The range value for PUCCH-GroupHopping is “neither”, “enable” and “disable” where - If PUCCH-GroupHopping is “neither”, it implies neither group or sequence hopping is enabled. - If PUCCH-GroupHopping is “enable”, it implies that group is enabled and sequence hopping is disabled. - If PUCCH-GroupHopping is “disable”, it implies that group is disabled and sequence hopping is enabled. - PUCCH-GroupHopping is provided to UE via RMSI before RRC connection. Agreements: - If group hopping enabled and not sequence hopping for PUCCH, the used pseudo-random sequence is initialized as , where the is configured by higher layer. - If sequence hopping but not group hopping is used for PUCCH, the used pseudo-random sequence is initialized as , where the is configured by higher layers. Agreements: - For parameter SR-offset, the SR offset is measured in slots for a given numerology. - For an SR-periodicity of maximum one slot duration for a given numerology, UE expects that the SR-offset is always 0 (slots) - For an SR-periodicity of more than one slot duration for a given numerology, the SR-offset is configured by RRC to be a non-negative value less than SR-periodicity. - For an SR-periodicity of maximum one slot duration for a given numerology, the starting symbol index for the SR transmission occasion on PUCCH within an SR period is determined by where startingSymbolIndex is obtained from the PUCCH resource configuration in the corresponding SR-resource. - Note: For an SR-periodicity of more than one slot duration for a given numerology, the starting symbol index for the SR transmission occasion on PUCCH within a slot is determined by startingSymbolIndex from the PUCCH resource configuration in the corresponding SR-resource. Agreements: - DMRS shifted mapping is not supported for PUCCH format 2. - In Rel-15, time-domain OCC is not supported for PUCCH format 0 with 2 symbols. Conclusion: - Regarding the notational misalignment between 38.211 and 38.213 in determination of PUCCH format 0 cyclic shift, the misalignment is to be fixed by 38.213 editor. Agreements: - If a UCI transmission on a PUCCH form a UE using format 2 or 3 or 4 overlaps in time with K PUCCH resources, each configured for a SR, X bits are used to represent a SR being indicated by the UE and appended to the end of the end of HARQ-ACK followed by CSI. - Note: X is used to indicated both the presence or absence of SR and which of the K configured SR is embedded. - FFS: X=ceil(log2(K+1)) - FFS PUCCH formats 0 & 1 Agreements: - is to be replaced by in and in clause of 6.3.2.6.4 in TS 38.211 Agreements: - Revise to and to . for UCI scrambling for PUCCH format 3 & 4 in clause of 6.3.2.6.1 of TS 38.211. Agreements: - To introduce a linkage in the spec between CS index of DMRS and index of OCC for pre-DFT-OCC (PUCCH PF4) as following - 0,6,3,9 (CS index of DMRS ) -> 0,1,2,3 (index for OCC) for SF = 4 - 0,6 (CS index of DMRS ) -> 0,1 (index of OCC) for SF =2 Agreements: - Revise “length of the orthogonal cover code” to “Spreading factor of PUCCH format 4” in clause of 9.2.1 of TS38.213 for multiplexing capacity of PUCCH format 4. Agreements: - For PUCCH format 1, time domain OCC index obtained from PUCCH resource is used as OCC index for both UCI and DMRS within each hop when frequency hopping is enabled or within the duration of PUCCH when frequency hopping is disabled. Agreements: - The ‘unknown’ symbols in semi-static DL/UL assignment can be used for long PUCCH transmission over multiple slots when a UE receives a grant to transmit the long PUCCH Agreements: - Add to clause of 6.3.2.1, 38.211 to clarify the hopping boundary in time when frequency hopping is enabled for PUCCH format 1, 3 and 4, the number of symbols in the first hop is given by floor(PUCCH length in symbols/2). Agreements: For inter-slot hopping pattern for long PUCCH over multiple slots, the following options can be down-selected - A first PRB for PUCCH transmission is provided by higher layer parameter PUCCH-starting-PRB and a second PRB for PUCCH transmission is provided by higher layer parameter PUCCH-2nd-hop-PRB (already agreed) - After long PUCCH over multiple slot starts, the first PRB is used for PUCCH transmssion in the even slot, the second PRB is used for PUCCH transmssion in the odd slot. - The slot index is counted after long PUCCH starts regardless if it is actually used for long PUCCH transmssion over multiple slots Agreements: - It is clarified that based on previous agreements, when UCI is piggybacked on PUSCH, UCI is not FDMed with DMRS - This applies to the case regardless of whether UL-SCH is present on PUSCH or not Agreements: - When UE determines to transmit 0, 1, or 2 HARQ-ACK bits, the amount of reserved REs for HARQ-ACK is calculated assuming 2-bits HARQ-ACK, along with the beta_offset determined for the particular transmission - In case the number of HARQ-ACK bits determined at UE is less than 2, the modulated HARQ-ACK symbols are mapped to a subset of the reserved REs. - FFS how to determine the subset Agreements: - Clarify in 38.212 that modulated HARQ-ACK symbols are not mapped to non-DMRS symbol before the first DMRS symbol(s) Working assumption: - Locations of the reserved REs for HARQ-ACK are determined following the same rule defined (in RAN1 #91) for mapping modulated HARQ-ACK symbols to REs. Agreements: - In step 5 in the pseudo codes to do “Data and control multiplexing” in Section 6.2.7 of 38.212, ACK puncture REs over set instead of Agreements - When piggybacked on PUSCH with UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK denoted as , CSI part 1 transmission denoted as , and CSI part 2 transmission denoted as , are determined based on following equations: where - ={0.5, 0.65, 0.8, 1} is a new RRC configured parameter of two bits, which is configured per UE (per CC) - FFS whether or not is applicable to the case when there is no UL-SCH - is the symbol index of first available non-DMRS symbol after the first DMRS symbol(s). Agreements: - For resource allocation for HARQ-ACK before RRC connection setup - 4-bit RMSI indicates an entry into a 16-row table and each row in the table configures a set of cell-specific PUCCH resources/parameters - PUCCH duration of at least {2, 14} - If frequency hopping is enabled for long PUCCH, the PRB(s) of the two hops are x PRBs away from each edge of the initial UL BWP. - FFS on x - FFS on whether to support frequency hopping for short PUCCH - FFS frequency hopping is always enabled or indicated in RMSI to enable/disable hopping - Check further offline on the UE feature related discussion Agreements: - For PUCCH duration = 2, the starting symbol is symbol 12 within the slot - HARQ-ACK is only one bit without bundling before RRC connection Conclusion: - To capture in the specification supported combinations of starting OFDM symbol, the number of OFDM symbols and mapping type for the time-domain allocation for PDSCH and PUSCH respectively - Discuss further offline to come up with the details - Note: the current RRC signalling related to the above is not changed Agreements: - Slot boundary is the reference point for starting time of resource allocation for PDSCH (type A only) and PUSCH (type A or type B) Agreements: - Slot boundary is the reference point for starting time of resource allocation for PDSCH for type B Conclusion: - Time domain information field in DCI formats 0-1 and 1-1 can be 0/1/2/3/4 bits depending on existing RRC configuration Agreements: - In case of slot-aggregation is configured - the same symbol allocation is used across slots in UL - Note: this aligns with the DL case - the TB is repeated across the slots - Discuss further offline the RV order for the DL/UL transmission (scheduled by DCI) spanning multiple slots (also checking the existing agreements made in the coding session) - In case of slot-aggregation is configured, the configuration is limited to rank 1 only for both DL and UL Working assumption: - Sizes of all DCI bitfields in DCI formats 0-1 and 1-1 in USS determined by current BWP. Data transmitted on the BWP indicated by the BWP index. If the BWP index activates another BWP, transform as follows: - Zero-pad too small bitfields to match the new BWP - Truncate too large bitfields to match the new BWP Agreements: For PDSCH: - Supported combinations for PDSCH mapping type A: - Starting symbol can be symbol index #0, 1, 2, 3 in a slot. - Length of the PDSCH is at least X symbols, up to 14 symbols within a slot, such that slot boundary is not crossed - FFS the value of X - Supported combinations for PDSCH mapping type B: - Length of the PDSCH can be 2, 4, or 7 symbols. - Starting symbol can be any position within a slot, such that slot boundary is not crossed. Agreements: For PUSCH - PUSCH mapping type A: - Starting symbol is symbol index #0 in a slot. - Length of the PUSCH is at least Y symbols, up to 14 symbols - FFS the value of Y - PUSCH mapping type B (All 105 combinations) - Length of the PUSCH can be 2 through 14 symbols, and with 1 symbol as a working assumption - Starting symbol can be any position within a slot, such that slot boundary is not crossed. Agreements: - The TP in the document containing 7.3.1.6 of 38.211 in R1-1801176 is adopted Conclusion: - Whether or not interleaving depends on the layer number is to be discussed in the MIMO session Agreements: - The TPs in R1-1801209 for 38.213 is endorsed with one update: - “aprt” to “part” Agreements: - The TPs in R1-1801210 for 38.214 is endorsed Agreements: - The TPs in R1-1801224 for 38.214 is endorsed Agreements: - The TPs in R1-1801259 for 38.212 is endorsed Agreements - The list of values for K0 which can be configured by RRC should be updated to {0,1,2,3,4,5,8,10,16,20,32}. - The list of values for K2 which can be configured by RRC should updated to {0,1,2,3,4,5,6,7,8,10,16,20,32}. Agreements: - For PDSCH transmission before RRC connection, the set of timing values K1 between PDSCH and HARQ-ACK is - Defined in specification to use values {1,2,3,4,5,6,7,8} - Note: the values should take into account the UE minimum timing requirements agreed in initial access - For PDSCH scheduled by fallback DCI, the set of timing values K1 between PDSCH and HARQ-ACK is - Defined in specification to use values {1,2,3,4,5,6,7,8} Agreements: - The set of K1 slot-timing values which are configurable in RRC at least includes the set - {0,1,2,3,4,5,6,7,8} - Some of the 16 available RRC values may be reserved Agreements: - Upon detection of a DL SPS PDSCH in slot n, UE transmits HARQ-ACK in slot n+k when there is no contradicting semi-static configuration, where k is given by the PDSCH-to-HARQ-timing-indicator field carried in the activation DCI for the DL SPS process. - FFS: how to handle the case when the semi-static configuration conflicts at slot n+k - FFS: how to handle the case with dynamic SFI - The contradicting semi-static configuration is defined as when there is at least one semi-statically configured DL symbol overlaps with the symbol(s) carrying HARQ-ACK in slot n+k Agreements: - The number of HARQ processes configurable in RRC for unicast PDSCH per cell for a UE includes the set of integers {2,4, 6, 8,10,12,16} - 4-bits are provided in fallback and non-fallback DCI for addressing the HARQ process regardless of configuration - UE does not expect DCI to address process ID larger than number of HARQ processes configured in RRC - Further discussion offline on the number of HARQ processes assumed by the UE before RRC configuration - FFS the number of HARQ processes for PUSCH is fixed at 16 Agreements: - Clarification that Table 2-1 in R1-1801124 for “slot-based scheduling” in previous UE processing time RAN1 #91 corresponds to the following conditions - Case 1-1: PDCCH monitoring periodicity of 14 or more symbols - PDCCH monitoring on up to three OFDM symbols at the beginning of a slot - PDSCH Type A, and PUSCH Type A or B - PDSCH duration of at least 7 symbols - For C-RNTI only - (working assumption) also applicable to the cases when C-RNTI and with other broadcast RNTIs are processed simultaneously by the UE Agreements: - The baseline UE processing time capability (Capability #1) in NR Release 15 given by Table 2.1-1 corresponds to the following conditions - Single numerology for PDCCH, PDSCH, and PUSCH and no UCI multiplexing - Note: mixed numerology processing time agreements from previous meeting can be applied here (i.e., take largest absolute processing time among numerologies between uplink and downlink) - PDCCH conditions - Case 1: PDCCH monitoring periodicity of 14 or more symbols - Case 1-1: PDCCH monitoring on up to three OFDM symbols at the beginning of a slot - Case 1-2: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot - Case 2: PDCCH monitoring periodicity of less than 14 symbols - First symbol of PDSCH assumed to be no earlier than the first symbol of PDCCH scheduling this PDSCH - PDSCH Type B and PUSCH type B time-domain allocation - FFS: increase N1 by d symbols if front-loaded DMRS comes d symbols after first PDSCH symbol - Note: if PDSCH Type A with less than 7 symbols is supported, the capability needs to be further discussed - One unicast PDSCH received and/or one unicast PUSCH transmitted within the same slot - FFS: More than one PDSCH and/or PUSCH case - Non-CA, and CA case with no cross-carrier scheduling - For C-RNTI only. - (working assumption) also applicable to the cases when C-RNTI and with other broadcast RNTIs are processed simultaneously by the UE Table 2.1-1. UE Processing Time and HARQ Timing Configuration; HARQ Timing Parameter; Units; 15 KHz SCS; 30 KHz SCS; 60 KHz SCS; 120 KHz SCS. Front-loaded DMRS only; N11; Symbols; 8; 10; 17; 20. Front-loaded + additional DMRS; N1; Symbols; 13; 13; 20; 24. Frequency-first RE-mapping; N21; Symbols; 10; 12; 23; 36. - If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2 in table. Agreements: - The UE is not expected to transmit or receive any TB across slot boundaries, where the slot boundary is determined by the numerology associated with the transmission or the reception - For the case of TB repetition, each repetition of the TB does not cross slot boundaries Agreements: - The number of HARQ processes for dynamic scheduling of PUSCH is fixed at 16 - Note: some of processes may be used for grant-free operation Agreements: - Confirm the value of d=1 symbols in the RAN1 #91 agreement regarding multiplexing of HARQ-ACK with uplink data on PUSCH, i.e., confirm the text (without change) in TS38.214 Subclause 5.3: - If the HARQ-ACK is transmitted on PUSCH, then d1=1. Agreements: For each HARQ process ID, the UE is not expected to receive a scheduled unicast PDSCH transmission with the same HARQ process ID until - The time after the end of the expected transmission of the HARQ-ACK for an earlier transmission on the same HARQ process ID - FFS the time condition under which soft combinining for the same HARQ process ID can be assumed The TPs in R1-1801275 are endorsed Agreements: - To describe the functionality of CBGTI and CBGFI in the specs: - In 38.213, for re-transmissions, a bit value of ‘1’ in the CBGTI field indicates that the corresponding CBG is transmitted, and a bit value of ‘0’ in the field indicates that the corresponding CBG is not transmitted. - In 38.214, at least for re-transmissions, a bit value of ‘1’ in the CBGFI field indicates for the UE to assume the CBG(s) currently in soft buffer corresponding to a bit value of ‘1’ in the CBGTI field as invalid, and a bit value of ‘0’ in the CBGFI field indicates for the UE to assume the CBG(s) currently in soft buffer corresponding to a bit value of ‘1’ in the CBGTI field as valid. Agreements: - When CBG is configured, UE assumes that the initial transmission is scheduled with whole TB (for both PDSCH and PUSCH) - The UE is not expected to be scheduled with partial TB for the initial transmission. Agreements: - When the configured number of CBGs is assumed to N, and in case when N-bit HARQ-ACK is to be transmitted by the UE in response to the PDSCH transmission scheduled by fallback DCI: - The TB-level HARQ-ACK is repeated over all the N bits for the configured number of CBGs. Conclusion: - The TPs reflecting the above agreemennts are captured in R1-1801226. Editors can use the TPs as a starting point with further update as necessary (including removing some parts deemed unnecessary) Agreements: - The value range for the parameter frequency-hopping-offset is any integer value between 1 and . - Confirm the value 50 as the boundary number of PRBs in the active UL BWP between the bandwidths having two or four frequency offsets for frequency hopping on PUSCH. - Note: above proposals are applicable for both UL transmission with dynamic grant and with configured grant. Agreements: - For both configured grant Type 1 and Type 2 UL transmissions, following RRC parameters are defined and configured for a UE by UE-specific RRC signaling, separately from the corresponding RRC parameters for grant-based PUSCH transmission: - At least some open-loop power-control parameters - FFS discussion the detailed parameters (to be handled by Havish’s session) Agreements: - For both configured grant Type 1 and Type 2, value of the following RRC parameter is updated: - nrofHARQ-processes: INTEGER (1 .. FFS16) Agreements: - For both configured grant Type 1 and Type 2 UL transmissions, a UE can be configured with the following parameter by UE-specific RRC signaling separately from the corresponding RRC parameter for grant-based transmission: - FrequencyHopping: ENUMERATED {mode1, mode2} - dmrs-Type: ENUMERATED {type1, type2} - dmrs-AdditionalPosition: ENUMERATED {pos0, pos1, pos2, pos3} - phaseTracking-RS - DMRSLength: ENUMERATED {len1, len2} - Note: original parameter is called “maxLength” in PUSCH-Config., while if maxLength is configured as len2, single-symbol or double-symbol DM-RS can dynamically indicated by DCI. For configured grant Type 1, the length for DMRS should be configured either len1 or len2. - scramblingID BIT STRING (SIZE (16)) for cp-OFDM - For dft-S-OFDM: - nDMRS-CSH-Identity: INTEGER(0..1007) - nPUSCH-Identity: INTEGER(0..1007) - disableSequenceGroupHopping: ENUMERATED {disabled} - sequenceHoppingEnabled: ENUMERATED {enabled} - activateDMRS-WithOCC: ENUMERATED {enabled} - cyclicShift: INTEGER (0..7) - groupAssignmentPUSCH: INTEGER (0..29) - mcs-Table: ENUMERATED {64QAM, 256QAM} - mcs-TableTransformPrecoder: ENUMERATED {64QAM, 256QAM} - uci-on-PUSCH: CHOICE {dynamic EQUENCE (SIZE (1..4)) OF BetaOffsets, semiStatic BetaOffsets} } - UCI on PUSCH for configured grant is supported. - Dropping/multiplexing rules for UCI to be further discussed. - Note: For Type 1 UL data transmission without grant, “uci-on-PUSCH” should be “semiStatic BetaOffsets” - resourceAllocation: CHOICE {resourceAllocationType0, resourceAllocationType1, dynamicSwitch} - Note: For Type 1 UL data transmission without grant, “resourceAllocation” should be semiStatic “resourceAllocationType0” or “resourceAllocationType1” - rbg-Size: ENUMERATED {config1, config2} - Note: rbg-size is used when the transformPrecoder parameter is disabled Agreements: - For configured grant Type 1, following RRC parameters are updated as following: - timeDomainOffset: INTEGER (0 … 5119) - Note: Indicating the offset in unit of a slot for a given numerology and a given periodicity - timeDomainAllocation: any one of valid combinations of start symbol and length and PUSCH mapping type that are specified for uplink grant-based transmission - frequencyDomainAllocation: 18 bits - Note: Indicating resource allocation as the same way in grant-based PUSCH for a given resourceAllocationType - UL-TWG-DMRS: split into four parameters {Precoding information and number of layers, Antenna ports, and DMRS_seq_intialization, SRS resource indicator} - Antenna ports: 5 bits - DMRS_seq_initialization: 1 bit - Note: Present only when transform-precoding is disabled. - Precoding information and number of layers: 6 bits - SRS resource indicator: 4 bits - UL-TWG-hopping - “Disabled” is deleted - Frequency-hopping-offset: the same range as grant-based case - Note: it is up to RAN2 how to configure configured grant Type 1 on UL or SUL Agreements: - In 38.212, sec 5.4.2.1, clarify (by adding red text) that for TBSLBRM calculation, the “max number of MIMO layers supported by the UE” is the “maximum number of MIMO layers for one TB supported by the UE for the serving cell”. Agreements: - Prior to RRC configuration, TBS_LBRM should be derived based on reference value for maximum number of layers and for maximum modulation order. - Reference values should be specified for Rel-15 at least for downlink Agreements: - Details of soft buffer dimensioning are up to UE implementation. RAN1 recommends RAN4 to define suitable test cases. Send an LS to RAN4 – R1-1801139 Agreements: - Support 4 as additional PI monitoring periodicity value as part of the RRC configuration for the UE - No other additional values in Rel-15 Agreements: - To adopt the TP in section 3 of R1-1801155 (for Section 11.2 of 38.213) Agreements: - DL BWP and UL BWP with the same BWP index are implicitly paired for unpaired spectrum - The previously introduced RRC parameter BWP-pair-index is no longer useful and thus is to be removed - This is to be reflected in the updated spreadsheet (to make it clear that the parameter is “cancelled”) - DL-BWP-index as BWP index in DL assignment and UL-BWP-index as BWP index in UL grant for unpaired spectrum Agreements: - To adopt the TP related to issue #1 in Section 2.2 of R1-1801158 (impact on 38.211 & 38.213) Offline to prepare a list of parameters, if any, that are deemed to be controversial regarding per-CC/BWP/UE configuration Agreements: - The following TP is adopted ------------Begin of Text Proposal for TS38.213 Section 12------------ … For unpaired spectrum operation, a UE can expect that the center frequency for a DL BWP is same as the center frequency for a UL BWP. For each DL BWP or UL BWP in a set of DL BWPs or UL BWPs, respectively, the UE is configured the following parameters for the serving cell as defined in [4, TS 38.211] or [6, TS 38.214]: - a subcarrier spacing provided by higher layer parameter DL-BWP-mu or UL-BWP-mu; - a cyclic prefix provided by higher layer parameter DL-BWP-CP or UL-BWP-CP; - a number of contiguous PRBs provided by higher layer parameter DL-BWP-BW or UL-BWP-BW; - an index in the set of DL BWPs or UL BWPs by respective higher layer parameters DL-BWP-index or UL-BWP-index for paired spectrum operation, or a link between a DL BWP and an UL BWP from the set of configured DL BWPs and UL BWPs by higher layer parameter BWP-pair-index for unpaired spectrum operation; For unpaired spectrum operation, a UE is not expected to receive a configuration where the center frequency for a DL BWP is different than the center frequency for a UL BWP when the DL-BWP-index of the DL BWP is equal to the UL-BWP-index of the UL BWP. … ------------Begin of Text Proposal for TS38.213 Section 12------------ Agreements: - Place HARQ-ACK bit(s) in response to SPS PDSCH after ones in response to dynamic PDSCH in the HARQ-ACK codebook Agreements: - For semi-static HARQ-ACK codebook, the UE provides CBG-based HARQ-ACK in response to detecting DCI format 1_0 for PDSCH reception on a cell with configuration for CBG-based HARQ-ACK Agreements: - No additional bundling for HARQ-ACK is supported when the actual coding rate for UCI on PUCCH exceeds the configured coding rate - In this case, no dropping of HARQ-ACK - If the actual coding rate on PUCCH carrying HARQ-ACK/SR only exceeds the configured coding rate, the UE is expected to proceed with the PUCCH transmission carrying the HARQ-ACK/SR Agreements: - DCI 0-0 doesn’t contain DAI - DCI 1-0 contains a 2-bit counter DAI but doesn’t contain total DAI Agreements: - Regarding how to select PUSCH for piggybacking UCI - Follows the LTE-approach - FFS whether or not there are any specific issues Working assumption: - In case a UE is configured for semi-static HARQ-ACK codebook determination, when the UE detects to receive only one PDSCH on the Pcell, the UE reports HARQ-ACK only for the one PDSCH - FFS whether additional constraints are necessary for the above operation Agreements: - The max number of Resource-set-BWP can be configured per each BWP is 4 Agreements: - The TPs in section 2 related to issue 2-2 of R1-1801159 are adopted (impact 38.211 and 38.214) Agreements: - The TP Alt2 in Section 3 of R1-1801159 is adopted (impact on 38.214) Agreements: - Include numerology µ in Resource-set-cell configuration (which was agreed previously but missing from the parameter Excel): - Resource-set-cell configuration applies only to PDSCH of the same numerology Agreements: - Update the previous agreements as shown below: - The center-subcarrier location (124 bits), bandwidth (3 bits) and MBSFN subframe configuration for a single LTE carrier can be indicated to the NR UE in addition to the Vshift and the number of CRS ports for rate-matching around LTE CRS - Note: this is to cover 4 (max numerology ratio w.r.t. reference SCS)* 12 (subcarriers/RB) * 275 (RBs) subcarriers Agreements: - Bitmap-3 of {1, 2, 4, 5, 8, 10, 20 or 40} units and max 40ms (already supported by RRC) - Bitmap-3 applies also to resource-set-BWP/cell associated with L1 signalling. Agreements: - When bitmap-3 is not configured and bitmap-1/2 pair is configured for L1 indication for PDSCH rate matching, - In case of unit size of bitmap-2 is one slot, bitmap-1/2 applies to each slot during slot(s) scheduled by DL assignment - In case of unit size of bitmap-2 is two slots, first slot of bitmap-1 and 2 pair applies to all even-numbered slot(s) and second slot of bitmap-1 and 2 pair applies to all odd-numbered slot(s) overlapping with slot(s) scheduled by DL assignment

Channel coding

Agreement: The text proposal in R1-1801225 for 38.214 is endorsed Agreement: - In Subclause 5.4.1.3 of TS 38.212, replace the text in “[A UE may assume the value of E is no larger than 8192.]” with: “The value of E is no larger than 8192.” - In Subclause 5.2.1 of TS 38.212, replace the text in “[A UE may assume the value of A is no larger than 1706.]” with: “The value of A is no larger than 1706.” Agreement: Endorse the following text proposal for 38.212 ---------------------------------- Start TP ------------------------------------ 7.3.2 CRC attachment Error detection is provided on DCI transmissions through a Cyclic Redundancy Check (CRC). The entire payload is used to calculate the CRC parity bits. Denote the bits of the payload by, and the parity bits by, where is the payload size and is the number of parity bits. Let be a bit sequence such that for and for . The parity bits are computed with input bit sequence and attached according to Subclause 5.1 by setting to 24 bits and using the generator polynomial . The output sequence after CRC attachmentis for for , where . ----------------------------------------- End TP -------------------------------------- Agreement: Table 6.3.1.4.1-1 of TS 38.212 [1] should ensure to be a multiple of PUCCH modulation order , as revised in Table 2: Table 2: Revision for UCI codeword size on PUCCH UCI(s) for transmission on a PUCCH; UCI for encoding; Value of. HARQ-ACK; HARQ-ACK. HARQ-ACK, SR; HARQ-ACK, SR. CSI (CSI not of two parts); CSI. HARQ-ACK, CSI (CSI not of two parts); HARQ-ACK, CSI. HARQ-ACK, SR, CSI (CSI not of two parts); HARQ-ACK, SR, CSI. CSI (CSI of two parts); CSI part 1. CSI part 2. HARQ-ACK, CSI (CSI of two parts); HARQ-ACK, CSI part 1. CSI part 2. HARQ-ACK, SR, CSI (CSI of two parts); HARQ-ACK, SR, CSI part 1. CSI part 2. Agreement: The rate matching output size after segmentation, Er, should be an integer. Agreement: Both segments are encoded by polar codes with the same configurations (mother code size, rate-matching, interleaver size) Agreement: The rate matching output sequence length is set to , where is the number of code blocks for UCI and the value of is the number of coded bits for UCI transmission. EUCI – (Er×CUCI) zero-valued bits are appended after codeblock concatenation

NR-LTE coexistence

Agreement: If a SUL/non-SUL indicator field is present in DCI format 0_0, a UE ignores the SUL/non-SUL indicator field in DCI format 0_0 when it is not configured for dynamic PUSCH switching between SUL/non-SUL, and transmits PUSCH on the uplink configured to the UE for PUCCH transmission. Agreement: If a SUL/non-SUL indicator field is not present in DCI format 0_0, the UE transmits PUSCH on the uplink configured to the UE for PUCCH transmission when it is scheduled by DCI format 0_0 - Note: This updates a previous agreement which stated that the PUSCH transmission alwas occurs on the non-SUL carrier in this case. Agreement: The MAC CE should include a mechanism to indicate whether the activation/deactivation of semi-persistent SRS applies to the SUL or the non-SUL (but not both). Conclusion: As per previous agreements, for HARQ case 1, LTE uplink transmission is allowed only in uplink subframes (i.e., not in special subframes) of the reference configuration.

UL power control

Agreement: For grant-based PUSCH, when the SRI field is present in the UL grant - The mapping between each state of the SRI field and the pathloss reference (k) is directly configured via RRC. - The mapping between each state of the SRI field and the p0,alpha (j) is directly configured via RRC. - If N=2 (number of closed loop process) is configured to the UE, the mapping between each state of the SRI field and the PUSCH closed loop process (l) is directly configured via RRC. - Note: The mappings above are separately configured for SUL and non-SUL Agreement: Define RRC parameter SRI-PUSCHPowerControl-Mapping which contains the following, where Ns is the number of valid values for the SRI field in the DCI (as defined in 38.212) - SRI-PathlossReferenceIndex-Mapping contains Ns pathloss reference ID values (Note: Maximum of four pathloss reference IDs can be configured) with the first value corresponding to SRI state 0, second value corresponding to SRI state 1 etc. - SRI-P0AlphaSetIndex-Mapping contains Ns p0-alpha set index values (Note: Maximum of 32 p0-alpha set values can be configured) with the first value corresponding to SRI state 0, second value corresponding to SRI state 1 etc. - SRI-PUSCHClosedLoopIndex -Mapping contains Ns closed loop index values (Note: Maximum of 2 closed loop index values can be configured) with the first value corresponding to SRI state 0, second value corresponding to SRI state 1 etc. Note: The RRC parameters above are separately configured for SUL and non-SUL Agreement: Add RRC parameter PathlossReferenceIndex at least for UL-TWG-type1 Agreement: For PUCCH, when the parameter PUCCH-Spatial-relation-info is configured to the UE, - The mapping between each entry of the RRC parameter PUCCH-Spatial-relation-info and the pathloss reference (k) is configured as part of PUCCH-Spatial-relation-info. - The mapping between each entry of the RRC parameter PUCCH-Spatial-relation-info and the p0 is directly configured as part of PUCCH-Spatial-relation-info. - If N=2 (number of closed loop process) is configured to the UE, the mapping between each entry of the RRC parameter PUCCH-Spatial-relation-info and the PUSCH closed loop process (l) is directly configured as part of PUCCH-Spatial-relation-info. - Note: Precise ASN.1 design can be decided by RAN2 Agreement: Define RRC parameter PUCCHPowerControl-Mapping which contains the following, where Ns is the number of configured entries in the parameter PUCCH-Spatial-relation-info - PathlossReferenceIndex-Mapping contains Ns pathloss reference ID values (Note: Maximum of four pathloss reference IDs can be configured) with the first value corresponding to the first entry of the parameter PUCCH-Spatial-relation-info, second value corresponding to the second entry of the parameter PUCCH-Spatial-relation-info etc. - P0PUCCHIndex-Mapping contains Ns p0 PUCCH index values (Note: Maximum of 8 p0 PUCCH index values can be configured) with the first value corresponding to the first entry of the parameter PUCCH-Spatial-relation-info, second value corresponding to the second entry of the parameter PUCCH-Spatial-relation-info etc. - PUCCHClosedLoopIndex -Mapping contains Ns closed loop index values (Note: Maximum of 2 closed loop index values can be configured) with the first value corresponding to the first entry of the parameter PUCCH-Spatial-relation-info, second value corresponding to the second entry of the parameter PUCCH-Spatial-relation-info etc. Agreement: - P_LTE and P_NR are configured separately via UE specific RRC (i.e., as dBm numbers with similar value range as p-Max in LTE) - P_LTE and P_NR are UE-specific - P_cmax for LTE and P_cmax for NR are derived based on P_LTE and/or P_NR (details to be decided by RAN4) - RAN4 to define maximum total LTE and NR power in FR1 (X_total) that the UE should never exceed. - When dynamic power sharing is used, - If total power for LTE and NR in FR1 exceeds X_total, UE reduces NR transmission power or drops NR transmission so that total power does not exceed X_total - Note: As per previous agreement LTE power control procedure is not changed Agreed with the following modifications: - Change title to “LS on power control for LTE-NR dual connectivity” - Change “power sharing” to “power control” in the first line of Section 1 - Change NRAH180 to NRAH1801 in the first line of Section 1 Final LS agreed in R1-1801231 Agreement: The delta function ΔTF,f,c(i) for PUCCH formats 0 and 1 is as follows, - ΔTF,f,c(i) = 10 log10( ), where - Nsymb is the number of symbols transmitted - Nref (the reference number of symbols configured for the PUCCH format) is set to - 2 for PUCCH format 0 - 14 for PUCCH format 1 Agreement: Virtual PHR for non-scheduled serving cell for CA/DC case is supported - For PHR reporting for multiple cells, if the UE does not transmit PUSCH in PUSCH transmission period i for carrier f of serving cell c, the UE computes power headroom for a Type 1 report as - MPR=0dB, A-MPR=0dB, P-MPR=0dB for serving carrier f of cell c - FFS: how to determine j, q_d and l

UE features

Conclusion Issues related to "initial access-related" or "forwards compatibility-related" in RP-172816 are discussed in RAN1#92 Conclusion - Need FR1/FR2 differentiation is discussed during this week. - Empty in this column means no need FR1/FR2 differentiation - This column may be not appeared in the LS to RAN Agreement Features related to initial access in (R1-1721707)Rel-15 NR UE feature list_v4.docx are agreed. Conclusion Scope of UE feature list includes both SA and NSA while application of each feature to SA and NSA needs further discussion. Agreement Features related to waveform, modulation, SCS, and CP in (R1-1721707)Rel-15 NR UE feature list_v5.docx are agreed R1-1801289 and R1-1801290 were agreed after email discussion

Outgoing LS is listed below.

  - R1-1801139	LS on NR Soft Buffer Dimensioning

  - R1-1801182	Reply LS on channel raster and SS raster

  - R1-1801189	LS on OSI and paging PDCCH configuration

  - R1-1801190	LS on CSI-RS based RRM

  - R1-1801191	LS on RLM reporting Interval

  - R1-1801231	LS on power scontrol for LTE-NR dual connectivity

  - R1-1801246	LS on RMSI CORESET configurations

  - R1-1801272	LS response to RAN2 on MAC CEs for beam management and CSI

  - R1-1801279	LS on OFDM symbol generation

  - R1-1801280	Agreements on Paging

  - R1-1801281	LS on correction to RRC parameters for NR

  - R1-1801282	Reply LS on Acquisition of TA Offset for Uplink Transmission

- RAN1 #92 (February 2018)

General

List of RAN1 agreements with ASN.1 impac is agreed in R1-1803529 with the following two additions: - The ffsValue for parameter scs-SpecificCarrierList is maxSCS - Remove RRC signal Scrambling ID for PUCCH corresponding to L1 parameter 'ScramblingID' Draft LS to RAN2 in R1-1803423, which is approved and final LS (including attachment in R1-1803529) in R1-1803530.

Initial access and mobility

Agreements: - The UE may assume that the ratio of PSS EPRE to SSS EPRE is either 0dB or 3dB for a given cell Discuss further till next meeting regarding power offset between SS/PBCH block and PDCCH - No RRC signalling is necessary Agreements: - For RRC parameter k0, it takes values -6, 0 and 6 Agreements: - Agreed to the TP to section 4.1 of 38.213 below. <Start of Text proposal for section 4.1 of 38.213> The candidate SS/PBCH blocks in a half frame are indexed in an ascending order in time from 0 to . A UE shall determine the 2 LSB bits, for , or the 3 LSB bits, for , of a SS/PBCH block index per half frame from a one-to-one mapping with an index of the DM-RS sequence transmitted in the PBCH. For , the UE shall determine the 3 MSB bits of the SS/PBCH block index per half frame by in the PBCH payload as defined in Subclause 7.1.1 of [4, TS 38.212]. <End of Text proposal for 38.213> Conclusion: - The indication of presence of cell-defining SS/PBCH block should be confined within a contiguous spectrum allocation of a same operator in which the SSB is detected. Agreements: - Signal an offset from a reference GSCN to the closest GSCN that UE should search for cell-defining SSB - The reference GSCN is the GSCN that SSB is transmitted (or the middle in the cluster for sub 2.7 GHz) Agreements: - Use reserved codeword(s) to indicate that there is no cell-defining SS/PBCH around the reference GSCN (Gr) in the range [Gr - Glow∆Ghigh, Gr + Ghigh∆Ghigh] - Signal Glow∆Ghigh ∆Glowby multiplication of the step size, X, and 4 MSBs of RMSI-PDCCH-Config corresponding to parameter ssb-subcarrierOffset set to 15 and aA+5=0aA+5=1 (for FR1) - Signal Ghigh∆Ghigh by multiplication of the step size, X, and 4 LSBs of RMSI-PDCCH-Config corresponding to parameter ssb-subcarrierOffset set to 15 and aA+5=0aA+5=1 (for FR1) - FFS: The step size X is either in sync raster step or frequency and may be different for FR1 and FR2 Working assumption: - Use the following tables to signal an offset from a reference GSCN to the closest GSCN that UE should search for cell-defining SSB Offset table for FR1 aA+5; SSB-subcarrier-offset; RMSI-PDCCH-Config; Offset. 1; 8; 0, 1, …, 255; 1, 2, …, 256. 1; 9; 0, 1, …, 255; 257, 258, …, 512. 1; 10; 0, 1, …, 255; 513, 514, …., 768. 1; 11; 0, 1, …, 255; -1, -2, …, -256. 1; 12; 0, 1, …, 255; -257, -258, …, -512. 1; 13; 0, 1, …, 255; -513, -514, …., -768. Offset table for FR2 SSB-subcarrier-offset; RMSI-PDCCH-Config; Offset. 12; 0, 1, …, 255; 1, 2, …, 256. 13; 0, 1, …, 255; -1, -2, …, -256. Working assumption: - Value of X = 1 and is in sync raster step Agreements: - The ffsValue for parameter scs-SpecificCarrierList is maxSCS Agreements: - For potential collision (i.e., in the same symbol) of SS/PBCH block and RMSI PDCCH with multiplexing pattern 1, add the clarification on UE’s behaviour as shown in the following text proposal (section 10, 38.213) A UE can be configured by higher layer parameter SSB-periodicity-serving-cell a periodicity of half frames for reception of SS/PBCH blocks in a serving cell. If the UE has received SSB-transmitted-SIB1 and has not received SSB-transmitted and if REs for a PDCCH reception overlap with REs corresponding to SS/PBCH block indexes indicated by SSB-transmitted-SIB1, the UE receives the PDCCH by excluding REs corresponding to SS/PBCH block indexes indicated by SSB-transmitted-SIB1. If a UE has received SSB-transmitted and if REs for a PDCCH reception overlap with REs corresponding to SS/PBCH block indexes indicated by SSB-transmitted, the UE receives the PDCCH by excluding REs corresponding to SS/PBCH block indexes indicated by SSB-transmitted. When a UE follows the procedure in Subclause 13 to monitor Type0-PDCCH common search space, a UE may assume that no SS/PBCH block is transmitted in REs used for the reception of the Type0-PDCCH. Agreements: For {SSB SCS, PDCCH SCS} = {120, 60} kHz, Remove the configurations with =2 for pattern 2 in Table 13-7. The LS is endorsed by updating the first paragraph to: “In RAN1#92, RAN1 has discussed the maximum TBS for PDSCH containing RMSI/OSI/Paging/RAR, and a max TB size of 3000 bits is proposed by numerous companies for PDSCH carrying RMSI/OSI/Paging and a max TB size of 3000 bits is proposed by one company for PDSCH carrying RAR.“ Final LS is in R1-1803375 Send LS to RAN4 as in R1-1803456, which is approved with the final LS in R1-1803469 Conclusion: - Recommend RAN2 to replace ‘FFS_value’ for broadcast OSI and paging search space with ‘SearchSpace’ (current ASN.1 name in 38.331) with following clarification: - the number of candidates per AL for broadcast OSI PDCCH and for paging PDCCH is fixed and the same as the ones for RMSI in 38.213 (already agreed) - SI monitoring window configuration, e.g., time offset, duration, and periodicity, is up to RAN2 (already agreed) - Paging occasion configuration, e.g., time offset, duration, periodicity, is up to RAN2 (already agreed) - USS-DCI-format is not needed for broadcast OSI and paging search space Agreements: - Regarding the working assumption on the RACH configurations for paired spectrum, FR1 and long sequence, the working assumption is confirmed - Configuration indices 0 to 86 in Table 6.3.3.2-2 are unchanged - In TS38.211, in Table 6.3.3.2-2 remove entry number 117 (preamble format A1/B1 with periodicity 10 ms, and in all subframes). PRACH Configuration Index; Preamble format; Subframe number; Starting symbol; Number of PRACH slots within a subframe; Number of time-domain PRACH occasions within a RACH slot. 117; A1/B1; 1; 0; 0,1,2,3,4,5,6,7,8,9; 0; 2; 7. Agreements: For RACH configuration for FR1, short sequence and unpaired spectrum: - More priority to Ax and Bx formats compared to Ax/Bx - Aim for the following number of configurations for the different formats (in total we will at least have 256-71 = 185 configurations) - A1: 20 - A2: 20 - A3: 20 - B1: 20 - B4: 20 - A1/B1: 15 - A2/B2: 15 - A3/B3: 15 - C0: 20 - C2: 20 - Regarding the number of RACH occasions within one RACH configuration period. - FDM is used to achieve 8 RACH occasions within one RACH configuration period - In addition, for preamble formats A1, A2 and for periodicities 80ms, 160ms at least one configuration supports 8 RACH occasions. - Note: This can be done by either have more subframes allocated or to have more frames allocated Agreements: - For unpaired spectrum FR1, short sequence, support at most one additional Starting Symbol for each format, on top of the already agreed values of 0 and 2. - A1: 7 - A2: 9 - A3: 7 - B1: 8 - B4: None - A1/B1: 8 - A2/B2: 6 - A3/B3: None - C0: 8 - C2: 8 Working assumption: - To adopt the following TP to section 6.3.3.2 of 38.211 - Note: in total now there are 71 (long sequences)+185 (short sequences) = 256 configurations - Note: - Editor to complete the indices for PRACH configuration (after the indices for the long sequences) ============ Text proposal 38.211, section 6.3.3.2 ======================= Table 6.3.3.2-3: Random access configurations for FR1 and unpaired spectrum (short sequences) PRACH Configuration Index; Preamble format; Subframe number; Starting symbol; Number of PRACH slots within a subframe; Number of time domain PRACH occasions within a RACH slot. A1; 16; 1; 9; 0; 2; 6. A1; 8; 1; 9; 0; 2; 6. A1; 4; 1; 9; 0; 1; 6. A1; 2; 1; 2,3,4,7,8,9; 0; 1; 6. A1; 2; 1; 8,9; 0; 2; 6. A1; 2; 1; 7,9; 0; 1; 6. A1; 2; 1; 7,9; 7; 1; 3. A1; 2; 1; 4,9; 7; 1; 3. A1; 2; 1; 4,9; 0; 2; 6. A1; 2; 1; 9; 0; 1; 6. A1; 1; 0; 9; 0; 2; 6. A1; 1; 0; 9; 7; 1; 3. A1; 1; 0; 9; 0; 1; 6. A1; 1; 0; 8,9; 0; 2; 6. A1; 1; 0; 4,9; 0; 1; 6. A1; 1; 0; 7,9; 7; 1; 3. A1; 1; 0; 3,4,8,9; 0; 1; 6. A1; 1; 0; 3,4,8,9; 0; 2; 6. A1; 1; 0; 0,1,2,3,4,5,6,7,8,9; 7; 1; 3. A1; 1; 0; 1,3,5,7,9; 0; 1; 6. A2; 16; 1; 9; 0; 2; 3. A2; 16; 1; 4,9; 0; 2; 3. A2; 8; 1; 9; 0; 2; 3. A2; 8; 1; 4,9; 0; 2; 3. A2; 4; 1; 9; 0; 1; 3. A2; 2; 1; 8,9; 0; 2; 3. A2; 2; 1; 7,9; 9; 1; 1. A2; 2; 1; 4,9; 9; 1; 1. A2; 2; 1; 4,9; 0; 2; 3. A2; 2; 1; 9; 0; 1; 3. A2; 1; 0; 9; 0; 2; 3. A2; 1; 0; 9; 9; 1; 1. A2; 1; 0; 9; 0; 1; 3. A2; 1; 0; 8,9; 0; 2; 3. A2; 1; 0; 4,9; 0; 1; 3. A2; 1; 0; 7,9; 9; 1; 1. A2; 1; 0; 3,4,8,9; 0; 1; 3. A2; 1; 0; 3,4,8,9; 0; 2; 3. A2; 1; 0; 0,1,2,3,4,5,6,7,8,9; 9; 1; 1. A2; 1; 0; 1,3,5,7,9; 0; 1; 3. A3; 16; 1; 9; 0; 2; 2. A3; 8; 1; 9; 0; 2; 2. A3; 4; 1; 9; 0; 1; 2. A3; 2; 1; 2,3,4,7,8,9; 0; 1; 2. A3; 2; 1; 8,9; 0; 2; 2. A3; 2; 1; 7,9; 0; 1; 2. A3; 2; 1; 7,9; 7; 1; 1. A3; 2; 1; 4,9; 7; 1; 1. A3; 2; 1; 4,9; 0; 2; 2. A3; 2; 1; 9; 0; 1; 2. A3; 1; 0; 9; 0; 2; 2. A3; 1; 0; 9; 7; 1; 1. A3; 1; 0; 9; 0; 1; 2. A3; 1; 0; 8,9; 0; 2; 2. A3; 1; 0; 4,9; 0; 1; 2. A3; 1; 0; 7,9; 7; 1; 1. A3; 1; 0; 3,4,8,9; 0; 1; 2. A3; 1; 0; 3,4,8,9; 0; 2; 2. A3; 1; 0; 0,1,2,3,4,5,6,7,8,9; 7; 1; 1. A3; 1; 0; 1,3,5,7,9; 0; 1; 2. B1; 16; 1; 9; 2; 2; 6. B1; 8; 1; 9; 2; 2; 6. B1; 4; 1; 9; 2; 1; 6. B1; 2; 1; 2,3,4,7,8,9; 2; 1; 6. B1; 2; 1; 8,9; 2; 2; 6. B1; 2; 1; 7,9; 2; 1; 6. B1; 2; 1; 7,9; 8; 1; 3. B1; 2; 1; 4,9; 8; 1; 3. B1; 2; 1; 4,9; 2; 2; 6. B1; 2; 1; 9; 2; 1; 6. B1; 1; 0; 9; 2; 2; 6. B1; 1; 0; 9; 8; 1; 3. B1; 1; 0; 9; 2; 1; 6. B1; 1; 0; 8,9; 2; 2; 6. B1; 1; 0; 4,9; 2; 1; 6. B1; 1; 0; 7,9; 8; 1; 3. B1; 1; 0; 3,4,8,9; 2; 1; 6. B1; 1; 0; 3,4,8,9; 2; 2; 6. B1; 1; 0; 0,1,2,3,4,5,6,7,8,9; 8; 1; 3. B1; 1; 0; 1,3,5,7,9; 2; 1; 6. B4; 16; 1; 9; 02; 2; 1. B4; 8; 1; 9; 02; 2; 1. B4; 4; 1; 9; 2; 1; 1. B4; 2; 1; 2,3,4,7,8,9; 02; 1; 1. B4; 2; 1; 8,9; 02; 2; 1. B4; 2; 1; 7,9; 2; 1; 1. B4; 2; 1; 4,9; 2; 1; 1. B4; 2; 1; 4,9; 02; 2; 1. B4; 2; 1; 9; 2; 1; 1. B4; 1; 0; 9; 02; 2; 1. B4; 1; 0; 9; 2; 1; 1. B4; 1; 0; 8,9; 02; 2; 1. B4; 1; 0; 4,9; 2; 1; 1. B4; 1; 0; 7,9; 2; 1; 1. B4; 1; 0; 3,4,8,9; 2; 1; 1. B4; 1; 0; 3,4,8,9; 02; 2; 1. B4; 1; 0; 0,1,2,3,4,5,6,7,8,9; 2; 1; 1. B4; 1; 0; 1,3,5,7,9; 2; 1; 1. A1/B1; 2; 1; 8,9; 2; 2; 6. A1/B1; 2; 1; 7,9; 2; 1; 6. A1/B1; 2; 1; 7,9; 8; 1; 3. A1/B1; 2; 1; 4,9; 8; 1; 3. A1/B1; 2; 1; 4,9; 2; 2; 6. A1/B1; 2; 1; 9; 2; 1; 6. A1/B1; 1; 0; 9; 2; 2; 6. A1/B1; 1; 0; 9; 8; 1; 3. A1/B1; 1; 0; 9; 2; 1; 6. A1/B1; 1; 0; 8,9; 2; 2; 6. A1/B1; 1; 0; 4,9; 2; 1; 6. A1/B1; 1; 0; 7,9; 8; 1; 3. A1/B1; 1; 0; 3,4,8,9; 2; 2; 6. A1/B1; 1; 0; 0,1,2,3,4,5,6,7,8,9; 8; 1; 3. A1/B1; 1; 0; 1,3,5,7,9; 2; 1; 6. A2/B2; 2; 1; 8,9; 0; 2; 3. A2/B2; 2; 1; 7,9; 6; 1; 2. A2/B2; 2; 1; 4,9; 6; 1; 2. A2/B2; 2; 1; 4,9; 0; 2; 3. A2/B2; 2; 1; 9; 0; 1; 3. A2/B2; 1; 0; 9; 0; 2; 3. A2/B2; 1; 0; 9; 6; 1; 2. A2/B2; 1; 0; 9; 0; 1; 3. A2/B2; 1; 0; 8,9; 0; 2; 3. A2/B2; 1; 0; 4,9; 0; 1; 3. A2/B2; 1; 0; 7,9; 6; 1; 2. A2/B2; 1; 0; 3,4,8,9; 0; 1; 3. A2/B2; 1; 0; 3,4,8,9; 0; 2; 3. A2/B2; 1; 0; 0,1,2,3,4,5,6,7,8,9; 6; 1; 2. A2/B2; 1; 0; 1,3,5,7,9; 0; 1; 3. A3/B3; 2; 1; 8,9; 0; 2; 2. A3/B3; 2; 1; 7,9; 0; 1; 2. A3/B3; 2; 1; 7,9; 2; 1; 2. A3/B3; 2; 1; 4,9; 2; 1; 2. A3/B3; 2; 1; 4,9; 0; 2; 2. A3/B3; 2; 1; 9; 0; 1; 2. A3/B3; 1; 0; 9; 0; 2; 2. A3/B3; 1; 0; 9; 2; 1; 2. A3/B3; 1; 0; 9; 0; 1; 2. A3/B3; 1; 0; 8,9; 0; 2; 2. A3/B3; 1; 0; 4,9; 0; 1; 2. A3/B3; 1; 0; 7,9; 2; 1; 2. A3/B3; 1; 0; 3,4,8,9; 0; 2; 2. A3/B3; 1; 0; 0,1,2,3,4,5,6,7,8,9; 2; 1; 2. A3/B3; 1; 0; 1,3,5,7,9; 0; 1; 2. C0; 16; 1; 9; 2; 2; 6. C0; 8; 1; 9; 2; 2; 6. C0; 4; 1; 9; 2; 1; 6. C0; 2; 1; 2,3,4,7,8,9; 2; 1; 6. C0; 2; 1; 8,9; 2; 2; 6. C0; 2; 1; 7,9; 2; 1; 6. C0; 2; 1; 7,9; 8; 1; 3. C0; 2; 1; 4,9; 8; 1; 3. C0; 2; 1; 4,9; 2; 2; 6. C0; 2; 1; 9; 2; 1; 6. C0; 1; 0; 9; 2; 2; 6. C0; 1; 0; 9; 8; 1; 3. C0; 1; 0; 9; 2; 1; 6. C0; 1; 0; 8,9; 2; 2; 6. C0; 1; 0; 4,9; 2; 1; 6. C0; 1; 0; 7,9; 8; 1; 3. C0; 1; 0; 3,4,8,9; 2; 1; 6. C0; 1; 0; 3,4,8,9; 2; 2; 6. C0; 1; 0; 0,1,2,3,4,5,6,7,8,9; 8; 1; 3. C0; 1; 0; 1,3,5,7,9; 2; 1; 6. C2; 16; 1; 9; 2; 2; 2. C2; 8; 1; 9; 2; 2; 2. C2; 4; 1; 9; 2; 1; 2. C2; 2; 1; 2,3,4,7,8,9; 2; 1; 2. C2; 2; 1; 8,9; 2; 2; 2. C2; 2; 1; 7,9; 2; 1; 2. C2; 2; 1; 7,9; 8; 1; 1. C2; 2; 1; 4,9; 8; 1; 1. C2; 2; 1; 4,9; 2; 2; 2. C2; 2; 1; 9; 2; 1; 2. C2; 1; 0; 9; 2; 2; 2. C2; 1; 0; 9; 8; 1; 1. C2; 1; 0; 9; 2; 1; 2. C2; 1; 0; 8,9; 2; 2; 2. C2; 1; 0; 4,9; 2; 1; 2. C2; 1; 0; 7,9; 8; 1; 1. C2; 1; 0; 3,4,8,9; 2; 1; 2. C2; 1; 0; 3,4,8,9; 2; 2; 2. C2; 1; 0; 0,1,2,3,4,5,6,7,8,9; 8; 1; 1. C2; 1; 0; 1,3,5,7,9; 2; 1; 2. C2; 8; 1; 9; 8; 2; 1. C2; 4; 1; 9; 8; 1; 1. ============================================================================ Conclusion: - It is understood by RAN1 that there is no paired spectrum defined in FR2 in Rel-15. Hence, there would be no RACH configuration table for paired spectrum in FR2 in Rel-15. Agreements: - For RACH configuration for FR2, short sequence and unpaired spectrum: - More priority to Ax and Bx formats compared to Ax/Bx - Aim for the following number of configurations for the different formats - A1: 28 - A2: 28 - A3: 28 - B1: 28 - B4: 28 - A1/B1: 20 - A2/B2: 20 - A3/B3: 20 - C0: 28 - C2: 28 - Regarding the number of RACH occasions within one RACH configuration period. - For preamble formats A1, A2, A3, B1, B4, C0 and C2 and all RACH configuration periodicities and all msg 1 subcarrier spacings and at least one configuration supports at least 16 RACH occasions. - Note: This can be done by either have more subframes allocated or to have more frames allocated Agreements: - For RACH configuration for FR2, short sequence and unpaired spectrum, support at most one additional Starting Symbol for each format, on top of the already agreed values of 0 and 2. The following values are as working assumption - A1: 7 - A2: 9 - A3: 7 - B1: 8 - B4: None - A1/B1: 8 - A2/B2: 6 - A3/B3: none - C0: 8 - C2: 8 Agreements: The parameter SubcarrierSpacingRACH as shown is removed. - Note: this is because msg1-SubcarrierSpacing is already defined in 38.331 SubcarrierSpacingRACH ::= ENUMERATED {ffsTypeAndValue} Agreements: - Regarding the FFS value for parameter ra-PreambleIndex: Value range: {0,1,...,63} - Regarding the ffs value for parameter prach-FreqOffset - Value range: {0,1,..., maxNrofPhysicalResourceBlocks - 2} - Regarding the ffs value for parameter numberOfRA-PreamblesGroupA - For SSB-per-rach-occasion {1/8, 1/4, ½, 1, 2}: numberOfRA-PreamblesGroupA is selected from the set of values 4*N, with N=1, …, floor(16/max(1, SSB-per-rach-occasion)) - For SSB-per-rach-occasion {4, 8, 16}: numberOfRA-PreamblesGroupA is selected from the set of values N, with N=1, …, floor(64/SSB-per-rach-occasion) - Note: This denotes the number of GroupARachPreambles per SSB in one RACH occasion. Agreements: - Adopt the following TP to Section 5.3.2 of 38.211: - is the subcarrier spacing of the initial active uplink bandwidth part during initial access. Otherwise, is the subcarrier spacing of the active uplink bandwidth; - is obtained from the higher-layer parameter k0 and is such that the lowest numbered subcarrier in a common resource block for subcarrier spacing configuration coincides with the lowest numbered subcarrier in a common resource block for any subcarrier spacing configuration less than ; - is the lowest numbered resource block of the initial active uplink bandwidth part based on common resource block indexing and is derived by the higher-layer parameter initial-UL-BWP initial Uplink Bandwidth Part during initial access. Otherwise, is the lowest numbered resource block of the active uplink bandwidth part based on common resource block indexing and is derived by the higher-layer parameter UL-BWP. - is the frequency offset of lowest PRACH transmission occasion in frequency domain with respect to PRB 0 of the initial active uplink bandwidth part given by the higher-layer parameter prach-frequency-start during initial access associated with the initial active UL BWP . Otherwise, is the frequency offset of lowest PRACH transmission occasion in frequency domain with respect to PRB 0 of the active uplink bandwidth part given by the higher-layer parameter prach-frequency-start associated with the active uplink bandwidth part; Agreements: - To adopt the following TP to section 6.3.3.2 of 38.211 Random access preambles can only be transmitted in the frequency resources given by parameter prach-FDM. The PRACH frequency resources are numbered in increasing order from 0 to prach-FDM -1 within the initial active uplink bandwidth part during initial access starting from the lowest frequency. Otherwise are numbered in increasing order from 0 to prach-FDM -1 within the active uplink bandwidth part, starting from the lowest frequency. Agreements: - For the previous working assumption: - If the Semi-static UL/DL configuration is in RMSI, only PRACH occasions within the UL part is transmitted - UE assumes that RACH occasions configured in RMSI are not collided with DL transmission - Introducing start symbol(s) larger than 2 for a limited number of entries in the Configuration table. - For FR2, it is replaced with the following working assumption (no change of the previous working assumption regarding FR1) - For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part are valid as long as it does not precede or collide with an SSB in the RACH slot - UE is not expected to receive DL signals during any valid RACH occasion - Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured - FFS how to handle the gap necessary in between SS/PBCH block or DL part and PRACH tx Working assumption: - For Random access configurations for FR2 and unpaired spectrum (short sequence) - The ones in yellow background are not final PRACH Configuration Index; Preamble format; nSFN mod x = y; Slot number; Starting symbol; Number of PRACH slots within a 60kHz slot; Number of time domain PRACH occasions within a RACH slot. 0; A1; 1; 0; 4,9,14,19,24,29,34,39; 0; 2 or 1; 6. 1; A1; 1; 0; 4,9,14,19,24,29,34,39; 7; 1; 3. 2; A1; 1; 0; 24,29,34,39; 0; 2 or 1; 6. 3; A1; 1; 0; 24,29,34,39; 7; 1; 3. 4; A1; 1; 0; 18,19,38,39; 0; 2 or 1; 6. 5; A1; 1; 0; 18,19,38,39; 7; 1; 3. 6; A1; 1; 0; 0,1,2,…,39; 0; 2 or 1; 6. 7; A1; 1; 0; 0,1,2,…,39; 7; 1; 3. 8; A1; 1; 0; 23,27,31,35,39; 0; 2 or 1; 6. 9; A1; 1; 0; 23,27,31,35,39; 7; 1; 3. 10; A1; 1; 0; 3,7,11,15,19,23,27,31,35,39; 0; 2 or 1; 6. 11; A1; 1; 0; 3,7,11,15,19,23,27,31,35,39; 7; 1; 3. 12; A1; 1; 0; 1,3,5,7,…,37,39; 0; 2 or 1; 6. 13; A1; 1; 0; 7,15,23,31,39; 0; 2 or 1; 6. 14; A1; 1; 0; 7,15,23,31,39; 7; 1; 3. 15; A1; 16; 0; 4,9,14,19,24,29,34,39; 0; 2 or 1; 6. 16; A1; 16; 1; [3,]7,[11,]15,[19,]23,[27,]31,[35,]39; 0 or 7; 1; 6 or 3. 17; A1; 8; 1; 4,9,14,19,24,29,34,39; 0; 2 or 1; 6. 18; A1; 8; 1; [3,]7,[11,]15,[19,]23,[27,]31,[35,]39; 0 or 7; 1; 6 or 3. 19; A1; 4; 1; 4,9,14,19,24,29,34,39; 0; 2 or 1; 6. 20; A1; 4; 1; [3,]7,[11,]15,[19,]23,[27,]31,[35,]39; 0 or 7; 1; 6 or 3. 21; A1; 2; 1; 4,9,14,19,24,29,34,39; 0; 2 or 1; 6. 22; A1; 2; 1; [3,]7,[11,]15,[19,]23,[27,]31,[35,]39; 0 or 7; 1; 6 or 3. 23; A1; 1; 0; 23,31,39; 0; 1; 6. 24; A1; 1; 0; 1,3,5,7,…,37,39; 7; 1; 3. Working assumption: - Minimum time gap between the end of PDSCH for Msg4 and the start of HARQ-ACK equals to the duration of N1 + L2. - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1 - L2=500 usec refers to the MAC layer processing time - Note: the reason for working assumption is due to potential concern over satisfying ITU requirement (to be checked) Agreements: - If a received Msg2 does not contain a response to the transmitted preamble sequence, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence after the duration of N1 + new + L2. - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1 - new>=0, FFS - L2=500 usec refers to the MAC layer processing time Agreements: - For PDCCH ordered CFRA, the minimum timing gap between PDCCH order reception and Msg1 transmission is - N2+BWPSwitching+ Delay, - If BWP switching is not required, BWPSwitching=0; otherwise, BWPSwitching is up to RAN4 - N2 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1 - Delay includes at least MAC layer delay in initializing PRACH - Value of Delay is FFS (to be decided in RAN1) Agreements: - In RA-RNTI equation (RA-RNTI= 1 + s_id + 14*t_id + 14*X*f_id + 14*X*Y*ul_carrier_id)), - Value of X and Y are 80 and 8 respectively. - LS to RAN2 regarding the above decision – Asbjorn (Ericsson) R1-1803335, which is endorsed, and final LS is in R1-1803372 Working assumption: - Time period for SSB to RO association is multiple of PRACH configuration periods - The number of multiple PRACH configuration periods in the time period is the smallest value in the set {1, 2, 4} such that the number is ≥ Actually transmitted SSBs / SSBs that can be mapped to one PRACH config period - The time period starts from frame 0 Agreements: - PDCCH order can be transmitted in both common and UE specific search space. - Note: Control session will decide which common search space UE should monitor to receive PDCCH order. Agreements: - UE assumes that the DMRS of both the received PDCCH order and the PDCCH of the corresponding Msg2 are QCLed with the same SSB/CSI-RS. - gNB configures 9 bits to indicate RACH occasion index. - 6 bits are used to indicate an SSB index - Note: This SSB index is just intended to find the RACH occasion to transmit Msg1 - 3 bits are used to indicate the relative RACH occasion index that corresponds to the indicated SSB index - Note: UE follows the SSB CBRA mapping rule to find the specific RACH occasion. - The SSB/CSI-RS that is QCLed with both the DM-RS of PDCCH order and the DM-RS of the PDCCH of the corresponding Msg2 is used for pathloss estimation associated with Msg1. Agreements: - The number of bits for RAR is 56 for a UE. Send LS to RAN2 – R1-1803435, which is approved by updating “if need” to “if needed”, and the final LS is in R1-1803475 - Working assumption: Bit fields according to the table below. - The LTE part is FYI - Note: Need to check the entries in the time domain allocation table to fulfill ITU requirements. LTE; NR; Comment. Reserved; 1; 3. Timing advance; 11; 12; Agreed in 38.321. Hopping flag; 1; 1; Need to ensure FH is mandatory for the UE, at least for msg3.. Frequency domain RA; 10; 12; Resource allocation type 1. Reuse LTE wording on interpretation (but with numbers adjusted accordingly) If FH, the first 1-2 bits are used for hopping information.. Time domain RA; 1; 4; Point to entries in the time allocation table. Check with resource allocation session regarding the number of bits.. Truncated MCS; 4; 4; Same MCS table as for “normal” transmission without 256QAM, only lowest part used.. TPC command; 3; 3; -6 dB to 8 dB with a 2 dB step size. Check with power control session.. CSI request; 1; 1; Only for contention-free RA in Rel-15. A CSI configuration to be used in case the bit is set needs to be defined/configured.. TC-RNTI; 16; 16. Total; 48; 56. Agreements: - If DCI format 1-0 with RA-RNTI is used to schedule msg 2, the DCI contents as a working assumption are according to the table below Field; Bits; Comment. Identifier for DCI formats; 1; Reserved. Frequency domain resource assignment. Time domain resource assignment; ?; Use default time allocation table. VRB-to-PRB mapping; 1. Modulation and coding scheme; 5; Use MCS table without 256QAM (UE capabilities not yet known). New data indicator; 1; Reserved. Redundancy version; 2; Reserved. HARQ process number; 4; Reserved. Downlink assignment index; 2; Reserved. TPC command for scheduled PUCCH; 2; Reserved. PUCCH resource indicator; 3; Reserved. PDSCH-to-HARQ_feedback timing indicator; 3; Reserved. Agreements: - Msg 3 retransmission: use DCI format 0-0 with TC-RNTI - working assumption according to the table below Field; Bits; Comment. Identifier for DCI formats; Indicate UL. Frequency domain resource assignment. Time domain resource assignment. Frequency hopping flag. Modulation and coding scheme; Use UE-capability-independent MCS table. New data indicator; Reserved. RV. HARQ proc; Reserved. HARQ process 0 is always used.. TPC for PUSCH. UL/SUL; Reserved (retransmission on same carrier as initial transmission). Agreements: - Msg4 using DCI format 1-0 with TC-RNTI - working assumption according to the table below Field; Bits; Comment. Identifier for DCI formats; 1; Indicate DL. Frequency domain resource assignment. Time domain resource assignment; ?. VRB-to-PRB mapping; 1. Modulation and coding scheme; 5; Use UE-capability-independent MCS table Assume LBRM in the UE? Check!. New data indicator; 1. Redundancy version; 2. HARQ process number; 4. Downlink assignment index; 2; Reserved from functionality point-of-view (ongoing discussion in PUCCH session whether to use these bits for other purposes or not). TPC command for scheduled PUCCH; 2. PUCCH resource indicator; 3. PDSCH-to-HARQ_feedback timing indicator; 3; Used but potentially different interpretation. Agreements: - To adopt the following for section 7.4 of 38.213 A UE determines a transmission power for a physical random access channel (PRACH) for carrier corresponding to the current determined SS block of serving cell in transmission period as - [dBm], where is the configured UE transmission power defined in [8, TS 38.101] for carrier of serving cell within transmission period , is the preamble transmission target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layer [11, TS 38.321] for carrier of serving cell , and is a pathloss calculated by the UE for carrier corresponding to the current determined SS block of serving cell in dB calculated as referenceSignalPower – higher layer filtered RSRP of the current determined SS block, where RSRP is defined in [7, TS 38.215], the higher layer filter configuration is defined in [12, TS 38.331], and referenceSignalPower is SS-PBCHBlockPower, where SS-PBCHBlockPower is provided by SystemInformationBlockType1. Agreements: - To adopt the following TP for section 8, 38.213: From the physical layer perspective, the L1 random access procedure encompasses some or all of the transmission of random access preamble (Msg1) in a PRACH, random access response (RAR) with a PDCCH/PDSCH (Msg2), Msg3 PUSCH, and PDSCH for contention resolution. Agreements: - Adopt the additional periodicity of 4ms for CSI-RS for L3 mobility Agreements: - Agree on the following text proposal for 38.215 - Note: this (E-UTRA RS-SINR) needs a NEW RRC parameter for a NR UE. It is understood that this parameter is intended for SA case >>>>>>>>>>>> Start text proposal >>>>>>>>>>>> 2 References <omitted> [13] 3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation” <omitted> 5.1.X E-UTRA RS-SINR Definition; E-UTRA reference signal-signal to noise and interference ratio (E-UTRA RS-SINR), is defined as the linear average over the power contribution (in [W]) of the resource elements carrying cell-specific reference signals divided by the linear average of the noise and interference power contribution (in [W]) over the resource elements carrying cell-specific reference signals within the same frequency bandwidth. For E-UTRA RS-SINR determination, the E-UTRA cell-specific reference signals R0 according TS 36.211 [13] shall be used. The reference point for the E-UTRA RS-SINR shall be the antenna connector of the UE. If receiver diversity is in use by the UE, the reported value shall not be lower than the corresponding E-UTRA RS-SINR of any of the individual diversity branches. If higher-layer signalling indicates certain subframes for performing E-UTRA RS-SINR measurements, then E-UTRA RS-SINR is measured in the indicated subframes.. Applicable for; RRC_CONNECTED inter-RAT. >>>>>>>>>>>> End text proposal >>>>>>>>>>>> Agreements: - resourceElementMappingPattern of CSI-RS-Resource-Mobility IE is identical to resourceMapping of the NZP-CSI-RS-Resource IE. - Note: it was already agreed earlier that the value range for resourceElementMappingPattern corresponds to single port only Agreements: Value Range. ssb-Index; 0, .., 63. maxNrofCSI-RS-CellsRRM; 96 in case associated-SSB is configured 64 in case associated-SSB is not configured. MaxNrofPCIsPerSMTC; 64. Agreements: - For reception of cell defining SSB from the target PCell, frequency offset is not required. NR only supports cell defining SSBs of PCell that are always on the synchronization signal raster. - For reception of SSB for the target SCell, frequency offset signaling will be required for SSB not on the synchronization signal raster. - FFS the case of PScell - Agree to update the value range for starting PRB index of CSI-RS configuration for L3 mobility as follows: Measurement-BW; Allowed measurement BW for CSI-RS; BW size: {24, 48, 96, 192, 264} PRBs in CSI-RS numerology Starting PRB index: {0, 1, …, [251][2169](=274*8-24+1)]} PRBs with respect to PRB0 in CSI-RS numerology; A common set of values are assigned across all the resources configured per cell. Draft LS in R1- 1803354 to respond to LS from RAN2 in R1-1801323, which is endorsed by updating question 3 with the addition of a bullet “FFS the case of PScell”, with the final LS is in R1-1803379 Conclusion: - It is RAN1 understanding that the selection of resources within the RSSI measurement window to obtain the RSSI measurement is up to UE implementation. UE is expected to meet the RAN4 RSSI requirements. Agreements: - For FDD/TDD, adopt RAN4 recommendation for default SS-RSSI measurement definition. Agreements: - The RRC classification table for RRM, as shown below: RRM; RRM-MeasurementConfig SMTC-Config SMTC-Config-Idle SSB-MeasurementTimingConfiguration csi-rs-ResourceConfig-Mobility ReferenceSignalConfig SS-RSSI-MeasurementConfig SS-RSSI-MeasurementSlotConfig SS-RSSI-MeasurementSymbolConfig; FFS. is replaced by RRC Parameter Name in 38.331 v1.01.1; RAN1 Parameter Name; Classification; Note. MeasObjectNR; -; Per UE. SS-RSSI-MeasurementConfig; Per UE (CONNECTED) Per Cell (IDLE); For IDLE mode UEs, this configuration is delivered through OSI. For CONNECTED mod UEs, this configured is delivered through RRC.. SS-RSSI-MeasurementSlotConfig; -; Sub IE of SS-RSSI-MeasurementConfig. SS-RSSI-MeasurementSymbolConfig; -; Sub IE of SS-RSSI-MeasurementConfig. Note: In RAN1 understanding, a UE can be configured with multiple MO. Conclusion: - The following RRC signalling is missing from the 38.331, even though RAN1 agreements require such signalling. - subcarrierSpacingSSB (for inter-frequency cell re-selection), useServingCellTimingForSync, and smtc1 of SSB-ConfigMobility IE should be also available in SI for IDLE mode UEs. Currently SSB-ConfigMobility IE is contained within the MeasObjectNR IE. - SS-RSSI-MeasurementConfig parameter is currently missing from the RRC specification 38.331 and should be available in both SI for IDLE mode UEs and dedicated RRC for CONNECTED mode UEs. Agreements: - maxSCSs INTEGER ::= ffsValue = 5 Conclusions: - No RRC configuration involvement in determining whether or not QCL of RLM-RSs in two or more different BWPs of the Pcell/PScell is assumed by the UE - Discuss further offline whehther QCL is always assumed or no-QCL is always assumed – (to be further discussed till next meeting) Agreements: - Agree the following TP: ========== Start of Text proposal for TS38.213 section 5 ========= A UE can be configured for each SpCell [11, TS 38.321] with a set of resource indexes for radio link monitoring by higher layer parameter RLM-RS-List. The UE is provided by higher layer parameter RLM-RS an association between a resource index, from the set of resource indexes, with either a CSI-RS resource configuration or a SS/PBCH block. For a CSI-RS resource configuration, the UE is provided a corresponding index by higher layer parameter RLM-CSIRS. The higher layer parameters CSI-IM-RE-pattern, CSI-IM-Resource, CSI-IM-ResourceId, CSI-IM-timeConfig, CSI-IM-FreqBand, CSI-IM-ResourceMapping, CDM-pattern, Pc, and Pc_SS in the CSI-RS configuration are not applicable. A two port CSI-RS configuration is not applicable for RLM. In CSI-RS resource configuration, only ‘No CDM’ is allowed for higher layer parameter CDM-Type, only ‘1’ and ‘3’ are allowed for higher layer parameter density, and only ‘1 port’ is allowed for higher layer parameter nrofPorts. For a SS/PBCH block, the UE is provided a corresponding index by higher layer parameter RLM-SSB. ========== End of Text proposal ========= Agreements: • If there is an overlap between two adjacent slots due to TA command, the latter slot is reduced compared with the former slot. Working assumption: - For the timing advance in MAC-CE in the case with multiple configured UL BWPs in the TAG, the granularity depends on the maximum SCS of all the activated UL BWPs including SUL within the TAG. - When the TA is adjusted based on the timing advance in MAC-CE in higher SCS, the TA in lower SCS may be rounded in order to align with lower SCS TA granularity - This may result in different timing across UL carriers within the same TAG - This revises the past agreement of SUL of the granularity of the smaller subcarrier spacing between UL and SUL.

MIMO

LS on MAC CEs for BM and CS is endorsed in R1-1803347. Agreement: In Rel-15, additionally support BFR on SCell - Number of SCells BFR needs to be supported on is 1 - UE is not mandated to support BFR on SCell - Note: There is no additional RAN1 specification impact for BFR on SCell. LS reply on beam failure recovery is endorsed in R1-1803348. Agreement For NR Rel-15, DMRS port ordering is performed as follows for one-CW transmission (DL) - Simple ordering with increasing index (port number) Agreement For VRB-to-PRB interleaving: - Layer-common VRB-to-PRB interleaving Agreement - If ulTxConfig is not configured, the default transmission is 1 PUSCH port based - The PUSCH is triggered by DCI format 0_0 - Note: there is no TPMI, no SRI indication in format 0_0 - Note: this does not imply new uplink transmission scheme shall be defined Agreement - gNB should configure at least 1 SRS resource when ULTXconfig is codebook based UL transmission. No restriction on gNB’s configuration of SRS for UL scheduling by DCI format 0_0. - If and when the SRS is triggered is up to gNB implementation Agreement: For codebook based UL transmission, the SRI in slot n is associated with the most recent transmission of the SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI before slot n. Agreement - UE is not expected to be configured with ULCodebookSubset of ‘partialAndNonCoherent’ when two antenna ports are configured. Agreement: - Clarify 38.212 to show that ‘precoding information and number of layers’ field size is 0 bits for DCI format 0_1 if UE is configured to transmit on only one antenna port with codebook based operation Agreement: Agree to the following text proposal: TP to Section 6.1.1.2 of 38.214: The UE shall use one or multiple SRS resources for SRS transmission, where the number of SRS resources which can be configured to the UE for simultaneously transmission in the same RBs is being part of UE capability signalling. Only one SRS port for each SRS resource is configured. Only one SRS resource set can be configured with higher layer parameter srsSetUse set to ‘nonCodebook’. The maximum number of SRS resources that can be configured for non-codebook based uplink transmission is 4. For non-codebook based transmission, the UE can measure NZP CSI-RS resource to calculate the precoder used for the transmission of precoded SRS. - If aperiodic SRS resource set is configured, the [CSI-RS information in the same slot TBD] for UL channel measurement is indicated via DCI, where the association among aperiodic SRS triggering state, triggered SRS resource(s) SRS-ResourceConfigId, CSI-RS resource ID NZP-CSI-RS-ResourceConfigID are higher layer configured by AperiodicSRS-ResourceTrigger. A UE may receive the dynamic SRS transmission request for aperiodic SRS transmission in the same slot as the reception of the DL CSI-RS resource, where the SRS transmission happens [TBD] symbols after the SRS transmission request. A UE may receive the SRI which shall be associated with the most recent SRS transmission. - If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceConfigID for measurement is indicated via higher layer parameter SRS-AssocCSIRS per set.. Agreement Add the following text for TS38.214 section 6.1.1.2 The UE shall perform one-to-one mapping from the indicated SRI(s) to the indicated DMRS ports(s) in the DCI format 0_1 increasing order. Agreement: A UE should not be expected to be configured with both SRS-SpatialRelationInfo for SRS resource and SRS-AssocCSIRS for SRS resource set for non-codebook based transmission. Agreement: For non-codebook based transmission, a UE can be configured with only one NZP CSI-RS resource for the SRS resource set. Agreement: The minimum SRS resource for non-codebook based transmission is one for DCI format 0_1. Conclusion: No explicit specification support of frequency selective precoding for non-codebook based transmission in Rel-15. Agreement: For non-codebook based uplink transmission, support the same time domain behavior at SRS resource set level as that of the codebook based uplink transmission case Agreement: If the higher layer parameter prb-BundlingType is set to 'dynamic', and the UE is scheduled a PDSCH with DCI format 1_0 scrambled with C-RNTI and CS-RNTI: - PRG value is fixed as 2; Agreement: The following text proposal for TS38.214 section 5.1.2.3 is agreed. ----Text Proposal for TS 38.214------------------------------------------------------------------ 5.1.2.3 Physical resource block (PRB) bundling A UE may assume that precoding granularity is multiple resource blocks in the frequency domain. Precoding Resource Block Group (PRGs) of size partition the carrier bandwidth part i with consecutive PRBs.and each PRG consists of one or multiple consecutive PRBs, can be one of the values among {2, 4, scheduled bandwidth wideband}. Actual number of consecutive PRBs in each PRG could be one or more. The PRG for each carrier bandwidth part is equal to 2 PRBs unless configured by the higher layer parameter prb-BundlingType prb-BundlingEnabled and pdsch-BundleSize. When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, the UE shall assume that is equal to 2 PRBs. If a UE is configured with PRG of is determined as " wideband scheduled bandwidth", the UE is not expected to be scheduled with non-contiguous resource allocation and the UE may assume that the same precoding is applied to the allocated resource. When the bundling size is configured as is determined as one of the values among {2, 4}, the first PRG size is given by and the last PRG size given by if , and the last PRG size is if . The UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs in a PRG partitioned above. If the higher layer parameter prb-BundlingType prbBundling is set to ' dynamic ON', the higher-layer parameter bundleSizeSet1pdsch-BundleSize and bundleSizeSet2 configures two sets of values, the first set includes one or two values among {2, 4, scheduled bandwidth wideband}, and the second set includes one value. The UE is not expected to be configured with (2, 4) in the first set. If the PRB bundling size indicator signalled in DCI format 1_1 as defined in Subclause 7.3.1.2.2 of [2, TS 38.212] - is set to '0', the UE shall use the value from the second set of PRG values when receiving PDSCH scheduled by the same DCI. - is set to '1' and one value is configured for the first set of values, the UE shall use this value when receiving PDSCH scheduled by the same DCI - is set to '1' and two values are configured for the first set of values as ‘n2-wideband’(corresponding to two values 2 and wideband) or ‘n4-wideband’ (corresponding to two values 4 and wideband)(2, scheduled BW) or (4, scheduled BW ), the UE shall use the value when receiving PDSCH scheduled by the same DCI as follows: - If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than , is the same as the scheduled bandwidth, otherwise, is set to the remaining configured value of 2 or 4, respectively. If the higher layer parameter prb-BundlingType prbBundling is set to 'static OFF', the value is configured with the single value indicated by the higher-layer parameter bundleSize pdschBudnleSize. When a UE is configured with RBG = 2 according to Section 5.1.2.2.1, or when a UE is configured with resource block bundle size of 2 for VRB to PRB mapping provided by the higher layer parameter VRB-to-PRB-interleaver, the UE is not expected to be configured with = 4. Agreement: The following text proposal for TS38.214 section 5.1.2.3 is agreed. ----Text Proposal for TS 38.214------------------------------------------------------------------ 5.1.2.3 Physical resource block (PRB) bundling < Unchanged parts are omitted > When the bundling size is configured as one of the values among {2, 4}, the first PRG size is given by and the last PRG size given by if , and the last PRG size is if . The UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs in a PRG partitioned above. For PDSCH carrying RMSI scheduled by PDCCH with CRC scrambled by SI-RNTI, PRG is partitioned from the lowest numbered resource block of the CORESET signalled in PBCH. < Unchanged parts are omitted > Agreement Agree to the following text proposal for TS38.211 section 7.3.1.6 7.3.1.6 Mapping from virtual to physical resource blocks < Unchanged parts are omitted > The UE may assume that the same precoding in the frequency domain is used within a PRB bundle, and the bundle size is determined by clause 5.1.2.3 in [TS38.214, 6]. The UE shall not make any assumption that the same precoding is used for different bundles of common resource blocks. < Unchanged parts are omitted > Conclusion: - RAN1 to discuss at least the following combinations of DL physical channels and reference signals for the simultaneous reception assuming FR2 operation: Channels/RS in same OFDM symbol; Same CC / BWP; Different CCs. SSB + PDSCH. CSI-RS + PDSCH (including DM-RS/PT-RS) - A-CSI-RS (below and above threshold), P CSI-RS, SP CSI-RS - BM CSI-RS, CSI-RS for CSI, CSI-RS for tracking - CSI-RS Repetition ‘ON’, ‘OFF’ - CSI-IM. PDCCH / CORESET+ PDSCH - Default CORESET (CORESET with lowest ID which is used for obtaining default QCL assumption) - Non default CORESET. CSI-RS + PDCCH / CORESET; Rx spatial QCL should be ensured by the NW configuration*. PDCCH / CORESET + PDCCH / CORESET; [N/A]. CSI-RS + SSB; Rx spatial QCL should be ensured by the NW configuration*. CSI-RS + CSI-RS. PDSCH + PDSCH with different RNTIs; TBD. SSB + SSB; N/A. PDSCH + PDSCH (C-RNTI + C-RNTI); N/A. *Note: Already agreed in the previous meetings - RAN1 to discuss at least the following combinations of UL physical channels and reference signals combinations for the simultaneous transmission assuming FR2 operation: Channels/RS in same OFDM symbol; Same CC; Different CCs. PUCCH + PUCCH; N/A. PUSCH + PUSCH; N/A. SRS + SRS. - RAN1 to consider at least the following solutions: - Skipping transmission of reference signal / physical channel, e.g. QCL assumptions are not the same - Overriding QCL assumptions - Prioritization of the processing of one physical channel /signal over another - NW configuration should ensure the same QCL assumption for the physical channel /signal - Other solutions are not precluded Agreement (New RRC parameter) Agree on the value ranges and include the following table as part of the RRC parameter update. RRC parameter; Value range; Description. ZP-CSI-RS-ResourceConfigType; periodic, semi-persistent or aperiodic; Time domain behavior of ZP-CSI-RS resource configuration. SP-ZP-CSI-RS-ResourceList; Up to 16 ZP CSI-RS resource set IDs per BWP; Contains list of SP ZP-CSI-RS resource set IDs Each resource set can have up to 16 ZP CSI-RS resources. - The maximum number of ZP CSI-RS resources per CC is 32 - The maximum number of ZP CSI-RS resources per BWP is 16 - The maximum number of ZP CSI-RS resources per ZP CSI-RS resource set is 16 Agreement: - NZP CSI-RS based IMR is only supported when single CMR per set is configured Agreement Following text proposal for TS38.214 section 5.2.2.4 is agreed: The following parameters for which the UE shall assume zero transmission power are configured via higher layer parameter CSI-IM-ResourceConfig for each CSI-IM resource configuration: Agreement For configuration details on trigger state for aperiodic CSI: - Specification should capture that None equals to 1 always Agreement If interference measurement is performed only on CSI-IM, each CSI-RS resource for channel measurement is resource-wise associated with a CSI-IM resource by the ordering of the CSI-RS resource and CSI-IM resource in the linked resource sets. - The number of CSI-RS resources for channel measurement equals to the number of CSI-IM resources. Agreement For P/SP CSI reporting, NZP CSI-RS based IMR is not supported. Conclusion: It’s up to UE implementation on how downlink interference estimation is performed. Agreement (RRC parameter update): For CSI acquisition, aperiodic CSI-RS triggering offset can be 0, 1, 2, 3, 4 slots. - If all the associated trigger states do not contain QCL Type D information, aperiodic CSI-RS triggering offset is fixed to zero. Agreement (RRC parameter update): The proposals and TP in R1-1803523 are endorsed with the following additional details. - The maximum number of CSI report settings configured per BWP is 12 - The maximum number of resource settings configured per BWP is 28 Agreement: For wideband CSI reporting on PUCCH, the number of padding bits, denoted as , is determined as follows: - =Nmax-Nreported, Nmax is the maximum payload (bits) of the WB CSI, and Nreported is the actual payload (bits) of the WB CSI that is reported Following text proposal for section 6.3.1.1 of TS 38.212 is agreed: CSI field; Bitwidth. Zero padding bits. Nreported=B(R), where R=reported rank Nmax=maxr∈SRankB(r), where SRank= set of rank values that are allowed to be reported For 2 CSI-RS ports: Br=NPMIr+NCQIr+NLI(r) For >2 CSI-RS ports: Br=NPMI i1r+NPMI i2r+NCQIr+NLI(r) Payload NPMI1=2 and NPMI2=1 if PMI is reported; NPMIr=0 otherwise Payload NPMI i1r as in Table 6.3.1.1.2-1/2 of [1] if PMI i1 is reported; NPMI i1r=0 otherwise Payload NPMI i2r as in Table 6.3.1.1.2-1/2 of [1] if PMI i2 is reported; NPMI i2r=0 otherwise Payload NCQIr as in Table 6.3.1.1.2-3/4 of [1] if CQI is reported; NCQIr=0 otherwise Payload NLI(r) as in Table 6.3.1.1.2-3/4 of [1] if LI is reported; NLI(r)=0 otherwise. Agreement - DCI Formats 0_1 is used to activate/deactivate SP-CSI reporting on PUSCH. - DCI Format 0_1 contains a CSI request field and can activate/deactivate any configured SP-CSI trigger state - Note: The DCI can only be used to activate/deactivate SP-CSI reporting Agreement (RRC parameter update): Support {32, 64, 640} slots as periodicities for periodic CSI reporting on PUCCH and semi-persistent CSI reporting on PUCCH and PUSCH Agreement: - Clarify that the J>=1 multi-CSI PUCCH resources are configured separately in PUCCH-Config and are not associated with any specific CSI Report Setting, following LTE approach. Each CSI Report Setting further has a dedicated PUCCH resource configured. Agreement (RRC parameter update): At most J=2 multi-CSI PUCCH resource can be configured to the UE per UL BWP Conclusion: Periodicity of the multi-CSI PUCCH resource does not need to be defined Agreement: - Which of the J>=1 multi-CSI PUCCH resources are used to transmit the colliding CSI reports is determined as follows: - Choose the multi-CSI PUCCH resource with the smallest capacity but such that the CSI payload of all colliding CSI reports is not larger than the capacity - If no such resource is configured, choose the largest multi-CSI PUCCH resource and apply dropping rules Agreement: - For P/SP-CSI-RS, periodicity and slot offset configured in the CSI resource setting is referred to its associated DL BWP - For P/SP-CSI report, periodicity and slot offset configured in the CSI report setting is referred to the UL BWP in which P/SP-CSI report is transmitted Conclusion: - When triggered for aperiodic CSI reporting with an aperiodic trigger state associated with multiple CSI reports: - Triggered CSI reports associated with non-active BWPs (in the slot of the CSI reference resource) are dropped and are not reported by the UE while the remaining CSI reports associated with active BWPs are reported Agreement: Agree to the following text proposal for TS 38.213 Subclause 9.2.5.2 <Start of text> If - a UE has periodic/semi-persistent CSI reports to transmit in a PUCCH and the UE determines a PUCCH format 2 to transmit HARQ-ACK/SR and PUCCH-F2-simultaneous-HARQ-ACK-CSI = TRUE - or the periodic/semi-persistent CSI reports to transmit in a PUCCH are wideband and the UE determines either a PUCCH format 3 or a PUCCH format 4 to transmit HARQ-ACK/SR, and PUCCH-F3-simultaneous-HARQ-ACK-CSI = TRUE, or PUCCH-F4-simultaneous-HARQ-ACK-CSI = TRUE, respectively, - if …; - else, …. If a UE has sub-band periodic/semi-persistent CSI reports to transmit in a PUCCH and the UE determines either a PUCCH format 3 or a PUCCH format 4 to transmit HARQ-ACK/SR, and PUCCH-F3-simultaneous-HARQ-ACK-CSI = TRUE, or PUCCH-F4-simultaneous-HARQ-ACK-CSI = TRUE, respectively - if …; else, …. <End of text> Agreement: - When multiple PUCCH-based CSI reports collide with HARQ-ACK/SR - The colliding CSI reports are first managed according to the CSI only collision mechanism, (potentially multiplexing CSI reports on a multi-CSI PUCCH resource and applying associated dropping rules) outputting a single PUCCH resource carrying one or more CSI reports - In the second step, collision between the CSI PUCCH resource and HARQ-ACK/SR collision is managed and CSI dropping/omission rules for PUSCH is applied Agreement: - LI is reported when indicated by reportQuantity in the CSI report setting irrespective of Downlink-PTRS-Config Agreement: Clarify the CSI omission criterion so that lower priority information bits in are omitted while is larger than Agreement: - CSI omission procedure is supported for the case of CSI only PUSCH - Details TBD in UCI multiplexing session Agreement: - UE is not expected to report CSI with the payload size more than 115 bits when configured with PUCCH format 4 Agreement: Clarify in 38.214 that CRI reporting is not supported for Type II CSI Agreement: Add the CRI bitwidth in Table 6.3.1.1.2-3, Table 6.3.1.1.2-4, and Table 6.3.1.1.2-5 of TS38.212 Agreement: - For the uplink non-CA case, a UE is not expected to transmit more than one aperiodic CSI reports triggered by different DCIs on overlapping OFDM symbols. - For the uplink CA case, the number of aperiodic CSI reports triggered by different DCIs on overlapping OFDM symbols is to be decided in the UL control session Agreement: - A CSI Reporting Setting with ‘CRI/RI/i1’ report quantity is said to have a wideband frequency-granularity - If CSI-ReportingBand indicates that CSI shall be reported for one subband only, this CSI Reporting Setting is said to have a wideband frequency-granularity The following text proposal for TS38.214 section 5.2.1.4 is agreed. <START OF TEXT PROPOSAL> A CSI Reporting Setting is said to have a wideband frequency-granularity if either - ReportQuantity is set to ‘CRI/RI/PMI/CQI’, ‘CRI/RI/i1/CQI’ or 'CRI/RI/LI/PMI/CQI', CQI-FormatIndicator indicates single CQI reporting and PMI-FormatIndicator indicates single PMI reporting, or - ReportQuantity is set to ‘CRI/RI/i1’ and PMI-FormatIndicator indicates single PMI reporting, or - ReportQuantity is set to ‘CRI/RI/CQI’ and CQI-FormatIndicator indicates single CQI reporting, or - ReportQuantity is set to ‘CRI/RSRP’, or - CSI-ReportingBand indicates that CSI shall be reported for one subband only otherwise, the CSI Reporting Setting is said to have a subband frequency-granularity. Agreement: SP-CSI reports on PUSCH cannot be multiplexed on the PUSCH with uplink data transmission. - SP-CSI report is dropped Agreement (RRC parameter update): - For indicating slot offset Y relative triggering DCI for SP-CSI report on PUSCH, re-use RRC parameter reportSlotOffset that can be configured for aperiodic CSI-ReportConfig also for semi-persistent on PUSCH CSI-ReportConfig - Note: The first report is transmitted in slot n+Y, second report in n+Y+P, where P is the configured periodicity Agreement - Re-use the same mechanism for activation / deactivation of SP-CSI on PUSCH as for UL grant free transmission in NR - Note: It’s up to UL scheduling session to decide if MAC CE confirmation is needed for SP-CSI Agreement: - When A-CSI reporting on CSI only PUSCH with single CSI report is triggered, - UE is not expected to receive a scheduling DCI with symbol offset such that M-L-N < Z - UE is not expected to receive a scheduling DCI if AP CSI-RS is used for channel measurement and with symbol offset such that M-O-N < Z’ - FFS for the condition when P/SP CSI-RS is used for channel/interference measurement - FFS if this agreement applies to beam reporting in addition to CSI reporting - Note: - L=the last symbol of PDCCH triggering the A-CSI reporting - M=the starting symbol of the PUSCH - N= the TA value in unit of symbols (e.g., TA=1.4 symbol) - O= the later symbol between the last symbol of AP CSI-RS resource for CMR, the last symbol of aperiodic NZP CSI-RS for IM (if present) and the last symbol of aperiodic CSI-IM (if present) Agreement: - Time offset of the CSI reference resource is derived from Z’ for a given CSI latency and numerology as follows: - nCQI_ref is the smallest value greater than or equal to Z'/Nsymbslot, such that slot n- nCQI_ref corresponds to a valid downlink slot. - This is applied for at least for aperiodic CSI reporting - FFS criterion for P / SP reporting - This is applied when AP/P/SP CSI-RS is used for CSI calculation. - When P/SP CSI-RS / CSI-IM is used for channel/interference measurement, UE is not expected to measure channel/interference on the CSI-RS / CSI-IM whose last OFDM symbol is received 0 to Z’ symbols before transmission time of first OFDM symbol of AP CSI reporting. - Note: This is not the only condition, the CSI-RS shall also be on or before the CSI reference resource, this also includes the AP CSI-RS case - FFS if this criterion is needed for P / SP reporting - FFS: When P/SP CSI-RS is used for channel/interference measurement in case of AP CSI reporting, UE does not expect that the latest CSIRS no later than CSI reference resource is received before triggering PDCCH. Agreement: - In Table I, (Z,Z’) values are baseline values which shall be supported by all UEs. - All (Z,Z’) values in Table I are TBD. - FFS whether (Z,Z’) values for low latency and high latency CSI in Table I are the same for a given numerology, in case of normal UE. - Note. If those two values are the same for all numerology, then low and high latency is merged for normal UE. - In Table II, for a given numerology and CSI latency, whether or not to support the (Z,Z’) value in Table II is reported as a UE capability. - For a given numerology and CSI latency, the (Z,Z’) values in Table II should be equal to or smaller than (Z,Z’) values in Table I. - All (Z,Z’) are fixed in specification and the values are TBD - If no agreement on the values for Table II can be achieved, advanced UE is not supported for Rel-15 Table1. CSI calculation time Z for normal UE. CSI latency; Units; 15kHz SCS; 30kHz SCS; 60kHz SCS; 120kHz SCS. Low latency; Symbols; (Z1,1, Z’1,1); (Z1,2, Z’1,2); (Z1,3, Z’1,3); (Z1,4, Z’1,4). High latency; Symbols; (Z2,1, Z’2,1); (Z2,2, Z’2,2); (Z2,3, Z’2,3); (Z2,4, Z’2,4). Table2. CSI calculation time Z for advanced UE. CSI latency; Units; 15kHz SCS; 30kHz SCS; 60kHz SCS; 120kHz SCS. Low latency; Symbols; (Z1,1, Z’1,1); (Z1,2, Z’1,2); (Z1,3, Z’1,3); (Z1,4, Z’1,4). High latency; Symbols; (Z2,1, Z’2,1); (Z2,2, Z’2,2); (Z2,3, Z’2,3); (Z2,4, Z’2,4). Agreement: RAN4 specifies the mapping between reported value of L1-RSRP and the measured quantity value and includes that mapping in 38.133. - A reference to the RAN4 specification with the above details will be added in TS38.214 Agreement RAN4 specifies the exact mapping for the differential RSRP values with the following input from RAN1. For differential reporting: - 16 states are supported - One state is used to indicate that the difference is larger than 30dB - Step size is 2dB Agreement (RRC parameter update) In group based beam reporting, NR supports the following aspects: - The reporting format of differential L1-RSRP for non-group based beam reporting is reused for group based beam reporting. - The RRC parameter nrofBeamsToReport can be removed (TS38.331). Corresponding RAN1 parameter name is number-of-beams-reporting. Agreement: If all configured TCI states do NOT contain QCL Type D i.e. QCL w.r.t. spatial Rx parameter, a UE shall obtain the other QCL assumptions from the indicated TCI state for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH Agreement (RRC parameter update): Maximum number of spatial relations in PUCCH-SpatialRelationInfo is 8. Update to 38.331: maxNrofSpatialRelationInfos = 8. Agreement (RRC parameter update): As previously agreed, maximum number of TCI states in TCI-StatesPDCCH is the same as in TCI-States, i.e., 64. Update to 38.331: maxNrofTCI-StatesPDCCH = 64. Agreement (RRC parameter update): As previously agreed, maximum number of TCI states is 64. Hence, the ID range is 0 .. 63. Update to 38.331: TCI-StateId = 0 .. 63. Agreement (New RRC parameter): Update the description of the RRC parameters TCI-RS-SetConfig and TRS-Info according to the table below RRC parameter; Description; Value range; Comments. TRS-Info; Indicating whether or not the antenna ports of NZP CSI-RS resources in the CSI-RS resource set is same; ON, OFF; Contained in NZP-CSI-RS-ResourceSetConfig. Agreements (RRC parameter update): Update to 38.331: maxNrofCSI-RS-ResourcesPerSet = 64. - UE is not expected to be configured with more than 8 CSI-RS resources per set for CSI acquisition - TS38.214 Editor to capture this restriction in the specification Agreements (RRC parameter update): The maximum number of configured ap-CSI-RS resources per set for beam management is 16. Introduce new constant in 38.331: maxNrofAP-CSI-RS-ResourcesPerSet = 16. Agreement (RRC parameter update): Update to 38.331: Size of list qcl-Info-aPeriodicReportingTrigger is maxNrofReportConfigIdsPerTrigger * maxNrofAP-CSI-RS-ResouresPerSet Agreement: UE expects a dedicated SearchSpace configuration that is one-to-one mapped to CORESET-BFR (RRC parameter CORESET-BFR remains) Agreement (RRC parameter update): Value range for the higher layer parameter BeamFailureInstanceMaxCount is {1,2,3,4,5,6,8,10} Agreement (RRC parameter update): Value range of beam failure recovery timer is {10, 20, 40, 60, 80, 100, 150, 200} in units of msec - Additional values can be added by RAN2 (e.g. 500msec) Agreement (RRC parameter update): Value range of maximum number of CF BFRQ transmissions is {3, 4, 5, 6, 7, 8, 10, 20, 50} - Additional values can be added by RAN2 (e.g. 100, 200) Agreement (RRC parameter update): maxNrofFailureDetectionResources is 2 per BWP Agreement: RRC parameter value: maxNrofCandidateBeams is 64 - Note: the total number of distinct resources for candidate beam RS and beam management RS is also constrained by 64 Agreement: For candidate SSB/CSI-RS beam selection L1-RSRP threshold determination, RRC parameter “beamFailureCandidateBeamThreshold” is used to provide L1-RSRP threshold of SSB beam selection. - Corresponding L1-RSRP thresholds for a CSI-RS resource is linearly scaled based on Pc_ss corresponding to the CSI-RS resource Agreement: Only 1-port CSI-RS and SSB are supported for BFD RS set q0 Agreement: The following text change for TS38.213 section 6 is agreed. A UE can be configured, for a serving cell, with a set of periodic CSI-RS resource configuration indexes by higher layer parameter Beam-Failure-Detection-RS-ResourceConfig and with a set of CSI-RS resource configuration indexes and/or SS/PBCH block indexes by higher layer parameter Candidate-Beam-RS-List for radio link quality measurements on the serving cell. If the UE is not provided with higher layer parameter Beam-Failure-Detection-RS-ResourceConfig, the UE determines the set to include SS/PBCH block indexes and periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by the TCI states for respective control resource sets that the UE is configured for monitoring PDCCH. If for a control resource set that the UE is configured for monitoring PDCCH, the RS indexes of SS/PBCH blocks or periodic CSI-RS resource configurations in the RS sets indicated by the TCI state for the control resource set do not have same values as indexes for SS/PBCH blocks or periodic CSI-RS resource configurations in the set , the UE determines that the set includes indexes of SS/PBCH blocks or periodic CSI-RS resource configurations with same values as the ones provided by higher layer parameter TCI-StatesPDCCH for the control resource set. Agreement: Specificiation to clarify that after PRACH transmission on slot n for BFR, gNB response observation starts from slot n+4 The following text proposal for TS38.213 section 6 is agreed. < Unchanged parts are omitted > A UE is configured with one control resource set by higher layer parameter Beam-failure-Recovery-Response-CORESET. The UE may receive from higher layers, by parameter Beam-failure-recovery-request-RACH-Resource, a configuration for a PRACH transmission as described in Subclause 8.1. After 4 slots from the slot of the PRACH transmission, With the PRACH transmited in slot n, starting from slot n+4, the UE monitors PDCCH for a DCI format with CRC scrambled by C-RNTI, within a window configured by higher layer parameter Beam-failure-recovery-request-window, and receives PDSCH according to an antenna port quasi co-location associated with periodic CSI-RS configuration or SS/PBCH block with index in set , in the control resource set configured by higher layer parameter Beam-failure-Recovery-Response-CORESET. < Unchanged parts are omitted > Agreement The following text proposal for section 5.2.2.1.1 of TS 38.214 is agreed. ---- Start of text proposal ---- - The PDSCH transmission scheme where the UE may assume that the gNB transmission on the PDSCH transmission would be performed with up to 8 transmission layers on antenna ports [1000-1011] as defined in Subclause 7.3.1.4 of [4, TS 38.211]. For CQI calculation UE should assume PDSCH signals on antenna ports in the set [1000-1008 + υ - 1] for υ layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,3000 + P - 1], as given by, , where is a vector of PDSCH symbols from the layer mapping defined in Subclause 7.3.1.4 of [4, TS 38.211], P ϵ [1,2,4,8,12,16,24,32] is the number of CSI-RS ports. If only one CSI-RS port is configured, is 1, otherwise is the precoding matrix corresponding to the reported PMI applicable to . The corresponding PDSCH signals transmitted on antenna ports [3000,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Subclause 4.1. ---- End of text proposal ---- Agreement The following text proposal is agreed for TS38.214 section 5.2.2.1.1. ---- Start of text proposal ---- 5.2.2.1.1 CSI reference resource definition The CSI reference resource for a serving cell is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CQI value relates. - In the time domain, for a UE configured with a single CSI resource set for the serving cell, the CSI reference resource is defined by a single downlink slot n-nCQI_ref, - where for periodic and semi-persistent CSI reporting nCQI_ref is the smallest value greater than or equal to [TBD], such that it corresponds to a valid downlink slot. - where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCQI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCQI_ref is the smallest value greater than or equal to [TBD], such that slot n-nCQI_ref corresponds to a valid downlink slot. A slot in a serving cell shall be considered to be a valid downlink slot if: - it is configured as a downlink slot for that UE, and - it does not fall within a configured measurement gap for that UE. and - the active DL BWP in the slot is the same as the DL BWP for which the CSI reporting is performed. - there is at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement no later than CSI reference resource for which the CSI reporting is performed. ---- End of text proposal ---- Agreement: The following text proposals for TS38.214 section 5.2.2 is agreed. ---- Start of text proposal ---- - A single PDSCH transport block with a combination of modulation scheme, target code rate, and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CSI reference resource, could be received with a transport block error probability not exceeding: - 0.1, if the higher layer parameter CQI-table configures Table 5.2.2.1-2, or Table 5.2.2.1-3, or - a higher layer configured BLER-target, if the higher layer parameter CQI-table configures Table 5.2.2.1-4. ---- End of text proposal ---- Agreement: The following text proposals for TS 38.214 v15.0.0 Section 6.1.4.1 is agreed. < Unchanged parts are omitted > elseif transform precoding is enabled and MCS-Table-PUSCH-transform-precoding is not set to '256QAM', - the UE shall use IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink uplink shared channel. - for MCS index 0 and 1, q=1 if UE has reported to support pi/2 BPSK modulation; and q=2 in other cases else - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel. end Table 6.1.4.1-1: MCS index table for PUSCH with transform precoding and 64QAM MCS Index IMCS; Modulation Order Qm; Target code Rate x 1024 R; Spectral efficiency. 0; 1q; 240/q; 0.2344. 1; 1q; 314/q; 0.3066. 2; 2; 193; 0.3770. 3; 2; 251; 0.4902. 4; 2; 308; 0.6016. 5; 2; 379; 0.7402. 6; 2; 449; 0.8770. 7; 2; 526; 1.0273. 8; 2; 602; 1.1758. 9; 2; 679; 1.3262. 10; 4; 340; 1.3281. 11; 4; 378; 1.4766. 12; 4; 434; 1.6953. 13; 4; 490; 1.9141. 14; 4; 553; 2.1602. 15; 4; 616; 2.4063. 16; 4; 658; 2.5703. 17; 6; 466; 2.7305. 18; 6; 517; 3.0293. 19; 6; 567; 3.3223. 20; 6; 616; 3.6094. 21; 6; 666; 3.9023. 22; 6; 719; 4.2129. 23; 6; 772; 4.5234. 24; 6; 822; 4.8164. 25; 6; 873; 5.1152. 26; 6; 910; 5.3320. 27; 6; 948; 5.5547. 28; 1; reserved. 29; 2; reserved. 30; 4; reserved. 31; 6; reserved. < Unchanged parts are omitted > Agreement: On the issue of whether the subcarrier spacing for SSB and DMRS can be different: - Support the same or different subcarrier spacing for SS/PBCH block and PDSCH DMRS within a CC. For the case of 240 kHz SCS, the same subcarrier spacing is precluded. - Support for different subcarrier spacing for SS/PBCH block and PDSCH DMRS within a CC is a UE capability Agreement: From a UE’s perspective, SRS and PRACH are not transmitted simultaneously - If SRS and PRACH have to be transmitted on overlapping OFDM symbols, SRS is not transmitted Agreement: The UE is not expected to receive PDSCH scheduling grant which indicates CDM group(s) with potential DMRS ports which overlap with any configured CSI-RS resource(s) for that UE - Above applies for the case of TRS as well Agreement: the UE is not expected to receive CSI-RS and SIB1 message in overlapping RBs Agreement (RRC parameter update): maxNrofCSI-IM-Resources INTEGER ::= 32 -- Maximum number of CSI-IM resources per CC. See CSI-IM-ResourceMax in 38.214. Agreement (RRC parameter update): Revise the higher-layer parameter CSI-RS-ResourceRep defined in TS 38.331 to be an optionally configurable parameter (not mandatory parameter as in the current specification), and include this into an LS to RAN2. - CSI-RS-ResourceRep can only be configured for CSI-RS resource sets which are associated with CSI report with report of L1 RSRP or “no report” Agreement: For the CSI-RS sequence mapping, the working assumption from RAN1 NR Ad Hoc 1801 for density 1 and 3 is confirmed Agreement: - By default, UE does not perform rate matching on REs overlapped with at least CSI-RS for mobility - Note: UE shall perform rate matching on REs overlapped with a CSI-RS for mobility only if ZP-CSI-RS covers the REs overlapped with the CSI-RS for mobility. Conclusion: UE performs the same measurement on NZP CSI-RS or CSI-IM regardless of whether they collide with ZP CSI-RS or not. Agreement: - The formula for SRS can be reused for CSI-RS: It’s up to the editor how to reflect the above agreement into the specification. Agreement (RRC parameter update): maxNrofNZP-CSI-RS-Resources-1 is 191 Agreement (RRC parameters update): Send as part of LS to RAN2 to inform that the parameters DL-DMRS-add-pos/UL-DMRS-add-pos, DL-DMRS-config-type/UL-DMRS-config-type, DL-DMRS-max-len/UL-DMRS-max-len can be configured separately for PDSCH/PUSCH Type A and PDSCH/PUSCH Type B. Agreement: In Section 6.4.1.1.1 in 38.211, 14 should be replaced by because of only 12 symbols in one slot for extended CP. Agreement: The following text proposal for TS38.211 section 5.2.2 is agreed. Base sequences are divided into groups, where is the group number and is the base sequence number within the group, such that each group contains one base sequence () of each length , and two base sequences () of each length , . The definition of the base sequence depends on the sequence length . Agreement Following text proposal for TS38.211 section 6.4.1.1.3 is agreed. For PUSCH mapping type A, the case UL-DMRS-add-pos equal to 3 is only supported when UDL-DMRS-typeA-pos is equal to 2.” Agreement: Make the following change in Section 6.4.1.1.3 of 38.211 if transform precoding is enabled - Agreement: Agree to the following changes in 5.1.6.2 in Section 38.214: “For PDSCH with allocation duration of 7 symbols with mapping type B, the UE shall assume one additional single-symbol DM-RS present in the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol respectively of within the PDSCH allocation duration, otherwise the UE shall assume that the additional DMRS symbol is not present.” Agree to the following changes in 7.4.1.1.2 in Section 38.211: “if the PDSCH duration is 7 symbols, the UE is not expected to receive the first DM-RS beyond the fourth symbol, and if one additional single-symbol DM-RS is configured, the UE should only expect the additional DM-RS to be transmitted on the 5th or 6th symbol relative to the first DM-RS symbol. when the front-loaded DM-RS symbol is in the 1st or 2nd symbol respectively of the PDSCH allocation duration, otherwise the UE should expect that the additional DM-RS is not transmitted.” Agreement Following text proposal is agreed. Section 7.4.1.1.1 in 38.211: < Unchanged parts are omitted > and, is given by the higher-layer parameter DL-DMRS-Scrambling-ID if provided and the PDSCH is scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI < Unchanged parts are omitted > Section 6.4.1.1.1 in 38.211: < Unchanged parts are omitted > and , is given by the higher-layer parameter UL-DMRS-Scrambling-ID if provided and the PUSCH is not a Msg3 PUSCH according to Clause 8.3 in [TS38.213]. < Unchanged parts are omitted > Section 6.4.1.1.2 in 38.211: < Unchanged parts are omitted > - if is configured by the higher-layer parameter nPUSCH-Identity-Transform-precoding and the transmitted PUSCH is not a Msg3 PUSCH according to Clause 8.3 in [TS38.213]. < Unchanged parts are omitted > Agreement - Text proposal for Tables 6.4.1.1.3-3, 6.4.1.1.3-4, 7.4.1.1.2-1, and 7.4.1.1.2-2 in TS38.211: - Table 6.4.1.1.3-3: PUSCH DM-RS positions for single-symbol DM-RS. Duration in symbols; DM-RS positions. PUSCH mapping type A; PUSCH mapping type B. UL-DMRS-add-pos; UL-DMRS-add-pos. 0; 1; 2; 3; 0; 1; 2; 3. <4; -; -; -; -. 4. 5; , [4]; , [4]; , [4]. 6; , 4; , 4; , 4. 7; , 4; , 4; , 4. 8; , [7]; , [7]; , [7]; , 6; , 3, 6; , 3, 6. 9; , 7; , 7; , 7; , 6; , 3, 6; , 3, 6. 10; , 9; , 6, 9; , 6, 9; , 8; , 4, 8; , 3, 6, 9. 11; , 9; , 6, 9; , 6, 9; , 8; , 4, 8; , 3, 6, 9. 12; , 9; , 6, 9; , 5, 8, 11; , 10; , 5, 10; , 3, 6, 9. 13; , 11; , 7, 11; , 5, 8, 11; , 10; , 5, 10; , 3, 6, 9. 14; , 11; , 7, 11; , 5, 8, 11; , 10; , 5, 10; , 3, 6, 9. - Table 6.4.1.1.3-4: PUSCH DM-RS positions for double-symbol DM-RS. Duration in symbols; DM-RS positions. PUSCH mapping type A; PUSCH mapping type B. UL-DMRS-add-pos; UL-DMRS-add-pos. 0; 1; 2; 3; 0; 1; 2; 3. <4; -; -; -; -. 4; -; -. 5; -. 6; -. 7; -. 8; , 5. 9; , 5. 10; , 8; , 7. 11; , 8; , 7. 12; , 8; , 9. 13; , 10; , 9. 14; , 10; , 9. - Table 7.4.1.1.2-3: PDSCH DM-RS positions for single-symbol DM-RS. Duration in symbols; DM-RS positions. PDSCH mapping type A; PDSCH mapping type B. DL-DMRS-add-pos; DL-DMRS-add-pos. 0; 1; 2; 3; 0; 1; 2; 3. 2; -; -; -; -. 3; -; -. 4. 5; -; -. 6; -; -. 7. 8; ,[7]; ,[7]; ,[7]; -; -. 9; ,7; ,7; ,7; -; -. 10; ,9; , 6, 9; , 6, 9; -; -. 11; ,9; , 6, 9; , 6, 9; -; -. 12; 9; , 6, 9; , 5, 8, 11; -; -. 13; ,11; , 7, 11; , 5, 8, 11; -; -. 14; ,11; , 7, 11; , 5, 8, 11; -; -. - Table 7.4.1.1.2-4: PDSCH DM-RS positions for double-symbol DM-RS. Duration in symbols; DM-RS positions. PDSCH mapping type A; PDSCH mapping type B. DL-DMRS-add-pos; DL-DMRS-add-pos. 0; 1; 2; 0; 1; 2. <4; -; -. 4; -; -. 5; -; -. 6; -; -. 7. 8; -; -. 9; -; -. 10; , 8; -; -. 11; , 8; -; -. 12; , 8; -; -. 13; , 10; -; -. 14; , 10; -; -. Note: Some DMRS pattern(s) with no additional DMRS but configured with UL/DL add_pos larger than 0 in the above table might not have sufficient performance in high mobility scenarios (ex: Table 6.4.1.1.3-3, 7 symbol case) Note: All remaining details of the above tables will be finalized in RAN1#92bis. Agreement: For PDSCH scheduled by PDCCH format 1_0 with CRC scrambled by C-RNTI, or PUSCH scheduled by PDCCH format 0_0 with CRC scrambled by C-RNTI, the following DMRS port is used - For downlink, DMRS port 1000 is used - For uplink, DMRS port 0000 is used - With no data FDMed unless it is 2-symbol PDSCH Type B - FFS: For the case of data FDMed for uplink, for the case of other RNTIs Agreement: The following text proposal is agreed. -----------------------------------------------------Start of CR to 38.211------------------------------------------------------------ 6.4.1.1.3 Mapping to physical resources Unaffected parts omitted… Table 6.4.1.1.3-3: PUSCH DM-RS positions for single-symbol DM-RS. PUSCH duration in symbols; DM-RS positions. PUSCH mapping type A; PUSCH mapping type B. UL-DMRS-add-pos; UL-DMRS-add-pos. 0; 1; 2; 3; 0; 1; 2; 3. <=4; []; 0. 5, 6, 7; -; -; -; 0; 0, 4; -; -. 8; -; -; -; 0; 0, 6; 0, 3, 6; -. 9; , 7; -; -; 0; 0, 6; 0, 3, 6; -. 10; , 9; , 6, 9; -; 0; 0, 8; 0, 4, 8; 0, 3, 6, 9. 11; , 9; , 6, 9; -; 0; 0, 8; 0, 4, 8; 0, 3, 6, 9. 12; , 9; , 6, 9; , 5, 8, 11; 0; 0, 10; 0, 5, 10; 0, 3, 6, 9. 13; , 11; , 7, 11; , 5, 8, 11; 0; 0, 10; 0, 5, 10; 0, 3, 6, 9. 14; , 11; , 7, 11; , 5, 8, 11; [0]; [0, 10]; [0, 5, 10]; [0, 3, 6, 9]. Unaffected parts omitted… --------------------------------------END of CR to 38.211---------------------------------------------------------------- Conclusion: Regarding PDSCH mapping type B beyond 2/4/7 symbol, no DMRS positions other than these durations are supported Agreement: Regarding PUSCH mapping type B of 14 symbol length, support the agreed DMRS locations of 13 symbols. Agreement: Replace the “0” value of the front-load DMRS of Table 6.4.1.1.3-3 and 6.4.1.1.3-4 in 38.211 with l0 Agreement: Update the corresponding text in Section 5.1.6.2. of 38.214 as follows: “When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, DL-DMRS-max-len and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B, and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted the UE assuming that all the orthogonal antenna ports are associated with transmission of its PDSCH, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.” Agreement: The following text proposal for TS38.211, section 6.4.1.1.2 is agreed. The reference signal sequence shall be generated according to where - and , and are defined in clause 6.4.1.1.3; - and are given by Tables 6.4.1.1.3-1 and 6.4.1.1.3-2; - the set of antenna ports is given by clause 6.4.1.1.2. < Unchanged parts are omitted > The reference signal sequence shall be generated according to where - , , and are defined in clause 6.4.1.1.3; - and are given by Tables 6.4.1.1.3-1 and 6.4.1.1.3-2; - the antenna port is given by clause 6.4.1.1.2. 6.4.1.1.2 Mapping to physical resources The precoded PUSCH DM-RS shall be mapped to physical resources according to type 1 or type 2 as given by the higher-layer parameter UL-DMRS-config-type. The UE shall map the sequence to resource elements according to - if transform precoding is not enabled, and the reference point for is subcarrier 0 in common resource block 0. - if transform precoding is enabled and the reference point for is subcarrier 0 of the lowest-numbered resource block of the scheduled PUSCH allocation. where and are given by Tables 6.4.1.1.3-1 and 6.4.1.1.3-2 and the following conditions are fulfilled: - the resource elements are within the common resource blocks allocated for PUSCH transmission < Unchanged parts are omitted > 6.4.1.1.3 Precoding The reference-signal sequence shall be precoded according to where - the precoding matrix is given by clause 6.3.1.5, – - the set of antenna ports is given by clause 6.3.1.5, and - the set of antenna ports is given by [TS38.214]. Agreement: The following text proposal is agreed. Text proposal for Section 6.2.2 of 38.214 6.2.2 UE DM-RS transmission procedure < Unchanged parts are omitted > If frequency hopping flag as defined in Subclause 7.3.1.1.2 of [5, TS 38.212] is enabled for PUSCH, the UE shall transmit either one single-symbol front-load DM-RS symbol when the higher layer parameters UL-DMRS-add-pos=0 in each hop, otherwise single-symbol front-load DM-RS symbol with one single-symbol additional DM-RS symbol in the 5th symbol with respect to the front-load DMRS symbol in each hop if within the PUSCH allocation of each hop. < Unchanged parts are omitted > Text proposal for Section 6.4.1.1.3 of 38.211 6.4.1.1.3 Mapping to physical resources < Unchanged parts are omitted > The position(s) of the DM-RS symbols is given by and the signalled duration between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PUSCH resources in the slot for PUSCH mapping type A, or the signalled duration of scheduled PUSCH resources for PUSCH mapping type B according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 and 6.4.1.1.3-X. If the frequency hopping flag is enabled, the indexes and start from the beginning of each hop. For PUSCH mapping type A, the case UL-DMRS-add-pos equal to 3 is only supported when UL-DMRS-typeA-pos is equal to 2. The time-domain index and the supported antenna ports are given by Tables 6.4.1.1.3-3 through 6.4.1.1.3-5: - if frequency hopping flag as defined in Subclause 7.3.1.1.2 of [5, TS 38.212] is enabled for PUSCH, Table 6.4.1.1.3-X shall be used. - otherwise, if the higher-layer parameter UL-DMRS-max-len is equal to 1, the tables shall be used according to single-symbol DM-RS - otherwise, if the higher-layer parameter UL-DMRS-max-len is equal to 2, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used. < Unchanged parts are omitted > Table 6.4.1.1.3-X: PUSCH DM-RS positions for frequency hopping PUSCH duration in each hop; DM-RS positions for (first; second) hop. PUSCH mapping type A; PUSCH mapping type B. UL-DMRS-add-pos; UL-DMRS-add-pos; UL-DMRS-add-pos. (=2); (=3). 0; 1; 0; 0; 1. ≤3; -; -; -; l0; 0; -. 4; l0; 0; -; l0; 0; l0; 0; -. 5, 6; l0; 0; -; l0; 0; l0; 0; (l0, l0+4); (0, 4). 7; l0; 0; (l0, l0+4); (0, 4); l0; 0; l0; 0; (l0, l0+4); (0, 4). < Unchanged parts are omitted > Agreement: The UE is not expected to be configured with more PT-RS ports in UL than what the UE has reported need for. Agreement: The text in {38.211:6.4.1.2.1.2 Sequence generation if transform precoding is enabled} is modified as >>>>>>>>>>>> Start text proposal >>>>>>>>>>>> where is the lowest OFDM symbol number in the PUSCH allocation in slot that contains PT-RS according to clause 6.4.1.2.2.2 and is given by the higher-layer parameter nPUSCH-Identity-Transform-precoding. >>>>>>>>>>>> End text proposal >>>>>>>>>>>> Agreement: The following text proposal related to PT-RS in 38.211 is agreed >>>>>>>>>>>> Start text proposal section 6.4.1.2.1.1 >>>>>>>>>>>> If transform precoding is not enabled, the precoded phase-tracking reference signal for subcarrier on layer is given by where - antenna ports or associated with PT-RS transmission are given by clause 6.2.3 of [TS38.214] - the precoding matrix is given by clause 6.3.1.5. - is given by clause 6.4.1.1.1.1 - at the position of the first DM-RS symbol in absence of PUSCH intra-slot frequency hopping - at the position of the first DM-RS symbol in hop h in presence of PUSCH intra-slot frequency hopping where >>>>>>>>>>>> End text proposal section 6.4.1.2.1.1 >>>>>>>>>>>> Agreement: Correct section 7.4.1.2.2 in 38.211 as follows >>>>>>>>>>>> Start text proposal section 7.4.1.2.2 >>>>>>>>>>>> If present, the UE shall assume the PDSCH PT-RS is scaled by a factor to conform with the transmission power specified in clause 4.1 of [6, TS38.214] and mapped to resource elements according to when all the following conditions are fulfilled - is within the OFDM symbols allocated for the PDSCH transmission - resource element is not used for DM-RS, CSI-RS, SS/PBCH block, a detected PDCCH, or by a configured CORESET declared as 'not available for PDSCH' according to clause 5.1.4.1 of [6, TS 38.214] >>>>>>>>>>>> End text proposal section 7.4.1.2.2 >>>>>>>>>>>> Check offline whether NZP CSI-RS should be added or not (yellow part) Conclusion On the issue of Head-Tail potential issue for DFT-s-OFDM, keep existing agreement and end this discussion for Rel.15 in RAN1. Agreement: In Section 6.4.1.2.2.1 and 7.4.1.2.2 of TS38.211, the parameter in the resource mapping formulas of PTRS is corrected as follows: >>>>>>>>>>>> Start text proposal >>>>>>>>>>>> - is the C-RNTI associated with the DCI scheduling the transmission. >>>>>>>>>>>> End text proposal >>>>>>>>>>>> Agreement: Update 38.214 as follows: >>>>>>>>>>>> Start text proposal Section 6.2.3.1 >>>>>>>>>>>> When transform precoding is not enabled and if a UE is configured with the higher layer parameter UL-PTRS-present set to 'ON', - if the additional higher layer parameters UL-PTRS-time-density and UL-PTRS-frequency-density are configured, and the RNTI equals C-RNTI, CS-RNTI or SPS-CSI-RNTI, the UE shall assume the PT-RS antenna ports' presence and pattern are a function of the corresponding scheduled MCS and scheduled bandwidth in a corresponding bandwidth part as shown in Table 6.2.3-1 and Table 6.2.3-2, - if the higher-layer parameter timeDensity is not configured, the UE may assume LPT-RS = 1. -if the higher-layer parameter frequencyDensity is not configured, the UE may assume KPT-RS = 2. - otherwise, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2 and the UE may assume that PT-RS is not present when - the scheduled MCS is smaller than 0, or - the number of scheduled RBs is smaller than 1, - the RNTI equals TC-RNTI Agreement: Update 38.214 as follows: >>>>>>>>>>>> Start text proposal Section 6.2.3.2 >>>>>>>>>>>> When transform precoding is enabled and if a UE is configured with the higher layer parameter UL-PTRS-present-transform-precoding, - the UE shall be configured with the higher layer parameters sampleDensity and the UE shall assume the PT-RS antenna ports' presence and PT-RS group pattern are a function of the corresponding scheduled bandwidth in a corresponding bandwidth part, as shown in Table 6.2.3-3. The UE shall assume no PT-RS is present when the number of scheduled RBs is less than or equal to NRB0 if NRB0 > 0 or if the RNTI equals TC-RNTI. - and the UE shall be configured PT-RS time density LPTRS ={1,2} with the higher-layer parameter timeDensity. Otherwise, the UE shall assume PT-RS is present with LPTRS = 1. >>>>>>>>>>>> End text proposal Section 6.2.3.2 >>>>>>>>>>>> Agreement: Agree to the following text proposal in section 5.1.6.3 in 38.214 The feature lead updated the TP as follows: >>>>>>>>>>>> Start text proposal >>>>>>>>>>>> If the higher layer parameter TCI-PresentinDCI is set as “Disabled”, the scheduled number of PT-RS ports for a UE PDSCH transmission is indicated by the TCI state applied for the CORESET used for the PDCCH transmission that schedules the PDSCH, when the scheduling offset is above the threshold indicated by the UE reported Threshold-Sched-Offset. If the scheduling offset is less or equal than the threshold, the scheduled number of PT-RS ports for a UE PDSCH transmission is indicated by the TCI state applied for the CORESET with the lowest CORESET-ID in the latest slot in which one or more CORESETs are configured for the UE. >>>>>>>>>>>> End text proposal >>>>>>>>>>>> Agreement: - For non-codebook based UL transmission, the association between PTRS port(s) and DMRS port(s) are indicated explicitly in the DCI format 0_1. - At least for 1 PTRS port case, reuse the table 7.3.1.1.2-25 and 7.3.1.1-26 defined in 38.212 (up to editor how to capture this) Agreement: Introduce the following correction to Sections 6.3.1.3, 6.3.1.4, 6.4.1.2.2 of TS 38.211 >>>>>>>>>>>> Start text proposal >>>>>>>>>>>> < Unchanged parts are omitted > 6.3.1.3 Layer mapping For the single codeword q=0, the complex-valued modulation symbols for each of the codeword to be transmitted shall be mapped onto up to four layers according to Table 7.3.1.3-1. Complex-valued modulation symbols for codeword =0 shall be mapped onto the layers xi=[x(0)i…xυ-1i]T where is the number of layers and is the number of modulation symbols per layer. 6.3.1.4 Transform precoding If transform precoding is not enabled, yλi=xλ(i) for each layer . If transform precoding is enabled, . If the procedure in [6, TS 38.214] indicates that phase-tracking reference signals are not being used, the symbols prior to transform precoding are x0i=x0i, i=0…Msymblayer-1 and the block of complex-valued symbols x00…x0Msymblayer-1 for the single layer shall be divided into sets, each corresponding to one OFDM symbol. If the procedure in [6, TS 38.214] indicates that phase-tracking reference signals are being used, the block of complex-valued symbols x(0)0…x(0)Msymblayer-1 shall be divided into sets, each set corresponding to one OFDM symbol, the l-th set containing MscPUSCH-εlNsampgroupNgroupPTRS symbols and being mapped to the complex-valued symbols prior to transform precoding x0(lMscPUSCH+i') in OFDM symbol l before transform precoding, with i'∈[0…MscPUSCH-1] and i’≠m. The index m of PT-RS samples in OFDM symbol l prior to transform precoding, the number of samples per PT-RS group Nsampgroup and the number of PT-RS groups NgroupPTRS are defined in clause 6.4.1.2.2.2. The parameter εl=1 when the l-th OFDM symbol contains PTRS according to clause 6.4.1.2.2.2 and εl=0 otherwise. Transform precoding shall be applied according to resulting in a block of complex-valued symbols . < Unchanged parts are omitted > 6.4.1.2.2.2 Mapping to physical resources if transform precoding is enabled The UE shall transmit phase-tracking reference signals only in the resource blocks used for the PUSCH, and only if the procedure in [6, TS 38.214] indicates that phase-tracking reference signals are being used. The sequence shall be multiplied by and mapped to complex valued symbols in x00 where - x00 are the complex-valued symbols in OFDM symbol before transform precoding according to Subclause 6.3.1.4 < Unchanged parts are omitted > >>>>>>>>>>>> End text proposal >>>>>>>>>>>> Note: The editor is respectfully asked to crosscheck whether the occurrences of variable x00 throughout the specification refer to the complex symbols after layer mapping or to the complex symbols before transform precoding, and in the latter case to modify them to x00, representing the complex-valued symbols in OFDM before transform precoding according to Subclause 6.3.1.4 Working Assumption The following text proposal is agreed. >>>>>>>>>>>> Start text proposal Section 5.1.6.3 of TS38.214>>>>>>>>>>>> If a UE is configured with the higher layer parameter Downlink-PTRS-Config, set to 'ON', - if either or both of the additional higher layer parameters timeDensity and frequencyDensity are both configured, and the RNTI equals C-RNTI or CS-RNTI, the UE shall assume the PT-RS antenna ports' presence and pattern are a function of the corresponding scheduled MCS of the corresponding codeword and scheduled bandwidth in corresponding bandwidth part as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2, - if the higher-layer parameter timeDensity is not configured, the UE may assume LPT-RS = 1. - if the higher-layer parameter frequencyDensity is not configured, the UE may assume KPT-RS = 2. - otherwise, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2 and the UE shall assume that PT-RS is not present when, - the scheduled MCS from Table 5.1.3.1-1 is smaller than 10, or - the scheduled MCS from Table 5.1.3.1-2 is smaller than 5, or - the number of scheduled RBs is smaller than 3, or - the RNTI equals RA-RNTI, SI-RNTI or P-RNTI >>>>>>>>>>>> End text proposal Section 5.1.6.3 >>>>>>>>>>>> Agreement - For codebook based UL transmission, the following table is used for UL PTRS power boosting UL-PTRS-power/; Full coherent; Partial coherent; Non-coherent. 1; 2; 3; 4; 1; 2; 3; 4; 1; 2; 3; 4. 00; 0; 3; 4.77; 6; 0; Q; Q; Q+3; 0; Q; Q; Q. 01; 0; 3; 4.77; 6; 0; 3; 4.77; 6; 0; 3; 4.77; 6. 10; reserved; reserved; reserved. 11; reserved; reserved; reserved. - Q=0 for 1 PT-RS port case, Q=3 for 2 PT-RS port case. - Note: If two PT-RS ports are configured, their EPREs are the same. Agreement (RRC parameter update): SRS resource set configuration is supported per BWP. SRS is always transmitted in an active BWP within an active CC. Agreement (New RRC parameter): - The slot offset with respect to DCI that triggers an SRS resource set is higher layer configured in a per SRS resource set - FFS: Details on the support for aperiodic SRS for 1T4R Agreement (RRC parameter update): maxNrofSRS-ResourceSets INTEGER ::= 16 -- Maximum number of SRS resource sets. maxNrofSRS-Resources INTEGER ::= 64 -- Maximum number of SRS resources in an SRS resource set. maxNrofSRS-ResourcesPerSet INTEGER ::= 16 Working Assumption - NR supports the case that different ports of one 4-port SRS resource have different comb offsets for comb 2 and 4. Agreement: The following text proposal for section 6.3.1.6 of 38.211 is agreed mapped in sequence starting with to resource elements in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission, and - the corresponding resource elements in the corresponding physical resource blocks are not used for transmission of the associated DM-RS, PT-RS, or SRS, or DM-RS intended for other co-scheduled UEs as described in clause 6.4.1.1.3 Agreement (RRC parameter update) - For FR2, additionally support TRS symbol position of {0, 4}, {1, 5}, {2, 6}, {3, 7}, {7, 11}, {8, 12} and {9, 13} Agreement The following text proposal for section 5.1.6.1.1 in TS38.214 is agreed. - the bandwidth of the CSI-RS resource, as given by the higher layer parameter csi-RS-FreqBand, is the minimum of 52 and resource blocks, or is equal to resource blocks - UE is not expected to be configured with the periodicity of slots if the bandwidth of CSI-RS resource is larger than 52 resource blocks Agreement: The following text proposal for TS38.214 subsection 5.1.5 is agreed. A UE should expect only the following QCL-Type configurations in TCI-RS-Set: - If a CSI-RS resource is configured with higher-layer parameter TRS-INFO, the UE should only expect {QCL-TypeC} or {QCL-TypeC and QCL-TypeD} configurations with SS/PBCH block or {QCL-TypeD} with CSI-RS configured with higher-layer parameter [CSI-RS-ResourceRep/for L1-RSRP reporting]. - If a CSI-RS resource is configured without higher-layer parameter TRS-INFO and [CSI-RS-ResourceRep/for L1-RSRP reporting], the UE should only expect {QCL-TypeA} or {QCL-TypeB} configuration with CSI-RS configured with higher-layer parameter TRS-INFO or {QCL-TypeD} with CSI-RS configured with higher-layer parameter [CSI-RS-ResourceRep/for L1-RSRP reporting]. - If a CSI-RS resource is configured with higher-layer parameter [CSI-RS-ResourceRep/for L1-RSRP reporting], the UE should only expect {QCL-TypeA} configuration with CSI-RS with higher-layer parameter TRS-INFO or {QCL-TypeC and QCL-TypeD} configurations with SS/PBCH block or {QCL-TypeD} with CSI-RS configured with higher-layer parameter [CSI-RS-ResourceRep/for L1-RSRP reporting]. - For the DM-RS of CORESET scheduling the PDSCH, the UE should only expect {QCL-TypeA} configuration with CSI-RS configured with higher-layer parameter TRS-INFO or {QCL-TypeA and QCL-TypeD} configuration with SS/PBCH block if UE is not configured with CSI-RS with higher-layer parameter TRS-INFO or {QCL-TypeD} with CSI-RS configured with higher-layer parameter [CSI-RS-ResourceRep/for L1-RSRP reporting] - For the DM-RS of PDSCH, the UE should only expect {QCL-TypeA} configuration with CSI-RS configured without higher-layer parameter TRS-INFO and [CSI-RS-ResourceRep/for L1-RSRP reporting] or {QCL-TypeA} configuration with CSI-RS configured with higher-layer parameter TRS-INFO or {QCL-TypeA and QCL-TypeD} configuration with SS/PBCH block if UE is not configured with CSI-RS with higher-layer parameter TRS-INFO or {QCL-TypeD} with CSI-RS configured with higher-layer parameter [CSI-RS-ResourceRep/for L1-RSRP reporting] or {QCL-TypeA and QCL-TypeD} configuration with CSI-RS configured without higher-layer parameter TRS-INFO and [CSI-RS-ResourceRep/for L1-RSRP reporting].

Scheduling/HARQ aspects

Agreements: - Adopt the following TP for 38.211, 7.4.1.1.1 === Start === The pseudo-random sequence generator shall be initialized with cinit=(217(14ns,f + l + 1)(2NID+1) + 2 NID) mod 231 Where l is the OFDM symbol number within the slot, ns,fμ ns,f is the slot number within a frame, and - NID{0, 1, …, 65535} is given by the higher-layer parameter PDCCH-DMRS-Scrambling-ID if provided NID=NIDcell otherwise. === End === Conclusion: - Regarding the issue of cyclic shift for PDCCH interleaver as discussed in Section 2 of R1-1803298: - No spec update is necessary Agreements: - For RMSI CORESET, interleaved CCE-to-REG mapping with - REG-bundle size of 6 - R=2 - All RMSI CORESET configurations are with precoder granularity = REG-bundle size. Agreements: - To adopt the following TP (Section 7.3.2.3, 38.211) === Start === The scrambling sequence generator shall be initialized with where - For a PDCCH in a UE-specific search space, nID{0, 1, …, 65535} equals the higher-layer parameter Control-scrambling-Identity if configured, - otherwise and where is given by the C-RNTI for a PDCCH in a UE-specific search space if Control-scrambling-Identity is configured and otherwise for a PDCCH in a common search space. === End === Agreements: - If a PDCCH decoding candidate having a CCE overlapped, even partially, with the configured SSB, the UE is not required to monitor the PDCCH with the decoding candidate. Agreements: - To adopt the following TP for 38.211: === Start === 7.3.2.5 Mapping to physical resources The UE shall assume the block of complex-valued symbols to be scaled by a factor and mapped to resource elements not used for the associated PDCCH DMRS in increasing order of first , then , in the resource-element groups used for the monitored PDCCH. The antenna port . === End === Agreements: - To adopt the following TP for 7.4.1.3.2 of 38.211 === Start === 7.4.1.3.2 Mapping to physical resources The UE shall assume the sequence is mapped to resource elements according to where the following conditions are fulfilled - they are within the resource element groups constituting the PDCCH the UE attempts to decode if the higher-layer parameter CORESET-precoder-granularity equals CORESET-REG-bundle-size, - all resource-element groups within the set of contiguous resource blocks in the CORESET where the UE attempts to decode the PDCCH if the higher-layer parameter CORESET-precoder-granularity equals the size of the CORESET in the frequency domain. === End === Agreements: - Adopt following TP: === Start === For interleaved CCE-to-REG mapping, for and for where is configured by the higher-layer parameter CORESET-REG-bundle-size. The interleaver is defined by where is given by the higher-layer parameter CORESET-interleaver-size and where - is a function of for a PDCCH transmitted in a CORESET configured by the PBCH or RMSI - is a function of the higher-layer parameter CORESET-shift-index Agreements: - Confirm the value for Case 1-2. X=0 and Y=0 for Case 2. No consensus on additional Case 2’. Max no. of PDCCH BDs per slot; SCS. 15kHz; 30kHz; 60kHz; 120kHz. Case 1-1; 44; 36; 22; 20. Case 1-2; [44]; -. Case 2; [44+X]; [36+Y]; [22+Y]; [20]. Agreements: - To adopt the TP for TS38.213 Section 10.1 - Also add one sentence in the spec saying “when the number of REGs is not sufficient for a given aggregation level, the UE is not required to monitor candidates of the given aggregation level” - Up to spec editor for final wording === Start === Table 10.1-1: CCE aggregation levels and max number of candidates per CCE aggregation level for Type0/Type0A/Type2-PDCCH common search space CCE Aggregation Level; Number of Candidates. 4; 4. 8; 2. 16; 1. === End === Agreements: - The number of CCEs for PDCCH channel estimation which refers to the union of the sets of CCEs for PDCCH candidates to be monitored, regardless of which REG-bundle size or precoder granularity. - Overlapped CCEs associated with different CORESETs are counted separately. - Overlapped CCEs associated with different PDCCH starting symbols with the same or different search space sets with the same CORESET are counted separately. - Overlapped CCEs associated with same or different search space sets with the same PDCCH starting symbol associated with the same CORESET are counted one. - Note: in the above, the overlapping CCEs for candidates for a given search space set with different starting symbols are assumed to be supported. Agreements: - Change Y_{p, kp} to Y_{p, ns,f ns,fμ} ns,fμ Yp,ns,fμ in the search space hashing function in subclause 10.1 of 38.213, where the index ns,f ns,fμ is the slot number. - (Working assumption) The reset of the update is per radio frame Agreements: - The UE capability signaling for PDCCH BDs in CA is integer value from {4, …, 16}. - Discuss further whether or not to restrict the combination of the number of CCs that a UE can support vs. the number of PDCCH BDs indicated via UE capability signalling Agreements: - Mp,maxL is Mp,s,maxL which is the maximum number of PDCCH candidates for the given aggregation level L across all serving cells for the given search space set s for the given CORESET p Agreements: - Specify PDCCH candidate mapping rules. - PDCCH candidates are mapped to search-space-sets until either or both limit(s) of (number of blind decodes, CCEs for channel estimation) is/are met at least with the following rule - SS type order, e.g. CSS before USS - FFS: further rule within a search space set/type Agreements: - Confirm the following working assumption, with updates: - At least for case 1-1 and case 1-2, all UE supports channel estimation capability for following numbers of 48 CCEs for a given slot per scheduled cell - 56 CCEs for SCS = 15kHz and 30kHz - 48 CCEs for SCS = 60kHz - 32 CCEs for SCS = 120kHz - FFS: cross-carrier scheduling - FFS: wideband RS - FFS: overbooking and/or nested structure - FFS: exceptional case of CCE counting - FFS: for case 2 Agreements: - Adopt following TP (38.213) === Start === A UE determines a PDCCH monitoring occasion from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For the search space set s in the control resource set p, the UE monitors PDCCH in a slot where nfNslotframe,μ+ns,fμ-op,smod kp,s=0 is satisfied, with nf nfbeing the frame number. === End === Agreements: - Remove variable slotFormatIndicator from PDCCH-Config from 38.331 - subcarrierSpacing and subcarrierSpacing2 currently in SlotFormatCombination should be moved to SlotFormatCombinationsPerCell - Recommend RAN2 to change the variable name maxNrofAggregatedCellsPerCellGroup to maxNrofSlotFormatCombinationsPerCell. - Use a value of 16 for the above parameter - For parameter maxNrofSlotFormatCombinationsPerSet, use a value of 4096 - For parameter maxNrofSlotFormatsPerCombination, use a value of 256 - For parameter maxSFI-DCI-PayloadSize, use a value of 128. - For cell-specific DL/UL assignment, which has tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationCommon2 as shown below, - Clarify that when tdd-UL-DL-ConfigurationCommon2 is configured, the cell specific DL/UL pattern is a concatenation of the pattern specified in tdd-UL-DL-ConfigurationCommon and the pattern specified in tdd-UL-DL-ConfigurationCommon2, which is tdd-UL-DL-ConfigurationCommon + tdd-UL-DL-ConfigurationCommon2. -- A cell-specific TDD UL/DL configuration. tdd-UL-DL-ConfigurationCommon TDD-UL-DL-ConfigCommon OPTIONAL, -- Cond TDD -- A second cell-specific TDD UL/DL configuration. -- FFS_CHECK: What does the UE do with two? Which one applies? A union of both? If so, how? tdd-UL-DL-ConfigurationCommon2 TDD-UL-DL-ConfigCommon OPTIONAL, -- Cond TDD Agreements: - For the CSS which a DCI format 2_0 is configured to be monitored on, the UE will only monitor the first one or two (from SFI configuration) PDCCH candidates of the configured aggregation level for DCI format 2_0 Agreements: - If a configured DCI format 2_0 is not received, PDCCH monitoring is performed till the next configured DCI format 2_0 monitoring occasion Agreements: - Restrict the combined periodicity for cell-specific DL/UL assignment to such that 20ms is a multiple of the combined periodicity Agreements: Adopt the following text in 38.213 Section 11.1.1 for SUL SFI configuration ===Start========= For unpaired spectrum operation with SUL on a serving cell, the SFI-index field in DCI format 2_0 indicates a combination of slot formats that includes a combination of slot formats for a reference non-SUL carrier and a combination of slot formats for a reference SUL carrier of the serving cell. A UE is provided by higher layer parameter SFI-scs a reference subcarrier spacing configuration of for the combination of slot formats indicated by the SFI-index field in DCI format 2_0 for the reference non-SUL carrier of the serving cell. The UE is provided by higher layer parameter SFI-scs2 a reference subcarrier spacing configuration of for the combination of slot formats indicated by the SFI-index field in DCI format 2_0 for the reference SUL carrier of the serving cell. For each () values, the first values for the combination of slot formats are applicable to the reference non-SUL carrier and the next value is applicable to the reference SUL carrier. The UE is expected to be provided with a reference subcarrier spacing configuration of so that for an active non-SUL BWP with subcarrier spacing configuration of , it is . The UE is expected to be provided with a reference subcarrier spacing configuration of so that for an active SUL BWP with subcarrier spacing configuration of , it is . Each slot format for a combination of slot formats indicated by the SFI-index field in DCI format 2_0 for the reference non-SUL BWP is applicable to consecutive slots for the active non-SUL BWP where the first slot starts at a same time as a first slot in the reference non-SUL BWP. Each slot format for the combination of slot formats for the reference SUL BWP is applicable to consecutive slots for the active SUL BWP where the first slot starts at a same time as a first slot in the reference SUL BWP. A reference subcarrier spacing configurations of , or , or , or , or is either 0, or 1, or 2 for frequency range 1 and is either 2 or 3 for frequency range 2. ==========End========================= Agreements: - When the cell-specific DL/UL configuration is configured, no explicit offset is provided for the starting slot of the configured period, but the first slot of each even radio frame should be a starting slot of the configured cell-specific DL/UL pattern Agreements: - If a configured DCI format 2_0 is not received, before the next configured DCI format 2_0 monitoring occasion, UE will cancel RRC configured transmission, and assume RRC configured DL transmission is not transmitted, during semi-static configured flexible symbols R1-1803515 contains TP capturing some of the above agreements, which can be used for editor as a starting point. Agreements: - DCI formats 0-0/1-0, 0-1, and 1-1 can have different sizes. - DCI formats 0-1 and 1-1 can be received in USS only. The size is determined by the active BWP. Agreements: Rearding the Working assumption: - The number of bits in the resource allocation field for format 0-0 and 1-0 depends on search space: - In CSS(s) in CORESET 0, use initial DL BWP for DCI size determination and RB numbering - FFS If a UE monitors 0-0/1-0 in CSS in CORESET 0 in a slot, it does not monitor formats 0-0 or 1-0 (or 2-x family in case they have a size aligned with 0-0/1-0) in any other search space - Otherwise, use active BWP for DCI size determination and RB numbering It is replaced by the following working assumption: - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWP - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled - FFS: Otherwise, for format 0-0/1-0, the size is given by the initial DL BWP - FFS: how to meet the C-RNTI size and DCI size budget per slot - align 0-1 and 1-1 - configure active BWP such that the DCI size is the same as of the initial BWP - do not configure 0-1 and 1-1 - do not configure 0-0/1-0 in USS - other are not precluded - FFS: for format 0-0/1-0, how to interpret the frequency-domain field in a DCI with a size defined from a BWP with a different size than the BWP it is applied to Agreements: - To adopt the following TP for Section 9.2.4 for 38.213: The UE is configured a PUCCH resource by higher layer parameter SR-resource providing a PUCCH format 0 resource through higher layer parameter PUCCH-format0 or a PUCCH format 1 resource through higher layer parameter PUCCH-format1 as described in Subclause 9.2.1. The UE is also configured a periodicity in symbols if the periodicity is less than a slot and in slots otherwise, by higher layer parameter SR-periodicity, an offset in slots by higher layer parameter SR-offset for a PUCCH transmission conveying SR. If is larger than one slot, the UE determines occasions for SR transmission in a PUCCH from the values of and in the slot satisfying,where nf is the frame number. If is one slot, the UE expects that =0 and every slot is an SR transmission occasion in a PUCCH. If is smaller than one slot, a first symbol index for an occasion of an SR transmission in a PUCCH satisfies is given as where startingSymbol is the value of higher layer parameter startingSymbol and ls is the symbol index within a slot, starting with ls=0 for the first symbol in the slot. If the UE determines that, for an occasion of an SR transmission in a PUCCH in a slot, the number of symbols available for the PUCCH transmission is smaller than the value provided by higher layer parameter nrofSymbols, the UE does not transmit the PUCCH in the slot. Agreements: - To adopt the following TP for Section 6.4.1.3.2.1 of 38.211 The reference-signal sequence shall be generated according to . where the pseudo-random sequence is defined in clause 5.2, m=0, …, Msymb/3-1, and Msymb Msymb is defined in clause 6.3.2.5.2. The pseudo-random sequence generator shall be initialized with where is the OFDM symbol number within the slot, is the slot number within the radio frame, and is given by the higher-layer parameter UL-DMRS-Scrambling-ID if provided and by otherwise. Agreements: - For PUCCH formats 0, 1, 3 & 4, the base sequence is hopped per frequency hop when frequency hopping is enabled for the PUCCH - FFS the details Agreements: - If a UCI transmission on a PUCCH from a UE in a slot using format 2 or 3 or 4 overlaps in time with K configured PUCCH SR resources which are overlapped in time, X bits are used to represent an SR being indicated by the UE where - All-zero X represents the absence of an SR. Otherwise, X represents the presence of an SR. - Note: X bits should be considered when the UCI is encoded irrespective of the SR being present of absent. - X=ceil(log2(K+1)) where K is the number of configured SR PUCCH IDs (schedulingRequestResourceId) that overlap in time with the UCI transmission on PUCCH in the slot. - The K IDs are ordered according to an increasing number of the corresponding SR IDs. The index of an ordered configured PUCCH resource ID is obtained from X bits when SR is triggered. Working assumption: - The maximum number of configured SR PUCCH resources per BWP of a cell for a UE is: 8 Agreements: - To adopt the following TP for Section 6.3.2.2.1 of 38.211 - if PUCCH-GroupHopping equals ‘enabled’ fgh=m=072mc82nsμ+nhop+mmod30 fss=nIDmod30 v = 0 where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with where is given by [5, TS 38.213]. - if PUCCH-GroupHopping equals ‘disabled’ fgh = 0 fss=nIDmod30 v=c(2ns+nhop) where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized at the beginning of each radio frame with where is given by [5, TS 38.213], and nhop={0, 1}nhop=0, 1is a frequency hopping index of each slot depends on the higher-layer parameter PUCCH-frequency-hopping as described in [5, TS 38.213] and the frequency hopping index: - nhop nhop = 0 if PUCCH-frequency-hopping = disabled - nhop nhop = 0 if PUCCH-frequency-hopping = enabled, and if the 1st frequency hop - nhop nhop = 1 if PUCCH-frequency-hopping = enabled, and if the 2nd frequency hop. Agreements: - Remove RRC signal Scrambling ID for PUCCH corresponding to L1 parameter 'ScramblingID' Agreements: - When AN/SR and CSI PUCCH resources have the same starting symbols, one PUCCH resource is used for transmission of AN/SR and CSI. - If the UE is configured with more than one PUCCH resource sets, the PUCCH resource set is determined based on the total number of AN/SR and CSI. The PUCCH resource is determined based on ARI. - FFS on UE assumption on the CSI part 2 is present - FFS if/how to multiplex CSI and AN/SR in case PUCCH format indicated by ARI for AN/SR and CSI is different from the configured PUCCH format for CSI. - If the UE is configured with only one PUCCH resource set, - FSS how to determine the PUCCH resource for transmission of UCI Agreements: - For intra-slot hopping pattern for long PUCCH over multi-slots, the intra-slot hopping pattern is the same for each slot Agreements: - To capture that the UE does not transmit PUCCH if the PUCCH transmission would have its transmission overlapping in symbols configured for SS/PBCH transmission Agreements: - To adopt the following TP for 9.2.6 of 38.213: If a UE is provided higher layer parameter UL-DL-configuration-common, or is additionally provided higher layer parameter UL-DL-configuration-common-set2, or is additionally provided higher layer parameter UL-DL-configuration-dedicated for the slot format per slot over the number of slots, as described in Subclause 11.1, the UE determines the slots for a PUCCH transmission starting from a slot indicated to the UE as described in Subclause 9.2.3 and having - an UL symbol or flexible symbol indicated by higher layer parameter PUCCH-F1-F3-F4-starting-symbol as a starting symbol, and - consecutive UL symbols or flexible symbols, starting from the symbol indicated by PUCCH-F1-F3-F4-starting-symbol, equal to or larger than a number of symbols provided by higher layer parameter PUCCH-F1-F3-F4-number-of-symbols. If a UE is not provided higher layer parameter UL-DL-configuration-common or higher layer parameter UL-DL-configuration-common-set2, the UE determines the slots for a PUCCH transmission as the consecutive slots starting from a slot indicated to the UE as described in Subclause 9.2.3. Conclusion: - Editor to check whether the following change is necessary or not. The UE transmits a PUCCH in the PUCCH resource for the corresponding SR configuration only when the UE transmits a positive SR. For a positive SR transmission using PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by setting . For a positive SR transmission using PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b(0)=0. Agreements: In Section 6.3.1.6 in 38.212, define NUCIsymbol as the number of modulated UCI symbols in each PUCCH symbol, as listed below - For PUCCH format 3, , where are number of PRBs that are determined by the UE for PUCCH formats 3 transmission. - For PUCCH format 4, , where is the spreading factor for PUCCH format 4. Agreements: In Section 6.3.1.6 in 38.212, adopt the following TP < Unchanged parts are omitted > for to for to < Unchanged parts are omitted > for to for to Agreements: - Confirm the following working assumption - Locations of the reserved REs for HARQ-ACK are determined following the same rule defined (in RAN1 #91) for mapping modulated HARQ-ACK symbols to REs. Agreements: - To adopt the following TPs for 38.212: ----------------------------------------------------------- Text Proposal Start ------------------------------------------------------ 6.3.2.4.1.1 HARQ-ACK … - if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; else, is the -th code block size for UL-SCH of the PUSCH transmission. … 6.3.2.4.1.2 CSI part 1 … - if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; else, is the -th code block size for UL-SCH of the PUSCH transmission. … 6.3.2.4.1.3 CSI part 2 … - if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; else, is the -th code block size for UL-SCH of the PUSCH transmission. … ----------------------------------------------------------- Text Proposal End ------------------------------------------------------ Agreements: - Ceiling operation is applied for the UCI resource upper bound computation, as indicated below: Conclusion: The current text in 38.212 v15.0.0 for mapping of CSI part 2 resolve the “FFS: how to map CSI part 2” in the following agreement made in RAN1#91 - Agreements: - Detail UCI mapping rule on PUSCH is as follows: - Map HARQ-ACK to REs around DMRS symbol(s) - If PUSCH punctured by HARQ-ACK, - Map CSI part 1 starting after certain amount of reserved HARQ-ACK REs. - FFS reserved HARQ-ACK REs - PUSCH can be mapped to reserved REs - If PUSCH rate matched by HARQ-ACK, - map HARQ-ACK first, followed by CSI part1. - FFS: how to map CSI part 2, e.g., - Map CSI part 2 after CSI part 1 - Map CSI part 2 after UL_SCH - No spec update is necessary Agreements: - When only 0 or 1 bit HARQ-ACK and CSI part 1 (without CSI part 2) are multiplexed on PUSCH without UL-SCH, fill the empty reserved REs with bits that are generated by appending NACK to HARQ-ACK payload to make sure 2 bits HARQ-ACK payload before encoding. Agreements: - When CSI part 2 piggybacked on PUSCH without UL-SCH, lower priority information bits are omitted until CSI Part 2 UCI code rate is below where - FFS: CSI-1 is based on code rate calculated at UE or signalled by gNB in DCI . - Note: if C_T is larger than 1, UE consider it is an error case. Agreements: - When a single slot PUCCH overlap with a single slot PUSCH with the same starting symbol and with different ending symbols, PUCCH is not transmitted and UCI is piggybacked on PUSCH using the same multiplexing rules defined in 38.212 for fully overlapped PUCCH and PUSCH. - FFS: overlap for multiple slot transmissions. Agreements: - In case a single slot PUCCH overlap with a multi-slot PUSCH transmission - If the starting symbol of PUCCH and PUSCH are aligned in a slot, piggyback UCI on PUSCH in that slot using the same multiplexing rules defined in 38.212 for fully overlapped PUCCH and PUSCH and drop PUCCH transmission Agreements: For UCI-only multiplexed on PUSCH without UL-SCH - Modulation order and code rate are signalled in DCI. - Resource determination following the same principle as UCI multiplexing on PUSCH with UL-SCH. - FFS: A-CSI only without UL-SCH on PUSCH is triggered explicitly based on adding one bit in DCI or triggered implicitly based on a special combination of certain existing fields in DCI. - FFS: how modulation and code rate are signalled. Agreements: - 3-bit ARI for DCI 1_0 and DCI 1_1 - At least 8 (up to 32) PUCCH resources can be configured in a resource set with - CCE-index-based implicit mapping is additionally used when >8 resources are configured. - Note: Increasing RRC value range from 8 to 32 - 8 PUCCH resources are configured in a resource set with . - No implicit mapping - Note: Changing RAN1#91 agreement. Agreements: For resource allocation for HARQ-ACK before a UE has a dedicated PUCCH configuration, - Additional PUCCH durations include: - 4-symbol, - starting from Symbol#10. - 10-symbol, - starting from Symbol#4 Working assumption: - For HARQ-ACK resource allocation before a UE has a dedicated PUCCH configuration, - Frequeny hopping is always enabled for PUCCH transmission for FR1 and FR2. Agreements: - For HARQ-ACK resource allocation before a UE has a dedicated PUCCH configuration, - PUCCH format is cell-specific and derived from RMSI indication. - For HARQ-ACK resource allocation before a UE has a dedicated PUCCH configuration, - PUCCH duration and starting symbol are cell-specific and derived by RMSI. Agreements: - For HARQ-ACK resource allocation before a UE has a dedicated PUCCH configuration, 16 UE-specific resources are specified within a PUCCH resource set indicated by RMSI. - PRB indices are determined based on ARI and RMSI. Agreements: - Adopt the following RBG sizes. BWP size (RBs); RBG size. Config 1; Config 2. 1 – 36; 2; 4. 37 – 72; 4; 8. 73 – 144; 8; 16. 145 – 275; 16; 16. Agreements: - Update 38.214 such that the same MCS table and TB size determination mechanism for data channels as defined for the C-RNTI is used for TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, or P-RNTI - Note: there could be limitation(s) for TBS/MCS determination, e.g., modulation order limit for certain cases, etc. Agreements: - Capture interpretation of ‘VRB-to-PRB mapping’ bit in 38.212 (0=non-interleaved, 1=interleaved). - Non-interleaved mapping is used as default in 38.211 when no ‘VRB-to-PRB mapping’ bit is present in the DCI. - The default bundling size is 2. Agreements: - Regarding “Length of the PDSCH is at least X symbols” (PDSCH mapping type A), X=3 - Regarding “Length of the PUSCH is at least Y symbols” (PUSCH mapping type A), Y=4 Agreements: For ECP adopt the following: - PDSCH mapping type A and ECP: - Starting symbol can be symbol index #0, 1, 2, 3 in a slot. - Length of the PDSCH is at least X=3 symbols, up to 12 symbols within a slot, such that slot boundary is not crossed - PDSCH mapping type B and ECP: - Length of the PDSCH can be 2, 4, or 6 symbols. - Starting symbol can be any position within a slot, such that slot boundary is not crossed. - PUSCH mapping type A and ECP: - Starting symbol is symbol index #0 in a slot. - Length of the PUSCH is at least Y=4 symbols, up to 12 symbols - PUSCH mapping type B and ECP - Length of the PUSCH can be 2 through 12 symbols, and with 1 symbol as a working assumption Agreements: - Request RAN2 to introduce possibility for providing the RRC-configured table in RMSI to configure PDSCH and PUSCH symbol allocation for PDSCH/PUSCH scheduling after RMSI, where the RRC-configurable table via dedicated signaling was previously agreed in RAN1 - Draft LS in R1-1803474 (Karri, Nokia), which is approved and final LS in R1-1803510 Agreements: - In case of CA operation, the extra UE processing time shall be introduced based on the max timing difference among CCs defined in RAN4 spec - Note : The value may be SCS dependent, depending on RAN4’s discussion Agreements: - For the processing time formulas given in Section 5.3 & 6.4, in TS 38.214, - (working assumption) Remove timing advance from the processing time formulas - Clarify that the conversion from symbol units to absolute time assumes NCP (with the shorter symbol length) as a nominal convention for both NCP and ECP cases - The UE processing time between PDSCH with SCS u1 and PUCCH with SCS u2 (or between PDCCH with u1 and PUSCH with u2) should be defined as max(T1, T2) based on the agreement in RAN1#91, where T1 or T2 is the absolute time of N1 in case with single SCS u1 or single SCS u2 for both PDSCH and PUCCH (or for both PDCCH and PUSCH). Agreements: - The default number of PDSCH HARQ processes is 8. Working assumption: - For broadcast PDSCH, MCS is limited to QPSK, rank is limited to 1. Agreements: - The UE is not expected to receive both C-RNTI PDSCH and CS-RNTI PDSCH from the primary cell if they are overlapped with at least one symbol. - If both RA-RNTI PDSCH and C-RNTI (or CS-RNTI) PDSCH for a given UE from the primary cell are overlapped with at least one symbol, the UE may skip decoding C-RNTI (or CS-RNTI) PDSCH. Working assumption: - For Capability #1 of UE processing time, the following adjustments to N1 are made: - If the last symbol of the PDSCH Type A allocation ends d symbols before the 7th symbol in the slot, then N1 is increased by d. Agreements: - The default set of K1 slot-timing values to use is {1,2,3,4,5,6,7,8} - For Capability #1 of UE processing time, for PDSCH mapping type B, the following adjustments to N1 are made: - For 7-symbol duration, no change to N1 - For 4-symbol duration, increase N1 by 3 symbols - FFS: For 2-symbol Working assumption: - P-RNTI PDSCH is not required to be decoded by UE in RRC_CONNECTED mode - Note: P-RNTI PDCCH may not schedule a P-RNTI PDSCH but rather carries the message by itself, which may be used for e.g. systemInfoModification, cmas-Indication, and etws-Indication - Send an LS to RAN2 asking for feedback (R1-1803508, which is approved by removing the 2nd subbullet, final LS is R1-1803537) Agreements: - For any two HARQ process IDs A and B for a given cell, if scheduled unicast PDSCH transmission for A comes before the scheduled unicast PDSCH transmission for B then the (baseline capability) UE is not expected to be triggered to send the HARQ-ACK for A after the HARQ-ACK for B - Note: this does not preclude a future capability for UEs to support out-of-order HARQ-ACK. - Send LS to RAN2 to address this capability (R1-1803509, which is approved by removing the 2nd subbullet, final LS is R1-1803538) Agreements: - For the case of DL SPS PDSCH with dynamic SFI, - The UE is not expected to have conflict between the HARQ-ACK feedback on link (DL or UL) direction between that of dynamic SFI and the A/N for SPS PDSCH in Rel-15 Agreements: - The TPs in R1-1803499 are endorsed Agreements: - The TPs in R1-1803500 are endorsed Agreements: - The TPs in R1-1803501 are endorsed Working assumption: - While UE acquires SI upon being triggered by Paging DCI - UE is not required to decode C-RNTI PDSCH if the SI-RNTI PDSCH is overlapped with at least one symbol - In case UE autonomously monitors SI-RNTI PDCCH while monitoring C-RNTI PDCCH, and both SI-RNTI PDSCH and C-RNTI PDSCH are overlapped with at least one symbol, the UE is not required to decode SI-RNTI PDSCH - The first two bullets apply unless TBS of SI-RNTI PDSCH ≤ 2216 for FR1, then UE decodes both SI-RNTI PDSCH and C-RNTI PDSCH - The first two bullets always apply in FR2 - Send an LS to RAN2 asking for feedback – R1-1803536, which is approved and final LS is in R1-1803541 Conclusion: - Regarding DCI signalling for CBG based partial TB retransmission - Both explicit MCS and implicit MCS are used for CBG based partial TB transmission - No spec update is necessary Agreements: - To adopt the following TP in 38.214: […] [In case of inter-slot frequency hopping, hopping happens at each slot. Tthe starting RB during slot is given by: , ] where is the slot number within a radio frame of the first PUSCH slot of a multi-slot PUSCH transmission, is the current slot number within a radio frame, where a multi-slot PUSCH transmission can take place, is the starting resource within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in sub-clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops.. Agreements: - Add to subclause of 6.3, 38.214 to clarify the hopping boundary in time domain when intra-slot frequency hopping is enabled for PUSCH without repetitions, the number of symbols in the first hop is given by floor(PUSCH length in symbols/2). - Text proposal is following: ## 6.3 UE PUSCH frequency hopping procedure […] In case of intra-slot frequency hopping is configured for PUSCH without repetitions, the number of symbols in the first hop is given by NsymbPUSCH,s/2, the number of symbols in the second hop is given by NsymbPUSCH,s-NsymbPUSCH,s/2, where NsymbPUSCH,s is the length of the PUSCH transmission in OFDM symbols in one slot. […]. Agreements: - (working assumption) Adopt the following table for frequency hopping on PUSCH for Msg.3 Table: Hopping bits for frequency hopping Number of PRBs in the initial active UL BWP; Number of hopping bits NUL_hop; Information in hopping bits; Frequency offsets corresponding to the value of hopping bits. < 50; 1; 0; NBWPsize/2. 1; NBWPsize/4. >=50; 2; 00; NBWPsize/2. 01; NBWPsize/4. 10; -NBWPsize/4. 11; Reserved. Agreements: - At least support following special fields for activation/release validation PDCCH for DL SPS, Configured Grant Type. - Corresponding TP will be provided until Friday Table 2: Special fields for DL SPS and Configured grant Type 2 UL transmission Activation PDCCH Validation DCI format 0_0/0_1; DCI format 1_0; DCI format 1_1. HARQ process number; set to all '0's; set to all '0's; set to all '0's. Redundancy version; set to '00'; set to '00'; For the enabled transport block: set to '00'. Table 3: Special fields for DL SPS and Configured grant Type 2 UL transmission Release PDCCH Validation DCI format 0_0; DCI format 1_0. HARQ process number; set to all '0's; set to all '0's. Modulation and coding scheme; set to all '1's; set to all '1's. Resource block assignment; set to all '1's; set to all '1's. Redundancy version; set to '00'; set to '00'. Agreements: - Remove Table 6.1.2.3-1, the periodicity table for configured grant PUSCH transmission, in subclause 6.1.2.3 of TS 38.214 to avoid the double specification - A set of allowed periodicities P are defined in table 6.1.2.3-1 [38.331]. - And remove the entire table 6.1.2.3-1. Agreements: - Data mapping type A for PDSCH and for PUSCH does not support more than one repetition within one slot. Agreements: - The SR configuration should additionally include the value set of {4,8,16}*14. - Support following updated periodicities (shown in red font color with yellow background) for UL transmission with configured grant Table 6.1.2.3-1: Allowed periodicities P for uplink transmission with configured grant CP; Possible values of periodicities P [symbols]. 0; Normal; 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 320, 640}. 1; Normal; 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 640, 1280}. 2; Normal; 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32,40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560}. 2; Extended; 2, 6, n*12, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560}. 3; Normal; 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32,40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120}. Agreements: - For DL TBLBRM calculation, by default: - the reference value for “maximum number of layers for one TB” is ref = 2, and the reference value for “maximum modulation order” is Qm,ref = 6. The “Maximum number of PRBs across all configured BWPs of a carrier” corresponds to that of the initial BWP - By default: - LBRM is disabled for PUSCH transmissions. Agreements: - In case of cross carrier PI with different numerologies, UE is not expected to be configured such that is not an integer number for the carrier being cross-carrier indicated. Agreements: - To adopt the following text proposal for TS38.213 section 11.2 ---------------------------------------Start of the text proposal---------------------------------------------------------------------------- If a UE is provided higher layer parameter Preemp-DL and Preemp-DL= ON, the UE is configured with an INT-RNTI provided by higher layer parameter INT-RNTI for monitoring PDCCH conveying DCI format 2_1 [5, TS 38.212]. The UE is additionally configured with: - control resource set(s) and respective search space sets for monitoring PDCCH with DCI format 2_1 as described in Subclause 10.1; - a set of serving cells by higher layer parameter INT-cell-to-INT; - a mapping for each serving cell in the set of serving cells to a field in DCI format 2_1 by higher layer parameter cell-to-INT; - an information payload size for DCI format 2_1 by higher layer parameter INT-DCI-payload-length; - an indication granularity for time-frequency resources by higher layer parameter INT-TF-unit for each serving cell in the set of serving cells. If a UE detects a DCI format 2_1 for a serving cell from the configured set of serving cells, the UE may assume that no transmission to the UE is present in PRBs and in symbols, from a set of PRBs and a set of symbols of the last monitoring period, that are indicated by the DCI format 2_1. The indication by the DCI format 2_1 is not applicable to receptions of SS/PBCH blocks. The set of PRBs is equal to the active DL BWP as defined in Subclause 12 and includes PRBs. If a UE detects a DCI format 2_1 in a PDCCH transmitted in a control resource set in slot , the set of symbols indicated by a field in DCI format 2_1 includes the last symbols prior to the first symbol of the control resource set in slot where is the value of higher layer parameter Monitoring-periodicity-PDCCH-slot, is the number of symbols per slot, is the subcarrier spacing configuration for a serving cell with mapping to a respective field in the DCI format 2_1, is the subcarrier spacing configuration of the DL BWP where the UE receives the PDCCH conveying the DCI format 2_1, and is a natural number. If the UE is configured with higher layer parameters UL-DL-configuration-common or UL-DL-configuration-common-Set2, symbols indicated as uplink by UL-DL-configuration-common or UL-DL-configuration-common-Set2 are excluded from the last symbols prior to the first symbol of the control resource set in slot . The resulting set of symbols includes a number of symbols that is denoted as . UE is not expected to be configured such that is not an integer number. The UE is configured with the indication granularity for the set of PRBs and for the set of symbols by higher layer parameter INT-TF-unit. If the value of INT-TF-unit is 0, 14 bits of a field in DCI format 2_1 have a one-to-one mapping with 14 groups of consecutive symbols from the set of symbols where each of the first symbol groups includes symbols, each of the last symbol groups includes symbols, a bit value of 0 indicates transmission to the UE in the corresponding symbol group and a bit value of 1 indicates no transmission to the UE in the corresponding symbol group. If the value of INT-TF-unit is 1, 7 pairs of bits of a field in the DCI format 2_1 have a one-to-one mapping with 7 groups of consecutive symbols where each of the first symbol groups includes symbols, each of the last symbol groups includes symbols, a first bit in a pair of bits for a symbol group is applicable to the subset of first PRBs from the set of PRBs, a second bit in the pair of bits for the symbol group is applicable to the subset of last PRBs from the set of PRBs, a bit value of 0 indicates transmission to the UE in the corresponding symbol group and subset of PRBs, and a bit value of 1 indicates no transmission to the UE in the corresponding symbol group and subset of PRBs. ---------------------------------------------end of text proposal--------------------------------------------------------------------------- Agreements: - For DCI format 2-1, the maximum configurable payload size is 126 bits Agreements: - Clarify that: General parameter. RAN2 parameter name; RAN1 feature; Guidance. SRS carrier switching; Per Cell. SUO; Per UE. Note: RAN1 guidance “Per UE” means that the related RRC configuration is not BWP-specific or cell-specific. PDCCH-Config. RAN2 parameter name; RAN1 parameter name; Guidance. slotFormatIndicator; SFI-SS in SFI-PDCCH; Same as search space configuration. Other parameters in SFI-PDCCH; Per Cell. Note: RAN1 guidance on CORESET & search-space-config, “Per Cell (but association with each configured BWP is needed)” in last LS (R1-1801281) means: - CORESETs/search spaces from 0 to the maximum numbers (11 for CORESETs & 39 for search spaces) are configured per cell - However, within each BWP, a UE is further configured with the association of search spaces (up to 10 per BWP per previous RAN1 agreements) with a configured CORESET (up to 3 in total per BWP per previous RAN1 agreements). - The removal of slotFormatIndicator is per the agreement in 7.1.3.1.3 PDSCH-Config. RAN2 parameter name; RAN1 parameter name; Guidance. codeBlockGroupTransmission; CBG-DL; Per Cell. maxCodeBlockGroupsPerTransportBlock; CBGs-per-TB-DL; Per Cell. codeBlockGroupFlushIndicator; CBG-DL-FI; Per Cell. PUSCH-Config. RAN2 parameter name; RAN1 parameter name; Guidance. codeBlockGroupTransmission; CBG-UL; Per Cell. maxCodeBlockGroupsPerTransportBlock; CBGs-per-TB-UL; Per Cell. Agreements: General parameter. RAN2 parameter name; RAN1 feature; Guidance. TPC-SRS-RNTI; Per Cell group. PUCCH-Config. RAN2 parameter name; RAN1 parameter name; Guidance. TPC-PUCCH-RNTI; Per Cell group. PUSCH-Config. RAN2 parameter name; RAN1 parameter name; Guidance. TPC-PUSCH-RNTI; Per Cell group. Agreements: - Add active-BWP-DL-Pcell & active-BWP-UL-Pcell in the RRC parameter list to allow activating DL or UL BWP by dedicated RRC signalling in BWP configuration - Value range is 0, 1, 2, 3 for both parameters, depending on the number of configured DL or UL BWP Agreements: - Adopt the following text for LS reply to Action 4 in R1-1801318: “RAN1 doesn’t have strong concerns to apply the same BWP inactivity timer restarting conditions in TDD to FDD and it’s up to RAN2’s decision.” To check further offline other actions in R1-1801318 – aim to have a single reply LS Agreements: - A UE is not expected to receive DL signals or transmit UL signals during the transition time of active DL or UL BWP switch - For DCI-based active BWP switch, from RAN1 perspective, the transition time of active DL or UL BWP switch is the time duration from the end of last OFDM symbol of the PDCCH carrying the active BWP switch DCI till the beginning of a slot indicated by K0 in the active DL BWP switch DCI or K2 in the active UL BWP switch DCI - For timer-based active BWP switch, from RAN1 perspective, the transition time of active DL or UL BWP switch is the time duration from the beginning of the subframe (FR1) or from the beginning of the half-subframe (FR2) immediately after a BWP timer expires till the beginning of a slot UE is able to receive DL signals or transmit UL signals in the default DL BWP for paired spectrum or the default DL or UL BWP for unpaired spectrum Agreements: - Value range of locationAndBandwidth is 0 ~ (275*(275+1)/2-1) = 37949, which requires 16 bits Agreements: - To adopt the TP below: ------------Begin of Text Proposal for TS38.213 Section 12------------ … For each DL BWP or UL BWP in a set of DL BWPs or UL BWPs, respectively, the UE is configured the following parameters for the serving cell as defined in [4, TS 38.211] or [6, TS 38.214]: - a subcarrier spacing provided by higher layer parameter DL-BWP-mu or UL-BWP-mu; - a cyclic prefix provided by higher layer parameter DL-BWP-CP or UL-BWP-CP; - a RB offset with respect to the RB determined by higher layer parameters offset-pointA-low-scs and ref-scs and a number of contiguous PRBs provided by higher layer parameter DL-BWP-BW or UL-BWP-BW; - an index in the set of DL BWPs or UL BWPs by respective higher layer parameters DL-BWP-index or UL-BWP-index; - DCI 1_0 or DCI 1_1 detection to a PDSCH reception timing values by higher layer parameter DL-data-time-domain, PDSCH reception to a HARQ-ACK transmission timing values by higher layer parameter DL-data-DL-acknowledgement, and DCI 0_0 or DCI 0_1 detection to a PUSCH transmission timing values by higher layer parameter UL-data-time-domain; - an offset of the first PRB of the DL bandwidth or the UL bandwidth, respectively, relative to a first PRB of a bandwidth by higher layer parameter DL-BWP-loc or UL-BWP-loc … ------------End of Text Proposal for TS38.213 Section 12------------ Agreements: - A UE is not expected to receive a DCI indicating active DL (UL) BWP change in OFDM symbols other than the first 3 OFDM symbols of a slot R1-1803539, draft LS is approved, final LS in R1-1803540 Agreements: - When a UE is configured with semi-static HARQ-ACK Codebook, the codebook is not a function of dynamic SFI Agreements: - When a UE is configured with semi-static HARQ-ACK Codebook, support restricting HARQ-ACK timings for DCI format 1_0 to be a subset of ones configured for DCI format 1_1 for a given cell - As an example: Configured HARQ-ACK timing for DCI format 1_1 are {2, 6, 9} then, timings for DCI format 1_0 of {2, 6} are possible, but not if any timing value from{1, 3, 4, 5, 7, 8} - UE is not expected to be indicated by DCI format 1_0 a HARQ-ACK timing which is not based on the above rule Conclusion: - It is understood that before dedicated RRC configuration, since it was agreed previously “HARQ-ACK is only one bit without bundling before RRC connection”, consequently, there is no need to define the default HARQ-ACK codebook before dedicated RRC configuration Agreements: - Confirm the following working assumption with updates: - Working assumption: In case a UE is configured for semi-static HARQ-ACK codebook determination, when the UE detects to receive only one PDSCH within a DL association set for HARQ-ACK feedback on the Pcell, the UE reports HARQ-ACK only for the one PDSCH - FFS whether aAdditional constraintsare necessary for the above operation: only if UE detects DCI format 1_0 with counter DAI value of 1 Agreements: - When a UE is configured with Semi-static HARQ-ACK Codebook, HARQ-ACK corresponding to PDSCH transmission(s) of the DL BWP before DL and/or UL BWP switching is not transmitted by the UE after the switching Working assumption: - When a UE is configured with semi-static HARQ-ACK Codebook, on a per cell basis: - If the UE indicates capability to receive more than one unicast PDSCH per slot, it assumes a max number of non-overlapping candidate unicast PDSCH occasions per slot as determined by the SLIV in the configured pdsch-symbolAllocation table. - Otherwise, the UE is expected to receive only one unicast PDSCH per slot, and HARQ-ACK association set assumes one unicast PDSCH per slot; - Handling HARQ-ACK for PDCCH for SPS release follows the same way as in LTE Agreements: - In Rel-15, UE is not expected to be configured to report HARQ-ACK on the same PUCCH for more than 2 SPS PDSCH receptions - To include this agreements in one of the LSs to RAN2 Agreements: - To respond the LS from RAN2 (on EN-DC): - Simultaneous configuration of EN-DC and NR PUCCH Scell (PScell) - Yes, but the UE is not expected to be configured with more than one PUCCH in NR under EN-DC - Also state that it’s the RAN1 understanding of the intention of the question from RAN2 - Simultaneous transmission of PUCCH and PUSCH in LTE for EN-DC - Yes Agreements: - To adopt the following TP: ======================= Begin of Text Proposal in Section 5.1.4.1 in TS38.214 ================== […] If rate-match-PDSCH-bitmap3 is not configured for bitmap pair and unit of symbol level bitmap is one slot, bitmap-pair applies to each slot of PDSCH scheduled by detected PDCCH. If rate-match-PDSCH-bitmap3 is not configured for bitmap pair and unit of symbol level bitmap is two slots, first slot of bitmap-1 and 2 pair applies to all even-numbered slot(s) and second slot of bitmap pair applies to all odd-numbered slot(s) overlapping with slot(s) of PDSCH scheduled by detected PDCCH. For a bitmap pair included in one or two groups of resource sets and not configured with rate-match-PDSCH-bitmap3, - if unit of rate-match-PDSCH-bitmap2 is one slot, the bitmap-pair applies to each slot of PDSCH scheduled by detected PDCCH. - if unit of rate-match-PDSCH-bitmap2 is two slots, first slot of bitmap pair applies to all even-numbered slot(s) and second slot of bitmap pair applies to all odd-numbered slot(s) overlapping with slot(s) of PDSCH scheduled by detected PDCCH […] ======================= End of Text Proposal in Section 5.1.4.1 in TS38.213 ================== Agreements: - To adopt the following TP ======================= Begin of Text Proposal in Section 7.4.3.1 in TS38.211================== […] Any common resource block partially or fully overlapping with an SS/PBCH block shall be viewed as occupied and not used for transmission of PDSCH or PDCCH in the OFDM symbols where SS/PBCH block is transmitted. The UE may assume that the complex values corresponding to resource elements that are part of such a partially overlapping common resource block, and are not used for SS/PBCH transmission are set to zero in the OFDM symbols where SS/PBCH block is transmitted. […] ======================= End of Text Proposal in Section 7.4.3.1 in TS38.211 ================== Conclusion: - There is ambiguity between AL8 and AL16 PDCCH candidates, and it happens only when non-interleaved CORESET which is 1 symbol long and AL8 and AL16 PDCCH candidates have the same starting CCE. - To address the above ambuiguity: - When non-interleaved CORESET which is 1 symbol long and AL8 and AL16 PDCCH candidates have the same starting CCE, when the UE successfully decodes a PDCCH with AL8 or AL16 satisfying the ambiguity condition, the UE performs rate-matching for PDSCH by assuming AL 16. - Note: the actual spec text wording can be refined

NR-LTE co-existence

Agreement: For a UE operating in EN-DC when configured with Case 1 HARQ timing on an FDD PCell - No changes to the current DCI format size for DCI formats 0 and 1A in the CSS - UE does not use PHICH - The UL scheduling/HARQ timing is as follows: - PUSCH HARQ RTT is 10ms - UL grant in subframe n scheduled PUSCH in subframe n+4, and the UL grant for the same UL HARQ process occurs in subframe n+10 - Note: This supersedes previous agreements on the scheduling/HARQ timing for the UL - The DAI field and the HARQ process number field in DCI formats in the USS follows the design of FDD SCell with TDD PCell - Support PUCCH format 3/4/5 procedures for HARQ-ACK feedback as in LTE FDD SCell with a TDD PCell, with the following exceptions - The PUCCH format 1a/1b resource corresponding to a PDSCH scheduled with a DCI is derived based on the resource determination procedure for FDD - A UE is not expected to receive both unicast PDSCH scheduled by CSS and unicast PDSCH scheduled by USS within a HARQ-ACK bundling window - If a unicast PDSCH is scheduled in CSS, UE assumes DAI=1 - Note: The above does not change the agreement that LTE UL transmissions only occur in the UL subframes (not including special subframes) of the reference UL/DL configuration - Note: Per previous agreement, the PDSCH-to-HARQ feedback timing follows the reference UL/DL configuration, for DL assignments in both CSS and USS. Agreement: NTA-offset of SUL is the same as that of UL - The granularity of NTA-offset is the same as that for CA operation

UL power control

The LS is endorsed in R1-1803351 The LS is endorsed in R1-1803350 Agreement: For the indication of {k} for PUSCH UL-TWG-type2: - Do NOT introduce one new RRC parameter PathlossReferenceIndex into UL-TWG-type2 and the pathloss reference index will be based on activation DCI for UL-TWG-type2 Agreement: At least for the case of initial access - UE will use the SSB identified during the initial access as the DL RS/SSB for pathloss estimation for PUSCH(including MSG3) before DL RS(s) is explicitly configured for pathloss measurement. - UE will use the SSB identified during the initial access as the DL RS/SSB for pathloss estimation for PUCCH before DL RS(s) is explicitly configured for pathloss measurement. Agreement: - For PUCCH format 2, 3, and 4, for the case of small UCI payload size (less than or equal to 11) - ΔPUCCH_TF,c(i) = 10log10(K2∙BPRES(i)) where - BPRES(i) = OUCI(i)/NRE(i) - OUCI(i) is the actual number of UCI bits transmitted in i excluding the known bits as in LTE - NRE(i) = MPUCCH,c(i) x number of subcarriers per PRB x number of DFT-s-OFDM/CP-OFDM symbols excluding DMRS symbols/tones - FFS: K2 (there is no new RRC parameter introduced) Working Assumption (RRC parameter update): Delta-preamble-msg3; Power offset between msg3 and RACH preamble transmission, i.e. similar as in LTE; INTEGER (-1..6) with 1dB step size. p0-pusch; P0 value for PUSCH with grant (except msg3); INTEGER (-16..15) with 1dB step size. p0-pucch; P0 value for PUCCH; INTEGER (-16..15) with 1dB step size. deltaF-pucch-f0; deltaF values configurable for PUCCH format 0; INTEGER (-16..15) with 1dB step size. deltaF-pucch-f1; deltaF values configurable for PUCCH format 1; INTEGER (-16..15) with 1dB step size. deltaF-pucch-f2; deltaF values configurable for PUCCH format 2; INTEGER (-16..15) with 1dB step size. deltaF-pucch-f3; deltaF values configurable for PUCCH format 3; INTEGER (-16..15) with 1dB step size. deltaF-pucch-f4; deltaF values configurable for PUCCH format 4; INTEGER (-16..15) with 1dB step size. Clarification that deltaF-pucch-f2, deltaF-pucch-f3, deltaF-pucch-f4 are included in higher layer PUCCH-config instead of PUCCH-configCommon. Agreement (New RRC parameter): For group-common TPC command in DCI format 2-2, when a UE is not configured with 2 closed loops, NR uses the same approach as LTE (i.e. TPC_index is configured by higher layers and used by the UE to determine the location of the TPC command in the DCI) - New RRC parameter needs to be introduced Agreement: To change PUCCH power control formula as below, where does not depend on . Agreement: A default SRS resource set ID for virtual SRS Type 3 PH is used among the configured SRS resource set IDs - Default SRS resource set ID = 0 Agreement: The following text proposal is agreed: - In Sections 7.1.1 of TS 38.213 - A UE shall reset accumulation for carrier of serving cell - When value is changed by higher layers; - When value is received by higher layers and serving cell is a secondary cell; - When value is changed by higher layers. In Sections 7.3.1 of TS 38.213 - A UE shall reset accumulation for carrier of serving cell - When value is changed by higher layers; When value is changed by higher layers. Agreement: For UE specific pucch-PowerControl and pusch-PowerControl configuration, at least including UL power control parameter set, DL RS for path loss estimate and closed-loop power control process: - Individual parameters are configured per BWP Working Assumption For the case of PUSCH with grant for DCI 0_0 and DCI 0_1 with no SRI field in uplink grant, at least the following is supported - DL RS for PL estimation is given by, the RS corresponding to pusch-pathlossreference-index=0 of pusch-pathloss-Reference-rs (i.e., q_d =0), if only one DL RS for path loss is configured - P0 and alpha are given by, the values corresponding p0alphasetindex =0 of p0-pusch-alpha-setconfig (i.e., j=2), if only one entry of p0-pusch-alpha-setconfig is configured for PUSCH transmission with grant; - Closed loop index l=0 Note that depending on the further agreement on the MIMO session, additional mapping rules for {j, q_d, l} for the PUSCH with grant and no SRI field in uplink grant can be considered. Agreement For the case of PUCCH without PUCCH-Spatial-relation-info, at least the following is supported - DL RS for PL estimation is given by, the RS corresponding to pucch-pathlossreference-index=0 of pucch-pathloss-Reference-rs (i.e., q_d =0), where only one DL RS for path loss is configured - P0 is given by, the values corresponding p0setindex =0 of p0-pucch-set (i.e., q_u=0), where only one entry of p0-pucch-set is configured; - FFS: Closed loop index l=0 Agreement: Send LS to RAN4 with regards to the power transition and possible combinations of the physical channels and reference signals. Inform RAN4 that RAN1 has identified the following combinations of the physical channels and the reference signals that can be transmitted by the UE on different OFDM symbols in the same slot with different transmit power - PUSCH+PUSCH - SRS+SRS - PUCCH+PUCCH - PUSCH+SRS - PUSCH+PUCCH - PUSCH+PRACH - SRS+PUSCH - SRS+PUCCH - SRS+PRACH - PUCCH+PUSCH - PUCCH+SRS - PUCCH+PRACH - PUSCH+PUCCH+PUCCH - PUSCH+SRS+PUCCH Inform RAN4 that RAN1 has identified the following combinations of the physical channels and the reference signals that can be transmitted by the UE on two consecutive slots with different transmit power and the PUSCH DMRS occupies the first OFDM symbol - PUSCH+PUSCH - PUCCH+PUSCH - SRS+PUSCH Inform RAN4 that for each combination different physical channels or reference signals may have different durations in the same slot - PUSCH: 2/4/5/6/7/8/9/10/11/12/13/14 symbols - SRS: 1/2/3/4/5/6 symbols - PUCCH: 1/2/4/5/6/7/8/9/10/11/12/13/14 symbols - PRACH: 2/4/6/12 symbols Note that transmission priority may be different depending on the channel, content that physical channel is carrying and reference signal. Note that the above list might not be complete. Ask RAN4 to take it into account in their work. The LS is endorsed in R1-1803349 Agreement: When PRACH is associated with CSI-RS resource(s), the pathloss measurement for PRACH is based on one of the associated CSI-RS resource. - FFS: Which of the associated CSI-RS is used if there are multiple resources associated Agreement: For the case of large UCI payload size (greater than 11), ΔPUCCH_TF,c(i) is equal to ΔPUCCH_TF,c(i) = 10log10(2K1∙BPRE(i) – 1) - BPRE(i) = OUCI(i)/NRE(i) - OUCI(i) is the number of UCI bits including CRC bits in i - NRE(i) = MPUCCH,c(i) x number of subcarriers per PRB x number of DFT-s-OFDM/CP-OFDM symbols excluding DMRS symbols/tones - FFS: K1 (there is no new RRC parameter introduced) Agreement: - For power control, the following priority rules for SRS transmission is adopted - A-SRS > SP-SRS/P-SRS - Additional prioritization rules for SRS carrier switching cases are not introduced. Agreement: Following working assumption is confirmed with the understanding that the CCs are in the same cell group: - In Case 2, (CCs/uplinks configured for UE have same or different numerologies and partially overlapping transmissions between different CCs/uplinks and same/different transmission duration and one or two PUCCH group(s)), when the UE is power limited due to simultaneous transmission on multiple serving CCs/uplinks, - PRACH of PCell > PUCCH/PUSCH with ACK/NACK and/or SR > PUCCH/PUSCH with other UCIs > PUSCH w/o UCI > SRS/PRACH of Scell - Within a same priority level, PCell is prioritized over Scell - In case that transmission power exceeds Pcmax, Scaling/dropping is applied to the lowest priority first until the aggregated power is within Pcmax. - Note: different priority of SRS used for carrier switching can be discussed further - Scaling or dropping of the whole or part(s) of a transmission is left to UE implementation. - Note: If the aggregated transmission power does not exceed Pc_max within any part of a transmission that overlaps with other transmission(s), the transmission is considered as non-power limited case. - Note: power control with look-ahead is not required at UE. FFS: Priority on the UL and SUL Agreement The proposed text change for 38.213 section 7.6.1 is agreed. “ … - If the UE indicates a capability for dynamic power sharing between EUTRA and NR - If the UE is not configured for operation with shortened TTI and processing time on the MCG [13, TS 36.213], and if the UE transmission(s) in subframe of the MCG overlap in time with transmission(s) in slot of the SCG, and if would exceed in any portion of slot of the SCG, the UE reduces transmission power on the SCG in any portion of slot so that in any portion of slot i2, where and are the linear values of the total UE transmission powers in subframe of the MCG and in slot of the SCG, respectively “ Agreement: Clarify the text in section 7.5 in TS38.213 such that the prioritization rules in the section are applied only for the case when UE is power limited for NR-NR CA/SUL (i.e., UE is not power limited due to simultaneous LTE-NR transmission) Agreement: - Regarding this issue on {j, q_d, l} configuration for virtual PHR, determination of the predetermined/default setting is done as follows: - UE uses a default {j, q_d, l} setting to compute the virtual PH of the serving cell/uplink for which there is no grant. - FFS: The details on default {j, q_d, l} setting

UE features

Agreement UE processing time capability#2 is not supported in Dec. 2017 version of the NR specification. Agreements: - R1-1803478 is agreed in R1-1803513 with the following update: - Remove “UE specific RRC configure UL/DL assignment” from 5-1 - Add “Dynamic UL/DL determination based on L1 scheduling DCI with cell specific RRC configured UL/DL assignment” to 5-1 - Replace 5-1a by: 5-1a; UE specific RRC configure UL/DL assignment; Dynamic UL/DL determination based on L1 scheduling DCI with cell-specific and UE specific RRC configured UL/DL assignment; Yes; Type 3; N.A.; N.A.; RAN1 needs to check component; RAN1. The draft LS is agreed with the following updates: - Add a 2nd paragraph “To RAN plenary: RAN1 did not have time to complete the initial access and forward compability categorizations requested by RAN LS. However, RAN1 plans to complete these aspects in the future unless instructed otherwise.” - Add in the first paragraph “UE processing time capability#2 is not supported in Dec. 2017 version of the NR specification”. - Add RANP to the “to” list Final LS is R1-1803480

NR URLLC in Release 15

Agreements: The two BLER targets for CQI reporting that are configurable for URLLC are to be down-selected from one of the following options: - Option A. (10-1, 10-4) - Option B. (10-1, 10-5) - Option C. (10-3, 10-5) - Option D. (10-2, 10-4) Companies are encouraged to consider the following when performing evaulations for down-selection of BLER targets for CQI reporting, e.g., - Resource efficiency: e.g., number of RE occupied, probability of blocking - Feasibility of UE producing accurate CQI estimation for CQI reporting. Each company can provide views from their perspective. Assume existing definition of CSI reference resource. - The distance in SNR (dB) between the two target is sufficient to generate distinct CQI in typical operation. - UE complexity of being able to generate CQI report for 3 BLER targets (e.g., Option (C) and (D) in certain cases) vs 2 BLER targets (Option (A) and (B)) - achieved latency Conclusion: - Regarding the number of CQI table to define for URLLC, finalize after the two BLER targets values for CQI reporting are agreed Agreements: - For new CQI table and MCS table constructed specifically for URLLC, 256QAM is not included. - Lowest spectral efficiency in any/all CQI table is not lower than 30/1024 * 2 (QPSK) - Highest spectral efficiency in any/all CQI table is not greater than a value, where the value is selected from one of the following: - 666/1024 * 6 - 772/1024 * 6 - 873/1024 * 6 - 948/1024 * 6 - Lowest spectral efficiency in any/all MCS table is not lower than 30/1024 * 2. - Highest spectral efficiency in any/all MCS table is not greater than a value, where the value is selected from the following: - 666/1024 * 6 - 772/1024 * 6 - 873/1024 * 6 - 948/1024 * 6 Agreements: - Only single transport block (i.e., a single CW) transmission is supported for URLLC in Rel-15. Conclusion: - Discuss further in the next meeting whether or not to support PDCCH repetition - Companies are encouraged to perform more evaulations/analyses Agreements: - To study the necessity of compact DCI and PDCCH repetition, the following link-level simulation assumptions are provided. Parameters; Value; Notes. DCI payload (excluding 24bits CRC); 40bits, 30bits, 24bits (optional). System bandwidth; 20MHz. Carrier Frequency; 4GHz, 700MHz; Reported by companies. Number of symbols for CORESET; 1, 2, 3; Reported by companies. CORESET BW (contiguous PRB allocation); 20MHz, 10MHz (optional for PDCCH repetition in frequency). Subcarrier spacing; 30KHz, other SCS are not precluded; Reported by companies. Aggregation level; Compact DCI study: 8, 16. (1,2,4 are optional) PDCCH repetition study (40bits): 4, 8, 16. Transmission type; Interleaved. REG bundling size; 6. Modulation; QPSK. Channel coding; Polar code (DCI). Transmission scheme; 1-port precoder cycling. Channel estimation; Realistic. Channel model; TDL-A (delay spread: 30ns) TDL-C (delay spread: 300ns) TDL-B (delay spread 100ns) (optional). UE speed; 3 km/h. Number of BS antennas; 2Tx. Number of UE antennas; 4Rx for 4G, 2Rx for 700MHz. Residual target BLER; 10^-5; Applied to one-shot tx, PDCCH repetition, HARQ, and others. Deployment; Urban macro as listed in 3GPP 38.802. SINR target; Compact DCI study: 5th percentile DL geometry PDCCH Repetition study: look at link curves directly. Agreements: - Study the options to support dynamic resource sharing between eMBB UL and URLLC UL from different UEs (comparing with existing techniques) - Option 1: eMBB UE cancels UL transmission when an indication is detected. Details to be discussed/clarified - UE processing timeline for cancelation - UE monitoring periodicity - Group common or UE specific signalling (including the possibility to use eMBB scheduling DCI) - reliability of indication - Any impact due to timing advance - Option 2: UL power control. URLLC UE transmits over the same resource with eMBB UE transmission. The transmission power for URLLC UL is boosted and/or transmission power for eMBB UL is reduced. Details need to be discussed/clarified - Performance impact to eMBB/URLLC transmission - How to signal the URLLC transmission power boosting - How to signal the eMBB transmission power reduction after UL grant - UE monitoring periodicity - Processing timeline - Feasibility of changing eMBB Tx power during the transmission - reliability of indication - Any impact due to timing advance - Other options including gNB receiver interference cancelation schemes are not precluded - Aspects to be included in the study - Processing timeline for grant-based procedure for URLLC in UL - Applicability of the options to TDD and/or FDD can be studied - Cases for GB-based & GF-based

Email discussions after the meeting are listed below.

  - [92-NR-01-spec#] Editors to prepare CRs by March 8, to be reviewed and endorsed by email by 3/13

  - [92-NR-02] CR to 36.214 on Inter-RAT Measurements of NR

  - [92-NR-03] Email discussion on UCI piggyback on PUSCH

  - [92-NR-04] LS on SRS switching

Outgoing LS is listed below.

  - R1-1803375	LS on Maximum TBS for PDSCH containing RMSI/OSI/Paging

  - R1-1803469	LS on RMSI CORESET configuration for supporting new SSB SCS and minimum CH BW combination

  - R1-1803372	Missing values for calculating the RA-RNTI

  - R1-1803475	LS on RAR

  - R1-1803379	Reply LS on Raster Configuration

  - R1-1803347	LS on MAC CEs for BM and CSI

  - R1-1803348	LS reply on beam failure recovery

  - R1-1803510	LS on time domain resource allocation

  - R1-1803537	LS on Simultaneous PDSCH Reception with Paging

  - R1-1803538	LS on UE HARQ Capability

  - R1-1803541	LS on Simultaneous PDSCH Reception with System Information

  - R1-1803540	Reply LS on Clarifications in MAC (BWP inactivity timer)

  - R1-1803351	Reply LS on PHR

  - R1-1803350	Reply LS on Clarifications in MAC (DELTA_PREAMBLE)

  - R1-1803349	LS on Power Transition and Uplink Channel and Reference Signal Combinations

  - R1-1803480	LS on NR UE feature list