3GPP TR 37.910 V0.1.0 (2018-03)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Study on Self Evaluation towards IMT-2020 Submission

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	6

4	Introduction	6

5	Self evaluation of eMBB technical performance	6

5.1	Peak spectral efficiency	7

5.2	Peak data rate	7

5.3	5th percentile user spectral efficiency	7

5.4	Average spectral efficiency	7

5.5	User experienced data rate	7

5.6	Area traffic capacity	7

5.7	Latency	7

5.7.1	User plane latency	7

5.7.2	Control plane latency	7

5.8	 Energy efficiency	7

5.9	Mobility	7

5.10	Mobility interruption time	7

6	Self evaluation of URLLC technical performance	7

6.1	Reliability	7

6.2	Latency	7

6.2.1	User plane latency	7

6.2.2	Control plane latency	7

6.3	Mobility interruption time	7

7	Self evaluation of mMTC technical performance	7

7.1	Connection density	7

8	Self evaluation of generic requirements	7

8.1	Bandwidth and scalability	7

8.2	Spectrum	7

8.3	Service	7

9	Conclusion	7

Annex A: Calibration for self evaluation Annex B: Simulation models and assumptions	8

Annex B: Simulation models and assumptions	9

B.1	TBD	9

Annex C: ITU-R Submission Templates for IMT-2020	10

C.1	Description template – characteristics	10

C.2	Description template – link budget	10

C.3	Compliance templates for services, for spectrum, for technical performance	10

Annex D: Change history	11

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

1	Scope

This clause shall start on a new page.

The present document …

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to 21.905 be the first in the list.

3	Definitions, symbols and abbreviations

Delete from the above heading those words which are not applicable.

Clause numbering depends on applicability and should be renumbered accordingly.

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

4	Introduction

5	Self evaluation of eMBB technical performance

[Editor’s note: The grouping of the technical performance requirements for eMBB usage scenario are based on Report ITU-R M.2410 “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”. However, it does not imply that each of the following technical performance requirements is only related to eMBB. It is just the minimum requirement that shall be evaluated under eMBB. This should also be noted for Section 6 and 7.]

5.1	Peak spectral efficiency

5.2	Peak data rate

5.3	5th percentile user spectral efficiency

5.4	Average spectral efficiency

5.5	User experienced data rate

5.6	Area traffic capacity

5.7	Latency

5.7.1	User plane latency

5.7.2	Control plane latency

As defined in Report ITU-R M.2410, control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state).

5.7.2.1	NR

5.7.2.2	LTE

For LTE Rel-15, control plane latency is evaluated from IDLE state to CONNECTED state. It is noted that, for LTE, when RRC connection is suspended, RRC connection resume is permitted by E-UTRAN. Figure 5.7.2.2-1 provides an example control plane flow for the above-mentioned IDLE to CONNECTED state transition for LTE Rel-15.

UE

eNB

1. Delay for RACH Scheduling Period

3. Processing delay in eNB

5. Processing delay in UE

7. Processing delay in eNB

9. Processing delay in UE

2. RACH Preamble

4. RA response

6. RRC Connection Resume Request

8. RRC Connection Resume

10. RRC Connection Resume Complete

Control plane procedure

Figure 5.7.2.2-1 C-plane activation procedure (example for Rel-15)

It is noted that for Step 9, the processing delay in UE might be different for the start of DL and UL continuous data transfer. Therefore the evaluation of control plane latency is conducted for DL and UL, respectively.

The evaluation results of LTE FDD on control plane latency are provided in Table 5.7.2.2-1.

Table 5.7.2.2-1 Control plane latency of LTE FDD

| Step | Description | CP Latency for UL data transfer [ms] |
| --- | --- | --- |
| 1 | Delay due to RACH scheduling period (1TTI) | 0 |
| 2 | Transmission of RACH Preamble | 1 |
| 3 | Preamble detection and processing in eNB | 2 |
| 4 | Transmission of RA response | 1 |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Resume Request) | 4 |
| 6 | Transmission of RRC Connection Resume Request | 1 |
| 7 | Processing delay in eNB (L2 and RRC) | 3 |
| 8 | Transmission of RRC Connection Resume | 1 |
| 9 | Processing delay in UE of RRC Connection Resume including grant reception | 7 |
| 10 | Transmission of RRC Connection Resume Complete and UP data | 0 |
|  | Total delay [ms] | 20 |
| Notes: - For step 1, the procedure for transition from a most “battery efficient” state has yet not begun, hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. - For step 5, the latency of 4 ms has been agreed by RAN1, see LS in R2-1806411. - For step 7, the processing delay in eNB (L2 and RRC) has been reduced to 3 ms. - For step 9 for UL data transfer, the processing delay in the UE (L2 and RRC) is considered, i.e., from reception of RRC Connection Resume to the reception of UL grant. The transmission of UL grant by eNB and processing delay in the UE (processing of UL grant and preparing for UL tx) are also considered. The RRCConnectionResume message only includes MAC and PHY configuration. No DRX, SPS, CA, or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRC Connection Resume Complete and the data is transmitted over common search space with DCI format 0. - For step 9 for DL data transfer, only the processing delay in the UE (L2 and RRC) is considered, i.e., from reception of RRC Connection Resume to the reception of DL grant. The RRCConnectionResume message only includes MAC and PHY configuration. No DRX, SPS, CA, or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRC Connection Resume Complete and the data is transmitted over common search space with DCI format 0. - For step 10, the beginning of this subframe is considered to be "the start of continuous data transfer", hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. |  |  |

For LTE TDD, the achieved latency will depend on when the procedure is initiated due to the TDD-pattern (i.e. in some cases the eNB and UE must wait for a subframe where DL and UL can be transmitted).

For DL data transfer, the evaluation results of LTE TDD are provided in Table 5.7.2.2-2.

Table 5.7.2.2-2 Control plane latency of LTE TDD for DL data transfer

| Step | Average CP Latency for DL data transfer [ms] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Config 0 | Config 1 | Config 2 | Config 3 | Config 4 | Config 5 | Config 6 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 3 | 3 | 2 | 2 | 2.7 | 2 | 2 | 2.5 |
| 4 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 5 | 4.8 | 5.4 | 5.8 | 4.8 | 5.7 | 5.9 | 4.5 |
| 6 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 7 | 3 | 3 | 3 | 3 | 3 | 3 | 3.3 |
| 8 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 9 | 4 | 3 | 3 | 3 | 3 | 3 | 3.8 |
| 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Total delay [ms] | 18.8 | 17.4 | 17.8 | 17.5 | 17.7 | 17.9 | 18.1 |
| Notes: - The description of each component is the same as in Table 5.7.2.2-1. TDD frame structure configuration 0~6 are defined in TS36.211. - For step 1, the procedure for transition from a most “battery efficient” state has yet not begun, hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. - For step 3, the eNB processing delay is 2ms as in FDD. Additional delay due to waiting for DL subframe is included. The delay value is the average delay taken over the starting subframes when the procedure is initiated under the corresponding TDD configuration. - For step 5, the UE processing delay is 4 ms as in FDD, see LS in R2-1806411. Additional delay due to waiting for UL subframe is included. The delay value is the average delay taken over the starting subframes when the procedure is initiated under the corresponding TDD configuration. - For step 7, the eNB processing delay (L2 and RRC) has been reduced to 3 ms as in FDD. Additional delay due to waiting for DL subframe is included. The delay value is the average delay taken over the starting subframes when the procedure is initiated under the corresponding TDD configuration. - For step 9 for DL data transfer, only the processing delay in the UE (L2 and RRC) is considered as in FDD. Additional delay due to waiting for DL subframe for receiving DL grant is included. The delay value is the average delay taken over the starting subframes when the procedure is initiated under the corresponding TDD configuration. - For step 10, the beginning of this subframe is considered to be "the start of continuous data transfer", hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. |  |  |  |  |  |  |  |

For UL data transfer, the evaluation results of LTE TDD are provided in Table 5.7.2.2-3 for specific starting subframes for TDD frame structure configurations.

Table 5.7.2.2-3 Control plane latency of LTE TDD for UL data transfer

| Step | Description | CP Latency for UL data transfer [ms] for the following cases |
| --- | --- | --- |
|  |  | Config 0, Starting Subframe = 2,7 Config 1, Starting Subframe = 2,7 Config 3, Starting Subframe = 1, 2 Config 4, Starting Subframe = 2 Config 6, Starting Subframe = 2,7 |
| 1 | Delay due to RACH scheduling period (1TTI) | 0 |
| 2 | Transmission of RACH Preamble | 1 |
| 3 | Preamble detection and processing in eNB | 2 |
| 4 | Transmission of RA response | 1 |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Resume Request) | 4 |
| 6 | Transmission of RRC Connection Resume Request | 1 |
| 7 | Processing delay in eNB (L2 and RRC) | 3 |
| 8 | Transmission of RRC Connection Resume | 1 |
| 9 | Processing delay in UE of RRC Connection Resume including grant reception | 7 |
| 10 | Transmission of RRC Connection Resume Complete and UP data | 0 |
|  | Total delay [ms] | 20 |
| Notes: - TDD frame structure configuration 0~6 are defined in TS36.211. The delay value is for the given starting subframes under the corresponding TDD configuration. - For step 1, the procedure for transition from a most “battery efficient” state has yet not begun, hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. - For step 3, the eNB processing delay is 2ms as in FDD. Additional delay due to waiting for DL subframe is 0 for the given starting subframes under the corresponding TDD configuration. - For step 5, the UE processing delay is 4 ms as in FDD, see LS in R2-1806411. Additional delay due to waiting for UL subframe is 0 for the given starting subframes under the corresponding TDD configuration. - For step 7, the eNB processing delay (L2 and RRC) has been reduced to 3 ms as in FDD. Additional delay due to waiting for DL subframe is 0 for the given starting subframes under the corresponding TDD configuration. - For step 9 for UL data transfer, the processing delay is considered as in FDD. Additional delay due to waiting for DL subframe for receiving UL grant is 0 for the given starting subframes under the corresponding TDD configuration. - For step 10, the beginning of this subframe is considered to be "the start of continuous data transfer", hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above. |  |  |

Based on the above analysis, 20ms control plane latency is fulfilled by LTE Rel-15 FDD, and for TDD for the above cases.

5.8		Energy efficiency

5.9	Mobility

5.10	Mobility interruption time

As defined in Report ITU-R M.2410, mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the mobile station and the radio access network, as applicable to the candidate RIT/SRIT.

5.10.1	NR

For NR Rel-15, the mobility interruption time is evaluated under the following cases.

- Case 1: In intra-cell beam mobility,

- Case 2: For CA operation, during addition and release of an Secondary cell (SCell) in response to mobility (no change to Primary cell, PCell).

5.10.1.1	Evaluation of Case 1

For intra-cell beam mobility case, the UE mobility may introduce the change of transmit-receive beam pair.

For DL data transmission, the UE can be configured with up to M TCI-States by higher layer signalling to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability. Each configured TCI (Transmission Configuration Indicator) state indicates one transmit – receive beam pair for PDSCH transmission. For each PDSCH transmission, one TCI state can be configured by DCI. During UE mobility, gNB can configure different TCI states for the UE at different slots. It ensures appropriate transmit – receive beam pair allocation to the UE for continuous DL transmission. Therefore DL data packet transmission is kept during beam pair switching at different slots.

For UL data transmission, PUSCH is sent using the beam configured by SRI (SRS resource indicator) by gNB. Accordingly, an appropriate gNB-side beam is selected for UL data reception. gNB may select different beams at different slots depending on the UE mobility. Therefore UL data packet transmission is kept during beam pair switching at different slots.

Based on the above analysis, the UE can always exchange user plane packets with gNB during transitions. Therefore 0ms mobility interruption time is achieved by NR for Case 1.

5.10.1.2	Evaluation of Case 2

For CA operation, the UE mobility may introduce the addition or release of an SCell in addition to the connection of PCell. The SCell addition procedure and SCell release procedure are shown below.

It is observed that during these procedures, the UE can always exchange user plane packets with gNB during transitions. Therefore 0ms mobility interruption time is achieved by NR for Case 2.

- SCell Addition / Activation

The SCell activation procedure is shown in Figure 5.10.1.2-1. In this figure, the RRC Connection Reconfiguration is for SCell activation. Data transmission between UE and PCell is kept.

Figure 5.10.1.2-1: SCell activation procedure

- SCell Release

The SCell release procedure is shown in Figure 5.10.1.2-2. During this procedure, data transmission between UE and PCell is kept.

Figure 5.10.1.2-2: SCell release procedure

5.10.2	LTE

For LTE Rel-15, the mobility interruption time is evaluated under the following cases.

- Case 1: In intra-frequency handover cases, under the assumption of a dual RX UE, with the combination of Make-Before-Break (MBB) and RACH-less handover (in the scenarios where RACH-less handover is applicable, i.e. no/negligible UE TA difference between the source and the target cell),

- Case 2: For dual connectivity (DC) operation, during addition and release of the secondary cell group (SCG) in response to mobility (no change to the PCell).

5.10.2.1	Evaluation of Case 1

The generic handover procedure of LTE-Advanced is shown in Figure 5.10.2.1-1.

In Make-Before-Break handover, the connection to the source eNB is not released until handover is completed at the target eNB. For intra-frequency handover case, a dual RX UE can monitor both source and target links simultaneously. For example, the UE can receive the data from source eNB while synchronizing with target eNB. When the UE performs successful RACH at the target eNB, it will release the connection to the source eNB and start the data transmission with the target eNB. In this case, the DL data transmission is kept between UE and source eNB until handover is completed.

On the other hand, if the source eNB, target eNB and UE are synchronized, the UE may be able to obtain the target eNB timing advance (TA) without explicit TA command. RACH-less handover can be applied. For example, the UE first obtains the DL propagation delay difference between the source eNB and the target eNB. By assuming the UL propagation delay is the same as the DL propagation delay, the UE can then derive TA of the target eNB from the source eNB. In some scenarios such as no/negligible UE TA difference between the source and target eNB, the UE can use its original TA for the source eNB to transmit the data to the target eNB. In this case, the delay of RACH procedure can be avoided and UL data transmission is kept continuous.

In RACH-less handover, 
No RACH procedure needed

In Make-Before-Break handover, 
connection to the source eNB is not released until handover complete.

Figure 5.10.2.1-1: U-Plane interruption in LTE-Advanced

Therefore, by combining Make-Before-Break and RACH-less handover for a dual RX UE in the scenario where no/negligible UE TA difference between the source and the target cell, the 0ms mobility interruption time is achieved by LTE for Case 1.

5.10.2.2	Evaluation of Case 2

For DC operation, the UE mobility may introduce addition or release of SeNB (Secondary eNB) connection in addition to Master eNB (MeNB) connection. The SeNB addition procedure and SeNB release procedure are shown below (see also Section 10.1.2.8 in 3GPP TS36.300).

It is observed that during these procedures, the UE can always exchange user plane packets with MeNB during transitions. Therefore 0ms mobility interruption time is achieved by LTE for Case 2.

- SeNB Addition

The SeNB Addition procedure is initiated by the MeNB and is used to establish a UE context at the SeNB in order to provide radio resources from the SeNB to the UE. This procedure is used to add at least the first cell (PSCell) of the SCG. Figure 5.10.2.2-1 shows the SeNB Addition procedure. Detailed description of each step is found in Section 10.1.2.8.1 in 3GPP TS36.300.

Figure 5.10.2.2-1: SeNB Addition procedure

- SeNB Release

The SeNB Release procedure may be initiated either by the MeNB or by the SeNB and is used to initiate the release of the UE context at the SeNB. The recipient node of this request cannot reject.

It does not necessarily need to involve signalling towards the UE, e.g., RRC connection re-establishment due to Radio Link Failure in MeNB.

MeNB initiated SeNB Release

Figure 5.10.2.2-2: SeNB Release procedure – MeNB initiated

Figure 5.10.2.2-2 shows an example signalling flow for the MeNB initiated SeNB Release procedure. Detailed description of each step is found in Section 10.1.2.8.3 in 3GPP TS36.300.

SeNB initiated SeNB Release

Figure 5.10.2.2-3: SeNB Release procedure – SeNB initiated

Figure 5.10.2.2-3 shows an example signalling flow for the SeNB initiated SeNB Release procedure. Detailed description of each step is found in Section 10.1.2.8.3 in 3GPP TS36.300.

6	Self evaluation of URLLC technical performance

6.1	Reliability

6.2	Latency

6.2.1	User plane latency

6.2.2	Control plane latency

6.3	Mobility interruption time

7	Self evaluation of mMTC technical performance

7.1	Connection density

8	Self evaluation of generic requirements

8.1	Bandwidth and scalability

8.2	Spectrum

8.3	Service

9	Conclusion

Annex A:
Calibration for self evaluation

To facilitate the self evaluation towards IMT-2020 submission, the system level simulators have been calibrated to ensure the results from different 3GPP entities are comparable.

The following metrics are selected for calibration of self evaluation:

- DL Geometry (wideband SINR)

- Coupling gain

The cell association mechanisms as defined in calibration assumptions are used to derive the above metrics, including multi-path based UE attachment model, and analog beamforming (BF) model, etc. Detailed calibration parameters and assumptions are found in Section 4 of RP-180524. It should be noted that these parameters are used for calibration purpose only.

The calibration was conducted to the corresponding evaluation configurations of Indoor Hotspot – eMBB, Dense Urban – eMBB, Rural – eMBB, Urban Macro – mMTC, and Urban Macro - URLLC test environment as defined in Report ITU-R M.2412. The two channel model variants (channel model A and channel model B) are applied in the calibration.

Twenty-one 3GPP entities provided the calibration results, including CATR, CATT, CEWiT, China Telecom, CMCC, Ericsson, Huawei, Intel, ITRI, LG Electronics, MediaTek, Motorola Mobility/Lenovo, NEC, Nokia, NTT DOCOMO, OPPO, Qualcomm, Samsung, Sharp, vivo and ZTE.

The calibration results for the five test environments are shown through Figure A.1 to Figure A.5, respectively. The results are based on the average of the independent results from the simulators of the contributing entities. The results of DL geometry (wideband SINR) from the independent samples are typically within 1~2 dB of the average SINR.

Figure A.1 Coupling gain and DL geometry of Indoor Hotspot - eMBB test environment

Figure A.2 Coupling gain and DL geometry of Dense Urban - eMBB test environment

Figure A.3 Coupling gain and DL geometry of Rural - eMBB test environment

Figure A.4 Coupling gain and DL geometry of Urban Macro - mMTC test environment

Figure A.5 Coupling gain and DL geometry of Urban Macro - URLLC test environment

A summary of the collected samples for calibration of each test environment and evaluation configuration is shown in Table A.1.

Table A.1. Sample statistics for ITU-R test environments.

| Test environment | Evaluation configuration | Channel model / Topology |  | Number of samples | DL wideband SINR difference compared to average SINR (at 50%-tile CDF point) |
| --- | --- | --- | --- | --- | --- |
| Indoor Hotspot - eMBB | Config. A (4 GHz) | Channel model A | 12TRxP | 16 | <0.8 dB |
|  |  |  | 36TRxP | 15 | <0.5 dB |
|  |  | Channel model B | 12TRxP | 18 | <0.9 dB |
|  |  |  | 36TRxP | 16 | <0.4 dB |
|  | Config. B (30 GHz) | Channel model A/B | 12TRxP | 17 | <2.2 dB |
|  |  |  | 36TRxP | 14 | <2.2 dB |
|  | Config. C (70 GHz) | Channel model A/B | 12TRxP | 16 | <1.6 dB |
|  |  |  | 36TRxP | 12 | <1.9 dB |
| Dense Urban - eMBB | Config. A (4 GHz) | Channel model A |  | 16 | <1.3 dB |
|  |  | Channel model B |  | 18 | <1.3 dB |
|  | Config. B (30 GHz) | Channel model A/B |  | 18 | <2.4 dB |
| Rural - eMBB | Config. A (1732 m, 700 MHz) | Channel model A |  | 18 | <0.8 dB |
|  |  | Channel model B |  | 20 | <0.9 dB |
|  | Config. B (1732 m, 4 GHz) | Channel model A |  | 18 | <0.9 dB |
|  |  | Channel model B |  | 20 | <1.2 dB |
|  | Config. C (LMLC, 6000 m, 700 MHz) | Channel model A |  | 15 | <0.9 dB |
|  |  | Channel model B |  | 16 | <1.0 dB |
| Urban Macro - mMTC | Config. A (500 m, 700 MHz) | Channel model A |  | 15 | <0.9 dB |
|  |  | Channel model B |  | 16 | <0.6 dB |
|  | Config. B (1732 m, 700 MHz) | Channel model A |  | 15 | <1.2 dB |
|  |  | Channel model B |  | 16 | <0.6 dB |
| Urban Macro - URLLC | Config. A (4 GHz) | Channel model A |  | 15 | <0.9 dB |
|  |  | Channel model B |  | 17 | <1.0 dB |
|  | Config. B (700 MHz) | Channel model A |  | 15 | <0.9 dB |
|  |  | Channel model B |  | 16 | <1.3 dB |

Annex B:
Simulation models and assumptions

B.1	Calculation of pre-processing SINR

Pre-processing SINR is used for mobility evaluation for SU-MIMO.

The pre-processing SINR is defined on an Rx antenna port with respect to a Tx antenna port.

The following pre-processing SINR formula can be used for a system comprising of S Tx antenna ports (TXRUs) and U Rx antenna ports (TXRUs),

(B.1-1)

where  is the average received signal power as given by (B.1-2), and  is the average interference plus noise power as given by (B.1-3).

The average received signal power  is over the S Tx antenna ports and U Rx antenna ports (similar to (8.1-1) in TR36.873) that are expressed as

(B.1-2)

where N is the number of paths and M is the number of sub-paths within one path; for NLOS path for n=1, …, N,

with the notations Pn, n,m,ZOA, n,m,AOA, n,m,ZOD, n,m,AOD,  , , ,  and  being according to equation (7.3-22) in TR36.873, and KR is the Ricean K-factor;

and for LOS path

with the notations LOS,ZOA, LOS,AOA, LOS,ZOD, LOS,AOD, and  being according to equation (7.3-27) in TR36.873;

and Ftx,p,θ and Ftx,p,ϕ are the field patterns of Tx antenna port p in the direction of the spherical basis vectors,  and  respectively, Frx,u,θ and Frx,u,ϕ are the field patterns of Rx antenna port u in the direction of the spherical basis vectors,  and  respectively; they are given by

where NT is the number of antenna elements that virtualizes the Tx antenna port p, NR is the number of antenna elements that virtualizes the Rx antenna port u; wk (k=1, …, NT) represents a complex weight vector used for virtualization of Tx antenna port p, gl (l=1, …, NR) represents a complex weight vector used for virtualization of Rx antenna port u, Ftx,k,θ and Ftx,k,ϕ are the kth transmit antenna element’s field patterns in the direction of the spherical basis vectors,  and  respectively, Frx,l,θ and Frx,l,ϕ are the lth receive antenna element’s field patterns in the direction of the spherical basis vectors,  and  respectively;

and TXpower is the total transmit power (over the S Tx antenna ports) per RE.

The average interference plus noise power  is over U Rx antenna ports that are expressed as

(B.1-3)

where RI+N = Q+N is the U×U covariance matrix of interference and noise on the same RE as TXpower is computed, Q and N are the covariance matrix of the interference and the noise, respectively, and are modeled according to section A.2.1.8 in TR36.814, and tr(RI+N) represents the trace of RI+N.

B.2	Link level assumption for mobility evaluation

B.2.1	Scaling factor for link level channel model

According to Report ITU-R M.2412, for the link level simulation part for mobility, reliability, and connection density evaluation, the delay spreads and angular spreads (for AoA, AoD, ZoA, and ZoD) in link-level channel model are scaled to the median values for the environment and channel type (LOS/NLOS) evaluated, and system-level channel model variant (model A or model B) selected.

According to system-level channel model defined in Report ITU-R M.2412, the scaling parameters of delay spreads and angular spreads are given in Table B.2.1-1.

Table B.2.1-1 Scaling parameters for delay spreads and angular spreads for related test environments

| Parameters | Indoor Hotspot-eMBB |  |  |  |  |  | Dense Urban-eMBB |  |  |  | Rural-eMBB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Config A (4 GHz) |  | Config B (30 GHz) |  | Config C (70 GHz) |  | Config A (4 GHz) |  | Config B (30 GHz) |  | Config A/B (700 MHz/4 GHz) |  |
| Link-level Channel model | LOS: CDL/TDL-iv | NLOS: CDL/TDL-i | LOS: CDL/TDL-iv | NLOS: CDL/TDL-i | LOS: CDL/TDL-iv | NLOS: CDL/TDL-i | LOS: CDL/TDL-v | NLOS: CDL/TDL-iii | LOS: CDL/TDL-v | NLOS: CDL/TDL-iii | LOS: CDL/TDL-v | NLOS: CDL/TDL-iii |
| Delay spread scaling parameter (ns) | 20 | 39 | 19.5 | 26 | 19.5 | 20 | 93 | 363 | 79 | 263 | 32 | 37 |
| AoA angular spreads scaling parameter (degree) | 42 | 59 | 32 | 50 | 27 | 46 | 65 | 74 | 65 | 48 | 33 | 33 |
| AoD angular spreads scaling parameter (degree) | 40 | 42 | 40 | 42 | 40 | 42 | 14 | 26 | 17 | 22 | 8 | 9 |
| ZoA angular spreads scaling parameter (degree) | 17 | 18 | 11 | 14 | 9 | 13 | 9 | 18 | 9 | 11 | 3 | 4 |
| ZoD angular spreads scaling parameter (degree) | 10 | 12 | 1 | 12 | 0.4 | 12 | TBD (NOTE 1) | TBD (NOTE 1) | TBD (NOTE 1) | TBD (NOTE 1) | TBD (NOTE 1) | TBD (NOTE 1) |
| K-factor (dB) | 13.3 | - | 13.3 | - | 13.3 | - | 22 | - | 22 | - | 22 | - |

NOTE 1: This value is derived by the 50%-tile point of CDF of the mean value of ZoD spread (as defined in system level channel model) of all UEs based on UE dropping in system level simulation. The mean value of ZoD spread of a UE is calculated according to the formula given in Table B.2.1-2 and Table B.2.1-3 for Dense Urban – eMBB and Rural – eMBB test environment, respectively, where d2D and hUT of a UE is determined by the UE dropping.

Table B.2.1-2 Mean value calculation for UMa ZoD spread

|  | Frequency | Parameters | UMa_x |  |
| --- | --- | --- | --- | --- |
|  |  |  | LOS | NLOS |
| UMa_A | 0.5 GHz≤ fc ≤6 GHz | lgZSD | max[-0.5, -2.1(d2D/1000) -0.01 (hUT - 1.5)+0.75] | max[-0.5, -2.1(d2D/1000) -0.01(hUT - 1.5)+0.9] |
|  | 6 GHz< fc ≤ 100 GHz | lgZSD | max[-0.5, -2.1(d2D/1000) -0.01 (hUT - 1.5)+0.75] | max[-0.5, -2.1(d2D/1000)-0.01(hUT - 1.5)+0.9] |
| UMa_B | 0.5 GHz≤ fc ≤ 100 GHz | lgZSD | max[-0.5, -2.1(d2D/1000)- 0.01 (hUT -1.5)+0.75] | max[-0.5, -2.1(d2D/1000)-0.01(hUT - 1.5)+0.9] |

Table B.2.1-3 Mean value calculation for RMa ZoD spread

|  | Frequency | Parameters | RMa_x |  |
| --- | --- | --- | --- | --- |
|  |  |  | LOS | NLOS |
| RMa_A RMa_B | RMa_A：0.5 GHz≤ fc ≤ 6 GHz, RMa_B：0.5 GHz≤ fc ≤ 7 GHz | lgZSD | max(-1, -0.17*( d2D /1000)-0.01*( hUT -1.5)+0.22) | max(-1, -0.19*( d2D /1000) - 0.01*( hUT -1.5)+0.28) |

B.2.2	TXRU pattern and inter-port spacing in link level simulation

The companies report the antenna port pattern used in link level simulation (LLS). If the CDL channel model is used in LLS, the following three options can be used for reference.

- Option 1: 0dBi omni-directional gain is assumed for Tx and Rx antenna ports

- Option 2: 0dBi gain (linear gain = 1) for the paths within the 3dB beamwidth; linear gain = 0 (-∞ dB gain) for the paths outside the 3dB beamwidth.

- Option 3: Use the pattern defined in Section 8.5 in Report ITU-R M.2412 for BS and UE side with maximum directional gain of 0dBi for related test environments.

The inter-port spacing calculation is reported by the companies. The calculation method given by equation (B.2.2-1) and (B.2.2-2) as shown below can be used for reference.

The Tx horizontal and vertical inter-port spacing (for the same polarization) can be calculated by

(B.2.2-1)

(B.2.2-2)

where dtx_element,H and dtx_element,V are the Tx inter-element spacing on horizontal and vertical domain, respectively, and  and  are the number of antenna elements on the same polarization that virtualizes the Tx antenna port on horizontal and vertical domain, respectively. An illustration is given in Figure B.2.2-1.

Figure B.2.2-1 Illustration of inter-port spacing

Annex C:
ITU-R Submission Templates for IMT-2020

C.1	Description template – characteristics

TBD

C.2	Description template – link budget

TBD

C.3	Compliance templates for services, for spectrum, for technical performance

TBD

Annex D:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-12 | RAN#78 | RP-172539 |  |  |  | Skeleton | 0.0.1 |
| 2018-03 | RAN#79 | RP-180523 |  |  |  | Calibration results are included in Annex A. | 0.1.0 |