3GPP TSG RAN Meeting #80                                                                    RP-181033

La Jolla, USA, 11th June – 14th June 2018

Source:	Thales, Nomor Research, Fraunhofer IIS

Title:	NR-NTN: RAN architecture

TDOC Type:	pCR

Agenda Item:	9.4.1  (SID = sFS_NR_nonterr_nw)

Document for:	Approval

Release:	Rel-15

Specification:	TR 38.811 (SID = sFS_NR_nonterr_nw)

# Abstract

The objective of this document is to complement the chap 7.3 with the identification of potential areas of NR impact associated to the RAN architecture supporting non-terrestrial networks.

# Proposed text for approval

It is proposed the following modifications to TR 38.811 “Study on NR to support Non-Terrestrial Networks” v0.4.0.

* * * Start of changes * * * * (added text in red)

### 7.3.1	Methodology

….

Table 7.3-1: Areas of impacts on NR to support Non-Terrestrial networks

| Non-Terrestrial network specifics | Effects | Impacted NR features |
| --- | --- | --- |
| Motion of the space/aerial vehicles (especially for Non GEO based access network) | Moving cell pattern | Hand-over/paging |
|  | Delay variation | TA adjustment |
|  | Doppler | Init synchro downlink |
|  |  | DMRS time density |
| Altitude | Long latency | HARQ |
|  |  | MAC/RLC Procedures |
|  |  | Physical layer Procedures (ACM, power control) |
| Cell size | Differential delay | TA in Random access response message |
|  |  | RACH |
| Propagation channel | Impairments | DMRS frequency density |
|  |  | Cyclic prefix |
| Duplex mode | Regulatory constraints | Access scheme (TDD/FDD) |
| Satellite or aerial Payload performance | Phase noise impairment | PT-RS |
|  | Back-off | PAPR |
| Network architecture | RAN Mapping | Protocols |

* * * End of Changes * * * *

* * * Start of changes * * * * (added text)

### 7.3.8	Network architecture

#### 7.3.7.1  Protocols

##### 7.3.7.1.1 Problem statement

Non-Terrestrial networks differ from typical cellular networks in terms of network architecture, deployment scenarios as well as coverage which may span across several countries.

Potential impacts on the NR specifications could be minimized through suitable mapping options of the Logical NG RAN architecture onto the NTN network architecture.

The respective NG-RAN logical architecture and the NTN network architecture are depicted below. The area of impacts associated to the NG-RAN mapping options need to be identified.

Figure 1: 3GPP NG-RAN (or NR radio access) architecture defined in TS 38.401

Figure 2: Typical Non-Terrestrial network physical architecture

Furthermore, when dealing with mobility of UEs, one should distinguish:

- The mobility induced by the motion of the NTN platform, in particular when considering Non-Geostationnary Satellites;The mobility of the terrestrial equipment:

  - when moving from one  beam to another beam  generated by a satellite or a HAPS

- The mobility of UE between beams generated by different satellites or HAPS

- The mobility of UE between NTN (Satellite or HAPS) and cellular access

Location update, paging and hand-over RAN related protocols may need to be adapted to accommodate

- The extended delay of NTN for intra NTN access mobility

- The differential delay when mobility between satellite access and cellular access

- The mobility of the cell pattern generated by non geo-stationary satellites

##### 7.3.7.1.2.	Assessment of conditions for NR operation in Non-Terrestrial Networks

RAN architecture mapping

There are several options to instantiate the RAN architecture in Non-Terrestrial networks. It depends on whether the satellite or HAPS implement a bent pipe payload or a processed payload. This leads to the following mapping options that prevent the need to create new interfaces or reference points:

Table 2: RAN architecture mapping options in Non-Terrestrial networks

| Satellite/HAPS payload | NG RAN architecture |
| --- | --- |
| Bent pipe | Mapping option 1 |
| Processed – low | Mapping option 2 |
| Processed - high | Mapping option 3 |

A bent pipe satellite/HAPS payload implements Radio frequency conversion, analogue filtering and amplification. A processed payloads implements some RAN functions. The type of gNB functions that can be embarked will be constraints by the available power and mass on board satellite/HAPS.

Here under are depicted different mapping options of the logical NG RAN architecture onto the physical architecture of Non-Terrestrial networks which prevents to modify the NG RAN architecture.

Figure 3: Mapping option 1 - NG RAN architecture in Non Terrestrial network with bent pipe payload

Figure 4: Mapping option 2 - NG RAN architecture in Non Terrestrial network with gNB-DU processed payload

Note: SRI refers to Satellite Radio Interface

Figure 5: Mapping option 3 - NG RAN architecture in Non Terrestrial network with gNB processed payload

The table below highlights the difference in terms of interfaces transported by the satellite links and NG RAN functions implemented in the NTN physical entities.

Table 3: Mapping options of the logical architecture of NR RAN onto the physical architecture of Non-Terrestrial networks

| Mapping options | Satellite/HAPS Service link | Satellite/HAPS feeder link | Terminal side | Network side |
| --- | --- | --- | --- | --- |
| 1 –Bentpipe payload | NR-Uu radio interface | NR-Uu radio interface | UE | NTN Remote Radio Unit + gNB |
| 2 –Processed payload (gNB-DU) | NR-Uu radio interface | F1 over Satellite radio interface (SRI) | UE | NTN Remote Radio Unit + SRI modems + gNB-CU |
| 3 –Processed payload (gNB) | NR-Uu radio interface | N1, N2, N3 over Satellite radio interface (SRI) | UE | NTN Remote Radio Unit + SRI modems |

Given that Non-terrestrial networks feature longer propagation delays, the timers associated to the protocols transported over the feeder and service links may require to be extended. This applies for example to F1 as well as N1, N2 and N3 reference points.

In mapping option 1, the Satellite or HAPS implements an RF repeater (with frequency conversion), whereas in mapping option 2, they implement some kind of “intermediate” node which could be based on the outcomes of  3GPP TR 38.874: "Study on Integrated Access and Backhaul”.

It is recommended to consider the outcome of this TR38.874 and see how it can be applicable to Non-Terrestrial networks (especially for NTN deployment scenarios 2 and 3) and what areas of impact it may create.Other mapping options may be considered depending on different CU-DU lower layer split for NR that may be defined (See TR 38.816).

Location update, paging and hand-over RAN related protocols The table below identifies the possible areas of impact on NR associated to Location update, paging and hand-over RAN related protocols in different types of non-terrestrial networks.

Table 4: Potential areas of impact on NR specifications associated to Location update, paging and hand-over RAN related protocols for different types of non-terrestrial networks

| NTN Deployment scenarios | GEO | Non GEO | HAPS |
| --- | --- | --- | --- |
| Cell pattern | Earth fixed : same as in cellular | Motion over earth or earth fixed | Motion over earth (e.g. rotating) & Possibly UE altitude dependent |
| Tracking area | Tracking area congruent with cell pattern : same as in cellular | For further study | Same as for GEO and cellular networks |
| Potential areas of impact on NR specifications | Mobility management procedures (Paging, hand-over, location update): Potential extension of some timers | Location update, paging and hand-over RAN related protocols to be adapted depending on tracking area design Handling of network identities to be adapted | Case 1: tracking area corresponds to HAPS coverage: no impact Case 2: Tracking area smaller than HAPS total coverage, same impact as for Non GEO are expected |

##### 7.3.7.1.3.	NR impact considerations

Thanks to suitable mapping options of the logical architecture of NG RAN onto the physical architecture of non terrestrial network, modifications of the NG RAN architecture are not needed in terms of reference points, interfaces or functions.

However the N1/N2/3 and GTP based F1 interface protocols may need to be adapted to accommodate the non terrestrial networks feeder link characteristics (long delay, BER).Depending on the non-terrestrial network deployment scenarios, other impacts on NR specification may have to be considered among which, location update, paging and hand-over RAN related protocols and handling of network identifies.

Last, it may be considered how the architecture being defined in 3GPP TR38.874 “Study on Integrated Access and Backhaul” can be used in the context of various Non-Terrestrial networks deployment scenarios.

* * * End of Changes * * * *