3GPP TSG-RAN2 Meeting #102	R2-1809041

# Busan, Korea, 21st May 2018 - 25th May 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1281 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PHR trigger for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current MAC spec, it is specified that MAC entity triggers PHR upon PSCell addition such that NW can obtain the PH info of PSCell as early as possible in case of initial PSCell addition and PSCell change. However, in EN-DC, PSCell change is modelled as reconfiguration with sync (not PSCell removal and addition) and it is not entirely clear that UE reports PH info for PSCell in case of PSCell change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clearified that “addition of PSCell” includes both SCG addition case and PSCell change case. Impact analysis: Impacted functionality: PHR Inter-operability: If the proposed change is implemented by the UE and not by the eNB, there is no interoperability issue. If the proposed change is implemented by the eNB and not by the UE, there is no interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NW cannot obtain PH information for PSCell in case of PSCell change. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

5.4.6	Power Headroom Reporting

The Power Headroom reporting procedure is used to provide the serving eNB with information about the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission or SRS transmission per activated Serving Cell and also with information about the difference between the nominal UE maximum power and the estimated power for UL-SCH and PUCCH transmission on SpCell and PUCCH SCell.

The reporting period, delay and mapping of Power Headroom are defined in subclause 9.1.8 of [9]. RRC controls Power Headroom reporting by configuring the two timers periodicPHR-Timer and prohibitPHR-Timer, and by signalling dl-PathlossChange which sets the change in measured downlink pathloss and the required power backoff due to power management (as allowed by P-MPRc [10]) to trigger a PHR [8].

A Power Headroom Report (PHR) shall be triggered if any of the following events occur:

-	prohibitPHR-Timer expires or has expired and the path loss has changed more than dl-PathlossChange dB for at least one activated Serving Cell of any MAC entity which is used as a pathloss reference since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

-	periodicPHR-Timer expires;

-	upon configuration or reconfiguration of the power headroom reporting functionality by upper layers [8], which is not used to disable the function;

-	activation of an SCell of any MAC entity with configured uplink;

-	addition of the PSCell (i.e. PSCell is newly added or PSCell is changed);

-	prohibitPHR-Timer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true in this TTI for any of the activated Serving Cells of any MAC entity with configured uplink:

-	there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the required power backoff due to power management (as allowed by P-MPRc [10]) for this cell has changed more than dl-PathlossChange dB since the last transmission of a PHR when the MAC entity had UL resources allocated for transmission or PUCCH transmission on this cell.

NOTE:	The MAC entity should avoid triggering a PHR when the required power backoff due to power management decreases only temporarily (e.g. for up to a few tens of milliseconds) and it should avoid reflecting such temporary decrease in the values of PCMAX,c/PH when a PHR is triggered by other triggering conditions.

If the MAC entity has UL resources allocated for new transmission for this TTI the MAC entity shall:

-	if it is the first UL resource allocated for a new transmission since the last MAC reset, start periodicPHR-Timer;

-	if the Power Headroom reporting procedure determines that at least one PHR has been triggered and not cancelled, and;

-	if the allocated UL resources can accommodate the MAC control element for PHR which the MAC entity is configured to transmit, plus its subheader, as a result of logical channel prioritization:

-	if extendedPHR is configured:

-	for each activated Serving Cell with configured uplink:

-	obtain the value of the Type 1 or Type 3 power headroom;

-	if the MAC entity has UL resources allocated for transmission on this Serving Cell for this TTI:

-	obtain the value for the corresponding PCMAX,c field from the physical layer;

-	if simultaneousPUCCH-PUSCH is configured or a serving cell operating according to Frame Structure Type 3 with uplink is configured and activated:

-	obtain the value of the Type 2 power headroom for the PCell;

-	obtain the value for the corresponding PCMAX,c field from the physical layer (see subclause 5.1.1.2 of [2]);

-	instruct the Multiplexing and Assembly procedure to generate and transmit an Extended PHR MAC control element for extendedPHR as defined in subclause 6.1.3.6a based on the values reported by the physical layer;

-	else if extendedPHR2 is configured:

-	for each activated Serving Cell with configured uplink:

-	obtain the value of the Type 1 or Type 3 power headroom;

-	if the MAC entity has UL resources allocated for transmission on this Serving Cell for this TTI:

-	obtain the value for the corresponding PCMAX,c field from the physical layer;

-	if a PUCCH SCell is configured and activated:

-	obtain the value of the Type 2 power headroom for the PCell and PUCCH SCell;

-	obtain the values for the corresponding PCMAX,c fields from the physical layer (see subclause 5.1.1.2 of [2]);

-	else:

-	if simultaneousPUCCH-PUSCH is configured for the PCell or a serving cell operating according to Frame Structure Type 3 with uplink is configured and activated:

-	obtain the value of the Type 2 power headroom for the PCell;

-	obtain the value for the corresponding PCMAX,c field from the physical layer (see subclause 5.1.1.2 of [2]);

-	instruct the Multiplexing and Assembly procedure to generate and transmit an Extended PHR MAC control element for extendedPHR2 according to configured ServCellIndex and the PUCCH(s) for the MAC entity as defined in subclause 6.1.3.6a based on the values reported by the physical layer;

-	else if dualConnectivityPHR is configured:

-	for each activated Serving Cell with configured uplink associated with any MAC entity:

-	obtain the value of the Type 1 or Type 3 power headroom;

-	if this MAC entity has UL resources allocated for transmission on this Serving Cell for this TTI or if the other MAC entity has UL resources allocated for transmission on this Serving Cell for this TTI and phr-ModeOtherCG is set to real by upper layers:

-	obtain the value for the corresponding PCMAX,c field from the physical layer;

-	if simultaneousPUCCH-PUSCH is configured or a serving cell operating according to Frame Structure Type 3 with uplink is configured and activated:

-	obtain the value of the Type 2 power headroom for the SpCell;

-	obtain the value for the corresponding PCMAX,c field for the SpCell from the physical layer (see subclause 5.1.1.2 of [2]);

-	obtain the value of the Type 2 power headroom for the SpCell of the other MAC entity;

-	if phr-ModeOtherCG is set to real by upper layers:

-	obtain the value for the corresponding PCMAX,c field for the SpCell of the other MAC entity from the physical layer (see subclause 5.1.1.2 of [2] or see [18]);

-	instruct the Multiplexing and Assembly procedure to generate and transmit a Dual Connectivity PHR MAC control element as defined in subclause 6.1.3.6b based on the values reported by the physical layer;

-	else:

-	obtain the value of the Type 1 power headroom from the physical layer;

-	instruct the Multiplexing and Assembly procedure to generate and transmit a PHR MAC control element as defined in subclause 6.1.3.6 based on the value reported by the physical layer;

-	start or restart periodicPHR-Timer;

-	start or restart prohibitPHR-Timer;

-	cancel all triggered PHR(s).

End of change