3GPP TSG RAN Meeting #81 RP-181642 Gold Coast, Australia, Sept 10 - 13, 2018 CR-Form-v11.2. CHANGE REQUEST. 38.213; CR; 0004; rev; -; Current version:; 15.2.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Correction on preemption indication. Source to WG:. Source to TSG:; Huawei, HiSilicon, Sony. Work item code:; NR_newRAT-Core; Date:; 2018-08-30. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; DL preemption indicator (PI) is used to indicate a set of time-frequency resource in which a UE shall not receive PDSCH. For a UE receiving PDSCH of different reliabilities (as distinguished by the RNTI, i.e. C-RNTI or MCS-C-RNTI, for the scheduling DCI), DL PI shall not preempt the UE’s reception of high reliable PDSCH (i.e. scheduled by a DCI with CRC scrambled by MCS-C-RNTI); otherwise the reliability of the corresponding PDSCH is reduced.. Summary of change:; Add the UE behavior that DL PI shall not preempt the time-frequency resources for PDSCH scheduled by a DCI with CRC scrambled by MCS-C-RNTI.. Consequences if not approved:; The reception reliability of PDSCH scheduled by a DCI with CRC scrambled by MCS-C-RNTI is reduced.. Clauses affected:; 11.2. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:.

11.2	Interrupted transmission indication

If a UE is provided higher layer parameter DownlinkPreemption, the UE is configured with an INT-RNTI provided by higher layer parameter int-RNTI for monitoring PDCCH conveying DCI format 2_1 [5, TS 38.212]. The UE is additionally configured with:

-	a set of serving cells by higher layer parameter INT-ConfigurationPerServingCell that includes a set of serving cell indexes provided by corresponding higher layer parameters servingCellId and a corresponding set of locations for fields in in DCI format 2_1 by higher layer parameter positionInDCI;

-	an information payload size for DCI format 2_1 by higher layer parameter dci-PayloadSize;

-	an indication granularity for time-frequency resources by higher layer parameter timeFrequencySet.

If a UE detects a DCI format 2_1 for a serving cell from the configured set of serving cells, the UE may assume that no transmission to the UE is present in PRBs and in symbols, from a set of PRBs and a set of symbols of the last monitoring period, that are indicated by the DCI format 2_1. The indication by the DCI format 2_1 is not applicable to receptions of SS/PBCH blocks and not applicable to reception of PDSCH scheduled by a PDCCH with DCI format 1_0/1_1 with CRC scrambled by MCS-C-RNTI.

The set of PRBs is equal to the active DL BWP as defined in Subclause 12 and includes  PRBs.

If a UE detects a DCI format 2_1 in a PDCCH transmitted in a control resource set in a slot, the set of symbols indicated by a field in DCI format 2_1 includes the last  symbols prior to the first symbol of the control resource set in the slot where  is the PDCCH monitoring periodicity provided by the value of higher layer parameter monitoringSlotPeriodicityAndOffset, as described in Subclause 10.1,  is the number of symbols per slot, is the subcarrier spacing configuration for a serving cell with mapping to a respective field in the DCI format 2_1,  is the subcarrier spacing configuration of the DL BWP where the UE receives the PDCCH conveying the DCI format 2_1. If the UE is configured with higher layer parameters tdd-UL-DL-ConfigurationCommon or tdd-UL-DL- ConfigurationCommon2, symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL- ConfigurationCommon2 are excluded from the last  symbols prior to the first symbol of the control resource set in the slot. The resulting set of symbols includes a number of symbols that is denoted as .

The UE does not expect to be provided values of , , and  resulting to a value of  that is not an integer.

A UE is provided the indication granularity for the set of PRBs and for the set of symbols by higher layer parameter timeFrequencySet.

If the value of timeFrequencySet is 0, 14 bits of a field in DCI format 2_1 have a one-to-one mapping with 14 groups of consecutive symbols from the set of symbols where each of the first  symbol groups includes  symbols, each of the last  symbol groups includes  symbols, a bit value of 0 indicates transmission to the UE in the corresponding symbol group and a bit value of 1 indicates no transmission to the UE in the corresponding symbol group.

If the value of timeFrequencySet is 1, 7 pairs of bits of a field in the DCI format 2_1 have a one-to-one mapping with 7 groups of consecutive symbols where each of the first  symbol groups includes  symbols, each of the last  symbol groups includes  symbols, a first bit in a pair of bits for a symbol group is applicable to the subset of first  PRBs from the set of  PRBs, a second bit in the pair of bits for the symbol group is applicable to the subset of last  PRBs from the set of  PRBs, a bit value of 0 indicates transmission to the UE in the corresponding symbol group and subset of PRBs, and a bit value of 1 indicates no transmission to the UE in the corresponding symbol group and subset of PRBs.