3GPP TSG RAN meeting #81																   	    RP-181744

Gold Coast, Australia, September 10-13, 2018                          Revision of RP-180867

Agenda item:	10.3.17

Source: 	CMCC

Title: 	Summary for WI Bluetooth/WLAN measurement collection in LTE Minimization of Drive Tests (MDT)

WI Code:	LTE_MDT_BT_WLAN

Leading WG:	RAN2

Release:	15

1. 	Introduction

This work item specifies Bluetooth (BT) and WLAN measurement collection in MDT to monitor and assess coverage performance of BT and WLAN network and also to provide location information for the associated other MDT measurements.

2. 	Description

In this WI, both logged MDT and immediate MDT functionality facilitating BT and WLAN measurements collection is introduced.

For WLAN measurement logging and Bluetooth measurement logging, the UE shall perform WLAN and Bluetooth measurements, respectively, only when indicated in the corresponding configuration. The measurement logging is performed only for logging intervals for which WLAN and Bluetooth measurements are available, respectively.

The measurement quantities for WLAN measurement logging are fixed and consist of BSSID, SSID, HESSID of WLAN APs. If configured by the network, optionally available RSSI and RTT can be included. The measurement quantity for Bluetooth measurement logging is fixed and consists of MAC address of Bluetooth beacons. If configured by the network, optionally available RSSI can be included.

3. 	References

- Last approved WID: 	RP-180306

- Last status report:		RP-181742

- R2-1808284	TS 36.306 CR to introduce Bluetooth and WLAN measurement collection in MDT	CMCC

- R2-1808794	TS 37.320 CR to introduce Bluetooth and WLAN measurement collection in MDT	CMCC

- R2-1812999	TS 36.331 CR to introduce Bluetooth and WLAN measurement collection in MDT	Huawei, HiSilicon

- R3-185228		TS 36.413 CR to introduce Bluetooth and WLAN measurement in MDT CMCC, Huawei, ZTE, Samsung, CATT, Nokia, Nokia Shanghai Bell

- R3-185229		TS 36.423 CR to introduce Bluetooth and WLAN measurement in MDT CMCC, Huawei, ZTE, Samsung, CATT, Nokia, Nokia Shanghai Bell