3GPP TSG RAN WG1 Meeting #94 	R1-1809648

# Gothenburg, Sweden, 20 – 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0003 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on COT length for AUL transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_unlic-Core |  |  |  |  |  |  | Date: |  | 2018-08-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The limitation on the total duration of AUL transmission and potential following DL transmission within a shared COT is not specified |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct 37.213 to ensure the total duration of AUL transmission does not exceed MCOT corresponding to priority class chosen by the UE. Also correct 37.213 to ensure the total duration of AUL transmission and following DL transmission shared by AUL transmission does not exceed MCOT corresponding to priority class chosen by the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The total duration of AUL transmission and potentially following DL transmission in shared COT initiated by AUL transmission can exceed MCOT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### < Unchanged parts are omitted >

### 4.2.1	Channel access procedure for uplink transmission(s)

The UE can access a carrier on which LAA Scell(s) UL transmission(s) are performed according to one of Type 1 or Type 2 UL channel access procedures. Type 1 channel access procedure is described in sub clause 4.2.1.1. Type 2 channel access procedure is described in sub clause 4.2.1.2.

If an UL grant scheduling a PUSCH transmission indicates Type 1 channel access procedure, the UE shall use Type 1 channel access procedure for transmitting transmissions including the PUSCH transmission unless stated otherwise in this sub clause.

A UE shall use Type 1 channel access procedure for transmitting transmissions including the PUSCH transmission on autonomous UL resources unless stated otherwise in this sub clause.

If an UL grant scheduling a PUSCH transmission indicates Type 2 channel access procedure, the UE shall use Type 2 channel access procedure for transmitting transmissions including the PUSCH transmission unless stated otherwise in this sub clause.

The UE shall use Type 1 channel access procedure for transmitting SRS transmissions not including a PUSCH transmission. UL channel access priority class =1 is used for SRS transmissions not including a PUSCH.

If the UE is scheduled to transmit PUSCH and SRS in subframe , and if the UE cannot access the channel for PUSCH transmission in subframe , the UE shall attempt to make SRS transmission in subframe  according to uplink channel access procedures specified for SRS transmission.

The total length of autonomous uplink transmission(s) obtained by the channel access procedure in this sub clause, including the following DL transmission if the UE sets ‘COT sharing indication’ in AUL-UCI to ‘1’ in a subframe within the autonomous uplink transmission(s), shall not exceed , where  is given in Table 4.2.1-1.

Table 4.2.1-1: Channel Access Priority Class for UL

| Channel Access Priority Class () |  |  |  |  | allowed sizes |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 7 | 2 ms | {3,7} |
| 2 | 2 | 7 | 15 | 4 ms | {7,15} |
| 3 | 3 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| 4 | 7 | 15 | 1023 | 6ms or 10 ms | {15,31,63,127,255,511,1023} |
| NOTE1: For , =10ms if the higher layer parameter 'absenceOfAnyOtherTechnology-r14' indicates TRUE, otherwise, =6ms. NOTE 2: When =6ms it may be increased to 8 ms by inserting one or more gaps. The minimum duration of a gap shall be 100 µs. The maximum duration before including any such gap shall be 6 ms. |  |  |  |  |  |

### < Unchanged parts are omitted >