5.9	Mobility

As defined in Report ITU-R M.2410, Mobility is the maximum mobile station speed at which a defined QoS can be achieved (in km/h). The QoS is defined as normalized traffic channel link data rate.

5.9.1	 NR

5.9.1.1	Indoor Hotspot – eMBB

Evaluation configuration A (carrier frequency = 4 GHz) and evaluation configuration B (carrier frequency = 30 GHz) with either 12TRxP or 36TRxP cases are applied for the evaluations of Indoor Hotspot– eMBB test environment for NR. Both NR FDD and TDD are evaluated. A wide range of antenna configurations and transmission schemes are considered. Detailed evaluation assumptions and results can be found in RP-181818.

5.9.1.1.1	Evaluation configuration A (CF = 4 GHz)

The evaluation results of mobility for NR FDD and NR TDD for evaluation configuration A with 12TRxP are provided in Table 5.9.1.1.1-1.

Table 5.9.1.1.1-1 NR mobility in Indoor Hotspot – eMBB 
(Evaluation configuration A, CF=4 GHz, for 12TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x8 SIMO | 15 | 1.5 | LOS | 2 | 2.09 | LOS | 2 | 2.11 |
|  |  |  | NLOS | 2 | 1.79 | NLOS | 2 | 1.80 |
| 1x2 SIMO | 30 |  | LOS | 1 | 2.8 | LOS | 1 | 2.8 |
|  |  |  | NLOS | 1 | 2.5 | NLOS | 1 | 2.5 |
| 2x8 SU-MIMO | 30 |  | LOS | / | / | LOS | / | / |
|  |  |  | NLOS | 1 | 3.85 | NLOS | / | / |
| 4x64 SU-MIMO | 30 |  | LOS | / | / | LOS | / | / |
|  |  |  | NLOS | 1 | 1.84 | NLOS | / | / |
| 2x8 SU-MIMO | 15 |  | NLOS | 1 | 6.6 dB (SNR margin) | NLOS | / | / |
| 2x8 SU-MIMO | 30 |  | NLOS | 1 | 6.39 dB (SNR margin) | NLOS | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x8 SIMO | 30 | DDDSU | 1.5 | LOS | 1 | 1.94 | LOS | 1 | 1.95 |
|  |  |  |  | NLOS | 1 | 1.59 | NLOS | 1 | 1.60 |
| 1x8 SIMO | 15 | DSUUD |  | LOS | 1 | 2.07 | LOS | 1 | 2.08 |
|  |  |  |  | NLOS | 1 | 1.78 | NLOS | 1 | 1.79 |
| 1x2 SIMO | 30 | UUUUU |  | LOS | 1 | 2.3 | LOS | 1 | 2.3 |
|  |  |  |  | NLOS | 1 | 2.0 | NLOS | 1 | 2.0 |
| 2x8 SU-MIMO | 30 | DDDDDDDSUU |  | NLOS | 1 | 3.85 | NLOS | / | / |
| 4x64 SU-MIMO | 30 | UUUUU |  | NLOS | 1 | 1.5 | NLOS | / | / |

The evaluation results of mobility for NR FDD and NR TDD for evaluation configuration A with 36TRxP are provided in Table 5.9.1.1.1-2.

It is observed that NR fulfill the mobility requirement in evaluation configuration A.

Table 5.9.1.1.1-1 NR mobility in Indoor Hotspot – eMBB 
(Evaluation configuration A, CF=4 GHz, for 36TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 2x8 SU-MIMO | 15 | 1.5 | NLOS | 1 | 0.39 dB (SNR margin) | NLOS | / | / |
| 2x8 SU-MIMO | 30 |  | NLOS | 1 | 0.38 dB (SNR margin) | NLOS | / | / |

5.9.1.1.2	Evaluation configuration B (CF = 30 GHz)

The evaluation results of mobility for NR FDD and NR TDD for evaluation configuration B with 12TRxP are provided in Table 5.9.1.1.2-1.

Table 5.9.1.1.2-1 NR mobility in Indoor Hotspot – eMBB 
(Evaluation configuration B, CF=30 GHz, for 12TRxP)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- |
| 1x4 SIMO | 60 | 1.5 | NLOS | 1 | 2.84 |
| 4x8 SU-MIMO | 120 |  | LOS | 1 | 4.76 |
|  |  |  | NLOS | 1 | 3.01 |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- | --- |
| 1x8 SIMO | 60 | UUUUU | 1.5 | NLOS | 1 | 13.26 dB (SNR margin) |
| 1x8 SIMO | 120 | UUUUU |  | NLOS | 1 | 13.32 dB (SNR margin) |

The evaluation results of mobility for NR FDD and NR TDD for evaluation configuration B with 36TRxP are provided in Table 5.9.1.1.2-2.

It is observed that NR fulfills the mobility requirement in evaluation configuration B.

Table 5.9.1.1.2-2 NR mobility in Indoor Hotspot – eMBB 
(Evaluation configuration B, CF=30 GHz, for 36TRxP)

(a) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- | --- |
| 1x8 SIMO | 60 | UUUUU | 1.5 | NLOS | 1 | 10.41 dB (SNR margin) |
| 1x8 SIMO | 120 | UUUUU |  | NLOS | 1 | 10.19 dB (SNR margin) |

5.9.1.2	Dense Urban – eMBB

Evaluation configuration A (carrier frequency = 4 GHz) and evaluation configuration B (carrier frequency = 30 GHz) are applied for the evaluations of Dense Urban – eMBB test environment for NR. Both NR FDD and TDD are evaluated. A wide range of antenna configurations and transmission schemes are considered. Detailed evaluation assumptions and results can be found in RP-181818.

5.9.1.2.1	Evaluation configuration A (CF = 4 GHz)

The evaluation results of mobility for NR FDD and NR TDD for evaluation configuration A are provided in Table 5.9.1.2.1-1.

It is observed that NR fulfills the mobility requirement in evaluation configuration A.

Table 5.9.1.2.1-1 NR mobility in Dense Urban – eMBB 
(Evaluation configuration A, CF=4 GHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x8 SIMO | 15 | 1.12 | LOS | 2 | 2.31 | LOS | 2 | 2.27 |
|  |  |  | NLOS | 2 | 2.00 | NLOS | 2 | 1.98 |
| 1x2 SIMO | 30 |  | LOS | 1 | 2.6 | LOS | 1 | 2.6 |
|  |  |  | NLOS | 1 | 1.8 | NLOS | 1 | 1.8 |
| 1x4 SIMO | 30 |  | NLOS | / | / | NLOS | 1 | 1.57 |
| 2x8 SU-MIMO | 30 |  | NLOS | 2 | 4.07 | NLOS | 1 | 3.552 |
| 4x64 SU-MIMO | 30 |  | NLOS | 1 | 1.85 | NLOS | / | / |
| 2x8 SU-MIMO | 15 |  | NLOS | / | / | NLOS | 1 | 3.352 |
| 2x8 SU-MIMO | 15 |  | NLOS | 1 | 3.02 dB (SNR margin) | NLOS | / | / |
| 2x8 SU-MIMO | 30 |  | NLOS | 1 | 3.03 dB (SNR margin) | NLOS | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x8 SIMO | 30 | DDDSU | 1.12 | LOS | 1 | 2.17 | LOS | 1 | 2.06 |
|  |  |  |  | NLOS | 1 | 1.82 | NLOS | 1 | 1.79 |
| 1x8 SIMO | 15 | DSUUD |  | LOS | 1 | 2.33 | LOS | 1 | 2.29 |
|  |  |  |  | NLOS | 1 | 2.03 | NLOS | 1 | 2.00 |
| 1x2 SIMO | 30 | UUUUU |  | LOS | 1 | 2.2 | LOS | 1 | 2.2 |
|  |  |  |  | NLOS | 1 | 1.5 | NLOS | 1 | 1.5 |
| 2x8 SU-MIMO | 30 | DDDDDDDSUU |  | NLOS | 1 | 4.58 | NLOS | / | / |
| 4x64 SU-MIMO | 30 | UUUUU |  | NLOS | 1 | 1.52 | NLOS | / | / |

5.9.1.2.2	Evaluation configuration B (CF = 30 GHz)

The evaluation results of mobility for NR FDD and NR TDD for evaluation configuration B are provided in Table 5.9.1.2.2-1.

It is observed that NR fulfills the mobility requirement in evaluation configuration B.

Table 5.9.1.2.2-1 NR mobility in Dense Urban – eMBB 
(Evaluation configuration B, CF=30 GHz)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| --- | --- | --- | --- | --- | --- |
| 1x4 SIMO | 60 | 1.12 | NLOS | 1 | 1.24 |

5.9.1.3	Rural – eMBB

Evaluation configuration A (carrier frequency = 700 MHz) and evaluation configuration B (carrier frequency = 4 GHz) are applied for the evaluations of Rural – eMBB test environment for NR. Both NR FDD and TDD are evaluated. The mobility class of 120km/h and 500km/h are considered. A wide range of antenna configurations and transmission schemes are considered. Detailed evaluation assumptions and results can be found in RP-181818.

5.9.1.3.1	Evaluation configuration A (CF = 700 MHz)

The evaluation results of mobility for NR FDD and NR TDD for evaluation configuration A for mobility class of 120km/h are provided in Table 5.9.1.3.1-1.

Table 5.9.1.3.1-1 NR mobility in Rural – eMBB 
(Evaluation configuration A, CF=700 MHz, Mobility class of 120km/h)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x4 SIMO | 15 | 0.8 | LOS | 1 | 2.90 | LOS | 1 | 2.90 |
|  |  |  | NLOS | 1 | 2.32 | NLOS | 1 | 2.31 |
| 1x2 SIMO | 15 |  | LOS | 1 | 1.80 | LOS | 1 | 1.80 |
|  |  |  | NLOS | 1 | 1.00 | NLOS | 1 | 1.00 |
| 1x4 SIMO | 15 |  | NLOS | / | / | NLOS | 1 | 1.75 |
| 2x2 SU-MIMO | 15 |  | NLOS | 2 | 1.79 | NLOS | 1 | 1.542 |
| 1x8 SIMO | 15 |  | LOS | 1 | 2.89 | LOS | 1 | 2.91 |
|  |  |  | NLOS | 1 | 2.86 | NLOS | 1 | 2.90 |
| 2x8 SU-MIMO | 15 |  | NLOS | 1 | 8.54 dB (SNR margin) | NLOS | / | / |
| 2x8 SU-MIMO | 30 |  | NLOS | 1 | 8.07 dB (SNR margin) | NLOS | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x4 SIMO | 15 | DDDSU | 0.8 | LOS | 1 | 2.63 | LOS | 1 | 2.63 |
|  |  |  |  | NLOS | 1 | 2.10 | NLOS | 1 | 2.09 |
| 1x2 SIMO | 15 | UUUUU |  | LOS | 1 | 1.60 | LOS | 1 | 1.60 |
|  |  |  |  | NLOS | 1 | 0.85 | NLOS | 1 | 0.85 |
| 2x2 SU-MIMO | 15 | DDDSU |  | NLOS | 1 | 2.01 | NLOS | / | / |
| 1x8 SIMO | 15 | DSUUD |  | LOS | 1 | 2.81 | LOS | 1 | 2.82 |
|  |  |  |  | NLOS | 1 | 2.77 | NLOS | 1 | 2.81 |

The evaluation results of mobility for NR FDD and NR TDD for evaluation configuration A for mobility class of 500km/h are provided in Table 5.9.1.3.1-2.

It is observed that NR fulfills the mobility requirement under both 120km/h and 500km/h.

Table 5.9.1.3.1-2 NR mobility in Rural – eMBB 
(Evaluation configuration A, CF=700 MHz, Mobility class of 500km/h)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x4 SIMO | 30 | 0.45 | LOS | 1 | 2.64 | LOS | 1 | 2.64 |
|  |  |  | NLOS | 1 | 2.07 | NLOS | 1 | 2.07 |
| 1x2 SIMO | 15 |  | LOS | 1 | 1.40 | LOS | 1 | 1.40 |
|  |  |  | NLOS | 1 | 0.70 | NLOS | 1 | 0.70 |
| 1x4 SIMO | 15 |  | NLOS | / | / | NLOS | 1 | 0.86 |
| 2x2 SU-MMO | 15 |  | NLOS | 1 | 1.92 | NLOS | / | / |
| 2x2 SU-MMO | 30 |  | NLOS | 1 | 1.28 | NLOS | 1 | 1.21 |
| 1x8 SIMO | 30 |  | LOS | 1 | 2.40 | LOS | 1 | 2.43 |
|  |  |  | NLOS | 1 | 2.39 | NLOS | 1 | 2.40 |
| 4x64 SU-MIMO | 30 |  | NLOS | 1 | 0.87 | NLOS | / | / |
| 2x8 SU-MIMO | 15 |  | NLOS | 1 | 12.04 dB (SNR margin) | NLOS | / | / |
| 2x8 SU-MIMO | 30 |  | NLOS | 1 | 11.77 dB (SNR margin) | NLOS | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x4 SIMO | 30 | DDDSU | 0.45 | LOS | 1 | 2.39 | LOS | 1 | 2.39 |
|  |  |  |  | NLOS | 1 | 1.88 | NLOS | 1 | 1.87 |
| 1x2 SIMO | 15 |  |  | LOS | 1 | 1.10 | LOS | 1 | 1.10 |
|  |  |  |  | NLOS | 1 | 0.60 | NLOS | 1 | 0.60 |
| 2x2 SU-MIMO | 15 | DDDSU |  | LOS | 0 |  | LOS | 0 |  |
|  |  |  |  | NLOS | 1 | 1.92 | NLOS | 0 |  |
| 1x8 SIMO | 30 | DSUUD |  | LOS | 1 | 2.30 | LOS | 1 | 2.33 |
|  |  |  |  | NLOS | 1 | 2.28 | NLOS | 1 | 2.29 |

5.9.1.3.2	Evaluation configuration B (CF = 4 GHz)

The evaluation results of mobility for NR FDD and NR TDD for evaluation configuration B for mobility class of 120km/h are provided in Table 5.9.1.3.2-1.

Table 5.9.1.3.2-1 NR mobility in Rural – eMBB 
(Evaluation configuration B, CF=4 GHz, Mobility class of 120km/h)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x4 SIMO | 30 | 0.8 | LOS | 1 | 1.74 | LOS | 1 | 1.68 |
|  |  |  | NLOS | 1 | 1.30 | NLOS | 1 | 1.28 |
| 1x4 SIMO | 30 |  | NLOS | / | / | NLOS | 1 | 1.08 |
| 2x8 SU-MIMO | 30 |  | NLOS | 1 | 2.68 | NLOS | 1 | 2.41 |
| 1x8 SIMO | 30 |  | LOS | 1 | 1.87 | LOS | 1 | 1.86 |
|  |  |  | NLOS | 1 | 1.74 | NLOS | 1 | 1.73 |
| 2x64 SU-MIMO | 30 |  | NLOS | 1 | 1.02 | NLOS | / | / |
| 2x8 SU-MIMO | 15 |  | NLOS | 1 | 9.47 dB (SNR margin) | NLOS | / | / |
| 2x8 SU-MIMO | 30 |  | NLOS | 1 | 8.30 dB (SNR margin) | NLOS | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x4 SIMO | 30 | DDDSU | 0.8 | LOS | 1 | 1.57 | LOS | 1 | 1.52 |
|  |  |  |  | NLOS | 1 | 1.18 | NLOS | 1 | 1.16 |
| 1x8 SIMO | 30 | DSUUD |  | LOS | 1 | 1.79 | LOS | 1 | 1.78 |
|  |  |  |  | NLOS | 1 | 1.66 | NLOS | 1 | 1.65 |
| 2x64 SU-MIMO | 30 | UUUUU |  | NLOS | 1 | 0.8 | NLOS | / | / |

The evaluation results of mobility for NR FDD and NR TDD for evaluation configuration B for mobility class of 500km/h are provided in Table 5.9.1.3.2-2.

It is observed that NR fulfills the mobility requirement under both 120km/h and 500km/h.

Table 5.9.1.3.2-2 NR mobility in Rural – eMBB 
(Evaluation configuration B, CF=4 GHz, Mobility class of 500km/h)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x4 SIMO | 60 | 0.45 | LOS | 1 | 1.33 | LOS | 1 | 1.33 |
|  |  |  | NLOS | 1 | 0.92 | NLOS | 1 | 0.91 |
| 2x8 SU-MIMO | 30 |  | NLOS | 1 | 1.56 | NLOS | 1 | 1.48 |
| 1x8 SIMO | 30 |  | LOS | 1 | 1.43 | LOS | 1 | 1.38 |
|  |  |  | NLOS | 1 | 1.11 | NLOS | 1 | 1.08 |
| 2x8 SU-MIMO | 15 |  | NLOS | 1 | 11.50 dB (SNR margin) | NLOS | / | / |
| 2x8 SU-MIMO | 30 |  | NLOS | 1 | 11.17 dB (SNR margin) | NLOS | / | / |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x4 SIMO | 60 | DDDSU | 0.45 | LOS | 1 | 1.22 | LOS | 1 | 1.22 |
|  |  |  |  | NLOS | 1 | 0.84 | NLOS | 1 | 0.83 |
| 1x8 SIMO | 30 | DSUUD |  | LOS | 1 | 1.37 | LOS | 1 | 1.31 |
|  |  |  |  | NLOS | 1 | 1.06 | NLOS | 1 | 1.03 |

5.9.2	LTE

5.9.2.3	Rural - eMBB

Evaluation configuration A (carrier frequency = 700 MHz) is applied for the evaluations of Rural – eMBB test environment for LTE. Both LTE FDD and TDD are evaluated. The mobility class of 120km/h and 500km/h are considered. Detailed evaluation assumptions and results can be found in RP-181818.

5.9.2.3.1	Evaluation configuration A (CF = 700 MHz)

The evaluation results of mobility for LTE FDD and TDD for evaluation configuration A for mobility class of 120km/h are provided in Table 5.9.2.3.1-1.

Table 5.9.2.3.1-1 LTE mobility in Rural – eMBB 
(Evaluation configuration A, CF=700 MHz, Mobility class of 120km/h)

(a) NR FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x4 SIMO | 15 | 0.8 | LOS | 1 | 2.79 | LOS | 1 | 2.79 |
|  |  |  | NLOS | 1 | 2.20 | NLOS | 1 | 2.19 |
| 1x2 SIMO | 15 |  | LOS | 1 | 1.70 | LOS | 1 | 1.70 |
|  |  |  | NLOS | 1 | 1.00 | NLOS | 1 | 1.00 |

(b) NR TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x4 SIMO | 15 | DSUDD | 0.8 | LOS | 1 | 2.49 | LOS | 1 | 2.49 |
|  |  |  |  | NLOS | 1 | 1.96 | NLOS | 1 | 1.94 |

The evaluation results of mobility for LTE FDD and TDD for evaluation configuration A for mobility class of 500km/h are provided in Table 5.9.2.3.1-2.

It is observed that LTE fulfills the mobility requirement under both 120km/h and 500km/h.

Table 5.9.2.3.1-2 LTE mobility in Rural – eMBB 
(Evaluation configuration A, CF=700 MHz, Mobility class of 500km/h)

(a) LTE FDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x4 SIMO | 15 | 0.45 | LOS | 1 | 2.59 | LOS | 1 | 2.59 |
|  |  |  | NLOS | 1 | 1.94 | NLOS | 1 | 1.93 |
| 1x2 SIMO | 15 |  | LOS | 1 | 1.05 | LOS | 1 | 1.05 |
|  |  |  | NLOS | 1 | 0.60 | NLOS | 1 | 0.60 |

(b) LTE TDD

| Scheme and antenna configuration | Sub-carrier spacing (kHz) | Frame structure | ITU Requirement (bit/s/Hz) | Channel model A |  |  | Channel model B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) | Channel condition | Number of samples | Normalized traffic channel link data rate (bit/s/Hz) |
| 1x4 SIMO | 15 | DSUDD | 0.45 | LOS | 1 | 2.31 | LOS | 1 | 2.31 |
|  |  |  |  | NLOS | 1 | 1.73 | NLOS | 1 | 1.72 |