3GPP TSG RAN Meeting #82  	RP-182317

# Sorrento, Italy, Dec 10th – 13th, 2018

Agenda item:	9.3.17

Source: 	Samsung

Title: 	Survey on regulatory requirements beyond 52.6 GHz in Korea

Document for:	Discussion and Decision

Introduction

In RAN#81 meeting, a new study item on NR beyond 52.6GHz was agreed with the following for the scope of the work [1]:

- Identify target spectrum ranges

  - Survey on global spectrum availability and regulatory requirements (including channelization and licensing regimes)

    - For 60 GHz bands, TR 38.805 can be a reference.

- Identify potential use cases and deployment scenarios

- Identify NR design requirements and considerations on top of regulatory requirements

Based on work plan of SI, we provide the survey result of regulatory requirements between 52.6 GHz and 100 GHz in Korea [2].

Discussion

The followings are regulatory requirements above 52.6 GHz including available spectrum band and related regulatory requirements [3][4].

Table 1. Korea Regulatory Requirements for Available Spectrum between 52.6 GHz and 100 GHz

| Frequency band |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | 52.6 – 54.25 GHz | All emission is prohibited | No requirement | No requirement | No requirement | All emission is prohibited |
| n2 | 57 – 66 GHz | Power below 500mW (for omni-directional power below 100mW) PSD below 13dBm/MHz EIRP below 43dBm (for fixed point to point EIRP below 57dBm) | For equipment with more than 27dBm EIRP operating in 57-58GHz, the following instruction shall be indicated in user the instruction manual. “User who would like to use this equipment within 300 m from Astronomical antenna have to obtain consent from astronomical observatory” | Occupied BW shall be within the range of 57-66 GHz | For unlicensed wireless communications | Antenna absolute gain shall be less than 16dBi. If antenna gain greater than 16dBi may be employed than conducted output power shall be reduced as much as the increased antenna gain. See Note 1 for additional regulatory requirements. |
| n3 | 71 – 76 GHz | Power below 3W, PSD below 21.7609dBm/100 MHz(150dBW/100MHz), EIRP below 85 dBm | Permits use under condition that do not cause interference to Astronomy stations that are already in use. | Occupied BW shall be within the range of 71-76 GHz | For licensed fixed point-to-point wireless communications. For amateur radio station (see Note 2 for channelization) | Antenna absolute gain shall be greater than 43dBi. The half power beamwidth of the antenna must be smaller than 1.2 degrees. See Note 3 for additional regulatory requirements. |
| n4 | 76 – 81 GHz | PSD below -3 dBm/MHz EIRP below 34dBm/50MHz | No requirement | Occupied BW shall be within the range of 76-81 GHz | For outdoor level detection radars. For amateur radio station (see Note 2 for channelization) | Fixed antenna orientation with down tilt angle. The half power beamwidth of the antenna array must be smaller than 8 degrees Unwanted emission outside the 76-81GHz shall be at least attenuated 20dB more than the PSD |
| n5 | 76 – 77 GHz | Power below 10mW EIRP below 55dBm | Function for interference avoidance shall be required. (No Specific technical requirement is described) | Occupied BW shall be within the range of 76-77 GHz | For automotive proximity detection radars | See Note 4 for unwanted emission requirements |
| n6 | 81 – 86 GHz | Power below 3W, PSD below 21.7609dBm/100 MHz(150dBW/100MHz), EIRP below 85 dBm | Permits use under condition that do not cause interference to Astronomy stations that are already in use. | Occupied BW shall be within the range of 81-86 GHz | For licensed fixed point-to-point wireless communications | Antenna absolute gain shall be greater than 43dBi. The half power beamwidth of the antenna must be smaller than 1.2 degrees. See Note 3 for additional regulatory requirements |
| n7 | 86 – 92 GHz | No requirement | No requirement | No requirement | No requirement | All emissions are prohibited |

*Note 1:

A user identification code shall be used in each device to prevent malfunction of other devices and to prevent malfunction caused by signals from other devices. However, it does not apply to fixed point-to-point communication.

- Unwanted emission shall not exceed emission level given in Table 2.

Table 2. Unwanted emission requirements

| Spectrum Range | Unwanted Emission Limit | Measurement Bandwidth |
| --- | --- | --- |
| Less than 1GHz | -36dBm | 100kHz |
| 1GHz to 40GHz | -30dBm | 1MHz |
| More than 40GHz | -10dBm | 1MHz |

- Tx or Rx spurious emission for Standby state (not operating) shall not exceed emission level given in Table 3.

Table 3 Spurious emission requirements

| Spectrum Range | Unwanted Emission Limit | Measurement Bandwidth |
| --- | --- | --- |
| Less than 1GHz | -54dBm | 100kHz |
| More than 1GHz | -47dBm | 1MHz |

*Note 2:

From 75.5 to 81GHz, 75.75, 77.75, 78.5, and 80 GHz are assigned for the carrier frequencies of amateur radio station.

*Note 3:

The unwanted emissions for 1 MHz measurement bandwidth shall be less than the followings

- Unwanted emission outside the 76-77GHz or 81-86GHz shall not exceed emission level given below:

A (dBc) = 11 + 0.4(S-50) + 10log(B)

where B is occupied bandwidth (MHz) and S is the percentage of the frequency separation from the center frequency for the given occupied frequency bandwidth.

- Spurious emission shall not exceed emission level given below:

A (dBc) = 43 + 10log(P)

where P is power (W).

*Note 4:

The unwanted emission outside the 76-77GHz shall not exceed emission level given in Table 4.

Table 4. Unwanted emission requirements

| Spectrum Range | Unwanted Emission Limit | Measurement Bandwidth |
| --- | --- | --- |
| Less than 1GHz | -26dBm * or 0dBm ** | 100kHz |
| More than 1GHz | -26dBm * or 0dBm ** | 1MHz |
| * In case that Conducted power level shall be less than 10mW ** In case that EIRP shall be less than 55dBm |  |  |

- References

- RP-182066	Revised SID on Study on NR beyond 52.6GHz	Intel

- No. 2018-36 	Ministry of Science and ICT notice

- No. 2018-71 	Ministry of Science and ICT notice

- No. 2018-26 	National Radio Research Agency notice