3GPP TSG RAN Meeting #82	RP-182328

Sorrento, Italy, December 10-13, 2018

Agenda item:		9.3.17

Source: 	Nokia, Nokia Shanghai Bell

Title: 	NR beyond 52.6GHz - Spectrum Availability and Regulatory Requirements

Document for: 		Discussion and Decision

# Introduction

This document provides a summary of spectrum availability and regulatory requirements for USA, Europe/CEPT and India per objective #1 as described in [1].

Note that we provide information for the range 52.6 GHz – 114.25 GHz, as discussed in [3, 4].

# Spectrum availability and regulatory requirements

The following spectrum availability and regulatory requirements have been summarized for USA, Europe/CEPT and India.

## USA

Table 1.  USA – Regulatory Requirements for Available and Potential Spectrum between 52.6 GHz and 114.25 GHz

| Frequency band |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 57 – 71 GHz | The average power of any emission shall not exceed 40 dBm and the peak power of any emission shall not exceed 43 dBm For fixed point-to-point transmitters located outdoors, the average power of any emission shall not exceed 82 dBm, and shall be reduced by 2 dB for every dB that the antenna gain is less than 51 dBi. The peak power of any emission shall not exceed 85 dBm, and shall be reduced by 2 dB for every dB that the antenna gain is less than 51 dBi. | No requirement | Fundamental emissions must be contained within the frequency bands specified in this section during all conditions of operation. | Unlicensed band | See Note 1 for spurious emissions |
|  | 71-76 GHz & 81-86GHz (Fixed Microwave Services) | Maximum allowable EIRP= 55 dBW Maximum power limit = 5dBW Transmitter maximum power spectral density (PSD)= 150 mW/100 MHz Minimum antenna gain: 43 dBi Antenna gain less than 50 dBi (but greater than or equal to 43 dBi) is permitted only with a proportional reduction in maximum authorized EIRP in a ratio of 2 dB of power per 1 dB of gain, so that the maximum allowable EIRP (in dBW) for antennas of less than 50 dBi gain becomes + 55−2(50-G), where G is the antenna gain in dBi. Minimum spectral efficiency: 0.125 bits/s/Hz | Sharing and coordination among non-government licensees and between non-government and government services. | 5GHz | Lightly licensed, First, a licensee applies for a non-exclusive nationwide license; second, the licensee registers individual point-topoint links with a database manager. Registration process verifies potential for harmful interference to or from all previously registered non-government links. | An entity may request any portion of the 71-76 GHz and 81-86 GHz bands, up to 5 gigahertz in each segment for a total of 10 gigahertz. |
|  | 92-95 GHz (Fixed Microwave Services) | Maximum allowable EIRP= 55 dBW Minimum antenna gain: 50 dBi Minimum spectral efficiency: 1.0 bits/s/Hz | Sharing and coordination among non-government licensees and between non-government and government services. | Maximum bandwidth is licensed in one segment of 2 GHz from 92-94 GHz and one 0.9 GHz segment from 94.1 to 95 GH | Lightly licensed. First, a licensee applies for a non-exclusive nationwide license; second, the licensee registers individual point-topoint links with a database manager. Registration process verifies potential for harmful interference to or from all previously registered non-government links. | The 92-95 GHz band is divided into three segments: 92.0-94.0 GHz and 94.1-95.0 GHz for non-government and government users, and 94.0-94.1 GHz for Federal Government use (for radioastronomy use, to be excluded from the study). Pairing is allowed and segments may be aggregated without limit. The 70 & 80GHz bands can be included for a possible 12.9 gigahertz maximum aggregation. |
|  | 92-95 GHz (Unlicensed) | The average power density of any emission, measured during the transmit interval, shall not exceed 9 uW/sq. cm, as measured at 3 meters from the radiating structure, and the peak power density of any emission shall not exceed 18 uW/sq. cm, as measured 3 meters from the radiating structure. | No requirement | Fundamental emissions must be contained within the frequency bands specified in this section during all conditions of operation. | Unlicensed band Indoor only | See Note 1 |
|  | 95-100 GHz, 102-109.5 GHz, 111.8-114.25 GHz | Rules being defined by FCC as part of ther Spectrum Horizons 95-275GHz proceeding | Rules being defined by FCC as part of ther Spectrum Horizons 95-275GHz proceeding | Rules being defined by FCC as part of ther Spectrum Horizons 95-275GHz proceeding | Licensed | There are passive services like radio astronomy service (RAS), Earth exploration-satellite service (EESS) (passive), and space research service (SRS) (passive) in the gaps (100-102GHz and 109.5-111.8 GHz) between bands considered See Note 2 |

Note 1:

(1) The power density of any emissions outside the 57-71 GHz band shall consist solely of spurious emissions.

(2) Radiated emissions below 40 GHz shall not exceed the general limits in § 15.209. (see https://www.law.cornell.edu/cfr/text/47/15.209 )

(3) Between 40 GHz and 200 GHz, the level of these emissions shall not exceed 90 pW/cm 2 at a distance of 3 meters.

Note 2:

On February 22nd, 2018, the FCC adopted their Spectrum Horizons Notice of Proposed Rulemaking (NPRM) that seeks comment on:bands above 95GHz; A mmW coalition [6] has been active in this proceeding and has petitioned FCC to include these passive bands (100-102GHz and 109.5-111.8 GHz), and other passive bands above 114.25GHz for consideration for terrestrial fixed and mobile use. If aproved, then the whole 95-114.25GHz range could be available in USA.

## Europe and CEPT

Table 2. Europt/CEPT – Regulatory Requirements for Available and Potential Spectrum between 52.6 GHz and 114.25 GHz

| Frequency band |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 57 – 66 GHz | 40 dBm mean e.i.r.p.. This refers to the highest power level of the transmitter power control range during the transmission burst if transmitter power control is implemented | Adequate spectrum sharing mechanism (e.g. Listen-before-Talk, Detect-And-Avoid) shall be implemented by the equipment | Not specified |  | The maximum mean e.i.r.p. density is limited to 13 dBm/MHz. Point-to-point links of the Fixed Service are regulated by ECC/REC/ (05)02 and ECC/REC/ (09)01 |
|  | 66 – 71 GHz | To be defined | To be defined | To be defined | To be defined |  |
|  | 71-76 GHz & 81-86GHz (Fixed Services) | Maximum power limit= 30dBm Maximum EIRP= 85dBm (55 dBW) Minimum antenna gain= Pout (dBm) + 15; or 38 (whichever is the greater) maximum antenna gain= 85-Pout(dBm) Provision for reduced EIRP, see ETSI EN 302 217 Minimum spectral efficiency, see ETSI Radio Interface capacity (RIC) Complete set of specification, ETSI EN 302 217 part 2-2 and part 3 | Access to the frequency bands by Authorization regimes or by Block assignment/auction regimes . | An aggregation of 250 MHz channels for wider channels as defined in ECC/REC/(05)07 (channel arrangements for channel sizes ranging from 250 to 4750 MHz) | To assist the planning of fixed links, self-coordination approach, similar to the “light licensing”, described in ECC Report 80 [can be considered. |  |
|  | 92-94 GHz, 94.1-100 GHz, 102-109.5 GHz and 111.8-114.25 GHz | For Fixed Service, defined in ECC Recommendation (18)02 | For Fixed Service, defined in ECC Recommendation (18)02 | Radio frequency channel/block arrangements for Fixed Service are defined in ECC Recommendation (18)02 | The choice of the appropriate assignment method and licensing regime for Fixed Service remains a decision for national administrations, based on several technical and not technical factors | To assist the planning of fixed links, self-coordination approach, similar to the “light licensing”, described in ECC Report 80 can be considered. |

## India

The Government of India Ministry of Communications’ Telecommunications Department has recently released a national frequency allocation plan [2].   Table 3 is populated per this plan.  For frequencies > 52.6 GHz, the power requirements, spectrum access, modulation and occupied bandwidth remain undefined.  The NFAP is released typically every 3 to 4 years and the next one is not expected until after 2020.

Table 3. India – Regulatory Requirements for Available and Potential Spectrum between 52.6 GHz and 114.25 GHz

| Frequency band |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 56.9 – 59.3 GHz | Not defined | Not defined | Not defined | -ESS (Passive) -Space Research (Passive) - Fixed - Mobile |  |
|  | 59.3 – 71 GHz | Not defined | Not defined | Not defined | -Fixed -Inter-satellite -Mobile -Radiolocation -Space Research |  |
|  | 71 – 78 GHz | Not defined | Not defined | Not defined | -fixed satellite(space-to-earth) -Fixed -Broadcasting -Space Research -Radio astronomy |  |
|  | 78 – 86 GHz | Not defined | Not defined | Not defined | -Radiolocation -Amateur Satellite -Radio astronomy -Space Research (Space-to-Earth) -Fixed -Mobile |  |
|  | 86-95 GHz | Not defined | Not defined | Not defined | -Earth Exploration Satellite -Radio astronomy -Space Research |  |
|  | 95-111.8 GHz | Not defined | Not defined | Not defined | -Fixed -Mobile -Radio Atronomy -Radiolocation -Radionavigation -Radionavigation-satellite -Space research |  |
|  | 111.8-114.25 GHz | Not defined | Not defined | Not defined | -Fixed -Mobile -Radio astronomy -Space Research Inter satellite |  |

# Summary

This document provides a summary of spectrum availability and regulatory requirements for USA, Europe/CEPT and India.

# References

- RP-182066	Revised SID on Study on NR beyond 52.6GHz

- NFAP-18		“National Frequency Allocation Plan – 2018”, Government of India Ministry of Communications

- RP-182326	“Frequency range for NR beyond 52.6 GHz”, Nokia, Nokia Shanghai Bell

- RP-182327	“Revised SID for Study on NR beyond 52.6GHz”, Nokia, Nokia Shanghai Bell

- FCC Spectrum Horizons Notice of Proposed Rulemaking and Order, FCC 18-17 (rel. Feb. 28, 2018)

- mmW Coalition http://mmwavecoalition.org/