3GPP TSG RAN meeting #82                                                            RP-182417

Sorrento, Italy, December 10-13, 2018

Source:	NTT DOCOMO, INC.

Title:	Survey on Japanese spectrum usage and regulatory requirements in the frequency band above 52.6GHz

Agenda Item:	9.3.17

Document for: 	Discussion and Decision

- Introduction

In this contribution, we provide a survey on Japanese spectrum usage and regulatory requirements in the frequency band above 52.6GHz.

- Survey on Japanese spectrum availability and regulatory requirements for above 52.6GHz

Table 1 and Figure 1 below summarize the survey on Japanese spectrum usage and regulatory requirements in the frequency band above 52.6GHz.

Figure 1: Major spectrum usage in the frequency band above 52.6GHz in Japan

(source: http://www.tele.soumu.go.jp/e/adm/freq/search/myuse/0002/index.htm)

Table 1. Major spectrum usage and regulatory requirements between 52.6 GHz and 100 GHz in Japan*

| Frequency band |  | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  | 52.6 – 54.25 GHz |  |  |  | For Earth Exploration Satellite Service (passive), Space Research Service (passive) |  |
|  | 54.25 – 55.78 GHz | Power below 1W |  | See Note 1 | For Broadcast Auxiliary Services (Mobile) | ARIB STD-B43 [1] See Note 2 for additional regulatory requirements. |
|  | 55.78 – 59 GHz |  |  |  | For Commercial Telecommunications, Public and General Services (Fixed/Mobile) |  |
|  | 57 – 66 GHz | Power below 250mW | See Note 3 | Occupied BW shall be below 9GHz | For Unlicensed low power data Communication systems | ARIB STD-T69 [2], ARIB STD-T117 [3] See Note 4 for additional regulatory requirements. |
|  | 60 – 61 GHz, 76 – 77 GHz | Power below 10mW |  | See Note 5 | For Vehicle Radars (Unlicensed Low-Power) | ARIB STD-T48 [4] See Note 6 for additional regulatory requirements. |
|  | 77 – 81 GHz | Power below 10mW or PSD below 5μW/MHz in case of occupied BW below 2GHz |  | Occupied BW shall be below 4GHz | For Vehicle Radars (Unlicensed Low-Power) | ARIB STD-T111 [5] See Note 7 for additional regulatory requirements. |
|  | 71 – 76 GHz, 81 – 86 GHz | Peak power below 1W |  | Occupied BW shall be below 5GHz | For Commercial Telecommunications, Public and General Services (Fixed/Mobile) | See Note 8 for additional regulatory requirements. |
|  | 77.5 – 78 GHz |  |  |  | For Amateur |  |
|  | 85.5 – 92 GHz |  |  |  | For Radio Astronomy |  |

Note *: Other radio stations may be used in the respective frequency bands.

Note 1: For system with 125MHz BW, PSK, QAM and OFDM are available. For system with 500MHz BW, PSK is available. For system with 1GHz BW, PSK and QAM are available.

Note 2: The unwanted emission shall satisfy below 50μW.

Note 3: The carrier sensing functionality is required for transmission power above 10dBm.

Note 4: The antenna absolute gain shall be less than 47dBi. In case of transmission power above 10mW, directivity with above 10dBi antenna absolute gain for main beam direction is necessary and EIRP shall be less than 40dBm.

In addition, the unwanted emission shall satisfy below Table 2.

Table 2. Unwanted emission requirement

| Frequency | Unwanted emission requirement |
| --- | --- |
| Below 55.62GHz | Below -30dBm/MHz |
| Between 55.62GHz and 57GHz, Between 66GHz and 67.5GHz | Below -26dBm/MHz |
| Above 67.5GHz | Below -30dBm/MHz |

Note 5: The allowed value for occupied channel bandwidth (which is defined as bandwidth containing 99.5% of transmission power) is shown in below Table 3.

Table 3. Allowed value for occupied channel bandwidth

| Frequency | Occupied channel bandwidth |
| --- | --- |
| Between 60GHz and 61GHz | Below 500MHz |
| Between 76GHz and 77GHz | Below 1GHz |

Note 6: The antenna gain shall be less than 40dB.

In addition, the unwanted emission shall satisfy below Table 4.

Table 4. Unwanted emission requirement

| Out-of-band domain emission limit | Spurious domain emission limit |
| --- | --- |
| Below 100μW | Below 50μW |

Note 7: The antenna absolute gain shall be less than 35dBi.

In addition, the unwanted emission shall satisfy Table 5.

Table 5. Unwanted emission requirement

| Out-of-band domain emission limit | Spurious domain emission limit |
| --- | --- |
| Peak power below 100μW/MHz | Peak power below 50μW/MHz |

Note 8: The antenna absolute gain shall be less than 55dBi.

In addition, the unwanted emission shall satisfy Table 5.

- Conclusion

In this contribution, we provided a survey on Japanese spectrum usage and regulatory requirements in the frequency band above 52.6GHz. We propose to capture the survey to TR38.807.

References

- ARIB STD-B43 v2.1, “PORTABLE MILLIMETER-WAVE DIGITAL TRANSMISSION SYSTEM FOR TELEVISION PROGRAM CONTRIBUTION,” Jan. 2018.

- ARIB STD-B69 v4.0, “LOW POWER DATA COMMUNICATION SYSTEM/MILLIMETER-WAVE VIDEO TRANSMISSION EQUIPMENT,” Sept. 2016.

- ARIB STD-B117 v1.0, “LOW POWER DATA COMMUNICATION SYSTEM/60 GHz-BAND WIRELESS LAN FOR VERY HIGH THROUGHPUT DATA COMMUNICATIONS,” Sept. 2016.

- ARIB STD-B48 v2.2, “MILLIMETER-WAVE RADAR EQUIPMENT FOR SPECIFIED LOW POWER RADIO STATION,” Dec. 2015.

- ARIB STD-B111 v1.1, “79 GHz BAND HIGH-RESOLUTION RADAR,” Mar. 2017.