3GPP TSG-RAN Meeting #82         							          RP-182476

Sorrento, Italy, December 10-13, 2018

Source: 	ZTE Corporation

Title: 	Survey on spectrum availability and regulatory requirements in ITU Region -3

Agenda Item:	9.3.17

Document for:	Discussion/Decision

Background

In RAN#81 a RAN-level Study on NR beyond 52.6GHz was updated [1], and its skeleton was approved [2].

The following work-plan is described in [1]:

- RAN#81: TR skeleton

- RAN#82-84: Objective 1

- RAN#83-84: Objective 2 and 3

- RAN#85: Conclude the SI and approve the TR

where the Objectives are given by [1] to be:

- Identify target spectrum ranges

  - Survey on global spectrum availability and regulatory requirements (including channelization and licensing regimes)

    - For 60GHz bands, TR 38.805 can be a reference.

- Identify potential use cases and deployment scenarios

- Identify NR design requirements and considerations on top of regulatory requirements

In this contribution we present our survey on spectrum availability and regulatory requirements in three countries in ITU region-3, including China, Australia and Singapore.

Discussion

Available spectrum

In China, MIIT (Ministry of Industry and Information Technology of the People's Republic China) publishes that short distant unlicensed applications can be applied between 59-64 GHz. While, 76-77GHz is considered for Vehicle Radar ranging application[3,4]. Moreover, details of their regulatory requirements can be found in Annex.

Secondly, ACMA (Australian Communications and Media Authority) publishes sensors using radar which can be applied in some different bands from 52.6GHz to 85GHz in Australia [5]. Moreover, details of their regulatory requirements can be found in Annex.

Finally, in Singapore, IMDA (Info-communications Media Development Authority) shows that 57-66 GHz is considered for Wireless LAN and broadband access application [6]. On the other hand, short range radar systems, such as automatic cruise control and collision warning systems for vehicle, are applied from76 to 77GHz. Moreover, details of their regulatory requirements can be found in Annex. Furthermore, it is notable that both spectrum band, 71 – 76 GHz paired with 81 – 86 GH, will be available for short-term commercial use in fixed-wireless backhaul and inter-building fixed-wireless network.

Proposal 1: Text proposal on spectrum availability and regulatory requirements of China, Australia and Singapore are shown in the Annex.

Potential spectrum

In 2015, the international telecommunication union (ITU) proposed 11 candidate millimeter-wave bands between 24 and 86 GHz for the deployment of future fifth mobile generation (5G/IMT-2020/NR) broadband systems.

Table1 IMT-2020 candidate bands in WRC-19 AI 1.13

| Candidate frequency bands | Candidate frequency bands of requirement additional conditions |
| --- | --- |
| 24.25-27.5 GHz | 31.8-33.4 GHz, |
| 37-40.5 GHz | 40.5-42.5 GHz |
| 42.5-43.5 GHz | 47-47.2 GHz |
| 45.5-47 GHz |  |
| 47.2-50.2 GHz |  |
| 50.4-52.6 GHz |  |
| 66-76 GHz |  |
| 81-86 GHz |  |

Moreover, in ITU region-3, the meeting of the APT Conference Preparatory Group for World Radiocommunication Conference 2019 (APG19-3) were held by Aisa-Pacfic Telecommunity (APT) to provide APT preliminary views on WRC-19 Agenda Items and issues associated with each WRC-19 Agenda Item.

With regarding WRC-19 Agenda Item 1.13, both China and Australia expressed their interesting bands beyond 52.6GHz:

- China is considering to identify the frequency bands above 43.5GHz especially 66-71GHz, 71-76 GHz and 81-86 GHz under AI 1.13 for IMT towards WRC-19, if sharing with relevant radio services is feasible depending on needs [7].

- Australia is also interested in investigating the possibility of an IMT identification in some or all of the 37-43.5 GHz and 66-76 GHz bands [8].

It is notable that it is possible to be updated views by administration, depending on further meetings.

Observation 1: China is considering to identify the frequency bands above 43.5GHz especially 66-71GHz, 71-76 GHz and 81-86 GHz under AI 1.13 for IMT towards WRC-19, if sharing with relevant radio services is feasible depending on needs.

Observation 2: Australia is interested in investigating the possibility of an IMT identification in some or all of the 66-76 GHz bands.

Proposal 2:  Preliminary views of beyond 52.6GHz of these countries for WRC-19 AI 1.13 can be considered in TR.

Conclusion

This contribution provides our survey results on available spectrum bands and regulatory requirements; and some potential available bands are shown in the discussion part.

The text proposals based on these discussions are included in Annex of this contribution. It is proposed to incorporate them into the TR of requirements for NR beyond 52.6 GHz.

Proposal 1: Text proposal on spectrum availability and regulatory requirements of China, Australia and Singapore are shown in the Annex.

Proposal 2:  Preliminary views of beyond 52.6GHz of these countries for WRC-19 AI 1.13 can be considered in TR.

Observation 1: China is considering to identify the frequency bands above 43.5GHz especially 66-71GHz, 71-76 GHz and 81-86 GHz under AI 1.13 for IMT towards WRC-19, if sharing with relevant radio services is feasible depending on needs.

Observation 2: Australia is interested in investigating the possibility of an IMT identification in some or all of the 66-76 GHz bands.

- References

- RP-182066, “Revised SID: Study on NR beyond 52.6GHz” Intel, RAN#81, Gold Coast, Australia, September 10-13, 2018

- RP-182147, “TR 38.807-004: Study on requirements for NR beyond 52.6 GHz”, Intel, RAN#81, Gold Coast, Australia, September 10-13, 2018

- MIIT, “Notice on wireless technical applications with very low transmission power (short distance) in 60GHz. 2006-082”  (关于60GHz 频段微功率（短距离）无线电技术应用有关问题的通知 2006-082) http://www.miit.gov.cn/n1146295/n1146592/n1146754/n1235566/n1235603/n1235609/n1235611/c3176263/part/3176264.pdf .

- MIIT, “Technical requiremeng of wireless devices with very low transmission power (short distance 2005-423).” (微功率（短距离）无线电设备的技术要求 2005-043) http://www.miit.gov.cn/n1146295/n1146592/n1146754/n1235566/n1235603/n1235609/n1235611/c3176241/part/3176242.pdf .

- ACMA (Australian Communications and Media Authority), “Radiocommunications (Low Interference Potential Devices) Class Licence 2015”, 2018. https://www.legislation.gov.au/Details/F2018C00500

- IMDA (Infocomm Media Development Authority), IMDA TS SRD, Aprail 2018, https://www.imda.gov.sg/-/media/imda/files/regulation-licensing-and-consultations/ict-standards/telecommunication-standards/radio-comms/imdatssrd.pdf?la=en

- APG19-3/INP-35, Australia, “PRELIMINARY VIEWS ON WRC-19 AGENDA ITEMS 1.13, 1.16, 9.1 (ISSUES 9.1.1, 9.1.5 AND 9.1.8)”, APG19-3, March, 2018, Australia

- APG19-3/INP-87, China, “PRELIMINARY VIEWS ON WRC-19 AGENDA ITEMS 1.13, 1.16, 9.1 (ISSUES 9.1.1, 9.1.5, 9.1.8)”, APG19-3, March, 2018, Australia

- FCC Part 95 Subpart M, The 76-81 GHz Band for Radar Service

- EN 301 091-1, Short Range Devices; Transport and Traffic Telematics (TTT); Radar equipment operating in the 76 GHz to 77 GHz range; Harmonised Standard covering the essential requirements under article 3.2 of Directive 2014/53/EU; Part 1: Ground based vehicular radar

- EN 302 567, Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard covering the essential requirements under article 3.2 of Directive 2014/53/EU

- EN 305 550, Electromagnetic compatibility and Radio spectrum Matters (ERM); Short Range Devices (SRD); Radio equipment to be used in the 40 GHz to 246 GHz frequency range; Part 1: Technical characteristics and test methods

Annex

Text proposal to TR

-------------Start of text proposal-----------------

- 4.1	Spectrum Availability

Editor’s Note: this section will describe available and potentially available spectrum in different regulatory domains/ITU-R. The section may be further broken down into countries of ITU region 1, 2, and 3.

- 4.2	Spectrum Regulatory Requirements

Editor’s Note: this section will describe available regulatory requirements for available spectrum. The section may be further broken down into ITU region 1, 2, and 3. TR 38.805 [3] can be a good reference for content for unlicensed bands between 54 ~ 71 GHz.

- 4.2.X   ITU Region 3

- 4.2.X.X1  China

# Table 4.2.x1. Regulatory Requirements for Available Spectrum between 52.6 GHz and 100 GHz in China

| Frequency band | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 59-64 GHz | Output power of antenna port: ≤ 10 dBm Maximum EIRP：47dBm Mean EIRP: ≤ 44 dBm | No requirement | Not specified | very low transmission power and short distant unlicensed applications | The unlicensed devices operating in 59-64 GHz shall not interfere to the space research, earth resources exploration, radio astronomy and other legal wireless applications, and shall not demand radio interference protection. Carrier frequency tolerance: 500 x 10-6 Out of Band Emission (EIRP): -5dBm/MHz Spurious Emission (corresponding to frequency range outside 2.5*carrier bandwidth ): -20dBm/1MHz |
| 76-77 GHz | Maximum EIRP：55dBm | No requirement | Not specified | Vehicle Radar ranging application | Spurious Emission (corresponding to frequency range outside 2.5*carrier bandwidth ): -20dBm/1MHz Spurious Emission in Idle/stand-by state (corresponding to frequency range outside 2.5*carrier bandwidth ): -47dBm/1MHz |

- 4.2.X.X2  Australia

# Table 4.2.x2. Regulatory Requirements for Available Spectrum between 52.6 GHz and 100 GHz in Australia

| Frequency band | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 61-61.5 GHz | Maximum EIRP：100 mW | No requirement | Not specified | Transmitters for non-specific applications |  |
| 59–63 GHz | Maximum EIRP：150 W The maximum transmitter power must not exceed 20 mW. | No requirement | Not specified | WiFi and RLAN transmitters | The transmitter must not be operated on board an aircraft. The transmitter must only be used outdoors. The transmitter must not cause spurious emissions outside the band at or greater than –30 dBm/MHz. |
| 57–66 GHz | Maximum EIRP：20 W | No requirement | Not specified | WiFi and RLAN transmitters | The transmitter must comply with FCC Rules Title 47 Part 15 Section 255. |
| 60-61 GHz | Maximum EIRP：20 mW | No requirement | Not specified | Radio determination: sensors using radar for measurement |  |
| 75-85 GHz | Maximum EIRP：75 nW | No requirement | Not specified | Same as 60-61GHz | The maximum EIRP applies outside the shielded room enclosure. The transmitter must meet the requirements of ETSI Standard EN 302 372. |
| 76-77 GHz | Maximum EIRP：25 W | No requirement | Not specified | Same as 60-61GHz |  |
| 77-81 GHz | See Note1 below | No requirement | Not specified | Same as 60-61GHz | The transmitter must meet the requirements of ETSI Standard EN 302 264. The transmitter must not be operated within a nominated distance of a specified Australian radio-astronomy site. |
| 57-64 GHz /75-85 GHz | See Note2 below | See Note3 below | Not specified | Same as 60-61GHz | The transmitter must be operated in a position such that emissions are directed towards: (i) the ground; or (ii) the floor or a wall of a building or similar structure. The transmitter must comply with ETSI Standard EN 302 729. The transmitter must not be operated within a nominated distance of a specified Australian radio-astronomy site. |

# Note1: Section 4.3.3.3 in ETSI EN 302 264 V2.1.1

The peak power for EUT with fixed beam or scanning antenna shall not be greater than 55 dBm.

# Note2: Section 4.3.4.3 in ETSI EN 302 729 V2.1.1

# 57-64 GHz: Maximum peak power limit EIRP is 35dBm, measured in 50MHz within main beam.

# 75-85 GHz Maximum peak power limit EIRP is 34dBm, measured in 50MHz within main beam.

# Note3: Section 4.7 in ETSI EN 302 729 V2.1.1

A reduction of 20dB shall be achieved by possible mitigation factors, including the APC(Adaptive power control),   activity factor and duty cycle, frequency domain mitigation, shielding effects.

- 4.2.X.X3  Singapore

# Table 4.2.x3. Regulatory Requirements for Available Spectrum between 52.6 GHz and 100 GHz in Singapore

| Frequency band | Power/Magnetic Field Requirements | Spectrum access and mitigation requirements | Modulation / maximum occupied bandwidth | Purpose/Node Placement requirements | Notes |
| --- | --- | --- | --- | --- | --- |
| 76 – 77 GHz Note1 | Maximum EIRP： 23.5 dBm when stationary Maximum EIRP： 37 dBm when vehicle is in motion | No requirement | Not specified | Short range radar systems such as automatic cruise control and collision warning systems for vehicle | A short range radar system is a movement and position detection device which is used to give a warning of collision by identifying the delay between a transmitted pulse and a return pulse. Standards for Measurement Methods FCC Part 95 Subpart M; or EN 301 091-1 Transmitter Spurious Emissions limit: -30dBm for f≥1GHz, refer to ITU-R Rec. SM.329-12 §4.3, Table 3 Category B limits |
| 57 – 66 GHz Note1 | Maximum EIRP： 10 W Indoor use is restricted to maximum mean EIRP density of 13 dBm/MHz Outdoor use is restricted to max e.i.r.p. of 25 dBm and max e.i.r.p. power spectral density of -2 dBm/MHz | No requirement | Not specified | Wireless LAN and broadband access | Standards for Measurement Methods EN 302 567 EN 305 550 Transmitter Spurious Emissions limit: -30dBm for f≥1GHz, refer to ITU-R Rec. SM.329-12 §4.3, Table 3 Category B limits |
| 71-76 GHz 81-86 GHz |  |  |  | fixed wireless system | See Note 2 for specific information |

Note1: The development of spectrum allocations for short range devices is an ongoing policy review process. IMDA will revise and update its frequency allocations for short range devices as new technologies emerge.

Note2 : IMDA will make available 71 – 76 GHz paired with 81 – 86 GHz of the E-band for commercial wireless applications such as fixed-wireless backhaul and inter-building fixed-wireless network. The spectrum will be available for short-term commercial use of up to 12 months until 31 December 2019. IMDA will monitor market and technology developments of the E-band, and take into consideration the World Radiocommunication Conference 2019 recommendations before allocating the spectrum in this band beyond 2020.

-------------End of text proposal-----------------